3GPP TSG RAN WG1 #111		                                                                 R1-2212199

Toulouse, France, November 14th - 18th, 2022

Title:	Draft reply LS on SL LBT failure indication and consistent SL LBT failure

Response to:	R1-2210805/R2-2210936

Release:	Rel-18

Work Item:	NR_SL_enh2-Core

Source:	[RAN1], ZTE, Sanechips

To:	RAN2

Cc:

Attachment:	-

Contact Person:

Name:	Yuzhou Hu

E-mail Address:	huyuzhou@zte.com.cn

Overall description

RAN1 thanks RAN2 for the LS (R1-2210805/R2-2210936) on SL LBT failure indication and consistent SL LBT failure. In the LS, the following question related to the SL LBT failure indication was raised by RAN2:

Question: When SL LBT failure is notified by PHY due to an intended SL transmission, what is the granularity in which MAC can consider that the SL LBT failure has been detected (e.g. whether MAC can consider that the SL LBT failure has been detected per SL BWP, per SL resource pool, per RB set, etc.).

Answer: RAN1 has discussed channel access procedures for SL transmission(s) in last few meetings, and agreed that NR-U UL channel access procedure is considered as baseline for transmission on multiple channels, as shown in following RAN1 agreements. Based on the understanding, SL LBT should be performed for an intended SL transmission on one or more RB sets in a resource pool. Accordingly, SL LBT failure would be notified by PHY due to LBT detection per SL transmission. Considering that a SL transmission could be on one or more RB sets, thus an SL LBT failure instance indicated in the granularity of RB set does not make sense. Moreover, only one SL BWP is configured on a SL-U carrier, in contrast, more than one resource pools could be configured with a SL BWP. And for a SL transmission, it could be performed in a selected/indicated resource pool, thus the granularity of resource pool for SL LBT failure indication for PSSCH/PSCCH transmission is appropriate.

Agreement(RAN1#109-e) Channel access procedures for transmission(s) on multiple channels are supported for NR sidelink operation as defined by TS37.213 for NR-U (wherever applicable) - FFS whether the downlink, uplink and/or semi-static multiple channel access procedure(s) (if supported) from NR-U should be used as a baseline and whether/how they are applied in SL mode 1 and mode 2 operation Agreement(RAN1#110bis-e) For dynamic channel access mode with multi-channel case in SL-U, NR-U UL channel access procedure is considered as baseline for transmission on multiple channels - FFS: whether transmission of PSFCH and/or S-SSB on a subset of RB sets is supported (using the NR-U DL channel access procedure as baseline) - FFS any necessary enhancement and modification for the SL-U operation

Actions:

To RAN2:

RAN1 respectfully requests RAN2 to take above response on the granularity of SL LBT failure indication into account for the future work.

Date of Next TSG-RAN1 Meetings:

RAN1#112			27th Feb -3rd Mar 2023				Athens, GR

RAN1#112bis-e			17th -26th Apr 2023	   			E-Meeting