3GPP TSG RAN WG1 #111	R1-2212263

Meeting, November 14th – 18th, 2022

Source: 	MediaTek Inc.

Title: 	L1 signal design and procedure for low power WUS

Agenda item:	9.13.3

Document for:	Discussion

# Introduction

RAN#97-e endorsed the revised SID [1] for LP WUS/WUR. Details of SID are in the appendix. One main objective is to study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals.

This contribution provides our views on L1 designs and procedures to support LP-WUS.

# L1 Designs

L1 designs depend on a system structure, including essential components required at the transmitter and receiver. Figure 1 shows a signal flow for OFDM-based multi-carrier on-ff keying (MC-OOK). Descriptions are below.

Figure 1: the signal flow of the OFDM-based MC-OOK system

Wake-Up Indication (including group ID)

LP-WUS can use a preamble sequence and/or a payload to deliver group ID. It is FFS how many groups or candidates of different wake-up indications/functionalities decide the required sequence number or bit number. It is noticed that there is a general trade-off in information amount and the required resource overhead.

Modulation and sequence generation

OOK demodulation requires an SNR estimate for an optimal threshold, which is complex to LP-WUR. Manchester encoding can simplify the demodulation by comparing the two consecutive received signals. In addition to OOK with Manchester code, pulse position modulation (PPM) is another scheme that can allow energy-based detection, where the UE detects the information bits from the identified pulse position. Combining modulation schemes (OOK and PPM) is also possible to optimize the detection performance in multiple path channels. Note that both modulations require a timing reference for correction decisions on the bit content. A set of OOK/PPM sequences can be designed to carry more information, subject to some auto-correlation and/or cross-correlation requirements. It is FFS whether/how to achieve the joint modulation and sequence design or utilize a preamble and payload structure in WIFI.

RE mapping

RE mapping may exclude the DC subcarrier (LTE and 802.11ba WUS do not use the DC) because LP-WUR may suffer interference from local oscillator (LO) leakage. However, if there is no LO at the WUR, the DC issue may not exist. Another need is introducing guard bands (GB) to relax accuracy for a low complexity LO or a low pass filter (LPF). With this regard, gNB may need to reserve some REs for no NR data and no LP-WUS.

Waveform generation

Waveform generation may use an FFT matrix (DFT-based OOK). A gNB allocates the OOK modulation following the RE mapping rule and uses waveform shaping and IFFT to generate the OOK waveform. Waveform shaping before IFFT can prevent inter-symbol interference (ISI) from and to NR data allocated in the same OFDM symbol. However, adding CP may break the OOK sequences if the sequence is longer than one OFDM symbol. It is FFS whether/how CP is removed and reused while keeping simple LP-WUR demodulation.

Mixer

A mixer impacts interference resilience. For example, a high-quality mixer supports a narrow-band LPF, achieving good interference suppression. However, the high-quality mixer potentially consumes more power.

Low pass filter (LPF)

An LPF impacts interference resilience. The narrow bandwidth of LPF results in better interference suppression but potentially more power consumption. LLS needs to determine a proper filer order.

ADC

An N-bit ADC impacts receiver sensitivity and interference resilience. The ADC needs to determine a sampling rate and the LP WUS bandwidth.

Envelope detector

The envelope detector (ED) converts any input frequency to DC. A DC blocker usually follows the ED to filter out the DC component. However, the need for the DC block can be FFS.

Wake-up indication/ID detector

The detailed design depends on the design of wake-up indications (including grouping design) and the method the information bits are carried (e.g., by OOK or PPM).

Preamble sequences and/or a payload can be considered to deliver group ID for LP-WUS

OOK with Manchester code and pulse position modulation (PPM) can be considered to simplify demodulation by allowing energy-based detection.

Guard bands can be considered to reduce inter-subcarrier interference and relax the receiver architecture requirements.

# L1 Procedures

## LP-WUR-Assisted Serving-Cell Measurement Relaxation

UE performs serving cell measurement in every DRX for link maintenance purposes. On the other hand, if the main receiver is waked-up for serving cell measurement every DRX cycle, there is no power-saving benefit with LP-WUR. In this regard, it should be investigated how to reduce the main receiver's serving cell measurements with the aid of LP-WUR. For example, after MR detects good serving cell quality, MR can relax serving cell measurement while having LP-WUR monitor whether a (coarse) serving cell quality becomes interior such that LP-WUR wakes up MR to perform normal serving cell measurements (no relaxation).

Evaluation

Assume LP-WUS is applied for paging monitoring. PO monitoring is only required when LP-WUS is received, which occurs with low probability (no paging assumption). We compare the following two cases:

- Baseline: Assume no measurement relaxation and no LP-WUR. MR measures 1 SSB and monitors 1 PEI.

- Case1: No meas. relaxation is on the serving cell. MR measures 1 SSB, and LR monitors 1 LP-WUS per DRX cycle.

- Case 2: K-times meas. relaxation is on the serving cell. MR measures 1 SSB every K DRX cycle, and LR monitors 1 LP-WUS per DRX cycle.

The UE processing timeline for case 1 is in Table 1, and the power model is in Table 2 in the appendix.

Table 1: MR and LR operations in a paging cycle for high SNR in case 1

| UE operations in a paging cycle | Time duration (ms) | Energy contribution (relative power * ms) | Example values |
| --- | --- | --- | --- |
| LP WUR ON | 1280 | PLP-WUR-ONNote1*1280 | 64 |
| SSB processing and intra-frequency RRM measurement | 2 | (PSSB + (Pintra, search+meas * [1/4] + Pintra, meas-only * (1 – [1/4]) ) ) * 0.85 * 2 | 446.25 |
| Deep sleep | 1266 | PDS * 1266 + 450 | 1728 |
| (Total) | 1280 | Total Energy | 2174.3 |
| Average power consumption = Total Energy / 1280 |  |  | 1.70 |
| Note 1: Assume LP-WUR is always ON with a relative power of 0.05. |  |  |  |

| Scheme | Deepest sleep | Average power | Gain |
| --- | --- | --- | --- |
| no meas. relax, no LP-WUR | deep sleep | 1.92 | N/A |
| no meas. relax |  | 1.75 | 9% |
| 4 x meas. relax | deep sleep | 1.22 | 36% |
|  | ultra-deep sleep | 2.39 | -25% |
| 12 x meas. relax | deep | 1.11 | 42% |
|  | ultra-deep sleep | 1.03 | 46% |
| 20 x meas. relax | deep | 1.08 | 43% |
|  | ultra-deep sleep | 0.76 | 60% |

We observe a 36% power-saving gain when four-times meas. relaxation is allowed in the serving cell. The most PS gain comes from skipping SSB measurement. However, ultra-deep sleep takes 12 times measurement relaxation to outperform deep sleep due to additional transition energy for ramping up and down (value: 10000).

WUR-assistant measurement relaxation provides a 36% PS gain when meaaurement can relax four times.

LP-WUR-assisted meas. relaxation to prolong the main receiver's sleep time can be considered.

## LP-WUR-Assisted SSSG Switching

In the RRC-connected mode, UE can monitor DCP to know whether to start the on-duration timer of the upcoming DRX cycle. On the other hand, DCP cannot be used to adjust PDCCH monitoring behavior during the on-duration time, or there is no DRX configured. In the use case of XR with large timing jitter (e.g. [-4 ms, 4 ms]), a large on-duration timer may be used, or there will be no DRX configured, and the DCP solution cannot reduce UE power consumption for such use cases. An alternative is to reuse the Rel-17 PS adaptation (SSSG switching). A gNB can configure one UE-specific SSSG for LR to monitor LP-WUS and another SSSG for MR to monitor DCI.

Evaluation

Suppose UE monitors LP-WUS in SSSG#0 via LP-WUR and monitors PDCCH in SSSG#1 via MR. In SSSG#0, MR enters micro sleep, and LR stays ON. In SSSG#1, MR performs PDCCH monitoring (and PDSCH processing), and LR stays OFF. UE switches from SSSG#0 to SSSG#1 if LP-WUR detects a wake-up indication. If there is no new data after the SSSG timer (4 ms) in SSSG#1, UE switches to SSSG#0. Settings and scenarios are in the appendix.

Figure 2: LP-WUS assistant SSSG switching for XR traffic

| # | Power saving scheme | PS Gain (w.r.t Rel-17) |  |  |
| --- | --- | --- | --- | --- |
|  |  | LP-WUS 0.005 | LP-WUS 0.05 | LP-WUS 4 |
| 1 | SSSG switching (XR 30 Mbps) | 19.02% | 19.01% | 17.84% |
| 2 | SSSG switching (XR 45 Mbps) | 17.58% | 17.56% | 16.30% |

About 16%-19% can be observed using LP-WUS-assisted SSSG switching w.r.t. Rel-17 SSSG switching. The PS gain highly depends on the time UE stays in SSSG#0, where MR enters micro sleep. On the other hand, power consumption for LP-WUS monitoring only contributes a few. For example, there is no significant difference if LP-ON takes a power value of 0.005 or 0.05.

Consider LP-WUR-assisted SSSG switching, where UE is configured with one SSSG for LR to monitor LP-WUS and another SSSG for MR to monitor DCI.

LP-WUR-assisted SSSG switching shows 16% to 19% PS gains compared to R17 SSSG switching.

# Conclusion

In this contribution, we have the following observations and proposals.

Proposal 1	Preamble sequences and/or a payload can be considered to deliver group ID for LP-WUS

Proposal 2	OOK with Manchester code and pulse position modulation (PPM) can be considered to simplify demodulation by allowing energy-based detection.

Proposal 3	Guard bands can be considered to reduce inter-subcarrier interference and relax the receiver architecture requirements.

Observation 1	WUR-assistant measurement relaxation provides a 36% PS gain when meaaurement can relax four times.

Proposal 4	LP-WUR-assisted meas. relaxation to prolong the main receiver's sleep time can be considered.

Proposal 5	Consider LP-WUR-assisted SSSG switching, where UE is configured with one SSSG for LR to monitor LP-WUS and another SSSG for MR to monitor DCI.

Observation 2	LP-WUR-assisted SSSG switching shows 16% to 19% PS gains compared to R17 SSSG switching.

# Reference

RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, vivo

IEEE 802.11-17/1037r0, Performance Evaluation of OOK Waveform Coding Schemes with Impairments, InterDigital

# Appendix

## Revised SID: study on LP WUS/WUR for NR

The study item includes the following objectives:

- Identify evaluation methodology (including the use cases) & KPIs [RAN1]

  - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables

    - Other use cases are not precluded

- Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4]

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals  [RAN2, RAN1]

- Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1]

  - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

## Evaluation Models for LP-WUR-Assisted Serving-Cell Measurement Relaxation

The power model is provided in this subsection.

Table 2: MR and LR power models

| Item | Power | Transition time (ms) | Transition energy |
| --- | --- | --- | --- |
| Ultra-deep sleep, PUDS | 0.3 | 400 | 10000 |
| Deep sleep, PDS | 1 | 20 | 450 |
| LP-WUR ON, PLP-WUR-ON | 0.05 |  |  |
| SSB proc., PSSB | 100 |  |  |
| RRM search and measurement, Pintra, search+meas | 200 |  |  |
| RRM measurement, Pintra, meas-only | 100 |  |  |

## Evaluation Models for LP-WUR-Assisted SSSG Switching

The power model and simulation parameters are in this subsection.

| Simulation parameters | Value |
| --- | --- |
| Deployment | 3GPP Dense Urban, 7 sites, 3 sectors per site, ISD = 200m 80% indoor UE, 20% outdoor UE |
| Channel model | 3D-UMa |
| Channel estimation | Ideal |
| SCS | 30 kHz |
| Frame structure for TDD | DDDSU |
| Scheduling scheme | Proportional fair |
| Bandwidth | 100MHz; 2.08% guard band (272 360kHz RB in 100MHz) |
| BS Tx antenna config. | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) with 64 TXRU (dH, dV) = (0.5λ, 0.5λ) Down tilt: 12 degrees SU-MIMO |
| BS height | 25 m |
| UE Rx antennas config. | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) 2T/4R (dH, dV) = (0.5λ, N/A) |
| UE height | 1.5 m |
| UE speed | 3 km/h |
| Antenna Gain | BS: 8 dBi, 3-sector antenna UE: 0 dBi, omni-directional |
| Noise figure | BS: 5dB; UE: 9dB |
| Max Tx power | BS: 44 dBm per 20 Mhz UE: 23 dBm |

| Traffic | AR/VR (Follow the setting in TR 38.838) |  |
| --- | --- | --- |
| Data rate (Mbps) | 30 | 45 |
| PDB (ms) | 10 |  |
| Packet size distribution | Truncated Gaussian |  |
| mean packet size (Bytes) | 62500 | 93750 |
| Ratio: STD/mean pkt size | 0.105 |  |
| Ratio: Max/mean packet size | 1.5 |  |
| Ratio: min/mean packet size | 0.5 |  |
| Packet arrival rate (sec) | Periodic (1/FR), FR = 60 |  |
| Jitter value distribution | Truncated Gaussian |  |
| Avg. jitter value (ms) | 0 |  |
| Jitter value STD (ms) | 2 |  |
| Max jitter value (ms) | 4 |  |
| Min jitter value (ms) | -4 |  |

| Basic power consumption for PDCCH/LP-WUS monitoring (uint) |  |
| --- | --- |
| PDCCH monitoring in SSSG 0/1 | 100 (PDCCH monitoring in TR 38.840) |
| LP-WUS monitoring | 45 (micro sleep power in TR 38.840) |
| Additional power consumption for LR (unit) |  |
| LP-WUS off | 0.001 |
| LP-WUS on | 0.005, 0.05, 4 |