3GPP TSG RAN WG1 #111	R1-2212418

Toulouse, Nov 14-18th, 2022

Agenda item:		9.13.2

Source:	Nordic Semiconductor ASA

Title:					On LP-WUS architecture

Document for:		Discussion and Decision

# Introduction

In RAN#97, study item in RP-222644 has been approved. And 3 different architectures have been agreed.

Agreement Study the architecture with RF envelope detection based on at least the following diagram for LP-WUR. - The RF signal is converted into baseband signal directly via an RF envelope detector. - There is no Local Oscillator (LO) and no Phase-Locked Loop (PLL). - 1-bit or multi-bit ADC is applied. - Some component(s), e.g., RF LNA and/or BB AMP, can be optionally applied. - High-Q matching network and/or RF BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers. - FFS the support of band and/or carrier tuning Agreement Study the heterodyne architecture with IF envelope detection based on at least the following diagram for LP-WUR. - The RF signal is down converted into IF signal via an RF mixer with a LO. The IF signal is converted into baseband signal via an IF envelope detection. - There may be one or multiple IF stages depending on design. - The choice of the LO is one of the major factors that determines the power consumption. - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such increased frequency offset and phase noise should be taken into account in the design and evaluation. - FLL (frequency locked loop) may replace PLL for non-coherent detection. - 1-bit or multi-bit ADC is applied. - High-Q matching network and/or RF BPF and/or IF BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers. - Some component(s), e.g., RF LNA and/or IF AMP and/or BB AMP, can be optionally applied. - Image rejection filter or an image rejection mixer is required. - FFS the support of band and/or carrier tuning - FFS the choice of IF frequency range Agreement Study the homodyne/zero-IF architecture with baseband envelope detection based on at least the following diagram for LP-WUR. - The RF signal is directly down converted into baseband signal via an RF mixer with a LO. - Baseband envelope detection can be done either in analog domain or in digital domain depending on design, which is not explicitly shown in the diagram. - The choice of the LO is one of the major factors that determines the power consumption. - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such increased frequency offset and phase noise should be taken into account in the design and evaluation. - FLL (frequency locked loop) may replace PLL for non-coherent detection. - 1-bit or multi-bit ADC is applied. - High-Q matching network and/or RF BPF and/or BB BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers. - No image rejection filter is required. - Some component(s), e.g., RF LNA and/or BB AMP, can be optionally applied. - FFS the support of band and/or carrier tuning

# Discussion

## On High Q filters and matching networks tuning

While different architectures to receive wake-up signals exist, not all are suitable for cellular deployments. To be more precise, architectures where RFFE requires High Quality RF filter (e.g. SAW) and/or matching network, for every possible location/ARFCN should be avoided. Tunable HQ-RF filters can be utilized. However, such systems require an oscillator, if tuning between bands is to be supported. Otherwise, separate components must be used per each band and components are not typically implemented in CMOS.

Moreover, to design a filter and/or matching network with narrow BW and high selectivity, high order/complexity filters are required,

Observations-1: Designs including non-tunable HQ-RF filters are not suitable for cellular systems. Designs assuming those should not be further considered.

Observation-2: High selectivity RF filters would be of higher-order, which are complex and power consuming and are not implemented in CMOS.

## On details of architectures

Matching network

Passive components can be used with negligible power consumption. Matching network can be tuned between the bands using oscillator, if available in the architecture. Such several bands can be covered by the same set of components and components can be implemented in CMOS.

Mixer->IF-Amp. vs LNA->Mixer

Mixer followed by IF/baseband amplifier consumes less power and takes less space compared to placing an LNA amplifier before mixer. On the other hand, Noise figure of mixer is worse than that of LNA. Therefore, LNA is often used up front to suppress noise in the following stages. For comparison, a mixer with CS IF amplifier consumption could consume 32uW and have ~7 dB NF, while CMOS implementable LNA would consume in an order of 1mW having ~3dB NF.

IF Filter to filter LP-WUS

Low-power filter implemented in CMOS could be e.g. a 2nd-order gm-C active filter, such filter could attenuate signals by -40dB at 10MHz from DC. Assuming LP-WUS is placed in the middle of carrier, it could be sufficient for rejecting an in-band blocker. Such filter could consume 10uW power.

Choice of oscillator

Oscillator is the main power-hungry element of the receiver. LC/DC-VCO can full-fill in-band blocking requirements, to ensure that receiver is not SNR-limited irrespective of signal strength coming in. On the other hand, power consumption is >400uW.

When using unlocked ring oscillator instead of LC/DC-VCO phase noise increases. Whether NR in-band requirements can be met is not clear. As mentioned, increased phase noise will mix ACI to own channel, flooring the SNR.

On the other hand, power consumption of unlocked ring oscillator is smaller, in order of 160uW (assuming high frequency such as 4GHz, number is smaller in lower bands).

In addition, XO needs to be turned on periodically, to maintain unlock ring oscillator within <300ppm error, this consuming part of above power budget. Frequency error, as discussed in our accompanying contribution, creates an uncertainty about location of LP-WUS in IF/Baseband.

Variable gain Amplifier

Variable gain amplifier could be placed still after filter to amplify signal to correct level for ADC, this to allow for ADC with less dynamic range, but better power consumption. Consumption of additional CS VGA could be 5uW.

ADC

Low power SAR 4Msample/sec 10-bit ADC with consumption of 32uW could be considered.

To summarize: Heterodyne architectures power consumption for RFFE could be somewhere around 250uW plus 50uW for power management. However, feasibility of using unlock ring oscillator will depend heavily on blocking requirements defined for LP-WUS and target SNR. Therefore, when sending LS to RAN4, RAN1 should ask RAN4 about what the in-band blocking requirements should be. RAN4 can consider as well an option, where LP-WUS is restricted to be located in the middle of carrier.

Observation-3: Power consumption for LP-WUR RFFE could be somewhere 250uW plus 50uW for power management.

Observation-4: Feasibility of using unlock ring oscillator depends highly on in-band blocking requirements and target SNR.

Proposal-1: When sending LS to RAN4 with agreed architectures, ask what the in-band blocking requirements should be for LP-WUS.

- RAN4 to consider an option as well, where LP-WUS location is restricted to middle of carrier.

# Conclusions

In this contribution we discussed issues related to LP-WUS architecture and we had the following observations and proposals:

Observations-1: Designs including non-tunable HQ-RF filters are not suitable for cellular systems. Designs assuming those should not be further considered.

Observation-2: High selectivity RF filters would be of higher-order, which are complex and power consuming and are not implemented in CMOS.

Observation-3: Power consumption for LP-WUR RFFE could be somewhere 250uW plus 50uW for power management.

Observation-4: Feasibility of using unlock ring oscillator depends highly on in-band blocking requirements and target SNR.

Proposal-1: When sending LS to RAN4 with agreed architectures, ask what the in-band blocking requirements should be for LP-WUS.

- RAN4 to consider an option as well, where LP-WUS location is restricted to middle of carrier.