3GPP TSG-RAN WG1 Meeting #111	R1-2212482

# Toulouse, France, November 14 – 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.213 | CR | xxxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on coexistance between Rel-16 and Rel-17 H/S/NA configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eIAB_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following two cases for Rel-17 dynamic indication of availability needs to be clarified in case only Rel-16 H/S/NA resource configuration is provided for a given a slot: - Case 1: If Rel-16 dynamic indication of availability is not configured by RRC or not provided by DCI, and Rel-17 dynamic indication of availability indicates the availability of RB sets of this slot - Case 2: If both Rel-16 and Rel-17 dynamic indication of availability are configured and received by IAB-MT for this this slot. In both cases, the IAB-MT shall ignore the Rel-17 dynamic availability indication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the IAB-MT ignores the Rel-17 dynamic availability indication in case only Rel-16 H/S/NA resource configuration is provided for a given a slot. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Parent node mistakenly assumes the H/S/NA configuration applied by child node DU when both Rel-16 and Rel-17 H/S/NA configuration or both Rel-16 and Rel-17 dynamic resource availability provided. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: No inter-operatability issue is identified. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

14	Integrated access-backhaul operation

========================= Unchanged parts =========================

With reference to slots of an IAB-DU cell, the IAB-DU can be provided an indication of hard, soft or unavailable type per RB set for symbols configured as downlink, uplink or flexible in a slot by Frequency-Domain HSNA Configuration List [16, TS 38.473]. The RB set size and the number of RB sets are configured by RB-Set-Configuration [16, TS 38.473]. If an indication of hard, soft or unavailable type is not provided for an RB set of a symbol in a slot, the IAB-DU applies the configuration of hard, soft or unavailable type provided by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] for the RB set of the symbol in the slot. If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by Frequency-Domain HSNA Configuration List [16, TS 38.473] unless the symbol is configured as soft type in HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] and the IAB-DU cell can transmit or receive in the symbol.

If an IAB-DU is only provided an indication of hard, soft or unavailable type for a symbol in a slot by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473], the IAB-MT shall ignore a DCI format 2_5 with an AI index field value indicating availability of soft RB sets of the symbol in the slot.

< Unchanged parts are omitted >