3GPP TSG RAN WG1 #111			R1-2212499

Toulouse, France, November 14th – 18th, 2022

Agenda Item:	5

Source:	Ericsson

Title: Draft reply  LS on Positioning Reference Units

Document for:	Discussion, Decision

- Discussion

The SA2 LS in R1-2210824 consists of the following questions:

- Question#1: Whether there is additional information that needs to be stored in the core network? If yes, what is the information?

- Question#2: Whether a PRU can be allowed to support the positioning signal transmission capability and signal measurement capability on PC5?

In our view, RAN1 is not competent to answer question 1 and should leave it to RAN3 or RAN2 to reply. Regarding question 2, from the RAN1 perspective it has been agreed that a PRU is a UE. Hence, our understanding is that   positioning signal transmission capability should be consisting of SRS transmission or, for sidelink, SL PRS transmission capability.  For signal measurement, similar to other UE, a PRU may measure downlink PRS or SL PRS.

Proposal 1: For the reply to SA2 to questions in R1-2210824:

- For question 1:, RAN1 is not the competent working group and suggest RAN3 or RAN2 takes up the question.

- For question 2: from the RAN1 perspective, a PRU is a UE.  Positioning signal transmission capability consists of SRS transmission or, for sidelink, SL PRS transmission capability.  For signal measurement, similar to other UEs, a PRU may measure downlink PRS or SL PRS.

- Draft reply LS

3GPP TSG RAN WG1 #111			R1-22NNNN

Toulouse, France, November 14th – 18th, 2022

Title:	Draft reply LS  on Positioning Reference Units

Response to:	R1-2210824

Release:	Rel-18

Work Item:	FS_eLCS_Ph3

Source:	RAN1

To:	SA2

Cc:	RAN2, RAN3

Contact Person:

# Name:	                Florent Munier

E-mail Address:	 florent.munier@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

1. Overall Description:

RAN1 thanks SA2 for its questions. RAN1 has the following answers:

- For question 1:, RAN1 is not the competent working group and suggest RAN3 or RAN2 takes up the question.

- For question 2: from the RAN1 perspective, a PRU is a UE.  Positioning signal transmission capability consists of SRS transmission or, for sidelink, SL PRS transmission capability.  For signal measurement, similar to other UEs, a PRU may measure downlink PRS or SL PRS according to its capabilities.

2. Actions:

To SA2

ACTION:

# RAN1 kindly requests SA2 to take the above information into  account.

3. Date of Next RAN1  Meetings:

TSG RAN WG1 Meeting #112		27 February – 3 March 2023		Athens GR