3GPP TSG RAN WG1 #111		                                                      R1-2212733

Toulouse, France, November 14th – 18th, 2022

Source:	Moderator (CATT)

Title:	Summary #1 of subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for:	Discussion and Decision

- Introduction

This document summarizes the inputs and the discussions on subband non-overlapping full duplex in RAN1#111.

- Proposals for online sessions

  - Nov. 15th (Tue)

Proposal 1-5b

Proposed Agreement:

For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

  - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived

  - FFS: UE does not expect to receive or transmit in the RBs which are not within DL subband(s) or UL subband

- DL receptions within DL subband(s) are allowed in the symbol

- FFS whether DL receptions within UL subband are allowed or not in the symbol

- FFS whether/how the symbols can be converted to DL-only symbols in the symbol

- Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol

Proposal 1-4a

Proposed Agreement:

For semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL subband is indicated with reference to CRB grid.

- If frequency location of DL subband(s) is explicitly indicated, the frequency location of DL subband(s) is indicated with reference to CRB grid.

- General aspects of SBFD schemes

This section discusses the general aspects of SBFD schemes except self-interference, inter-gNB and inter-UE CLI handling schemes.

  - Summary of input contributions

The inputs from companies’ contributions on SBFD operation except self-interference, inter-gNB and inter-UE CLI handling schemes are summarized below as per moderator’s understanding. Moderator would like to apologize in advance if your views are not correctly captured or are missed. Companies are encouraged to correct/update the summary with revision marks if needed.

    - Subband location indication

The following agreement was made in RAN1#110bis-e that for baseline SBFD operation both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

Agreement For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline. - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

      - Semi-static configuration of subband time and frequency location

The following agreements were made in RAN1#110 to study semi-static configuration of subband time and frequency location as baseline.

Agreement For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline.

        - Cell-common vs. UE-specific subband location indication

ZTE, CATT, Pansonic, Qualcomm, MediaTek, Nokia and CEWiT propose that semi-static configuration of subband time and frequency location should be cell-common. Spreadtrum think cell-common frequency location of subbands can be the baseline.

Vivo think that the UL subband is common to all SBFD aware UEs served by the TDD carrier and can be configured in carrier/serving cell level. But it is also feasible to configure the UL subband in BWP level and it is easier and straightforward to apply the subband division within the active BWP pair from UE’s perspective so vivo proposed to discuss and determine whether subband(s) are configured in carrier/serving cell level or BWP level.

ZTE additionally proposed to FFS whether UE-specific UL subband configuration is needed, e.g. for UEs with restricted bandwidth capability (e.g., Redcap UE), they may only support UL transmission on partial bandwidth of the UL subband. In this case, UE-specific UL subband frequency resource configuration is beneficial.

Ericsson think frequency location of subband can be cell-common since the most common mode of operation would be that the subband locations are the same for all UEs. If there is a need to have slightly different guardband sizes for different UEs, then dedicated signaling could be introduced to indicate the frequency domain information.

ETRI and Xiaomi prefers UE-specific RRC signaling to configure SBFD subband time and frequency locations considering various possibilities on the UE implementations for CLI mitigation and/or different traffic.

Based on companies’ views, majority companies prefer cell-common configuration of subband location so an initial proposal in Proposal 1-1 is provided to agree cell-common indication of subband time and frequency location as the baseline.

        - Subband time location indication

The following agreements were made in RAN1#110bis-e for semi-static configuration of subband time locations for SBFD operation.

Agreement For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

Spreadtrum, OPPO, xiaomi, Ericsson, Qualcomm, Sharp propose that the period for SBFD operation is based on the periodicity of the TDD UL-DL pattern(s).

Vivo discussed the period determined by periodicity of the configured TDD pattern and by a new configured periodicity.

Xiaomi discussed the following four cases and observed that TDD periodicity provided by tdd-UL-DL-ConfigurationCommon cannot be directly reused as the period for configuring SBFD subband time domain location.

- Case#1: TDD frame structure with single periodicity is configured by tdd-UL-DL-ConfigurationCommon only

- Case#2: TDD frame structure with two periodicities is configured by tdd-UL-DL-ConfigurationCommon only

- Case#3: TDD frame structure is configured by tdd-UL-DL-ConfigurationCommon, and  tdd-UL-DL-ConfigurationDedicated additionally

  - Case#3-1: single periodicity

  - Case#3-2: dual periodicity

- Case#4: Network doesn’t provide any TDD UL-DL configuration, i.e. all the symbols are regarded as semi-static flexible symbol

According to moderator’s understanding, the period determined by the periodicities of TDD UL-DL pattern(s) seems to be straightforward. In case two TDD UL-DL patterns are provided, the SBFD subband time locations can be separately configured within the periodicity of each TDD UL-DL pattern. Given that it is proposed that cell-common indication of subband time and frequency location is proposed as baseline in Proposal 1-1, only TDD UL-DL pattern(s) configured in TDD-UL-DL-ConfigCommon should be considered. Considering that it is possible that TDD-UL-DL-ConfigCommon is not provided from specification point of view, explicit configuration should be supported in this case. An initial proposal in Proposal 1-2 is provided.

New H3C, TCL, ZTE, Qualcomm proposed that consecutive SBFD symbols are configured within a period to reduce the switching between SBFD and non-SBFD symbols. Xiaomi and MediaTek support non-contiguous SBFD symbols within a period. Spreadtrum and LG think both continuous and non-contiguous slots in a period can be discussed.

In addition, Spreadtrum think at least slot level time location can be supported and symbol level can be with lower priority. Samsung think SBFD subband(s) with symbol level granularity at least for RRC_CONNECTED mode is desirable in terms of forward-compatibility and design flexibility.

From moderator’s understanding the above details can be discussed and decided later.

Companies’ views on the time domain locations of UL subband are summarized below. Majority companies support both symbols configured as DL and flexbile in TDD-UL-DL-ConfigCommon. Panasonic think that in legacy flexible symbols, non-SBFD aware UEs (including legacy UEs) can transparently utilize DL subband and UL subband based on gNB’s scheduling. gNB is also allowed not to restricted to DL subband and UL subband in legacy flexible symbol. If SBFD aware UE takes into account both time and frequency locations of subbands and non-SBFD aware UE does not take into account such subbands in legacy flexible symbol, the gNB scheduling operation is more complex. So it is proposed that in legacy flexible symbols, time and frequency locations of subbands for SBFD operation should not be applied.

- UL subband can be configured in symbols configured as DL in TDD-UL-DL-ConfigCommon

  - Support: Huawei, vivo, ZTE, Fujitsu, CATT, Spreadtrum, Intel, OPPO, CMCC, Panasonic, NEC, LG, Ericsson, DOCOMO, Samsung, Qualcomm, Nokia, ITRI, WILUS

- UL subband can be configured in symbols configured as flexible in TDD-UL-DL-ConfigCommon

  - Support: Huawei, vivo, ZTE, Fujitsu, CATT, Spreadtrum, Intel, OPPO, CMCC, NEC, LG, Ericsson (conditionally), DOCOMO, Samsung, Qualcomm, Nokia, ITRI, WILUS

  - Not support: Panasonic (not applicable)

Considering the strong support from companies to study both SBFD operation in DL and flexible symbols configured in TDD-UL-DL-ConfigCommon, an initial proposal is provided in Proposal 1-3.

        - Subband frequency location indication

The following agreements were made for semi-static configuration of subband frequency locations for SBFD operation.

Agreement (RAN1#110) For semi-static configuration of subband location, consider same subband frequency resources across different SBFD symbols as baseline. Agreement (RAN1#110bis-e) For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. - FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined.

Majority companies support that frequency location of UL subband is configured with respect to carrier as summarized below. An initial proposal in Proposal 1-4 is provided.

Frequency location of UL subband is configured with reference to carrier.

- Support: ZTE, CATT, xiaomi, Intel, OPPO, CMCC, ETRI, Qualcomm, Samsung (at least for RRC_CONNECTED), MediaTek, CEWiT

- Not support: NEC?

- FFS: vivo (FFS carrier/cell level or BWP level)

      - Dynamic subband location indication

It was agreed to study semi-static configuration of subband time and frequency location as baseline.

Companies discussed dynamic indication of time and frequency location of UL subband for efficient resource utilization and DL/UL traffic adaptation and the views are summarized below.

Dynamic indication of size and/or frequency location of UL subband

- Supported by: New H3C, TCL, Sony, Lenovo

- Not supported by: ZTE, vivo, Ericsson, Intel, MediaTek, Ericsson, DOCOMO, Nokia

- FFS: Qualcomm

Dynamic indication of time location of UL subband

- Supported by: Huawei, New H3C, vivo, TCL, ZTE, Intel, Sony, DOCOMO, Qualcomm, Nokia, WILUS (deactivate semi-static SBFD configuration)

- Not supported by: Ericsson, MediaTek

It is moderator’s understanding that whether/how to support dynamic subband location indication is highly related to the discussion in sections 3.1.2.1 and 3.1.2.2. So it is suggested to discuss the proposals in those sections directly.

    - SBFD aware UE behaviors

Based on the following agreement in RAN1#110bis-e, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

Agreement For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline. - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

      - DL symbols configurd in TDD-UL-DL-ConfigCommon

In RAN1#110bis-e meeting, the following proposal was supported by companies but it was not agreed since some companies prefer SBFD operation in flexible symbols configured in TDD-UL-DL-ConfigCommon.

Proposed Agreement: For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - A set of contiguous RBs in a carrier at least excluding RBs of UL subband is defined as a DL subband for discussion purpose - FFS whether frequency location of DL subband(s) is explicitly indicated or implicitly determined - FFS whether guardband(s) need to be defined and whether UE is aware of such guardband(s) - Note: if guardband(s) are defined, the DL subband(s) additionally exclude the RBs of the guardband(s) - DL receptions within DL subband(s) are allowed in the symbol - FFS whether DL receptions within UL subband are allowed or not in the symbol - FFS whether/how the symbols can be converted to DL-only symbols in the symbol - Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol as in existing specification

CATT, Spreadtrum, KT and Nokia proposed to agree the above agreement.

All companies agree that UL transmissions outside UL subband are not allowed in a symbol configured as DL in TDD-UL-DL-ConfigCommon as summarized below.

UL transmissions outside UL subband are not allowed in a symbol configured as DL in TDD-UL-DL-ConfigCommon

- Support: vivo, Fujistu, ZTE, CATT, OPPO, ETRI (baseline), CMCC, NEC, LG, Ericsson, DOCOMO, QC, Samsung

- Not support:

Regarding whether to introduce flexible subband in addition to UL subband, DL subband and guardband, Huawei proposed to introduce flexible subband without differentiating symbols configurd as DL or flexible in TDD-UL-DL-ConfigCommon. It is defined as a set of consecutive flexbile RBs, where

  - the flexible RB can be used for UL transmission, DL transmission, and guard band, and

  - the flexible RB can be reconfigured as UL RB or DL RB by UE dedicated RRC and/or DCI.

Panasonic think that instead of lengthy description of "the flexible-symbol-like operation in UL subband in semi-static legacy DL symbols", it would be more natural to call "flexible subband" as the actual operation is flexible symbol operation and proposed to support flexible subband in semi-static legacy DL symbols to achieve flexible-symbol-like operation in UL subband in semi-static SBFD symbols.

ZTE think the flexible subband is beneficial but also pointed out that if DL transmission is allowed to be transmitted in the UL subband, then even without introducing the concept of “flexible subband”, DL transmission may also be allowed in UL subband. In this sense, UL subband can serve as the “flexible subband”.

Intel discussed flexible subband and proposed flexible subband is not considered in semi-static DL symbols.

Samsung think that there is no need for a separate ‘flexible’ subband type.

It is assumed that other companies do not see the need for a flexible subband.

Regarding whether guardband(s) are defined and informed to UE, Huawei prefer that guardband(s) are implicitly indicated based on scheduling, i.e., gNB will not schedule UL and DL transmission on guard band because Huawei considers DL subband, UL subband and flexible subband. Other companies think guardband(s) are indicated to UE either explicitly or implicitly as summarized below.

Indication of guardband(s)

- Support: New H3C, vivo, ZTE, CATT, Spreadtrum, Intel, OPPO, InterDigital, NEC, Panasonic, Samsung, Ericsson, DOCOMO, Qualcomm, Nokia, ITRI, CEWiT

- Not support (implicitly indicated based on scheduling, i.e., gNB will not schedule UL and DL transmission on guard band): Huawei

Indication of other subband(s)

- Explicit: Huawei (DL subband), New H3C, TCL, Spreadtrum, Panasonic, Samsung, ITRI

- Implicit: Huawei (flexible subband), ZTE, CATT, InterDigital, Ericsson, Qualcomm, CEWiT

For DL receptions within UL subband, companies’ views are summarized below and the views are still divergent.

DL receptions within UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon:

- Allowed: ZTE, Fujistu, CMCC, NEC, Sharp, Nokia, Samsung

- Not allowed: vivo, ZTE, CATT (baseline), OPPO, ETRI (baseline), LG (baseline), Ericsson, DOCOMO, QC, WILUS, Nokia

  - symbol can be converted to DL-only symbol: vivo, DOCOMO, LG, QC, WILUS, Nokia

Based on the above views, an initial proposal in Proposal 1-5 is provided.

      - Flexible symbols configurd in TDD-UL-DL-ConfigCommon

In RAN1#110bis-e meeting, the following proposal was proposed for SBFD operation in flexible symbols configured in TDD-UL-DL-ConfigCommon.

Proposed Agreement: For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, - UL transmissions within UL subband are allowed in the symbol - FFS whether DL receptions within UL subband are allowed or not in the symbol - FFS whether UL transmissions outside UL subband are allowed or not in the symbol - FFS whether DL receptions outside UL subband are allowed or not in the symbol - FFS interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI - FFS whether/how the symbols can be converted to non-SBFD symbols - FFS whether/how guardband(s) are explicitly indicated

Compared with the SBFD operation in DL symbols configured in TDD-UL-DL-ConfigCommon, the views are more divergent for the SBFD operation in flexible symbols configured in TDD-UL-DL-ConfigCommon.

Panansonic proposed that for SBFD-aware UE, time and frequency locations of subbands for SBFD operation is not applied to legacy flexible symbols. This means SBFD operation in flexible symbol is transparent to both SBFD-aware and legacy UE.

CATT, Ericsson and Samsung think SFBD operation when the UL subband is configured on legacy D or legacy F symbols should be the same. In this case, the benefit of SBFD operation in flexible symbols is to allow non-SBFD aware UEs to benefit from UL subband which cannot be achieved in DL symbols configured in TDD-UL-DL-ConfigCommon.

Vivo, Spreadtrum and Intel think that different from the SBFD operation in DL symbols, in flexible symbols, the RBs excluding RBs of UL subband and guardband(s) can be used as either UL or DL. It can be achieved by introducing flexible subband(s) as shown below. But it was mentioned that the the direction of multiple flexible subbands in the same symbol should be the same, e.g. {FUF} can be indicated as {DUD, UUU (full UL)} but not {DUU} or {UUD}. Spreadtrum does not prefer flexible subband and Intel also think whether flexible subband is needed should be further discussed.

Figure 3-1: flexible subband in flexible symbol [15]

Fujitsu considers that the UE behavior is determined based on flexible symbol/slot determination by SFI as below.

  - Flexible is determined to DL: Same behaviour with UL subband on DL

  - Flexible is determined to UL: converting to UL-only

  - Remaining Flexible: Same behaviour with current specification assuming UL subband as Flexible

DOCOMO considers that the UE behavior is determined based on the configuration of TDD-UL-DL-Dedicated as below:

- If the symbol is configured as DL by TDD-UL-DL-Dedicated, SBFD operation with UL subband in the symbol can be supported without additional specification impact, based on support of SBFD operation with UL subband in symbol configured as DL by TDD-UL-DL-ConfigCommon.

- If the symbol is configured as UL by TDD-UL-DL-Dedicated, SBFD operation should be de-prioritized, according to RAN#97 guidance that SBFD operation in UL symbol is de-prioritized.

- If the symbol is configured as flexible by TDD-UL-DL-Dedicated, or if TDD-UL-DL-Dedicated is not configured, regarding whether/how to support SBFD operation with UL subband in the symbol, interaction with DCI 2_0 (i.e. dynamic SFI indication) should be considered. Possible options include:

  - Option 1: Not support SBFD operation with UL subband in the symbol, i.e. the symbol should be non-SBFD DL/UL/flexible symbol as legacy.

  - Option 2: Whether to support SBFD operation with UL subband in the symbol depends on indication by DCI 2_0, e.g. when DCI 2_0 indicates DL (and/or flexible) for the symbol, SBFD operation with UL subband in the symbol is supported, otherwise, SBFD operation with UL subband in the symbol is not supported.

  - Option 3: Support SBFD operation with UL subband in the symbol, regardless of the direction indication by DCI 2_0.

Companies’ views on the FFS points are summarized below

DL receptions within UL subband in a symbol configured as flexible in TDD-UL-DL-ConfigCommon

- Allowed: ZTE, NEC, Sharp, Samsung

- Not allowed: vivo, ZTE, CATT, OPPO, ETRI (baseline), LG (baseline), Ericsson, DOCOMO, WILUS

  - symbol can be converted to DL-only symbol: vivo, LG, WILUS

UL transmissions outside UL subband in a symbol configured as flexible in TDD-UL-DL-ConfigCommon

- Allowed: vivo, OPPO, WILUS

  - symbol can be converted to UL-only symbol: vivo, LG

- Not allowed: ZTE, CATT, NEC, LG (baseline), Ericsson, DOCOMO, Samsung

DL receptions outside UL subband in a symbol configured as flexible in TDD-UL-DL-ConfigCommon:

- Allowed: vivo, CATT, OPPO, Ericsson, WILUS, Samsung

- Not allowed:

Considering the divergent views, an initial proposal in Proposal 1-6 is provided which hopefully provides more information on the design principle compared with the proposal in the last meeting.

      - SSB symbols

The following agreement was made in RAN1#110bis-e.

Agreement Study whether SBFD operation in SSB symbols is supported or not.

It was agreed in RAN1#110bis-e to study whether SBFD operation in SSB symbols is supported or not.

Companies’ views are summarized below:

SBFD operation in SSB symbols is

  - Supported: Huawei, QC, Sharp, ITRI

    - Except the UEs which are under initial access or cell reselection, a short SSB measurement period is not necessary and these UEs can perform the SBFD operation on SSB symbols to improve the performance [Huawei]

    - More uplink resource is available for better coverage and reduced latency [Sharp]

    - Restriction of SBFD operation considering the case that the network is using many SS/PBCH blocks [Huawei, Intel, Sharp]

    - frequent switch between DL and SBFD symbols if SBFD in SSB is not supported [Intel]

    - UE-UE CLI between two UEs occurs rarely [ITRI]

  - Not supported: CATT, OPPO, InterDigital (at least for initial access), CMCC, Sharp, NEC, Ericssion, DOCOMO (and CORESET#0), Samsung, MediaTek, CEWiT

    - Avoid impact on SSB due to intra-cell/inter-cell UE-to-UE CLI [CATT, OPPO, CMCC, Ericsson, MediaTek, Sharp, CEWiT]

    - UE cannot transmit UL and measure SSB simultaneously [CATT, NEC, Sharp]

    - Self-interference from SSB is not easy to be cancelled considering that SSB signaling is a broadcast signaling with high power [OPPO]

    - UE may search SSB in UL subband which is a waste of time and power at the UE side [InterDigital]

    - lower sync raster opportunities and lower flexibility in initial access procedure [InterDigital]

    - Rel-15 UE-side EPRE assumptions may not hold depending on the antenna configuration selected to support duplexing [Samsung]

Moderator thinks that the benefit to allow SBFD operation in SSB symbols should be obvious to companies. But there are concerns from objecting companies as listed above. Unless the concerns are addressed, it is difficult to agree that SBFD operation in SSB symbols is supported. An initial proposal for conclusion in Proposal 1-7 is provided, which highlight some concerns from multiple objecting companies.

      - Transmission/Reception enhancements

The following agreement was made in RAN1#109-e.

Agreement Study the impact/potential enhancements of resource allocation in symbols with subbands that gNB would use for SBFD operation.

Companies’ inputs on impact/potential enhancements of resource allocation in SBFD symbols are summarized on a per physical channel/signal basis in the following sub-sections. A set of related proposals are provided in Proposal 1-8 to Proposal 1-11.

        - PDCCH

Lenovo proposed to study enhanced CORESET frequency resource allocation considering that some groups of 6RBs may not be usable for CORESET, e.g. a RB offset applicable to an SBFD symbol.

ZTE proposed to study the impact and benefits of potential enhancements for RBG for CORESET to resource allocation granularity in frequency domain for SBFD operation not just related to unaligned boundaries of DL/UL subband and configuration granularity of PDCCH/CORESET.

Intel proposed to study solutions for PDCCH in SBFD and non-SBFD symbol.

MediaTek observed that configuring PDCCH monitoring over different CORESETs for SBFD and DL-only slots/symbols may require configuring separate search spaces per each slot in the periodic SBFD partition pattern, each with the same slot periodicity but different slot offsets, soon using up the maximum number of search spaces that can be configured and proposed to study enhancements to Search Space configuration which allow adaptation to SBFD partitioning.

Similarly, Nokia proposed to study solutions enabling dynamic adaptation of PDCCH resources based on the occurrence of SBFD symbols and the corresponding DL-UL frequency domain partitioning.

Spreadtrum proposed that PDCCH does not need to study the impact when crossing SBFD and non-SBFD symbols, it can be guaranteed by PDCCH monitoring occasion and CORESET configuration.

        - PDSCH

With {DUD} subband frequency pattern, the available DL resources are partitioned into two DL subbands.

The existing configuration of rate matching resource cannot be used to resolve the issues since a UE is not expected to handle the case where PDSCH DMRS REs are overlapping with rate matching resources according to current specification.

For RA type 0, there can be fractional RBGs at DL subband boundaries which cannot be allocated for PDSCH.

Figure 3-2: Partial RBG at boundary of the DL subband [29]

For RA type 1 and non-interleaved VRB-to-PRB mapping, only contiguous PRBs can be allocated so that non-contiguous FDRA across DL subbands cannot be achieved. For RA type 1 and interleaved VRB-to-PRB mapping, scheduling restriction/difficulty are observed to avoid overlapping with UL subband.

Enhancements including enhanced VRB-to-PRB mapping, rate matching around UL subbands, puncturing the PRB/symbols within the UL subband, mirror image FDRA, interleaved mapping of odd and even VRBs over physical RBGs allocated by Type-0 FDRA, similar handling as fractional RBGs at BWP edges  and a 3rd more optimal set of RBG set sizes were proposed.

Issues of FDRA of PDSCH due to different available resources in SBFD symbols and non-SBFD symbols are identified.

For the above issue, OPPO proposed three potential schemes, which applies to PUCCH/PUSCH repetition as well.

- Scheme 1: A PUCCH/PUSCH/PDSCH can be repeated within non-SBFD slots or within SBFD slots, but not between a SBFD slot and a non-SBFD slot. For this scheme, current transmission scheme can be reused directly.

- Scheme 2: A PUCCH/PUSCH/PDSCH can be repeated between SBFD slot and non-SBFD slot, and the same RB resources are used in both types of slots. In other words, the valid RB resource for the first PUCCH/PUSCH/PDSCH repetition is copied in subsequent slots.

- Scheme 3: A PUCCH/PUSCH/PDSCH can be repeated between SBFD slot and non-SBFD slot, and the different RB resources can be used in different types of slots. In other words, the valid RB resources for PUCCH/PUSCH/PDSCH repetition are determined separately in SBFD slots and non-SBFD slots.

CMCC has similar proposals for PDSCH slot aggregation with the following two options.

- Opt 1: PDSCH slot aggregation is only across SBFD slots or non-SBFD slots;

- Opt 2: PDSCH slot aggregation can be across SBFD slots and non-SBFD slots with some automatic adjustment on FDRA between SBFD slots and non-SBFD slots.

Figure 3-3: Potential PDSCH slot aggregation methods in SBFD [19]

DOCOMO discussed similar issue in case of multiple PDSCHs scheduled by single DCI as shown below. One possible enhancement is to introduce an additional FDRA field in the DCI scheduling multiple PDSCHs/PUSCHs. Another possibility is to interpretate the FDRA field differently on SBFD symbol and non-SBFD. It is also possible that the PDSCH/PUSCH overlapping with UL/DL subabnd is simply dropped. The same applies to PUSCH.

Figure 3-4: Example of single DCI scheduling multiple PDSCHs [27]

Nokia discussed the similar issue for SPS PDSCH and proposed to study solutions to configure the UE with SBFD-specific SPS-PDSCH resources to be used during SBFD slots/symbols and it applies to CG PUSCH as well.

Besides, Huawei and OPPO discussed TBS determination of PDSCH for SBFD operation and proposed that the amount of RBs used to determine the TB size does not count the RBs overlapping with UL subband and the guard bands (if any) around the UL subband.

        - CSI-RS

The following agreement was made in RAN1#110bis-e.

Agreement Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands.

In current specifications, CSI-RS is limited to only contiguous FDRA and the CSI-RS frequency occupancy must be configured in integer multiple of 4 RBs and has size of minimum DL BWP or 24 RBs. With {DUD} subband frequency pattern, different options can be considered for CSI-RS resource set frequency domain resource allocation and CSI reporting configuration as summarized below as per moderator’s understanding from companies’ inputs.

- Option 1: Two CSI-RS resources are configured

  - Option 1-1: Two contiguous CSI-RS resources link to two CSI reports

  - Option 1-2: Two contiguous CSI-RS resources link to one CSI report [Huawei, Qualcomm]

- Option 2: Non-contiguous CSI-RS resource

  - Option 2-1: non-contiguous CSI-RS resource allocation [Huawei, vivo, Spreadtrum, Lenovo, Ericsson, DOCOMO, Sharp, WILUS]

  - Option 2-2: contiguous CSI-RS resource allocation and non-contiguous CSI-RS resource derived by excluding frequency resources of UL subband and guardband(s) [Huawei, Spreadtrum, Lenovo, Ericsson, DOCOMO?, Qualcomm, Sharp, Nokia, WILUS]

- Option 3: Contiguous CSI-RS resource configuration with overlapping resources with UL subband and guardband(s)

  - Option 3-1: UE skips a CSI measurement and report for a CSI reporting configuration if any CSI-RS resource collides with UL subband or guardband [vivo]

  - Option 3-2: UE skips the subband CSI reporting to a CSI subband colliding with UL subband and guardband(s) [vivo, ETRI?]

  - Option 3-3: CSI reporting setting configures that the CSI is not reported for CSI subband(s) colliding with UL subband and guardband(s) [CATT, Samsung]

Figure 3-5: CSI-RS resource/CSI report options

For Option 1, two CSI-RS resources each within a DL subband are configured. The two CSI-RS resources can link to two CSI reports (Option 1-1) or one CSI report (Option 1-2). Option 1-1 is supported in current specification but several companies think it is not sufficient for SBFD operation considering the restriction of maximum number of CSI-RS supported by UE. DOCOMO discussed potential support of narrow band RB allocation for CSI-RS (e.g. smaller value than 24 for number of RBs).

Option 2 is to define a non-contiguous CSI-RS by explicit non-contiguous FDRA (Option 2-1) or by implicit derivation (Option 2-2).

Option 3 allows CSI-RS resource allocation to be overlapped with UL subband and guardband. UE skips CSI report for a CSI reporting configuration or a CSI subband in case CSI-RS resource collides with UL subband or guardband(s) in Option 3-1 and Option 3-2. Option 3-3 is supported by existing specification that CSI report configuration linked to the CSI-RS resource set can toggle off CSI subbands colliding with UL subband and guardband(s) which is supported from Rel-15.

CATT think the motivation and potential benefit for enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands is unclear assuming CSI-RS sequence generation is unchanged since Option 3-3 can be supported already.

CEWiT proposed to support reception of CSI-RS within an UL subband and guardband if any.

In addition, Qualcomm see potential impact on CSI processing latency and propose to study impact on UE processing latency.

Huawei, CATT , Qualcomm, Samsung and WILUS discussed the issue that the subband boundary for CSI reporting may not be aligned with DL subband boundary. Qualcomm think it may be beneficial for the UE to measure CSI in partial CSI subband. WILUS think that no enhancement is needed due to no ambiguity between SBFD aware UE and gNB on DL/UL subband locations assuming RBs of UL subband are excluded.

CATT proposed to study the benefit of separate CSI report configurations for SBFD symbols and non-SBFD symbols considering that the CSI-RS resource may be invalid in SBFD symbols.

DOCOMO proposed to study whether to separately report CSI for non-SBFD symbol/slot and for SBFD symbol/slot considering the interference level at SBFD symbol and at non-SBFD symbol may be different.

Samsung discussed potential different CSI-RS Tx EPRE settings by the gNB in the two DL subbands of the SBFD slot.

        - PUSCH

For the similar reason as for PUCCH, separate PUSCH configurations for SBFD and non-SBFD slots are discussed/proposed by several companies. To be more specific, CMCC, Samsung and WILUS considered separate configuration for PUSCH frequency hopping. MediaTek considered separate FDRA for CG-PUSCH.

For PUSCH repetition, similar issue as for PDSCH exists. Samsung considered one possibility to restrict multi-slot PUSCH transmissions to only use the SBFD slots but not the normal UL slot in time-domain. MediaTek considered possible approach to support two FDRA for DG-PUSCH repetition, or to allow the repetitions to be performed on specific set of slots (e.g., UL-only slots) and skipped on other set of slots (e.g., SBFD slots). In addition, the UE can skip repetitions that overlap with the DL subband in SBFD slots.

For CG PUSCH, similar issue as for SPS PDSCH exists. MediaTek, Nokia proposed to support two FDRA for CG-PUSCH transmission based on slot type and support skipping of periodic transmissions (e.g., CG-PUSCH) in SBFD slots. Panasonic think it is preferable to decouple the configured UL transmission from the configuration/operation of SBFD symbol/slot. For example, the configured UL transmission can be associated with an applicable symbol type (e.g. SBFD symbol vs. normal UL symbol), such that UE will skip the UL transmission instance if it is not overlapping with the applicable symbol type.

For PUSCH frequency hopping, similar issue as for PUCCH exists and similar enhancements are considered.

DOCOMO proposed to study SBFD impact on PUSCH repetition type B segmentation.

Figure 3-6: Example of PUSCH repetition type B

In addition, Qualcomm proposed to further study the impact/potential of enhancement of available slot counting in SBFD.

        - PUCCH

Considering the different available resources in SBFD slot and non-SBFD slot for UL transmissions and potential different link quality in SBFD slot and non-SBFD slot, separate PUCCH configurations for SBFD and non-SBFD slots are discussed/proposed by several companies.

To be more specific, vivo proposed to consider separate antenna configurations, power control, planned frequency resources etc. and think the solution can be extended to other physical channels or signals, e.g. PDCCH, SPS PDSCH and CSI-RS in DL, and CG PUSCH, SRS, PRACH in UL. CMCC proposed to separately configure PUCCH frequency hopping offsets, PUCCH resource sets. In addition, separate configuration of PUSCH FH offsets and SRS resources for SBFD symbols and non-SBFD symbols are considered for PUSCH and SRS respectively. MediaTek proposed to support two frequency domain resource allocations for periodic and semi-persistent PUCCH transmission in SBFD slot and non-SBFD slot.

InterDigital proposed to study the feasibility and potential benefits of providing supplementary configurations to the UE to be used in case PUCCH/PUSCH frequency resources are mapped to outside of the SBFD UL boundaries.

For PUCCH repetition, similar as the issue discussed for PDSCH slot aggregation, MediaTek proposed to support two frequency domain resource allocations for PUCCH repetition based on slot type and UE can skip repetitions that overlaps with the DL subband in SBFD slots. In addition, frequency hopping can be enabled for PUCCH repetition. MediaTek proposed to support two frequency domain resource allocations for intra-slot and inter-slot frequency hopping for PUCCH transmission in SBFD and non-SBFD slot. In addition, a mechanism to disable frequency hopping in SBFD slots could help in allowing flexible frequency hopping in the UL-only slots. Ericsson proposed to study enhancements to frequency domain resource allocation and frequency hopping mechanisms for PUSCH and PUCCH configured with repetition in order to allow repetitions to occur in both SBFD and UL-only slots. Qualcomm think the issue can be resolved by adjusting the location of the hops that occur within the UL subband. Samsung proposed that PUCCH repetitions can be configured to only use the SBFD slots/symbols and the same applies to multi-slot PUSCH transmissions. Sony proposed that UE performs frequency hopping in SBFD slots if the bandwidth size of the UL subband is larger than a configured threshold and if there is resource available for the second hop, otherwise the UE disables frequency hopping in the SBFD slot.

DOCOMO proposed to study SBFD impact on SPS HARQ-ACK deferring.

Figure 3-7: Example of SPS HARQ-ACK deferring

        - SRS

Separate SRS configurations for SBFD and non-SBFD slots are discussed/proposed by ZTE, CATT, CMCC, Lenovo.

MediaTek considered SRS resource colliding with DL subband in {UDU} pattern and observed that for periodic and semi-persistent SRS transmission, a single frequency domain resource allocation is configured for all UL slots. This may impact flexibility in resource allocation in SBFD since it has two types of slots with different bandwidths and for SRS transmission in SBFD, the resource allocation with and without frequency hopping may not be confined within the UL subband of a SBFD partitioned slot. Corresponding enhancements were proposed.

      - UE collision handling

The following agreement was made in RAN1#110bis-e.

Agreement Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE - If there are, whether/how to avoid/handle such collision cases (as second step)

There are two types of collision between UL and DL for a SBFD aware UE as below.

- Type A: Collision between UL transmissions and DL receptions in the same SBFD symbol

- Type B: Collision between transmissions/receptions with transmission direction of subbands

For Type A collision, the following cases are discussed by companies.

- Collision between dynamic UL transmissions and dynamic DL receptions

  - ZTE, CATT (for repetition case), ITRI (with different priorities)

  - Error case: vivo, CATT (for non-repetition case), Intel, xiaomi

- Collision between dynamic DL receptions and semi-statically configured UL transmissions

  - Vivo, ZTE, CATT, OPPO, Intel, xiaomi, ITRI, MTK, Sony, QC, WILUS

- Collision between dynamic UL transmissions and semi-statically configured DL receptions

  - Vivo, ZTE, CATT, OPPO, Intel, xiaomi, ITRI, MTK, Sony, QC, Nokia (dynamic UL vs. SSB/Type0 CSS), WILUS

- Collision between semi-statically configured UL transmissions and semi-statically configured DL receptions

  - New H3C, vivo, ZTE, CMCC, CATT, Intel, Lenovo, Nokia, QC, WILUS

  - Error case: xiaomi, Sharp

In addition, several companies discussed collision between DL reception and PRACH transmissions. It is not clear yet whether PRACH resources can be configured in UL subband and it is related to the discussions in section 3.1.4. So this case is not considered for now in this section. Similarly, for the collision between SSB and UL transmissions, it is related to the discussion in section 3.1.2.3 and is not considered for now in this section.

For Type B collision, the transmission direction of subband(s) depends on the discussions in section 3.1.2.1, 3.1.2.2 and 3.1.2.3. For example, assuming only semi-static subband indication is supported and only UL transmissions are allowed in UL subband, there is collision if the time-frequency resource of a DL reception overlaps with UL subband in SBFD symbol. As another example, assuming that dynamic disabling of UL subband is supported, there is no collision if the time-frequency resource of a DL reception overlaps with UL subband in SBFD symbol when UL subband is dynamically disabled. CATT and Intel discussed the following two cases:

- Collision between dynamic UL transmissions/DL receptions and transmission direction of subbands

- Collision between semi-statically configured UL transmissions/DL receptions and transmission direction of subbands

CATT think that Case 1 can be avoided by gNB scheduling, but the collision between subsequent repetitions of dynamic transmissions except the first repetition and the subband/guardband should be further studied. Case 2 may exist in SBFD symbols considering that gNB may not be able to avoid these collision types in configuration.

Intel think whether resource collision still exists after enhancement of resource allocation and L1 procedure needs further discussion. For example, in case of separate configuration for SPS PDSCH in SBFD or non-SBFD symbol, the collision between configured SPS PDSCH resource and UL subband in SBFD symbol may be avoided, while if only single configuration for all symbols is allowed and UE may perform rate matching around the subband, the collision between configured SPS PDSCH resource and UL subband in SBFD symbol may happen.

In addition, CATT proposed to study whether UCI multiplexing is performed before or after Type B collision handling considering that the PUCCH resources after UCI multiplexing may change. Also, the outcome can be different if Type A collision or Type B collision is handled first. So the order or collision handling needs to be discussed.

    - DL-UL switching

Xiaomi proposed to further study how to configure or determine the guard period between DL region and UL subband

Intel discussed that for a SBFD aware UE, the gap for DL/UL switching is also needed between SBFD symbols, or between DL symbol and SBFD symbol. How to ensure such gap and the impact on back-to-back DL and UL signals/channels should be studied.

InterDigital discussed that non-zero timing advance or switching time could result in dropping of respective slots, especially for back-to-back scheduling cases between DL and UL and proposed to consider a means for UE to detect if the allocated UL/DL timing alignment is enough.

MediaTek proposed per-subband, per-symbol indication of RB-symbol regions reserved for DL-UL switching as shown in the following figure.

Figure 3-8: Symbol-level SBFD layout configuration [32]

    - SBFD operation in RRC idle/inactive state

It was agreed to study SBFD operation at least for RRC_CONNECTED state.

Ericsson proposed that UEs in IDLE mode are not aware of whether or not symbols/slots are used for SBFD operation and UL transmission during initial access should occur only within UL-only slots.

Fujitsu’s observation is that due to the CLI from PRACH transmission procedure on UL subband, the overall system performance will get decreased and thus proposed to wait for the discussion results of the PRACH coverage enhancement in the NR coverage enhancement agenda.

Huawei, ZTE, CATT, Intel, LG, Samsung, Sharp, DOCOMO, Qualcomm and Nokia support to study potential benenfits and enhancements for initial access in UL subband.

ZTE think at least the random access in RRC_CONNECTED mode can be studied.

    - SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies

One potential use case for SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies is for {D U} pattern shown below.

LG proposed to clarify the meaning of center frequency of BWP for at least following cases.

- Case 1: BWP is used for both half duplex and SBFD operation including two types of frequency resource partitioning (i.e., {D U D}, {D U})

- Case 2: BWP is used for only SBFD operation including two types of frequency resource partitioning (i.e., {D U D}, {D U})

Ericsson does not support misaligned center frequencies between DL and UL BWPs.

Qualcomm see useful scenario to support a single configured DL and UL BWP pair with unaligned center frequencies where network configures the UE with narrowband initial UL/DL BWP and then wideband first active BWP after RRC connection. If SBFD is indicated to only RRC connected UE, then during initial access, all UEs will be configured with center aligned narrowband UL/DL BWP as shown on left figure in the example below. This will restrict leveraging the UL-SB resources and hence don’t achieve UL coverage gain and latency reduction. On the other hand, if this restriction is lifted, two narrowband UL/DL BWP could be configured for the new UEs as shown in the right figure in the example below. This can be applicable for default BWP as well where UE could be scheduled only in one of the two DL subbands.

Figure 3-9: Narrowband UL/DL BWP with aligned and non-aligned center frequency

    - SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies

Example of multiple configured DL and UL BWP pairs with aligned center frequencies is illustrated below, where gNB configures BWP 1, 2 and 3 for UE and gNB may also configure BWP 4 with full bandwidth with the restriction that BWP 4 can be used only in 1st DL slot or last UL slot.

Figure 3-10: Multiple configured DL and UL BWP pairs with aligned center frequencies [15]

Given that there is either DL or UL within a BWP in a symbol, no new UE behaviour needs to be introduced to resolve UL/DL collision. In addition, separate configuration for different BWPs can be supported.

SBFD aware UE is required to support more than one BWP pair and there are potential issues due to frequent BWP switching and the interruption caused by BWP switching delay. Intel discussed the impact on SPS PDSCH/Type-2 CG PUSCH, HARQ-ACK dropping and repetition due to BWP switching. To reduce BWP switching delay, multiple active BWPs or shorter BWP switching delay needs to be considered as discussed by many companies, which would increase the UE implementation complexity.

ZTE, CMCC and NEC proposed to further study the BWP-based solution for SBFD configuration with aligned center frequency for a BWP pair.

Ericsson proposed to not support SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair considering the fundamental change to current specifications and negative impact on UE complexity.

Qulacomm raised concerns on SBFD operation using more than one configured DL and UL BWP pair with aligned center frequency where each subband configured as BWP. Instead, it is proposed that there is one UL/DL BWP pair configured for TDD operation and one UL/DL BWP pair configured for SBFD operation and UE would need to switch from one BWP to the other based on some semi-static configuration of the BWP switching. UE knowledge of the semi-static BWP switch may result into optimized BWP switching delay. It is noted that the UL/DL BWP pair configured for SBFD operation is with unaligned center frequencies and DL BWP can be defined as non-contiguous RBs to cover both DL subbands.

Figure 3-11: Multiple configured DL and UL BWP pairs with unaligned center frequencies

Xiaomi proposed that BWP-based SBFD operation is not supported in Rel-18 duplex enhancement.

Intel analyzed if both DL BWP and UL BWP in SBFD symbols shown above are within the same UE channel BW, the retune is not needed thus no interruption time between a SBFD symbol using DL BWP and next SBFD symbol using UL BWP; otherwise, similar retune time as existing BWP switch is needed.

KDDI observed that smaller BWP can suppress the inter-UE CLI (compared to the BWP including all RBs) by help of lower sampling rate and therefore proposed to study the potential benefit of SBFD scheme with more than one configured DL and UL BWP pair to utilize BWP adaptation based inter-UE CLI suppression.

    - SBFD operation across carriers

It was agreed in RAN1#109-e to at least study SBFD operation within a TDD carrier.

Agreement At least study SBFD operation within a TDD carrier

SBFD operation across carriers achieved by intra-band TDD CA with different TDD configurations is considered by Huawei, Intel and Samsung. Huawei proposed to study SBFD operation within a carrier and across carriers with equal priority. Intel proposed to study CA-based SBFD focusing on scenarios wherein multiple component carriers may be configured within a significantly wide frequency band. Samsung proposed to further study potential enhancements to CA-based SBFD operation in FR2-1 at a later stage and one area for such potential enhancements in FR2-1 is the Rel-16 directional collision handling.

Figure 3-12: SBFD operation across carriers [6]

Qualcomm discussed two options for SBFD operation across multiple component carriers as shown below. The first option is to utilize intra-band CA framework with different TDD DL/UL configurations across the component carriers. The other option is by fully reusing the same concept of SBFD within a component carrier across the component carriers. While it is considered that CA-based SBFD operation is interesting for e.g. FR 2-1, it is proposed that SBFD operation across multiple components is studied at later stage in Rel-18 after establishing the baseline study of SBFD operation within component carrier.

Figure 3-13: SBFD using multiple component carriers (intra-band CA)

Xiaomi proposed that half duplex CA based SBFD operation is not supported in Rel-18 duplex enhancement.

    - SBFD operation in legacy UL sybmol

The following conclusion was made in RAN#96.

Conclusion: UL symbol as second priority is accepted, no intended suspension of continuation of work in WGs

Xiaomi proposed that for subband non-overlapping full duplex, it cannot be applied to UL symbols.

Intel proposed to deprioritize SBFD in a UL symbol in which at least one legacy UE transmits UL, or in a UL symbol which is semi-statically configured as UL for at least one legacy UE.

Qualcomm observes that SBFD operation at legacy UL slot is beneficial in multiple deployment scenarios, e.g., greenfield deployment and UL heavy deployment (InH/InF) to reduce DL blockage and improve DL coverage.

  - [Open] 1st round discussion

Proposal 1-1

Proposed Agreement:

For indication of subband locations for SBFD operation, cell-common indication of subband time and frequency location is the baseline.

|  | Company |
| --- | --- |
| Support | Sony, New H3C, DOCOMO, xiaomi, NEC, Panasonic, CEWiT, LG Electronics, KDDI, WILUS, Nokia, NSB, Fujitsu, Lenovo |
| Not support | Samsung, InterDigital,OPPO, ETRI |

| Company | Comments |
| --- | --- |
| Sony | Indication similar to TDD Slot Format can be used where we have a cell common slot format indicator and UE dedicated indicator. If dynamic SBFD is considered we can further have SFI based and Grant based indicators. |
| Samsung | We have serious concerns about this proposal. SBFD operation only in RRC_CONNECTED mode is baseline for now. We have not yet agreed agreed if SBFD for UEs in RRC_IDLE/INACTIVE is supported, e.g., for RACH during IA. Such an agreement would be needed first. Like many other companies, we also see SBFD subband configuration as a property of the NR carrier BW depending on BS SIC implementation capabilities. This however by no means implies that cell-common signaling must then be used for the UEs. This is similar to the handling of the Rel-15 CSI-RS in the RRC signaling. Cell-common and/or UE-dedicated signaling for RRC_CONNECTED, e.g., like BWP config common/dedicated or PDSCH config common/dedicated can be left to RAN2 in the WID. Cell-common, e.g., SIB1-based, for RRC_IDLE would require agreeing on the possibility for SBFD operation for IA in RRC_IDLE first. It must also be considered that any cell-common SIB-1 based signaling solution necessarily needs to account for payload size constraints. This is one of the reason why Rel-15 SIB1 UL-DL config common signals consecutive DL slots from start of p1/p2 and UL slots from end of p1/p2, then only uses symbol level resolution for F. Similar for RB level resolution when indicating the SBFD UL/DL SB or GB. CSI-RS for example can be allocated in step size of 4 RBs, but this would result in an unacceptably high payload for the SBFD configuration when in SIB1. In our view, cell-common SIB-1 based indication is a complementary signaling mechanism for UEs in RRC_IDLE during RACH for IA (if agreed). The baseline is RRC signaling to UEs of the SBFD time/frequency locations for subbands using dedicated RRC signaling. Indication of the SBFD configuration in SIB1 and using RRC dedicated signaling must be compatible in the sense that UEs in RRC_IDLE can use the same SBFD configuration at least as subset of resources also signaled to UEs configured with SBFD operation in RRC_CONNECTED mode., respectively. RAN2 can then later decide how to encode the desired behavior, e.g., symbol level resolution, step sizes in F-D, using common/dedicated IEs during the WID stage. |
| New H3C | We are fine with this proposal |
| InterDigital | Similar view as Samsung on this issue. We suggest modifying the proposal for down-selection like: Modified Proposal 1-1: For indication of subband locations for SBFD operation, downselect: - Alt.1: cell-common indication of subband time and frequency location is the baseline - Alt.2: UE-dedicated RRC signaling on subband time and frequency location is the baseline. And, we support Alt.2, as the baseline should be for UEs with RRC-CONNECTED mode. We can further study cases for RRC-IDLE mode which is related with Alt.1 in our view. |
| Xiaomi | We slightly prefer UE-dedicated RRC signaling for UL subband configuration. However, we can live with the proposal. |
| OPPO | Three reasons for us not to support the proposal. - If SBFD symbol configuration is cell-common, RAN1 neeeds to further study the UE behavior of whether this cell-common SNBD symbol can be further indicated/modified in UE dedicated TDD UL/DL signaling. RAN1 has not yet study whether/how to modify Rel-18/19 cell-common configuration by a Rel-15 UE-dedicated confiuration. - We wonder whether the cell-common subband indication means mandatory UE support of subband (at least to recognize the subband), as what is done for TDD-UL-DL-Common. - If UL subband is cell-common, the guardband is also likely cell-common. But it is difficult in specification logic to utilize resources in a cell-common configured guardband. - Further, the cell common indication is necessary only if the contention-based PRACH is transmitted in UL subband before RRC connection is established, which however is not agreed yet. |
| Spreadtrum | We support the proposal. For the comments from Samsung, we have the following understanding. First, cell-common singling does mean SBFD can be applied for RRC-IDLE or Inactive state. If this is the concern, we are fine to add a Note like “Note: it does not mean SBFD for UEs in RRC_IDLE/INACTIVE is supported” Second, cell-common singling can make all UE have same understanding towards the location of SBFD, especially the frequency domain. There would be diverse decisions or handlings when different UE shares different understanding for SBFD location. It makes gNB scheduling more difficult. Furthermore, for the payload size constrains, it is an important aspect when design the granulary of SBFD indication, such as symbol or slot level. So it has to decide cell-common or UE-specific type first. |
| Intel | Does ‘cell-common indication’ mean the singaling is carried by SIB? Though we agree cell-specific semi-static subband configuration should be supported, we prefer to keep by SIB or UE specific RRC (but the information is same for all UEs) open, and discuss it later. |
| Huawei, HiSilicon | We support to study the SBFD operation in IDLE mode, and how to indicate the SBFD configurations in IDLE mode could be further studied once the SBFD operation in IDLE mode is agreed. So we may need to decide the study of SBFD in IDLE mode firstly. And as another aspects, the IDLE mode and CONNECTED mode may be both supported, it is not clear the “baseline” means, the configuration of the configuration or the indication may not be the same for IDLE mode and CONNECTED mode. So we suggest to dicuss to agree on the SBFD operation in IDLE mode first. |
| LG Electronics | We are fine with this proposal. Similar with TDD cell specific configuration, cell-common indication of subband time and frequency location is the baseline seems quite natural. |
| ZTE | Although we prefer to have a cell-common configuration, but we sympathize with companies that this can be discussed later after deciding whether to support SBFD in IDLE. |
| Sharp | We are fine with the proposal. On the other hand, we think Samsung and Intel’s concern is valid. It doesn’t imply we should use SIB1 or similar cell-specific signaling just because the cell-common configurability is baseline. |
| Nokia, NSB | We support the proposal and have similar view as Sony. Cell-specific and UE-specific signaling are not mutually exclusive. Since SBFD configuration is cell specific, we think it would be extremely inefficient if only UE-specific signal is agreed as baseline as this would require signaling the same configuration separately to each UE. |
| Lenovo | Support. Subband location for SBFD operation should be common and same for SBFD aware UEs. |

Proposal 1-2

Proposed Agreement:

For explicit configuration of SBFD subband time locations within a period, the one or two period(s) are determined by the periodicities of TDD UL-DL pattern(s) configured in TDD-UL-DL-ConfigCommon if provided and the one periodicity is explicitly configured otherwise.

- If two TDD UL-DL pattern(s) are provided, the SBFD subband time locations are separately configured within the periodicity of each TDD UL-DL pattern.

|  | Company |
| --- | --- |
| Support | Sony New H3C, NEC,OPPO, Panasonic, Sharp, Nokia, NSB, Lenovo |
| Not support | Samsung, InterDigital, Intel, LG Electronics, ZTE |

| Company | Comments |
| --- | --- |
| Samsung | We think this proposal should considered only after the need for cell-common SIB1-based signaling to support SBFD operation for UEs in RRC_IDLE/INACTIVE has been agreed. Like many other companies, we also think that the indication of SBFD time locations must be designed around the Rel-15 legacy patterns p1(and p2 if present) of the TDD UL-DL config common/dedicated. For SBFD configuration of time-domain symbols in RRC_CONNECTED, signaling of symbol level bitmap(s) for indication of configured SBFD time-locations however can achieve the same desired effect without making any explicit assumption on p1/p2. |
| New H3C | We are fine with this proposal |
| InterDigital | Share similar view as Samsung on this issue, too. This may be better discussed after the above Proposal 1-1, where we think the baseline should be for UEs with RRC-CONNECTED mode. The current Proposal 1-2 seems premature as there may be other simple way of explicit configuration by UE-dedicated RRC signaling on SBFD subband time locations, which can be futher listed up and determined later. |
| Xiaomi | Fine with the proposal. |
| NEC | We support the point that 2 UL subband patterns should be allowed to be configured for the case when 2 TDD UL-DL patterns are present. This will allow gNB to avoid configuring UL subband occasions overlapping the SSB/CORESET0 occasions in one period, while in another period maximixing the time occasions of UL subbband. |
| Intel | We think this is a WI aspect, no need to discuss now. |
| Huawei, HiSilicon | We support to study the SBFD operation in IDLE mode, and how to indicate the SBFD configurations in IDLE mode could be further studied once the SBFD operation in IDLE mode is agreed. So we may need to decide the study of SBFD in IDLE mode firstly. And as another aspects, the IDLE mode and CONNECTED mode may be both supported, it is not clear the “baseline” means, the configuration of the configuration or the indication may not be the same for IDLE mode and CONNECTED mode. So we suggest to dicuss to agree on the SBFD operation in IDLE mode first. |
| LG Electronics | We are fine with one period of SBFD subband time locations within a period which is same as the periodicities of a TDD UL-DL pattern configured in TDD-UL-DL-ConfigCommon. In the main bullet, the motivation of two period(s) is not clear if one TDD UL-DL pattern(S) is configured. |
| ZTE | The periodicity of SBFD may be multiple times of the TDD periodicity in order to have different patterns in different TDD periodicities. |
| Sharp | Regarding Samsung’s comment, we don’t think this proposal is related to Proposal 1-1. In other words, the proposal doesn’t say that UEs in IDLE/INACTIVE needs to recognize the SBFD subbands just because a part of information (i.e., periodicity) is provided by information in SIB1. In that sense, we suggest adding an FFS stating whether or not UEs in IDLE/INACTIVE needs to recognize the SBFD subbands or not. |
| Nokia, NSB | We agree that the periods used in TDD-UL-DL-ConfigCommon and TDD-UL-DL-ConfigDedicated can be used to signal the semi-static configuration of SBFD symbols/slots in time domain. |

Proposal 1-3

Proposed Agreement:

Study SBFD operation with UL subband in symbols configured as DL in TDD-UL-DL-ConfigCommon and in symbols configured as flexible in TDD-UL-DL-ConfigCommon.

|  | Company |
| --- | --- |
| Support | Sony, Samsung, New H3C, DOCOMO, InterDigital, ITRI, xiaomi, NEC,OPPO, Intel, Huawei, Hisilicon, CEWiT, LG Electronics, ZTE, Sharp, Qualcomm, ETRI, KDDI, WILUS, Nokia, NSB, Fujitsu, Lenovo |
| Not support | Panasonic (would like to clarify the technical merit to use flexible symbol), MediaTek (modification needed) |

| Company | Comments |
| --- | --- |
| Samsung | We support the proposal as shown. |
| New H3C | We are fine with this proposal |
| InterDigital | Support. |
| ITRI | Support. |
| Xiaomi | Support |
| NEC | We need to further discuss how UL subband occasions are interpreted once UE receives dedicated configuration of flexible symbols/slots. For example: - What is the UE behavior for receiving UL/DL when network does not update the flexible symbols/slots Is guard band applicable for the case when flexible symnbol/slot is indicated as UL |
| Panasonic | We are fine with SBFD operation with UL subband in symbols configured as DL in TDD-UL-DL-ConfigCommon. However, for symbols configured as flexible in TDD-UL-DL-ConfigCommon, gNB is allowed to schedule both DL and UL not to restricted to DL/UL subband in a transparent manner. If SBFD aware UE takes into account both time and frequency locations of subbands and non-SBFD aware UE does not take into account such subbands in legacy flexible symbol, the gNB scheduling operation is more complex. If potential enchancements based on non-transparent operation for SBFD aware UE are needed (e.g., different resource allocation between SBFD symbol and non-SBFD symbol if introduced), legacy DL symbol can be used. Thus, we would like to clarify the technical merit to use flexible symbol. Or we can postpone the discussion until more specific behaviour is clarified in DL symbols |
| LG Electronics | Support |
| MediaTek | We have concern on using “SBFD operation with UL subband”. SBFD, by definition, will have UL and DL subbands (regardless of how these subbands signalled). We had the following agreement from last meeting, we think the FFS should be addressed before having more agreements that only consider “UL subband” Agreement For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. - FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined. We can accept the proposal with the following modification: Study SBFD operation with UL subband in symbols configured as DL in TDD-UL-DL-ConfigCommon and in symbols configured as flexible in TDD-UL-DL-ConfigCommon. |
| Sharp | We think that the technical merit to have SBFD subbands is frequency domain resource assignment, or UE’s digital/analog filtering behavior. |
| QC | Instead of study agreement, we could use similar formulas in the last meeting agreement as suggested below For semi-static configuration of subband, it is agreed that UL subband can be configured in symbols configured as DL in TDD-UL-DL-ConfigCommon and in symbols configured as flexible in TDD-UL-DL-ConfigCommon |

Proposal 1-4

Proposed Agreement:

For semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL subband is configured with reference to the corresponding TDD carrier.

|  | Company |
| --- | --- |
| Support | Samsung (support intent of proposal, but modified wording). New H3C, DOCOMO, xiaomi, NEC,OPPO, Intel, Panasonic, CEWiT, ZTE, Sharp, Qualcomm, ETRI, WILUS, Nokia, NSB, Fujitsu, Lenovo |
| Not support | InterDigital (modification suggested), MediaTek (modification needed) |

| Company | Comments |
| --- | --- |
| Sony | Need clarification on how this works if multiple BWPs are configured. For example we can have 4 BWP configured since Rel-15. In a particular slot, we configure a {DUD} SBFD for the system bandwidth as shown below. For BWPs smaller than system bandwidth, are the subbands in a BWP depends on which subbands that BWP overlaps with? For example BWP2 here overlaps {DUD} but would have smaller DL subbands compared to BWP1. BWP3 would results in a {DU} configuration and BWP4 would results in a {UD} operation. It would be good to clarify whether this how we intend to operate under multi BWP configurations? |
| Samsung | Proposed alternative wording with intent to hopefully better capture the intent of the proposal: For semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL subband is indicated configured with reference to the CRB grid corresponding TDD carrier. |
| New H3C | We are fine with this proposal |
| InterDigital | Sony raised a valid question, which seems needed to be carefully checked in the group. As we believe UEs with RRC-CONNECTED mode should be the baseline at this stage, we think how to inform UE of the subband frequency locations can be simply based on UE-dedicated RRC signaling for a BWP, as what UE needs to know is the frequency location within the active BWP “UE-specifically”. Then, we suggest modifying the proposal for down-selection like: Modified Proposal 1-4: For semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL sub-band is configured: - Alt.1: with reference to the corresponding TDD carrier indication of subband locations for SBFD operation - Alt.2: by UE-dedicated RRC signaling for a BWP, UE-specifically. And, we support Alt.2, which resolves any potential misalignment issues raised by Sony, whereas Alt.1 seems needed to address such raised issues, which we failed to see any benefits on Alt.1 other than just UE-dedicated RRC signaling as eventually the SBFD operation is per UE (UE-specific feature). |
| Xiaomi | Regarding to Sony’s comments, I think what really matters is to determine the frequency location of UL subband, instead of the DL BWP. Although dynamic BWP switching is supported, UL subband location is not impacted. It is gNB’s business to make sure DL BWP covers the UL subband. {DUD} or {DU} or {UD} doesn’t impact SBFD operation. On the other hand, a single UL subband within a carrier has been agreed. From this perspective, configure UL subband within carrier is straightforward. |
| NEC | We are okay with keeping frequency location with reference to TDD carrier if other companies are in support of this. We need to further discuss the case on the UE procedural impact if UL subband bandwidth is greater than the active UL bandwidth part or when part of UL subband frequencies fall outside the active bandwidth part. Specifically, - Is it okay to have the case where UE’s active bandwidth part does not contain any DL subband or if DL subband bandwidth is very small within the active bandwidth part - How does UE determine the DL, UL and guard band resources within the active bandwidth What reference numerology is used by UE to determine the UL subband occasions for each bandwidth part |
| OPPO | We agree with the principle, but just wonder what “with reference to the corresponding TDD carrier” actually means. Does it mean “with reference to common RB #0 of the TDD carrier”? |
| Spreadtrum | We support FL’s proposal. Also fine with a further step from Samsung, use CRB grid as reference, more specificly, SBFD frequency location indicated with relative to common resource block 0 and subcarrier spacing. |
| Intel | We surpport the proposal. Regarding Sony’s question, we think cell-common subband does not depend on BWP. The subband which is available for a UE when the UE is configured with a BWP depends on the subband. Only RBs of a subband within the active BWP can be used. For exmaple, for BWP 3, RBs of UL subband within the brace can be used by UE for UL subband. |
| Huawei, HiSilicon | One clarification is that the for the configuration of the subband, does “reference to the corresponding TDD carrier” mean that the reference point for the configuration is the first PRB of the carrier bandwidth not point A of the carrier bandwidth? For Samsung’s comments, the revision seems that the reference point of the configuration of the subband is the the point A. Overall we think that the clarification should be made, and we think that this a very detailed signaling design and can be discussed later, e.g. can be discussed after whether the subband configuration is carrier based or BWP based. |
| CEWiT | We are fine with the proposal. Our understanding is that the UL subband is defined wrt the gNB. Thus, even if only a part of the UL subband overlaps with the active BWP of the UE, the gNB should inform the frequency domain location of the whole subband to the UE. The frequency domain location of the subband should not change for different UEs. Hence, a cell common indication can be used to inform the frequency domain location of the subband. |
| LG Electronics | We are fine with the principle. We are also thinking configuration for BWP should be clarified. In Rel-15, bandwidth related information is configured by cell-specific indication. Also, for BWP, bandwidth related information is configured in addition because a BWP can be composed within BW configured by cell-specific indication. In the same principle, it is quite natural that BW for SBFD operation for BWP can be configured on top of the frequency location of UL subband configured with reference to the corresponding TDD carrier. |
| MediaTek | Same as our comment for Proposal 3-1 We had the following agreement from last meeting, we think the FFS should be addressed before having more agreements that only consider “UL subband” Agreement For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined. |
| Sharp | We are OK with the proposal. If we want to further clarify the meaning of “with reference to the corresponding TDD carrier”, we suggest revising it as “with reference to the starting CRB of the TDD carrier”, which is not the same as the Point A. |
| QC | Generally supportive and okay with the proposal. Some minor edits needed to clarify that the UL subband is configured within the resource grid of the carrier with reference to the CRB0. For semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL subband is configured with reference to the CRB0 within the corresponding TDD carrier resource grid.. |
| Nokia, NSB | We support having carrier-specific configuration. We see no reason to have the UL subband configured with reference to the UE’s BWP as we think there is no point in configuring UE’s with different UL subbands. |
| Lenovo | Support. "with reference to the corresponding TDD carrier" should be clarified to wrt CRB0 in resource grid. |

Proposal 1-5

Proposed Agreement:

For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- A set of contiguous RBs in a carrier at least excluding RBs of UL subband and guardband(s) are is defined as a DL subband for discussion purpose

  - Either the number of RBs for guardband(s) or the frequency location of DL subband(s) is explicitly indicated

  - FFS whether frequency location of DL subband(s) is explicitly indicated or implicitly determined

  - FFS whether guardband(s) need to be defined and whether UE is aware of such guardband(s)

    - Note: if guardband(s) are defined, the DL subband(s) additionally exclude the RBs of the guardband(s)

- DL receptions within DL subband(s) are allowed in the symbol

- FFS whether/how DL receptions within RBs of semi-statically configured UL subband are allowed or not in the symbol

- FFS whether DL receptions within UL subband are allowed or not in the symbol

- FFS whether/how the symbols can be converted to DL-only symbols in the symbol

- Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol as in existing specification

|  | Company |
| --- | --- |
| Support | Sony, Samsung, New H3C, DOCOMO, NEC, Panasonic, CEWiT, LG Electronics (with modification), ZTE, Sharp, Qualcomm (w/ edit), ETRI, KDDI, Nokia, NSB, Lenovo |
| Not support | InterDigital (the 3rd bullet contradicts to the next proposal 1-6, modification suggested), MediaTek (modification needed) |

| Company | Comments |
| --- | --- |
| Moderator | This proposal is proposed based on the latest proposal discussed in the last meeting with revisions in red related to guardband(s) given that explicit/implicit indication of guardband(s) is widely supported by companies. In addition, for the last two FFS, the intention of both FFSs is to allow DL reception within RBs of semi-statically configured UL subband with different detailed solutions. It is better to merge the two FFS at this stage. |
| Samsung | We can support this updated FL proposal which will hopefully resolve the concerns from Qualcomm re: guard bands in RAN1#110bis-e. |
| New H3C | We are fine with this proposal |
| InterDigital | The 3rd bullet “…and guardband(s) are is defined as a DL subband” conflicts to the next proposal 1.6 with Option 2. So, suggest discussing the next proposal 1.6 first. BTW, we don’t think the guard-band RBs shall be explicitly configured to the UE, as those can be just implicitly determined as remaining RBs other than the UL (and/or) DL subbands, (or the UE actually doesn’t need to know it). So, in short, we can modify the Proposal 1-5 like: Modified Proposal 1-5: For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - A set of contiguous RBs in a carrier at least excluding RBs of UL subband is defined as a DL subband for discussion purpose - The number of RBs for the frequency location of DL subband(s) can be explicitly indicated - FFS whether frequency location of DL subband(s) is explicitly indicated or implicitly determined - FFS whether guardband(s) need to be defined and whether UE is aware of such guardband(s) - Note: if guardband(s) are defined, the DL subband(s) additionally exclude the RBs of the guardband(s) - DL receptions within DL subband(s) are allowed in the symbol - FFS whether/how DL receptions within RBs of semi-statically configured UL subband are allowed or not in the symbol - FFS whether DL receptions within UL subband are allowed or not in the symbol - FFS whether/how the symbols can be converted to DL-only symbols in the symbol - Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol as in existing specification |
| Xiaomi | For the first sub-bullet, we think the key point is how to determine guard band. If companies think guard band needs to be explicitly configured, either DL subband or guard band can be explicitly configured. Otherwise, it is enough for gNB to configured UL subband explicitly only. |
| NEC | We support DL receptions within configured UL subband dynamically. As it allows network to use the frequency resources of UL subband for DL allocation in case of intermittent high DL traffic demand and low UL traffic. |
| OPPO | Regarding to the following sub-bullet in red: - Either the number of RBs for guardband(s) or the frequency location of DL subband(s) is explicitly indicated We think there is a 3rd option, which explicitly defines a “DL blocking subband” that includes both UL subband and guard bands around it. This option has two benefits: - It avoids defining two DL subbands, which has some redundant configuration given the configuration of carrier bandwidth. - It avoids confiuraing UE with UL subband in case the UE does not need to perform UL tansmission in the DL symbol. |
| Spreadtrum | We support the proposal. And we prefer to explicit indicate the frequency location of DL subband(s). The subsequence operations can be decided based on the UL and DL subbands locations. |
| Ericsson | Generally we think that if an UL subband is configured, it doesn’t really make sense to change it to downlink. If that is the desired mode of operation, then a full flexible symbol should be configured (like dynamic TDD). However, we understand that this is just FFS for now. We have a concern about the wording “as in existing specification” in the Note. We think this is too restrictive, since it may be necessary to make some small modifications to the BWP definition to make SBFD work. For example, if a DL BWP spans both the subbands, the RBs of the DL BWP will not be contiguous due to the UL BWP in the middle which is different than existing specifications. Furthermore, if the UL BWP spans the whole carrier in order to avoid BWP switching between SBFD and UL-only symbols, then similarly the BWP definition may need some modification. |
| Intel | We are fine with this proposal. The sub-bullet in red captures possibility of explicit guardband or DL subband configuration. The revised 1st FFS point in red addresses the intention of allow DL reception on semi-static UL subband in high-level, which would be sufficient for now. In our understanding, ‘how’ covers using dedicated signaling to indicate SBFD/DL symbol switch and using dynamic scheduling grant, which can be further discussed as 2nd step. |
| Huawei, HiSilicon | We think that the subband is configured for uplink transmission, and it is also to be further decided whether the subband is allowed for downlink transmission. If the subband is allowed for downlink transmission, then subband may not be a dedicated uplink subband. So we suggested in the propoposal modifying “UL subband” to “subband” as follows For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: UL transmissions within UL subband are allowed in the symbol UL transmissions outside UL subband are not allowed in the symbol A set of contiguous RBs in a carrier at least excluding RBs of UL subband and guardband(s) are is defined as a DL subband for discussion purpose Either the number of RBs for guardband(s) or the frequency location of DL subband(s) is explicitly indicated FFS whether frequency location of DL subband(s) is explicitly indicated or implicitly determined FFS whether guardband(s) need to be defined and whether UE is aware of such guardband(s) Note: if guardband(s) are defined, the DL subband(s) additionally exclude the RBs of the guardband(s) DL receptions within DL subband(s) are allowed in the symbol FFS whether/how DL receptions within RBs of semi-statically configured UL subband are allowed or not in the symbol FFS whether DL receptions within UL subband are allowed or not in the symbol FFS whether/how the symbols can be converted to DL-only symbols in the symbol Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol as in existing specification |
| LG Electronics | We are fine with the proposal with following modification. - FFS whether/how the symbols can be converted to DL-only symbols in the symbol gNB and UE already support TDD operation which is baseline operation. It is more natural operation to be converted to DL-only symbols in the symbol. Also, this operation can provide comparable DL performance of SBFD. The UL performance of victim gNB can be kept if gNB-to-gNB co-channel CLI is considered by scheduler (e.g., lower MCS indication for UL, etc.) |
| MediaTek | We have concern on the “implicit” knowledge of DL subbands. It is not clear to us why it is essential to explicitly signal the UL subband but not the DL subband. We already had the following agreement on the knowledge of DL/UL subbands at the UE. Thus, the proposals should consider both subbands. Agreement For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline. SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation. |
| QC | Thanks to Samsung for flexibility on the guardband(s) issue! We believe that the FFS on SBFD symbols update shouldn’t be removed. To allow UE to receive in UL-SB, this can be done by scheduling DCI which is one shot override or by means of dynamic update of SBFD symbol. We understand that there are different opitions by companies on which approach to utilize. At this moment, it is better to keep both approaches as FFS bullet and not to exclude one or the other. - FFS whether/how the symbols can be converted to DL-only symbols in the symbol |
| WILUS | We are fine with the FL’s proposal in principle. For the guard band, please clarify whether “explicitly indicated” only includes that SBFD aware UE is indicated with a set of RBs (starting/length) for the guard band or also includes that SBFD aware UE is aware of the guard band (e.g., default value by RAN4). |
| Nokia, NSB | We are ok with the proposal. |
| Fujitsu | Regarding the guard band in the first sub-bullet, we don’t think it is needed to explicitly indicate the DL subband to UE because DL subband will be naturally defined if UL subband and guard band are defined. For the guard band, the gNB scheduling can control the guard band without any indication. However, we are open to discuss about explicit guard band indication if there is a merit to adopt explicit guard band indication. |
| Lenovo | Frequency locations of UL subband and DL subband(s) are sufficient to the SBFD aware UE for transmission/reception and explicit guard band awareness is not needed. We don't think downlink transmission in semi-static configured UL subband should be allowed, but can further study its merits. |

Proposal 1-6

Proposed Agreement:

For SBFD operation with an UL subband in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, down-select from the following options:

- Option 1:

  - Flexible subband is not introduced

  - UE behaviors are the same as in symbols configured as DL in TDD-UL-DL-ConfigCommon

- Option 2:

  - The symbol can be used as DL-only symbol, UL-only symbol or SBFD symbol from gNB perspective

    - Two sets of contiguous RBs in a carrier excluding RBs of UL subband, if any, are not expected to be used as UL and DL respectively in the symbol

    - FFS whether flexible subband is introduced

    - FFS conditions for DL-only symbol, UL-only symbol and SBFD symbol

    - FFS UE behaviors

Other options are not precluded.

|  | Company |
| --- | --- |
| Support | New H3C with option 2, DOCOMO, InterDigital (with Option 2), Xiaomi(option1), NEC, Huawei, Hisilicon, CEWiT, LG Elctronics (option 2), Sharp (Option 1), Qualcomm, ETRI (option 2), KDDI, WILUS |
| Not support | Sony, Samsung,OPPO, MediaTek (modification needed) , Nokia, NSB |

| Company | Comments |
| --- | --- |
| Moderator | Please refer to the summary in section 3.1.2.2 for more information. |
| Sony | On the sub-bullet of Option 2 as highlighted yellow below: - Option 2: - The symbol can be used as DL-only symbol, UL-only symbol or SBFD symbol from gNB perspective - Two sets of contiguous RBs in a carrier excluding RBs of UL subband, if any, are not expected to be used as UL and DL respectively in the symbol Why do we need to have this restriction that for {FUF} the two Flexible subbands must be either DL or UL? Why can’t it be {DUU} or {UUD}? |
| Samsung | Thank you @FL for identifying the two possible options. In our view, Option 1, same UE behavior on symbols configured as DL or F, is the only meaningful option when considering gNB SIC implementation capabilities. If UL transmission outside the configured UL subband is not possible on the DL symbol due to gNB SIC, why would this behavior now become possible (FFS) on the F symbol? It might be best to first understand from the proponents of Option 2 which gNB SIC implementation would actually allow for different behavior in time-domain symbols using the same underlying gNB implementation before inclusion of Option 2 is considered. We think that a separate (3rd) flexible subband type (in addition to UL SB and DL SB types) is not needed. When the gNB configures the UL SB, UL transmission outside the UL SB is not possible due to gNB SIC. If we allow for DL transmission in the configured UL SB (FFS), then the UL SB is inherently flexible from the scheduling perspective. |
| New H3C | We prefer option 2 |
| DOCOMO | We prefer option 1. Confusion on option 2 is: When we say SBFD operation in a flexible symbol (as in the main bullet), why DL-only and UL-only are considered? In our understanding, DL-only and UL-only are possible when the flexible symbol is not configured for SBFD operation. |
| InterDigital | We support FL proposal, and we prefer Option 2, which offers flexibility in the network. We believe it’s better that the specification can allow different gNB/network implementation on how to utilize efficiently over UL subband and DL (or even Flexible) subband. We also share Sony’s question, where the usefulness of Flexible subband can be the flexibility so that it’s better to allow the case that Sony mentioned. |
| Xiaomi | We prefer option 1. We believe the same SFBD operation on flexible as that of DL symbol is sufficient. Furthermore, as option 1 is relevant to SBFD operation on DL symbols, it is better to come back when we achieve consensus on proposal 1-5. |
| NEC | We support Option-2 of above. It is not clear from our perspective how flexible slots are different than normal DL slots for Option-1 if UE behavior is same as DL slots. Networks may have requirement to update the slot type from SBFD to UL-only or from SBFD to DL-only depending on immediate load characteristics. This requirement can be easily accomplished using Option-2 where network can configure good number of slots as flexible initially and can later update them dynamically e.g. using SFI. However, before agreeing on terminology “flexible subband” it is better to clarify this definition as different companies have different interpretations of this. |
| OPPO | Option-1 is from UE perspective, while Option-2 is from gNB perspective, which makes the whole proposal vague. What’s more, we have a concern to treat SBFD symbol as a concept in parallel with DL symbol, UL symbol and then flexible symbol as well. |
| Spreadtrum | Thank you for providing two options. But any of them are a complete solution. For Option 1, if the same UE behavious as in symbols configured as DL applied, how about the FFS point that whether/how DL receptions within RBs of semi-statically configured UL subband are allowed. Does it also have same handling for DL and flexible symbol? And if Flexible symbol is override into UL by UE-specific signaling, it is obvious different from DL operation. For Option 2, we agree with the first sub-bullet. But there are still some FFS need to be listed. Such as: - FFS whether/how SBFD application in symbols configured as UL/DL in TDD-UL-DL-ConfigDedicated |
| Ericsson | Generally we can support configuring an UL subband in a ‘F’ symbol since it will allow legacy UEs to be scheduled UL in the UL subband in a transparent way as long as there are no backwards compatibility issues. We still think there is a risk of backwards compatibility issues, and for that reason as long as configuring UL subband in a ‘D’ symbols is supported, we can be okay with supporting configuring an UL subband in an ‘F’ symbol. We think the baseline behavior for ‘F’ symbols should be the same as for ‘D’ symbols, i.e., if an UL subband is configured then the symbol is used as D-U-D. We don’t think it makes sense at all to dynamically switch between D-U-D/DL-only/UL-only. If that is the desired mode of operation, then the symbol should be left as ‘F’ and operated as dymamic TDD. We don’t see a performance advantage in configuring the UL subband if the symbol ends up being fully dynamic anyway. In summary, we think symbols configured as ‘D’ and ‘F’ should have the same behavior as baseline. We think it is strange to “downselect between Option 1 and Option 2” but then say “other options not precluded.” |
| Intel | We think it makes sense to allow a flexible symbol to be DL only or UL symbol which is supported since Rel-15. The controversial point would be, if gNB wants to support such flexibility, whether gNB should semi-statically configure UL subband for flexible symbol or not. If the answer is yes, that is option 2, otherwise, it is option 1. Comparing opt 1 and opt 2, finer DL/UL granuarlity is achieved by opt 2. With not so large requirement for UL traffic, UL subband is sufficient for SBFD UE and legacy UE, and SBFD UE can achiveve benefit of non-trasparent SBFD in the flexible symbol. If there is large dynamic UL or DL traffic, SBFD UE as well as legacy UE can fully utilize whole bandwidth for UL-only symbol or DL-only symbol as in Rel-15. Therefore, we think option 2 would be beneficial. Regarding Samsung’s comment, we’re not sure why we can not allow a flexibe symbol acting as a full UL symbol due to gNB SIC. Switching from Flexible symbol to UL symbol is already well-supported in legacy system. For the FFS of flexible subband, while we are open for the discussion, we think new subband type may not be necessary, though the resources in flexible symbol can be considered as flexible resource due to dynamic switch between different symbol types. For exmaple, we can still use the concept of DL and UL subband + additional overriding rule by dynamic symbol type switch. |
| Huawei, HiSilicon | Option 2 is preferred. In current stage, flexible subband is the best way to solve concerns from different companies. It can provide more scheduling flexibility to achieve better resource utilization for each subband. the SIC implementation is not changed, as long as the subband location is not changed So the for option 2 can be modified to Option 2: The symbol can be used as DL-only symbol, UL-only symbol or SBFD symbol from gNB perspective Two sets of contiguous RBs in a carrier excluding RBs of UL subband, if any, are not expected to be used as UL and DL respectively in the symbol The SBFD operation on the flexible symbol is the same as in the DL symbol configured for SBFD operation from gNB perspective. FFS whether flexible subband is introduced FFS conditions for DL-only symbol, UL-only symbol and SBFD symbol FFS UE behaviors For option 1, the UE behavior should not be changed compaired with the current specification, i.e. the modification is Option 1: Flexible subband is not introduced UE behaviors are the same as in symbols configured as DL flexible in TDD-UL-DL-ConfigCommon |
| MediaTek | Same as our comment for Proposal 3-1 We had the following agreement from last meeting, we think the FFS should be addressed before having more agreements that only consider “UL subband” Agreement For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. - FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined. |
| QC | We are okay with Option 1 for consistent UE behaviour in both DL and flexible slots. |
| KDDI | We support FL proposal for listing possible options at this stage, and we prefer option 1. |
| Nokia, NSB | We think that at least a third option should be included where flexible subband is not introduced but the the symbol can be used as DL-only symbol, UL-only symbol or SBFD symbol from gNB perspective. Option 1 only support the symbol to be used as DL-only symbol or SBFD. Otherwise we are ok with Option 2 (where our view on ‘FFS whether flexible subband is introduced’ is that flexible subband should not be introduced). |
| Lenovo | We are ok to study the two options. |

Proposal 1-7

Proposed Conclusion:

In order to decide whether SBFD operation in SSB symbols is support or not, at least consider the following aspects:

- Impact on SSB detection/measurement due to UE-to-UE CLI

- Incapable of simultaneous UL transmission and DL reception of SSB at UE side

|  | Company |
| --- | --- |
| Support | Sony, Samsung (added more aspects) New H3C, DOCOMO, ITRI, xiaomi, Intel (with updates), Panasonic, CEWiT, LG Electronics (with modification), ZTE, MediaTek, Sharp, ETRI, KDDI, Nokia, NSB, Lenovo |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | The intention of the proposal is to highlight some concerns from multiple objecting companies so that we can focus on these aspects for the futher study to see whether the concerns can be addressed. It is not an exhaustive list and some concerns brought up by single company have not be included which can be discussed later if the above concerns can be address. |
| Samsung | We support the intent of the proposal. We suggest using the following list of aspects to consider (replacing all sub-bullets above). Modified proposed conclusion: (…) - Potential impacts to UE measurement accuracy and detection performance for SSB-based measurements from UE-to-UE CLI - Potential imapcts on cell (re-)selection performance for legacy and SBFD-aware UEs - Potential impacts to DL beam/cell coverage from reduced or non-constant DL Tx EPRE for the SSB-carrying symbols The case of simultaneous UL transmission and DL reception of SSB at UE side is not meaningful according to us. Such a scenario would be gNB mis-configuration of the UE even if DL subbands on SSB symbols are supported. |
| New H3C | We are fine with this proposal |
| InterDigital | Support in principle |
| Xiaomi | We are fine with the proposal. Also fine with the newly added subbullets from Samsung. |
| NEC | Given that most of the companies have shown opposing view of supporting SBFD during SSB occasions, the question that we need to also ask is what is the benefit we are getting in supporting this? We know that UEs will need to monitor SSB e.g. for radio link monitoring, then how can network schedule UL during these occasions is not clear to us. So, before discussing how to address above issues, perhaps we need to confirm the scenarios where supporting SBFD during SSB occasions is actually going to bring any perceivable benefits. |
| OPPO | There maybe another point on DL power allocation: whether the DL EPRE for resources falling into UL subband can be re-allocated to REs not falling into UL subband, and if yes whether such EPRE re-allocation can be done over SSB REs. |
| Intel | We agree that pros and cons of supporting SBFD in SSB symbols should be well-investigated, but we think one important aspect is missing, which is a cirtial aspect. As we explained in our tdoc, if SBFD operation in SSB symbol is not allowed, it would be difficult to support contiguous UL subband resource in time domain (especially when the number of SSBs is relatively large) which leads to frequent switch between DL and SBFD symbols thus almost useless for UL repetition for coverage improvement. And for 2nd sub-bullet, we share same view with Samsung that it can be removed. We think it is common understanding that UE only works in half-duplex mode, no matter SBFD is supported or not in SSB symbol. Therefore, we suggest revision for the proposal as below: In order to decide whether SBFD operation in SSB symbols is support or not, at least consider the following aspects: - Impact on SSB detection/measurement due to UE-to-UE CLI, if SBFD operation in SSB is allowded - Incapable of simultaneous UL transmission and DL reception of SSB at UE side - Limited symbols for SBFD operation and incapable of continuous UL resource in time domain, if SBFD operation in SSB is not allowed and number of symbols for SSBs is relatively large. |
| Huawei, HiSilicon | We are OK with the proposal, however, we can highlight the significance of the allowing UL transmission in SSB symbols, i.e. if SSB is precluded from SBFD operation, the performance of SBFD operation may be reduced seriously. For example, if SSB with a duration of 4m is repeated with 5ms in the case of XXXXU to achieve better performance of mobility management, it equals to legacy TDD with DDDSU if SBFD operation is not supported on SSB symbol. At lease to reduce the latency, the SSB symbols should not be precoluded from SBFD operation. So we think we suggested to add also the disadvantages if SSB symbols is not allowed for SBFD operation. The modification of the proposal is as follows In order to decide whether SBFD operation in SSB symbols is support or not, at least consider the following aspects: Impact on SSB detection/measurement due to UE-to-UE CLI Incapable of simultaneous UL transmission and DL reception of SSB at UE side The performance loss of the in terms latency and coverage if SSB symbols is not allow for SBFD operation. |
| CEWiT | We support Samsung to include the bullet point on “Potential imapcts on cell (re-)selection performance for legacy and SBFD-aware UEs”. UL transmission in the UL subband happens in an SSB symbol which has a certain SSB ID and periodicity. In that case, a legacy UE will try to perform SSB measurements in that symbol. This will lead to unnecessary monitoring since UL is scheduled in those symbols. |
| LG Electronics | We are fine with the proposal with following modification. - Potential Impact on SSB detection/measurement due to UE-to-UE CLI If gNB dosen’t assign UL resource aroud frequency resource for SSB, the impact on SSB detection/measurement due to UE-to-UE CLI is not expected. |
| KDDI | We support in principle. We should add the performance aspects if SSB in SBFD symbol is not supported. |
| Nokia, NSB | We agree that especially impact on SSB detection by legacy UEs needs to be carefully considered. |

Proposal 1-8

Proposed Agreement:

Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following:

- Scheduled/configured PUCCH/PUSCH/PDSCH/PDCCH without repetition in SBFD symbols and non-SBFD symbols

- Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols

- Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition

- Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols

- Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols

Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered.

|  | Company |
| --- | --- |
| Support | Sony New H3C, DOCOMO, xiaomi, NEC,OPPO, Intel, Panasonic, Huawei, Hisilicon, CEWiT, LG Electronics, ZTE, MediaTek, Sharp, ETRI, KDDI, WILUS, Nokia, NSB, Fujitsu, Lenovo |
| Not support | Samsung |

| Company | Comments |
| --- | --- |
| Samsung | As we commented in RAN1#110bis-e on the FL proposal, we are in principle ok with the sub-bullets, but still have serious concerns on the wide scope of any potential enhancements. Please see our Round 4 and 5 comments and proposed wording on this proposal from RAN1#110bis-e. |
| New H3C | We are fine with this proposal |
| InterDigital | Support in principle |
| Xiaomi | We are fine with the proposal. For the first bullet, scheduled PDCCH is confusing. Hence we propose to capture PDCCH with a separate subbullet, such as “PDCCH in SBFD symbols and non-SBFD symbols” |
| Nokia, NSB | We are ok with the proposal though we think that a pragmatic approach shall be used, e.g., simple solution based on rate matching around physical resource blocks allocated for UL subband and guardband(s) should be assumed as baseline when possible. |

Proposal 1-9

Proposed Agreement:

Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering potential overlap of resource block group(s)/reporting subband(s) with a guardband and/or incompatible subband, including at least the following

- RBG for PDSCH/PUSCH FDRA

- CSI reporting subband

- PRG of PDSCH

- FFS: RBG for CORESET

Note: in this context an "incompatible subband" means, e.g., PDSCH RBG either fully or partially overlapping an UL subband

|  | Company |
| --- | --- |
| Support | Sony, Samsung, New H3C, DOCOMO, NEC,OPPO, Intel, Huawei, Hisilicon, LG Electronics, ZTE (prefer to study RBG for CORESET as well), MediaTek, Sharp, ETRI, KDDI, WILUS, Nokia, NSB, Lenovo |
| Not support |  |

| Company | Comments |
| --- | --- |
| Samsung | We support inclusion of the FFS: RBG for CORESET as compromise suggested by the FL. |
| New H3C | We are fine with this proposal |
| InterDigital | Support in principle |
| Xiaomi | Support. Clarification: the overlapping subband here is UL subband or UL/DL subband? |
| Spreadtrum | We have two comments: 1. Guardband can be involved in incompatible subband. So no need to explicit list it out. Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering potential overlap of resource block group(s)/reporting subband(s) with a guardband and/or incompatible subband, including at least the following … Note: in this context an "incompatible subband" means, e.g., PDSCH RBG either fully or partially overlapping an UL subband or a guardband. 2. For the FFS point, we prefer not to enhance RBG for CORESET. It would be significant complexity increasement but with little gain. Since RBG for CORESET has aready very flexible. |
| Intel | We can be fine with the proposal, though we think it is reasonable to support the study for RBG for CORESET. As we commented in last meeting, we’d like to remind the group that we did enhancement for CORESET with consideration of RB set in Rel-16 NRU. And we defined rules when a PDCCH candidate should be dropped, e.g., if it is outside of valid RB set for Rel-16 NR or collides with SSB for Rel-15. These designs are to improve frequency resource efficiency for PDCCH, and we think similar study for PDCCH for duplex should be performed. CORESET configruations are semi-statically provided. Depending on SS set configurations, PDCCH MOs may fall in regular DL or SBFD symbols. It would be undesirable if the CORESET configuration is conservatively set in view of SBFD symbols, thereby increasing PDCCH blocking even in regular DL symbols. Solutions based on defining UE behavior should be studied in this regard. |
| LG Electronics | We fine with the proposal. |

Proposal 1-10

Proposed Agreement:

Study the impact and benefits of potential enhancements for RA type 1 for PDSCH considering non-contiguous DL resources in frequency.

|  | Company |
| --- | --- |
| Support | Sony, New H3C, DOCOMO, xiaomi, NEC, Intel, LG Electronics, MediaTek, Sharp, KDDI, WILUS, Nokia, NSB, Lenovo |
| Not support | Samsung |

| Company | Comments |
| --- | --- |
| Samsung | In our view, enhancements to PDSCH RA Type 1 (contiguous) due to introduction of SBFD operation are not needed. PDSCH RA Type 1 for DCI F1_0 serves fallback, and the ability to schedule PDSCH inside 1 of the 2 SBFD DL subband(s) is sufficient. For scheduling using all DL RBs in both DL subbands, PDSCH RA Type 0 (RBG) is available. |
| New H3C | We are fine with this proposal |
| InterDigital | Support for study |
| Xiaomi | We are open to study. |
| NEC | Given that RA type 1 is used for fallback DCI, it is essential to support it for SBFD. Also, we need to discuss applicability of interleaving procedure for RA type 1 e.g. whether interleaving is supported or not supported for SBFD and if supported then what enhancements are required. So, we suggest adding following FFS: FFS whether/how to support interleaving for SBFD slots/symbols |
| Spreadtrum | We are supportive of the proposal. Since CSI-RS was agreed to study the enhancements of frequency domain resource allocation across non-contiguous DL subbands. PDSCH is suffering the same situation. PDSCH and CSI-RS can try to use unified solution for this problem. |
| LG Electronics | We are fine with the proposal. |
| MediaTek | RA Type-1 is not just for fallback DCI, it is more compact signalling of resource allocation. Efficient operation of frequency and time domains resource allocation should eb maintained for SBFD operation. Scheduling the PDSCH only one subband (out of two) will half the peak data rate. |
| Sharp | We think the priority is DCI format 1_0 in USS in which the allocatable bandwidth is the whole carrier. The DCI format 1_0 in CSS can only allocate bandwidth same as the CORESET0 where the CORESET0 size is up to 20 MHz for FR1. |
| Nokia, NSB | Support for study, see also our comments to proposal 1-8. |

Proposal 1-11

Proposed Agreement:

For CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subband, consider the following options:

- Option 1: Two contiguous CSI-RS resources link to one CSI report

- Option 2: Non-contiguous CSI-RS resource

  - Option 2-1: Non-contiguous CSI-RS resource allocation

  - Option 2-2: Contiguous CSI-RS resource allocation and non-contiguous CSI-RS resource derived by excluding frequency resources of UL subband and guardband(s)

- Option 3: Contiguous CSI-RS resource configuration with overlapping resources with UL subband and guardband(s)

  - Option 3-1: UE skips a CSI measurement and report for a CSI reporting configuration if any CSI-RS resource collides with UL subband or guardband

  - Option 3-2: UE skips the subband CSI reporting to a CSI subband colliding with UL subband and guardband(s)

  - Option 3-3: CSI reporting setting configures that the CSI is not reported for CSI subband(s) colliding with UL subband and guardband(s)

|  | Company |
| --- | --- |
| Support | Sony (need clarification), Samsung, DOCOMO, xiaomi, NEC, Panasonic, CEWiT, LG Electronics, ZTE, MediaTek, Sharp, ETRI, Nokia, NSB (need clarification), Lenovo |
| Not support | Intel (clarification whether this proposal is only for CSI-RS used for CSI measurement/report is needed) |

| Company | Comments |
| --- | --- |
| Moderator | Please refer to section 3.1.2.4.3 for more information. |
| Sony | On Option 3, it seems Option 3-1, 3-2 and 3-3 are not mutually exclusive. Option 3-1 seems to be applicable for wideband CSI whilst Option 3-2 is for subband CSI reporting. So we can have both Option 3-1 and Option 3-2. |
| New H3C | We are fine with this proposal |
| DOCOMO | We prefer option 2-1 which is the simplest solution. |
| InterDigital | Support in principle |
| Xiaomi | Support in principle. |
| NEC | We support Option 2-1 and 2-2 for CSI-RS allocation as it allows more compatibility with respect to dynamic slot type switching between SBFD and DL-only slot. Based on the presence of UL subband, UE would be able to determine which frequency resources to exclude for CSI-RS measurement. We also have some sympathy for Option-3 because we also need to look at the impact of CSI-RS report due to SBFD operation. For e.g., if UE is configured with subband level CQI/PMI reporting, then does it make sense to also report the subbands containing UL subband? This would just result in higher CSI reporting overhead, hence we need to consider how to enhance CSI reporting as well. |
| Spreadtrum | For Option 2-2 and Option 3-2/3-3, they might be not exclusive. Although Option 3-2/3-3 are based on CSI subband, Option 2-2 only mentions unvailable resource. CSI subband enhancement may based on unvailable frequency resources, and CSI measurement and report are relative to CSI subband. Thus, Option 2-2 and Option 3-2/3-3 may result in same solution. We suggest to consider Proposal 1-9 together. |
| Intel | We have a basic question, whether we only consider CSI-RS enhancmement for CSI measumrent/report? In our understanding, CSI-RS can be used for other purposes, e.g., as TRS. Option 1 & option 3 is only applicable for CSI measumrent/report, while option 2 can be applicable to any CSI-RS usage. So, is this proposal only for CSI-RS for CSI measurement/report? Does it imply, we will have different enhancement for different CSI-RS with different purposes, if we go with option 1/option 3? |
| CEWiT | We support option 3. We feel that option 3 has the least specification impact when it comes to CSI-RS sequence generation and resource allocation. However, for option 3, we feel that CSI reporting can also be done in the UL subband. This report can be used for DL in the latter slots where UL subband in not there. |
| LG Electronics | We are fine with the proposal in principle. Question is the intention of the proposal is down selection of one or more option(s) among the identified options. |
| WILUS | We are generally fine with the proposal. However, there is no description on CSI reporting only for Option 2. For the clarification, a note can be added on Option 2 as following: Note: no impact on CSI reporting. |
| Nokia, NSB | We support the proposal bu think some clarification is needed. For example, the difference between option 2-2 (‘non-contiguous CSI-RS resource derived by excluding frequency resources of UL subband and guardband(s)) and option 3-3 (‘contiguous CSI-RS resource configuration with overlapping resources with UL subband and guardband(s)’ and CSI reporting setting configures that the CSI is not reported for CSI subband(s) colliding with UL subband and guardband(s)) is not fully clear. In both cases, the UE is configured with contiguous CSI-RS resources but assumes no RS is transmitted on REs that collides with UL subband and guardband, hence CSI is not measured/reported in the corresponding subbands). |

Proposal 1-12

Proposed Conclusion:

At least the following cases of time domain conflict of UL transmission and DL reception in the same SBFD symbol are identified.

- Dynamic DL reception vs. semi-statically configured UL transmission

- Dynamic UL reception vs. semi-statically configured DL transmission

- Semi-statically configured DL reception vs. semi-statically configured UL transmission

Note: PRACH transmissions and SSB receptions are not included.

|  | Company |
| --- | --- |
| Support | Sony, Samsung (modified wording), New H3C with new note, DOCOMO, ITRI, xiaomi, NEC,OPPO, Intel, LG Electronics, ZTE, MediaTek, Sharp, ETRI, WILUS, Nokia, NSB, Lenovo |
| Not support |  |

| Company | Comments |
| --- | --- |
| Samsung | We do not think that it is meaningful to consider only time-domain collision cases when trying to agree on a list of possible conflict cases introduced by SBFD operation. In principle, the above list is what we would expect even with Rel-15 time-domain behavior if “SBFD” is removed from the wording. Possible new and additional collision cases introduced by SBFD operation are due to SBFD DL and/or UL subband configuration and if DL scheduling using the configured UL SB is allowed or not. This should be captured better by the FL proposed conclusions. We propose the following modified sub-bullets: - Dynamic DL reception in a configured DL subband vs. semi-statically configured UL transmission in a configured UL subband - Dynamic UL reception transmission in a configured UL subband vs. semi-statically configured DL transmission reception in a configured DL subband - Semi-statically configured DL reception vs. semi-statically configured UL transmission We consider the last sub-bullet as not needed, e.g., gNB mis-configuration. |
| New H3C | We are fine with this proposal. It is better to add note to address that repetition transmissions are included for time domain conflict case. |
| InterDigital | Support in principle |
| Xiaomi | Support |
| Spreadtrum | We support the intention, and additional with two comments: 1. The clear definition of dynamic DL/UL and semi-statically configured UL/DL can give as a bullet. Such as - Dynamic DL/UL is the PDSCH/CSI-RS/PUSCH/SRS/PUCCH triggered by DCI, including the first Type 2 CG-PUSCH activated by DCI, the first one if PDSCH/PUSCH/PUCCH with repetition - Semi-static configured DL/UL is the PDCCH/PDSCH/CSI-RS/PUSCH/SRS/PUCCH configured by higher layer singnalling, including - The other CG-PUSCH except the first Type 2 CG-PUSCH activated by DCI - PDSCH/PUSCH/PUCCH with repetition except the first one 2. The interaction with Proposal 1-5 and 1-6 can be added as a bullet. E.g. - Applicability to a symbol configured as DL/flexible in TDD-UL-DL-ConfigCommon |
| LG Electronics | Support in principle |
| Nokia, NSB | We disagree with Samsung and think that Type A and Type B collisions should be discussed separately. Type B collisions are more related to the SBFD aware UE behavior discussed in section 3.1.2). |

Proposal 1-13

Proposed Agreement:

Study random access in SBFD symbols at least in RRC_CONNECTED state.

|  | Company |
| --- | --- |
| Support | Sony, Samsung, New H3C with modification, DOCOMO, xiaomi, Intel, Panasonic, MediaTek, Sharp, ETRI, KDDI, WILUS, Nokia, NSB, Lenovo |
| Not support | NEC, LG Electronics, Fujitsu |

| Company | Comments |
| --- | --- |
| Samsung | We think that it is also important to reach an agreement if random access in SBFD symbols for RACH is supported in RRC_IDLE/INACTIVE mode (or not). |
| New H3C | During study pahse, idle mode and inactive mode should have the same priority with connection mode. Proposal should be modified as below Study random access in SBFD symbols at least in RRC_CONNECTED state. |
| DOCOMO | Agree with Samsung and New H3C that random access in RRC_IDLE/INACTIVE mode can also be studied. |
| InterDigital | OK for study |
| Xiaomi | Support |
| NEC | We don’t see a very urgent need to support RACH within UL subbands. We believe that some of the arguments provided by other companies in favor of this scheme can be addressed using gNB configuration adjustment. For e.g., UL fragmentation occurring during UL-sonly slots due to RACH occasions, can be avoided by configuring RACH resources towards edge of BWP. And given the number of issues we need to resolve for connected state, supporting RACH in SBFD slots/symbols can be taken up in next release. |
| Intel | Share similar view with Samsung for RRC_IDLE/INACTIVE mode. |
| LG Electronics | We are thinking random access in SBFD symbols can be operated for UE in RRC_IDLE/INACTIVE state. If the RO(s) in SBFD symbols are assigned only for RRC_CONNTECTED state, it is quite wasteful. |
| ZTE | Similar view as Samsung and LG. |
| KDDI | Random access in SBFD operation in RRC_IDLE/INACTIVE should also be studied as well. |
| WILUS | We agree to study RA in SBFD symbols for RRC_IDLE/INACTIVE state. |
| Nokia, NSB | Agree with Samsung, New H3C and DOCOMO that the agreement could also be extended to random access in RRC_IDLE/INACTIVE mode. |
| Fujitsu | We don’t have strong opinion but at the same time, no strong motivation to adopt PRACH transmission in SBFD symbol. If the purpos is for the PRACH coverage perspective, the enhancement has been discussing in other agenda (NR_Cov2). |

- gNB self-interference handling schemes

This section discusses gNB self-interference handling schemes for SBFD.

  - Summary of input contributions

Timing alignment

In legacy TDD system, a UE is provided a cell-specific TA offset NTA,offset. It can be provided in SIB1 and if not, UE assumes TA offset in Table 7.1.2-2 in 38.133. The cell-specific TA offset is used to reserve enough time for UL/DL switching, as shown in the following figure.

Figure 4-14: cell-specific TA offset in legacy TDD [6]

For SBFD operation at gNB side, several companies discussed inter-slot/inter-symbol interference due to non-aligned slot/symbol boundary which would impact self-interference cancellation in SBFD symbol at gNB side.

One possible solution proposed by companies is to configure NTA,offset=0. Some companies think that there can be backward compatibility issue. Nokia commented that though in principle NTA, offset can be configured by the network to any value among 0, 25600*Tc and 39936*Tc by parameter n-TimingAdvanceOffset, in practice, legacy UEs always assume the value specified in Table 7.1.2-2 in 38.133 (for the corresponding frequency range and co-existence scenario) independently of what the network indicates in system information. In addition, with NTA,offset=0, the gap for UL-to-DL switching may disappear and the gap realized based on scheduling avoidance can be larger.

An alternative solution is to have separate NTA,offset for SBFD slot and UL slot, i.e. NTA,offset=0 in SBFD symbols and NTA,offset>0 in non-SBFD symbols. Potential overlap between UL transmission in a SBFD symbol and next UL transmission in a non-SBFD symbol were identified as shown below.

Figure 4-15: Different NTA,offset values for SBFD symbols and non-SBFD symbols [12]

BS self-interference related to timing and SCS was discussed in RAN4#104bis-e with the following agreement in R4-2217464.

## 4.1 BS self-interference related to timing and SCS Agreement: - For the BS self-interference issue related to timing and SCS of D and U for both BS and UE: - RAN4 understanding is that this issue will be studied in RAN1. - RAN4 has not evaluated the timing and SCS impacts to BS SI. RAN4 for now assumes they are negligible and do not impact RAN4 requirement work. - RAN4 will not consider this issue in the BS SI feasibility study if no request from RAN1.

Feasibility study

Ericsson provided initial feasibility analysis of self-inteference mitigation solutions for FR1 SBFD operations, including self-inteference from direct TX to RX leakage, self-inteference from RX non-linearity spectrum regrowth, reciprocal mixing of phase noise, impact on ADC and additional filtering, example digital self-inteference cancellation solution for FR1 homodyne/heterodyne type base station receivers, summary of initial SBFD implementation analysis for a single-carrier single-sector BS, impact of multi-sector base station sites, alternative opportunistic TDD operation modes, further factors to study and modern gNB receiver architectures for multi-carrier and multi-band use cases.

Samsung provided analysis of self-interference cancellation including spatial-domain and antenna isolation, frequency-domain and digital cancellation and additional desing aspects. In addition, FR1 and FR 2-1 testbed performance are provided.

Qualcomm discussed self-interference mitigation techniques including spatial isolation, frequency isolation, Rx filtering, beam isolation and beamforming/nulling and digital self-interference mitigation with self-inteference link budget. In addition, Qualcomm OTA SBFD demonstration is provided.

Nokia discussed feasibility of SBFD in particular gNB RF architecture change for SBFD. It is observed that At least 145 dB of RSI and double number of BS antenna elements (Opt 2) are needed to achieve an acceptable performance with some tradeoffs between coverage gain and average system spectral efficiency degradation especially at high load based on preliminary evaluation results.

  - [Open] 1st round discussion

Proposal 2-1

Proposed Agreement:

Study impact/potential enhancements of unaligned slot/symbol boundary for DL and UL subbands in SBFD symbols.

|  | Company |
| --- | --- |
| Support | Sony, Samsung, New H3C, DOCOMO, xiaomi, NEC, Intel, Panasonic, LG Electroncis, ZTE, Sharp, KDDI, Nokia, NSB, Lenovo |
| Not support |  |

| Company | Comments |
| --- | --- |
| Moderator | This proposal was extensively discussed in the last meeting. Some companies commented that RAN4 is discussing the same issue and proposed to wait for RAN4’s outcome. As indicated above, RAN4 understanding is that this issue will be studied in RAN1. With the RAN4 agreement, is the above proposal agreeable now? |
| Samsung | We can support the FL proposal as is. |
| New H3C | We are fine with this proposal. |
| InterDigital | OK for study |
| Intel | We are open for the study, but we’d like to point out, whether slot/symbol boundary for DL and UL subbands in SBFD symbol is unaligned or aligned depends on Nta_offset and Nta, and implementation based on current UL timing mechanism can provide aligned boundary. Then, to avoid any wrong impression that unaligned slot/symbol boundary must happen, we suggest to add ‘potential’ as below. Study impact/potential enhancements of potential unaligned slot/symbol boundary for DL and UL subbands in SBFD symbols. |
| LG Electronics | Fine with the proposal |

- Inter-gNB and inter-UE CLI handling schemes

The guidance from Mr. Chair on discussions of CLI handling in AI 9.3.2 and AI 9.3.3 is as follows. Accordingly, this section discusses the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

Guideline for future meetings - Note: AI 9.3.3 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD. - Note: AI 9.3.2 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

In addition, according to the guidance from Mr. Chair, L1/L2 based CLI measurement/report for UE-to-UE CLI handling is to be discussed in AI 9.3.3 and exchange of intended subband configurations for SBFD operation across gNBs is to be handled in AI 9.3.2.

  - Summary of input contributions

The inputs from companies’ contributions on inter-gNB and inter-UE CLI handling schemes are summarized below as per moderator’s understanding. Moderator would like to apologize in advance if your views are not correctly captured or missed and encourage companies to correct/update the summary with revision marks if needed.

    - UE-to-UE CLI handling

Potential enhancements to UE-to-UE CLI measurement/reporting

The following agreements were made in RAN1#110.

Agreement (RAN1#110) Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for SBFD, at least includes: - Measurement resource/reporting configuration - Measurement/reporting details (including UE processing delay) - Relevant information exchange (between gNBs) if needed - Usage of measurement at gNB Note: other enhancement(s) for gNB-to-gNB and UE-to-UE CLI handling specific for SBFD are not precluded.

CMCC proposed to study the following two methods for inter-UE inter-subband CLI measurement.

- Method#1: victim UE measures RSSI/SINR within DL subband;

- Method#2: victim UE measures RSRP of aggressor UE’s CLI-SRS within UL subband.

Figure 5-16: Inter-UE inter-subband CLI measurement method#1 & method#2

Qualcomm also think it is beneficial for gNB to have UE CLI reports based on the CLI measurements within the UL subband and DL subbands.

CLI measurement in UL subband was discussed in last meeting. Companies discussed whether the restriction in Rel-16 that CLI is only measured within DL BWP would forbid UE to measure CLI in UL subband, and whether any enhancement is needed to support CLI measurement in UL subband. Intel discussed in [15] that Rel-16 CLI measurement mechanism supports CLI measurement in UL subband as long as the frequency resources are within DL active BWP, as for Case 1 and Case 2 below. For CLI measurement in UL subband outside of active DL BWP, Case 3, below, no clear motivation is identified.

In addition, whether a UE can perform CLI measurement when the UE receives DL channels/signals is per UE capability in Rel-16 and can be applied for enhanced duplex.

| Case 1 | Case 2 | Case 3 |
| --- | --- | --- |

Figure 5-17: CLI measurement in UL subband [15]

Companies are encouraged to answer Question 3-1.

The following agreement was made in RAN1#110bis-e.

Agreement Study impact/potential enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency.

CATT think that if the CLI in two DL subbands is considered to be non-symmetric, separate CLI-RSSI measurement resources/reports can be configured in different DL subbands according to existing Rel-16 CLI-RSSI mechanism; otherwise if the CLI in two DL subbands is considered to be symmetric, i.e. CLI in RBs in two DL subbands with the same frequency separation from UL subband is the same, it seems sufficient to measure CLI in only one DL subband. So it is proposed to further discuss the motivation and potential benefits for enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency.

LG think that the measured strength of CLI may be different depending on the position of DL. But, if the UE-to-UE CLI-RSSI measurement/report is taken non-contiguous measurement resource in frequency into account, the measured strength of CLI is averaged over the non-contiguous measurement resource. Hence, it is hard to determine which part of DL subband is suffered from intra-cell inter-UE CLI. So it is proposed that the use cases and potential benefits for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency should be justified.

Lenovo support non-contiguous CLI-RSSI measurement resource configuration in frequency for inter-subband CLI measurement.

Huawei proposed to study the following options:

- Option 1: Configure two CLI-RSSI resources associated with a same CLI-RSSI report ID

- Option 2: Configure non-consecutive CLI-RSSI frequency domain resources across DL subbands by one CLI-RSSI resource

- Option 3: Define subband report mechanism for CSI-RSSI, where each subband only contains DL RBs in the DL subband

Spreadtrum proposed to further study the following options:

- Option 1: One contiguous frequency domain resource allocation for UE-to-UE CLI-RSSI resource which the resources overlapping with UL subband or guardband are blank out.

- Option 2: There are two separate frequency domain resource allocations for a UE-to-UE CLI-RSSI resource if there are two DL subbands.

Intel think the mechanism to enable single measurement for different PRBs in the different DL subband would be desirable considering interference would be similar for DL resources in different DL subbands with same distance between DL and UL resources. Enhancement of configuration for non-contiguous PRBs for a CLI measurement resource can be studied. Alternatively, gNB may configure separate CLI measurement resource in two DL subbands while enhancement to associate multiple resources with single measurement result can be considered.

Figure 5-18: Single CLI measurement resource in two DL subbands [15]

Qualcomm proposed that UE implicitly determines the non-contiguous frequency resources for CLI-RSSI measurement in the DL subband(s) by excluding the UL subband and any guardband.

Ericsson think the same solution for non-contiguous CSI-RS resource could be envisioned for extending CLI-RSSI measurement resources, i.e. one solution is to make use of the RRC configuration of RB sets as an implicit indication of RBs that are not available for DL transmission, and another solution is to extend the current CSI-RSSI resource definition to explicitly configure non-contiguous FDRA in the two non-contiguous 'D' RB sets.

Companies are encouraged to answer Question 3-2.

CLI measurement and reporting based on a finer granularity are considered by ZTE, Sony, CMCC, Intel, DOCOMO, NEC, Qualcomm and Nokia considering the CLI is not frequency flat. Intel pointed out that separate CLI measurement on PRBs in the same DL subband is already supported by Rel-16 CLI-RSSI measurement, as shown below.

Figure 5-19: Two CLI measurement resources in a DL subband [15]

Some companies propose to support subband-based CLI reporting. In addition, Qualcomm proposed that UE report CLI measurements in specific subband(s) where CLI exceeds a configured CLI threshold to reduce subband CLI reporting overhead.

Companies are encouraged to answer Question 3-3.

InterDigital proposed to study measurement resources and reporting configurations for subband-edge CLI measurement. For example, the UE can measure a delta parameter that is based on measuring and calculating the difference between a first CLI-RSSI measured from the resources in the edge of the configured RBs and a second CLI-RSSI measured from the resources located in the middle of the configured RBs.

Apple proposes that potential aggressor UE, UEA, is indicated to measure SRS (transmitted by potential victim UE) before PUSCH transmission. DL CLI indication, e.g., based on DL-PI, indicates which symbols were impacted by cross-link interference from aggressor UE(s).

LG proposed to study

- Measurement gap for measurement inter-subband for UE-to-UE SRS-RSRP measurement/report for non-contiguous measurement resource

- UE-to-UE SRS-RSRP measurement within active DL BWP with wide bandwidth

Qualcomm proposed that gNB to exchange information of the CLI resource configurations and/or CLI measurements for the inter-cell CLI use-case which is beneficial to identity aggressor UEs for scheduling decision and UE grouping.

Power control based solution

ZTE proposed that UL subband resources can be divided into multiple areas and each area is mapped with a dedicated power control parameter set for compensating the inter-subband interference with different levels.

Panasonic proposed to study UL transmission power limitation, e.g. UL transmission power is differentiated between SBFD symbol and normal UL symbol especially for higher layer configured UL transmission (i.e., the maximum UL transmission power is limited on SBFD symbols).

Sony proposed to support non-uniform MCS and non-uniform power control in a PUSCH considering that inter subband CLI is non-uniform across a subband.

InterDigital proposed to study dynamic UL power control mechanism based on some dynamic factors such as the frequency gap, beam/spatial-domain parameter, or a priority indication on the UL, to mitigate the effects of the CLI dynamically.

UE and gNB transmission and reception timing

Nokia observed that by knowing the time difference of arrival between DL (transmitted by the serving gNB) and UL RS (transmitted by the aggressor UE) at the victim UE, the gNB can estimate the intra-cell (aggressor)UE-to-(victim)UE propagation delay. This information can be used to assist the UE by e.g. providing the specific timing offset to be used when performing a CLI measurement. It is proposed to study schemes for measurement and reporting of time difference of arrival between DL RS and UL RS, and potential extensions to the CLI measurement framework to include assistance information consisting of e.g. specific timing offset (with respect to DL timing) to be used when performing a CLI measurement.

Others

ZTE proposed that different frequency densities can be configured for reference signals transmitted in different areas with different interference levels.

Nokia think it would be beneficial if the UE could measure and report to the gNB its own in-band emissions. In principle the same measurement and reporting framework could be extended to out-of-band emissions. This functionality could be included within the subband CLI measurement framework by requesting UE to be able to measure the RSSI on a subband while transmitting on another subband.

InterDigital proposed to consider mechanisms to apply measurement skipping on some SBFD slots/symbols and power adjustment in deriving a CSI, depending on a level of dynamic power management occurred in the SBFD scenario. In addition, it is proposed to study a conditional CLI handling behavior based on monitoring the beams at the victim UE side, where the condition can at least include a case when the victim UE detects a PDSCH reception failure, which initiates a subband-wise CLI measurement/reporting for a subband switching to avoid the CLI.

    - gNB-to-gNB CLI handling

For gNB-to-gNB CLI handling, exchange of intended subband configurations for SBFD operation across gNBs are considered to be beneficial by ZTE, Intel, Panasonic, NEC, DOCOMO, Qualcomm and Nokia.

Spreadtrum and Qualcomm proposed to consider subband-based inter-gNB CLI reporting for accurate measurement of CLI leakage in SBFD considering that inter-gNB CLI leakage to adjacent RBs/subbands is not uniform over the measurement bandwidth.

CMCC proposed to study the following two methods for inter-gNB inter-subband CLI measurement.

Figure 5-20: Inter-gNB inter-subband CLI measurement method#1 & method#2 [19]

In addition, CMCC proposed to support inter-gNB coordination in time-domain, frequency-domain, spatial-domain, and power domain and backhaul signalling enhancement is needed to support inter-vendor cooperation.

- Method#1: victim gNB measures leakage power from aggressor gNB within UL subband;

- Method#2: victim gNB measures RSRP of aggressor gNB’s RS within DL subband.

Huawei proposed to study the following aspects related to advanced IRC receivers in SBFD.

- Feasibility and performance of muting the REs on the DL subband in UL DMRS symbols and the REs on the UL subband in DL DMRS to improve channel estimation and inter-cell interference estimation and suppression.

- Feasibility and performance of specific CLI measurement resources to improve gNB-gNB and UE-UE co-channel and adjacent-channel inter-subband CLI estimation and suppression.

Figure 5-21: Interference management for interference suppression based on advanced receiver [6]

In addition, Huawei proposed to study the feasibility and performance of beam nulling for gNB-to-gNB CLI suppression with gNB-to-gNB channel.

Similar as ZTE’s proposal for UE-to-UE CLI, ZTE proposed that different frequency densities can be configured for reference signals (e.g., RS for gNB-to-gNB CLI measurement, RS for uplink transmission channel estimation) transmitted in different areas with different interference levels.

  - [Open] 1st round discussion

Question 3-1

Do you see any necessity of enhancements for CLI measurement and report to support CLI measurent in UL subband? If your answer is yes, please elaborate.

|  | Company |
| --- | --- |
| Yes | Sony, LG Electronics, ZTE, Nokia, NSB,Yes |
| No | Samsung New H3C, xiaomi, NEC, Intel |

| Company | Comments |
| --- | --- |
| Moderator | Several companies proposed to support CLI measurement in UL subband. Based on the discussions in last RAN1 meeting and the contribution from Intel, it seems that the Rel-16 CLI measurement/report scheme can already support CLI measurement in UL subband as long as the frequency resources are within DL active BWP. So companies are encouraged to answer the above question to see whether any further enhancement is needed. |
| New H3C | Rel-16 CLI meas and report can be reused. Non-continuous measure resource configuration need be further considered |
| LG Electronics | Interference scenario of SBFD is quite different from the interference scenario of dynamic TDD. That is main motivation of enhancment of UE-to-UE CLI measurement for SBFD, especially for SRS-RSRP measurement. UE with SBFD capability can operate DL-only mode which is baseline operation. Hence, UE may operate UE-to-UE CLI measurement within wider BW of active DL BWP for SBFD operation. |
| ZTE | It seems the victim needs to identify the potential aggressor. This can be one of the potential enhancements. |
| Nokia, NSB | We think that based on new L1/L2 framework that could be agreed in A.I. 9.3.3, there could be specific enhancements for SBFD operation to facilitate the measurement and reporting of intra-cell UE-2-UE CLI. |
| Lenovo | Rel-16 CLI-RSSI and SRS-RSRP measurement and reporting mechanism can be used with support of non-contiguous CLI-RSSI measurement resource configuration in frequency for inter-subband CLI measurement. |

Question 3-2

What are the motivation/use case/potential benefits for enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency?

| Company | Comments |
| --- | --- |
| Moderator | It was agreed to study impact/potential enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency. Based on the inputs, the motivation/use case/potential benefits for such enhancements seem to be not clear to some companies. Please refer to section 2.1.1 for more details. Before discussing the detailed enhancements, proponent companies are encouraged to share your views on the motivation/use case/potential benefits for enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency. |
| Sony | Is this question targeting {DUD} configuration? If yes, it makes sense to study non-contiguous measurement since the upper DL Subband can have different CLI compared to the lower DL Subband. |
| New H3C | For DUD case, non-contiguous measurement resource in frequency can be used for evaluationon CLI in both two sperated DL-subbands |
| DOCOMO | We think non-contiguous measurement resource is useful for SBFD operation with {DUD} pattern. Otherwise multiple CLI-RSSI resources may be required for the non-contiguous DL subbands. |
| Intel | For {DUD case}, we tend to agree with CATT’a analysis, if the interference to be measured is symmetric, it seems CLI-RSSI resource in one DL subband is sufficient, while if the interference to be measured is asymmetric, CLI-RSSI resource in two DL subband is needed, and enhancement for non-contiguous measurement resource in frequency can be studied. However, for the second use-case, the baseline consideration should be to use different CLI-RS resources. |
| ZTE | We think it can be combined with the potential solution discussed in question 3-3, finer granularity. |
| Nokia, NSB | We think that motivation/use case/potential benefits for enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency are the same as the motivation/use case/potential benefits for enhancements of CSI-RS measurement/report considering non-contiguous measurement resource in frequency. |
| Lenovo | Non-contiguous CLI-RSSI measurement resource corresponding to DL subbands of SBFD symbols instead of needing multiple CLI-RSSI resources for non-contiguous DL subbands. |

Question 3-3

What is the motivation and potential benefits to support CLI measurement and reporting based on a finer granularity?

| Company | Comments |
| --- | --- |
| Moderator | A number of company proposed CLI measurement and reporting based on a finer granularity considering the CLI is not frequency flat. But as pointed out by Intel, separate CLI measurement on PRBs in the same DL subband is already supported by Rel-16 CLI-RSSI measurement. Proponent companies are encouraged to share your views on the motivation and potential benefits for further enhancements. |
| Sony | It is obvious in an inter-subband CLI, the frequency portion of a DL subband closer to an UL subband would experience higher inter-subband CLI compared to a frequency portion of a DL subband further away from the UL subband and vice-versa for the CLI experience in UL subband. Hence it makes sense to have finer granularity of CLI reporting so that the gNB can have more accurate scheduling on different RBs of the UL/DL subband. |
| New H3C | Rel-16 CLI-RSSI measurement mechanism shoud be reused as much as possible. |
| DOCOMO | The interference level in different subband positions may be different., considering different leakage due to inter-subband CLI. |
| Intel | No strong motivation is seen as dsicusseed in last RAN1 meeting. |
| ZTE | Similar views as Sony, DOCOMO. CLI measurement and report with finer granularity is required for reflecting the changes of interference in different frequency resources. |
| Nokia, NSB | To our understanding, separate CLI-RSSI measurement on PRBs in the same DL subband is supported in Rel-16 by configuring multiple RSSI resource configurations. The enhancement discussed by several companies is about a single RSSI resource configuration with UE reporting the RSSI measured in different subbands, in similar way as it is possible for CSI subband reporting. Therefore, CLI-RSSI resource configuration and reporting in line with CSI framework could be supported for SBFD. As discussed in our Tdoc, we think that UE should have the capability of measuring the CLI-RSSI in DL subband(s) while transmitting in UL subband, and this should also be be configurable by the network. |

Proposal 3-4

Study exchange of intended subband configurations for SBFD operation across gNBs for coordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB co-channel CLI handling.

|  | Company |
| --- | --- |
| Support | Sony, Samsung, New H3C, DOCOMO, NEC, Panasonic, LG Electronics, ZTE, Nokia, NSB, Lenovo |
| Not support | Intel (we think such information exchange is also beneficial for UE-to-UE CLI handling) |

| Company | Comments |
| --- | --- |
| Moderator | The following proposal was extensively discussed in the last meeting. Mr. Chair has provided guidance that exchange of intended subband configurations for SBFD operation across gNBs is to be handled in AI 9.3.2. With the above guidance, is the proposal agreeable? |
| New H3C | We are fine with this proposal |
| Intel | It is unclear to us, why SBFD information exchange is only for gNB-to-gNB CLI handling? It is also useful for UE-to-UE CLI handling. Same information exchange is applicable to both gNB-to-gNB and UE-to-UE CLI handling. Explanation would be appreciated. |
| LG Electronics | We are fine with the proposal in principle. |
| Nokia, NSB | We think this is a simple extension of the exchange of information on the intended TDD frame structure agreed for dynamic TDD in Rel-16. |

- Offline discussion

  - Nov. 14th (Mon)

Proposal 1-3

Proposed Agreement:

Study SBFD operation [for SBFD aware UE] with UL subband in symbols configured as DL in TDD-UL-DL-ConfigCommon and in symbols configured as flexible in TDD-UL-DL-ConfigCommon.

Proposal 1-3a

Proposed Agreement:

For semi-static configuration of subband, it is agreed in RAN1 study that UL subband can be configured in symbols configured as DL in TDD-UL-DL-ConfigCommon and in symbols configured as flexible in TDD-UL-DL-ConfigCommon

Proposal 1-5a

For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

  - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived

  - FFS: UE does not expect to receive or transmit in the RBs which are not within DL subband(s) or UL subband

- A set of contiguous RBs in a carrier excluding RBs of UL subband and guardband(s) are defined as a DL subband for discussion purpose

  - Either the number of RBs for guardband(s) or the frequency location of DL subband(s) is explicitly indicated

- DL receptions within DL subband(s) are allowed in the symbol

- FFS whether/how DL receptions within RBs of semi-statically configured UL subband are allowed in the symbol

- FFS whether DL receptions within UL subband are allowed or not in the symbol

- FFS whether/how the symbols can be converted to DL-only symbols in the symbol

- Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol as in existing specification

- Contact person

Please provide/update the information of the contact person in the following table to facilitate the discussions.

| Company | Name | Email address |
| --- | --- | --- |
| Sony | Shin Horng Wong | shinhorng.wong@sony.com |
| InterDigital | Jonghyun Park | jonghyun.park@interdigital.com |
| Sharp | Tomoki Yoshimura | yoshimurat@sharplabs.com |
| Qualcomm | Muhammad Abdelghaffar | mabdelgh@qti.qualcomm.com |
| New H3C | Lei Zhou | zhou.leih@h3c.com |
| New H3C | Lei Kong | Kong.lei@h3c.com |
| vivo | Lihui Wang | wanglihui@vivo.com |
| NEC | Pravjyot Singh Deogun | pravjyot.deogun@emea.nec.com |
| Xiaomi | Lei Wang | wanglei25@xiaomi.com |
| OPPO | Wenfeng Zhang | zhangwenfeng@oppo.com |
| Ericsson | Stephen Grant | stephen.grant@ericsson.com |
| Spreadtrum | Huan Zhou | Huan.Zhou@unisoc.com |
| CATT | Yanping Xing | xingyanping@catt.cn |
| Panasonic | Tomoya Nunome | nunome.tomoya@jp.panasonic.com |
| Intel | Yi Wang | yi5.wang@Intel.com |
| ITRI | Jen-Hsien Chen | itriA40175@itri.org.tw |
| Lenovo | Hyejung Jung | hyejung@motorola.com |
| ETRI | Hoondong Noh | hoondong.noh@etri.re.kr |
| ZTE | Xingguang WEI | wei.xingguang@zte.com.cn |
| Samsung | Marian Rudolf Kyungjun Choi | m.rudolf@partner.samsung.com kyungj.choi@samsung.com |
| CMCC | Tuo Yang Fei Wang | yangtuo@chinamobile.com wangfei@chinamobile.com |
| DOCOMO | Qiping Pi | piqp@docomolabs-beijing.com.cn |
| WILUS | David (Geunyoung) Seok | david.seok@wilusgroup.com |
| CEWiT | Priyanka Dey | priyanka@cewit.org.in |
| Nokia, NSB | Jingyuan Sun | Jingyuan.sun@nokia-sbell.com |
| Huawei, HiSilicon | Xinghua Song | songxinghua@huawei.com |
| MediaTek | Mohammed Al-Imari | Mohammed.Al-Imari@mediatek.com |
| LG Electronics | Hyunsoo Ko | hyunsoo.ko@lge.com |
| SK Telecom | Sanghoon Cho | seanc.cho@sk.com |
| KDDI | Masahito Umehara | ma-umehara@kddi.com |
| TCL | Shahid Jan | shahid.jan@tcl.com |
| Fujitsu | Taewoo LEE | lee.taewoo@fujitsu.com |
| Apple | Ali Fakoorian | sfakoorian@apple.com |

- Reference

- RP-220633		Revised SID: Study on evolution of NR duplex operation	CMCC

- R1-2205399		TR 38.858 skeleton for study on evolution of NR duplex operation	Rapporteur (CMCC)

- R1-2205520		Summary #2 of [109-e-R18-Duplex-03] Email discussion on subband non-overlapping full duplex	Moderator (CATT)	RAN1#109-e

- R1-2208122		Summary #5 of subband non-overlapping full duplex	Moderator (CATT)	RAN1#110

- R1-2210317		Summary #4 of subband non-overlapping full duplex	Moderator (CATT)	RAN1#110bis-e

- R1-2210877		Discussion on potential enhancement on subband non-overlapping full duplex	Huawei, HiSilicon

- R1-2210932		Discussion for subband non-overlapping full duplex	New H3C Technologies Co., Ltd.

- R1-2211005		Discussion on subband non-overlapping full duplex	vivo

- R1-2211043		Discussion of subband non-overlapping full duplex	ZTE

- R1-2211065		Discussion on Subband non-overlapping Full Duplex 	TCL Communication Ltd.

- R1-2211084		Discussion on subband non-overlapping full duplex	Fujitsu

- R1-2211196		Discussion on subband non-overlapping full duplex	CATT

- R1-2211233		Discussion on subband non-overlapping full duplex	Spreadtrum Communications

- R1-2211362		Discussion on subband non-overlapping full duplex	xiaomi

- R1-2211398		On SBFD in NR systems	Intel Corporation

- R1-2211485		Discussion on subband non-overlapping full duplex	OPPO

- R1-2211559		Discussion on subband non-overlapping full duplex enhancements	ETRI

- R1-2211610		Considerations on Subband Full Duplex TDD Operations	Sony

- R1-2211680		Discussion on subband non-overlapping full duplex	CMCC

- R1-2211737		Discussion on subband non-overlapping full duplex operations		InterDigital, Inc.

- R1-2211748		Discussion on subband non-overlapping full duplex	NEC

- R1-2211779		Subband non-overlapping full duplex	Lenovo

- R1-2211812		Views on subband non-overlapping full duplex	Apple

- R1-2211876		Discussion on subband non-overlapping full duplex	Panasonic

- R1-2211922		Study on Subband non-overlapping full duplex	LG Electronics

- R1-2211942		Subband non-overlapping full duplex	Ericsson

- R1-2211983		Discussion on subband non-overlapping full duplex	NTT DOCOMO, INC.

- R1-2212043		SBFD feasibility and design considerations for NR duplex evolution	Samsung

- R1-2212115		Feasibility and techniques for Subband non-overlapping full duplex	Qualcomm Incorporated

- R1-2212149		Discussion on subband non-overlapping full duplex	Sharp

- R1-2212194		Details of subband non-overlapping full duplex	ASUSTeK

- R1-2212249		Discussion on subband non-overlapping full duplex for NR	MediaTek Inc.

- R1-2212284		On subband non-overlapping full duplex for NR	Nokia, Nokia Shanghai Bell

- R1-2212289		Discussion on subband non-overlapping full duplex	KDDI Corporation

- R1-2212290		Discussion on subband non-overlapping full duplex	KT Corp.

- R1-2212334		Discussion on sub-band non-overlapping full duplex	ITRI

- R1-2212424		Discussion on subband non-overlapping full duplex	CEWiT

- R1-2212438		Discussion on subband non-overlapping full duplex	WILUS Inc.

Appendix A: Observations/proposals from companies in RAN1#111

| R1-2210877 | Discussion on potential enhancement on subband non-overlapping full duplex | Huawei, HiSilicon |
| --- | --- | --- |
| Observation 1: SBFD operation across carriers does not require specific collision handling rules than the ones that also required by SBFD operation within a carrier. Observation 2: Introducing flexible subband can achieve flexible scheduling of option 2 to 4 without deviating from the definition of UL subband and DL subband. Proposal 1: SBFD operation within a carrier and across carriers can be studied with equal priority. Proposal 2: Define the UL subband, DL subband, and flexible subband as follows: - UL subband is defined as a set of consecutive UL RBs, where the UL RB can only be used for UL transmission; - DL subband is defined as a set of consecutive DL RBs, where the DL RB can only be used for DL transmission; - Flexible subband is defined as a set of consecutive flexible RBs, where - the flexible RB can be used for UL transmission, DL transmission, and guard band, and - the flexible RB can be reconfigured as UL RB or DL RB by UE dedicated RRC and/or DCI. Proposal 3: The frequency domain location indication of DL subband, flexible subband, and guard band are given as follows: - DL subband should be explicitly indicated, at least for the length of DL subband; - Flexible subband can be implicitly indicated as a set of consecutive of RBs which are not indicated as UL RB and DL RB; - Guard band can be implicitly indicated based on scheduling, i.e., gNB will not schedule UL and DL transmission on guard band. Proposal 4: Study dynamic time domain location indication of subband for SBFD operation. Proposal 5: Study RA type 0 enhancement for PDSCH and PUSCH transmission in a RBG overlapping partially with UL and DL subband boundary, respectively, for SBFD operation. Proposal 6: Study RA type 1 enhancement for PDSCH transmission across DL subbands in the case of subband pattern {DUD} for SBFD operation. Proposal 7: Study multiple-slots PUSCH/PUCCH enhancement for SBFD operation to avoid multiple-slots PUSCH/PUCCH transmission across different slot types, including: - Available slots determination for Rel-17 PUSCH repetitions and TBoMS PUSCH; - Collision handling rules for Rel-15/16 PUSCH/PUCCH repetitions. Proposal 8: Study PUSCH/PUCCH frequency hopping for SBFD operation to keep the PUSCH/PUCCH frequency hopping always within the UL subband on SBFD symbol. Proposal 9: Study new RM pattern for PDSCH, which is associated with UL subband. Proposal 10: Study the enhancement for the TBS determination for SBFD operation. Proposal 11: Study following potential enhancement of CSI-RS measurement and report for SBFD operation in the case of subband pattern {DUD}: - Option 1: Configure two CSI-RS resources associated with a same CSI-RS report ID. - Option 2: Configure non-consecutive CSI-RS frequency domain resources across DL subbands by one CSI-RS resource. - Option 3: Update the RBG division mechanism to ensure that each RBG only contains DL RBs in the DL subband. Proposal 12: Study potential enhancement of SBFD operation on SSB symbols. Proposal 13: Study potential enhancement of initial access enhancement for SBFD operation. Proposal 14: UE half-duplex on handling conflict UL/DL indicating signaling for the same OFDM symbol should be studied, e.g., - Any DL signals or channels are indicated to transmit on the DL subband in a symbol configured as semi-U. - Any UL signals or channels are indicated to transmit on the UL subband in a symbol configured as semi-D. - Any UL signals or channels are indicated to transmit on the UL subband in a symbol indicated to receive SSB on the DL subband. - SSB is indicated to receive on the DL subband in a symbol configured as semi-U. - Valid PRACH is indicated to transmit on UL subband in a symbol configured as semi-D. - CORESET 0 configured in the DL subband in a symbol configured as semi-U. Proposal 15: Study the following aspects on gNB-gNB co-channel CLI measurement in SBFD. - Feasibility and performance of muting the REs on the DL subband in UL DMRS symbols and the REs on the UL subband in DL DMRS to improve channel estimation and inter-cell interference estimation and suppression. - Feasibility and performance of specific CLI measurement resources to improve gNB-gNB and UE-UE co-channel and adjacent-channel inter-subband CLI estimation and suppression. Proposal 16: Study the feasibility and performance of beam nulling for gNB-to-gNB CLI suppression with gNB-to-gNB channel. Proposal 17: Study the feasibility and performance of applying filters at both transmitter and receiver sides in SBFD involving RAN4 on the following aspects. - Filter at transmitter to suppress the leakage interference. - Filter at receiver to suppress the blocking interference. - Guard band for filters. Proposal 18: For subband non-overlapping full duplex, the timing advance offset NTA,offset can be configured as 0 to avoid the inter-slot interference. Proposal 19: Study following potential enhancement of UE-UE CLI-RSSI measurement and report for SBFD operation in the case of subband pattern {DUD}: - Option 1: Configure two CLI-RSSI resources associated with a same CSI-RS report ID. - Option 2: Configure non-consecutive CLI-RSSI frequency domain resources across DL subbands by one CLI-RSSI resource. - Option 3: Define subband report mechanism for CSI-RSSI, where each subband only contains DL RBs in the DL subband. |  |  |
| R1-2210932 | Discussion for subband non-overlapping full duplex | New H3C Technologies Co., Ltd. |
| Proposal 1：Extend the functionality of flexible symbol for supporting SBFD operation, and the frame structure configuration and slot format indication mechanism in legacy TDD can be reused. Proposal 2：Support the configurations of a number of dedicated symbols as SBFD symbols, the dedicated SBFD symbols should be consecutive and start from the first symbol in a period of the frame structure. Proposal 3: For semi-static configuration of subband frequency locations for SBFD operation explicit indication of frequency location of DL subband that a set of contiguous RBs for DL transmission excluding RBs of UL subband is required. Proposal 4: Support dynamic SBFD resource allocation and coordination between cells for evolution of NR duplex operation by DCI. Proposal 5: RO for Type-1 random access procedure is supported to be configured in semi-static SBFD resource. Proposal 6: MsgA RO and MsgA PUSCH for Type-2 random access procedure is supported to be configured in semi-static SBFD resource. Proposal 7: The SSB overlapping with SBFD sub-band should carefully studied. The SSB can be configured inside the SBFD sub-band in the legacy UL symbols. The SSB may be configured overlapping with UL SBFD sub-band in legacy Flexible or DL symbols, in this case the SSB has priority. Proposal 8: The CORESET overlapping with SBFD sub-band should carefully studied. In legacy UL symbol with SBFD configured, only CORESET0 can be configured inside the SBFD sub-band, but has lower priority than RO. Proposal 9: In legacy F symbol with UL SBFD configured, common CORESET has priority, then RO, then UE-specific CORESET. In legacy DL symbol with UL SBFD configured, CORESET has priority. Proposal 10: UE is dynamically scheduled to transmit PUSCH w/o repetition in semi-static SBFD time-frequency resource in connection mode. Proposal 11: UE is dynamically scheduled to transmit PUSCH w/o repetition in dynamic allocated SBFD time-frequency resource in connection mode. Proposal 12: UE is scheduled to transmit SDT PUSCH in SBFD time-frequency resource in inactive mode. Proposal 13: UE is scheduled PDSCH in the DL SBFD resource. The confliction between PDSCH and UL channels and signals should be further studied. Proposal 14: At least collision between CG PUSCH/PUCCH and PDCCH / SPS PDSCH should be considered and handled. |  |  |
| R1-2211005 | Discussion on subband non-overlapping full duplex | vivo |
| Proposal 1: Both semi-static DL symbols and semi-static flexible symbols are considered as candidates to be configured as SBFD symbols. Proposal 2: RAN1 discusses and determines whether subband(s) are configured in carrier/serving cell level or BWP level. Proposal 3: The gNB configures the location and bandwidth for one or more subbands, and the following two options can be considered: - Option 1: More than one subband can be configured and the transmission direction for each subband is also configured. - Option 2: Only the location of the UL subband is configured, without configuring the transmission direction outside the UL subband. Proposal 4: For SBFD operation in semi-static flexible symbol(s), if there are flexible subband(s), resources in flexible subband(s) can be used as UL or DL without changing the UL subband size in frequency domain based on existing signaling. Proposal 5: For explicit configuration of SBFD subband time locations within a period, the following two options can be considered to determine the period: - Option 1: The period is determined by periodicity of the configured TDD pattern. - Option 2: The period is determined by a new configured periodicity. Proposal 6: RAN1 first discusses the desired SBFD operation, e.g., whether a symbol can be determined dynamically as a SBFD symbol or non-SBFD symbol before discussing the detailed signaling design and related UE behavior. Proposal 7: Study dynamic indication of whether or not to apply the SBFD frequency pattern in time domain. Proposal 8: For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - A set of contiguous RBs in a carrier excluding RBs of UL subband and guardband(s) are defined as a DL subband for discussion purpose - FFS whether frequency location of DL subband(s) and guardband(s) are explicitly indicated or implicitly determined - DL receptions within DL subband(s) are allowed in the symbolStudy dynamic indication of whether or not to apply the SBFD frequency pattern in time domain. - Allow conversion from/to SBFD symbols to/from DL-only symbols - FFS explicit and/or implicit conversion Proposal 9: For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - DL receptions outside UL subband are allowed in the symbol - Allow conversion from/to SBFD symbols to/from UL-only or DL-only or legacy flexible symbols - FFS explicit and/or implicit conversion - FFS: interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI - FFS: whether/how guardband(s) are explicitly indicated Proposal 10: For time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE, the following cases can be considered: - Case 1: Configured UL transmission(s) and configured DL reception(s) in the same SBFD symbol. - Case 2: Configured UL transmission(s) and scheduled DL reception(s) in the same SBFD symbol. - Case 3: Scheduled UL transmission(s) and configured DL reception(s) in the same SBFD symbol. - Case 4: Scheduled UL transmission(s) and scheduled DL reception(s) in the same SBFD symbol. - Case 4 can be agreed as an error case. Proposal 11: Different transmission/reception parameters can be configured and applied to SBFD slot and UL/DL slot to better match conditions of different slot types for better performance. Proposal 12: For potential enhancements for CSI measurement and reporting for SBFD, the following can be considered: - For CSI-RS resource configuration, an NZP CSI-RS resource or CSI-IM resource can be allocated non-contiguous frequency resources. - When performing CSI reporting, the UE can handle collision between NZP CSI-RS/CSI-IM resources and UL subband/guard band(s) based on predefined rules. Proposal 13: No UE RF impact for CLI handling is expected to avoid additional UE complexity in Rel-18 SBFD operation. Proposal 14: CSI measurement and reporting can be used for interference measurement for UE-to-UE CLI handling. |  |  |
| R1-2211043 | Discussion of subband non-overlapping full duplex | ZTE |
| Overview Proposal 1: The solution/scheme of duplex evolution takes the following challenges of conventional TDD operation into account. - Challenge : Ensuring UL throughput + UL coverage simultaneously. - Challenge : Ensuring UL throughput + DL&UL Latency simultaneously. - Challenge : Ensuring UL coverage + DL&UL Latency simultaneously. - Challenge : Ensuring DL throughput + DL&UL Latency simultaneously. - Challenge : Ensuring DL throughput + UL throughput simultaneously. - Challenge : Ensuring DL throughput + UL coverage simultaneously. SBFD framework and configuration Proposal 2: In addition to the RB-set based solution, RAN1 further studies the BWP-based solution for SBFD configuration. Proposal 3: RAN1 clarifies and further studies whether to introduce “flexible subband” in addition to UL subband. Proposal 4: Regarding the suband configuration, - Frequency-domain resource configuration and time-domain resource configuration are cell-common. - Detailed signal/channel configuration for each UE within the subband is UE-specific. - FFS whether UE-specific UL subband configuration is needed for UE. Proposal 5: For SBFD operation, both of the followings are supported: - A SBFD aware UE is semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon - A SBFD aware UE is semi-statically configured with UL subband in a symbol configured as flexible in TDD-UL-DL-ConfigCommon Proposal 6: For semi-static configuration of subband time locations for SBFD operation, time domain window (e.g., period, offset and length) can be configured by RRC for SBFD operation for better compatibility with less impact on legacy UE and procedures. Proposal 7: For SBFD operation, guard band can be reserved between DL subband and UL subband explicitly or implicitly. - UE is not expected to transmit uplink transmission in the guard band. - UE is allowed to receive DL transmission in the guard band. Proposal 8: Regarding the frequency domain configuration of UL subband, RRC parameter locationAndBandwidth can be reused for configuring the starting point and bandwidth of the UL subband, Point A can be served as the reference point. Proposal 9: UE is allowed to transmit UL transmission in UL subband only if the UL subband is confined within it activated UL BWP. Proposal 10: RAN1 further studies the following alternatives for UL subband configuration. - Alt.1: Each UL BWP can be associated with up to one UL subband. More than one UL subband for different UL BWPs configured in one carrier can be configured, while up to one UL subband can be activated at one time. - Alt.2: One UL subband is configured in one carrier. The UL subband is activated only if the UL subband is confined within the activated UL BWP. Proposal 11: RAN1 studies explicitly/implicitly updating the time domain location of the subband dynamically. Proposal 12: For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, prioritize the study of following options: - Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol - Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol FFS: Whether DL transmission is allowed in all the UL subband resources or only a certain area. FFS: gNB transmits DL reference signal in UL subband. Proposal 13: Regarding confliction of Case 1: UL vs. DL, consider the following potential solution. - Option 1: When DL transmission and UL transmission have the same priority and both of them are DG or CG, if DL transmission is scheduled in DL subband and UL transmission is scheduled in UL subband, UL transmission scheduled in UL subband is always prioritized. If both DL transmission and UL transmission are scheduled in the UL subband, DL transmission is always prioritized. - Option 2: When DL transmission and UL transmission have different priorities, DL/UL transmission with high priority always survives. - Option 3: When dynamic DL (or UL) transmission and semi-static UL (or DL) transmission have the same priority and conflict in the time domain, the dynamic transmission is prioritized. Proposal 14: RAN1 studies UE collision handling at least for the following cases. - Case 1: UL vs. DL - Case 2: Semi D vs. UL - Case 3: SSB vs. UL - Case 4: Type 0 CSS vs. UL Proposal 15: Study the impact and benefits of potential enhancements to resource allocation granularity in frequency-domain for SBFD operation, including at least the following: - RBG for PDSCH/PUSCH FDRA - CSI reporting subband - PRG of PDSCH - RBG for CORESET Proposal 16: Study impact and potential enhancements for scheduled/configured UL transmissions and scheduled/configured DL receptions across SBFD symbols and non-SBFD symbols, including at least the following: - Scheduled/configured PUCCH/PUSCH/PDSCH/PDCCH without repetition in SBFD symbols and non-SBFD symbols - Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols - Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition - Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols - Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols - Shared/separate configuration for PUCCH/PUSCH/PDSCH/PDCCH/SRS/CSI-RS in SBFD symbol/slot and non-SBFD symbol/slot Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered. Proposal 17: RAN1 further studies the potential enhancements for initial access in the UL subband at least for the triggered events related to the RRC_CONNECTED mode. CLI handling Observation 1: The potential backward compatibility problem exists if NTA,offset=0 is set for all UL transmission. Proposal 18: For avoiding inter-slot interference under SBFD, two values of NTA,offset can be defined for UL transmission in UL subband and UL slot, respectively. - NTA,offset=0 for UL transmission in UL subband - NTA,offset>0 for UL transmission in UL slot Observation 2: The uplink transmissions in the UL subband are subject to different interference levels due to the following aspects, - The frequency domain isolation between the uplink transmission and the DL subband. - Time domain areas where DL transmissions are scheduled or configured in DL subband. Proposal 19: UL subband resources can be divided into multiple areas and each area is mapped with a dedicated power control parameter set for compensating the different levels of inter-subband interference. - The resources contained in each area can be semi-statically configured via RRC or indicated by DCI. Proposal 20: UE-to-UE CLI measurement and reporting mechanisms for non-uniform inter-subband interference should be discussed in AI 9.3.2. Observation 3: CLI measurement in DL subband is helpful for obtaining inter-subbands interference with a non-uniform feature. Wideband CLI measurement and reporting may fail to reflect the changes of inter-subband interference in different frequency resources. Proposal 21: UE-to-UE CLI measurement/reporting in both of DL subband and UL subband should be supported, - Further study subband CLI measurement and reporting for UE-to-UE CLI handling under SBFD, e.g., configuration and determination of the measurement subband size and measurement reporting overheads reduction, etc. Proposal 22: Different frequency densities can be configured for reference signals (e.g., RS for UE-to-UE CLI measurement, RS for downlink transmission channel estimation) transmitted in different areas with different interference levels. Proposal 23: Regarding UE-to-UE CLI, UL subband resources can be divided into multiple areas and each area is mapped with a dedicated power control parameter set for suppressing the UE-to-UE CLI with different interference levels. - The resources contained in each area can be semi-statically configured by RRC or indicated by DCI. Proposal 24: Different frequency densities can be configured for reference signals (e.g., RS for gNB-to-gNB CLI measurement, RS for uplink transmission channel estimation) transmitted in different areas with different interference levels. Proposal 25: Time and frequency domain configuration of UL subband should be exchanged between gNBs. |  |  |
| R1-2211065 | Discussion on Subband non-overlapping Full Duplex | TCL Communication Ltd. |
| Observation 1: An explicit indication of frequency location of other subbands types (DL Subbands), may let the UEs to implicitly determine the guard bands between the DL and UL subbands. Observation 2: Time window based solution to indicate x number of time slots/symbols to the UEs for SBFD operation and legacy TDD operation may provide assistance in interference management between SBFD operation and legacy TDD operation in the neighbor cells. Observation 3: Indication of time location of subbands to UE may requires an extension of TDD-UL-DL-ConfigCommon and TDD-UL-DL-ConfigDedicated to include an indication of the slots/symbols which is used for SBFD operation. Observation 4: Dynamic Indication of frequency location indication provides flexibility to adapt to the DL and UL traffic and to handle the DL and UL tranmission collision. Observation 5: The current SFI may requires an extension to include an indication for starting of slots/symbols and number of slots/symbols which is used for SBFD operation. Observation 6: The following scenarios may create gNB to gNB and UE to UE co-channel intra sub-band interference; - Allocating different bandwidth to the UL to UL and DL to DL sub-bands among neighbor gNBs - Allocation of dissimilar sub-bands to UL and DL transmission between neighbor gNBs - Allocation of dissimilar quantity of sub-bands to UL and DL transmission between neighbor - Configuration of different numbers of sub-bands between neighbor gNBs Proposal 1: Support an explicit indication of the frequency locations of all subbands (DL and UL) within a TDD band to the UE. Proposal 2: Study a time domain window based solution to indicate the time slots/symbols of SBFD operation and legacy TDD operation to the SBFD UEs and legacy operation UEs respectively. Proposal 3: Support Dynamic Indicaiton of frequency location of subbands. Proposal 4: For dynamic Indicaiton of frequecy location of subbands, DCI based indication and/or MAC CE based indication can be further studied. Proposal 5: Consider semi static configuration of multiple subband sizes within a given TDD band, and dynanmic indication of time/frequency location of subbands to activate/de-activate an UL or DL subbands. Proposal 6: To avoid the intra sub-band interference in SBFD operation the neighbor’s gNB shall: - Keep the same bandwidth of UL to UL and DL to DL sub-bands among the neighbor gNBs - Assign alike sub-bands to UL and DL transmission among the neighbor gNBs - Allocate similar quantity of sub-bands to DL and UL transmission among the neighbor gNBs - Configure similar numbers of sub-bands among the neighbor gNBs |  |  |
| R1-2211084 | Discussion on subband non-overlapping full duplex | Fujitsu |
| Proposal 1: - Support UL subband on semi-static DL symbol, - UL transmissions outside UL subband are not allowed - DL receptions within UL subband are allowed - Guardband can be configured by gNB scheduling - CSI measurement is not supported on the resource overlapped with UL subband. Proposal 2: - The UL subband on Flexible symbol should be operated depending on Flexible symbol determination - Flexible is determined to DL: Same behaviour with UL subband on DL - Flexible is determined to UL: converting to UL-only - Remaining Flexible: Same behaviour with current specification assuming UL subband as Flexible Observation 1: - Due to the CLI from PRACH transmission procedure on UL subband, the overall system performance will decrease. Proposal 3: - PRACH coverage enhancement is better to be discussed in further NR coverage enhancement agenda (NR_cov_enh2) |  |  |
| R1-2211196 | Discussion on subband non-overlapping full duplex | CATT |
| Observation 1: The motivation and potential benefit for enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands is unclear assuming CSI-RS sequence generation is unchanged. Observation 2: RBs for msg3 PUSCH and PUCCH during initial access may be located outside UL subband if time and frequency location of UL subband is not known to the UE. Proposal 1: For indication of subband locations for SBFD operation, cell-common indication of subband time and frequency location is the baseline. Proposal 2: For semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL subband is configured with reference to the corresponding TDD carrier. Proposal 3: It is agreed that for baseline SBFD operation, intra-subband CLI can be completely avoided by using same SBFD subband configuration across all the cells in the same carrier. Proposal 4: For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - A set of contiguous RBs in a carrier at least excluding RBs of UL subband is defined as a DL subband for discussion purpose - FFS whether frequency location of DL subband(s) is explicitly indicated or implicitly determined - FFS whether guardband(s) need to be defined and whether UE is aware of such guardband(s) - Note: if guardband(s) are defined, the DL subband(s) additionally exclude the RBs of the guardband(s) - DL receptions within DL subband(s) are allowed in the symbol - FFS whether DL receptions within UL subband are allowed or not in the symbol - FFS whether/how the symbols can be converted to DL-only symbols in the symbol - Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol as in existing specification Proposal 5: For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, it is agreed as baseline that DL receptions within UL subband are not allowed. - FFS enhancements to support DL receptions within UL subband Proposal 6: For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, guardband(s) between DL and UL subbands shall be defined and the configuration shall be informed to SBFD aware UEs. Proposal 7: It is preferred that frequency location(s) of DL subband(s) are implicitly determined based on frequency locations of UL subband and guardband(s). Proposal 8: For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, - UL transmissions within UL subband are allowed in the symbol - FFS whether DL receptions within UL subband are allowed or not in the symbol - FFS whether UL transmissions outside UL subband are allowed or not in the symbol - FFS whether DL receptions outside UL subband are allowed or not in the symbol - FFS: interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI - FFS whether/how the symbols can be converted to non-SBFD symbols - FFS: whether/how guardband(s) are explicitly indicated - Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol as in existing specification Proposal 9: For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, it is agreed as baseline that UE behaviors are the same as in symbol configured as downlink in TDD-UL-DL-ConfigCommon. Proposal 10: Study the benefit and specification impacts to support separate configurations for PUCCH transmissions in SBFD symbols and non-SBFD symbols. Proposal 11: FFS whether enhancement on PDCCH resource allocation in SBFD symbols and non-SBFD symbols is needed. Proposal 12: The invalid SPS PDSCH or CG PUSCH occasions in SBFD symbols can be ignored. Proposal 13: Study frequency hopping for PUSCH due to different available frequency resources in SBFD symbols and non-SBFD symbols for PUSCH. Proposal 14: Study the benefit and specification impacts to support separate SRS resource configurations for SBFD symbols and non-SBFD symbols. Proposal 15: Study the benefit of separate CSI report configurations for SBFD symbols and non-SBFD symbols. Proposal 16: Study how to define the available slot for TBoMS in SBFD operation. Proposal 17: Study whether enhancement is needed for multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols. Proposal 18: For PUCCH repetitions or PUSCH repetitions with available slot counting, study how to define the available slot in SBFD operation. Proposal 19: For PDSCH repetitions or PUSCH repetitions without available slot counting, omit the transmission in conflict slots. Proposal 20: Study assignment of fractional RBGs at subband boundaries for PDSCH and PUSCH with RA type 0. Proposal 21: Study non-contiguous FDRA across DL subbands for PDSCH with RA type 1. Proposal 22: FFS whether CSI-RS sequence generation enhancements considering non-contiguous DL subbands can provide performance gain. Proposal 23: Study CSI report for fractional CSI report subband at DL subband boundaries. Proposal 24: Study defining fractional PRGs at subband boundaries for PDSCH PRB bundling. Proposal 25: Study reinterpretation of wideband PRG assuming the same precoder is applied to the allocated resource in each DL subband if non-contiguous PDSCH RA type 1 is supported in SBFD symbols. Proposal 26: SBFD operation in SSB symbols is not supported. Proposal 27: For SBFD aware UEs, study the following collision cases: - Collision between dynamic UL transmissions and dynamic DL receptions during repetition of UL transmission or DL reception - Collision between dynamic DL receptions and configured UL transmission - Collision between dynamic UL transmission and configured DL receptions - Collision between configured UL transmission and configured DL receptions Proposal 28: For SBFD aware UE, study the following kinds of collision between transmissions/repetitions and resources: - Collision between dynamic transmission/repetitions except the first repetition and the subband/guardband - Collision between configured transmissions/repetitions and subband/guardband - Collision between PRACH and DL subband/guardband Proposal 29: For SBFD aware UEs, study the order of collision handling of different collision types. Proposal 30: Study potential enhancements for initial access in UL subband. Proposal 31: Study NTA,offset=0 for SBFD symbols and NTA,offset>0 for non-SBFD symbols for SBFD aware UEs. Proposal 32: Motivation and potential benefits for enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency need more discussions. |  |  |
| R1-2211233 | Discussion on subband non-overlapping full duplex | Spreadtrum Communications |
| - In co-channel co-existence case of legacy UE/gNB and SBFD, there is no impact to the legacy gNB, but impact to legacy UE. CLI mitigation scheme for aggressor SBFD UEs should be studied. - Both for R18 duplex operation enhancement and R17 RedCap, to achieve good co-existence performance with legacy network/UE, the bandwidth part location needs to be carefully designed. - DL resource fragmentation caused by NR duplex operation may not be friendly to legacy UEs from UE power consumption perspective. - In addition to Alt 4, Alt 2 can be further studied for SBFD operation. - UE does not aware of any SBFD information on time and frequency domain. - Regarding the informing method of time locations of subbands for SBFD operation, new signalling is a proper choice. - The slot configuration period of tdd-UL-DL-ConfigurationCommon can be used as the period of SBFD time domain location, including slot configuration period in pattern1 and pattern2 (if any). - Both of Cell-common and UE-specific subband time location indication can be study further. - At least slot level can be study for SBFD time location, including continuous and discontinuous slots in the period. - Regarding the informing method of frequency locations of subbands for SBFD operation, the cell-common starting CRB and length of continuous CRB number for UL subband can be the baseline. - The cell-common explicit indication of frequency location of DL subband(s) are the baseline for study, and guard bands locations are not included in any of DL and UL subbands. - Support the latest version of Proposal 1-18 for link direction of UL subband in DL symbols. - When UL subband located at the middle part of the carrier, the final frequency pattern cannot convert to {U(U+U)-D, D-U(U+U)} in symbols configured as flexible in TDD-UL-DL-ConfigCommon with UL subband configured by semi-static configuration. - Flexible subband is not preferable for flexible symbols. - For CSI-RS, UE-to-UE CLI-RSSI resource and PDSCH resource set frequency domain resource allocation across non-contiguous DL subbands, the following options can be further studied - Option 1: One contiguous frequency domain resource allocation for CSI-RS/UE-to-UE CLI-RSSI resource/PDSCH which the resources overlapping with UL subband or guardband are blank out. - Option 2: There are two separate frequency domain resource allocations for a CSI-RS/UE-to-UE CLI-RSSI resource/PDSCH if there are two DL subbands. - Support Proposal 1-12b without RBG for CORESET Study the impact and benefits of potential enhancements to resource allocation granularity in frequency-domain for SBFD operation, including at least the following: - RBG for PDSCH/PUSCH FDRA - CSI reporting subband - PRG of PDSCH - For the enhancements to RBG/PRG/CSI subband resource allocation granularity in frequency-domain for SBFD operation, the following options can be further studied. - Option 1: One implementation method is the resource group in RBG/PRG/CSI subband overlapping with UL subband/DL subband cannot be indicated for PUSCH/PDSCH transmission or CSI-RS within SBFD symbols. - Option 2: legacy contiguous frequency resources for RBG/PRG/CSI subband are unchanged, with the resources overlapping with different link direction subband or guardband are blank out. - Option 3: frequency resources for RBG/PRG/CSI subband are changed, with only the valid resources non-overlapping with different link direction subband or guardband. - Support proposal 1-13c for further study every cases. - Scheduled PUSCH/PDSCH/PUCCH without repetition do not need to study the impact when crossing SBFD and non-SBFD symbols, it can be guaranteed by scheduling. - PDCCH does not need to study the impact when crossing SBFD and non-SBFD symbols, it can be guaranteed by PDCCH monitoring occasion and CORESET configuration. - Victim UE measures CLI within DL subband for UE-to-UE inter-subband CLI handling. - Subband-level information can be considered for gNB-to-gNB’s information sharing. - Victim gNB measures CLI within UL subband for gNB-to-gNB inter-subband CLI handling. - The introduction of SBFD operation should study co-existence performance with the legacy NR operation including RedCap deployment, which allow network to configure SBFD and RedCap BWP in a legacy NR carrier simultaneously, meanwhile minimizing both UL resource fragmentation and DL resource fragmentation. |  |  |
| R1-2211362 | Discussion on subband non-overlapping full duplex | xiaomi |
| Observation 1: TDD periodicity provided by tdd-UL-DL-ConfigurationCommon cannot be directly reused as the period for configuring SBFD subband tome domain location. Observation 2: Before deciding whether dynamic indication is supported for SBFD subband time location, we need to firstly clarify UE behaviour in SBFD slot. Observation 3: Before deciding whether dynamic indication is supported for SBFD subband frequency location, we need to firstly clarify UE behaviour in SBFD slot. Proposal 1: Bitmap based method can be used to indicate SBFD time location within a time period. - FFS: the time unit of bitmap Proposal 2: UE-dedicated RRC signalling is slightly preferred to configure SBFD subband time location. Proposal 3: Send LS to RAN4 and consult RAN4 how guard band between DL subband and UL subband is determined. Proposal 4: Reuse the same mechanism of configuring BWP to configure the frequency locations of subband for SBFD operation, i.e. the starting position and length are indicated. Proposal 5: UE-dedicated RRC signalling is slightly preferred to configure SBFD subband frequency location. Proposal 6: For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, further study the following two options: - Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol - Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol Proposal 7: BWP-based SBFD operation is not supported in Rel-18 duplex enhancement. Proposal 8: Half duplex CA based SBFD operation is not supported in Rel-18 duplex enhancement. Proposal 9: Further study how to configure or determine the guard period between DL region and UL subband. Proposal 10: If UL subband is configured via RRC signalling, the bit length of FDRA field in a DCI scheduling uplink on the UL subband is determined by the active UL BWP. Proposal 11: The following principles should be considered for handling UL/DL collision on the SBFD slots: - SBFD UE doesn’t expect collision between dynamic DL and dynamic UL - SBFD UE doesn’t expect collision between semi-static DL and semi-static UL - Dynamic DL/UL has higher priority when it conflicts with semi-static UL/DL - Broadcast DL has higher priority when it conflicts with UL on the UL subband Proposal 12: For subband non-overlapping full duplex, it cannot be applied to UL symbols. |  |  |
| R1-2211398 | On SBFD in NR systems | Intel Corporation |
| Observation 1: SBFD operation with DL subband in a legacy UL symbol impacts legacy gNB/UE, which leads to UL resource fragmentation, may affect Msg 4 PUCCH transmission and PRACH transmission, degrades received SINR of UL reception due to co-channel gNB-to-gNB CLI interference, and may decrease frequency diversity gain for typical configurations. Observation 2: In a semi-static flexible symbol, the remaining RBs other than the RBs for UL subband (and guard band, if any) in the symbol can be considered as flexible resource, which may be used for UL or DL per gNB scheduling, to enable dynamic switch between SBFD symbol and full DL/UL symbol. No use case for flexible resource in a semi-static DL symbol is identified. Observation 3: Dynamic time location switch for subband can be achieved by both explicit and implicit indication. - Explicit indication using dedicated signaling for symbol type indication can help proper operation for both dynamic and configured channel/signal. - Implicit indication based on DL assignment or UL grant for dynamic scheduling may not support proper operation for configured channel/signal. Observation 4: Rel-16 CLI measurement mechanism supports CLI measurement in UL subband at least for some cases with aligned centre frequency for DL/UL BWP. No clear motivation is identified to enhance Rel-16 CLI mechanism for CLI measurement in UL subband outside of active DL BWP. Observation 5: Separate CLI measurement on PRBs in the same DL subband is already supported by Rel-16 CLI-RSSI measurement. Proposal 1: Deprioritize SBFD in a UL symbol in which at least one legacy UE transmits UL, or in a UL symbol which is semi-statically configured as UL for at least one legacy UE. - Note: Here, a “legacy UE” refers to a UE that does not support SBFD operation. Proposal 2: Support SBFD operation with UL subband in DL and flexible symbols configured by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated. - FFS whether support SBFD operation with UL subband in a cell-specific flexible symbol & UE-specific DL symbol for a SBFD-aware UE. Proposal 3: Further study the necessity of flexible subband and whether/how to define the flexible subband in a semi-static flexible symbol. Proposal 4: Further study dynamic subband location, with dynamic time location only as baseline. Proposal 5: For dynamic time location, further study following cases, - Case A: A semi-static SBFD symbol in legacy DL symbol or flexible symbol dynamically switches to a full DL symbol - Case B: A semi-static full DL symbol dynamically switches to a SBFD symbol - Case C: A semi-static SBFD symbol in legacy flexible symbol dynamically switches to a full UL symbol - FFS Case D: A semi-static SBFD symbol in legacy flexible symbol dynamically switches to a full flexible symbol - Case E: A semi-static flexible symbol dynamically switches to a SBFD symbol Proposal 6: Study methods for indication of guard bands between DL and UL subbands to a UE and related UE behaviour. Proposal 7: DL/UL subband is configured with reference to a carrier. - A UL subband can partially overlap with UL BWP. - UE does not expect to be scheduled/configured for UL transmission in a PRB in a UL subband, if the PRB is outside of active UL BWP. Proposal 8: Further study the necessity of explicit configuration for other subband types and guard band, depending on study on SBFD operation/configuration in legacy flexible symbol. Proposal 9: Further study methods for explicit indication using dedicated signaling for symbol type indication and implicit indication based on DL assignment or UL grant for dynamic scheduling. Mechanism to avoid DL/UL collision caused by miss-detected the indication should be considered. Proposal 10: Further study BWP-based and CA-based BWP for SBFD operation Alt 4: - Scheme 2: BWP-based SBFD - Fast inter-BWP or intra-BWP (between DL/UL BWP with same BWP index) switching with or without aligned centre frequency of DL/UL BWP with same BWP index. - Enhancements to avoid interruption/dropping of signals due to BWP switching and handling configurations common across different UEs, e.g., PRACH or CSS. - Scheme 3: CA-based SBFD - Overlapped carriers for CA - Cross-carrier transmission, transmission over multiple carriers and enhancement to resolve collision between carriers. - Focus on scenarios wherein multiple component carriers may be configured within a significantly wide frequency band. Proposal 11: Study SBFD operation for RRC idle/inactive mode. - Study Msg 1 PRACH/PUSCH in UL subband with consideration of challenge for UE-to-UE CLI handling caused by uncertainty of UE transmitting Msg 1 PRACH/PUSCH. - Study PUSCH/PUCCH in UL subband scheduled by gNB, e.g., Msg 3 PUSCH and PUCCH for Msg 4 PDSCH. Proposal 12: For frequency domain confliction of channels/signals and valid subband (not overridden for dynamic time location of subband) in different directions, whether resource collision is allowed depends on detailed enhancement of resource allocation and L1 procedure in section 5. - No matter resource collision is allowed or not, UE can only transmit within valid UL subband or receive within valid DL subband. Proposal 13: Do not support time domain confliction of dynamic DL channel/signals and dynamic UL channel/signals in different directions in the same SBFD symbol for SBFD-aware UE. Proposal 14: The following cases may be considered as valid case with time domain confliction of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE - semi-statically configured DL channel/signals and semi-statically configured UL channel/ signals in different directions, - if at least one configured DL/UL is cell-specific signal/channel - if at least one configured DL/UL is with repetition or multi-PDSCH/PUSCH repetition - if configured DL is PDCCH - if DL and UL is with different priorities (LP and HP) - semi-statically configured channel/signal and dynamically scheduled channel/signal signals in different directions Proposal 15: Study potential enhancements for resource allocation and L1 procedures to support efficient SBFD operation. - The resource allocation includes at least resource for PDSCH/PDCCH/PUSCH/PUCCH/CSI-RS/SRS, and PRACH if SBFD operation for initial access is supported. L1 procedure includes at least CSI measurement /report, power control, DMRS bundling, UL timing and DL/UL switching. - Study non-contiguous FDRA for RA type 1 across DL subbands for PDSCH. - Study PDSCH/PUSCH FDRA, CSI reporting subband and CORESET for PDCCH to reduce impact of non-aligned boundaries of DL/UL subband and frequency resource granularity - Study resource configuration/determination for PUCCH/PUSCH/PDSCH/PDCCH/RS with or without repetition/multi-PDSCH/PUSCH scheduling/TBoMs, with or without frequency hopping, in SBFD and non-SBFD symbols. - Study UL timing and handling of back-to-back DL/UL signals/channels without sufficient switching time or overlapped UL transmissions. Proposal 16: Study single CLI resource configuration/determination over non-contiguous DL subbands or multiple CLI resources in different DL subbands associated with single CLI report. Proposal 17: Study exchange of intended subband configurations for SBFD operation across gNBs for coordinated scheduling for time/frequency resources between gNBs for gNB-to-gNB or UE-to-UE co-channel CLI handling. |  |  |
| R1-2211485 | Discussion on subband non-overlapping full duplex | OPPO |
| Observation 1: The legacy UE can run with half-duplex mode in SBFD symbols with both time-domain transparency and frequency-domain transparency to UL subband allocation at gNB. Observation 2: The SBFD subband allocation does not have to be the same over adjacent carriers, but the adjacent edge subbands are desired to have the same transmission direction. Proposal 1: Carrier-specific SBFD configuration is preferred to ensure that only one continuous UL subband is configured within one carrier. Proposal 2: UL subband and guard band is indicated explicitly. - Within UL subband, only UL transmission is allowed. - The other subband except from UL subband and guard band, transmission direction depends on TDD configuration and scheduling. Proposal 3: UL subband can be explicitly configured in DL symbol and flexible symbol. Proposal 4: The periodicity for time resource configuration of SBFD should align with periodicity of TDD configuration. Proposal 5: SBFD operation in SSB symbols is not supported. Proposal 6: Only time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD should be specified. - When SBFD operation is supported, dynamic scheduling is prioritized when UL and DL operation collide. Proposal 7: The amount of RBs used to determine the TB size does not count the RBs overlapping with UL subband and the guard bands (if any) around the UL subband. Proposal 8: Considering that different slot types in multiple repetition slots, PUCCH/PUSCH/PDSCH repetition enhancement needs to be studied. |  |  |
| R1-2211559 | Discussion on subband non-overlapping full duplex enhancements | ETRI |
| Observation 1. In case of UE with high capabilities, the channel coherence may be maintained longer than either of the allocated SBFD resource or TDD resource. Proposal 1. Multiple SBFD subbands with different D-U directions can be explicitly configured on a given time instance from a UE perspective. Proposal 2. Consider UE-specific RRC signaling for time location and frequency location of the SBFD subbands as the baseline. - A RB set corresponding to a UL subband is represented by a set of CRBs with reference to Point A. - FFS: Cell-specific RRC signaling for time location and frequency location of the SBFD subbands (subject to SBFD support during initial access) Proposal 3. RAN1 to study the detailed signaling methods for the UL subband location indication for the SBFD-capable UEs considering the following aspects: - Interaction with slot format configuration - Tradeoff between signaling overhead and UL/DL resource partitioning flexibility - Need of explicit indication of RB set(s) for DL or flexible subband(s) - SFI enhancement for dynamic SBFD symbol allocation for a given time instance. Proposal 4. Capture the following use cases of DL-UL direction change for an SBFD aware UE in SBFD symbols in the TR: - Use case #1: Ease of planning on cell-specific signals / channels - Use case #2: Latency reduction for UL/DL beam indication, channel measurement, CSI feedback, etc. - Use case #3: Ease of co-existence with legacy UEs Proposal 5. Study semi-static and/or dynamic DL-UL direction change of an SBFD aware UE in SBFD symbols. Proposal 6. Consider the following options for SBFD aware UEs. - Option A (baseline): UE does not receive a DL transmission if the DL transmission overlaps, even partially, with a UL subband (and potentially corresponding guard band(s)), and UE does not transmit a UL transmission if the UL transmission overlaps, even partially, with a DL subband (and potentially guard band(s)). - Option B: If a DL transmission partially overlaps with a UL subband (and potentially corresponding guard band(s)), the UE can receive the non-overlapped DL transmission. - Option C: If a UL transmission partially overlaps with a DL subband (and potentially corresponding guard band(s)), the UE can transmit the non-overlapped UL transmission. Proposal 7. Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols. Proposal 8. RAN1 to clarify that for a CSI report with a frequency resource configuration, only the frequency resources within the DL subband are considered as the valid frequency resources for that CSI report where the corresponding CSI reference resource contains SBFD symbols. |  |  |
| R1-2211610 | Considerations on Subband Full Duplex TDD Operations | Sony |
| Observation 1: Configuration of SBFD by only explicitly indicating the UL subband locations assumes that an SBFD OFDM symbol consists of only UL subband and DL subband(s), which may not be true in all cases since there may be Flexible subband and/or Guard subband. Observation 2: For the {DUD}, subband configuration, FDRA Type 1 cannot schedule a PDSCH to occupy both DL subbands since it only allocates contiguous sets of RBs for a PDSCH. Observation 3: For the {DUD} subband configuration, FDRA Type 0 can be used to schedule a PDSCH to occupy RBs in both DL subbands. However, since RBG is the unit of allocation, FDRA Type 0 has a coarser frequency granularity compared to FDRA Type 1 and if the finer RBG granularity (i.e. RBG Configuration#1) is used, then FDRA Type 1 consumes more DCI bits compared to FDRA Type 0. Observation 4: FDRA Type 0 is not supported in Fallback DCI (Format 1_0). Observation 5: The Mirror Image FDRA is applicable regardless of whether the subband configuration is transparent or known to the UE. Observation 6: If the subband configuration is semi-statically signalled to the UE, a smaller FDRA bit size can be used in the DL Grant since the FDRA needs only to address the number of RBs in one of the DL subbands rather than the entire BWP. Observation 7: Frequency hopping in UL subband with small bandwidth may not be feasible if the PUSCH/PUCCH consumes most of the UL subband or may not provide any gain. Observation 8: Whether frequency hopping is enabled or disabled in UL subbands depends on the bandwidth size of the UL subbands. Observation 9: Inter subband CLI is non-uniform across a subband, where it is stronger for RBs in a subband that are closer to an adjacent subband compared to RBs that are further away from the adjacent subband. Observation 10: Since in SBFD, inter subband CLI is non-uniform across the victim subband, the CLI measurement reports should take this aspect into account. Observation 11: DL & UL messages may collide in SBFD slots and a mechanism is required to resolve such collisions. Proposal 1: In addition to UL subband and DL subband, study also Flexible subband and Guard subband in SBFD operations. Proposal 2: Support dynamic configuration of SBFD locations. Proposal 3: Use reserved indices in the SFI, i.e., indices 56-254, to indicate SBFD formats. Proposal 4: Consider using a Mirror Image FDRA, where the DL Grant indicates a 1st set of RBs and a 2nd set of RBs is determined by reflecting the 1st set of RBs across the middle of the BWP. The scheduled PDSCH occupies the 1st set and the 2nd sets of RBs. The Mirror Image FDRA can be enabled or disabled in the DL Grant. Proposal 5: For a UE configured with frequency hopping for a PUSCH/PUCCH, the UE performs frequency hopping in SBFD slots if the bandwidth size of the UL subband is larger than a configured threshold and if there is resource available for the second hop, otherwise the UE disables frequency hopping in the SBFD slot. Proposal 6: Support non-uniform MCS in a PDSCH and PUSCH so that RBs of a PDSCH/PUSCH that are closer to an adjacent subband uses more robust MCS compared to RBs that are further away from an adjacent subband. Proposal 7: Support non-uniform power control in a PUSCH so that RBs closer to an adjacent subband are transmitted with lower power compared to RBs further away from the adjacent subband. Proposal 8: Support finer frequency granularity for CLI measurement and reporting, by dividing the BWP or the victim subband into smaller frequency blocks, where CLI measurement and reporting are performed on each frequency block. Proposal 9: CLI measurements are preformed and reported at the physical layer. Proposal 10: If DL & UL messages collide in an SBFD slot, the UE prioritises the dynamically scheduled transmission over the semi-statically configured resources (e.g. PDCCH Search Space, SPS, CG-PUSCH). |  |  |
| R1-2211680 | Discussion on subband non-overlapping full duplex | CMCC |
| Proposal 1: For inter-gNB inter-subband CLI measurement, the following two methods can be further studied: - Method#1: victim gNB measures leakage power from aggressor gNB within UL subband; - Method#2: victim gNB measures RSRP of aggressor gNB’s RS within DL subband. Proposal 2: For inter-gNB inter-subband CLI handling, inter-gNB coordination in time-domain, frequency-domain, spatial-domain, and power domain should be supported. - Backhaul signalling enhancement is needed to support inter-vendor cooperation. Proposal 3: For inter-UE inter-subband CLI measurement, the following two methods can be further studied: - Method#1: victim UE measures RSSI/SINR within DL subband; - Method#2: victim UE measures RSRP of aggressor UE’s CLI-SRS within UL subband. Proposal 4: Subband based inter-UE CLI measurement and report can be supported for SBFD which both uniform and non-uniform CLI subband configuration can be further studied. Proposal 5: SBFD scheme with more than one configured DL and UL BWP pair can be further studied taking into account of the following aspects: - reducing BWP switching delay; - UE supporting multiple active BWPs simultaneously. Proposal 6: For a SBFD aware UE, SBFD operation can be configured in a symbol configured as DL or flexible in TDD-UL-DL-ConfigCommon. Proposal 7: Two steps signalling can be considered for indication of time location of subband: - Step 1: Cell-specific signalling to indicate SBFD slots/symbols which can override the DL/flexible slots/symbols in cell-specific TDD UL/DL configuration. - Step 2: UE-specific RRC signalling/scheduling DCI/SFI to indicate the DL or UL link direction for SBFD slots/symbols. Proposal 8: The SBFD subband time locations indication in Step 1 is indicated within a period equals to the TDD periodicity configured in TDD-UL-DL-ConfigCommon. Proposal 9: The UL subband is configured with the starting PRB location and bandwidth which reference to Point A. Proposal 10: For SBFD aware UE’s UL transmission, the UL subband is defined as available UL transmission resources within UE’s active UL BWP in SBFD slots/symbols. Proposal 11: For SBFD aware UE’s DL reception, the UL subband is defined as DL resource allocation enhancement information or a common rate matching pattern for all DL signals/channels within UE’s active DL BWP in SBFD slots/symbols without the explicit DL subband frequency location indication. Proposal 12: For SBFD operation Alt 4, support Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol. Proposal 13: Support non-continuous PRB-to-CRB mapping of DL BWP on SBFD slots. Proposal 14: For PDSCH slot aggregation, the following two options can be further studied: - Opt 1: PDSCH slot aggregation is only across SBFD slots or non-SBFD slots; - Opt 2: PDSCH slot aggregation can across SBFD slots and non-SBFD slots with some automatic adjustment on FDRA between SBFD slots and non-SBFD slots. Proposal 15: For SBFD symbols and non-SBFD symbols, the following parameters can be separately configured: - PUSCH/PUCCH frequency hopping offsets; - PUCCH resource sets; - SRS resources. Proposal 16: It is preferred to not allow the SBFD symbols overlap with the symbols that are indicated for reception of SS/PBCH blocks. Proposal 17: ROs for contention-free RACH can be configured in UL subband in SBFD symbols. ROs for contention-based RACH cannot be configured in UL subband in SBFD symbols. Proposal 18: The following case of time domain conflict of UE’s UL and DL operation in the same SBFD symbol can be considered for SBFD aware UE: - Collision between periodic CSI-RS/CORESET and periodic SRS/CG PUSCH. |  |  |
| R1-2211737 | Discussion on subband non-overlapping full duplex operations | InterDigital, Inc. |
| Observation 1. Mixed D/U regions informed to a UE per symbol/slot in a cell can be used for a subband-wise UL transmission or DL reception, which results in UL coverage/capacity enhancement while achieving parallel DL transmissions over non-overlapped RBs. Observation 2. In cases with dynamic RB gap indication, the DL subbands can be determined based on the indicated UL subbands and the RB gap. Observation 3. Using SBFD schemes with more than one configured DL and UL BWP pair, where each BWP pair is associated with a SBFD configuration could result in potential benefits in scheduling flexibility, optimal resource allocations, lower latency, and so forth. Observation 4. Considering UL subbands in SSB symbols could result in increased latency and complexity at the UE-side and lower flexibility at the NW-side for the purpose of initial access procedure. Observation 5. In case the UE is configured with physical channel priorities, the UE could use them to handle collisions in SBFD symbols. Observation 6. DL throughput performance suffers considerably at high MCSs and approaches to almost zero when there is any degree of intra-subband CLI overlap with DL signal. Observation 7. Inter-carrier interference, resulting from time advance on inter-subband CLI may impact DL throughput performance significantly, especially at high MCS indices, when there is no adjacent inter-subband distance between the DL signal and the CLI, i.e., 0-RB gap. Observation 8. At high MCS indices, DL throughput performance recovers to 90% of the maximum throughput when the inter-subband distance between the DL signal and the CLI signal is at least 2-RB in this example scenario. Observation 9. In addition to the frequency gap between the DL signal and the CLI (UL) signal, the amount of frequency resources (PRBs) allocated to the DL signal relative to that for the CLI signal also impacts the normalized throughput performance significantly. Observation 10. Restricting DL subband transmissions on slots that correspond to UL slots in legacy TDD can improve uplink performance but negatively impacts downlink performance. Observation 11. The static/fixed subband partitioning, e.g., [DUD] = [40 20 40] RB split all the time, results in worse performance for SBFD compared with legacy TDD in downlink. Thus, flexible/versatile subband partitioning and its dynamic indication mechanisms should be considered to cope with varying traffic/channel conditions. Observation 12. Utilizing SBFD option 2 (total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD) improves SBFD performance. Observation 13. In the SBFD scenario, a UL transmission over a subband causes significant UE-to-UE CLI leakage on the adjacent DL subband depending on a frequency gap between the UL RBs and DL RBs of each subband. Observation 14. As a part of gNB implementation, the gNB may apply a downlink power backoff on some SBFD slots or symbols to deal with self-interference caused by the FD operation, which can impact to UE behaviours depending on the amount of power backoff. Observation 15. Inter-subband UE-to-UE CLI measurement in SBFD slots based on measuring over configured RB resources and averaging may result in down-estimation, as the subband-edge RBs experience higher CLI compared to RBs in the middle of the subband. Observation 16. Since a general CSI/beam reporting in NR is not based on dynamic CLI-related information, a victim UE may unpredictably experience DL performance degradation if a UE-to-UE CLI occurs especially when an aggressor UE is served by a different serving gNB/TRP. Observation 17. The issues in UL/DL timing alignment (between UL/DL SBs) in SBFD slots could result in inter-slot interference and dropping of respective slots, especially for back-to-back scheduling cases between DL and UL. Observation 18. In case the scheduled frequency resources for an UL transmission are located outside of the boundaries of the SBFD UL subbands, the UE may need further configurations to re-interpret the frequency resources to be mapped within the SBFD UL boundaries. Proposal 1. Study mechanisms on how to inform UE of mixed D/U regions per symbol/slot as an enhancement of SFI to achieve subband non-overlapping FD (SBFD). - Consider the cases where multiple BWPs/subbands are configured for SBFD within the same carrier. Proposal 2. Consider implicit indication of the subband frequency locations for the DL subbands based on the indicated UL subbands and the RB gap between the UL and DL subbands. Proposal 3. Study the aspects of BWP switching for SBFD schemes with more than one configured DL and UL BWP pair, where each BWP pair is associated with a SBFD configuration. Proposal 4. At least for the purpose of initial access procedure, application of SBFD operation in SSB symbols needs to be avoided. Proposal 5. Consider configuration of physical channel priorities for handling the collisions in SBFD symbols, including uplink, downlink, control channels, data channels, dynamic grant, configured grant, and so forth. Proposal 6. Study performance of applying a frequency gap or guard RBs for a UL transmission in an SBFD framework for interference mitigation with regards to adjacent DL subbands. Proposal 7. Analysis on various downlink performance degradation aspects due to the SBFD operations compared with legacy TDD systems should also be an important part of the NR-Duplex study. Proposal 8. To overcome the degraded downlink performance due to the static/fixed subband partitioning, flexible/versatile subband partitioning and its dynamic indication mechanisms should be further discussed to cope with varying traffic/channel conditions. Proposal 9. Study dynamic UL power control mechanism based on some dynamic factors such as the frequency gap, beam/spatial-domain parameter, or a priority indication on the UL, to mitigate the effects of the CLI dynamically. Proposal 10. Consider mechanisms to apply measurement skipping on some SBFD slots/symbols and power adjustment in deriving a CSI, depending on a level of dynamic power management occurred in the SBFD scenario. Proposal 11. Study measurement resources and reporting configurations for subband-edge CLI measurement. Proposal 12. Study a conditional CLI handling behavior based on monitoring the beams at the victim UE side, where the condition can at least include a case when the victim UE detects a PDSCH reception failure, which initiates a subband-wise CLI measurement/reporting for a subband switching to avoid the CLI. Proposal 13. Study UL/DL timing alignment issues in subband non-overlapping full duplex systems. Proposal 14. Study the feasibility and potential benefits of providing supplementary configurations to the UE to be used in case PUCCH/PUSCH frequency resources are mapped (partially or totally) to outside of the SBFD UL boundaries. |  |  |
| R1-2211748 | Discussion on subband non-overlapping full duplex | NEC |
| Observation-1: Full duplex operation can be currently achieved in NR by providing different tdd-UL-DL-ConfigurationDedicated or SFI to different UEs in a cell, however, higher DL/UL performance gains are expected if UEs are indicated SBFD time/frequency resources Observation-2: Among different options considered for SBFD operation using multiple antenna panels, Option-1 most efficiently utilizes the available antenna elements. Observation-3: For semi-static physical channels following conflicts may occur during SBFD time/frequency resources: - UE may try to receive DL receptions (e.g. CSI-RS) within UL sub-bands resulting in incorrect channel estimation or reduced DL performance - UE may try to perform UL transmissions (e.g. SRS) within DL sub-bands resulting in increased interference to nearby UE receiving DL Observation-4: Following interference scenarios are possible for sub-band non-overlapping full duplex operation: - CLI between UEs of same cell i.e. UL transmission of one UE interfering with the DL reception of nearby UE - gNB experiencing interference in UL reception due to its DL transmission in adjacent sub-band - gNB experiencing interference in UL reception due to DL transmission from nearby gNB Proposal 1: - The SBFD aware UE can be dynamically scheduled with DL reception within the UL subband in the SBFD symbols Proposal 2: - The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband Proposal 3: - UL subband frequency resources are configured to UE per BWP - FFS how to define rules in RAN1 specification to ensure single UL subband per TDD carrier Proposal 4: - UL subband time occasions are configured in either DL or flexible slots/symbols of tdd-UL-DL-ConfigurationCommon - FFS UE behavior for UL subband occasions configured in flexible slots/symbols on receiving SFI/ tdd-UL-DL-ConfigurationDedicated Proposal 5: - Study SBFD time domain configuration to avoid SSB occasions while maximizing available SBFD occasions Proposal 6: - Support multiple BWP pairs configured to UE (with aligned center frequencies for a DL and UL BWP pair) where each BWP pair is associated with a different TDD/subband configuration Proposal 7: - Support frequency guard band between UL and DL sub-bands for interference mitigation during SBFD - Study on how to apply frequency guard band adaptively by UE and gNB only on SBFD occurrences Proposal 8: - Study application of multiple antenna panels for SBFD operation at gNB, where different antenna panels are used for UL reception and DL transmission by gNB Proposal 9: - For SBFD slot/symbols, study simultaneous Tx and Rx using different sets of antenna elements belonging to single antenna panel Proposal 10: - Study the impact on DL channel estimation, codebook aspects, CSI-RS procedures to support SBFD operation using multiple antenna panels Proposal 11: - Study how to improve UL decoding performance during SBFD slots/symbols for SBFD operation using multiple antenna panels Proposal 12: - Study support of 2-set of frequency resource configurations for semi-static UL/DL channels/signals where the first set is used during UL-only/DL-only slots/symbols and the second set is used for SBFD slots/symbols Proposal 13: - If a single BWP is used to enable subband non-overlapping full duplex operation, study the impact for multi-slot PUSCH transmission scheduled by single DCI cross symbols with different duplex type, e.g., UL symbol only and SBFD symbol with UL sub-band. Proposal 14: - Multi-slot PUSCH/PDSCH spanning SBFD and UL-only/DL-only slots can be transmitted on both SBFD slots and UL-only/DL-only slots - FFS whether additional network control is required to disable/enable transmission on SBFD slots Proposal 15: - Study how to support frequency hopping and frequency interleaving procedures for UL and DL transmissions performed during SBFD slots/symbols - Study some enhancement for Type 0 FDRA for PDSCH/PUSCH for SBFD slots/symbols Proposal 16: - In Rel-18, for sub-band non-overlapping full duplex at the gNB side, study interference management for - inter-UE CLI in a cell - self-interference management for gNB - inter-gNB CLI Proposal 17: - In Rel-18, for sub-band non-overlapping full duplex at the gNB side, traffic characteristics served by gNBs should be taken into the interference management. Proposal 18: - Study exchange of SBFD configuration between gNBs for inter-gNB CLI mitigation Proposal 19: - Study the CSI report enhancement for SBFD operation and different type of CLI interference |  |  |
| R1-2211779 | Subband non-overlapping full duplex | Lenovo |
| Proposal 1: PUSCH/PUCCH/CG-PUSCH/RACH/SRS configurations for operation in a full duplex UL subband can be indicated by an associated UL BWP ID. Separate CG PUSCH/PUCCH/RACH/SRS configuration(s) can be provided for the full duplex UL subband. Proposal 2: Study PUSCH repetition type A/B and TBoMS across SBFD and non-SBFD symbols/slots. Proposal 3: Study PUCCH repetition across SBFD and non-SBFD symbols/slots. Observation 1: Radio resources may not be efficiently used with a semi-static full duplex UL subband. Proposal 4: Study UL subband bandwidth adaptation for flexible SBFD operation. Proposal 5: For PDSCH reception with downlink type 0 resource allocation or with non-interleaved downlink type 1 resource allocation, a SBFD aware UE performs rate matching around an active UL subband. Proposal 6: For PDSCH reception with interleaved downlink type 1 resource allocation, a SBFD aware UE indexes VRBs and defines resource block bundles based on resource blocks not overlapping with an active UL subband. Proposal 7: Study enhanced CORESET frequency resource allocation for SBFD operation. Proposal 8: Study solutions such as non-available resources for CSI-RS or non-contiguous CSI-RS resource allocation to enable non-contiguous CSI-RS in frequency. Proposal 9: Study potential enhancements for UL/DL collision handling within SBFD symbols in time domain for use case such as when both UL and DL are configured by RRC message. Proposal 10: Support a non-contiguous CLI-RSSI measurement resource configuration in frequency for inter-subband CLI measurement. |  |  |
| R1-2211812 | Views on subband non-overlapping full duplex | Apple |
| Proposal 1: De-prioritize Option 3 and Option 4 for SBFD operation under Alt4. Proposal 2: The CSI-RS (or CLI-RSSI) measurement resource can be configured on separate DL sub-bands or even within UL sub-bands if victim UE (DL UE) is indicated to measure CLI (not UL UE). Proposal 3: Aggressor UE does not expect to measure CLI over a resource that is not within UL sub-band. Proposal 4: Potential aggressor UE, UEA, is indicated to measure SRS (transmitted by potential victim UE) before PUSCH transmission. The indication can be through DCI scheduling the PUSCH for aggressor UE. - One (or more) aperiodic SRS resource sets are tagged with a RRC parameter indicating CTS purpose - Once SRS request bit-field activates such SRS resource set, (potential) UEA performs SRS-RSRP and/or CLI-RSSI over the activated SRS resource(s) Proposal 5: Study feasibility of a mechanism to indicate SBFD-aware UE about cell duplex operation mode. ## Proposal 6: DL CLI indication, e.g., based on DL-PI, indicates which symbols were impacted by cross-link interference from aggressor UE(s). |  |  |
| R1-2211876 | Discussion on subband non-overlapping full duplex | Panasonic |
| Observation 1: It is not sufficient to operate SBFD only over legacy semi-static Flexible symbol/slot. Method to utilize legacy semi-static DL symbol/slot should be studied. Proposal 1: For SBFD-aware UE, time and frequency locations of subbands for SBFD operation is not applied to legacy flexible symbols. This means SBFD operation in flexible symbol is transparent to both SBFD-aware and legacy UE. Proposal 2: In legacy UL symbols, UL subband is not applicable. Proposal 3: UL subband are applicable only to legacy DL symbols. Proposal 4: In Option 1 for SBFD operation Alt 4, dynamic indication of SBFD symbol is necessary. In Option 2, dynamic indication of UL or DL direction in semi-static SBFD symbols is necessary. Both operations can be seen as flexible-symbol-like operation in UL subband in semi-static legacy DL symbols. Proposal 5: "Flexible-symbol-like operation in UL subband in semi-static legacy DL symbols" should be renamed to "flexible subband in semi-static legacy DL symbols". The subband pattern {DFD}{DF}{DF} should be supported. Proposal 6: Frequency location of subbands types other than UL subband (i.e., DL subband, guard band if supported) is also explicitly indicated. Proposal 7: The legacy semi-static configuration of RB set can be basically reused but the configuration should be sent over SIB. Proposal 8: Consider to introduce a new semi-static slot format for Rel-18 (and beyond) where the legacy semi-static DL symbol/slot can be re-configured as UL or Flexible symbol/slot. Proposal 9: Study potential enhancement on configured UL transmission to associate with either normal UL symbol or SBFD symbol. Proposal 10: For PUSCH with dynamic grant, the same FDRA is used between SBFD symbol and normal symbol. Proposal 11: Study how to configure timing advance offset for SBFD symbol and normal symbol Proposal 12: Study potential schemes for interference mitigation such as UL transmission power limitation, and exchange SBFD configurations. |  |  |
| R1-2211922 | Study on Subband non-overlapping full duplex | LG Electronics |
| 1. General Aspects of SBFD scheme SBFD schemes across BWP(s) within a carrier Propose 1: Clarify the meaning of center frequency of BWP for at least following cases - Case 1: BWP is used for both half duplex and SBFD operation including two types of frequency resource partitioning (i.e., {D U D}, {D U}) - Case 2: BWP is used for only SBFD operation including two types of frequency resource partitioning (i.e., {D U D}, {D U}) Proposal 2: For SBFD operation, allow the assumption that UE can expect to receive a configuration where the center frequency for a DL BWP is different than the center frequency for an UL BWP. Proposal 3: SB-FD operation can be allowed in the time resource configured as DL and/or Flexible. Proposal 4: Study whether TDD configuration mechanism should be updated for supporting SB-FD operation. If it is agreed that enhancement of TDD configuration for SB-FD operation is studied, following can be studied. - Which type of TDD configuration needs to be enhanced for supporting SB-FD operation Proposal 5: Study whether multiple type of TDD UL/DL pattern can be allowed for BWPs when SB-FD operation is adopted. Subband Location Indication Proposal 6: Study whether/how different length of available BW of a BWP depending on time resource for HD or SB-FD is supported. - UE behavior according to frequency resource of DL/UL for half duplex time resource and SBFD time resource (e.g., UL frequency hopping, PUCCH resource allocation in frequency domain, SRS resource configuration, UL/DL FDRA, CSI-RS resource, DL Measurement, CSI measurement and reporting, etc.) Proposal 7: For semi-static configuration of subband time locations for SBFD operation, following options can be studied - Option 1: Indicate each slot within a period where SB-FD operation is allowed - Option 2: Indicate start time resource and number of contiguous time resource within a period where SB-FD operation is allowed. Proposal 8: For a BWP / Carrier for SBFD operation, additional parameter for resource indicator value (RIV) is introduced (e.g., indication of a resource blocks which is non-contiguous resource, or indication of a resource blocks with narrower bandwidth, etc.). Proposal 9: For an SBFD aware UE configured with an UL subband in an SBFD symbol, option 1 (i.e., Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol) is the baseline for the study. In addition, fallback operation of an SBFD symbol (i.e., returning to symbol for the half duplex operation) is studied. UE collision handling Proposal 10: Study whether/how DL/UL collision rule is enhanced for efficient DL/UL operation at the SB-FD symbols SBFD operation for UE in RRC_IDLE/Inactive states Proposal 11: Study SBFD operation for UE in IDLE/Inactive state. 2. Interference handling Cross Link Interference Handling Observation 1. If the UE-to-UE CLI-RSSI measurement/report is taken non-contiguous measurement resource in frequency into account, the measured strength of CLI is averaged over the non-contiguous measurement resource. Hence, it is hard to determine which part of DL subband is suffered from intra-cell inter-UE CLI. Proposal 12. For UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency, the use cases and potential benefits should be justified. Proposal 13: For SB-FD specific UE-to-UE Cross Link Interference handling, followings can be studied. - Measurement gap for measurement inter-subband for UE-to-UE SRS-RSRP measurement/report for non-contiguous measurement resource - UE-to-UE SRS-RSRP measurement within active DL BWP with wide bandwidth - L1/L2 based CLI measurement and reporting for intra-cell UE2UE CLI management Self-Interference Cancellation/Mitigation Proposal 14: Study time boundary alignment between UL and DL within a slot for SB-FD. Following options can be studied. - Option 1: Symbol boundary alignment between DL signal/channel and UL signal/channel - Option 2: Slot boundary alignment without symbol boundary alignment between DL signal/channel and UL signal/channel Proposal 15: For self-Interference cancellation/mitigation method for SB-FD, followings can be studied. - Simultaneous beam management for DL transmission and UL reception - Simultaneous power control for DL transmission and UL reception - gNB implementation for Self-Interference cancellation/mitigation (e.g., antenna separation, RF level SI cancellation, baseband level SI cancellation, subband filtering, etc.) 3. MIMO operation Proposal 16: Study whether MIMO related configuration (e.g., antenna configuration, beam management, power control, CSI measurement/report, reference signal, etc.) can be differently applied according to duplex schemes (i.e., HD TDD and SB-FD) operated in gNB side. |  |  |
| R1-2211942 | Subband non-overlapping full duplex | Ericsson |
| Observation 1 To avoid excessive receiver desensitization, the total interference due to SBFD operation from all sources (residual self-interference, IMD due to Rx non-linearities, interference due to reciprocal mixing of phase noise, ADC quantization noise, etc.) needs to be maintained well below the noise/reference sensitivity level for the gNB receiver. Observation 2 Digital self-interference cancellation solutions are more feasible for lower power BS with small coverage than for high power BS with massive MIMO capabilities. Observation 3 For a low power BS such as the representative local area (LA) BS class, can likely operate in SBFD mode using existing hardware components without digital self-IC if antenna isolation of 80 dB is achieved. Observation 4 For a medium power BS such as the representative medium range (MR) BS class, in a single-carrier, single-sector deployment, interference mitigation solutions such as (1) hardware component upgrades to avoid spectrum regrowth caused by RX non-linearities, or (2) digital self-interference cancellation, or combination of (1) and (2), are needed even with 80 dB antenna isolation. Observation 5 For mitigating interference due to SBFD operation, the cost, complexity, energy consumption, and heat dissipation scales (1) linearly with the number of TX/RX chains for mitigation through hardware component upgrades, and (2) quadratically with the number of TX/RX chains for mitigation through digital self-interference cancellation. Observation 6 For a high-power BS, such as the representative wide area (WA) BS class, interference powers due to SBFD operation are far above what current typical WA BS hardware is designed for even with 80 dB antenna isolation, even in a single-carrier, single-sector deployment. Either of the SBFD related interference mitigation approaches discussed for the medium range (MR) BS class would result in excessive cost, complexity, energy consumption, and heat management. Observation 7 With inter-sector isolation of 60 dB, the receivers for WA and MR BS classes suffer from strong interference powers leaked from adjacent sectors far above the current BS operation specs. In some cases, BS receiver components may be permanently damaged. Observation 8 Installation of isolation enhancing material between sectors of one operator and/or between antennas of different operators with antennas at different heights on a tower/pole would face significant practical challenges, and does not appear feasible. Observation 9 The implementation complexity and difficulty of sharp analog filtering that is suitable separating DL and UL subbands with a few RBs of guard frequencies in a SBFD carrier depends strongly on the operating frequency at which such analog filtering takes place. Such analog filtering is more feasible with homodyne type receivers but becomes increasingly infeasible with the heterodyne or direct RF sampling type receivers. Observation 10 For high power BS (e.g., WA BS class), the feasibility of SBFD in a modern multi-carrier, multi-band, AAS architecture is highly questionable given the excessive requirements on (1) hardware component upgrades which translate to significant issues on product size, power consumption, and heat management, and (2) site solutions for mitigation of co-site interference. Alternative approaches can be considered for WA deployments that do not rely on simultaneous transmission and reception at the gNB, e.g., opportunistic TDD operation with equivalent amount of UL resources as SBFD. Observation 11 For low and medium power BS (e.g., LA and MR BS classes), assuming 80 dB antenna isolation is achieved, the feasibility of SBFD appears more likely at least for single-sector, single-carrier deployments for which there are lower demands on hardware component upgrades and mitigation of self-interference compared to WA BS class. Further study under realistic assumptions is needed in the area of multi-sector, multi-carrier, and multi-band deployments. Observation 12 Dynamic "conversion" of an SBFD symbol configured as 'F' by TDD-UL-DL-ConfigCommon amongst {D-U-D, UL-only, DL-only, and F-only} use is not expected to offer a performance advantage compared to dynamic TDD operation. Observation 13 For both legacy and new (Rel-18) UEs in IDLE mode, it is feasible that DL reception (i.e., during initial access) can occur in only one of the 'D' subbands within SBFD slots/symbols configured as D-U-D. By "feasible," it is meant that assuming a certainminimum channel bandwidth, CORESET0 "fits" within a single 'D' subband in a symmetric D-U-D configuration in either FR1 or FR2. For 48 RB CORESET0, at least 40/100/200 MHz channel bandwidth is required for 15/30/120 kHz SCS. Observation 14 For legacy UEs in CONNECTED mode, DL reception of some channels may occur within both 'D' RB sets, while reception of others is restricted to a single 'D' RB set. Observation 15 For new (Rel-18) UEs in CONNECTED mode, DL reception of some channels may occur within both 'D' RB sets according to current specifications, while reception of others is restricted to a single 'D' RB set unless enhancements are made. Observation 16 For both legacy and new (Rel-18) UEs in IDLE mode, UL transmissions (i.e., during initial access) should occur only within UL-only slots. Observation 17 For legacy UEs in CONNECTED mode, UL transmissions should occur only within UL-only slots. |  |  |
| R1-2211983 | Discussion on subband non-overlapping full duplex | NTT DOCOMO, INC. |
| Observation 1: Rel-16 legacy SRS-RSRP measurement and reporting is not applicable for SBFD. Observation 2: Enhancements like beam specific CLI measurement/reporting and Layer 1 based CLI measurement/reporting are also applicable for SBFD. Proposal 1: - If guard band between DL and UL subband in SBFD symbol is required, either DL subband frequency location or guard band size should be explicitly indicated. - If guard band between DL and UL subband in SBFD symbol is not required, DL subband frequency location can be implicitly determined based on UL subband frequency location. Proposal 2: - Study dynamic indication of DL/UL subband time domain location, at least for converting an SBFD symbol into non-SBFD symbol. - NOT support dynamic indication of DL/UL subband frequency domain location. Proposal 3: - Support SBFD operation with UL subband in symbol configured as flexible by TDD-UL-DL-ConfigCommon, at least for symbol configured as DL by TDD-UL-DL-Dedicated. - Study whether/how to support SBFD operation with UL subband in symbol configured as flexible by TDD-UL-DL-ConfigCommon and TDD-UL-DL-Dedicated, with considering interaction with dynamic SFI indication. Proposal 4: - Support only DL reception in DL subband (i.e. not support UL transmission in DL subband). - Support only UL transmission in UL subband (i.e. not support DL reception in UL subband). - Not support UL transmission outside UL subband in SBFD symbol. - Not support DL transmission outside DL subband in SBFD symbol. Proposal 5: For single DCI scheduling multiple PDSCHs/PUSCHs, study possible enhancements when any of the multiple scheduled PDSCHs/PUSCHs overlapping with UL/DL subband. Proposal 6: - For collision handling of higher layer configured channels/signals with SBFD DL/UL subband, study possible enhancements for channels/signals with higher priority first, e.g. PDCCH as highest priority for study. - Study collision for PDSCH/PUSCH/PUCCH repetitions with UL or DL subband in SBFD symbol. Proposal 7: For Rel-18 subband non-overlapping full duplex, at least for higher layer configured channels/signals, three alternatives can be considered: - Alt 1: UE does not expect collision of higher layer configured DL reception or UL transmission with UL subband or DL subband in SBFD symbol. - Alt 2: Separate higher layer DL reception or UL transmission configuration for SBFD symbols and non-SBFD symbols, e.g. separate PDCCH monitoring configuration or PUCCH resource configuration. - Alt 3: No separate configuration, but special handling is needed if higher layer configured DL/UL channel/signal colliding with UL/DL subband in SBFD symbol, e.g. the DL/UL channel/signal is canceled or rate matched around DL/UL subband, or different interpretations are applied for frequency resource allocation for SBFD symbol and non-SBFD symbol. Proposal 8: Study SBFD impact on PUSCH repetition type B segmentation. Proposal 9: Study SBFD impact on SPS HARQ-ACK deferring. Proposal 10: Study possible enhancements on CSI-RS configuration and CSI reporting, including: - Study non-contiguous RB allocation for CSI-RS or narrow band RB allocation for CSI-RS (e.g. smaller value than 24 for number of RBs). - Study possible enhancements for CSI reporting overhead reduction for SBFD, e.g. to allow different frequency band configurations (i.e. different starting PRB and num-ber of RBs) for CSI-RS resources in the same CSI-RS resource set, or to associate different CSI-RS resources with different frequency band configurations and treat them as one “effec-tive CSI-RS”. - Study whether to separately report CSI for non-SBFD symbol/slot and for SBFD symbol/slot. Proposal 11: For DL channel/signal in DL subband and UL channel/signal in UL subband in the same SBFD symbol, study predefined rules to determine whether UE will receive in DL subband or transmit in UL subband in the same SBFD symbol. - To define the predefined rules, following general principles can be considered: - Whether the channels/signals are scheduled by DCI - Whether the channels/signals are periodic or semi-persistent - Whether the channels/signals are with repetitions or without repetitions Proposal 12: Not support SBFD operation with UL subband on SSB symbol or CORESET #0 symbol. Proposal 13: - Support PRACH and Msg 3 PUSCH transmission in UL subband in SBFD symbol. - Support UE monitoring RAR in DL subband in SBFD symbol, if UE doesn’t transmit PRACH in any valid RO on the symbol. Proposal 14: Study SBFD impact on UL cancellation DCI format 2_4. Proposal 15: Study CLI-RSSI measurement and reporting with multiple subbands for CLI-RSSI measurement resource. Proposal 16: Support inter-gNB coordination enhancement with DL/UL subband allocation exchanged between gNBs. |  |  |
| R1-2212043 | SBFD feasibility and design considerations for NR duplex evolution | Samsung |
| Observation 1: Backwards-compatibility for legacy UEs when SBFD is configured in the TDD cell can be achieved by using DDDSU in SIB1 tdd-UL-DL-ConfigCommon Observation 2: It cannot be assumed that SBFD using transparent mode when configuring DFFFU in SIB1 tdd-UL-DL-ConfigCommon and scheduling/configuring DL/UL transmissions for legacy UEs in the SBFD UL subband results in consistent UE behavior Observation 3: 80 dB in FR1 and 87 dB in FR2-1 antenna isolation using spatial separation and RF barrier can be achieved Observation 4: Stopgap performance of the RF barrier for FR1 100 MHz and FR2-1 100 MHz channel BW is feasible Observation 5: 45 dBc subband leakage ratio between the SBFD DL and UL subband when using non-overlapping frequency resources with digital pre-distortion can be achieved in FR1 Observation 6: 28 dBc subband leakage ratio between the SBFD DL and UL subband when using non-overlapping frequency resources can be achieved in FR2-1 Observation 7: Both in FR1 and FR2-1, SBFD can operate with only a few guard RBs between DL and UL subband when sufficient spatial isolation is guaranteed Observation 8: Digital SIC to remove Tx-to-Rx interference in the Rx path results in a noise rise of 0.9dB for SFBD in FR1 Observation 9: Digital SIC to remove Tx-to-Rx interference in the Rx path results in a noise rise of 0.7 dB with 1T1R and 1 dB with 2T2R panel configurations for SBFD in FR2-1 Observation 10: Additional Rx filtering can be applied for FR1 and FR2-1 receivers to increase robustness of the gNB Rx path with respect to ADC and LNA dynamic range without incurring undue insertion losses Observation 11: gNB power consumption aspects are considered in the SBFD feasibility analysis Proposal 1: RAN1 should evaluate the potential impacts from SBFD when a TDD serving cell using SBFD is configured as PCell, SCell or SpCell for a legacy UE Proposal 2: Further study potential enhancements to CA-based SBFD operation in FR2-1 Proposal 3: SBFD operation in SSB symbols is not supported. Proposal 4: Further study whether SBFD operation in DL symbols configured for CORESET#0 or scheduled for PDSCH carrying SI is supported or not. Proposal 5: Further study whether SBFD operation in DL symbols configured for TRS is supported or not. Proposal 6: At least for RRC_CONNECTED mode, the frequency location(s) of the DL subband(s) are explicitly indicated. Proposal 7: At least for RRC_CONNECTED mode, the frequency location(s) of the guard band(s) are implicitly determined by the UE as RBs not configured as either UL or DL subband(s). Proposal 8: For the SBFD-aware UE semi-statically configured with the UL subband in legacy D and F symbols, - UL transmissions within the configured UL subband are allowed in the symbol - UL transmissions outside the configured UL subband are not allowed in the symbol - DL receptions inside the configured UL subband are allowed in the symbol Proposal 9: At least for RRC_CONNECTED mode, the SBFD subbands are configured with respect to the CRB grid. Proposal 10: Frequency locations and time locations of SBFD subbands can be configured independently for a UE in RRC_CONNECTED mode and a UE in RRC_IDLE/INACTIVE modes (if supported). Proposal 12: Multi-slot PUSCH transmissions and PUCCH repetitions can be configured to only use the SBFD slots/symbols Proposal 13: RAN1 to study enhancements to SBFD operation for PDSCH resource allocation type 0 Proposal 14: RAN1 to study enhancements to SBFD operation for PUSCH resource allocation type 1 Proposal 15: RAN1 to study enhancements to resource allocation with SBFD operation for CSI-RS resource set configuration and CSI reporting Proposal 16: RAN1 to study potential benefits and specification impacts when using SBFD slots/symbols for random access |  |  |
| R1-2212115 | Feasibility and techniques for Subband non-overlapping full duplex | Qualcomm Incorporated |
| Observation 1: SBFD operation across multiple component carriers can be achieved using two different design alternatives. - Alt1: intra-band CA using different TDD-DL-UL pattern across the CCs Alt2: Reusing the same design concept of SBFD within component carrier across the CCs. Observation 2: SBFD operation across multiple CCs requires UE supports of CA as prerequisite while CA framework has some inherent UE complexity. Observation 3: Compared to single-CC SBFD, CA-based SBFD has some limitation where DL and UL BW is restricted to the component carrier bandwidth while the inter-channel guardband can’t be utilized. Observation 4: CA-based SBFD operation is interesting for higher band (e.g. FR2-1). Observation 5: SBFD operation at legacy UL slot is not precluded in Rel-18 study item. Observation 6: SBFD operation at legacy UL slot is beneficial in multiple deployment scenarios, e.g., greenfield deployment and UL heavy deployment (InH/InF) to reduce DL blockage and improve DL coverage. Observation 7 Legacy DL slot is important to protect DL reception of UEs that suffer from strong CLI especially when receiving common signalling and UEs that don’t support Rel-16 CLI framework. Observation 8 Transparent SBFD operation (Alt 1) using current 3GPP specification is possible. However, there are restrictions and limitations. - gNB to rely on dynamic scheduling within the SBFD slots and signals these slots as flexible on the cell-specific configuration. - Limited usage of configured UL signals/channels within the SBFD slots (e.g. SRS, CG, hopping, etc) - CSI-RS report overheads for the two DL subbands and limitation on PDSCH scheduling on both subbands Observation 9 Non-Transparent SBFD operation (Alt 4) resolves the limitation/restriction of transparent SBFD (Alt 1) and allow for the following benefits: - Improved UE selectivity, filtering and possible power savings due to reduced sampling rate. - SBFD-aware can transmit UL in RRC configured DL and vice versa - Enable some enhancement on resource allocation (e.g. CSI-RS) and subband scheduling. - Enable slots dependent configurations (e.g power control and timing) Observation 10: It is beneficial for the HD UE to be aware of gNB full duplex operation in specific slot format and the frequency resource’s locations of the DL and UL subbands. Observation 11: There is no extra benefits or gains for SBFD operations using alternative 2-3 over alternative 4. Observation 12: SBFD operation using only Alt 4 achieves all gains of gNB SBFD operations and simplify UE behaviour (only one scheme can be specified). Observation 13 SBFD gNB may not need a guardband between UL and DL subband. However, from UE perspective, a guardband may be needed to reduce inter-UE CLI given there is no (or small) UE selectivity. Observation 14 Based on LLS, increasing the guardband between the scheduled DL and UL helps reducing the inter-UE CLI and recovering some TPUT loss. When inter-UE CLI is too large due to close UEs proximity, increasing the guardband is not helpful. Observation 15: Semi-static configuration of the UL/DL subbands is essential for the SBFD operation. Observation 16: Whether to explicitly or implicitly indicate the frequency location of other subbands depends on the gNB capability of SBFD operation with or without a guardband respectively. Observation 17: Dynamic indication/update of the UL/DL subband can be useful in some scenarios (e.g. gNB fallback to HD mode due to strong interference and dynamic DL or UL scheduling after slots updates or adapting to the UL/DL traffic loads). Observation 18: For SBFD scheme within a single configured UL/DL BWP pair: - Non-aligned UL/DL BWP could be beneficial for some scenarios, e.g., narrowband UL/DL BPW for initial access or default BWP. Observation 19: For SBFD operation using more than one configured DL and UL BWP pair where each subband configured as BWP, it requires a lot of specification impacts and complicate UE behavior. - Requires two active BWPs at a time, each configured with its own DL/UL TDD pattern. - Requires cross-BWP scheduling and cross-BWP HARQ feedback. - Complicates BWP switching mechanism - Requires non-aligned BWP center frequency - Requires some restriction rules may be needed to have common parameters for both DL BWPs - RRC signalling overhead Observation 20: For SBFD operation using more than one configured DL and UL BWP pair where one BWP pair is configured for TDD operation and the pair is configured for SBFD operation, it requires less specification impact mainly to enhance the BWP framework (e.g. non-contiguous RB for BWP, non-aligned UL/DL center freq.) - One UL/DL BWP pair is active at a time - Semi-static configuration of BWP switching pattern to reduce BWP switching delay. - Enable UE to enhance BWP-based selectivity/filtering to reduce inter-UE CLI. Observation 21: SBFD operation should target both SBFD-aware UE and non SBFD-aware UE including both legacy UE and Rel-18+ UEs that are not SBFD-aware. Observation 22: Restriction SBFD operation to symbols configured as DL in TDD-UL-DL-ConfigCommon will prevent non SBFD-aware UE from UL subband transmission. Observation 23: Option #1 is sufficient baseline for SBFD operation of SBFD-aware in downlink symbols with UL-subband UE and conforms with basic assumption/understandings of UL/DL subband operation. Observation 24: Options 2-4 increases the CLI, contradicts with basic understanding of UL/DL subband traffic direction and increases complexity and timeline for adaptive RF tunning and filtering. Observation 25: Dynamic SBFD symbol update (e.g., fallback to TDD mode or adapting subbands) provides the same functionality and flexibility of options 2, 3 and 4. Observation 26: UE indication of the UL/DL subband configurations at initial access is beneficial to enable: - Improve UL coverage for RACH messages by enabling repetition and/or frequency hopping - Enable additional RACH occasions which reduce the collision of the CBRA - Reduce the latency for random/initial access procedure. Observation 27: Transparent SBFD operation based on current 3GPP specification is possible. - SBFD symbols configured as flexible to enable dynamic UL/DL scheduling. - DL scheduling across both DL SBs using RA Type 0 with some limitation on granularity. - CORESET #0, SIB1 and Type-0 CSS can be configured in one the DL subband. Other CORESETs are very flexibly configured using bitmaps. - CSI-RS per each DL subband or wideband CSI-RS configuration in DL slot - Wideband SRS in UL symbols to enable DL CSI acquisition. Observation 28: For CSI-RS in SBFD symbols, gNB can configure: - Option1: Two contiguous CSI-RS resources per each subband and a single CSI report linked to the two resources. - Option 2: Non-contiguous CSI-RS across the two DL subbands. Observation 29: UE complexity increases to process the CSI-RS across the two DL subbands which increase CSI processing latency. Observation 30: The FDRA for RA Type 0 is flexible to enable DL scheduling across the two subbands. There could be some restriction on scheduling flexibility if subbands are not aligned with the RBG grid. Observation 31: A separate PUCCH resource set for SBFD operation is useful to enable PUCCH configuration at edge of the UL subband and to account for the different UL link quality and the different antenna/panels configuration at SBFD symbols than normal UL slot. Observation 32: Available slot counting considered time availability of all symbols based on tdd-UL-DL patterns. Observation 33: The available frequency resources for UL transmission are not the same across the legacy UL symbols and SBFD symbols. Observation 34: There is difference in link quality between SBFD slots and TDD slots due to residual self-interference , increased cross-link interference in SBFD slots and the different number of antennas between slots. Observation 35: It is challenging or restricting to configure semi-static signals and channels within SBFD and TDD symbols. Observation 36: R15/16 introduced resections on multiplexing DL/UL signals and channels at some slots. - UE does not expect to have both dedicated configured reception and transmission on Flexible symbol. - UE doesn’t transmit UL signal/channel at SSB symbol(s) and doesn’t receive DL signal/channel during valid RO (including gap) - UE doesn’t receive on RRC UL symbols and doesn’t transmit on RRC DL symbols. Observation 37: Tx-CLI will affect the dynamic range for the reception of the DL signal while the CLI -leakage (NL) will reduce the DL SINR. Both factors will have direct impact on DL reception quality. Observation 38: It is beneficial for the network to configure the UE with CLI resources at the UL and DL subband to measure the CLI and CLI leakage accordingly. Observation 39: The CLI measurements metrics depends on the CLI resource configuration, whether configured at UL or DL subband and which RS configured for the measurement (e.g., SRS, CSI-RS). Observation 40: Rel-16 CLI framework does not support subband CLI reporting, i.e., reporting CLI metric for one or more subbands in the measurement bandwidth. Observation 41: In SBFD, CLI leakage to adjacent subbands is not uniform over the measurement bandwidth and may require subband CLI reporting. Observation 42: The exchange of the UL/DL subband locations is beneficial for inter-gNB CLI avoidance in SBFD deployment, e.g. for deployment case where cells have different UL/DL subband configurations. Observation 43: Two separate panels with added EM spatial duplexer enables large spatial isolation which facilitate gNB full duplex without the need of complex RF circuitry of analogy interference cancelation or subband filters. Observation 44: More than 80 dB of spatial isolation could be achieved using two separate panels with spatial duplexer. Observation 45: For FR2, more than 80-90 dB of spatial isolation could be achieved using two separate panels at 28 GHz frequency. Observation 46: The frequency isolation could be approximated as flat, non-frequency selective profile and its value per-RB is ACLR+10×log10RBDL Observation 47: Evaluation results show more than 45 dB of frequency isolation for FR1 is achievable with 5 RBs guard band and max DL Tx Power which is aligned with the assumption of 45 dB ACLR. Observation 48: Subband filtering may improve gNB Rx selecting for self-interference, however, it is very challenging for massive MIMO deployment, add extra cost and complexity for supporting SBFD in multiple channels and adds insertion loss. Observation 49: With enough spatial isolation between the panels, there is no need for RF subband filtering. Observation 50: A baseband analog LPF may be used to reject the DL blocker and improve the ADC dynamic range. Observation 51: For FR2, it is feasible for implementation to achieve ACLR requirement without RF filtering. Observation 52: In massive deployment, the large number of digital and analog degrees of freedom can be utilized to provide spatial Tx/Rx beamform nulling for self-interference and clutter mitigation Observation 53: For FR2, the measured 28/39GHz path loss between Tx and Rx antennas including clutter reflections is typically approximately 80 dB or better for empty conference room environment. - Higher path loss is generally observed for larger angular separation between Tx and Rx beams. Observation 54: The residual self-interference including both direct leakage and clutter echo can be cancelled using non-linear digital cancellation algorithm. Observation 55: Digital NLIC can provide additional isolation and improvement to alleviate self-interference. Observation 56: Self-interference could be mitigated by means of spatial isolation, frequency isolation and digital IC which makes SBFD feasible with minimal impact on UL degradation Observation 57: For co-site deployment, gNB should have mitigation capability for the CLI of the co-sited sectors by means of improved spatial isolators and additional digital interference cancellation. - In addition, there could be some specification/requirement on the maximum radiation pattern towards the other co-sited sectors. Observation 58: A prototype of full duplex base station was demonstrated and validated feasibility of Sub-band full duplex gNB in wide-area deployments. Proposal 1: SBFD operation across multiple components is studied at later stage in Rel-18 after establishing the baseline study of SBFD operation within component carrier. Proposal 2: Not all DL slots are used for SBFD operation. Proposal 3: The maximum number of DL subbands for SBFD operation in legacy UL symbols within a TDD carrier is one. The DL subband can be located at one side of the carrier (UDU pattern) or at the middle of carrier (UD, DU) pattern depending on RAN4 feasibility Proposal 4: Non-transparent SBFD operation using Alt-2 and Alt-3 are not considered in Rel-18. Proposal 5: RAN1 to further discuss UE-specific guardband configuration. Proposal 6: The frequency location of the guardband(s) are explicitly indicated (if any). Proposal 7: The other subband(s) are implicitly determined based on the bandwidth of the component carrier bandwidth and excluding the frequency location of the UL subband and the guardband Proposal 8: The period for SBFD operation is based on the periodicity of the TDD-DL-UL pattern(s). Proposal 9: Support cell-common semi-static configuration of the time and frequency location of UL/DL subbands and guardband for SBFD operation. Proposal 10: Support dynamic indication/update of the at least time locations of the SBFD subbands. - FFS: dynamic indication/update of the frequency locations of the DL and/or UL SBFD subbands. Proposal 11: RAN1 to further discuss enhancement for SBFD operations using two or more BWP pairs with single active UL/DL BWP pair and semi-static switching of BWP pattern. Proposal 12: Support semi-static configuration of UL subband in symbols configured as ‘downlink’ or ‘flexible’ in TDD-DL-UL-ConfigCommon. Proposal 13: Support only Option 1 as the baseline for SBFD-aware UE scheduling. UE can be configured to measure CLI in the UL subband for AGC blocking and/or LNA compression Proposal 14: RAN1 to study potential benefits of initial access for SBFD-aware UE Proposal 15: Support broadcast of the UL/DL subband locations for the initial UL/DL BWP for SBFD-aware UE. Proposal 16: Support configurations of initial access messages on UL subbands of SBFD symbols for both RRC-idle and RRC connected UEs. Proposal 17: Study mechanism to facilitate SBFD aware UE to select SBFD capable cells. Proposal 18: gNB should handle legacy UE by utilizing Rel-16 CLI framework and proper scheduling. Proposal 19: For the coexistence study of legacy UE, No change in UE RF requirements. Proposal 20: UL-subband and guardband are considered as non-available resources for DL reception. PDSCH symbols are rate-matched around these resources and DMRS Proposal 21: RAN1 to further study non-contiguous frequency resources configuration of the CSI-RS and impact to UE CSI processing and reporting latency. Proposal 22: RAN1 to further study partial CSI subband reporting at the edges of the DL subband(s) boundaries. Proposal 23: Any further optimization for the FDRA field in scheduling DCI should be well motivated, e.g., partial RBG at DL subband(s) boundaries. Proposal 24: RAN1 to discuss separate PUCCH configuration for SBFD and UL slot. Proposal 25: RAN1 to discuss PUCCH transmission across with inter-slot frequency hopping across multiple SBFD and UL slots. Proposal 26: RAN1 to further study the impact/potential enhancement of frequency hopping with SBFD operation. Proposal 27: RAN1 to further study the impact/potential of enhancement of available slot counting in SBFD. Proposal 28: RAN1 to further study SBFD specific signals and channel configuration (e.g. CG and SPS). Proposal 29: RAN1 to further study separate operation parameters can be pre-configured for TDD and SBFD slots. Proposal 30: The restriction rules on the DL/UL channel/RS multiplexing can be relaxed in SBFD symbols for a SBFD-aware UE to improve resource utilization, reduce DL/UL switching delay and traffic latency. Proposal 31: RAN1 to further study the SBFD-aware UE collision scenarios in SBFD symbols for the following cases, e.g. - Semi SFI D/U vs RRC U/D - Semi SFI D/U vs Dynamic U/D - RRC D/U vs RRC U/D - SBFD symbols with valid ROs - SBFD symbols with SSBs Proposal 1: SSB can be configured in DL subband in SBFD symbol. - FFS: whether SSB can be multiplexed with UL signal can be further discussed Proposal 33: RO can be configured in UL subband in SBFD symbol. - FFS: whether RO can be multiplexed with DL signal can be further discussed. - FFS: whether RO in SBFD symbols can be used at least for only SBFD-aware Proposal 34: The restriction rules on the DL/UL channel/RS multiplexing can be relaxed in SBFD symbols for both connected UEs and idle UEs. Proposal 35: RAN1 to further discuss CLI measurement resources configuration in the DL and/or UL subbands Proposal 36: For UL subband, RAN1 to further discuss CLI measurements reporting based on CLI-SRS-RSRP, and CLI-RSSI for UL-subband configured CLI resources and CLI-SINR and CLI-RSSI for CLI resources configured at the DL subband. Proposal 37: UE implicitly determines the non-contiguous frequency resources for CLI-RSSI measurement in the DL subband(s). Proposal 38: Support subband-based CLI reporting for accurate measurement of CLI leakage in SBFD. RAN1 to further discuss subband configurations within the CLI resource. Proposal 39: To reduce subband CLI reporting overhead, UE report CLI measurements in specific subband(s) where CLI exceeds a configured CLI threshold. - In addition, differential CLI reporting could be considered to reduce the overhead. Proposal 40: gNB to exchange information of the CLI resource configurations and/or CLI measurements. Proposal 41: Support exchange of the UL/DL subband locations between the cells. Proposal 42: Support subband-based inter-gNB CLI reporting for accurate measurement of CLI leakage in SBFD. |  |  |
| R1-2212149 | Discussion on subband non-overlapping full duplex | Sharp |
| Observation 1: As baseline for study, subbands are a set of resources defined by time/frequency region which is semi-statically configured by gNB. The subbands are not necessarily used for SBFD operation, but maybe used for SBFD operation. Proposal 1: From gNB perspective, UL subband can be used to perform DL transmission dedicated for legacy UEs as well as UL reception. Proposal 2: From SBFD capable UE perspective, UL subband can be used to perform DL reception for cell-specific signalling. Proposal 3: From SBFD capable UE perspective, UL subband can be used to perform DL reception as well as UL transmission. Proposal 4: Explicit/implicit configuration of length of the period for SBFD subband time location indication should be considered as a baseline. Proposal 5: New dynamic signalling should be studied to specify which direction of DL/UL is applied to the UL subband. Proposal 6: When a SS/PBCH block overlaps with the UL subband in time and frequency resource, the UE should not expect UL transmission in the resource and the UE should be able to receive the SS/PBCH block. Proposal 7: When a SS/PBCH block overlaps with the UL subband in time resource whereas the SS/PBCH block doesn’t overlap with the UL subband in frequency domain, the following two options should be further studied: - The UE shouldn’t expect UL transmission in the resource and the UE should be able to receive the SS/PBCH block - The UE should expect UL transmission in the resource and the UE is not required to receive the SS/PBCH block Proposal 8: The following two options can be further studied: - CSI-RS allocation overlapping with the UL subband - CSI-RS configured with two partitions Proposal 9: For PDSCH resource allocation, type-1 RA with DCI format 1_0 in USS and type-0 RA is priority. Proposal 10: RAN1 should discuss whether simultaneous SS/PBCH block transmission and UL reception in SBFD symbols would be supported or not, before discussing the collision between SS/PBCH blocks and any UL transmission. Proposal 11: No further study on handling cases of collision between higher layer configured DL and UL. Proposal 12: RAN1 further study the impact on procedures for UE-specific uplink repetition. Proposal 13: Msg3 PUSCH repetition should be studied for SBFD. |  |  |
| R1-2212194 | Details of subband non-overlapping full duplex | ASUSTeK |
| Observation 1: Different option of scheduling handling/constraint would be preferred subject to signaling details of subband SFI. Proposal 1: A joint design of scheduling expectation and signaling details of subband SFI is conducted by RAN1. Proposal 2: If subband SFI is designed in a semi-static manner, at least one of option 2, 3, or 4 should be considered. Observation 2: Collision handling rules are separately designed for DCI-based transmission/reception and configured-based transmission/reception. Proposal 3: when discussing the following four option, the “scheduling” based on DCI or “scheduling” based on configuration may need to be distinguished/separately considered as preferred options for the two types of scheduling could be different: - Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol - Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol - Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol - Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol |  |  |
| R1-2212249 | Discussion on subband non-overlapping full duplex for NR | MediaTek Inc. |
| Observation 1: Non-contiguous and contiguous subband layout yield similar latency gain in single retransmission scenario high-data-rate, low-latency traffic scenario. But non-contiguous SBFD symbols cause less perturbation to synchronized TDD layout. Observation 2: The TDD validation rule for PRACH occasions should be preserved. Overlap between SBFD symbols and valid PRACH occasions should only be allowed if UL-DL and DL-UL CLI from CBRA can be controlled by careful scheduling, while still improving CFRA latency. Observation 3: Configuring a single MCS for semi-static transmissions in SBFD systems will negatively impact system performance since it has two slot types with different interference levels Observation 4: CORESET allocation readily supports non-contiguous frequency-domain allocation and monitoring up to three different CORESETs in a BWP. Observation 5: Configuring PDCCH monitoring over different CORESETs for SBFD and DL-only slots/symbols may require configuring separate search spaces per each slot in the periodic SBFD partition pattern, each with the same slot periodicity but different slot offsets, soon using up the maximum number of search spaces that can be configured. Observation 6: When using FDRA Type-0 for PDSCH scheduling, certain RBG(s) may be unavailable due to partial overlap with UL-subband (and/or any guard band) leading to resource fragmentation. Observation 7: VRB-interleaving is not supported by FDRA Type-0 and is mostly incompatible with SBFD by FDRA Type-1 allocation, since PRB segments likely overlap with RBs that are not available to downlink. This prohibits VRB-interleaving altogether. Observation 8: For semi-static UL transmissions, a single FDRA is configured for all UL slots. This may impact flexibility in resource allocation in SBFD since it has two types of slots with different bandwidths. Observation 9: For UL transmissions with repetition, a single FDRA is defined for all slots within a repetition. This may impact flexibility in resource allocation in SBFD since it has two types of slots with different bandwidths. Observation 10: When frequency hopping is enabled for PUSCH/PUCCH transmission in SBFD, the resource allocation with the first and second hops may not be confined within the UL subband of SBFD partitioned slots/symbols. Observation 11: For periodic and semi-persistent SRS transmission, a single frequency domain resource allocation is configured for all UL slots. This may impact flexibility in resource allocation in SBFD since it has two types of slots with different bandwidths. Observation 12: For SRS transmission in SBFD, the resource allocation with and without frequency hopping may not be confined within the UL subband of a SBFD partitioned slot. Observation 13: For CG PUSCH transmission in SBFD systems PUSCH fragmentation will occur on UL-only slots since the frequency domain resource allocation on SBFD slots is at the centre of the UL BWP. Observation 14: The impact of PUSCH fragmentation on UL performance can be reduced by placing periodic PUSCH resources originating from a SBFD slot at the edge of an UL-only slot. Observation 15: Non-contiguous resource allocation using FDRA Type 0 is an optional feature for legacy UEs and cannot be relied upon in the case of PUSCH fragmentation in SBFD systems. Proposal 1: Frequency domain configuration is broadcasted as part of SIB, listing at least the UL subband(s), specifying their starting CRB(s) and RB length(s). Proposal 2: On symbols indicated as flexible by SFI group common signalling, the SBFD partition status is assumed to be unknown by UE, irrespective of other configurations. Proposal 3: Any transmission or reception overlapping with either a subband or a symbol indicated as ‘flexible’ must be deprioritized unless it is scheduled by a DCI, in connected mode. Proposal 4: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol, with the following exceptions: Inter-subband UE-UE CLI measurement are not confined within the DL subband. UE assumes that the symbol is not partitionned if symbol is indicated to be ‘flexible’ by group common SFI signalling of UE TDD link direction. DL reception scheduled by DCI is allowed to overlap with subband indicated as flexible. Proposal 5: The time-domain configuration of the SBFD layout should support two back-to-back periodic patterns, to accommodate different layout in slots containing SSB. Proposal 6: SBFD layout in frequency and time is configured per carrier. Proposal 7: Allow configuration of non-contiguous SBFD symbols in temporal pattern. Proposal 8: Do not allow overlap between SSB and SBFD symbols. Proposal 9: Adopt the following prioritization rules (assuming the specific collision can occur): Prioritize dynamic scheduling over semi-static scheduling, except in the cases of SSB and Msg1/MsgA in CBRA. Leave prioritization to UE implementation when Msg1/MsgA in CBRA collides with reception. Prioritize SSB over semi-statically configured transmission other than Msg1/MsgA in CBRA. Prioritize monitoring PDCCH Type 0/0A/1/2 in CSS over UE dedicated semi-statically scheduled uplink transmission. When uplink and downlink scheduling does not allow for the gurad-gap required for Tx-Rx and Rx-Tx turn-around by the standard then deprioritize one of them as if a collision have happened according to the respective rules. Proposal 10: Study the feasibility of enabling Two MCSs based on slot type Proposal 11: Study enhancements to Search Space configuration which allow adaptation to SBFD partitioning. Proposal 12: Support allocation of fractional RBGs in Type-0 FDRA at DL subband boundaries. Proposal 13: Support interleaved mapping of odd and even VRBs over physical RBGs allocated using Type-0 FDRA. Proposal 14: Support two FDRA for semi-static UL transmissions (e.g., CG PUSCH, periodic/semi-persistent PUCCH) based on slot type. Proposal 15: For semi-static UL transmissions, support skipping of periodic transmissions in SBFD partitioned slots/symbols. Proposal 16: Support two FDRA for UL repetitions (e.g., DG PUSCH repetition, PUCCH repetition) based on slot type. Proposal 17: Allow the support of skipping of UL repetitions that occur in SBFD slots. Proposal 18: Modify the frequency hopping procedure for PUSCH transmission to ensure that the frequency domain position is confined within the UL-SB in the SBFD slots Proposal 19: Support two frequency domain resource allocations for intra-slot and inter-slot frequency hopping for PUCCH transmission in SBFD based on slot type Proposal 20: Support PUSCH/PUCCH frequency hopping in the UL-only slots for inter-slot frequency hopping. Proposal 21: Support two frequency domain resource allocations for periodic and semi-persistent SRS transmission based on slot type. Proposal 22: Modify the frequency domain resource allocation procedure for SRS transmission to ensure that the frequency domain position is confined within the UL-SB in the SBFD slots Proposal 23: Support skipping of periodic and semi-persistent SRS transmissions in SBFD slots. Proposal 24: To avoid the problem of fragmentation in SBFD systems, support the option of configuring two FDRA for semi-static UL transmissions based on slot type. |  |  |
| R1-2212284 | On subband non-overlapping full duplex for NR | Nokia, Nokia Shanghai Bell |
| Observation 1: SBFD cannot be operated without changes to the RF architecture and as such SBFD needs new physical implementations and cannot be software upgraded to existing and deployed base stations. Observation 2: With clustered UE distribution in UMa Scenario, SBFD provide UL throughput gains across the 5%/50%/95%-iles for low load conditions. For medium and high load, large performance degradation is observed for all the percentiles of the UE throughput primarily as a consequence of the inter-site gNB-gNB inter-subband interference. Observation 3: Assuming similar ratio of DL resources for SBFD and TDD (XXXXX vs DDDSU), SBFD provides a DL throughput degradation of approximately 20% at low loads mainly as a consequence of the guardband (overhead) between DL and UL subbands, and lower availability of DL RBs to deliver the 0.1 MB payload. At medium and high loads, the performance degradation can reach up to 99% at the 5%-ile UE UPT, where some UEs experience close to zero DL throughput due to the presence of strong UE to UE CLI. Observation 4: With clustered UE distribution in UMa Scenario, performance degradation in the order of 20%-100% is generally observed with SBFD when compared to static TDD in both DL and UL directions. Only exception is the UL throughput performance at low load conditions where approximately 300%/30%/20% improvement in the 5%/50%/95% average UE UPT is obtained Observation 5: The basic BWP based scheme is rather straightforward but has severe UE peak rate and signaling overhead issue. Observation 6: For the advanced BWP based scheme, there should be no additional gap due to BWP switch in the transition from SBFD slots to TDD slots, and vice versa. Observation 7: In terms of digital filter bandwidth adaptation at the UE, there is substantially no difference between the RB-set based and the advanced BWP based approach. Observation 8: Unless new in-band emission requirements are specified, better UL transmission and DL reception filtering should not be considered as an advantage of any of the proposed schemes. Observation 9: Operation according to the basic BWP based scheme is not precluded by RB-set based signaling. Observation 10: Enabling initial access in SBFD symbols can increase the PRACH capacity, PRACH coverage, as well as reduce initial access delay. Observation 11: (Extension of) existing rate matching techniques based on the knowledge of UL subband location is a straightforward solution that works for both Type 0 and Type 1 FDRA. Observation 12: Options available with current standard specifications to provide different CORESET configurations in different slots/symbols all present limitations in terms of either limited flexibility or excessive signaling overhead. Observation 13: Use of NTA, offset = 0 is only possible in SBFD slots and it increases the required guard time between the end of a DL transmission in a DL slot and the start of an UL reception in a SBFD slot. Observation 14: Use of NTA, offset = 0 in SBFD slots and NTA, offset > 0 in TDD UL slots further increases the required overhead needed to switch between legacy TDD DL/UL slots and SBFD slots. Observation 15: One obvious way to mitigate intra-cell inter-subband UE-to-UE CLI is to tighten the UE in-band emission requirements. The UE being able to match its UL/DL digital filter’s bandwidth to the UL/DL subband in SBFD symbols may also help reducing the UE in/band emissions. However, such advance UE RF requirements are for RAN4 to be discussed. Observation 16: By knowing the time difference of arrival between DL (transmitted by the serving gNB) and UL RS (transmitted by the aggressor UE) at the victim UE, the gNB can estimate the intra-cell (aggressor)UE-to-(victim)UE propagation delay. This information can be used to assist the UE by e.g. providing the specific timing offset to be used when performing a CLI measurement. Proposal 1: Explicit indication of the semi-static frequency location of at least one of the DL subband(s) and the guardband(s) is required. The one that is not explicitly signalled can be implicitly determined. Proposal 2: Signaling of the frequency and time location of the UL subband via SIB is assumed as baseline. Signaling via dedicated RRC can be considered as an additional (not alternative) option. Proposal 3: Study the required enhancements to the TDD frame format signaling framework to enable semi-static configuration of SBFD. Proposal 4: SBFD operation with the possibility to configure the SBFD UL subband only in a subset of the legacy DL symbols is agreed as the baseline. Proposal 5: Assume RB-set based scheme as the baseline. With the RB-set based scheme, a UE’s active BWP may include PRBs that are used by the serving cell for the opposite link direction. Proposal 6: Confirm the following discussed but not agreed proposal from RAN1 #110bis-e meeting: For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - A set of contiguous RBs in a carrier at least excluding RBs of UL subband is defined as a DL subband for discussion purpose - FFS whether frequency location of DL subband(s) is explicitly indicated or implicitly determined - FFS whether guardband(s) need to be defined and whether UE is aware of such guardband(s) - Note: if guardband(s) are defined, the DL subband(s) additionally exclude the RBs of the guardband(s) - DL receptions within DL subband(s) are allowed in the symbol - FFS whether DL receptions within UL subband are allowed or not in the symbol - FFS whether/how the symbols can be converted to DL-only symbols - Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol as in existing specification Proposal 7: For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, at least one of the following options are assumed as the baseline: - DL receptions within the UL subband are allowed in the symbol - the symbol can be converted to a DL-only symbol Proposal 8: For a SBFD aware UE in a symbol semi-statically configured as DL or Flexible in tdd-UL-DL-ConfigurationCommon, study mechanisms enabling dynamic configuration of the SBFD UL subband in the symbol. Proposal 9: Study support of initial access on SBFD symbols by e.g. introducing a new SBFD RACH configuration enabling at least Msg1/MsgA transmissions during SBFD symbols. Proposal 10: Study requirements for supporting initial access transmissions other than Msg1/MsgA in SBFD symbols. Proposal 11: Study the severity of PDSCH resource fragmentation problem (and potential solutions) for Type 0 and Type 1 FDRA, considering remaining issues when rate matching solutions already supported in current specifications are applied and/or extended. Proposal 12: Study and down-select solutions to solve the frequency resource fragmentation problem of CSI-RS in SBFD slots/symbols. Proposal 13: Study solutions to configure the UE with SBFD-specific CG-PUSCH or SPS-PDSCH resources to be used during SBFD slots/symbols. Proposal 14: How the multi-slot PDSCH/PUSCH is mapped on different type of slots based on the same DCI or grant, should be studied in RAN1. Proposal 15: Study solutions enabling dynamic adaptation of PDCCH resources based on the occurrence of SBFD symbols and the corresponding DL-UL frequency domain partitioning. Proposal 16: consider using NTA, offset = 0 in SBFD slots and NTA, offset > 0 in TDD UL to solve the FFT misalignment problem between UL Rx and DL Tx in SBFD slots, and study solutions to reduce the increased overhead when switching between TDD DL/UL slots and SBFD slots. Proposal 17: Study L1/L2 based CLI, including potential enhancements to the RS configuration for the purpose of L1/L2 based CLI measurement/reporting. Proposal 18: Study schemes for measurement and reporting of time difference of arrival between DL RS and UL RS, and potential extensions to the CLI measurement framework to include assistance information consisting of e.g. specific timing offset (with respect to DL timing) to be used when performing a CLI measurement. Proposal 19: Study subband CLI measurements and reports, including subband CLI-RSSI measurements performed on a subband while the UE is transmitting on a different subband. Proposal 20: Study possible enhancements to the exchange of intended TDD configuration over Xn to include SBFD subband configuration. Proposal 21: The potential benefits of boosting the UE Tx power and/or reducing the gNB power in SBFD slots/symbols shall be further investigated as a potential method to boost the UL received SINR in slots/symbols affected by gNB self-interference. Proposal 22: RAN1 to study the potential benefits of SBFD aware UEs prioritizing SBFD cells and/or legacy UEs de-prioritizing SBFD cells when performing cell (re)selection. Detailed solutions are for RAN2 to discuss and specify if/when normative work for SBFD is agreed. Proposal 23: for time domain conflict of SBFD aware UE’s UL and DL operation in the same SBFD symbol, at least study whether/how to avoid/handle the following collision cases: - Semi-static UL (including valid RO) vs. semi-static DL (including SSB/Type 0 CSS) - Dynamic UL vs. SSB / Type 0 CSS |  |  |
| R1-2212289 | Discussion on subband non-overlapping full duplex | KDDI Corporation |
| Observation 1: For the inter-UE CLI, - Increasing the guard RBs between DL and UL is helpful to reduce the inter-UE CLI - Small DL BWP helps reducing the inter-UE CLI - When the UE-to-UE distance is less than 1 m, the DL performance is still degraded even with the higher number of guard RBs due to limited dynamic range of ADC Proposal 1: RAN1 should study inter-UE CLI when UE-to-UE distance is less than 1m and investigate mitigation methods. Proposal 2: RAN1 to study the feasibility and potential benefit of SBFD scheme with more than one configured DL and UL pair with aligned/unaligned center frequencies for a DL and UL BWP pair. |  |  |
| R1-2212290 | Discussion on subband non-overlapping full duplex | KT Corp. |
| Proposal 1: Confirm below proposal: For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - A set of contiguous RBs in a carrier excluding RBs of UL subband and guardband(s) are defined as a DL subband for discussion purpose - FFS whether frequency location of DL subband(s) and guardband(s) are explicitly indicated or implicitly determined - DL receptions within DL subband(s) are allowed in the symbol - FFS whether/how the symbols can be converted to DL-only symbols Proposal 2: Confirm below proposal: For a SBFD aware UE semi-statically configured with UL subband in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, - UL transmissions within UL subband are allowed in the symbol - Allow conversion from SBFD symbol to DL-only, or UL-only symbol - FFS whether DL receptions within UL subband are allowed or not in the symbol - UL transmissions outside UL subband are not allowed in the symbol - DL receptions outside UL subband are allowed in the symbol - FFS: interaction of transmissions and receptions with TDD-UL-DL-ConfigDedicated and dynamic SFI - FFS whether/how the symbols can be converted to non-SBFD symbols - FFS: whether/how guardband(s) are explicitly indicated |  |  |
| R1-2212334 | Discussion on sub-band non-overlapping full duplex | ITRI |
| Proposal 1: The frequency location of DL subband is explicitly indicated and the guardband can be implicitly determined. Proposal 2: UL transmissions within UL subband in symbols configured as DL or F in TDD-UL-DL-ConfigCommon are supported. Proposal 3: Support SBFD operations in SSB symbols. Proposal 4: Following conflict cases could be studied in Rel-18 - Dynamic scheduled DL reception in the DL subband v.s. RRC configured UL transmission in the UL subband, - Dynamic scheduled UL transmission in the UL subband v.s. RRC configured DL reception in the DL subband, - Dynamic scheduled DL reception with a first priority in the DL subband v.s. Dynamic scheduled UL transmission with a second priority in the UL subband - SSB reception in SBFD symbols - RACH transmission in SBFD symbols |  |  |
| R1-2212424 | Discussion on subband non-overlapping full duplex | CEWiT |
| Observation 1: BWPs allocated to UEs might contain parts of the full UL SB. Proposal 1: The frequency domain location of the full UL SB is provided to the SBFD aware UE where the frequency domain location is a combination of the starting subcarrier of the UL SB and the bandwidth in terms of subcarriers/RBs/RBGs. Observation 2: Since, the guard band is always in conjunction with the UL SB in time domain, there is no need to provide explicit time domain location for the guard band. Proposal 2: The time location is not explicitly provided for guard bands. Observation 3: The guard band is expected to start at the end(s) of the UL SB. Hence, the frequency domain start location for guard bands need not be explicitly signalled. Proposal 3: For signalling of guard bands, only the bandwidth of the guard band(s) is provided in terms of subcarriers/RBs/RBGs. Proposal 4: gNB configuring separate TDD configuration for each BWP pair of the UE is supported. Observation 4: To allow such flexibility of scheduling and efficient utilization of available bandwidth for SBFD, TDD configurations and SFI formats starting with UL slots/symbols can be introduced. Proposal 5: TDD configurations starting with UL slots/symbols are supported. Observation 5: Allowing reception of CSI-RS within an UL subband and guard band (if any) has the following benefits: - No changes are required to the specification wrt CSI-RS. - The CSI-RS remains wideband. The quality of channel estimated might be affected if 2 separate CSI- RS resources are allocated to the 2 DL subbands depending on the size of the DL subbands in frequency domain. Similarly, in case of non-contiguous CSI-RS resource, the channel estimation quality will be affected. - The CSI can be estimated over the whole BWP which can be later used fully as DL. Proposal 6: Support reception of CSI-RS within an UL subband and guard band (if any). Proposal 7: Study FDRA related enhancement for SPS PDSCH and CG PUSCH. Proposal 8: Study enhancements related to repetition of UL signals/channels e.g., PUCCH, SRS etc. for coverage enhancement. Proposal 9: SBFD operation is not allowed in SSB symbols. Observation 6: Using different panels at the gNB for DL and UL operation reduces interference but impacts reciprocity assumption between UL and DL channels. Proposal 10: Study the impacts of using different panels at the gNB for DL and UL operations on reciprocity assumption of channels. |  |  |
| R1-2212438 | Discussion on subband non-overlapping full duplex | WILUS Inc. |
| - Proposal 1: RAN1 to study dynamic deactivation of semi-static SBFD configuration. - SBFD aware UE can be indicated to deactivate the SBFD operation via dynamic SFI. - FFS: impact and potential enhancements on dynamic SFI for the co-existence of SBFD aware UE and non-SBFD aware UE in a cell. - Proposal 2: RAN1 to study link direction of UL/DL subband in a symbol configured as DL and flexible by tdd-UL-DL-ConfigurationCommon. - In a symbol configured as DL and flexible by tdd-UL-DL-ConfigurationCommon, DL reception is not allowed within UL subband. - In a symbol configured as flexible by tdd-UL-DL-ConfigurationCommon, both DL reception and UL transmission are allowed outside UL subband. - Proposal 3: RAN1 to study impact and potential enhancements due to unaligned boundaries of DL/UL subband and configuration granularity of RBG for PDSCH FDRA (i.e., FDRA type 0). - Enhancements on RBG mapping of PDSCH FDRA type 0 or reusing PDSCH rate-matching mechanism. - Proposal 4: RAN1 to study impact and potential enhancements of PDSCH receptions with slot aggregation (repetitions) across non-SBFD symbols and SBFD symbols. - Same TBS can be determined among PDSCH receptions with slot aggregation (repetitions) across non-SBFD symbols and SBFD symbols. - Proposal 5: RAN1 to study Option 3 (one contiguous CSI-RS resource configuration across non-contiguous DL subbands) in terms of specification impact. - A UE can assume that PRBs in UL subband (and guard band, if specified) are not available for CSI-RS reception. - Observation 1: Current CSI reporting mechanism can be reused without enhancement for SBFD aware UE due to no ambiguity between SBFD aware UE and gNB on DL/UL subband locations. - Proposal 6: RAN1 to study enhancement on intra-slot frequency hopping and inter-slot frequency hopping for PUSCH and PUCCH in SBFD operation. - Different frequency hopping patterns can be determined across non-SBFD symbols and SBFD symbols. - Proposal 7: RAN1 to study following collision cases for SBFD aware UE: - Valid RO vs. DL signal/channel. - Higher layer configured DL (or UL) signal/channel vs. higher layer configured UL (or DL) signal/channel. - DCI indicated DL (or UL) signal/channel vs. higher layer configured UL (or DL) signal/channel. |  |  |

Appendix B: Previous agreements of SBFD

RAN1#109-e

Agreement

Study whether/how to inform the UE of the time and/or frequency location of subbands that gNB would use for SBFD operation.

Agreement

Study the impact/potential enhancements of resource allocation in symbols with subbands that gNB would use for SBFD operation.

Agreement

At least study SBFD operation within a TDD carrier

Conclusion

For discussion purpose only, SBFD symbols is defined as symbols with subbands that gNB would use for SBFD operation.

Conclusion

For discussion purpose, for SBFD operation within a TDD carrier, a SBFD subband consists of 1 RB or a set of consecutive RBs for the same transmission direction.

Agreement

The time and frequency location of subbands within a TDD carrier are not fixed in the specification.

- Subject to any RAN4 guidance on minimum or maximum subband and guardband size and subband location within TDD carrier.

- Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed.

Guideline for future meetings

- Note: AI 9.3.3 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD.

- Note: AI 9.3.2 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

RAN1#110

Agreement

Study the following alternatives with Alt 4 prioritized, for SBFD operation at least for RRC_CONNECTED state.

- SBFD operation Alt 1:

  - Time and frequency locations of subbands for SBFD operation are not known to UEs.

  - UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side.

- SBFD operation Alt 2:

  - Time and frequency locations of subbands for SBFD operation are not known to UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs

- SBFD operation Alt 3:

  - Only time location of subbands for SBFD operation is known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

UE capability discussion is held in work item phase.

Agreement

For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline.

Agreement

For semi-static configuration of subband location, consider same subband frequency resources across different SBFD symbols as baseline.

Working Assumption

For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline.

- FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies

- FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair

Agreement

For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, study the following options:

- Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol

- Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol

Agreement

Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for SBFD, at least includes:

- Measurement resource/reporting configuration

- Measurement/reporting details (including UE processing delay)

- Relevant information exchange (between gNBs) if needed

- Usage of measurement at gNB

Note: other enhancement(s) for gNB-to-gNB and UE-to-UE CLI handling specific for SBFD are not precluded.

RAN1#110bis-e

Agreement

For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline.

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

Agreement

For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required.

- FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined.

Agreement

Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands.

Agreement

Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE

- If there are, whether/how to avoid/handle such collision cases (as second step)

Agreement

Study impact/potential enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency.

Agreement

Study whether SBFD operation in SSB symbols is supported or not.

Agreement

For SBFD operation within a TDD carrier, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline.

Agreement

The maximum number of UL subbands for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier is one for the study in RAN1.

- The UL subband can be located at one side of the carrier.

- The UL subband can be located at the middle part of the carrier

Note: RAN1 considers the above two possibilities unless RAN4 concludes that any one is infeasible.

Note: Two UL subbands for SBFD operation in an SBFD symbol within a TDD carrier due to SBFD operation in legacy UL symbols is subject to further RAN1 discussions which is 2nd priority as per RAN guidance.

Send an LS to RAN4 to inform the above agreement. If RAN4 has response, it will be taken into account but in the meanwhile, RAN1 work will continue based on the above.

LS on maximum number of UL subbands for duplex evolution to RAN4 is endorsed. Final LS in R1-2210671.

Agreement

For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.