3GPP TSG RAN WG1 #111	R1-2212784

Toulouse, Nov 14-18, 2022

Agenda item:	8.13

Source:	Moderator (Ericsson)

Title:	Summary of Rel17 Maintenance on NR Dynamic spectrum sharing (DSS)

WI:	NR_DSS

Document for:	Discussion and Decision

# 1	Introduction

This document summarizes discussions for RAN1#111 for Rel17 NR DSS WI considering below documents submitted for A.I. 8.13

| Ref# | TDoc | Title | Source | Agenda item |
| --- | --- | --- | --- | --- |
| 1 | R1-2211492 | Remaining issue in Rel-17 SCell-to-PCell scheduling | OPPO | 8.13 |
| 2 | R1-2212096 | Clarification for cross-carrier scheduling from SCell to P(S)Cell | Qualcomm Incorporated | 8.13 |
| 3 | R1-2212162 | Clarifications for Cross-carrier scheduling from SCell to P(S)Cell | Ericsson | 8.13 |

# 2	Discussion

All the submitted documents discuss clarification related to CSI-MeasConfig when SCell to PCell scheduling is used. This issue was initially raised in [4] and discussed as Topic 6 of [110bis-e-R17-DSS-01] email thread in previous meeting [5].

R1-2212096 [2] proposes the following

Proposal: RAN1 to clarify either of the following:

- Opt.1: when a UE is configured with cross-carrier scheduling from sSCell to P(S)Cell, for A-/SP-CSI report on PUSCH on P(S)Cell, the UE refers to the CSI reporting related parameters provided in CSI-MeasConfig of P(S)Cell, regardless of whether the DCI that triggers/activates the A-/SP-CSI on PUSCH on P(S)Cell is received on P(S)Cell or sSCell.

- Opt.2: when a UE is configured with cross-carrier scheduling from sSCell to P(S)Cell, , for A-/SP-CSI report on PUSCH on P(S)Cell, the UE refers to the CSI reporting related parameters provided in CSI-MeasConfig of P(S)Cell if the DCI that triggers/activates the A-/SP-CSI on PUSCH on P(S)Cell is received on P(S)Cell, and refers to those provided in CSI-MeasConfig of sSCell if the DCI that triggers/activates the A-/SP-CSI on PUSCH on P(S)Cell is received on sSCell. Network ensures that the parameters in CSI-MeasConfig of P(S)Cell and sSCell are configured such that joint set of these parameters across P(S)Cell and sSCell does not result in exceeding any of the UE’s capabilities for A-/SP-CSI reporting on PUSCH on P(S)Cell, such as:

  - maxNumberAperiodicCSI-triggeringStatePerCC

  - maxNumberAperiodicCSI-PerBWP-ForCSI-Report

  - maxNumberAperiodicCSI-PerBWP-ForBeamReport

  - maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report

  - maxNumberSemiPersistentCSI-PerBWP-ForBeamReport

- Opt.3: when a UE is configured with cross-carrier scheduling from sSCell to P(S)Cell, for A-/SP-CSI report on PUSCH on P(S)Cell, the CSI reporting related parameters provided in CSI-MeasConfig of P(S)Cell and sSCell are identical such that the UE can refer to the parameters provided on either of the P(S)Cell and sSCell for A-/SP-CSI on PUSCH on P(S)Cell.

Proposal: RAN1 to clarify either of the following:

- Opt.1: when a UE is configured with cross-carrier scheduling from sSCell to P(S)Cell, for A-/SP-CSI report on PUSCH on P(S)Cell, the UE refers to the CSI reporting related parameters provided in CSI-MeasConfig of P(S)Cell, regardless of whether the DCI that triggers/activates the A-/SP-CSI on PUSCH on P(S)Cell is received on P(S)Cell or sSCell.

- Opt.2: when a UE is configured with cross-carrier scheduling from sSCell to P(S)Cell, , for A-/SP-CSI report on PUSCH on P(S)Cell, the UE refers to the CSI reporting related parameters provided in CSI-MeasConfig of P(S)Cell if the DCI that triggers/activates the A-/SP-CSI on PUSCH on P(S)Cell is received on P(S)Cell, and refers to those provided in CSI-MeasConfig of sSCell if the DCI that triggers/activates the A-/SP-CSI on PUSCH on P(S)Cell is received on sSCell. Network ensures that the parameters in CSI-MeasConfig of P(S)Cell and sSCell are configured such that joint set of these parameters across P(S)Cell and sSCell does not result in exceeding any of the UE’s capabilities for A-/SP-CSI reporting on PUSCH on P(S)Cell, such as:

  - maxNumberAperiodicCSI-triggeringStatePerCC

  - maxNumberAperiodicCSI-PerBWP-ForCSI-Report

  - maxNumberAperiodicCSI-PerBWP-ForBeamReport

  - maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report

  - maxNumberSemiPersistentCSI-PerBWP-ForBeamReport

- Opt.3: when a UE is configured with cross-carrier scheduling from sSCell to P(S)Cell, for A-/SP-CSI report on PUSCH on P(S)Cell, the CSI reporting related parameters provided in CSI-MeasConfig of P(S)Cell and sSCell are identical such that the UE can refer to the parameters provided on either of the P(S)Cell and sSCell for A-/SP-CSI on PUSCH on P(S)Cell.

R1-2211492 [1] proposes the following (related to Proposal1_Alt1 in [5] and Opt 2 in [2])

Proposal 1

- Proposal 1: The proposed Note in RAN1 #110bis Proposal 1-Alt1 is interpreted as: The sum (or the union) of what results from the CSI-MeasConfig for P(S)Cell and what results from the CSI-MeasConfig for sSCell does not exceed any of the UE’s capabilities for A-/SP-CSI reporting on PUSCH on P(S)Cell.

Proposal 1

- Proposal 1: The proposed Note in RAN1 #110bis Proposal 1-Alt1 is interpreted as: The sum (or the union) of what results from the CSI-MeasConfig for P(S)Cell and what results from the CSI-MeasConfig for sSCell does not exceed any of the UE’s capabilities for A-/SP-CSI reporting on PUSCH on P(S)Cell.

R1-2212162 [3] proposes the following (also related to Proposal1_Alt1 in [5] and Opt 2 in [2])

Proposal 1

- Clarify capability description for FG 34-1 and 34-2 by including below additional text as a Note

  - “parameters in CSI-MeasConfig of P(S)Cell and sSCell are configured such that joint set of these parameters across P(S)Cell and sSCell does not result in exceeding any of the UE’s capabilities for A-/SP-CSI reporting on PUSCH on P(S)Cell”

Proposal 1

- Clarify capability description for FG 34-1 and 34-2 by including below additional text as a Note

  - “parameters in CSI-MeasConfig of P(S)Cell and sSCell are configured such that joint set of these parameters across P(S)Cell and sSCell does not result in exceeding any of the UE’s capabilities for A-/SP-CSI reporting on PUSCH on P(S)Cell”

Companies are requested to provide comments the above proposals

| Company Name | Comments |
| --- | --- |
| ZTE | To follow our comment in RAN1#110bis-e meeting, we would like to clarify the meaning of “joint set of these parameters”. Take the “maxNumberAperiodicCSI-triggeringStatePerCC” as an example, if network configures 3 triggering states for PCell and 3 triggering states for sSCell, if one triggering state for PCell is the same as one triggering state for sSCell, then which of the followings is the correct understanding? Understanding#1: joint set of these parameters = 3 triggering states; Understanding#2: joint set of these parameters = 6 triggering states; Understanding#3: joint set of these parameters = 5 triggering states since one triggering state for PCell is the same as one triggering state for sSCell ; The above understandings need to be clarified first. For the above example, if UE indicates maxNumberAperiodicCSI-triggeringStatePerCC as 3 for PCell and 3 for sSCell, then the above example should be considered as “not exceeding UE’s capability”. |
| Nokia, NSB | On QC proposal: Isn’t this question relevant to any cross-carrier scheduling as known since Rel-15 rather than being specific to SCell scheduling PCell? Our understanding of the behaviour is according to Opt1, the location of the triggering DCI is of no interest. |
| vivo | We support the proposal, i.e., option 2 in QC’s proposal. Agree that ZTE’s question needs to be clarified. We prefer Understanding#3 as ZTE describes. |
| Qualcomm | Same as the last meeting, we are OK with the FL proposal. @ZTE: Our understanding is #2. For non-CA case, no matter whether there are multiple trigger states that have identical trigger state configurations, each trigger state is counted as one. For example, if the UE reports maxNumberAperiodicCSI-triggeringStatePerCC = 3, then the UE is not supposed to be configured with 128 trigger states with 126 trigger states having identical configurations. Same for Rel-17 DSS, our understanding is that it is #2. @ Nokia: for A-CSI/SP-CSI on PUSCH, the relevant higher-layer parameters in CSI-MeasConfig are based on the scheduling cell. Opt.1 requires a special handling for Rel-17 DSS scenario and hence would require spec change in 38.331. |
| Intel | One clarification question based on ZTE’s example and understanding #2. Which value of CSI request should be used in the DCI on PCell and sSCell? One possibility is that CSI request = 1, 2, 3 for DCI on PCell, and CSI request = 4, 5,6 for DCI on sSCell. The other way is to use CSI request = 1, 2, 3 in both DCIs. Further, if it is understanding #5, do we need to obtain a numbering of 1, 2,3, 4, 5 for the 6 CSI states on two DCIs, or still use CSI request = 1, 2, 3 in both DCIs? |
| Moderator Notes1 | Suggest considering below proposal Proposal 1 - Clarify capability description for FG 34-1 and 34-2 by including following Note - Note: Parameters in CSI-MeasConfig of P(S)Cell and sSCell are configured such that [joint set of these parameters across P(S)Cell and sSCell does not]/ [combination of P(S)Cell and sSCell configurations does not] result in exceeding any of the UE’s capabilities for A-/SP-CSI reporting on PUSCH on P(S)Cell @ZTE/vivo/QC/Intel -- Can try blue highlighted text (i.e., instead of yellow part) to further clarify the intention. @Intel – Opt.2 is related to capability discussion. There is no impact to how trigger indexing is handled compared to current specs. |

# 3	Conclusion

TBD

# 4	References

- R1-2211492, Remaining issue in Rel-17 SCell-to-PCell scheduling, OPPO, RAN1#111, Nov 2022, Toulouse, France.

- R1-2212096, Clarification for cross-carrier scheduling from SCell to P(S)Cell, Qualcomm Incorporated, RAN1#111, Nov 2022, Toulouse, France.

- R1-2212162, Clarifications for Cross-carrier scheduling from SCell to P(S)Cell, Ericsson, RAN1#111, Nov 2022, Toulouse, France.

- R1-2209962, Discussion on clarification for cross-carrier scheduling from SCell to P(S)Cell, Qualcomm Incorporated, RAN1#110bis-e, Oct 2022.

- R1-2210780, Summary#3 of Email discussion [110bis-e-R17-DSS-01]	Moderator (Ericsson), RAN1#110bis-e, Oct 2022.