3GPP TSG RAN WG1 #111			     R1-2212865

Toulouse, France, November 14th – 18th, 2022

Source:	Moderator (NTT DOCOMO, INC.)

Title:	Summary #5 on 9.11.1 Coverage enhancement for NR NTN

Agenda Item:	9.11.1

Document for: 	Discussion and Decision

- Introduction

This document is made for discussion on coverage enhancement for NR NTN. Schedule for discussion is below in UTC time. FL requests companies to consider the schedule.

- 1st offline session: 14:30 – 17:45 on Monday

- 1st online session: 11:00 – 12:00 on Tuesday

- 2nd offline session: 16:30 – 17:45 on Wednesday

- 2nd online session: 8:00 – 10:30 on Thursday

- 3rd offline session: 11:00 – 12:00 on Thursday

- 3rd online session: 11:00 – 12:00 on Friday

This topic is mentioned in Rel-18 NR NTN WID as captured in Appendix-1. As discussed and concluded at the last RAN plenary meeting, we focus on coverage enhancement of PUCCH for Msg4 HARQ-ACK and discussion of DMRS-bundling for PUSCH. Although FL found that several companies propose other mechanisms in their contributions as summarized in section 5.3, they will not be handled since not aligned with the WID description.

In this meeting, FL’s plan is to agree at least the following aspects.

- For PUCCH of Msg4 HARQ-ACK

  - High-level procedure – down-selection

  - Signaling for request/capability report – option list or down-selection

  - Configuration/indication details – option list or down-selection

- For PUSCH DMRS-bundling

  - Phase rotation issue – RAN1 assumption and an LS to RAN4

  - High-level concept – option list or down-selection

In the end, we have agreed below:

- For PUCCH of Msg4 HARQ-ACK

  - High-level procedure – compromised one

- For PUSCH DMRS bundling

  - Constant frequency error and phase rotation issue

  - Conclusion of necessity of DMRS bundling

FL assumes that at least the following should be discussed in the next meeting while plan may be changed after further consideration.

- For PUCCH of Msg4 HARQ-ACK

  - For semi-static: request/capability report

  - For dynamic: request/capability report, dynamic indication details, etc.

  - FH

- For PUSCH DMRS bundling

  - What is expected UE behavior within a TDW in NTN

  - Required TDW length and maximum available length

  - Listing options of what RAN1 need to do for DMRS bundling in NTN

In addition, ‘contact information’ in the last section is copied from the summary at the last meeting. Anyone can use/add/update/remove some of the list if necessary.

- Collections of agreements/conclusions in RAN1#111

Conclusion

For the study of NTN-specific PUSCH DMRS bundling, RAN1’s understanding is that Phase variation due to constant frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1 does not have impact on the phase continuity requirement for two adjacent slots specified as Table 6.4.2.5-1 in 38.101-1, according to annex F.9 and F.4 of 38.101-1.

Conclusion

RAN1 concluded that PUSCH DMRS bundling with sufficient TDW size should be applicable in NTN to meet the performance requirement for VoIP

- FFS: How to determine TDW size, including UE capability.

- Note: The above does not mean the performance requirements will be satisfied with DMRS bundling

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK,

- One or more repetition factors may be configured via SIB

  - If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor

    - FFS: whether UE requests repetition or indicates repetition capability

  - If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB

    - FFS: whether UE requests repetition or indicates repetition capability

    - FFS: whether repetition factor is indicated by UE

  - FFS: UE behavior when repetition factor is not configured via SIB

  - FFS: whether one or more UE capabilities are needed for the above is for further discussion

- Proposals for agreements/conclusions

For PUCCH repetition

Proposal 1-2_v2

[Repetition request or Repetition capability report] of PUCCH for Msg4 HARQ-ACK is transmitted by:

- Option A: PRACH preamble and/or occasion

  - FFS: relationship with Msg3 repetition request

  - FFS: whether/how to avoid further fragmentation of PRACH resources

- Option B: Msg3 PUSCH

  - FFS: how to transmit the information (e.g., higher layer signaling, scrambling, DMRS port, etc.)

- Option C: PUCCH for Msg4 HARQ-ACK

  - FFS: how to transmit the information

Proposal 1-4_v2

For dynamic indication of PUCCH repetition for Msg4 HARQ-ACK,

- Maximum N value(s) of repetition factor can be candidates of dynamic indication.

  - Alt 1: N = 1

  - Alt 2: N = 2

  - Alt 3: N = 4

  - FFS: configuration per cell or per beam or per PUCCH resource included in a PUCCH resource set provided by pucch-ResourceCommon

- PUCCH repetition factor or whether repetition is performed is indicated by:

  - Alt A: some of the existing fields in DCI scheduling the Msg4 PDSCH

    - FFS: which field (e.g., MCS information field)

  - Alt B: MCS information field of DCI scheduling Msg3 PUSCH jointly with indication of Msg3 repetition factor

    - FFS: details on the joint indication

- A configured PUCCH resource set is included in:

  - Alt X: Table 9.2.1-1 of TS 38.213

    - Note: no spec change is assumed

  - Alt Y: A newly defined table for PUCCH repetition for Msg4 HARQ-ACK

    - FFS: details

Proposal 1-5_v1

For PUCCH repetition for Msg4 HARQ-ACK,

- Further study the following options.

  - Option X: Apply intra-slot frequency hopping as currently specified by extending applicability to repetitions

  - Option Y: Apply inter-slot frequency hopping

  - Option Z: no frequency hopping in case of repetitions

For PUSCH DMRS bundling

Proposal 2-4_v1

- For NTN-specific PUSCH DMRS bundling,

  - UE shall not perform pre-compensation update of TA if it causes phase discontinuity within an actual or nominal TDW, which may violate RAN4 requirement.

  - UE may perform pre-compensation update of TA if it does not cause phase discontinuity within an actual or nominal TDW.

Proposal 2-2_v6

For NTN-specific PUSCH DMRS bundling,

- Further study the following options of applying time/frequency compensation update with DMRS bundling

  - Option 1: NTN-specific configuration/indication is provided by gNB and actual/nominal TDW is determined based on the configuration/indication.

  - Option 2: UE reports pre-compensation timing and actual/nominal TDW is determined based on the report.

  - Option 3: Actual/Nominal TDW is determined as in the existing specification

    - FFS: whether/how to define new UE capability report

- Discussion

As in the last meeting, FL recommends companies to use the following values for discussion in this meeting if any and for future evaluations. Still FL found that some companies are using different values to make observations/proposals. Observations/proposals may be changed dependent on the applied CNR value; thus different values should not be chosen among companies.

| Case | Satellite orbit | Satellite parameter set | Elevation angle [degree] | Frequency [GHz] | UE antenna gain [dBi] | TX: EIRP [dBm] | RX: G/T [dB/T] | No. of PRBs | Bandwidth [MHz] | Free space path loss [dB] | Atmospheric loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | LEO-1200 | 1 | 30 | 2.0 | -5.5 | 18.0 | 1.1 | 1 | 0.18 | 164.5 | 0.1 | 3.0 | 2.2 | 3.0 | 0.0 | -8.1 |
|  |  |  |  |  |  |  |  | 2 | 0.36 |  |  |  |  |  |  | -11.1 |
|  |  |  |  |  |  |  |  | 3 | 0.54 |  |  |  |  |  |  | -12.9 |
|  |  |  |  |  |  |  |  | 4 | 0.72 |  |  |  |  |  |  | -14.1 |
|  |  |  |  |  |  |  |  | 6 | 1.08 |  |  |  |  |  |  | -15.9 |

  - PUCCH enhancements for Msg4 HARQ-ACK

    - [Open/High] High-level procedure

Although possible high-level procedure to perform PUCCH repetition for Msg4 HARQ-ACK has been agreed with 3 options, FL found from companies’ contributions that interpretation of each option is not aligned. For example, some companies supporting option 1 are proposing indication of whether to perform repetition. FL would like to clarify the original intention (at least in FL’s mind) as follows:

Option 1: gNB configures a repetition factor in cell-specific manner. Any UE capable of Msg4 HARQ-ACK PUCCH repetition shall perform repetition with the configured repetition factor. How to handle UE incapable of Msg4 HARQ-ACK PUCCH repetition is FFS.

Option 2: UE capable of Msg4 HARQ-ACK PUCCH repetition indicates whether repetition is necessary or not; that is, UE capable of Msg4 HARQ-ACK PUCCH repetition may not request repetition. gNB can indicate Msg4 HARQ-ACK PUCCH repetition but it is up to gNB implementation. Whether single value or multiple values is configured is FFS. Note that this option is the same way as R17 Msg3 repetition.

Option 3: UE capable of Msg4 HARQ-ACK PUCCH repetition reports whether repetition is applicable or not; that is, UE capable of Msg4 HARQ-ACK PUCCH repetition will send the information as ‘capable’ regardless of whether PUCCH repetition is necessary or not. gNB can indicate Msg4 HARQ-ACK PUCCH repetition but it is up to gNB implementation. Whether single value or multiple values is configured is FFS.

On top of this understanding, FL observed the following supporters for each option:

Option 1: 5 companies [1/HW, HiSi (w/ modification)] [3/MTK] [9/xiaomi] [19/LGE] [23/Baicells]

Option 2: 15 companies [2/vivo (?)] [4/ZTE] [6/CATT] [7/Spreadtrum] [9/xiaomi] [11/OPPO] [25/Nokia, NSB] [12/ETRI (?)] [13/Pana] [15/CMCC (?)] [16/NEC] [17/Apple] [18/Lenovo] [20/DCM] [21/Samsung (?)]

Option 3: 5 companies [7/Spreadtrum] [10/Intel] [22/QC] [24/Ericsson] [25/Nokia, NSB]

A lot of companies supporting Option 2/3 argues that cell-specific concept is too wasted and inflexible since channel quality, antenna type, UE power class, etc. are different among UEs (cyan-highlighted part in section 5.1). FL recommends selecting either Option 2 or Option 3 based on the above argument and the fact of more supporters.

Regarding Option 2 vs Option 3 (yellow-highlighted part in section 5.1), there is an argument that ‘request’ includes capability report. In addition, a lot of companies supporting Option 2 prefer to reuse the same mechanism of R17 Msg3 repetition. Meanwhile, some companies proposing Option 3 explain that gNB can perform UL measurement e.g. on Msg3 repetition, which is more reliable. Essentiality of changing rule from R17 Msg3 repetition has not been discussed sufficiently yet; thus FL would like to recommend discussing this aspect further.

FL found that why UE measurement is used in R17 Msg3 repetition is that UE transmission power of PRACH is unknown at gNB side. UE transmission power of Msg3 PUSCH would also be unknown at gNB side, thus UL measurement at gNB side may not be appropriate.

Proposal 1-1_v0

For PUCCH repetition for Msg4 HARQ-ACK,

- UE [requests repetition or indicates repetition capability] and is dynamically indicated to perform repetition and/or repetition factor.

Q: Do you agree the above proposal? If YES, which should be agreed: request or capability report?

| Company | YES/NO | Request / Cap report | Comment |
| --- | --- | --- | --- |
| Lenovo | Yes | Request | We think capability may be related to UE capability and it may be transmitted in RRC connected state, and the UE request can indicate repetition number to be 1 or larger than 1, which can also cover the case without UE repetition. |
| Apple | Yes | Both | UE requests repetition if it has the capability and it detects the need of PUCCH repetition. If UE has the capability but does not detect the need of PUCCH repetition, then it does not request; If UE does not have the capability but detects the need of PUCCH repetition, then it also does not request repetition. |
| Panasonic | Yes | Request | UE request repetition if UE is capable of repetition and the need of the repetition. The request includes an information on the propagation condition. gNB can use UL measurement results based on PRACH and/or Msg3 by its implementation if needed. |
| Xiaomi | Yes | Request |  |
| ZTE | Yes | Request |  |
| QC | Yes | Both | We think both are fine. For Msg 4 HARQ-ACK, gNB should already have the idea if repetition is needed. |
| OPPO | YES | Request | PUCCH repetition request takes both repetition capability and coverage performance into account, which is also aligned with R17 Msg3 PUSCH repetition. |
| Baicells |  |  | Need options for cell specific configuration. |
| Sharp | Yes | Request | We could reuse msg3 repetition design principle. |
| NTT DOCOMO | Yes | Request | Similar mechanism to Msg3 repetition can be used. |
| Huawei, HiSilicon | No | Request repetition | It is OK to request the repetition, which we think meanwhile the capability is also carried. However, for dynamic indication of PUCCH repetition for Msg4 HARQ-ACK, we don’t think it is necessary. Some justification to use dynamic indication is to save the resource. However, within the same beam of a satellite, the CNR value difference would be small. Therefore, we think using option1 to configure beam specific repetition number is sufficient. Compared with significant change on the specification to support option2, we prefer to go the direction of option1. |
| Ericsson | Yes | Cap report | Regarding the FL comment "UE transmission power of Msg3 PUSCH would also be unknown at gNB side, thus UL measurement at gNB side may not be appropriate.": If the received power of Msg3 is low, it can be due to that the UE is power-limited, or due to that it has down-regulated TX power due to an under-estimated PL. Either way, it could be assumed that the UE will be in the same situation when transmitting Msg4 HARQ-ACK, so the need for repetitions if Msg4 HARQ-ACK could be determined based on measurements on Msg3, even if the absolute TX power level is not known. Further, if UE requests repetition, it seems difficult/costly to find more than one bit for this, so the repetition factor to use still needs to be determined by the gNB by other means, e.g., UL measurements on Msg3. There are pros and cons of both options: Request: UE indicates its view on the need for repetitions, which can be useful info for the gNB. But if UE underestimates the need for UL repetitions, gNB does not know the UE capability and thus cannot correct the mistake based on other info (e.g. UL measurements). Cap report: gNB always knows whether the UE is capable of repetitions but has no info about the UEs view on the need for repetitions. |
| CMCC | Yes |  | We are open to discuss the two options since both of them can work and requesting repetitions is aligned with Rel-17 CE. |
| ETRI | YES | Request | We prefer option 2, but option 1 is also fine. |
| Nokia, Nokia Shanghai Bell | Yes | Capability reporting | gNB would have better knowledge of the expected or potential coverage issues for the PUCCH, so we think that it is sufficient that gNB knows the capability of the UE. |
| LG | Yes | Request |  |

      - 1st offline

YES: 14

NO: 1

Request: 12

Cap report: 4

2 companies supporting ‘Cap report’ are also fine with ‘Request’

Proposal 1-1_v1

For PUCCH repetition for Msg4 HARQ-ACK,

- UE capable of PUCCH repetition for Msg4 HARQ-ACK can [request repetition or indicate repetition capability] and may dynamically be indicated to perform repetition and/or repetition factor.

      - 2nd input

Proposal 1-1_v1 with online update

For PUCCH repetition for Msg4 HARQ-ACK,

- UE capable of PUCCH repetition for Msg4 HARQ-ACK can be dynamically indicated to perform repetition and/or repetition factor.

  - FFS: SIB-based RRC configuration to perform repetition and/or to configure repetition factor is also supported

  - FFS: whether UE requests repetition or indicates repetition capability

Based on the current situation, FL tries to update this proposal with compromise. Whether semi-static configuration is used or not is decided according to the configured number of repetition factors. This does not have any negative impact on dynamic indication.

In FL’s understanding, QC/vivo did not agree with compromised way. At least QC is OK with this direction while vivo is not OK.

Proposal 1-1_v2

For PUCCH repetition for Msg4 HARQ-ACK,

- UE capable of PUCCH repetition for Msg4 HARQ-ACK can be dynamically indicated to perform repetition and/or repetition factor.

  - If only one repetition factor is configured via SIB and if the value is one of {2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK always performs repetition with the repetition factor

  - If multiple factors are configured via SIB, dynamic indication is used

  - FFS: whether UE requests repetition or indicates repetition capability

Q: Do you agree the above proposal (v2)?

| Company | YES/NO | Comment |
| --- | --- | --- |
| MediaTek | NO | The revised proposal now only mention dynamic indication. It was discussed online that RRC configuration should also be supported. We think the proposal discussed in GTW could be revised with 1st sub-bullet moved as a sub-bullet “FFS” removed as below For PUCCH repetition for Msg4 HARQ-ACK, - UE capable of PUCCH repetition for Msg4 HARQ-ACK can be dynamically indicated to perform repetition and/or repetition factor. - FFS: whether UE requests repetition or indicates repetition capability - FFS SIB-based RRC configuration to perform repetition and/or to configure repetition factor is also supported - FFS Details |
| NEC |  | Our understanding of the first sub bullet of revised proposal is that the UE capable of PUCCH repetition for Msg4 HARQ-ACK should always perform repetition if only one repetition factor is configured via SIB and thus FFS part in third sub bullet ‘whether UE requests repetition’ is rendered irrelevant for this case. Can FL please confirm if our understanding is correct? |
| Ericsson | Yes | On MTK's comment, our understanding is that the first sub-bullet of proposal 1-1_v2 describes the SIB-based RRC configuration case. |
| OPPO |  | We also have concerns on the 1st sub-bullet. The UE always performing repetition regardless of UL coverage performance will result in unnecessary waste of resource. In our understanding, the repetition factor configured via SIB should only apply to the UE which requests PUCCH repetition to the gNB. However, considering whether UE requests repetition or indicates repetition capability is FFS, we suggest to modify the 1st sub-bullet as “If only one repetition factor is configured via SIB and if the value is one of {2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK always can perform repetition with the repetition factor” |
| NTT DOCOMO | Yes | We think this is an appropriate compromise since both semi-static and dynamic indication are possible depending on the configuration. |
| Lenovo |  | We are generally fine with the intention to address the contradiction issue of RRC configuration and dynamic indication. We don’t agree with MTK’s version as it doesn’t mention how to deal with the case when the indication by dynamic signaling and RRC configuration are different. We think the UE performs repetition based on configuration is only applicable when it indicates repetition request. |
| LGq | Yes | We are fine to have both SIB-based indication and dynamic indication. If gNB configure SIB-based repetition number for all of capable UE based on its decision, we think there is no reason to prohibit that operation. It would be harder to optimize the number of repetitions for each UE and signal them individually. |
| ZTE | No | In our view, UE is in better knowledge of whether and how many repetitions are needed. Hence, if multiple factors are configured via SIB, UE may directly report the requested repetition factor and the factor is applied in PUCCH repetition. In this case, the dynamic selection of repetition factor is determined by UE and reported to network, instead of dynamically indicated from network to UE. Moreover, even if only one repetition factor is configured, UE capable of PUCCH repetition may not want to repeat since it may have good channel condition. Hence, “UE request” is more preferred than “UE capable” in subbullet 1. Hence, following updates are recommended: Proposal 1-1_v2 For PUCCH repetition for Msg4 HARQ-ACK, - UE capable of PUCCH repetition for Msg4 HARQ-ACK can be dynamically indicated to perform repetition and/or repetition factor. - If only one repetition factor is configured via SIB and if the value is one of {2, 4, 8}, UE capable request of PUCCH repetition for Msg4 HARQ-ACK always performs repetition with the repetition factor - If multiple factors are configured via SIB, dynamic request or dynamic indication is used - FFS: whether UE requests repetition or indicates repetition capability |
| FL |  | To MTK As commented by Ericsson, the first bullet is for SIB-based RRC configuration, so I assume this is a compromise based on the yesterday’s online discussion. To OPPO/NEC Let’s keep whether ‘can’ or ‘always’ open. To all, Now I updated a bit. Please share comment for this version. Proposal 1-1_v3 For PUCCH repetition for Msg4 HARQ-ACK, - UE capable of PUCCH repetition for Msg4 HARQ-ACK can be dynamically indicated to perform repetition and/or repetition factor. - If only one repetition factor is configured via SIB and if the value is one of {2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor - FFS: whether the UE always performs repetition or not - If multiple factors are configured via SIB, dynamic indication is used - FFS: whether UE requests repetition or indicates repetition capability |
| Xiaomi | Yes | We support this proposal. From our understanding, this proposal supports both RRC configuration based repetition and dynamic repetition indication among multiple RRC configured repetition factors, which is a compromise between RRC based repetition and dynamic repetition indication, |
| Samsung |  | For proposal 1-1_v3, it is unclear to us why first FFS is needed because first-sub-sub bullet has no mechanism of (explicitly and implicitly) dynamic indication in our understanding. |
| Nokia, Nokia Shanghai Bell | Yes with comment | From network side, it is crucial that we know whether or not the UE is able to perform repetitions since this is related to the uplink resources that the network will have to pre-reserve for the UL (PUCCH) transmission, at least in the case multiple repetition factors are configured via SIB. As stated during the online session, there is only one entity in the network that is able to determine whether or not there is a need for repetitions, and that is the gNB. Hence, the gNB would need to know the UE’s capability prior to applying dynamic indication of amount of PUCCH repetitions. With respect to the potential update, we are a bit confused. We are in the case where the network has configured the repetition to be available, the network has determined that repetitions are needed, and UE is capable of performing the repetitions. For which reasons would it make sense to leave it to the UE to determine whether or not to actually do the repetitions? |
| Panasonic | No | We are ok to support both dynamic indication and RRC configuration as a compromise. But, we prefer the formulation like the following. - UE capable of PUCCH repetition for Msg4 HARQ-ACK can be dynamically indicated to perform repetition and/or repetition factor. - SIB based RRC configuration can disable the dynamic indication of PUCCH repetition for Msg4 HARQ-ACK. If disabled, repetition factor configured by SIB is used. FFS: whether UE requests repetition or indicates repetition capability As discussed in Proposal 1-4_v1, if N=4, we don’t need RRC configuration for repetition factors which can be dynamically indicated. |
| Baicells |  | We support both dynamic indication and RRC/SIB configuration. We prefer to delete ‘dynamic’ in the main bullet: UE capable of PUCCH repetition for Msg4 HARQ-ACK can be dynamically indicated to perform repetition and/or repetition factor. ... ... |
| ETRI |  | Same opinion with LG and Baicells. We support both cell SIB-based and dynamic indications, and prefer to delete ‘dynamic’ in the main bullet. |
| Spreadtrum | Yes | We are fine with the updated Proposal 1-1_v3. |

      - 2nd offline (1)

Proposal 1-1_v4

For PUCCH repetition for Msg4 HARQ-ACK,

- Zero or one or more repetition factors can be configured via SIB

  - If only one repetition factor is configured via SIB and if the value is one of {2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor without dynamic indication from gNB

    - FFS: whether UE requests repetition or indicates repetition capability

  - If zero or multiple factors are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically indicated

    - FFS: whether UE requests repetition or indicates repetition capability

    - [Open/High] Signaling for request or capability report

If dynamic indication of performing repetition is supported,

How to send the request or the capability needs to be solved. The following three solutions are proposed:

PRACH preamble/occasion: 9 companies [1/HW, HiSi] [2/vivo] [6/CATT] [9/xiaomi] [12/ETRI] [13/Pana] [15/CMCC] [17/Apple] [18/Lenovo]

Msg3 PUSCH: 7 companies [4/ZTE] [9/xiaomi] [18/Lenovo] [20/DCM] [22/QC] [24/Ericsson] [25/Nokia, NSB]

PUCCH for Msg4 HARQ-ACK: [25/Nokia, NSB]

The first solution is aligned with R17 Msg3 repetition while the second solution, i.e., Msg3 PUSCH, can be the candidate for PUCCH repetition of Msg4 HARQ-ACK since this PUCCH is transmitted after Msg3 PUSCH. Considering most companies are proposing either the first or the second solution, FL recommends this proposal below. Note that several detailed aspects are discussed in contributions; they are listed as FFS for better information.

Proposal 1-2_v0

[Repetition request or Repetition capability report] of PUCCH for Msg4 HARQ-ACK is transmitted by:

- Option A: PRACH preamble and/or occasion

  - FFS: relationship with Msg3 repetition request

- Option B: Msg3 PUSCH

  - FFS: how to transmit the information (e.g., higher layer signaling, scrambling, DMRS port, etc.)

Q: If proposal 1-1 is agreed, which option should be agreed? Also please share the reason.

| Company | Option | Comment |
| --- | --- | --- |
| Lenovo | Option B | There are RACH resource segmentation for different purposes. We want to avoid additional segmentation. |
| Apple | Option A | We could simply follow the Rel-17 Msg3 repetition design. For Option B, the motivation why the trigger repetition request and/or repetition capability report is as late as in Msg 3 PUSCH is unclear. Here, UE knows its capability of PUCCH repetition ahead of time and SSB RSRP measurement is performed before PRACH transmission (i.e., Proposal 1-3). Also, we do not think high layer signaling of Msg3 should be changed here. The CRC scrambling has some restrictions and is not preferred. In polar decoding, CRC based listed decoding is used. Note that only the last 17 CRC bits are not distributed in the polar encoding. Hence, only the last 17 CRC bits can be used for scrambling purpose. |
| Panasonic | A | We prefer similar mechanism as Msg3 repetition rather than introducing a new mechanism using Msg3 PUSCH. |
| Xiaomi | B | If option A is adopted and two different RSRP thresholds are defined for Msg4 HARQ-ACK repetition request and Msg3 repetition request, the PRACH resource would be divided as set of Msg4 HARQ-ACK & Msg3 repetition request, set of non-Msg4 HARQ-ACK & Msg3 repetition request, set of Msg4 HARQ-ACK & non-Msg3 repetition request, set of non-Msg4 HARQ-ACK & non-Msg3 repetition request. Considering the PRACH resource is limited, option B is preferred. |
| ZTE | B | With option B, UE can directly report the request repetition number through higher layer signaling. Then network may send a confirmation information to UE and follow the request of UE, or just follow the UE request and not send any signaling. The signaling procedure can be simplified in this way. And UE is better in control of preferred repetition number. |
| QC | B | Additional PRACH resource partition will result collision issue. |
| Baicells | Option A | Capability report can share with Msg3. |
| Sharp | B | More efficient than PRACH due to no RACH partitioning. |
| NTT DOCOMO | B | Option A would further fragment PUCCH resources. This might result in higher RACH collision probability. To keep the collision probability low, gNB may have to reserve more UL resources for PRACH. Option B would be more appropriate in terms of resource efficiency. |

| Huawei, Hisilicon | A | We prefer to use the mechanism for Msg3 repetition in Rel-17. By the way, Option B description seems a little bit unclear for us. Whether it includes the one proposed by some companies that no specification is needed and gNB implementation can evaluate whether repetition of PUCCH is needed. For some other option B solutions, it would have big impact on gNB e.g. blind decoding by assuming different scrambling, DMRS ports etc. This is not preferred. |
| --- | --- | --- |
| Ericsson | B | For R17 Msg3 repetition, Option B was not possible for obvious reasons but for R18 Msg4 HARQ-ACK, Option B is better since it avoids further fragmentation of PRACH resources. |
| CMCC | Option A | If UE indicates repetition capability in Msg3, then gNB send the repetition factor (if any) in Msg4. But if UE cannot successfully decode the Msg4, repetition factor is missed, and UE have no idea about how to send this NACK message. Thus, we prefer Option A. |
| ETRI | A | We prefer Option A because of small impact on specification. |
| Nokia, Nokia Shanghai Bell | B | Further PRACH segmentation should be avoided. In addition, using the Msg3 repetition request for requesting PUCCH repetition would be too limiting because the PUCCH for Msg4 HARQ-ACK and Msg3 PUSCH have completely different coverage requirements. We would also like to ask FL to add an Option C as: - Option C: PUCCH for Msg4 HARQ-ACK - FFS: how to transmit the information As explained in our contribution, both Msg1 and Msg3 have limitations for conveying capability of PUCCH repetitions, so that we would at least like to hear other companies’ views on reporting such capability via the same PUCCH for Msg4 HARQ-ACK. |
| LG | A | We prefer to support similar mechanism as Msg3 PUSCH repetition. |

      - 1st offline

A: 7

Follow the Rel-17 Msg3 repetition design

B: 8

Avoid additional segmentation

C (new): 1

Proposal 1-2_v1

[Repetition request or Repetition capability report] of PUCCH for Msg4 HARQ-ACK is transmitted by:

- Option A: PRACH preamble and/or occasion

  - FFS: relationship with Msg3 repetition request

  - FFS: whether/how to avoid further fragmentation of PRACH resources

- Option B: Msg3 PUSCH

  - FFS: how to transmit the information (e.g., higher layer signaling, scrambling, DMRS port, etc.)

- Option C: PUCCH for Msg4 HARQ-ACK

  - FFS: how to transmit the information

      - 2nd input

Now the number of supporters of Option A and Option B is almost the same. One critical point of Option A is further fragmentation of PRACH resources, as commented by several companies.

For now, FL recommends just listing possible options with the above point as FFS, and further analysis can be done for the next meeting. On option C, this is suggested by Nokia, but FL is not sure this option is feasible; please share your view on this option as well.

Proposal 1-2_v1

[Repetition request or Repetition capability report] of PUCCH for Msg4 HARQ-ACK is transmitted by:

- Option A: PRACH preamble and/or occasion

  - FFS: relationship with Msg3 repetition request

  - FFS: whether/how to avoid further fragmentation of PRACH resources

- Option B: Msg3 PUSCH

  - FFS: how to transmit the information (e.g., higher layer signaling, scrambling, DMRS port, etc.)

- Option C: PUCCH for Msg4 HARQ-ACK

  - FFS: how to transmit the information

Q: Do you agree the above proposal?

| Company | YES/NO | Comment |
| --- | --- | --- |
| MediaTek | NO | Wait for 4.1.2.2 discussion as currently no agreement on dynamic indication. It’s not clear if repetition request or repetition capability report is needed if dynamic indication is not supported. |
| NEC | Yes | Whether dynamic indication is agreed or not, we think it can be useful for gNB to know if a UE is capable of PUCCH repetition or not. |
| Ericsson | Partially | We are not sure what option C means. How does the UE request repetition of PUCCH for Msg4 HARQ-ACK in the PUCCH for Msg4 HARQ-ACK? We do not support inclusion of option C unless this is clarified. |
| OPPO | Yes | Whether for SIB-based RRC configuration or dynamic indication, request or capability report is necessary to ensure the gNB and UE have common understanding, which is supported by most of the companies in the 1st round discussion in Section 4.1.1. |
| NTT DOCOMO | Partially | We doubt that option C is feasible. |
| Lenovo |  | We are not clear about Option C. In our understanding, the repetition request or repetition capability report should be transmitted before PUCCH for msg4 HARQ-ACK, so that gNB can configure suitable repetition factor. We don’t know how Option C can work properly. |
| LG | Yes in principle. | We think Option C is not aligned with current RACH procedure. |
| ZTE | Yes |  |
| Xiaomi | No | How option C works is unclear to us. For Option A, the 1st FFS seems to be one solution of the 2nd FFS, not sure whether the 1st FFS is still necessary with the 2nd FFS added. |
| Samsung | No | Option C should be removed. Now, we are talking about PUCCH coverage issue and how to solve this issue. How option C can be workable in this issue? For Option B, we would like to understand how Msg.3 PUSCH can provide capability information to NW because different companies seems to have different details. Thus, before down-selecting option, details of option B should be clarified with one solution in order to check pros and cons. |
| Nokia, Nokia Shanghai Bell | Yes with comments | For the sake of progress we are OK to list the options. With respect to Option A we have strong concerns related to the PRACH fragmentation – especially considering the large cells that will naturally be created due to the satellite height above earth. |
| Panasonic | Partially | Feasibility of option C is not clear. Blind detection by gNB might be needed and this is not preferred. |
| CMCC | Partially | We are fine to further study Option A and B, but how Option C can be used for repetition request or Repetition capability report is not clear to us. |
| Baicells | Partially | Feasibility of option C is not clear. |
| ETRI | No | We have same concern as Samsung on Option B. Option C can be removed. |
| Spreadtrum | Partially | Feasibility of option C is not clear and Option C can be removed. |

      - 2nd offline (3)

Proposal 1-2_v2

[Repetition request or Repetition capability report] of PUCCH for Msg4 HARQ-ACK is transmitted by:

- Option A: PRACH preamble and/or occasion

  - FFS: relationship with Msg3 repetition request

  - FFS: whether/how to avoid further fragmentation of PRACH resources

- Option B: Msg3 PUSCH

  - FFS: how to transmit the information (e.g., higher layer signaling, scrambling, DMRS port, etc.)

- Option C: PUCCH for Msg4 HARQ-ACK

  - FFS: how to transmit the information

    - [Closed/High] Determination of whether to request

If dynamic indication of performing repetition and repetition request from UE are supported,

A rule on whether to request or not would be necessary as such a mechanism exists in R17 Msg3 repetition. FL observed that 5 companies [1/HW, HiSi] [13/Pana] [17/Apple] [18/Lenovo] [20/DCM] propose to reuse the mechanism in R17 Msg3 repetition. FL suggest agreeing the same mechanism. Besides, 1 company [1/HW, HiSi] propose to define a different threshold from that defined for Msg3 repetition request. It may be better to study this aspect further.

Proposal 1-3_v0

Repetition request of PUCCH for Msg4 HARQ-ACK is transmitted when the following condition is met:

- The RSRP of the downlink pathloss reference is less than a threshold.

  - FFS: relationship with rsrp-ThresholdMsg3

Q: If proposal 1-1 is agreed with repetition request, do you agree this proposal? Also please share the reason.

| Company | YES/NO | Comment |
| --- | --- | --- |
| Lenovo |  | We are fine to support RSRP threshold. Additionally, we also think some satellite specific parameter, such as satellite angle, satellite position can also be considered. |
| Apple | Yes | We could reuse Rel-17 Msg3 repetition design for the trigger of PUCCH repetition for Msg4. |
| Panasonic | Yes | Relationship with rsrp-ThresholdMsg3 and potentially a threshold for PRACH repetition under discussion in AI9.14.1 can be for further study. |
| Xiaomi | Yes |  |
| ZTE | No | The repetition request can be up to UE implementation. In NTN, UE is able to estimate link budget using the satellite ephemeris. Additional definition of RSRP threshold may not be needed. |
| QC | No | gNB should have sufficient information to make decision for Msg4 HARQ-ACK. If UE evaluation is deemed helpful, UE antenna gain should also be considered. |
| OPPO | Yes | Similar to Msg3 PUSCH repetition, the UL coverage performance can be firstly evaluated through the RSRP of the downlink pathloss reference signal. |
| Sharp | Yes | We could reuse msg3 repetition design. |
| NTT DOCOMO | Yes | Since SNR threshold that requires repetition may be different for Msg3 and PUCCH for Msg4 HARQ-ACK, it would be better to introduce a new threshold. |
| Huawei, HiSilicon | Yes | RSRP threshold is enough and in our view a separate threshold can be used for flexibility, and it can be configured the same or different as rsrp-ThresholdMsg3. |
| Ericsson | Yes | In principle yes, but we think proposal 1-1 should be agreed with capability indication. |
| CMCC | Yes | The mechanism for Msg3 repetition can be reused. Considering the different payload of Msg3 and PUCCH for Msg4 HARQ-ACK, a new threshold or offset of the rsrp-ThresholdMsg3 can be considered. |
| ETRI | Yes | The repetition mechanism for Rel-17 Msg3 can be reused. |
| Nokia, Nokia Shanghai Bell |  | As indicated in Proposal 1-1 we do not prefer this option. The need for repetitions is better determined at the gNB side. |
| LG | Yes |  |

      - 1st offline

YES: 11

Reuse Rel-17 Msg3 repetition design

NO: 3

Up to UE implementation, UE antenna gain should also be considered, ‘request’ should not be supported

Proposal 1-3_v0 (no update)

Repetition request of PUCCH for Msg4 HARQ-ACK is transmitted when the following condition is met:

- The RSRP of the downlink pathloss reference is less than a threshold.

  - FFS: relationship with rsrp-ThresholdMsg3

This proposal will be postponed until agreeing either repetition request or capability report.

    - [Open/High] Repetition factor configuration/indication, PUCCH resource set table

If dynamic indication of performing repetition is supported,

Repetition factor configuration/indication mechanism is necessary.

For configuration, the following two options are proposed. FL recommends further discussion on which should be adopted. Besides, configuration granularity is also a discussion point. This aspect is raised by e.g., [1/HW, HiSi] proposing configuration per beam and [21/Samsung] proposing configuration per PUCCH common resource. FL would like to ask companies to consider this further.

Configure only one value: 5 companies [4/ZTE] [11/OPPO] [12/ETRI] [19/LGE] [21/Samsung]

Configure one or multiple values: 3 companies [19/LGE] [21/Samsung] [24/Ericsson]

For indication, there are several proposals. FL recommends focusing on solutions with more supporters unless reasonable justification to consider other option is presented. On PRACH repetition, FL recommends not considering linkage with it since the details of PRACH repetition has not been concluded, unless essentiality is found.

Reuse some field in DCI scheduling Msg4 PDSCH: 10 companies [9/xiaomi] [10/Intel] [12/ETRI] [13/Pana] [18/Lenovo] [19/LGE] [20/DCM] [21/Samsung] [22/QC] [24/Ericsson]

Joint indication with Msg3 repetition indication and/or PRACH repetition indication: 6 companies [10/Intel] [15/CMCC] [16/NEC] [17/Apple] [18/Lenovo] [19/LGE]

Define new field in DCI scheduling Msg4 PDSCH: 1 company [21/Samsung]

Transmit via Msg2: 1 company [15/CMCC]

CRC scrambling of DCI scheduling Msg4: 1 company [25/Nokia, NSB]

Besides, two companies are discussing PUCCH resource set table. It would be better to discuss to take either reusing the existing table defined in 213 or introducing a new table for PUCCH transmission with repetition.

Proposal 1-4_v0

For dynamic indication of PUCCH repetition for Msg4 HARQ-ACK,

- Maximum N value(s) of repetition factor can be configured via SIB.

  - Alt 1: N = 1

  - Alt 2: N = 2

  - Alt 3: N = 4

  - FFS: configuration per cell or per beam or per PUCCH resource included in a PUCCH resource set provided by pucch-ResourceCommon

- PUCCH repetition factor or whether repetition is performed is indicated by:

  - Alt A: some of the existing fields in DCI scheduling the Msg4 PDSCH

    - FFS: which field

  - Alt B: MCS information field of DCI scheduling Msg3 PUSCH jointly with indication of Msg3 repetition factor

    - FFS: details on the joint indication

- A configured PUCCH resource set is included in:

  - Alt X: Table 9.2.1-1 of TS 38.213

    - Note: no spec change is assumed

  - Alt Y: A newly defined table for PUCCH repetition for Msg4 HARQ-ACK

    - FFS: details

Q: If proposal 1-1 is agreed, which alternatives should be agreed? Also please share the reason.

| Company | Alt 1/2/3 | Alt A/B | Alt X/Y | Comment |
| --- | --- | --- | --- | --- |
| Lenovo | Alt 3 | Alt A or B | Alt Y |  |
| Apple | Alt 3 | Both are fine to us | Comment | We suggest considering another alternative to modify Table 9.2.1-1 of TS38.213 with a new column to indicate the number of repetitions. It does not need to be a totally new table, which is different from Alt Y. We are also fine with Alt X. |
| Panasonic | 3 | A | X | Repetition factor 1, 2, 4 and 8 should be indicatable. Existing fields in DCI scheduling the Msg4 PDSCH can be reused (repurposed) by limiting the flexibility of original purpose considering NTN usage, e.g. flexibility of PDSCH-to-HARQ timing indicator (3 bits) would not be necessary for FDD. The motivation to introduce the new table is not clear. |
| Xiaomi | 3 | A | X |  |
| ZTE | 2 | A | X | For bullet 1, since coverage performance is not significantly varied within a cell, indicating too many candidate repetition factors may not be needed. For bullet 2, indication may not be needed if UE directly report the requested repetition number. If a confirmation information on performing repetition is agreed necessary, alt A is more preferred than alt B to decouple with Msg3 repetition. For bullet 3, Alt X is preferred to minimize spec change. |
| QC | 3 | A | X | For bullet 2, the two DAI bits can be used and hence we can allow 4 values (option 3) for bullet 1. |
| OPPO | 1 or 2 | comment | X | For the 1st bullet, we prefer Alt 1 with a cell-specific indication of PUCCH repetition factor, and the UE can apply it after requesting repetition to gNB. In addition, we are also fine with Alt2. For the 2nd bullet, the indication of PUCCH repetition factor should be discussed after the signaling for request (or capability report) is determined in Section 4.1.2 since the Repetition request (or Repetition capability report) of PUCCH for Msg4 HARQ-ACK may be transmitted by Msg3 PUSCH. |
| Baicells | Alt 2/3 | Alt A | Alt X | For Alt A, the existing field can be HARQ process number, Downlink assignment index, for example. |
| Sharp | 3 | B | X | We don’t see motivation to introduce new table. |
| NTT DOCOMO | 3 | A | X | Candidates for the number of repetitions are 1,2,4 and 8, so 2 bits are needed. We think that it is possible to include 2 bits for indicating the number of PUCCH repetitions in the DCI format for scheduling Msg4 by repurposing the existing DCI field, so there would be no need to limit N to less than 4. Considering the standardization effort to define a new table, we think Table 9.2.1-1 should be reused. |
| Huawei, HiSilicon | 1, however there is no need for dynamic indication if only one repetition number is configured. |  |  | We think repetition factor configured via SIB and beam level configuration is sufficient. Generally, the SNR difference of UE within one beam is smaller than 3dB, beam level repetition configuration is sufficient and will not cause large resource waste. |
| Ericsson | 2 or 3 | A | X or Y | Alt 2 or 3 to give the gNB some flexibility to dynamically chose the number of repetitions. Alt A since Alt B assumes that the gNB knows the need for Msg4 HARQ-ACK repetitions prior to reception of Msg3. No strong opinion on X or Y. |
| CMCC | 2/3 | A/B | X | Multiple values of repetition factor configured via SIB can provide more scheduling flexibility. For Alt A, whether 2 or 4 values can be configured may depend on how many bits of existing fields can be repurposed at most. For Alt B, a joint indication for both Msg3 and Msg4 HARQ-ACK repetitions can be considered, which can directly support 2 or 4 values of repetition factor, and we prefer not to modify the existing DCI field. |
| ETRI | 1 or 3 | A | X | For bullet 2, the existing fields in DCI scheduling the Msg4 PDSCH can be reused. |
| Nokia, Nokia Shanghai Bell | 3 | None | X | If the dynamic indication of the repetition factor is agreed to be carried out via other means than a hard-coded table, we do not see the need for Alt Y, as the current PUCCH resource set table offers different possibilities for UE in coverage shortage. For the indication of the PUCCH repetition factor, we think that Alt A may have a backward compatibility problem whereas we do not understand Alt B. - The DCI scheduling the Msg4 PDSCH is scrambled by a TC-RNTI that could be shared by legacy and Rel-18 UEs, so that a repurposing may either confuse the legacy or the Rel-18 UE interpretation of the repurposed DCI field. - Regarding Alt B, the Msg3 repetitions are indicated via the MCS field of the RAR UL grant in Msg2 and not via DCI. But again, using Msg3 repetition factor (or a proportional factor) for Msg4 HARQ-ACK repetitions is greatly limiting. |
| LG | 3 | Both | Both |  |

      - 1st offline

Alt 1: 3

Alt 2: 5

Alt 3: 12

N is max value

Alt A: 11

Alt B: 5

Most companies supporting Alt B is OK with also Alt A

Alt X: 13

No motivation is found

Alt Y: 3

Proposal 1-4_v1

For dynamic indication of PUCCH repetition for Msg4 HARQ-ACK,

- Maximum N value(s) of repetition factor can be configured via SIB.

  - Alt 1: N = 1

  - Alt 2: N = 2

  - Alt 3: N = 4

  - FFS: configuration per cell or per beam or per PUCCH resource included in a PUCCH resource set provided by pucch-ResourceCommon

- PUCCH repetition factor or whether repetition is performed is indicated by:

  - Alt A: some of the existing fields in DCI scheduling the Msg4 PDSCH

    - FFS: which field

  - Alt B: MCS information field of DCI scheduling Msg3 PUSCH jointly with indication of Msg3 repetition factor

    - FFS: details on the joint indication

- A configured PUCCH resource set is included in:

  - Alt X: Table 9.2.1-1 of TS 38.213

    - Note: no spec change is assumed

  - Alt Y: A newly defined table for PUCCH repetition for Msg4 HARQ-ACK

    - FFS: details

      - 2nd input

For maximum number of configured values, more companies think 4 should be agreed to cover different UE performance or different UE type and so on. N is max, so Alt 1/2 are covered by Alt 3.

For indication signaling, probably both can work, and some of Alt B supporters also are OK with Alt A; so we can go with Alt A.

Regarding PUCCH table for this feature, most companies think that the existing table can be reused. Alt X can be agreed.

Proposal 1-4_v1

For dynamic indication of PUCCH repetition for Msg4 HARQ-ACK,

- Maximum N value(s) of repetition factor can be configured via SIB.

  - Alt 1: N = 1

  - Alt 2: N = 2

  - Alt 3: N = 4

  - FFS: configuration per cell or per beam or per PUCCH resource included in a PUCCH resource set provided by pucch-ResourceCommon

- PUCCH repetition factor or whether repetition is performed is indicated by:

  - Alt A: some of the existing fields in DCI scheduling the Msg4 PDSCH

    - FFS: which field

  - Alt B: MCS information field of DCI scheduling Msg3 PUSCH jointly with indication of Msg3 repetition factor

    - FFS: details on the joint indication

- A configured PUCCH resource set is included in:

  - Alt X: Table 9.2.1-1 of TS 38.213

    - Note: no spec change is assumed

  - Alt Y: A newly defined table for PUCCH repetition for Msg4 HARQ-ACK

    - FFS: details

Q: Do you agree the above proposal?

| Company | YES/NO | Comment |
| --- | --- | --- |
| MediaTek | YES | Support proposal |
| NEC | YES | Support the proposal |
| Ericsson | Mostly | Regarding alt X or Y, it is premature to agree on this level of specification detail before the solution is agreed. |
| NTT DOCOMO | Yes |  |
| Lenovo | Yes | Support. |
| LG | Yes |  |
| ZTE |  | Fine for first and third bullet. While for second bullet, the discussion can be postponed until agreeing on detailed solution on repetition determination. For example, if the repetition factor is determined and reported by UE, or if the repetition factor is indicated by network through RAR, there is no need to modify the DCI scheduling the Msg4 PDSCH. |
| Xiaomi | Yes |  |
| Samsung |  | We have a concern on the first FFS bullet. In our understanding, SIB is “system information” and is not associated any beam/PUCCH resource. Thus, it should be removed. |
| Nokia, Nokia Shanghai Bell | No | It would be preferable to have the proposal split into the separate elements for easier separation of the functionality. First bullet: OK for us Second bullet: Unless we know which field of the DCI is expected to be repurposed, it is difficult to determine if we can support. If no consensus can be reached on which field to repurpose, does this mean that the entire concept will fall? In addition, there could be backward compatibility issues with this approach in some cases, so we suggest to further study the mechanism of indication. Third bullet: How would we ensure that no specification change will happen? Changing/modifying an existing table will mean change of specifications, and on top of this we need to consider impact to legacy UEs (as well as impact to UEs not supporting the feature). |
| Panasonic | Yes with modification | If we go for Alt3 (N=4), candidate repetition factors does not need to be configured via SIB. DCI can directly indicate 1, 2, 4 or 8. We suggest to modify the first bullet to: - N value(s) of repetition factor can be dynamically indicated. - N = 4 |
| CMCC | Yes |  |
| Baicells | Yes |  |
| ETRI | Yes | Support. |
| Spreadtrum | Yes |  |

      - 2nd offline (4)

Proposal 1-4_v2

For dynamic indication of PUCCH repetition for Msg4 HARQ-ACK,

- Maximum N value(s) of repetition factor can be candidates of dynamic indication.

  - Alt 1: N = 1

  - Alt 2: N = 2

  - Alt 3: N = 4

  - FFS: configuration per cell or per beam or per PUCCH resource included in a PUCCH resource set provided by pucch-ResourceCommon

- PUCCH repetition factor or whether repetition is performed is indicated by:

  - Alt A: some of the existing fields in DCI scheduling the Msg4 PDSCH

    - FFS: which field (e.g., MCS information field)

  - Alt B: MCS information field of DCI scheduling Msg3 PUSCH jointly with indication of Msg3 repetition factor

    - FFS: details on the joint indication

- A configured PUCCH resource set is included in:

  - Alt X: Table 9.2.1-1 of TS 38.213

    - Note: no spec change is assumed

  - Alt Y: A newly defined table for PUCCH repetition for Msg4 HARQ-ACK

    - FFS: details

    - [Open/High] Inter-slot FH

Regarding inter-slot FH, FL observed that still companies’ views are quite divergent unfortunately. 6 companies [2/vivo] [10/Intel] [13/Pana] [17/Apple] [19/LGE] [23/Baicells] believe that inter-slot FH should be supported e.g., for better performance, due to low workload by reusing R17 Msg3 repetition, etc. 3 companies [1/HW, HiSi] [6/CATT] [25/Nokia, NSB] are not convinced with inter-slot FH since gain is limited and just max 8 repetitions without inter-slot FH can meet the requirement.

On this issue, FL found that the current specification does not have clear text to do ‘intra-slot’ FH as below.

### 9.2.1 PUCCH Resource Sets … The UE transmits a PUCCH using frequency hopping if not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon; otherwise, the UE transmits a PUCCH without frequency hopping. …

For PUCCH without repetition, there is no choice other than performing ‘intra-slot’ FH. However, when repetition is introduced, whether intra-slot or inter-slot should be applied is unclear in the current spec. No spec change does not mean applying intra-slot FH. At least corresponding spec change would be necessary. Therefore, FL believes that continuing this discussion and concluding explicitly are essential.

Proposal 1-5_v0

For PUCCH repetition for Msg4 HARQ-ACK,

- Down-select from the following options.

  - Option X: Apply intra-slot frequency hopping

  - Option Y: Apply inter-slot frequency hopping

      - 2nd input

Proposal 1-5_v0

For PUCCH repetition for Msg4 HARQ-ACK,

- Down-select from the following options.

  - Option X: Apply intra-slot frequency hopping

  - Option Y: Apply inter-slot frequency hopping

Q: Do you agree the above proposal?

| Company | YES/NO | Comment |
| --- | --- | --- |
| MediaTek | YES | Down selection can be further discussed |
| Ericsson | No | Intra-slot frequency hopping is already supported for PUCCH and it should be made clear in the proposal that this is the baseline while inter-slot frequency hopping needs to be justified by performance gains. |
| NTT DOCOMO | Yes | We are open to discuss frequency hopping. |
| Lenovo | Yes |  |
| LG | Yes | We are open to discuss frequency hopping. |
| ZTE | Yes | We prefer option X. The gain of inter-slot FH in addition to intra-slot FH is not clear, especially with consideration that the repetition factor is limited. |
| Samsung | No | Similar view with Ericsson. In our understanding, if option Y is considered, UE needs to apply different hopping scheme depending on the number of transmissions. That should be avoided. |
| Nokia, Nokia Shanghai Bell | Yes with comments | OK to list the options, but according to our understanding we would not have any benefits from the inter-slot frequency hopping, since the radio channel is expected to be frequency flat due to the lack of time dispersion. |
| Panasonic | Yes | We slightly prefer Option Y because of better channel estimation accuracy for low SNR region (or higher number of repetitions). |
| CMCC | Yes | We are fine to further discuss frequency hopping. |
| Baicells |  | We prefer Option Y (inter-slot frequency hopping) for PUCCH repetition. Inter-slot FH can be an enhancement or supplement to the existing specification, not necessarily in a manner of down-selection, thus more companies can make compromise. |
| ETRI | Yes | We are open to discuss frequency hopping. |
| Spreadtrum | Yes | We are fine to further discuss frequency hopping. |

      - 2nd offline (6)

Proposal 1-5_v0

For PUCCH repetition for Msg4 HARQ-ACK,

- Down-select from the following options.

  - Option X: Apply intra-slot frequency hopping

  - Option Y: Apply inter-slot frequency hopping

    - [Low] Terminology / Focused situation

Several companies suggest discussing the terminology issue, which was raised at the last meeting. One company [10/Intel] believes that ‘PUCCH for Msg4 HARQ-ACK which corresponds to PUCCH resource indicated by a DCI format 1_0 with CRC scrambled by TC-RNTI’ should be used. 4 companies [2/vivo] [11/OPPO] [19/LGE] [21/Samsung] propose to use ‘PUCCH transmission when dedicated PUCCH resource configuration is not provided’ or with a bit modification. One company [23/Baicells] suggest postponing this discussion.

Although FL thinks that this issue should be solved, available time in this meeting will be consumed for the features themselves. Discussion in the next RAN plenary may be considerable.

Proposal 1-6_v0

For coverage enhancement of PUCCH for Msg4 HARQ-ACK,

- Down-select from the following options

  - Option 1: RAN1 discuss PUCCH transmission when dedicated PUCCH resource configuration is not provided

  - Option 2: RAN1 discuss PUCCH transmission when PUCCH resource is indicated by a DCI format 1_0 with CRC scrambled by TC-RNTI

  - Option 3: Keep ‘PUCCH for Msg4 HARQ-ACK’ now and discuss later

    - [Low] Additional repetition factor

5 companies [2/vivo] [6/CATT] [11/OPPO] [12/ETRI] [24/Ericsson] propose to close this issue without any additional repetition factor while one company [23/Baicells] believes that larger value is needed.

Based on the majority companies’ views, FL recommends the following proposal.

Proposal 1-7_v0 (for conclusion)

For PUCCH repetition for Msg4 HARQ-ACK, no additional repetition factor is supported.

    - [Low] DMRS bundling

On DMRS bundling, only two companies [4/ZTE] [17/Apple] propose to support this feature, but more companies object the direction. The reason is little gain and infeasibility to apply this feature before RRC connection establishment. FL thinks these arguments are valid. In addition, if this situation is not changed and thereby no spec change is agreed, this means PUCCH repetition is performed without DMRS bundling, which can work. Therefore, FL recommends that companies discuss offline with the objecting companies.

Proposal 1-8_v0 (for conclusion)

For PUCCH repetition for Msg4 HARQ-ACK, deprioritize DMRS bundling.

  - DMRS bundling for PUSCH taking into account NTN-specifics

    - [Open/High] Phase difference due to timing drift and/or doppler shift

38.101-1 Table 6.4.2.5-1: Maximum allowable phase difference for DMRS bundling UL channel; Modulation order; Phase difference between any slot p-1 and slot p (NOTE 2); Phase difference between slot 0 and any slot p (NOTE 3). PUSCH; Pi/2 BPSK, QPSK; [25] degrees; [30] degrees. PUCCH; Pi/2 BPSK, BPSK, QPSK. NOTE 1: The UE capability of the length of maximum duration refers to the maximum time duration during which UE is able to meet the phase continuity requirements, assuming no phase consistency violating events defined in TS 38.214 in between. NOTE 2: This requirement applies for FDD and TDD bands, for supported DMRS bundling configurations ≤ 8 slots. NOTE 3: This requirement applies only for FDD bands, for supported DMRS bundling configurations of 16 slots.. 38.101-5 ### 6.4.1 Frequency error The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. 38.133 Table 7.1C.2-1: Te_NTN Timing Error Limit Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te_NTN. 1; 15; 15; 29*64*Tc. 30; 24*64*Tc. 60; N/A. 30; 15; 24*64*Tc. 30; 22*64*Tc. 60; N/A. Note 1: Tc is the basic timing unit defined in TS 38.211 [6].

Regarding phase difference due to timing drift and/or doppler shift, FL observed that at least 3 companies [8/NTPU] [22/QC] [25/Nokia, NSB] states that the existing requirement (e.g., 0.1 PPM) cannot be met for two adjacent slots unless further gNB implementation assumption and/or UE requirement while 1 company [9/xiaomi] believes that the existing requirement can be met for 20 repetitions assuming 0.09 PPM/s.

Then, there are two directions as the solution: post-compensation by gNB (4 companies [1/HW, HiSi] [5/Hyundai] [20/DCM] [22/QC]) or pre-compensation with higher requirement (4 companies [5/Hyundai] [9/xiaomi] [20/DCM] [25/Nokia, NSB]).

Besides, FL found that at least 2 companies [13/Pana] [22/QC] believe that UE pre-compensation update within an actual TDW is possible such that phase discontinuity does not occur, while at least 6 companies [2/vivo] [5/Hyundai] [9/xiaomi] [14/Sony] [17/Apple] [19/LGE] assume the update is not performed within an actual TDW. However, FL does not understand relationship between the phase difference issue and the pre-compensation update within an actual TDW; even if such an operation is possible, the requirement of the operation outcome is the same and hence the phase difference issue will not disappear.

In FL’s understanding, the above issue is basically RAN4 discussion, and thus it is better to share this issue to RAN4 and to ask RAN4 what RAN1 should do. Without RAN4 confirmation, it would not be a good way that RAN1 agree some assumption and continue discussion based on the assumption.

Proposal 2-1_v0

For NTN-specific PUSCH DMRS bundling,

- RAN1 assume that UE cannot meet for two or more adjacent slots the phase continuity requirement specified as Table 6.4.2.5-1 in 38.101-1 when frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-5 and timing error specified in Table 7.1C.2-1 of 38.133 are allowed.

- Send an LS to RAN4 to ask the following questions:

  - Whether the above assumption is valid or not

  - Whether/how this issue (if valid) should be solved

    - e.g., UE should meet the phase continuity requirement with smaller time/frequency error and/or gNB should do post-compensation based on satellite movement and/or the UE requirement(s) will be changed for PUSCH DMRS bundling in NTN

Q: Do you agree with this proposal? If NO, what should be agreed?

| Company | YES/NO | Comment |
| --- | --- | --- |
| Lenovo |  | We are fine to send LS to RAN4 to ask the corresponding requirement. If there is anyway phase continuity issue, we think segmentation is necessary. |
| Apple |  | Fine to send LS to RAN4. |
| Panasonic | Yes | Agree to send LS to RAN4. Besides, frequency error requirement of 0.1 ppm is not specific to NTN but common for both TN and NTN. Assuming phase continuity can be met in TN, it would be better to discuss impact of NTN specific timing and frequency error on the phase continuity requirement. |
| Xiaomi |  | Agree to send LS to RAN4. In our understanding, there are two “pre-compensation” in NTN specific DMRS bundling, one is time/TA pre-compensation, the other is phase pre-compensation. Based on RAN1’s previous agreements in Rel-17, Agreement - UE should not perform UE autonomous TA adjustment during the actual time domain window. It is clear that TA pre-compensation during the actual TDW is not allowed. As the time drift and/or time drift variation is known to UE, the phase can be pre-compensated before the start of actual TDW, then the phase difference observed by gNB can meet the phase difference requirement defined by RAN4. Therefore, we think another question for RAN4 is whether the phase difference requirement is for phase difference observed by UE or gNB? |
| ZTE | No | In our view, the phase continuity issue is mainly caused by the timing/frequency drift as discussed in proposal 2-2. The 0.1 ppm fixed frequency error also exists in TN and is not a new NTN specific issue. Hence, how this issue is resolved in legacy TN may first be checked before conclusion. |
| QC | No | Agree with ZTE. |
| OPPO | YES | Agree with FL that the phase difference due to timing drift and/or doppler shift should be discussed by RAN4. |
| Baicells | Yes | We agree that 0.1 ppm fixed frequency error is not a new NTN specific issue. But also agree to send LS to RAN4 for further discussion. |
| Sharp | Yes | We are OK to send an LS to RAN4. |
| NTT DOCOMO | Yes | Agree to send LS to RAN4. |
| Huawei, HiSilicon | No | We have not decided yet whether any enhancement to enable DMRS bundling is needed. We prefer to firstly decide whether DMRS bundling should be enhanced to be used for NTN based on simulation results. |
| Ericsson | No | UE TX frequency error of 0.1 ppm is tolerated also in a legacy terrestrial network. Therefore, it can be assumed that gNB has frequency offset post-compensation implemented. Our understanding is that the RF frequency error is corrected in the RAN4 test before phase continuity is measured. LS to RAN4 could be sent if we cannot conclude without it. |
| Nokia, Nokia Shanghai Bell | Yes | Perhaps clarify that we specifically ask whether this assumption of phase stability in Table 6.4.2.5-1 is also applicable for use with NR over NTN (and whether the phase stability is to be assumed at the gNB side of the system (such that this refers to the signals received at the antenna side). That is, the potential Doppler effects on the uplink direction on the service link. |
| LG |  | Fine to send LS to RAN4. Regarding frequency error requirement of 0.1 ppm, we think it is not an new issue. If we consider it as maximum frequency error which UE suffers, it means Rel-17 DMRS bundling is not applicable also in TN. If we consider NTN-specific Doppler shift included in the error of 0.1 ppm, it can be pre-compensate at gNB side. Timing drift would be an issue since UE originally managed it by adjusting TA, even for the moving of the satellite. |

      - 1st offline

YES: 10

NO: 4

0.1PPM frequency error is existing even in TN

Proposal 2-1_v1

For NTN-specific PUSCH DMRS bundling,

- RAN1 assume that UE cannot meet for two or more adjacent slots the phase continuity requirement specified as Table 6.4.2.5-1 in 38.101-1 when frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-5 and timing error specified in Table 7.1C.2-1 of 38.133 are allowed.

  - RAN1 assume that in TN, UE cannot meet for two or more adjacent slots the phase continuity requirement specified as Table 6.4.2.5-1 in 38.101-1 when frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1 is allowed.

- Send an LS to RAN4 to ask the following questions:

  - Whether the above assumption is valid or not

  - Whether/how this issue (if valid) should be solved

    - e.g., UE should meet the phase continuity requirement with smaller time/frequency error and/or gNB should do post-compensation based on satellite movement and/or the UE requirement(s) will be changed for PUSCH DMRS bundling in NTN

      - 2nd input

It seems that the main issue is 0.1PPM error. How to handle this error in TN is unclear now.

FL think what we can do is just to agree assumption on TN and whether/how to update the assumption for NTN.

Proposal 2-1_v2

For NTN-specific PUSCH DMRS bundling,

- Send an LS to RAN4 to ask the following questions:

  - Q1: Whether the following assumption for TN is correct

    - Phase variation due to frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1 is larger than the phase continuity requirement for two adjacent slots specified as Table 6.4.2.5-1 in 38.101-1, and post-compensation of frequency error will be performed at gNB

  - Q2: Whether/how to change this assumption for NTN

  - Q3: Whether/how to change the above requirements and timing error requirement specified in Table 7.1C.2-1 of 38.133 for NTN

Q: Do you agree with this proposal? If NO, what should be agreed?

| Company | Option | Comment |
| --- | --- | --- |
| MediaTek | None | We do not think LS to RAN4 should be sent at this stage. Further discussion in RAN1 is needed to align understanding on how phase continuity issue is resolve in legacy TN and discuss further NTN-specific aspects and potential enhancements for DM RS bundling.. |
| Ericsson | No | Q1, Q2: After discussion with RAN4 colleagues, our understanding is that the frequency error is corrected by the test equipment before the phase continuity is measured, as described in 38.101-1 annex F.9 and F.4. Therefore, the phase drift due to frequency error does not impact the phase continuity. Post-compensation in gNB may be needed but this is up to implementation. Q3: We currently don't see a need to change these requirements. |
| OPPO | Yes | In our understanding, at least whether the phase difference due to timing drift and/or doppler shift in NTN can meet the phase continuity requirement should be discussed by RAN4. |
| NTT DOCOMO | Yes |  |
| Lenovo |  | We are fine with Q1. If the answer to Q1 is yes, we think we also need to ask how the issue is addressed in TN. Regarding Q2 and Q3, we don’t think it is necessary at this stage. |
| LG | Yes | We are fine with Q1 and Q2. If Ericsson’s comment is the common understanding for our group, we can make a conclusion for that rather than sending out LS. Regarding Q3, we are not sure that it is necessary to change the requirements. |
| ZTE |  | RAN1 may first discuss and make it clear how 0.1 ppm error is handled in legacy TN. If RAN1 cannot make conclusion, send LS to RAN4 for clarification can also be considered if majority agree. |
| Xiaomi | Partially | We are fine to send LS to RAN4, but the questions need further update. For Q1, we think it is not necessary. Firstly the frequency error is the residual estimated CFO using the SSB signal for frequency adjustment, the common offset can be post-compensated by the gNB. Another reason is that the measurement conditions are defined in Table 6.4.2.5-2 for the requirements listed in Table 6.4.2.5-1, while in Table 6.4.2.5-2 the DL signal frequency is not changed, in other words, the frequency error is assumed to be constant of different slots during the repetition. Therefore, the assumption for Q1 for TN is correct. For Q2, we think it is still valid. For example, whether DL signal timing and DL signal frequency can be assumed as constant in NTN. In other words, will the frequency drift (e.g., 0.09PPM/s) and time drift (e.g., 0.1us/ms) impact the previous measurement condition? For Q3, we think it is relevant to the answer of other questions, which is not necessary at this moment. Table 6.4.2.5-2: Measurement conditions for the maximum allowable phase difference Parameter; Unit; Level. UE Output Power; dBm; PCMAX,f,c in clause 6.2.4, P-MPR = 0. UE downlink received power; Not change. Operating conditions; Normal conditions. Transmission bandwidth; Confined within FUL_low + [4] MHz and FUL_high – [4] MHz. DL signal frequency; Not change before and during the measurement window. DL signal timing; Maintained constant before and during the measurement window. UL slots for testing; Tested on consecutive UL slots. |
| Nokia, Nokia Shanghai Bell | No | In general we are OK with sending LS to RAN4, but for Q1 it is important to highlight to RAN4 that the frequency error will happen on a per-UE basis (meaning that different UE may have different frequency error when receiving multiple UEs at the same time). |
| Baicells | Yes | We have same understanding with the Ericsson’s comment on Q1 and Q2. Yet, sending LS to RAN4 for clarification is still necessary. |
| Spreadtrum | Yes |  |

      - 2nd offline (5)

Proposal 2-1_v2

For NTN-specific PUSCH DMRS bundling,

- Send an LS to RAN4 to ask the following questions:

  - Q1: Which is the correct assumption for TN, or if neither is correct, what is correct assumption for TN

    - Assumption 1: Phase variation due to frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1 is larger than the phase continuity requirement for two adjacent slots specified as Table 6.4.2.5-1 in 38.101-1, and post-compensation of frequency error will be performed at gNB

    - Assumption 2: Phase variation due to frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1 does not have impact on the phase continuity requirement for two adjacent slots specified as Table 6.4.2.5-1 in 38.101-1, according to annex F.9 and F.4 of 38.101-1

  - Q2: Whether/how to change this assumption for NTN

  - Q3: Whether/how to change the above requirements and timing error requirement specified in Table 7.1C.2-1 of 38.133 for NTN

    - [Open/High] Procedure to perform UL pre-compensation with DMRS bundling

Regardless of discussion outcome in the last section, FL observed that several companies (at least [2/vivo] [3/MTK] [4/ZTE] [8/NTPU] [13/Pana] [15/CMCC] [24/Ericsson]) are discussing applicable/required window length and believe that some partitioning for 20 ms PUSCH repetitions is necessary at least for some scenarios.

Then, there is discussion in companies’ contributions on whether UE pre-compensation should be defined as an event which causes power consistency and phase continuity not to be maintained and whether segmentation concept should be introduced. From reviewing their contributions, current FL’s understanding is that there are two high-level directions.

Direction 1: Timing of UE pre-compensation update is determined first, and then actual TDW is decided based on the timing

Direction 2: Actual TDW is determined first, and then UE pre-compensation update is allowed only at each boundary of two adjacent actual TDWs

In direction 1, the timing is reported to have the same understanding between gNB and UE, and the timing is considered as the event. FL’s reading is that 7 companies [2/vivo] [4/ZTE] [6/CATT] [11/OPPO] [17/Apple] [18/Lenovo] [19/LGE] assume this direction. In addition, R17 IoT-NTN segmentation concept is proposed by some of them in order to achieve direction 1. On the other hand, FL found that one company [13/Pana]’s comment that nominal TDW configuration works as ‘segment’ is valid and thus necessity of segmentation concept beyond nominal TDW configuration would be unclear.

In direction 2, what specification needs to define is to clarify that UE allows UL pre-compensation update with phase discontinuity only at the boundaries and to report from UE the maximum nominal TDW duration in consideration of the NTN-specific rule. FL’s reading is that 5 companies [1/HW, HiSi (?)] [13/Pana] [20/DCM] [22/QC (?)] [24/Ericsson (?)] assume this direction. Such a capability-related issue is discussed in contributions from [3/MTK] [10/Intel] [20/DCM].

FL recommends discussing the high-level concept first; otherwise, we cannot have consensus on detailed UE behavior.

Proposal 2-2_v0

For NTN-specific PUSCH DMRS bundling,

- Down-select from the following options of applying time/frequency compensation update with DMRS bundling

  - Option 1: Actual TDW is determined based on when UE performs pre-compensation update causing phase discontinuity and power inconsistency

  - Option 2: Actual TDW is determined as in the existing specification and UE can perform pre-compensation update causing phase discontinuity and power inconsistency only at boundaries of two adjacent actual TDWs

Q: Which option should be agreed? Also please share the reason.

| Company | Option | Comment |
| --- | --- | --- |
| Lenovo | Option 1 | We think this is more aligned with the actual TDW determination in coverage enhancement in R17. |
| Apple | Comment | We think even the configured TDW (not actual TDW) should be updated based on UE’s pre-compensation capability. |
| Panasonic | 2 | Option 1 would not work because gNB needs to know for appropriate joint channel estimation the timing of UE pre-compensation update causing phase discontinuity and/or power inconsistency but the timing is up to UE implementation. Existing mechanism of UE capability report and nominal TDW would be sufficient. |
| Xiaomi | 1 | Our understanding of option 1 is defining a new event of time/frequency pre-compensation for determining actual TDW. |
| ZTE | 1 | If TDW is first determined and UE can only adjust pre-compensation at boundary, then the phase difference/timing error may exceed tolerable range, which is not desired. Hence, option 1 should be agreed. |
| OPPO | 1 | Option 1 provides sufficient feasibility for time/frequency pre-compensation in NTN. In our understanding, the actual TDW should only be used for DMRS bundling and should not be bundled with UL pre-compensation update. |
| Baicells | 2 | Option 2 is more clear from the point view of specifications. Option 1 need much more additional signaling. |
| Sharp | 1 | Option 2 would restrict scheduling timing of UE pre-compensation update, which should be UE implementation currently. Regarding Panasonic comment, we don’t agree that nominal TDW would be reused. Nominal TDW starts at the first PUSCH repetition, and this would also restrict UE’s scheduling timing of UE pre-compensation. |
| NTT DOCOMO | 2 | In the current specification, the timing of time/frequency pre-compensation update by UE is up to UE implementation, which is unknown to gNB. Option 1 requires a clear specification of the pre-compensation update timing, but we don’t think this is appropriate considering that there are various possible implementations of pre-compensation. |
| Huawei, HiSilicon | comment | We think TDW should be updated based on the event which may impact the phase continuity in NTN, e.g. ephemeris update, antenna switching etc. Also, as we commented before, the performance of JCE should be summarized and conclude whether JCE needs to be enhanced for NTN, rather than using time to discuss the details on how to support JCE in NTN. |
| Ericsson | 1 | The length of the TDW in the existing specification can be too long for transmission without TA update in NTN. The goal should be to fulfil existing timing accuracy requirements for each transmitted slot in the TDW. Therefore, it is necessary to limit the actual TDW length based on the timing drift and ensure that the UE and NW have a common understanding. |
| CMCC | 1 | It can be regarded as a event which cause power consistency and phase continuity not to be maintained when the UE changes its time and/or frequency pre-compensation. |
| Nokia, Nokia Shanghai Bell | Slight preference for option 1 | There is a risk that under Option 2, the UE may have to withhold the UE autonomous update of the Common TA parameters (and Doppler compensation) since it may get limited by the potential TDM windows restricting the updates. |
| LG | 2 | To solve the issue by TDW, the determination of TDW has to be known between UE and gNB. As Panasonic mentioned, since the timing of the TA update is up to UE implementation, gNB cannot expect when UE changes phase and power so Option 1 doesn’t work. Considering gNB can configure maximum tolerable TDW size to UE, Option 2 should be enough. |

      - 1st offline

1: 8

In Option 2, the phase difference/timing error may exceed tolerable range, which is not desired

2: 4

gNB needs to know the timing of UE pre-compensation update

Proposal 2-2_v1

For NTN-specific PUSCH DMRS bundling,

- Down-select from the following options of applying time/frequency compensation update with DMRS bundling

  - Option 1: Actual TDW is determined based on when UE performs pre-compensation update causing phase discontinuity and power inconsistency

  - Option 2: Actual TDW is determined as in the existing specification and UE can perform pre-compensation update causing phase discontinuity and power inconsistency only at boundaries of two adjacent actual TDWs

After offline discussion:

Proposal 2-2_v2

For NTN-specific PUSCH DMRS bundling,

- Down-select from the following options of applying time/frequency compensation update with DMRS bundling

  - Option 1: Actual TDW is determined based on reported information regarding when UE performs pre-compensation update causing phase discontinuity and power inconsistency

    - FFS: what information is reported

    - FFS: how to report the information

  - Option 2: Actual TDW is determined as in the existing specification and UE can perform pre-compensation update causing phase discontinuity and power inconsistency only at boundaries of two adjacent actual TDWs

      - 2nd input

There are more supporters for option 1, and some of Option 2 supporters can live with Option 1, for example if the issues with red colors are clarified. Then FL recommends going with Option 1.

Regarding details of Option 1, FL received one comment that nominal TDW can be used instead of actual TDW. FL thinks the comment is correct, and at least in this stage both possibilities can be kept.

Proposal 2-2_v3

For NTN-specific PUSCH DMRS bundling,

- Down-select from the following options of applying time/frequency compensation update with DMRS bundling

  - Option 1: Actual/Nominal TDW is determined based on reported information regarding when UE performs pre-compensation update causing phase discontinuity and power inconsistency

    - FFS: what information is reported

    - FFS: how to report the information

  - Option 2: Actual TDW is determined as in the existing specification and UE can perform pre-compensation update causing phase discontinuity and power inconsistency only at boundaries of two adjacent actual TDWs

Q: Do you agree the above proposal?

| Company | YES/NO | Comment |
| --- | --- | --- |
| MediaTek | NO | We do not understand the need for “reported information regarding”. If it is meant “based on UE capability” then this should be mentioned explicitly. If something else, then more discussion needed before details in FFS bullets are considered. |
| Ericsson | Partially | Our understanding of the proposal is that it is the UE that reports the information. But our view is that the TDW should be under network control and it is ffs if any information from the UE is needed for this. The required update rate of pre-compensation is known by the network based on the serving cell assistance information. |
| OPPO | NO | The reported information in the proposal is also unclear to us. |
| NTT DOCOMO | Yes | Since the timing of pre-compensation update is up to UE implementation, UE will need to report some information about pre-compensation to gNB to align the understanding of the update timing between UE and gNB. |
| Lenovo | Yes |  |
| LG | Yes | For our understanding, method of the timing pre-compensation is totally up to UE implementation so far. To facilitate Option 1, it is necessary to align the timing of UE’s updates between UE and gNB so they get same TDW at both sides. Any kind of information would be necessary. If the information is based on UE capability and TDW is under gNB control, This approach could be in the light of Option 2. |
| ZTE | No | We agree with MediaTek that “reported information regarding” is not clear. In IoT-NTN, the segment length for pre-compensation is configured by network. If this is reused in NR-NTN, the TDW may be determined by network configuration on pre-compensation instead of reported information. With this consideration, we prefer the original option 1 in proposal 2-2_v1. |
| FL |  | After I read the above kind comments, it seems that there are two ways for Option 1. One is UE decides when the update is performed and actual TDW is decided by that. The other is gNB decides when the update is updated and actual TDW is decided by that. Now I think it is better to agree option list in this meeting and to discuss further for the next meeting, so all options are kept as below. Please share comment for this latest version. Proposal 2-2_v4 For NTN-specific PUSCH DMRS bundling, - Down-select from the following options of applying time/frequency compensation update with DMRS bundling - Option 1: Actual/Nominal TDW is determined based on when UE performs pre-compensation update causing phase discontinuity and power inconsistency - Option 1-1: UE reports information regarding when UE performs pre-compensation update causing phase discontinuity and power inconsistency - FFS: details of information report - Option 1-2: UE is configured/indicated regarding when UE can perform pre-compensation update causing phase discontinuity and power inconsistency - FFS: details of configuration/indication - Option 2: Actual/Nominal TDW is determined as in the existing specification and UE can perform pre-compensation update causing phase discontinuity and power inconsistency only at boundaries of two adjacent actual TDWs |
| Samsung |  | For proposal 2-2_v4, we think that option 1-2 and option 2 are same because TDW duration is determined by NW, not UE. Would FL clarify difference between and option 2 and option 1-2? |
| Nokia, Nokia Shanghai Bell | Yes |  |
| Panasonic |  | OK with Proposal 2-2_v4 |
| CMCC | Yes | A UE self-estimated TA to pre-compensate for the service link delay, which is calculated using the UE position and the serving satellite ephemeris, may be transparent for gNB. Thus, reported information to align the understanding between network and UE can help gNB to configure proper TDW. |
| Baicells |  | Agree on Proposal 2-2_v4. We prefer option2. |
| Spreadtrum | Yes |  |

      - 2nd offline (2)

Proposal 2-2_v4

For NTN-specific PUSCH DMRS bundling,

- Down-select from the following options of applying time/frequency compensation update with DMRS bundling

  - Option 1: Actual/Nominal TDW is determined based on when UE performs pre-compensation update causing phase discontinuity and power inconsistency

    - Option 1-1: UE reports information regarding when UE performs pre-compensation update causing phase discontinuity and power inconsistency

      - FFS: details of information report

    - Option 1-2: NTN-specific configuration/indication is provided regarding when UE can perform pre-compensation update causing phase discontinuity and power inconsistency

      - FFS: details of configuration/indication

  - Option 2: Actual/Nominal TDW is determined as in the existing specification and UE can perform pre-compensation update causing phase discontinuity and power inconsistency only at boundaries of two adjacent actual TDWs

    - Note: No NTN-specific configuration/indication to determine actual/nominal TDW is assumed

    - [Low] New event which causes power consistency and phase continuity not to be maintained

Besides, there are proposals to define another new event which causes power consistency and phase continuity not to be maintained. [1/HW, HiSi] proposes to define antenna switching as the event. [2/vivo] [9/xiaomi] propose to define indication/update/arrival of epoch time as the event.

FL believes that at least the second proposal is related to proposal 2-2, thus the discussion should be postponed. For the first proposal, FL thinks similar discussion to proposal 2-2 is assumed; i.e., whether actual TDW should be determined based on the feature or just timing restriction to perform the feature is sufficient.

Proposal 2-3_v0

For NTN-specific PUSCH DMRS bundling,

- Down-select from the following options of applying antenna switching with DMRS bundling

  - Option 1: Actual TDW is determined based on when UE performs antenna switching causing phase discontinuity and power inconsistency

  - Option 2: Actual TDW is determined as in the existing specification and UE can perform antenna switching causing phase discontinuity and power inconsistency only at boundaries of two adjacent actual TDWs

- Contribution summary

  - PUCCH enhancements for Msg4 HARQ-ACK

- Terminology

  - PUCCH for Msg4 HARQ-ACK which corresponds to PUCCH resource indicated by a DCI format 1_0 with CRC scrambled by TC-RNTI: [10/Intel]

  - PUCCH transmission when dedicated PUCCH resource configuration is not provided: [2/vivo] [11/OPPO] [19/LGE] [21/Samsung]

    - [2/vivo] Update as: PUCCH transmission before a UE has dedicated PUCCH resource configuration

  - Discuss later: [23/Baicells]

- Whether to perform rep: Cell-specific vs UE-specific

  - Option 1 (cell-specific): [3/MTK] [9/xiaomi] [19/LGE] [23/Baicells]

    - [3/MTK] For example, assuming the network configures all UEs with 8 repetitions, some UEs could use 4 repetitions only if they have an UL link budget that is up to 5 dBs better. It would make the system more robust to have conservative assumption for the number of repetitions

    - [9/xiaomi] both option 1 and option 2/3 should be supported for different deployment cases.

  - Option 1’ (Request + cell-specific): [1/HW, HiSi]

    - [1/HW, HiSi] Observation 4: Dynamic repetition indication induces considerable specification change, and has an impact on the existing fields in RAR UL grant or DCI 1_0.

  - Option 2 (Request + UE-specific): [2/vivo (?)] [4/ZTE] [6/CATT] [7/Spreadtrum] [9/xiaomi] [11/OPPO] [25/Nokia, NSB] [12/ETRI (?)][13/Pana] [15/CMCC (?)] [16/NEC] [17/Apple] [18/Lenovo] [20/DCM] [21/Samsung (?)]

    - [4/ZTE] with different elevation angle, UE may experience different path loss because of the location of UE, satellite and gNB, UEs with different channel environments may need more or not need repetitions to mitigate performance loss.

    - [6/CATT] If one cell is corresponding to multiple beams, the number of repetitions configured in cell-specific manner is inflexible and will lead to resource waste. Allowing UE to request repetition for msg4 HARQ-ACK is helpful to improve coverage and reduce the resource waste.

    - [6/CATT] Actually even if one has the capability to do PUCCH repetition it doesn’t mean UE always to do repetition.

    - [9/xiaomi] both option 1 and option 2/3 should be supported for different deployment cases.

    - [11/OPPO] The repetition request is a more reasonable approach, which takes both repetition capability and coverage performance into account

    - [16/NEC] we think dynamic indication can help to avoid PUCCH performance degradation by adapting the number of repetitions if the channel conditions change. This can  help avoid PUCCH performance degradation (e.g. by indicating higher repetition factor when channel conditions get worse) and can also be useful for avoiding waste of network resources (e.g. indicating lower value if channel conditions are favorable).

    - [17/Apple] the large satellite beam footprint indicates that UEs in the coverage of the same satellite have quite various channel conditions. The PUCCH repetition for Msg4 HARQ-ACK may be needed for one UE, while it is unnecessary for another UE.

    - [18/Lenovo] There will be some blind detection at gNB side, and the resource should be reserved based on the cell-specific repetition number, so there will be some resource waste.

    - [21/Samsung] For option 3, it is not possible for a UE to indicate the repetition capability to gNB since it is before RRC connection. If UE capability indication is indicated via Msg3 PUSCH, it needs non-trivial specification impact for other working groups since such information in Msg3 PUSCH has not been defined before.

  - Option 3 (Capability + UE-specific): [7/Spreadtrum] [10/Intel] [22/QC] [24/Ericsson] [25/Nokia, NSB]

    - [10/Intel] In our view, given that gNB may measure the UL SNR directly based on UL signals before PUCCH for Msg4 reception (e.g., PRACH, Msg3 PUSCH DMRS), there is no need to support UE request for PUCCH repetition.

    - [22/QC] Backward compatibility issue, Due to the differences in transmit power and antenna gain

    - [24/Ericsson] The SNR can differ between different UEs depending on location in cell, antenna type, UE power class, etc. The SNR can also vary with time – especially for a pseudo-earth-fixed cell – and adapting the broadcast repetition factor to these variations may be too slow.

    - [24/Ericsson] NW determination can be based on UL measurements on Msg3. UL measurements are more reliable when determining the need for repetitions of Msg.4 HARQ-ACK. Further, if indicating a preferred number of repetitions, Option 2 requires more bits for the UE to indicate preferred number of repetitions while Option 3 only requires a single capability bit.

- Details on cell-specific manner

  - Cell-specific configuration

    - Configuration per beam: [1/HW, HiSi]

    - Configured in PUCCH-CondigCommon: [3/MTK] [23/Baicells]

    - ‘1’ can be explicitly/implicitly configured: [23/Baicells]

  - Incapable/legacy UE’s behavior

    - Not perform repetition: [9/xiaomi] [23/Baicells]

    - Not perform transmission: [23/Baicells]

- Details on request / capability report

  - Request signaling / capability report signaling

    - PRACH preamble/occasion: [1/HW, HiSi] [2/vivo] [6/CATT] [9/xiaomi] [12/ETRI] [13/Pana] [15/CMCC] [17/Apple] [18/Lenovo]

    - PRACH preamble/occasion for request of both Msg3 repetition and Msg4 PUCCH repetition: [1/HW, HiSi] [2/vivo] [13/Pana]

    - Msg3: [4/ZTE] [9/xiaomi] [18/Lenovo] [20/DCM] [22/QC] [24/Ericsson] [25/Nokia, NSB]

    - PUCCH for Msg4 HARQ-ACK (detectable before HARQ-ACK decoding): [25/Nokia, NSB]

  - Determination of whether to request

    - Measurement threshold different from that for Msg3 repetition request: [1/HW, HiSi]

    - UE measurement: [13/Pana] [18/Lenovo] [20/DCM]

    - SSB RSRP and capability: [17/Apple]

- Repetition indication

  - ‘1’ can be configured/indicated: [2/vivo] [23/Baicells]

    - [2/vivo] from the whole network system perspective, gNB may not want to indicate UEs to perform more than 1 repetition for PUCCH for msg4 HARQ-ACK when the link quality is not so bad, even though UE requests the PUCCH repetition.

    - [23/Baicells] explicitly or implicitly

  - Only one value is configured via SIB: [4/ZTE] [11/OPPO] [12/ETRI] [19/LGE] [21/Samsung]

    - [4/ZTE] the coverage performance is relatively stable across the cell. Supporting all repetition number may not be needed.

    - [11/OPPO] It can be observed that the variation range of UL CNR within an NTN cell is small, even less than 1.5dB, which is the NTN-specific characteristic

    - [12/ETRI] Because all UEs in an NTN beam/cell suffer from a bad link condition and the channel quality b/w UEs is not so different

    - [21/Samsung] configuring a number of repetitions per PUCCH common resource

  - Multiple values are configured via SIB: [19/LGE] [21/Samsung] [24/Ericsson]

  - Reuse some field in DCI scheduling Msg4 PDSCH: [9/xiaomi] [10/Intel] [12/ETRI] [13/Pana] [18/Lenovo] [19/LGE] [20/DCM] [21/Samsung] [22/QC] [24/Ericsson]

  - New DCI field: [21/Samsung]

  - Linkage between PRACH/Msg3 and PUCCH, joint indication: [10/Intel] [15/CMCC] [16/NEC] [17/Apple] [18/Lenovo] [19/LGE]

    - [16/NEC] it is preferred to support a solution with minimum specification impact and standards effort

  - Via Msg2: [15/CMCC]

  - CRC scrambling of DCI scheduling Msg4: [25/Nokia, NSB]

    - [25/Nokia, NSB] only the 16 LSB of the CRC are scrambled with the TC-RNTI, whereas the remaining 8 parity bits of the CRC are transmitted as generated. This means that such parity bits could be scrambled with information on the number of PUCCH repetitions (or whether to perform repetitions), without substantially impacting the DCI and TC-RNTI detection performance.

- PUCCH resource set table

  - Reuse table 9.2.1-1 of 213: [17/Apple] [19/LGE]

    - [19/LGE] Proposal 3. The NTN UE expects the parameter "pucch-ResourceCommon" to indicate one of indexes 11 to 15 in Table 9.2.1-1 in TS38.213, if the PUCCH repetition for Msg4 HARQ-ACK is configured.

  - Define new table: [17/Apple] [19/LGE]

  - [4/ZTE] Proposal 5: The PUCCH format-1 with 14 symbols should be used as the baseline for enhancement.

  - [21/Samsung] Proposal 2: Support at least PUCCH repetition for PUCCH format 1.

- Other repetition factor

  - No new factor: [2/vivo] [6/CATT] [11/OPPO] [12/ETRI] [24/Ericsson]

  - Support larger value: [23/Baicells]

    - [23/Baicells] Based on the SNR requirement (-7.4dB) and CNR value (-8.1dB), we think repetition 8 may not be adequate for LEO-1200 with 30deg elevation angle, if the UE antenna gain is -5.5dBi.

- Inter-slot FH

  - Support: [2/vivo] [10/Intel] [13/Pana] [17/Apple] [19/LGE] [23/Baicells]

    - [2/vivo] Observation 2: The performance gain with inter-slot FH and intra-slot FH decreases with the larger repetition number. Compared to PUCCH repetitions for msg4 HARQ-ACK without frequency hopping, at most 0.2dB and 0.4dB additional performance gain from PUCCH repetitions with inter-slot FH and intra-slot FH can be obtained, respectively.

    - [2/vivo] In msg3 repetition mechanism, inter-slot frequency hopping is supported and the frequency hopping flag information field in UL RAR grant or DCI format 0_0 with CRC scrambled by TC-RNTI is reused.

    - [10/Intel] it may not require additional specification effort as it is already supported for PUCCH repetition with dedicated configuration

    - [13/Pana] For 8 repetitions, NACK-to-ACK error is more critical in case of intra-slot FH due to poor channel estimation under low SNR region. Then, inter-slot FH and JCE can reduce the required SNR by around 1 dB.

    - [19/LGE] Because it is advantageous in terms of frequency diversity, the inter-slot frequency hopping was introduced in Msg3 PUSCH repetition in Rel-17 NR CE. Similarly, we think it is desirable that inter-slot frequency hopping is supported for PUCCH repetition for Msg4 HARQ-ACK

    - [23/Baicells] Observation 7.2: For NTN scenario, intra-slot FH has negative effect on the performance of repetition transmission, because phase continuity is destroyed by intra-slot FH in every half a slot, which is bad for channel estimation. The effect is more significant when SNR is lower. Observation 7.3:  For repetition transmission, one way to obtain the benefit of frequency hopping without bring much negative effect to channel estimation is to perform inter-slot FH instead of intra-slot FH.

  - Not support: [1/HW, HiSi] [6/CATT] [25/Nokia, NSB]

    - [1/HW, HiSi] Observation 2: The performance gain of enabling inter-slot frequency hopping during PUCCH repetitions is less than 0.5dB, in TDL-C channels.

    - [6/CATT] we think that existing repetition factor is enough to meet the coverage enhancement requirement, and it would be a simpler solution. Since gains of DMRS bundling and inter-slot FH are limited for PUCCH

    - [25/Nokia, NSB] Considering that intra-slot frequency hopping is already supported for the PUCCH of the Msg4 HARQ-ACK, we see no immediate need for additional support of inter-slot frequency hopping for such channel. Except for some very specific scenarios, the NTN channel can generally be considered flat in the frequency domain, and therefore the expected gains from additional frequency hopping would be marginal and would not justify support of such a feature.

  - Reuse frequency hopping flag information field in UL RAR grant: [2/vivo]

  - Reuse some field in DCI scheduling Msg4 PDSCH: [2/vivo]

  - Discuss hopping rule: [19/LGE]

- Repetition slot counting for FDD

  - Postpone discussion: [25/Nokia, NSB]

- Others

  - DMRS bundling

    - Support: [4/ZTE] [17/Apple]

      - [4/ZTE] That is, higher performance gain can be achieved with more DMRS sharing in repetitions which can further improve the coverage performance of PUCCH Msg.4 HARQ-ACK.

      - [17/Apple] to further enhance the coverage of PUCCH for Msg4 HARQ-ACK via joint channel estimation over the slots of PUCCH repetition

    - Not support: [1/HW, HiSi] [2/vivo] [6/CATT] [10/Intel] [13/Pana] [25/Nokia, NSB]

      - [1/HW, HiSi] Observation 3: DMRS bundling offers no performance gain on top of repetition transmission for PUCCH of Msg4 HARQ-ACK.

      - [2/vivo] the performance of PUCCH for Msg4 HARQ-ACK with 4 or 8 repetitions can already compensate the gap and meet the requirement of the CNR calculated in link budget calculation. Over-optimizing the performance of PUCCH repetition for msg4 HARQ-ACK is unnecessary

      - [6/CATT] we think that existing repetition factor is enough to meet the coverage enhancement requirement, and it would be a simpler solution. Since gains of DMRS bundling and inter-slot FH are limited for PUCCH

      - [10/Intel] DMRS bundling is specified for PUSCH as an optional UE feature with the corresponding UE capability signalling. Since PUCCH repetition for Msg4 HARQ-ACK corresponds to initial access, support of feature which may not be supported for all the UEs and requires additional UE capability report is not preferred.

      - [13/Pana] Because miss-detection rate becomes more critical when inter-slot FH is used, additional gain of JCE is not obtained although gain of JCE is seen in NACK-to-ACK error rate.

      - [25/Nokia, NSB] The DMRS bundling feature is currently RRC configured and applicable only to UEs in RRC_CONNECTED mode, for improving the reliability of the channel estimation at low SNR range. In addition, support of such feature is not mandatory from a UE point of view, and therefore support of DMRS bundling for the PUCCH of the Msg4 HARQ-ACK would have a large specification impact since it would require additional capability signalling and network configurations in initial access

  - [10/Intel] Proposal 1: Same Tx beam and same PUCCH resource is applied for all the repetitions of PUCCH transmission with HARQ-ACK in response to Msg4 PDSCH

  - DMRS bundling for PUSCH taking into account NTN-specifics

- Phase difference due to timing drift and/or doppler shift

  - Post-compensation by gNB: [1/HW, HiSi] [5/Hyundai] [20/DCM] [22/QC]

  - Pre-compensation by UE / Higher requirement: [5/Hyundai] [9/xiaomi] [20/DCM] [25/Nokia, NSB]

    - [25/Nokia, NSB] It is worth noticing that a residual frequency error of 0.1 PPM (as by RAN4 specifications) may not affect the DMRS bundling in terrestrial networks (TN) much since frequency offset mainly originates due to a misaligntment of the transmitter and receiver local ocillators, which may not be very time sensitive and may be more stable over time to allow a gNB to track such misalignments and compensate for them when receiving an uplink channel.

    - [25/Nokia, NSB] in NTN, the residual frequency offset of 0.1 PPM after UE-precompensation takes into account not only the effect of misalignment between the transmitter and receiver local ocillators but also the residual error from UE estimation and precompensation of doppler shift in the service link, which changes continuously due to both satellite and UE movements. This creates a challenge to the receiver frequency offset estimator, which cannot rely on an averaged estimate of the frequency offset based on multiple observations, especially important in cases of coverage shortage wherein the SNR is very low and the single shot estimates are less reliable.

  - The requirement cannot be met for two adjacent slots: [8/NTPU] [22/QC] [25/Nokia, NSB]

  - The requirement can be met: [9/xiaomi]

    - [9/xiaomi] Assuming the frequency drift is kept with the maximum value during the whole 20 repetitions, the average frequency shift is f_Doppler=1/2 at, where a is the maximum frequency drift which equals to 0.09 ppm/s. Then after 1ms, phase difference between slot p-1 and slot p equals to 0.0468 degree, while after 20ms, phase difference between slot 0 and any slot p equals to or less than 18.72 degree.

  - The requirement cannot be met within an actual TDW with a required size: [13/Pana]

  - Dependent on PUSCH frequency resource: [21/Samsung]

    - [21/Samsung] Proposal 5: Sends LS to RAN4 asking the feasibility of DMRS bundling for NTN when the UE applies TA pre-compensation.

- Whether UE pre-compensation update within an actual TDW is possible

  - Yes: [13/Pana] [22/QC]

    - [13/Pana] Proposal 6: Send LS to RAN4 to ask the feasibility of updating time pre-compensation with keeping the phase different within the maximum allowable phase difference.

  - No: [2/vivo] [5/Hyundai] [9/xiaomi] [14/Sony] [17/Apple] [19/LGE] ([22/QC])

- An event which causes power consistency and phase continuity not to be maintained

  - UE pre-compensation update as the event

    - Yes: [2/vivo] [4/ZTE] [6/CATT] [11/OPPO] [17/Apple] [18/Lenovo] [19/LGE]

      - [2/vivo] Although gNB can try to not allow the common TA to take effect within a nominal TDW by implementation when DMRS bundling is enabled, TACommonDrift and TACommonDriftVariation if configured could still result in the update of common TA.

      - [4/ZTE] In Rel-17 IoT-NTN, to address the impacts of time and Doppler drift for long transmission, the segmented pre-compensation is introduced. The similar mechanism can also be introduced to indicate the proper length for DM-RS bundling, e.g., to address the impact of timing drift/or Doppler shift, the segment pre-compensation segment length less than 6 slots can be considered

      - [6/CATT] timing shift can cause performance reduction, so segment time-frequency pre-compensation should be applied at UL repetition transmission.

      - [11/OPPO] the UE is required to use the UE self-estimated TA and doppler shift to perform UL time and frequency pre-compensation for the service link, which is not known by the network. In this case, phase continuity is not maintained anymore, but the network is not aware of the event.

      - [17/Apple] In IoT NTN, a UE does not adjust its TA value and frequency offset value within an uplink segmented transmission duration. In this case, the uplink PUSCH segment duration is considered in the DMRS time duration window (TDW) configuration, i.e., TDW length is no larger than uplink segment duration.

      - [19/LGE] we propose to introduce the concept of UL segment in Rel-17 IoT-NTN. UL segments can be indicated or configured to UE, so that a UL segment spans one or more DMRS bundling windows.

    - No: [1/HW, HiSi (?)] [13/Pana] [20/DCM] [22/QC (?)] [24/Ericsson (?)]

      - [1/HW, HiSi (?)] Proposal 8: For NR NTN, the nominal TDW should be less than the maximum time duration, in which timing adjustment is not performed while satisfying the timing error requirement from RAN4.

      - [13/Pana] Because such event terminates the actual TDW, its timing should be predictable by gNB to allow for an appropriate joint channel estimation. On the other hand, when to update the time and frequency pre-compensation due to satellite movement is up to UE implementation and is not known to gNB.

      - [13/Pana] Proposal 8: It is not required to introduce “segment” because TDW can work as “segment”.

    - ???: [15/CMCC]

      - [15/CMCC] UE may have the restriction of only change its self-estimated TA in between two actual TDWs. Depending on how frequent UE would perform self-estimated TA pre-compensate adjustment, the priority between the time pre-compensation and DMRS bundling should be discussed.

  - Antenna switching as the event

    - Yes: [1/HW, HiSi]

      - [1/HW, HiSi] Observation 5: The upper bound performance of utilizing DMRS bundling only, i.e. the SNR @2%rBLER with the ideal channel estimation performance, is still 2.4dB worse than the achievable CNR for the Set-1 LEO 1200 PUSCH and elevation of 30 degree for VoIP.

      - [1/HW, HiSi] Observation 16: For PUSCH VoIP, jointly using DMRS bundling with antenna switching can reduce the coverage gap to 0.26 dB compared to the 1.17 dB minimum coverage gap that can be provided by only using antenna switching.

    - No:

  - Indication/update/arrival of epoch time as the event

    - Yes: [2/vivo] [9/xiaomi]

      - [2/vivo] Observation 6: DMRS bundling should be performed within the validity duration.

      - [2/vivo] If the validity timer is restarted due to the update of epoch time within a TDW, the phase continuity and/or power consistency would be violated.

      - [9/xiaomi] In NTN, the UE need to adjust TA at the epoch time of new UL sync assistance information which would break the phase continuity

      - [9/xiaomi] Proposal 6: UE notify the gNB whether the nominal TDW is broken by the new events.

  - Capability report

    - Max TDW duration: [3/MTK] [10/Intel] [20/DCM]

      - [3/MTK] Proposal 4: Re-use Rel-17 NR Coverage Enhancement solution for NTN UE where UE reports the maximum duration expressed in consecutives slots during which power consistency and phase continuity can be maintained.

      - [10/Intel] the capability to maintain the phase and amplitude continuity also depends on the UE implementation for autonomous TA adjustment and frequency adjustment

    - Multiple configurations for multiple elevation angles: [9/xiaomi]

      - [9/xiaomi] the length of the time window that UE can maintain phase continuity is varying with the elevation angle.

    - Whether UE performs pre-compensation update within an actual TDW(?): [22/QC]

- Others

  - Window length

    - Max 16 ms (w/o spec impact): [2/vivo]

    - 4 ms to 20 ms: [3/MTK]

    - Max 6 slots: [4/ZTE]

    - Max 13 slots: [8/NTPU]

    - 10 slots are sufficient: [13/Pana]

    - 10 ms: [15/CMCC]

    - Max 5/10/19 slots: [17/Apple]

      - [17/Apple] Observation 1: The phase difference for 20 PUSCH repetitions is larger than the maximum allowable phase difference for DMRS bundling, given the timing drift rate of 24 ppm. Observation 2: The timing error requirement for 20 PUSCH transmissions in DMRS bundling is satisfied, given the timing drift rate of 24 ppm.

    - Max 8 slots: [24/Ericsson]

  - Additional gaps

    - No: [10/Intel]

      - [10/Intel] At the RAN1#110 meeting, it was agreed to support UE capability on the gaps for segmented UL transmission for both eMTC and NB-IoT NTN. The gaps may be required to allow low-complexity implementation for adjustment of time/frequency shift for UL transmission at the UE. Considering that NR devices allow higher UE complexity compared to eMTC/NB-IoT devices, there is no need to support additional gaps to adjust the UE pre-compensation values. Moreover, support of symbol-level or slot-level gaps may reduce coverage performance of PUSCH transmission

  - RAN4 requirement

    - [14/Sony] Proposal 1: RAN1 should focus on the discussion of physical layer enhancements that do not depend on UE RF implementations, and the discussion should only focus on the NTN-specific characteristics.

    - [22/QC] RAN1 asks RAN4 to consider associated requirement for DMRS bundling for segmented pre-compensation.

    - [24/Ericsson] RAN1 to discuss how to model the timing drift during DMRS bundling in terms of initial timing error at the start of the TDW and maximum timing drift within the TDW.

      - [24/Ericsson] there is likely a timing error already at the start of a transmission of DMRS bundled slots. (e.g., UE position inaccuracy, satellite ephemeris/propagation inaccuracy, TAC granularity, TA adjustment inaccuracy, DL synchronization inaccuracy and other UE-internal sources of error)

  - Details on segment

    - [17/Apple] Proposal 10: The PUSCH segmented transmission duration is configured by the network and indicated by SIB or dedicated RRC signaling with the unit of slot.

    - [17/Apple] Proposal 11: The TDW size of PUSCH DMRS bundling (if configured) is upper bounded by the PUSCH segmented transmission duration.

    - [17/Apple] Proposal 12: If the TDW size of PUSCH DMRS bundling is not configured, then the TDW size is equal to the minimum of maximum duration reported by UE, PUSCH segmented transmission duration, PUSCH repetition duration and the frequency hopping interval for PUSCH DMRS bundling.

  - Indication of pre-compensation update by gNB

    - [18/Lenovo] Proposal 2: Time-frequency pre-compensation at UE side can be indicated by gNB in slot level to satisfy RAN4 requirement.

  - Others

- [2/vivo] Proposal 11: Do not support RAN protocol overhead reduction for NTN coverage enhancement in NR Rel-18.

- [2/vivo] Proposal 12: Circular polarization enhancement on Tx diversity could be studied for downlink coverage enhancements in NR NTN if needed.

- [4/ZTE] Proposal 4: Other enhancements including 2Tx, high power UE and polarization matching should also be supported to improve the performance for VoIP on PUSCH.

- [16/NEC] Proposal 3: At least two sets of RACH resources are required to be configured with different RACH formats or repetitions, to allow both VSAT UEs and commercial UEs gain access to NTN cell.

- [16/NEC] Proposal 4:  Discuss how to configure multiple RACH resource sets per cell for initial random-access procedure:

  - Option-1: Configure two separate UL cells (similar to SUL and NUL) to allow configuring different set of RACH resources for commercial UEs and VSAT UEs

  - Option-2: Configure two sets of RACH resources with different RACH formats/repetitions within initial active BWP

- [18/Lenovo] Proposal 1: Updated K-offset MAC CE is applied at the start of the first repetition of an uplink channel. Proposal 4: Application of updated K-offset is only at the start of the joint channel estimation window.

- [18/Lenovo] Proposal 7: Study the impact of polarization loss on the coverage of initial access procedure. Proposal 8: Study the scenario where gNB adopts different polarization modes to serve UEs with different polarizations modes at different time instances. Proposal 9: Study the association between polarization mode and RS.

- [18/Lenovo] Proposal 10: Study the polarization based scheme for uplink initial access channels.

- [18/Lenovo] Proposal 11: Study the impact of CSI prediction on system performance in NLOS channel conditions.

- [19/LGE] Proposal 11. For NTN coverage enhancement, it can be considered to configure exclusive RACH occasion for low power class UE suffering high path loss.

- [19/LGE] Proposal 12. It can be discussed whether and how to support handling open loop TA (e.g., UE specific TA and/or common TA) during repeated transmission of UL signal/channel (e.g., PRACH).

- [19/LGE] Proposal 13. If antenna polarization configuration of gNB are provided to UE, It can be necessary to discuss how to consider the antenna polarization for open-loop UL power control.

- References

- R1-2210872	Discussion on coverage enhancement for NR NTN	Huawei, HiSilicon

- R1-2211026	Discussions on coverage enhancements for NR NTN	vivo

- R1-2211093	Coverage enhancement for NR NTN	MediaTek Inc.

- R1-2211109	Discussion on coverage enhancement for NTN	ZTE

- R1-2211115	Discussion on coverage enhancement for NR NTN	Hyundai Motor Company

- R1-2211176	Further discussion on UL coverage enhancement for NR NTN	CATT

- R1-2211247	Discussion on coverage enhancements for NTN	Spreadtrum Communications

- R1-2211328	Discussion on DMRS bundling for NR NTN	NTPU

- R1-2211342	Discussion on coverage enhancement for NR-NTN	xiaomi

- R1-2211416	On coverage enhancement for NR NTN	Intel Corporation

- R1-2211460	Discussion on coverage enhancement for NR NTN	OPPO

- R1-2211567	Discussion on coverage enhancement for NR NTN	ETRI

- R1-2211594	Discussion on coverage enhancement for NR-NTN	Panasonic

- R1-2211626	On coverage enhancement for NR NTN	Sony

- R1-2211699	Discussion on coverage enhancement for NR NTN	CMCC

- R1-2211754	Coverage enhancement for NR NTN	NEC

- R1-2211828	Discussion on Coverage Enhancement for NR NTN	Apple

- R1-2211883	Discussion on coverage enhancement for NR NTN	Lenovo

- R1-2211929	Discussion on coverage enhancement for NR NTN	LG Electronics

- R1-2212002	Discussion on coverage enhancement for NR NTN	NTT DOCOMO, INC.

- R1-2212064	On coverage enhancement for NR NTN	Samsung

- R1-2212136	Coverage enhancements for NR NTN	Qualcomm Incorporated

- R1-2212213	Discussion on coverage enhancement for NR NTN	Baicells

- R1-2212325	On coverage enhancements for NR NTN	Ericsson

- R1-2212401	Considerations on coverage enhancements for NR over NTN	Nokia, Nokia Shanghai Bell

- Appendix-1 (Copy from WID RP-222654)

4.1.1 Coverage enhancement The Rel-18 NTN objectives are focused on the applicability of the solutions developed by general NR coverage enhancement to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage). The following sentence will be revisited in RAN#99 as part of the DL enhancements discussion: “The evaluation should also take into account any related regulatory requirements, e.g., ITU limitation of power flux density.” No work on this topic will take place in RAN WGs before the discussion on DL enhancements in RAN#99. The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port). Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO. The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps. The detailed objectives are for NTN: - To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4] - To study DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) and, if necessary, specify enhancements to the Rel-17 procedures [RAN1]

- Appendix-2 (Outcomes of post meetings)

  - RAN1#109-e

Agreement

For NR NTN coverage enhancement, evaluate only handset terminals as UE type.

- i.e., VSAT is not considered.

Agreement

Coverage performance in NR NTN is evaluated according to the following steps.

- Step 1: CNR is calculated as defined in 6.1.3.1 of TR38.821

  - For polarization loss,

    - 3 dB polarization loss is assumed as baseline, and companies are encouraged to report the value and corresponding justification if other value is used

- Step 2: Required SNR of target service is evaluated by LLS

- Step 3: The CNR and the required SNR are compared

Agreement

Coverage performance in NR NTN is evaluated for GEO/LEO-1200/LEO-600 scenarios.

- Note: Service type for each scenario is discussed separately

- Note: Parameter set (Set-1/2) is discussed separately

- Note: MEO can be evaluated optionally

Agreement

For evaluation of coverage performance in NR NTN,

- It is assumed that carrier bandwidth is sufficiently large to transmit each channel.

- Companies are encouraged to report BWP bandwidth, when necessary (e.g. for frequency hopping).

- Note: each channel bandwidth is discussed separately.

Agreement

For VoIP, AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval is used in the evaluations.

- Each packet is transmitted within 20 ms, if packet combining is not used.

  - Companies are encouraged to evaluate at least packet transmission without combining

  - Companies are encouraged to report how to apply packet combining, if used.

  - Note: in packet combining, two packets can be combined into a single packet at TX side

    - Companies should report the impact on E2E latency

- VoIP is evaluated only in LEO scenario.

- Note 1: PRB/MCS/TBS determinations are discussed separately

- Note 2: companies should report if HARQ is used in the evaluations, and if evaluations depart from the assumption that each packet is transmitted within 20 ms

Agreement

Reuse Set-1/2 satellite parameters as in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band, and as in table 6.1.1.1-1/2 of RP-220590 for MEO and S-band.

- In addition, evaluations assuming relevant ITU regulatory limitations on power flux density can be reported in the study phase.

  - Companies should report which value of EIRP density is used and corresponding justification.

Agreement

For link budget calculation, parameters in the following table is assumed.

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2 GHz for DL and UL (S-band) |
| Channel bandwidth | FFS |
| Satellite altitude | 600 km, 1200 km, 10000 km, 35786 km |
| Target elevation angle | [30 (LEO), 12.5 (GEO-Set 1) , 20° (GEO –Set 2), 30° (MEO)] |
| Atmospheric loss | Equation (6.6-8) in [2] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [2] Ionospheric loss: = 2.2 dB (note 1) Tropospheric loss: Table 6.6.6.2.1-1 of [2] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Satellite antenna polarization | Circular polarization |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| Free space path loss | Equation (6.6-2) in [2] |
| Terminal RF parameters | FFS |
| Satellite RF parameters | FFS |
| Polarization loss | As agreed separately |
| Outcome | CNR |
| - NOTE 1: Based on P3 curve for 1% of time from Figure 6.6.6.1.4-1 of [2] after frequency scaling. - dB - NOTE 2: [2] in this table is 3GPP TR 38.811 v15.2.0: "Study on New Radio (NR) to support non-terrestrial networks (Release 15)" |  |

Agreement

If corresponding channel (including SCS) is agreed as evaluation target channel, the following features introduced in Rel-17 Coverage enhancement WI can be applied in coverage evaluation of NR NTN.

- For VoIP, max 20 PUSCH repetitions if SCS = 15 kHz and packet combining/HARQ are not applied; otherwise, max 32 PUSCH repetitions with consideration of the impact on E2E latency

- For low-data rate service, max 32 PUSCH repetitions

- TBoMS

- Joint channel estimation (DMRS bundling)

  - Companies are encouraged to report how to apply

- Max 16 Msg.3 PUSCH repetitions

Agreement

For low-data rate service, the following target data rate is assumed.

- For DL, 3 kbps if satellite EIRP density lower than values in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band, or values in table 6.1.1.1-1/2 of RP-220590 for MEO and S-band due to ITU regulatory limitations on power flux density is considered; otherwise, 1 Mbps

- For UL, 3 kbps and 100 kbps

  - FFS: which data rate applies for GEO/MEO/LEO

Agreement

For NR NTN coverage enhancement, the following channels/signals can be evaluated.

- PUSCH for VoIP

- PUSCH for low data rate service

- PUCCH format 1 with 2 bits

- PUCCH format 3 with 11 bits

- PRACH format 0

- PRACH format 2

- PRACH format B4

- PUSCH Msg.3

- PUCCH for Msg.4 HARQ-ACK

- SSB

- PDSCH for VoIP

- PDSCH for low data rate service

- PDSCH Msg.2

- PDSCH Msg.4

- PDCCH

- Broadcast PDCCH (PDCCH of Msg.2)

Agreement

Evaluate coverage performance for the following UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration.

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | 1 TX, 2TX (optional) / 2 RX with omni-directional antenna element Note: companies should provide their assumption on polarization |
| Polarisation | Linear |
| Rx Antenna gain | [X] dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | [X] dBi per element |

- X = -5 as working assumption

  - Send an LS to RAN4 to ask whether above antenna gain is valid and if invalid, appropriate value.

R1-2205622	[Draft] LS on UE antenna gain for NR NTN coverage enhancement	Moderator (NTT DOCOMO, INC.)

R1-2205623	LS on UE antenna gain for NR NTN coverage enhancement	RAN1, NTT DOCOMO, INC.

Final LS is endorsed in R1-2205623.

Agreement

For coverage performance evaluation, the following elevation angle is assumed.

- 30 deg for LEO, 12.5 deg for GEO-Set 1, 20 deg for GEO-Set 2, as in in Table 6.1.3.2-1 of TR38.821

  - Note: For GEO-Set 1, channel parameters for 10 deg is used in LLS.

- 30 deg for MEO

- Other elevation angles can be evaluated as optional

- Note: these values are elevation angles at the edge of the edge beam.

Agreement

For NR NTN coverage enhancement, evaluate the following cases.

| Case | Satellite orbit | Satellite parameter set | Elevation angle (deg) | Terminal | Frequency band | Service type |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | 1 | 12.5 | Handset | S-band | Low-data rate service |
| 2 | GEO | 2 | 20 | Handset | S-band | Low-data rate service |
| 3 (Optional) | LEO-1200 | 1 | 30 | Handset | S-band | VoIP |
| 4 | LEO-1200 | 2 | 30 | Handset | S-band | VoIP |
| 5 | LEO-1200 | 2 | 30 | Handset | S-band | Low-data rate service |
| 6 (Optional) | LEO-600 | 1 | 30 | Handset | S-band | VoIP |
| 7 | LEO-600 | 2 | 30 | Handset | S-band | VoIP |
| 8 (Optional) | LEO-600 | 2 | 30 | Handset | S-band | Low-data rate service |
| 9 (Optional, with higher priority than case 10) | MEO | 1 | 30 | Handset | S-band | Low-data rate service |
| 10 (Optional) | MEO | 2 | 30 | Handset | S-band | Low-data rate service |

Agreement

For coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread

  - Channel model as in Table 6.1.2-4 of TR38.821, assuming NTN-TDL-A (NLOS) and NTN-TDL-C (LOS)

- Evaluation scenario

  - Rural (LOS/NLOS)

  - Sub-urban (LOS/NLOS) (optional)

- Channel estimation: Realistic estimation

  - Companies are encouraged to report channel estimation method.

- SCS

  - 15 kHz only

- UE speed: 3 km/h

- Frequency drift: Not assumed

- Frequency offset: 0.1 ppm

Agreement

For coverage evaluation of PUSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains | 1, 2 (optional) |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM, CP-OFDM (optional) |
| PUSCH duration | 14 OS |
| Repetitions | w/ type A repetition, optional for type B repetition. The actual number of repetitions is reported by companies. |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| PRBs/TBS/MCS for low data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK, pi/2 BPSK (optional) |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PUCCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 2bits UCI. Format 3, 11 bits UCI |
| Frequency hopping | w/ frequency hopping |
| BLER | - For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. - For PUCCH format 3: BLER for Ack/Nack, SR: 1% BLER for CSI: 1%, optional for 10%. |
| Number of UE transmit chains | 1 |
| DMRS configuration | Number of DMRS symbols for PUCCH Format 3: Reported by companies |
| Repetitions | w/ repetition. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| Number of PRBs | 1 PRB |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PRACH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Format | Format 0, Format B4, Format 2 |
| SCS | Reported by companies. |
| Performance metric | 1% missed detection at 0.1% false alarm probability 10% missed detection: reported by companies if this value is used |
| Number of UE transmit chains | 1, 2 (optional) |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of PUSCH Msg.3 in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| Number of UE transmit chains | 1, 2 (optional) |
| Number of DMRS symbol | w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop |
| Waveform | DFT-s-OFDM |
| HARQ configuration | Whether/How is adopted is reported by companies. |
| PUSCH duration | 14 OS |
| Number of PRBs | 2 |
| TBS | 56 bits |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of SSB in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Periodicity | 20ms |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of PDSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 for 2GHz |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| DMRS configuration | 3 DMRS symbols is used for PDSCH of Msg.2. For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| PRBs/TBS/MCS for low data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK |
| PDSCH duration | 12 OS |
| Payload size for PDSCH of Msg.4 | 1040 bits |
| Other parameters | Reported by companies. |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PDCCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | Reported by companies |
| BLER | 1% BLER optional for 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

  - RAN1#110

Conclusion

For Rel-18 coverage enhancement in NTN, NLOS environment is deprioritized.

Agreement

For NR-NTN coverage enhancement, RAN1 concludes that coverage enhancements specifically for GEO and MEO are de-prioritized in Rel-18.

- Potential enhancements for LEO can also apply to GEO and MEO

Agreement

For NR-NTN coverage enhancement in Rel-18, link budget of parameter set-1 for LEO-1200 operating at LOS is considered as the target to evaluate whether each channel/signal with the existing specification needs to be enhanced or not. The targeted performances are used to evaluate the following services:

- VoIP using AMR 4.75 kbps.

- Low data rate of 3 kbps.

- Potential enhancements for deployments with parameter set-1 can also apply for deployments for parameter set-2

Observation

For PUCCH format 1 with parameter set-1 for LEO-1200 operating at LOS,

- Five sources observed that the existing specification can meet the performance requirement

Conclusion

RAN1 concluded that enhancement is unnecessary for PUCCH format 1 with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PUCCH format 3 with parameter set-1 for LEO-1200 operating at LOS,

- Six sources observed that the existing specification can meet the performance requirement

- One source observed that the existing specification cannot meet the performance requirement with at least 0.6 dB gap

Conclusion

RAN1 concluded that enhancement is unnecessary for PUCCH format 3 with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PUCCH for Msg4 HARQ-ACK with parameter set-1 for LEO-1200 operating at LOS,

- One source observed that the existing specification can meet the performance requirement

- Three sources observed that the existing specification cannot meet the performance requirement with a gap of 1.8 to 6 dB.

Conclusion

RAN1 concluded that PUCCH for Msg4 HARQ-ACK should be enhanced to meet the coverage requirements for parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PUSCH for low data rate of 3 kbps with parameter set-1 for LEO-1200 operating at LOS,

- Eight sources observed that the existing specification can meet the performance requirement

Conclusion

RAN1 concluded that enhancement is unnecessary for PUSCH for low data rate of 3 kbps with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

Observation

For PRACH format 0 with parameter set-1 for LEO-1200 operating at LOS,

- One source observed that the existing specification can meet the performance requirement

- Eight sources observed that the existing specification cannot meet the performance requirement with a gap of 0.3 to 5.3 dB

For PRACH format 2 with parameter set-1 for LEO-1200 operating at LOS,

- Ten sources observed that the existing specification can meet the performance requirement

- Two sources observed that the existing specification cannot meet the performance requirement with a gap of 1.9 to 8.8 dB

For PRACH format B4 with parameter set-1 for LEO-1200 operating at LOS,

- Ten sources observed that the existing specification cannot meet the performance requirement with a gap of 1.2 to 11.9 dB

Note: for the observations above, some sources used 1 Rx antenna and some sources used 2 Rx antennas at the satellite.

Observation

For PUSCH for VoIP with parameter set-1 for LEO-1200 operating at LOS,

- Six sources observed that the existing specification can meet the performance requirement with a margin of 0 to 1.7 dB

  - One company simulated by using 20 repetitions without DMRS bundling

  - Four companies simulated by using 20 repetitions with DMRS bundling

  - One company simulated by using 32 repetitions with DMRS bundling

    - Note: this is the only result using frame combining by application layer

- Nine sources observed that the existing specification cannot meet the performance requirement with a gap of 0.3 to 8.6 dB

  - Eight companies simulated by using 20 repetitions without DMRS bundling

  - Seven companies simulated without frequency hopping

  - One company simulated by using 16 repetitions with DMRS bundling

Note: for the observations above, some sources used 1 Rx antenna and some sources used 2 Rx antennas at the satellite.

Observation

RAN1 concluded that enhancement for PUSCH for VoIP may be needed to meet the coverage requirements for parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain, when DMRS bundling is not applied.

Observation

For Msg3 PUSCH with parameter set-1 for LEO-1200 operating at LOS,

- Eight sources observed that the existing specification can meet the performance requirement

- One source observed that the existing specification cannot meet the performance requirement with a gap of 1.5 dB.

Conclusion

RAN1 concluded that enhancement is unnecessary for Msg3 PUSCH with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

  - RAN1#110bis-e

Agreement

For PUCCH for Msg4 HARQ-ACK,

- Support PUCCH repetition

  - Further discuss the specification impact for at least the following

    - Procedure and signaling (e.g., cell-specific configuration, request to gNB and dynamic indication from gNB, UE capability indication before Msg4, etc.)

    - Repetition factor

    - Repetition slot counting for FDD

  - Further study whether to enhance or support the following

    - Frequency hopping

    - DMRS bundling

Agreement

For PUCCH repetition for Msg4 HARQ-ACK,

- Discuss the following options of procedure to perform repetitions

  - Option 1: UE always performs repetition if configured in cell-specific manner

    - FFS: details of cell-specific configuration

    - FFS: behavior of UE being incapable of repetition

  - Option 2: UE requests repetition and is dynamically indicated to perform repetition

    - FFS: details of repetition request

    - FFS: details of configuration and dynamic repetition indication

  - Option 3: UE indicates repetition capability and is dynamically indicated to perform repetition

  - How UE indicates repetition capability before Msg4

Conclusion

For PUCCH repetition for Msg4 HARQ-ACK,

- The existing mechanism on repetition slot counting (as in section 9.2.6 of TS 38.213) can be applied.

  - FFS: whether specification update to apply the existing mechanism to PUCCH repetition for Msg4 HARQ-ACK is needed.

Agreement

For NTN-specific PUSCH DMRS bundling,

- Discuss further the need of enhancement in consideration of at least the following:

  - Phase difference due to timing drift and/or doppler shift.

    - e.g., whether/how long a UE can meet phase continuity requirement specified as Table 6.4.2.5-1 in 38.101-1 in consideration of frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-5 and timing error specified in Table 7.1C.2-1 of 38.133, whether RAN1 should introduce enhancement to meet the requirement and/or recommend RAN4 to update the requirement or UE should pre-compensate phase difference by UE implementation, etc.

  - An event which causes power consistency and phase continuity not to be maintained.

    - e.g., whether the new event is necessary to determine actual TDW(s) from each nominal TDW or the existing specification can work without any specification change or whether such event may not occur depending on implementations, etc.

  - Note: baseline performance for legacy UEs can include antenna switching

Agreement

For PUCCH transmission for Msg4 HARQ-ACK,

- Supported number of transmissions are 1, 2, 4, 8.

  - Note: single PUCCH transmission is performed as in the existing specification, and/or (if supported for single PUCCH transmission) according to configuration/indication e.g., in signaling with respect to number of transmissions.

  - FFS: whether larger number of transmissions is supported

  - FFS: whether/how single PUCCH transmission can be configured and/or indicated

- Appendix-3 (Contact information)

| Company | Name | Email |
| --- | --- | --- |
| FL (DCM) | Shohei Yoshioka | shohei.yoshioka@docomo-lab.com |
| Lenovo | Hongmei Liu | Liuhm6@lenovo.com |
| Apple | Chunxuan Ye | Chunxuan_ye@apple.com |
| Apple | Chunhai Yao | Chunhai_yao@apple.com |
| Xiaomi | Min Liu | Liumin10@xiaomi.com |
| Xiaomi | Yajun Zhu | zhuyajun@xiaomi.com |
| vivo | Zhipeng Lin | zhipeng.lin@vivo.com |
| vivo | Yong Wang | wy.wang.5g@vivo.com |
| Nokia | Frank Frederiksen | Frank.frederiksen@nokia.com |
| OPPO | Hao LIN | lin.hao@oppo.com |
| OPPO | Zuomin WU | wuzuomin@oppo.com |
| OPPO | Nande Zhao | zhaonande@oppo.com |
| Huawei, HiSilicon | Xiaolei TIE | tiexiaiolei@huawei.com |
| Huawei, HiSilicon | Ying Chen | chenying18@huawei.com |
| Huawei, HiSilicon | Xinghua Song | songxinghua@huawei.com |
| ZTE | Fangyu Cui | cui.fangyu@zte.com.cn |
| CATT | Deshan Miao | miaodeshan@catt.cn |
| Ericsson | Stefan Eriksson Löwenmark | stefan.g.eriksson@ericsson.com |
| Thales | Mohamed EL JAAFARI | mohamed.el-jaafari@thalesaleniaspace.com |
| Spreadtrum | Zhenzhu Lei | reven.lei@unisoc.com |
| MediaTek | Gilles Charbit | Gilles.charbit@mediatek.com |
| InterDigital | Moon-il Lee | Moonil.lee@interdigital.com |
| Sony | Samuel Atungsiri | Sam.Atungsiri@sony.com |
| Lockheed | Robert Olesen | robert.l.olesen@lmco.com |
| ETRI | Dukhyun You | dhyou@etri.re.kr |
| ETRI | Jung-Bin Kim | jbkim777@etri.re.kr |
| ETRI | Gyeongrae Im | imgrae@etri.re.kr |
| Panasonic | Akihiko Nishio | nishio.akihiko@jp.panasonic.com |
| Samsung | Sungjin Park | sj100.park@samsung.com |
| Samsung | Carmela Cozzo | carmela.c@samsung.com |
| Omnispace | Ron Olexa | rolexa@omnispace.com |
| NEC | Pravjyot Singh Deogun | pravjyot.deogun@emea.nec.com |
| Ligado | Clive Packer | clive@ligado.com |
| Hughes/EchoStar | Munira Jaffar | Munira.Jaffar@EchoStar.com; munirajaffar@hughes.com |
| Qualcomm | Xiao Feng Wang | wangxiao@qti.qualcomm.com |
| Qualcomm | LiangPing Ma | lpma@qti.qualcomm.com |
| Novamint | Thierry Bérisot | tberisot@novamint.com |
| GateHouse | Robert van der Pool | rvp@gatehouse.com |
| FGI | YenHua Li | danielli@fginnov.com |
| LG | Haewook Park | haewook.park@lge.com |
| LG | Seokmin Shin | seokmin.shin@lge.com |
| LG | Duckhyun Bae | duckhyun.bae@lge.com |
| NTT DOCOMO | Yoshinori Ojima | yoshinori.ojima@docomo-lab.com |
| Baicells | Xiang Yun | yunxiang@baicells.com |
| Baicells | Yong Ding | dingyong@baicells.com |
| Sharp | Tomoki Yoshimura | yoshimurat@sharplabs.com |