3GPP TSG RAN WG1#111			R1-2212830

Toulouse, France, Nov 14th – 18th, 2022

Title:	LS to RAN4 and RAN2 on L3-RSSI measurement for NR up to 71GHz

Release:	Rel-17

Work Item:	NR_ext_to_71GHz-Core

Source:	RAN1

To:	TSG RAN WG4, TSG RAN WG2

Cc:

Contact person:	Jing Sun

jingsun@qti.qualcomm.com

# 1	Overall description

In RAN1 #110, RAN4 sent an LS response with clarification of the following:

“When a UE has no serving cell in FR2-2, it is not clear if the explicit TCI state should be configured to the UE for FR2-2 RSSI measurement. If explicit TCI state should be configured, how does the UE use such explicit TCI?”

In RAN1 #110b-e, RAN1 responded in R1-2201590 to the above question as follows

- When a UE has no serving cell in FR2-2, the UE does not expect that a TCI-state is provided in RMTC-Config for inter-frequency RSSI measurement on FR2-2.

- For a UE that has no serving cell in FR2-2 and configured with inter-frequency RSSI measurement in FR2-2, it is up to UE implementation how to determine the spatial domain filter for the inter-frequency RSSI measurement in FR2-2.

In RAN1 #111, RAN1 further agreed the following

- For intra-frequency and inter-frequency RSSI measurements for FR2-2, when a UE has a serving cell in FR2-2, the UE does not expect to be configured with an explicit TCI-state in RMTC-Config with a reference serving cell in FR1 or FR2-1.

# 2	Actions

To TSG RAN4 and RAN2

ACTION: 	RAN1 respectfully asks RAN4 and RAN2 to consider the above agreement and capture it in RAN4 and RAN2 spec.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #112	Feb 27 – March 03, 2023 		Athens, GR

TSG RAN WG1 Meeting #112-bis-e	April 17 – April 26, 2023	 	Online