3GPP TSG RAN WG1 #111	R1-2212963

Toulouse, France, November 14th – 18th, 2022

Title:	LS on interference modelling for duplex evolution

Response to:

Release:	Rel-18

Work Item:	FS_NR_duplex_evo

Source:	RAN1, CMCC

To:	RAN4

Cc:

Contact person:	Fei Wang

wangfei@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1		Overall description

In RAN1#111, RAN1 made the following agreements.

Agreement-1

Regarding the modelling of inter-site gNB-gNB co-channel inter-subband CLI agreed in RAN1#110bis for the case that both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the second part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at one UL RB, caused by receiver selectivity at victim gNB, can be modelled as

Iselectivity=z0n,z1n,…,zNR-1nT

  - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise

  - σz,n2=PblockerICSBS×NDLRB

  - Pblocker=1NR*m∈Used DL RBsHCLImWmsm2

  - HCLIm is the NR×NT channel matrix between aggressor gNB and victim gNB at DL RB m, the analog beams of the aggressor gNB and the victim gNB can be taken into account by HCLIm,

  - Wm is the digital precoder at DL RB m at aggressor gNB, WmF=1,

  - sm is the symbol transmitted at DL RB m at aggressor gNB with transmission power for each layer as Ptxper-RB=Ptxmax/NDLRB.

  - NDLRB is the total number of DL RBs in the DL subbands,

  - RAN1 can assume ICSBS (in channel selectivity) is given by gNB ACS unless further RAN4 guidance is received.

    - Send LS to RAN4 to confirm RAN1 understanding and check whether ICSBS can be modelled depending on the value of the blocker interference, e.g.,

ICSBS= ICS1            Pblocker<P1 ICS2,   P1< Pblcoker<P2ICS3             Pblocker>P2

  - Note: Pblocker can be reported by companies

Agreement-2

For SLS in RAN1, for co-site inter-sector co-channel inter-subband CLI modelling, reuse similar method as gNB self-interference modelling as follows.

Ico-siteper-RB=Ico-site-sector-1per-RB+Ico-site-sector-2per-RB

Ico-site-sector-xper-RB=Ptxsector-x, per-RBαco-site*Nused-DL-RBsector-xNDLRB

- Ptxsector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxsector-x, per-RB=Ptxsector-x, max/NDLRB.

- Ptxsector-x, max is the maximum DL Tx Power of sector x on the two DL subbands (in linear scale).

- NDLRB is the total number of DL RBs in the DL subbands.

- Nused-DL-RBsector-x is the number of DL RBs allocated for DL transmission of sector x.

- αco-site is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI.

  - 10*log10αco-site=spatial isolationdB+10*log1011ACLRBS+1ACSBS+…

    - Note: ACLRBS and ACSBS are in linear scale. gNB ACLR (i.e.,ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e.,ACSBS) is provided as the candidate for Receiver impairment.

  - Companies shall report the value of αco-site assumed in the simulations with feasibility of how these values were derived.

  - Send LS to RAN4 confirming the model and asking the value ranges for spatial isolation, and values of  10*log10ACLRBS and  10*log10ACSBS.

Agreement-3

For SLS in RAN1, regarding Tx leakage model of UE-UE co-channel inter-subband CLI modelling, Option 1 is used as starting point.

- Option 1: RAN1 to take in-band emission (IBE) defined in TS38.101-1 and TS38.101-2 as starting point.

- Send LS to RAN4 to ask them whether it can be modelled as an equivalent frequency flat model (e.g., IBEUE,ave) based on RAN4 IBE requirement, and if possible, what is the value of IBEUE,ave

# 2		Actions

To: RAN4

ACTION: 	RAN1 respectfully asks RAN4 to provide feedback on the questions above.

# 3		Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #112                                   	27th Feb – 3rd Mar, 2023			               Athens, GR

TSG RAN WG1 Meeting #112-bis-e                           17th Apr – 26th Apr, 2023			               E-meeting