3GPP TSG RAN WG1 Meeting #112	R1-2300182

Athens, Greece, February 27th – March 3rd, 2023

Source:	ZTE

Title:	TA enhancement for multi-DCI

Agenda Item:	9.1.1.2

Document for:	Discussion and Decision

- Introduction

In RAN#94-e meeting, a new Rel-18 WID on MIMO evolution for both DL and UL was reached as in [1]. As identified below, TA enhancement for multi-DCI based MTRP operation is one of the objectives according to the WID.

- Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation. - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this contribution, we concentrate on some potential methods to facilitate timing advance enhancements.

- Discussion

  - Association between TAG and UL channels/signals

In RAN1#110b-e meeting, the following agreement on the association between TA and UL channels/signals has been endorsed [2].

Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, the four options agreed in RAN1#110bis-e are refined as below (down-selection of one or a combination of the options to be performed in RAN1#111): - Option 1: Associate TAG to TCI-state/spatial relation - Configure TAG ID as part of UL/joint TCI state or spatial relation - for UL transmission, the TAG ID associated with the UL/joint TCI state or spatial relation is utilized - Option 2: Associate TAG to CORESETPoolIndex - for dynamically scheduled/activated PUSCH, TAG associated with the CORESET pool index of the CORESET carrying the scheduling/activating PDCCH is utilized for UL transmission - for Type 1 CG, P/SP-SRS, and P/SP-PUCCH, coresetPoolIndex is RRC-configured. - FFS: Other signals/channels: AP-SRS, and dynamic HARQ-ACK - Option 3: Associate TAG to SSB group (if such an association is agreed in agenda 9.1.1.2). For a UL transmission, UE adopts the TAG associated with the SSB group such that - if the PL RS is an SSB, then the UE adopts the TAG associated with the SSB group which the PL RS of the UL transmission belongs to - if the PL RS is a CSI-RS, then the UE adopts the TAG associated with the SSB group which the QCL source SSB of the PL RS belongs to - Option 4: TAG association performed as follows: - for dynamically scheduled/activated channels/signals, TAG associated with the CORESET pool index of the CORESET carrying the scheduling PDCCH is utilized for UL transmission - for P/SP UL channels / signals (not scheduled or activated by DCI), TAG ID is RRC-configured.

- A total of four options were raised as the candidate solutions for associating TAG to target UL channels/signals, and the pros and cons of each option are analyzed as follows.

- Option 1

- Basically, Option 1 can be workable as long as UL TCI-state or spatial relation is present, i.e., in FR2. In contrast, it is intuitive that Option 1 cannot be used in FR1 due to lack of UL TCI-state or spatial relation, and it means separate frameworks in FR1 and FR2 have to be used, which is undesired and should be avoided. On the other hand, if the association is only based on UL TCI-state or spatial relation, it will lead to two error cases, 1) two TAGs associate with one TRP; 2) one TAG associates with two TRPs simultaneously. Both of the two cases deviate from TRP-specific TA enhancement as stated in WID. To address such issue, some further restrictions have to be specified, e.g., UL TCI states or spatial relations associated with the same TRP/CORESETPoolIndex should not be associated with different TAGs. In this way, it is equivalent to one TAG has to be associated with one TRP/CORESETPoolIndex, that is exactly option 2 in fact.

- Option 2

Given that CORESETPoolIndex has been introduced for multi-DCI based multi-TRP operation to identify TRP-ID since Rel-16, where multiple TRP-specific mechanisms (i.e., out-of-order PUSCH/PUCCH, separate HARQ-ACK codebook generation, TDMed MDCI based MTRP PUSCH/PUCCH transmission) are based on CORESETPoolIndex, it is reasonable to reuse CORESETPoolIndex as the identification of TRP to associate TAG with UL channels/signals.

Besides, it has been agreed that a coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value for Rel-18 unified TCI framework extension for M-DCI based MTRP [2]. Therefore, it can also be workable to associate TA with CORESETPoolIndex under Rel-18 unified TCI framework.

To associate TAG with CORESETPoolIndex, the UE can determine timing advance of an UL channel/signal based on the CORESETPoolIndex of the corresponding scheduling/activating DCI or CORESETPoolIndex configured in RRC configuration associated with the UL signal/channel. With regards to the FFS part in Option 2, it can be fulfilled by:

  - For AP-SRS, it is the same to dynamically scheduled PUSCH that the TAG is associated with the CORESETPoolIndex of the scheduling DCI.

  - For PUCCH carrying dynamic HARQ-ACK information, the following should be considered.

      - If the PUCCH only carries HARQ-ACK information which is indicated to be provided in the corresponding PUCCH, the PUCCH resource is determined by the PRI field in the last DCI among all the DCI formats of which PDSCH-to-HARQ feedback timing indicator indicates to provide HARQ-ACK information in the same slot. In such case, TAG associated with the CORESETPoolIndex of the DCI which schedules the PUCCH can be applied, or TAG associated with the CORESETPoolIndex configured for the PUCCH resource can be applied.

      - If the PUCCH carries multiplexed UCI bits, it is complicated to determine which TAG should be associated with the PUCCH transmission based on DCI format, especially when UCI bits with different types and UCI bits associated with different TRPs are multiplexed on the same PUCCH. In such case, TAG associated with the CORESETPoolIndex configured for the PUCCH resource can be applied.

      - Unlike SRS resources, PUCCH resources are not configured per resource type, i.e., aperiodic, semi-persistent and periodic, therefore it is unnecessary to specify different association rules for aperiodic, semi-persistent and periodic PUCCH transmissions.

      - To guarantee the scheduling flexibility of PUCCH spatial relation or beam indication/update, it is better to configure CORESETPoolIndex per PUCCH resource group rather than per PUCCH resource.

- Based on the analysis above, TAG associated with the CORESETPoolIndex configured for the PUCCH resource group should be used for PUCCH transmissions.

- One may argue that Option 2 cannot be workable to the case of cross TRP uplink scheduling, e.g., for the UL transmission which is scheduled by a DCI associated with CORESETPoolIndex 0 but towards another TRP corresponding to CORESETPoolIndex 1, or vice versa. However, such case can be avoided by gNB configuration or implementation. In particular, one option of cross TRP triggering AP-SRS can be to configure CORESETPoolIndex per SRS triggering state set as mentioned in our contribution [3]. In the light of the above, a couple of solutions can be introduced to complete option 2 in the case for cross TRP scheduling.

- Option 3

Given that PL-RS can be an SSB or a CSI-RS and it is configured in multiple RRC information elements, hence too much specification efforts are needed to specify the association between DL RS group and TAG in particular. Option 3 provides a multilevel association rather than a direct association as Option 1 or Option 2, hence more clarifications in the specification are required. Moreover, it can be noted that the association between TAG and DL RS will cause two error cases as that mentioned in Option 1, hence CORESETPoolIndex must be used to identify TRP-specific association additionally. Therefore, instead of associating DL RS group with TAG on top of CORESETPoolIndex, it is better to associate CORESETPoolIndex with TAG directly.

- Option 4

Option 4 is basically equivalent to Option 2, the only difference is that TAG ID rather than CORESETPoolIndex is configured for P/SP UL channels/signals, but this option might not be needed. For two TAs enhancement, two TAGs are supported for two TRPs and CORESETPoolIndex has been used to identify TRP for dynamic uplink signals, therefore the one-to-one correspondence between TAG and CORESETPoolIndex should be there at least for dynamic uplink signals. For P/SP UL signals, it is unclear to not obey the one-to-one correspondence, i.e., P/SP UL signals associated with one TRP have to apply the TA value associated with another TRP. As for some P/SP UL signals which are not dedicated to one CORESETPoolIndex, the association to TAG can be up to UE implementation. In such case, if a TAG ID is configured as in Option 4, it still implies that the P/SP UL signals are associated with one TRP. Thus one-to-one correspondence between TAG and CORESETPoolIndex should be considered as baseline, and then Option 4 is the same as Option 2 in principle.

- Based on the analysis above, the pros and cons of these four options are given in Table 1. Due to Option 2 can provide a unified association between TA and UL signals/channels, and also it is aligned with the framework of Rel-16 multi-DCI multi-TRP as well as Rel-18 unified TCI, it should be adopted to fulfill the association between TAG and UL channels/signals for MDCI based MTRP operation.

Table 1 The pros and cons of the four options

|  | pros | cons |
| --- | --- | --- |
| Option 1 | (1) Good backward compatibility if two TAs enhancement is required for single DCI based multi-TRP operation. | (1) Cannot be workable in FR1. (2) Requires separate frameworks in FR1 and FR2. (3) Lead to some error cases, e.g., two TAGs associate with one TRP, or one TAG associates with two TRPs simultaneously. (4) Considerable specification efforts required. |
| Option 2/4 | (1) Align with Rel-16 multi-DCI multi-TRP framework, including HARQ-ACK codebook generation, DCI-PUSCH/PUCCH out-of-order, TDMed MDCI MTRP, etc. (2) Straightforward and unified association. (3) Workable in both FR1 and FR2. (4) Minor specification efforts required. | (1) May cannot completely work in case of cross TRP scheduling, but such issue can be addressed by gNB implementation or a tolerable specification impact. |
| Option 3 | (1) Unified association due to each uplink signal/channel is associated with a PL-RS. (2) Workable in both FR1 and FR2. | (1) Complex and multilevel association. (2) Lead to some error cases, e.g., two TAGs associate with one TRP, or one TAG associates with two TRPs simultaneously. (3) Considerable specification efforts required. |

Proposal 1: For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support to associate TAG with CORESETPoolIndex corresponding to the scheduling/activating DCI or configured in the RRC configuration (Option 2).

- For dynamically scheduled/activated PUSCH, TAG associated with the CORESET pool index of the CORESET carrying the scheduling/activating PDCCH is utilized for UL transmission.

- For Type 1 CG, coresetPoolIndex is RRC-configured.

- For P-PUCCH, SP-PUCCH, and AP-PUCCH carrying HARQ-ACK, coresetPoolIndex is RRC-configured per PUCCH resource group.

- For P-SRS and SP-SRS, coresetPoolIndex is RRC-configured per SRS resource set.

- For AP-SRS, TAG associated with the CORESET pool index of the CORESET carrying the PDCCH or the CORESET triggering the AP-SRS is utilized for UL transmission.

  - Initial Timing Advance Acquisition

Although TA acquisition based on RACH procedure is being discussed under another agenda item, i.e., A.I. 9.12.2 of Enhancements on TA management to reduce latency, and it has made progresses on acquiring timing advance values based on PDCCH ordered RACH, the design requirements and principles of TA enhancements for these two agenda items are actually different. For instance, two TAs enhancement for multi-DCI multi-TRP mainly focuses on associating TA and TRP under the legacy multi-DCI multi-TRP framework, and most of the enhancements related to TA are basically based on the legacy rather than being re-designed. While TA management for LTM focuses on acquiring timing advance with lower latency, and L1/L2 triggered mobility can be regarded as a new framework different from that of L3 triggered handover, hence quite a lot new mechanisms of TA acquisition are considered, e.g., initial TA acquisition and TA updating are both based on RACH procedure, and RAR can be not needed.

For the issue whether RAR is needed or not in a RACH procedure for TA acquisition, it is being discussed in A.I. 9.12.2 to further reduce latency of TA acquisition in LTM. In one aspect, as analyzed in our contribution [4], transmitting RAR still is simpler and has less specification efforts than not transmitting RAR, and the issue of how to terminate PRACH transmissions in case of RAR is not needed can have giant impacts on random access procedure defined by RAN2. In another aspect, when RAR is not needed in LTM, TA related information will be indicated in cell switch command, and TA values for the candidate cells will not be applied or be valid before receiving the cell switch command. However, TAC indicated by RAR in multi-DCI multi-TRP operation should be applied to adjust uplink transmission timing of the corresponding TRP after it is received by UE. If RAR is not needed, an additional signaling must be specified to replace RAR to indicate TAC.

Based on the analysis above, we recommend to treat TA enhancement for L1/L2 triggered mobility and multi-DCI multi-TRP as two independent designs.

Observation 1: Timing advance management for L1/L2 triggered mobility and two TAs enhancement for multi-DCI multi-TRP should be treated as two independent enhancements.

    - Cross TRP PDCCH order

In RAN1#111 meeting, the following working assumption and agreement have been endorsed [5].

Working Assumption For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI - the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support CFRA triggered by PDCCH order for both intra-cell and inter-cell cases. Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP at least for inter-cell Multi-DCI. - FFS: for intra-cell Multi-DCI - FFS: whether there are any restrictions needed - FFS: if cross TRP RACH triggering is an optional feature

For inter-cell Multi-DCI Multi-TRP, one leftover issue is that cross TRP PDCCH order might lead to latency due to the transfer of the PDCCH order between two cells. However, such delay does not exist for intra-cell Multi-DCI Multi-TRP, hence it is reasonable to support cross TRP PDCCH order for both intra-cell and inter-cell Multi-TRP. Furthermore, in case the link of a TRP to UE is not reliable enough, PDCCH order might be missed by UE for several times, and the corresponding RACH procedure will be triggered with high latency. In such case, if cross TRP PDCCH order sent by another TRP can be allowed to trigger random access procedures of the TRP, PRACH procedures can be more efficient and reliable.

For intra-cell Multi-TRP, one RACH configuration can be used by both TRPs. Based on the analysis in sub-section 2.2.1, RACH resources reserved for PDCCH order are insufficient in some cases, hence it is reasonable to share RACH resources by the two TRPs, and cross TRP RACH triggering should be supported without doubt.

Proposal 2: For multi-DCI based Multi-TRP operation with two TAs enhancement, support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP for intra-cell Multi-DCI.

Since PDCCH ordered RACH triggering depends on network implementation, it is up to gNB to decide whether the transmitted PDCCH order is to trigger RACH procedure towards the same TRP or a different TRP. Meanwhile, supporting cross TRP RACH triggering will not introduce additional UE complexity or impacts on random access procedure, and the main difference is that UE requires to determine RACH resources configured for different TRPs based on an indication in the PDCCH order. Therefore, cross TRP RACH triggering should be a basic feature, and additional UE capability or configuration of enabling/disabling cross TRP RACH triggering is not needed.

Observation 2: For multi-DCI based Multi-TRP operation with two TAs enhancement, additional rules, UE capability and restrictions are not needed for cross TRP RACH triggering.

According to the working assumption that one additional PRACH configuration is supported for each configured additional PCI in inter-cell Multi-TRP, the random access preambles, RACH occasions, and the mapping of SSB to ROs configured for two TRPs can be individual. Therefore, implicitly indicating which PRACH configuration is to be used based on the preamble index, RO index or CORESETPoolIndex associated with the DCI format can not work in cross TRP RACH triggering.

In order to support cross TRP PDCCH order, a TRP related indication should be introduced in PDCCH order for either inter-cell or intra-cell Multi-TRP and then the UE can know the TRP for which the RACH procedure is triggered and the corresponding RACH resources to be used based on the PDCCH order. To align with the principle of association between TA and UL signals/channels in Section 2.1, CORESETPoolIndex should be introduced to identify TRP in this case.

Proposal 3: For multi-DCI based inter-cell Multi-TRP operation with two TAs enhancement, support to introduce an indication of CORESETPoolIndex in DCI format for PDCCH order to indicate which PRACH configuration is to be used in the triggered RACH procedure.

    - Enhancements on random access procedure

In RAN1#110b-e meeting, the following agreements on random access procedure related enhancements were endorsed [2].

Agreement For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, support one of the alternatives (down selection to be done in RAN1#111): - Alt 1: PDCCH scheduling RAR will always be received from serving cell there is no need for additional type 1 CSS configuration per additional PCI - Alt 2: In addition to PDCCH scheduling RAR being received from serving cell, reception of PDCCH scheduling RAR from a TRP corresponding to an additional PCI for a RACH procedure associated to the additional PCI is supported additional type 1 CSS configuration per additional PCI needs to be supported Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support at least one of the following alternatives (down selection to be done in RAN1#111): - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 2: indicate TAG ID as part of PDCCH order - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 4: divide RACH resources into two groups, where for a RACH procedure, if the corresponding RACH resource belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 5: divide preambles into two groups, where for a RACH procedure, if the corresponding preamble belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP - Alt 6: TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order - Alt 7: Each TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the TCI state used to receive the PDCCH order Note: If Alt 1 or Alt 2 is downselected, then it does not preclude indication of two TAG IDs (if supported) Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support enhancements related to indicating TAG ID via absolute TA command: - FFS: whether the indication is implicit or explicit - Detailed indication schemes are FFS - This does not preclude indication of two TAG IDs (if supported) - Note: This applies at least to MSGB in case of C-RNTI

For inter-cell MDCI MTRP operation in Rel-16, it has been concluded that UE is not required to monitor a Type0/0A/1/2 CSS in a CORESET when the active TCI state is associated with a PCI different from serving cell PCI. Unless the practical use case can be identified, the legacy rule should be kept.

Proposal 4: For inter-cell multi-DCI based Multi-TRP operation with two TAs enhancement, additional Type 1 CSS configuration per additional PCI is not needed (Alt 1).

For intra-cell Multi-TRP, additional PRACH configuration for the second TRP is unnecessary in principle, and the PRACH configuration shared by two TRPs is sufficient. Hence, the generated preambles, the configured PRACH occasions, and the mapping of SSBs to ROs should be commonly used by two TRPs. Based on this consideration, seven alternatives for the association between RACH procedure and TAG in intra-cell multi-DCI based Multi-TRP operation are considered as follows.

- Alt 1

- For PDCCH order triggered CFRA, network should be capable of indicating different preamble indexes to random access procedures for different TRPs. Due to PRACH transmission and the corresponding RAR message are associated with a RAPID and a RA-RNTI, implicit association between a RAR message and a TRP can be determined based on the RAPID and RA-RNTI associated with the PRACH transmission in such case. Since PRACH transmission is triggered by gNB, the implicit association can be known at both UE side and gNB side. Therefore, Alt 1 is unnecessary unless network can not guarantee to indicate different preamble indexes for RACH procedures of different TRPs.

- Alt 2

When a TAG ID is indicated in PDCCH order to identify TRP, as analyzed in sub-section 2.2.2, UE can know the TRP for which the RACH procedure is triggered and the corresponding RACH resources to be used. However, CORESETPoolIndex rather than TAG ID should be introduced in PDCCH order to reach a unified design principle.

- Alt 3&4&5

- Alt 3, 4 and 5 provide solutions to associate random access procedure to TRP without introducing additional fields in PDCCH order or RAR, but they require to divide RACH resources into two parts for two TRPs.

In legacy, RACH configurations can comprise RACH-ConfigCommon and RACH-ConfigDedicated, where RACH-ConfigCommon is cell specific and used for CBRA and PDCCH order, and RACH-ConfigDedicated is UE specific and used for CFRA. For intra-cell Multi-TRP operation, only one PRACH configuration might be configured and RACH resources reserved for CFRA might be quite less, and preamble grouping can further decrease the number of available preambles for PDCCH ordered RACH for each of the TRPs.

If CBRA can be triggered, the issue above can be not severe as that in CFRA, but impacts on the decrease of the number of preambles for each TRP still needs evaluation.

- For PRACH transmission, PRACH occasions and the mapping of SSB-RO are specified to transmit PRACH using a proper beam. In some ways, TRP can be regarded as different beam directions from gNB’s perspective, hence it is natural to identify TRP based on the beam directions and to divide SSBs into two groups to associate TAG with RACH procedures.

  - If SSBs for different TRPs can be mapped in the same RO, preambles are divided into two groups accordingly. The issue of insufficiency of CFRA preambles for each TRP can exist as well.

  - If SSBs of different TRPs are mapped in different ROs, PRACH occasions available for PRACH transmissions of each TRP will be divided into two groups accordingly as in Alt 4. When the number of valid PRACH occasions for each TRP decreases, latency to the random access procedure caused by determining a proper RO should be taken into account.

- Alt 6&7

Alt 6 and 7 are related to the discussion in Section 2.1, i.e., to associate TAG/TRP with CORESETPoolIndex or TCI state.

Alt 6 can work in the case that PDCCH order triggers RACH procedure towards the same TRP. When cross TRP RACH triggering is supported in intra-cell Multi-TRP operation, CORESETPoolIndex field should be additionally introduced in PDCCH order as analyzed for Alt 2. UE determines the TAG which is triggered RACH procedure based on the CORESETPoolIndex field in the PDCCH order rather than the CORESETPoolIndex associated with the PDCCH order.

Proposal 5: For multi-DCI based intra-cell Multi-TRP operation with two TAs enhancement, support to associate TAG ID with CORESETPoolIndex, and TAG ID associated with the RACH procedure is determined based on the CORESETPoolIndex indicated in PDCCH order.

According to TS 38.321, Absolute Timing Advance Command is only for PTAG and is associated with the MSGB. For two TAs enhancement, Absolute Timing Advance Command should be used for TAGs containing the SPCell for two TRPs, and TRP related identification should be introduced implicitly or explicitly in Absolute Timing Advance Command to associate the TAC with PTAG of the corresponding TRP.

For inter-cell Multi-TRP, only introducing a TRP related identification field in Absolute Timing Advance Command can associate a MSGB with a TRP due to the individual RACH configuration for serving cell and non-serving cell. For intra-cell Multi-TRP, only if dividing SSBs into two groups is supported and SSBs mapped on the same RACH occasion are from the same group, ROs for two TRPs can be divided accordingly, and then MSGB receptions for two TRPs can be differentiated based on the MSGB-RNTI. Otherwise, introducing a TRP related identification field in Absolute Timing Advance Command is required as well. However, such restriction might not be reasonable enough. Hence, introducing a TRP related identification field in Absolute Timing Advance Command should be supported for both intra-cell and inter-cell Multi-TRP operation.

There is no obvious difference in including TAG ID or CORESETPoolIndex in Absolute Timing Advance Command, since TAG ID is naturally associated with a CORESETPoolIndex as analyzed in Section 2.1. For example, if two TAGs are configured for the PCell and each TAG is associated with a CORESETPoolIndex, UE can apply the TAC to the TAG associated with CORESETPoolIndex indicated by the Absolute Timing Advance Command.

In order to align with the principle of association between TA and UL signals/channels, CORESETPoolIndex should also be used to identify TRP in this case.

Proposal 6: For multi-DCI based Multi-TRP operation with two TAs enhancement, support to introduce an indication of CORESETPoolIndex in absolute TA command.

  - Overlapping handling due to two TAs

In RAN1#110b-e meeting, the conclusion below was endorsed and some further considerations are needed [2].

Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, it cannot always be assumed that both TRPs have knowledge of the overlapping region between transmissions corresponding to the two TAs. - Note: This doesn’t prevent the network from applying scheduling restrictions even if the TRPs have no knowledge of the overlapping region

In legacy rules, when two adjacent slots overlap due to a TA command, the latter slot is reduced in duration relative to the former slot, and such overlapping can happen only when a TAC is received. However, overlapping between two uplink transmissions associated with two TAs can be quite normal and frequent, such dropping rules will lead to re-transmission of plenty of uplink transmissions.

Observation 3: Dropping one of the uplink transmissions associated with two TAs to handle overlapping issue will lead to undesirable performance degradation for uplink transmission.

For overlapping between two uplink transmissions associated with two TAs, assuming that NTA,offset for different TRPs are configured the same, and TACs for TRPs are received and applied by the UE at the same time. If TA is indicated by the TAC to be 0 and 63 for TRP-1 and TRP-2 respectively, the overlapping length will be 63·16·64·Tc, and it is multiples of the CP length and less than the length of an OFDM symbol. Considering initial timing advance acquisition and TA updating for two TRPs are not requested to be synchronized, therefore the overlapping length might be larger than the values analyzed above.

An effective method is to introduce a scheduling restriction by which the UE does not expect to be scheduled an uplink transmission within a time gap, and the overlapping issue can be prevented accordingly. Further details of the time gap are as follows.

  - As shown in Figure 1, the timing advance values are NTA,1 and NTA,2 respectively, and the time gap is Ntg. In order to prevent from uplink overlapping, the start timing of the uplink transmission 2 should be later than the end timing of the uplink transmission 1, which means that NTA,1+Ntg should be larger than NTA,2, in other words, the time gap should be at least larger than NTA,2 - NTA,1.

  - The minimum time gap satisfying the demand changes with the updating of timing advance values, hence determination of the time gap should be considered. For example, a predefined value set can be specified, where multiple {difference between two TAs, time gap} pairs are defined.

Figure 1 Overlapping avoidance between two UL transmissions associated with two TAs

Proposal 7: Regarding overlapping handling for two uplink transmissions associated with two TAs, support to introduce a time gap in which UE does not expect to transmit any uplink signals/channels if STxMP UL transmission is not supported.

- UE does not expect to transmit an uplink transmission associated with a TA value within the time gap starting from the end of an uplink transmission associated with a different TA value.

- FFS: Candidate values of the time gap.

If STxMP UL transmission is to be considered to handle overlapping in case of two TAs, the following aspects should be considered.

  - If two uplink transmissions are logically overlapped, whether to simultaneously transmit them or drop one of them should not be considered here since the collision is not caused by two TAs.

  - If two uplink transmissions are not logically overlapped but actually overlapped due to two TAs, and they are configured to transmit via the different panel, whether to allow simultaneous transmission should be considered in A.I. 9.1.4.1.

  - If two uplink transmissions are not logically overlapped but actually overlapped due to two TAs, and they are configured to transmit via the same panel, it is hardly workable to switch single panel transmission scheme to STxMP scheme due to it is not clear how to apply a new transmission configuration to one of the uplink transmissions. Even if such switching is reasonable, it should be discussed in A.I. 9.1.4.1.

With the considerations above, whether to allow multi-DCI based STxMP UL transmission to handle overlapping issue due to two TAs requires further study.

Proposal 8: For overlapping handling for two uplink transmissions associated with two TAs, the mechanism through multi-DCI based STxMP UL transmission should depend on the outcome of A.I. 9.1.4.1.

  - Other Issues

In RAN1#109-e meeting, the following agreement has been endorsed [6].

Agreement Enhancement on two TAs for UL multi-DCI for multi-TRP operation is supported in Rel-18. Note 1: whether (1) the network signals two TACs or (2) the network signals one TAC and the UE deriving the second TA can be further studied. Note 2: evaluations can be considered on as-needed basis.

Basically, since the indication of TAC is determined by the network based on the measurement of uplink signals from the UE currently, the amount of timing advance adjustment is determined by the network and then the UE can directly apply the received TAC. As the transmission timing of uplink signals is determined using the downlink reception timing as reference, the determination of timing advance by the network can take both the uplink and downlink propagation delay into account.

Alternatively, if the determination of timing advance is up to UE and also based on the difference of downlink reception timing, timing advance value might be twice of the difference of downlink reception timing assuming the same values of downlink and uplink propagation delay. The determination above might be inappropriate when the values of downlink and uplink propagation delay are different (e.g., corresponding to different physical path in a separate TCI indication). Hence the determination of timing advance by the network should be more reliable than that by the UE.

Observation 4: Timing advance adjustment amount should be determined by the signaling from network rather than downlink reception timing measurement by UE.

Indicating individual TACs for two TAGs within a serving cell is a straightforward solution and the legacy timing advance indication framework can be reused. By this way, UE behaviour of adjusting uplink transmission timing for two TRPs are basically the same, i.e., UE applies a received TAC to a TRP according to the association between TRP and TAG.

Once receiving a TAC associated with a TRP, UE adjusts the current timing advance value for the TRP to the new timing advance value based on the corresponding TAC. In such case, TAC indication is entirely individual for both TRPs, hence it can work well even in case of non-ideal back-haul for the assumption of Multi-DCI Multi-TRP operation.

Proposal 9: Regarding determination of timing advance values of two TAGs within a serving cell, support to indicate two individual TACs, where each TAC is associated with a TAG.

In RAN1#110 meeting, the following agreement on supporting two TAGs within a serving cell has been endorsed [7].

Agreement For multi-DCI based multi-TRP operation with two TAs, support configuring two TAGs belonging to a serving cell.

In the current specification, up to four TAGs can be configured, hence it is natural to double the maximum number of TAGs to be 8 when two TAGs can be configured.

Proposal 10: For multi-DCI based multi-TRP operation with two TAs, up to 8 TAGs can be configured.

The index of TAG including the primary serving cell is specified to be 0 in legacy, and the corresponding TAG is regarded as the PTAG. When two TAGs are configured within the primary serving cell, the definition of PTAG in this case and the number of PTAGs (e.g., 1 or 2) should be clarified. Consequently, the follows can be taken into consideration:

  - If only one PTAG is supported, the TAG associated with the primary serving cell and CORESETPoolIndex = 0 is the PTAG.

  - If two PTAGs are supported, both of the two TAGs associated with the primary serving cell are the PTAGs.

Proposal 11: For multi-DCI based multi-TRP operation with two TAGs, RAN1 shall further study how many PTAGs can be supported.

Time alignment timer (TAT) is configured in TAG-Config per TAG in the current specification and is related with the determination of whether uplink timing is synchronized or not. In one aspect, it has been agreed that two TAGs can be configured within a serving cell, and each TAG corresponds to one TRP, hence configuring TRP-specific TAT in the corresponding TAG-Config is a straightforward option. In another aspect, considering the non-ideal back-haul characteristic might be there for multi-DCI multi-TRP operation, starting or restarting the timer can be asynchronous, hence it is not reasonable to run one timer for two TAGs.

Proposal 12: For multi-DCI based multi-TRP operation with two TAs enhancement, support to configure time alignment timer (TAT) per TRP for TAGs within a serving cell in the perspective TAG-Config as legacy.

If timing advance command is separately indicated for each TRP, the reception timing of TAC for each TRP may differ. Hence it is unavoidable that the TAT of one TRP expires and the TAT of another TRP does not, and UE’s behaviour in such case should be further considered, e.g., only to cancel the uplink transmission associated with TRP of which the TAT expires, or fallback to single TA operation, i.e., both TRPs share the same timing advance. Based on the analysis above, whether one or two PTAGs can be supported should be discussed, accordingly UE behaviour in case of TAT associated with a PTAG expires should be considered.

Proposal 13: For multi-DCI based multi-TRP operation with two TAs enhancement, RAN1 shall study UE behaviour in case of TAT expiry if TAT is configured per TAG as legacy.

- Conclusion

In this contribution, we discuss the potential candidate solutions and alternatives to support two TAs for multi-DCI multi-TRP operation with the following observations and proposals.

Observation 1: Timing advance management for L1/L2 triggered mobility and two TAs enhancement for multi-DCI multi-TRP should be treated as two independent enhancements.

Observation 2: For multi-DCI based Multi-TRP operation with two TAs enhancement, additional rules, UE capability and restrictions are not needed for cross TRP RACH triggering.

Observation 3: Dropping one of the uplink transmissions associated with two TAs to handle overlapping issue will lead to undesirable performance degradation for uplink transmission.

Observation 4: Timing advance adjustment amount should be determined by the signaling from network rather than downlink reception timing measurement by UE.

Proposal 1: For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support to associate TAG with CORESETPoolIndex corresponding to the scheduling/activating DCI or configured in the RRC configuration (Option 2).

- For dynamically scheduled/activated PUSCH, TAG associated with the CORESET pool index of the CORESET carrying the scheduling/activating PDCCH is utilized for UL transmission.

- For Type 1 CG, coresetPoolIndex is RRC-configured.

- For P-PUCCH, SP-PUCCH, and AP-PUCCH carrying HARQ-ACK, coresetPoolIndex is RRC-configured per PUCCH resource group.

- For P-SRS and SP-SRS, coresetPoolIndex is RRC-configured per SRS resource set.

- For AP-SRS, TAG associated with the CORESET pool index of the CORESET carrying the PDCCH or the CORESET triggering the AP-SRS is utilized for UL transmission.

Proposal 2: For multi-DCI based Multi-TRP operation with two TAs enhancement, support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP for intra-cell Multi-DCI.

Proposal 3: For multi-DCI based inter-cell Multi-TRP operation with two TAs enhancement, support to introduce an indication of CORESETPoolIndex in DCI format for PDCCH order to indicate which PRACH configuration is to be used in the triggered RACH procedure.

Proposal 4: For inter-cell multi-DCI based Multi-TRP operation with two TAs enhancement, additional Type 1 CSS configuration per additional PCI is not needed (Alt 1).

Proposal 5: For multi-DCI based intra-cell Multi-TRP operation with two TAs enhancement, support to associate TAG ID with CORESETPoolIndex, and TAG ID associated with the RACH procedure is determined based on the CORESETPoolIndex indicated in PDCCH order.

Proposal 6: For multi-DCI based Multi-TRP operation with two TAs enhancement, support to introduce an indication of CORESETPoolIndex in absolute TA command.

Proposal 7: Regarding overlapping handling for two uplink transmissions associated with two TAs, support to introduce a time gap in which UE does not expect to transmit any uplink signals/channels if STxMP UL transmission is not supported.

- UE does not expect to transmit an uplink transmission associated with a TA value within the time gap starting from the end of an uplink transmission associated with a different TA value.

- FFS: Candidate values of the time gap.

Proposal 8: For overlapping handling for two uplink transmissions associated with two TAs, the mechanism through multi-DCI based STxMP UL transmission should depend on the outcome of A.I. 9.1.4.1.

Proposal 9: Regarding determination of timing advance values of two TAGs within a serving cell, support to indicate two individual TACs, where each TAC is associated with a TAG.

Proposal 10: For multi-DCI based multi-TRP operation with two TAs, up to 8 TAGs can be configured.

Proposal 11: For multi-DCI based multi-TRP operation with two TAGs, RAN1 shall further study how many PTAGs can be supported.

Proposal 12: For multi-DCI based multi-TRP operation with two TAs enhancement, support to configure time alignment timer (TAT) per TRP for TAGs within a serving cell in the perspective TAG-Config as legacy.

Proposal 13: For multi-DCI based multi-TRP operation with two TAs enhancement, RAN1 shall study UE behaviour in case of TAT expiry if TAT is configured per TAG as legacy.

- References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator)

- Chair’s Notes RAN1#110bis-e v16

- R1-2300181, Enhancements on unified TCI framework extension for multi-TRP, ZTE

- R1-2300189, Enhancements on TA management to reduce latency, ZTE

- Chair’s Notes RAN1#111 v15

- Chair’s Notes RAN1#109-e v19

- Chair’s Notes RAN1#110 v21