3GPP TSG RAN WG1 #112		          R1-2300205

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:     9.1.2

Source:	Spreadtrum Communications

Title:	Discussion on CSI enhancement for high/medium UE velocities and coherent JT

Document for:	Discussion and decision

Introduction

In RAN1#111 meeting, RAN1 has discussed on Type-II codebook refinement for CJT mTRP, Type-II codebook refinement for high/medium UE velocities exploiting time-domain correlation/Doppler-domain information, and UE reporting of time-domain channel properties (TDCP) measured via CSI-RS for tracking [1]. During the meeting, some important agreements have been achieved.

In this contribution, we provide our view on the detailed design for each of the features.

Discussion

CSI enhancement for CJT

On W2 quantization group and Strongest Coefficient Indicator (SCI) design, the following agreements have been agreed.

Agreement On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group and Strongest Coefficient Indicator (SCI) design, for each layer: - One (common) SCI applies across all N CSI-RS resources - Further down-select one from the following alternatives by RAN1#110bis-e: - Alt1. One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2) - FFS: Amplitude quantization table considering transmission power difference between multiple TRPs - For each of the amplitude groups (other than the group associated with the SCI), the reference amplitude is reported - Alt3. One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N) - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported FFS: The need for “strongest” TRP/TRP-group indicator in addition to the SCI Agreement On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group, for each layer: - Support the following: (Alt1) One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2) - FFS: Amplitude quantization table enhancement - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported - Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111) - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N) - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported - If the support Alt3 in addition to Alt1 is confirmed, only one of the two schemes will be a basic feature for UEs supporting Rel-18 Type-II CJT codebook

Regarding the working assumption, we support to confirm it. The use case of Alt3 is that when the pathloss difference between UE and multiple TRPs are too large, Alt1 will result in low quantization accuracy of NZCs corresponding to weaker TRPs. Besides, for Alt3, UE can perform CSI omission per TRP, and gNB can construct the codebook corresponding to a subset of N TRPs.

Proposal 1: For W2 quantization group design, confirm the working assumption on supporting Alt3.

During the last meeting, whether and how to use per-CSI-RS-resource FD basis selection offset was discussed, and the corresponding agreements can be found below.

Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, study and down select (no later than RAN1#112) only one from the following schemes: - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources - Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) - Alt3. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf,n where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf,n is independently selected across N CSI-RS resources For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FS basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources

Before the start of this meeting, FL triggered an offline discussion on this AI [2]. Based on the discussion, we have similar view with FL that the main difference between Alt2 and Alt3 is the use of per-resource FD basis offset. Also, per-resource FD basis selection offset for Alt3 and per-resource Minit for Alt2 are functionally equivalent since only a resource-common Minit is needed for Alt3. However, Alt3 has more advantages.

Figure 2.1-1. Illustration of per-CSI-RS-resource FD basis selection offset determination

As shown in Figure 2.1-1, for Alt3, before FD basis selection, per-resource FD basis selection offset can be detemined by remapping the strongest FD basis for each resource to FD basis 0. Then a resource-common intermediate FD basis subset is selected with a single Minit. Finially, for each resource, FD basis selection can be independently performed in the resource-common intermediate FD basis subset. Based on the procedure described above, the FD basis remapping operation in Alt3 will increase the possibility that the reference amplitude is the strongest amplitude within each group for both W2 grouping methods. Besides, the overhead of SCI can be reduced as well. What’s more, compared with Alt2, the support of resource-common intermediate FD basis subset is beneficial to achieve unified design for mode-1 and mode-2.

Different from Alt3, Alt1 implies that the FD basis selection is resource-common. Therefore, Alt1 may save some bits by reporting resource-common FD basis indexes at the cost of losing some performance gain. For example, when the delay profiles for each resource are different, the distribution of strong FD basises corresponds to different resources will be different. Without increasing the number of selected FD basises, some of the strong FD basises may not be selected. What’s more, in order to cover the strong FD basises for different resources, the per-resource FD basis selection offset should be used to remap the first selected FD basis to FD basis 0 (rather than remapping the strongest FD basis for each resource to FD basis 0 in Alt3). Compared with Alt3, the remapping operation in Alt1 cannot reduce the overhead of SCI especially when W2 coefficients are grouped per polarization per resource. In summary, our preference is Alt3 since the performance is expected to be very stable the delay profiles for each resource are different.

Proposal 2: Regarding FD basis selection offset for mode-1, support Alt3.

Regarding the location of non-zero coefficients (NZCs), RAN1 has agreed to support separate bitmap per TRP.

Agreement On the Type-II codebook refinement for CJT mTRP, regarding the bitmap(s) for indicating the locations of NZCs, reuse the legacy design. This implies that the size of the bitmap for selected CSI-RS resource n (Bn) is, Bn=2LnMv - FFS: additional mechanism to reduce bitmap overhead for larger N values, e.g. including via Parameter Combination

For Rel.17 Type II PS codebook, it was agreed that the bitmap is not reported if all the coefficients are non-zero, i.e., KNZ=K1*M*rank. Similarly, we suggest to introduce this feature to Rel.18 Type II codebook for CJT. Additionally, the dropping of bitmap can be conducted either per TRP or jointly for all TRPs. Therefore, we have the following proposal.

Proposal 3: For Rel.18 Type II codebook for CJT, the bitmap is absent when all the coefficients are non-zero.

- FFS: The absent for the bitmap corresponding to each CSI-RS resource is determined separately or jointly

Regarding CBSR for Rel.18 Type II codebook for CJT, we think it is straightforward to adopt SD basis subset restriction per resource. For each resource, beam-group-based restriction analogous to Rel.15/16 Type II codebook can be supported. For each SD basis in the beam group, both hard restriction (0 or 1) and sum power ratio can be supported based on UE capability.

Proposal 4: Regarding CBSR for Rel.18 Type II codebook for CJT, for each resource, support beam-group-based restriction analogous to Rel.15/16 Type II codebook.

In RAN1#110b, the following agreement was agreed on CSI measurement and reporting.

Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, - Only CSI reporting over PUSCH is supported - FFS: Whether AP only, or both AP and SP (following legacy), is supported - An associated Resource Setting includes a CMR comprising K≥1 NZP CSI-RS resources from one CSI-RS resource set - Periodic, semi-persistent, and aperiodic NZP CSI-RS are supported - The supported CSI-RS resource parameter settings follow the legacy specification (without additional enhancement) - FFS: Whether or not the K NZP CSI-RS resources are constrained to be in the same slot

Based on the agreement above, there’s a FFS on whether to limit the K CSI-RS resource to be in the same slot. From performance point of view, if K CSI-RS resource are transmitted within the same slot, it is straightforward to calculate PMI for CJT. On the other hand, if K CSI-RS resources are transmitted across multiple slots, the calculated PMI for CJT may not be accurate enough.

In Rel.17, two CMRs within the same CMR pair configured for NCJT measurement hypothesis to be restricted within up to 2 continuous slot(s). For CJT, ideal backhaul and synchronization is assumed for all TRPs. Therefore, the requirement for channel measurement should also be enhanced, we think it’s necessary to limit the K CSI-RS resource to be within the same slot.

Proposal 5: For CJT, support to configure K NZP CSI-RS resources to be transmitted in the same slot.

CSI enhancement for High/medium velocity UE

One of the fundamental issue on CSI for high/medium velocities is whether to support codebook refinement based on Rel-17 FeType-II port selection (PS) codebook.

Agreement in RAN1#109 The work scope of Type-II codebook refinement for high/medium velocities includes refinement of the following codebooks, based on a common design framework: - Rel-16 eType-II regular codebook - Rel-17 FeType-II port selection (PS) codebook FFS: Whether to prioritize/down-select from the two

It has been discussed for several meeting without consensus. The main concern is the additional workload required and the potential overhead caused by transmission of multiple beamformed CSI-RS. In our views, since RAN1 has only a few meetings left for Rel.18, we prefer to focus on completing the CSI refinement based on Rel-16 eType-II regular codebook.

Proposal 6: For CSI enhancement for high/medium velocities, support codebook refinement based on only Rel-16 eType-II regular codebook.

Regarding the W2 quantization scheme, the following FL proposal was discussed during the last meeting.

FL proposal in RAN1#111: For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, reuse the following components of the legacy per-coefficient quantization scheme: - Alphabets for amplitude and phase - Quantization of phase and quantization of differential amplitude relative to a reference, reference amplitude (with SCI determining the location of one reference amplitude), where the reference is defined for each layer and each “group” of coefficients

Since the same quantization scheme has been used for Rel.15/16/17 Type II codebooks, we prefer to reuse the quantization scheme for Rel.18 Type II codebook for high/medium velocities, including alphabets for amplitude and phase, and also differential quantization. We can support the FL proposal.

Proposal 7: For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, reuse the following components of the legacy per-coefficient quantization scheme:

- Alphabets for amplitude and phase

- Quantization of phase and quantization of differential amplitude relative to a reference, reference amplitude (with SCI determining the location of one reference amplitude), where the reference is defined for each layer and each “group” of coefficients

On W2 quantization group and Strongest Coefficient Indicator (SCI) design, the following FL proposal was discussed during the last meeting.

FL proposal in RAN1#111: For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, for each layer: - One (common) SCI (Strongest Coefficient Indicator) applies across all Q selected DD basis vectors - One group comprises one polarization across all Q selected DD basis vectors (Cgroup,phase=1, Cgroup,amp=2) - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported

For Rel-18 Type-II codebook refinement for high/medium velocities, the . We think it is reasonable to divide the quantization matrix W2 into 2 groups according to polarization. Hence, one SCI is needed. Therefore, we support the FL proposal above.

Proposal 8: For the Rel-18 Type-II codebook refinement for high/medium velocities, on the  W2 quantization scheme when N4>1, for each layer:

- One (common) SCI (Strongest Coefficient Indicator) applies across all Q selected DD basis vectors

- One group comprises one polarization across all Q selected DD basis vectors (Cgroup,phase=1, Cgroup,amp=2)

  - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported

For coefficient subset selection, RAN1 has agreed to support •	Q different 2-dimensional bitmaps. Besides, potential overhead reduction on the bitmap design was also proposed. The candidate solutions was collected in the following FL proposal.

Agreement For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, support the following: - Q different 2-dimensional bitmaps are introduced for indicating the location of the NZCs, where the qth (q=1,…., Q) 2-dimensional bitmap corresponds to qth selected DD basis vector - The number of selected DD basis vectors is denoted as Q - This implies that for each layer, the location of NZCs in SD-FD can be different for different selected DD basis vectors. FFS: Further overhead reduction on bitmap(s) FFS: Whether the number of NZCs is upper bounded across all DD basis vectors or per DD basis vector FL proposal in RAN1#111: For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, down-select one from the following alternatives: - Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv - Alt2. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design and further compressed using source-coding (e.g Huffman code) - Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector. - Alt3B: A single 2-dimensional bitmap of size 2LQ to report the selected S pairs of SD components and DD basis vector and a single 2-dimensional bitmap of size MS for indicating the location of the NZCs, where each row corresponds to a selected FD basis vector and each column corresponds to one of the selected S pairs of SD component and DD basis vector. - Alt3C: A single 2-dimensional bitmap of size 2LM to report the selected S pairs of SD component and FD basis vector and a single 2-dimensional bitmap of size QS for indicating the location of the NZCs, where each row corresponds to a selected DD basis vector and each column corresponds to one of the selected S pairs of SD component and FD basis vector. - Alt4. A bitmap that includes bits associated with the set of {(f1, s1, t1)} with d(f1, s1, t1)<=d, where d is the threshold that can be configured by gNB, df1, s1,t1= minf1-f0, Mv-f1-f0+ mins1-s0, L - s1-s0+ mint1-t0, Q - t1-t0. s0, f0 and t0 denotes a reference SD basis index and a reference FD basis index and a reference DD basis index associated with SCI, respectively.

Among the alternatives, Alt1 can be considered as baseline without overhead reduction.

For Alt2, the main difference is to introduce Huffman code for each bitmap. However, since Huffman code has been proposed during Type II refinement for other purpose but not agreed, we tend to support legacy design here.

For Alt3A/3B/3C, two step indication was considered assuming the same 2-dimensional bitmap can be applied for the third dimension. Based on the agreement listed above, only Alt3A/3B are valid since ‘for each layer, the location of NZCs in SD-FD can be different for different selected DD basis vectors’.

For Alt4, the bitmap only indicates a group of coefficients around the reference coefficient. It is assumed that the coefficients far from the reference coefficient are fixed to zero.

In our views, further overhead reduction may not be needed since the PMI overhead is mainly affected by the number of reported NZCs. However, we are open to Alt3A/3B/4 as long as the assumptions are valid and the performance loss is acceptable. In summary, Alt1 is a simple solution and can be considered as default choice if there’s no consensus on supporting the other alternatives.

Proposal 9: Regarding the bitmap(s) for indicating the locations of the NZCs, support Alt1: Q different 2-dimensional bitmaps where each bitmap reuses the legacy design.

Conclusion

In this contribution, we provide our view on CSI enhancement for high/medium UE velocities and coherent JT. The following proposals and observations are achieved:

CSI enhancement for CJT

Proposal 1: For W2 quantization group design, confirm the working assumption on supporting Alt3.

Proposal 2: Regarding FD basis selection offset for mode-1, support Alt3.

Proposal 3: For Rel.18 Type II codebook for CJT, the bitmap is absent when all the coefficients are non-zero.

- FFS: The absent for the bitmap corresponding to each CSI-RS resource is determined separately or jointly

Proposal 4: Regarding CBSR for Rel.18 Type II codebook for CJT, for each resource, support beam-group-based restriction analogous to Rel.15/16 Type II codebook.

Proposal 5: For CJT, support to configure K NZP CSI-RS resources to be transmitted in the same slot.

CSI enhancement for High/medium velocity UE

Proposal 6: For CSI enhancement for high/medium velocities, support codebook refinement based on only Rel-16 eType-II regular codebook.

Proposal 7: For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, reuse the following components of the legacy per-coefficient quantization scheme:

- Alphabets for amplitude and phase

- Quantization of phase and quantization of differential amplitude relative to a reference, reference amplitude (with SCI determining the location of one reference amplitude), where the reference is defined for each layer and each “group” of coefficients

Proposal 8: For the Rel-18 Type-II codebook refinement for high/medium velocities, on the  W2 quantization scheme when N4>1, for each layer:

- One (common) SCI (Strongest Coefficient Indicator) applies across all Q selected DD basis vectors

- One group comprises one polarization across all Q selected DD basis vectors (Cgroup,phase=1, Cgroup,amp=2)

  - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported

Proposal 9: Regarding the bitmap(s) for indicating the locations of the NZCs, support Alt1: Q different 2-dimensional bitmaps where each bitmap reuses the legacy design.

References

[1] Draft_Minutes_report_RAN1#111_v020

[2]