3GPP TSG RAN WG1 meeting #112                                       R1-2300366

Athens, Greece, February 27th – March 3rd, 2023

Title: 	Discussion on 1024QAM capabilities

Source: 	ZTE, Sanechips

Agenda Item:	5

Document for:	Discussion and Decision

- Introduction

RAN2 sent LS [1] about the impacts due to introduction of the reduced 1024 QAM capability pdsch-1024QAM-2MIMO-FR1-r17 based on R2-2212595 [2]. The following agreements and assumptions were made in RAN2:

Agreements in RAN2 [1] Clarify that UE shall at most report one of pdsch-1024QAM-2MIMO-FR1-r17and pdsch-1024QAM-FR1-r17. [2] Assume to Extend supportedModulationOrderDL to include 1024 QAM (confirm with R1). [3] The MIMO layer for 1024 QAM is Min (2, maxNumberMIMO-LayersPDSCH) for the CC where 1024 QAM is reported [4] Assume that Max data rate shall be derived from the higher data rate between 1024 QAM or 256 QAM for CC where 1024 QAM is indicated and the UE support reduced 1024 capability (confirm with R1) [5] Assume to Clarify that both scalingFactor and scalingFactor-1024QAM-FR1-r17 can be included for in one per CC capability and legacy scalingFactor is used when non-1024 QAM is scheduled (confirm with R1)

In this contribution, the impacts due to introduction of the reduced 1024 QAM capability are analyzed..

- Discussion

In the current specification, there are two 1024QAM capabilities as below [3]:

| pdsch-1024QAM-2MIMO-FR1-r17 Indicates whether the UE supports 1024QAM modulation scheme for PDSCH with maximum 2 MIMO layers for FR1 as defined in TS 38.211 [6], MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214 [12]. UE indicating support of this feature shall also indicate support of pdsch-256QAM-FR1. | Band | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |
| pdsch-1024QAM-FR1-r17 Indicates whether the UE supports 1024QAM modulation scheme for PDSCH for FR1 as defined in TS 38.211 [6], MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214 [12]. UE indicating support of this feature shall also indicate support of pdsch-256QAM-FR1. | Band | No | N/A | FR1 only |

The pdsch-1024QAM-2MIMO-FR1-r17 is introduced to indicate 1024 QAM with restricted MIMO layer to reduce the requirement for UE supporting 1024 QAM.

[1] Clarify that UE shall at most report one of pdsch-1024QAM-2MIMO-FR1-r17and pdsch-1024QAM-FR1-r17.

For the first bullet, according to current specification, 1024QAM capabilities of pdsch-1024QAM-FR1-r17 and pdsch-1024QAM-2MIMO-FR1-r17 are signalled per band. When only pdsch-1024QAM-FR1-r17 is signalled, the UE assumes that there is no restriction for the MIMO layer. When only pdsch-1024QAM-2MIMO-FR1-r17 is signalled, the UE assumes that the MIMO layer for 1024QAM should not be larger than 2. Therefore, it is easy to know that when both pdsch-1024QAM-2MIMO-FR1-r17 and pdsch-1024QAM-FR1-r17 are signalled for a band, the UE can support 1024QAM scheduling with no restriction for the MIMO layer. It is okay to clarify that UE shall at most report one of pdsch-1024QAM-2MIMO-FR1-r17 and pdsch-1024QAM-FR1-r17 for a band.

UE shall at most report one of pdsch-1024QAM-2MIMO-FR1-r17 and pdsch-1024QAM-FR1-r17 for a band.

[2] Assume to Extend supportedModulationOrderDL to include 1024 QAM (confirm with R1).

For the second bullet, even though the supportedModulationOrderDL is not extended to 1024QAM in current specification [3], it would work according to the current specification. We agree it may have a forward compatible issue, e.g. if the modulation order higher than 1024QAM was introduced in the future, the absent of supportedModulationOrderDL would not be able to imply 1024 QAM anymore. Therefore, we support RAN2’s assumption to extend supportedModulationOrderDL to include 1024 QAM.

Support RAN2’s assumption to extend supportedModulationOrderDL to include 1024 QAM.

[3] The MIMO layer for 1024 QAM is Min (2, maxNumberMIMO-LayersPDSCH) for the CC where 1024 QAM is reported

For the third bullet, according to the specification in TS 38.331, the value indicated by maxNumberMIMO-LayersPDSCH is not smaller than 2 [4]. If the UE indicates support of pdsch-1024QAM-2MIMO-FR1-r17, the maximum MIMO layer for 1024QAM can be assumed to be equal to 2 and the maximum MIMO layer for other modulation orders is provided by maxNumberMIMO-LayersPDSCH. While maxNumberMIMO-LayersPDSCH is absent, the UE does not support MIMO on this carrier, i.e. the number of MIMO layer is less than 2. So it is necessary to clarify that the maximum MIMO layer for 1024 QAM is Min (2, maxNumberMIMO-LayersPDSCH) for the CC where 1024 QAM is reported.

If the UE indicates support of pdsch-1024QAM-2MIMO-FR1-r17, the maximum MIMO layer for 1024 QAM is Min (2, maxNumberMIMO-LayersPDSCH) for the CC where 1024 QAM is reported.

[4] Assume that Max data rate shall be derived from the higher data rate between 1024 QAM or 256 QAM for CC where 1024 QAM is indicated and the UE support reduced 1024 capability (confirm with R1)

For the fourth bullet, according to current specification, when 1024QAM is indicated and the UE supports reduced 1024 capability, the maximum number of MIMO layers can be assumed as 2 if maxNumberMIMO-LayersPDSCH is not absent. Then the max data rate for 1024QAM with 2 MIMO layers can be less than that of 256QAM and larger MIMO layers. So we support RAN2’s assumption to assume that max data rate shall be derived from the higher data rate between 1024 QAM or 256 QAM for CC where 1024 QAM is indicated and the UE support reduced 1024 capability.

Support RAN2’s assumption that max data rate shall be derived from the higher data rate between 1024 QAM or 256 QAM for CC where 1024 QAM is indicated and the UE support reduced 1024 capability.

[5] Assume to Clarify that both scalingFactor and scalingFactor-1024QAM-FR1-r17 can be included for in one per CC capability and legacy scalingFactor is used when non-1024 QAM is scheduled (confirm with R1)

For the fifth bullet, according to current specification, when 1024QAM is used for data scheduling, only the scalingFactor-1024QAM-FR1-r17 is available for the maximum data rate calculation. And if the legacy scalingFactor and scalingFactor-1024QAM-FR1-r17 are absent, the scaling factor 1 shall be used for 1024QAM. However, because there is no description that scalingFactor is not used for non-1024QAM in [3], whether the legacy scalingFactor or the scaling factor 1 is used for 1024QAM should be clarified if the legacy scalingFactor is presented and scalingFactor-1024QAM-FR1-r17 is absent. Therefore, we support RAN2’s assumption to clarify that both scalingFactor and scalingFactor-1024QAM-FR1-r17 can be included for in one per CC capability and legacy scalingFactor is used when non-1024 QAM is scheduled.

Support RAN2’s assumption to clarify that both scalingFactor and scalingFactor-1024QAM-FR1-r17 can be included for in one per CC capability and legacy scalingFactor is used when non-1024 QAM is scheduled.

- Conclusion

In this contribution, we discuss the LS about 1024QAM capabilities and have the following proposals.

Proposal 1: UE shall at most report one of pdsch-1024QAM-2MIMO-FR1-r17 and pdsch-1024QAM-FR1-r17 for a band.

Proposal 2: Support RAN2’s assumption to extend supportedModulationOrderDL to include 1024 QAM.

Proposal 3: If the UE indicates support of pdsch-1024QAM-2MIMO-FR1-r17, the maximum MIMO layer for 1024 QAM is Min (2, maxNumberMIMO-LayersPDSCH) for the CC where 1024 QAM is reported.

Proposal 4: Support RAN2’s assumption that max data rate shall be derived from the higher data rate between 1024 QAM or 256 QAM for CC where 1024 QAM is indicated and the UE support reduced 1024 capability.

Proposal 5: Support RAN2’s assumption to clarify that both scalingFactor and scalingFactor-1024QAM-FR1-r17 can be included for in one per CC capability and legacy scalingFactor is used when non-1024 QAM is scheduled.

- References

R1-2300019. LS on reduced 1024 QAM capability. 2022-11.

R2-2212595. Discussion on the capability for 1024QAM with 2 layers MIMO. Huawei, Hisilicon, 2022-12.

3GPP TS 38.306 V17.3.0 (2022-12).

3GPP TS 38.331 V17.3.0 (2022-12).