3GPP TSG RAN WG1 #112  														       R1- 2300384

Athens, Greece 27th Feb – 3rd March, 2023

Agenda item:		9.12.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Layer-1 Enhancements for L1/L2-triggered Mobility

Document for:		Discussion and Decision

# Introduction

In order to reduce the mobility latency, in Rel-18 WID on Further NR mobility enhancements, RP-222332 [1], one of the objectives was agreed to specify mechanism and procedures of Layer 1/Layer 2 (L1/L2)-triggered mobility (LTM) which includes the following items:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

In the last two RAN1 meetings on Rel-18 LTM, several issues including measurements, reporting, target cell beam indication, cell switch command, and candidate cell procedures to be performed before cell switch were discussed and some agreements were made. In this contribution, we further discuss these issues and provide our additional views on the open items.

# Discussion

## L1 Measurements of Candidate Cell(s)

### Scenarios

RAN1 has agreed to support intra- and inter-frequency measurements for candidate cells in RAN1 #110-bis-e [4] and RAN1 #111 [3], respectively. Also, in the RAN1 #110-bis-e, RAN1 asked to RAN4 about the scenarios and requirements for intra and inter-frequency measurements for which the following definition of SSB based intra-frequency L1 measurement has been agreed in RAN4#104bis-e [2]:

- For SSB L1-RSRP measurement, follow the definition of L3 measurement:

  - A measurement is defined as a SSB based intra-frequency L1 measurement provided the center frequency and SCS of the SSB of the neighbor cell is the same as SSB of the serving cell indicated in ServingCellConfigCommon

    - Note: RAN4 will revisit the definition based on RAN1/2 conclusion

Further, it was mentioned by RAN4 that the supported scenarios not included in intra-frequency L1 measurement are regarded as inter-frequency L1 measurement. That is, if a SSB of a neighbour cell has the same centre frequency and SCS as the serving cell, then this scenario is an intra-frequency L1 measurement; otherwise, it is an inter-frequency L1 measurement irrespective of which bandwidth part (BWP) is used.

Based on the above reply from RAN4, it seems that the requirements for SSB based L1-RSRP intra-frequency measurements for Rel-18 LTM would be similar to requirements defined for L1-RSRP intra-frequency measurements using SSB(s) of a non-serving cell (a cell with different PCI from serving cell) [section 9.13, TS 38.133] which was used for Rel-17 inter-cell mTRP and beam management.

For the inter-frequency measurements, as per the current assumption, RAN4 considers all the measurements which can not satisfy the intra-frequency measurement requirements. For such measurements, frequency information, e.g., ARFCN-ValueNR, FreqBandIndicatorNR along with other resource config information of the RS need to be provided. Additionally, measurement gaps would also be required for the UE for RF tuning and making measurements on a different frequency.

Proposal 1: For inter-frequency measurements, additional parameters containing the frequency information of the RS such as ARFCN, frequency band indicator, and measurement gaps to enable inter-frequency measurements need to be provided along with the RS information.

### Measurement RS and Metric

RAN1 has made the following agreement in RAN1 111 [3] for the measurement RS and metric

- For candidate cell measurement for Rel-18 LTM,

  - SSB based L1-RSRP is supported for intra-frequency measurement

  - SSB based L1-RSRP is supported for inter-frequency measurement from RAN1 point of view

  - FFS: L1-SINR, CSI-RS based L1-RSRP

As the CSI-RS measurements can be performed over larger bandwidth with shorter periodicity and narrower beams, it would be beneficial to support measurements using CSI-RS of candidate cell(s). L1-RSRP can be supported as the starting point. CSI-RS can be supported as measurement RS for both intra and inter-frequency for Rel-18 LTM. CSI-RS used for beam management can be supported as the starting point. Other types of CSI-RS can be studied and supported based on the progress on other issues. For example, if early TRS or/and CSI measurements for a candidate cell are supported, then CSI-RS used for tracking or/and CSI acquisition can be supported as measurement RSs as well. Similarly, CSI-IM can also be considered if L1-SINR is supported. Note that depending on the RAN1 agreements on the measurement RS and metric, RAN4 would need to define separate requirements for each of the supported scenarios, i.e., intra and inter-frequency for CSI-RS based L1-RSRP.

Proposal 2: For Rel-18 LTM, L1-RSRP using CSI-RS for beam management can be supported L1 measurement of a candidate cell as starting point.

Observation 1: Depending on the RAN1 agreements on supporting L1-RSRP using CSI-RS for intra- or/and inter-frequency measurements, RAN4 would need to define the relevant requirements.

One concern with CSI-RS measurements of a candidate cell can be the relevant configuration of CSI-RSs for the UE. As the CSI-RSs are not always-on signals like SSBs and are typically configured for the connected UEs. Without any additional information of the serving cell’s measuring UE (e.g., location), the candidate cell may either provide the CSI-RS configuration which may have CSI-RS in the directions not relevant to the measuring UE or may configure CSI-RS in all possible directions which would incur unnecessary network overhead, and also UE may need to attempt performing measurements on a larger set of CSI-RSs than needed. There can be multiple ways to handle this issue,

- Only the relevant CSI-RS indices are configured to the UE - the UE first can only be configured with SSB measurements, then based on the SSB measurement reporting, the serving cell can request the target cell to configure relevant CSI-RSs (e.g., associated with the strongest SSBs in terms of QCL reference). After this, the UE can be configured to measure CSI-RSs received from the target cell, or

- The UE can be configured with both SSBs and a larger set of CSI-RS of candidate cells, but it can select a subset of CSI-RSs to perform measurements based on its SSB measurements. For example, it can only perform measurements with CSI-RSs associated with the strongest measured SSB or only the CSI-RS that are associated with the SSBs that UE has detected.

Observation 2: Without any additional information of the UE (e.g., location), a candidate cell’s CSI-RS configuration may not contain CSI-RSs relevant for the measuring UE.

Proposal 3: If CSI-RS measurements with a candidate cell are supported, RAN1 to study mechanism(s) for the configuration of CSI-RSs of a candidate cell relevant to the measuring UE with minimal overhead.

In terms of measurement metric, L1-RSRP should be supported for both intra and inter-frequency scenarios. For an L1-SINR measurement, it typically requires a RS (SSB/CSI-RS) configuration for the channel measurement resource (CMR) and a RS (SSB/CSI-RS) configuration for interference measurement resource (IMR). That may not be an issue since the CSI configuration would typically be formed by the CU which may access to RS configuration of both serving and candidate cells. However, given that the RAN1 has limited TU for this work item, support of L1-SINR may be discussed once the overall framework for L1 measurement configuration is in place for L1-RSRP.

Proposal 4: Support of L1-SINR can be discussed later after the basic overall framework for L1 measurement configuration is in place for L1-RSRP.

### Filtering for L1 measurements

In RAN1# 111, there was no agreement made on filtering of L1 measurements as views were split in terms of importance and handling of this issue. In this section, we provide our views with some simulation results showing the importance of filtering of L1 measurements; however, such operation can be left for the network to handle if we want to minimize the UE impacts.

Given that L1 beam measurements follow more the channel variations than L3 measurements, i.e., L3 filtering introduces delay to L3 measurements, the network can react faster to serving cell signal strength degradations by handing over the UE to another cell. This ensures that the UE is served with the best radio link. However, this comes at the expense of an increase in the number of ping-pongs between cells (forth and back handovers due to fast reaction to temporal changes in the L1 measurements).

As previously captured in our RAN2 contribution [7], LTM suffers from increased number of ping-pongs. In the following we compare the performance of LTM with and without filtering of L1 beam measurements. Specifically, we consider two cases:

- LTM without filtering (marked as L1 in Figure 1), where the decision to trigger the cell change is based on the L1 beam measurements, i.e., trigger the handover if L1 beam measurement of target cell is 3 dB better than that of serving cell, and,

- LTM with filtering (marked as L1-F in Figure 1), where the decision to trigger the cell change is based on filtered L1 beam measurements, i.e., trigger the handover if the filtered L1 beam measurement of the target cell is 3 dB better than that of the serving cell). In this case, additional filtering is applied by MAC to L1 beam measurements (similar to L3 filtering).

The simulations are generated for a network in FR1 with a 3GPP compliant three-sector Urban Macro scenario with 200 meters Inter-site distance (ISD). The network consists of 7 sites with three cells per each site. The users are dropped randomly with 20 UEs per cell and they are moving in a random direction along straight lines with constant speed of 60 km/h. The further details of the simulation scenarios and assumptions can be found in appendix.

As handover preparation delay, 40 ms has been used and the candidate cells are prepared early using L3 filtered measurements (IIR filter). The cell preparation is initiated if the L3 cell quality cell measurement of a candidate cell is weaker than the serving cell quality by at most 1 dB margin – TTT for the A3 preparation event is 160 ms. For LTM, UE can only report the L1-RSRP of the prepared candidate cells and the network can send a lower layer message to trigger the cell change to one of the prepared candidate cells. The interruption time for L1/L2 is set to 1 ms [8], assuming that UE has decoded all the RRC configurations of the target cell beforehand and UE does not need to perform TA acquisition after the cell switch command is received. Note that for the LTM no TTT is considered.

The mobility performance of L1 LTM (without filtering) and L1-F LTM is presented in Figure 1 with respect to normalized number of a) ping-pongs (PPs), i.e., back-and-forth handovers to same cell within short time, b) radio link failures (RLFs), and c) radio link problems (RLPs), i.e., reception of out-of-sync indications from lower layer.

As expected, L1 LTM (without filtering) has a high number of ping pongs due to the use of L1 measurements for mobility decisions. When IIR filtering is performed for L1 beam measurements, the number of ping pongs achieved by L1-F LTM is significantly reduced, i.e., almost a reduction of 36% compared to L1 LTM (without filtering) as shown in Figure 1-a. At the same time L1 LTM and L1-F LTM perform similarly in terms of RLF and RLP (Figure 1-b,c)

Observation 3: Filtering the L1 beam measurements can decrease the number of ping-pongs substantially for LTM.

| (a) | (b) | (c) |
| --- | --- | --- |

Figure 1 Performance evaluation of L1 LTM and L1-F LTM with respect to (a) Ping pongs, (b) RLFs, and (c) RLPs

The higher number of ping-pongs increases the signalling overhead, specially when the target cell is controlled by a DU that is different from the one serving the UE. For instance, the source DU may need to communicate with the target DU (via the CU) before/upon cell switch, e.g., to obtain the RS measurement configuration, and the path needs to be switched from the source cell to the new target cell, etc. Further in case of inter-DU LTM, RLC/MAC need to be re-established and PDCP recovery shall be enabled to re-transmit the PDCP SDU that have not been acknowledged by the UE. As such, high number of inter-DU ping-pongs would increase the latency in re-transmitting the pending packets. This may not be issue for intra-DU where the same RLC/MAC are controlled by the same DU. Therefore, the cost of ping-pong is higher for inter-DU than intra-DU case in terms of signalling overhead and latency in re-transmitting the pending packets.

Observation 4: The cost of ping-pong is higher for inter-DU than intra-DU case in terms of signalling overhead and latency in re-transmitting the pending packets.

One way to solve the above issue is to use different filtering configurations for the measurements performed with candidate cells of different DU compared to the candidate cells of the serving DU. This may be performed either at the UE or at the network side as discussed above. In case of UE side filtering, multiple configurations can be given to the UE to apply for different groups of cells, i.e., candidate cells that are controlled by serving DU or different DU.

Proposal 5: Different L1 filtering configurations are supported for different groups of cells, where one group can contain the candidate cells belong to serving DU and another group can contain the candidate cells belong to different DU than the serving DU.

### Measurement and Reporting Configuration Framework

To provide the measurement configuration of candidate cells, the following options were listed based on the proposals from different companies in RAN1 #111 [3], but no further agreements were made due to the lack of discussion on this issue:

- Option-1: Based on CSI measurement configuration specified in Rel-17 ICBM with necessary enhancements to support inter-frequency measurements.

- Option-2: Independent measurement configuration for all candidate cells decoupled from serving cell configuration, i.e., separate CSI-MeasCofig is configured for candidate cell from the CSI-MeasConfig of serving cell.

- Option-3: Measurement configuration for each candidate cell within its candidate cell configuration.

- Option-4: No explicit RS/PCI information in measurement configuration for candidate cells, e.g., L3 measurement type framework.

In terms of candidate cell config, RAN2 has made the following agreement:

- RAN2 to confirm that the CellGroupConfig IE is (mandatory) needed within an LTM candidate cell configuration

The reporting of measurements RSs associated with the serving and candidate cells needs to be performed to the current serving cell. Currently, each CSI reporting config indicates a CSI resource config which needs to be used for channel measurements, and both reporting and resource configs are part of the same CSI-MeasConfig. In solutions where the measurement RSs (CSI resource config) of a candidate cell are configured separately, i.e., outside of the current serving cell’s CSI-MeasConfig configuration (option 2 and option 3), it is not clear, how the reporting configuration of the serving cell could indicate/link the RSs of that candidate cell. Therefore, enhancements would be needed to enable such association. Also, if the RSs of candidate cells are given within their respective candidate cell configurations, it would incur additional processing overhead at the UE because the UE would need to read and process multiple candidate cell configurations to extract the associated RS configuration information before the cell switch.

Observation 5: For L1 measurement and reporting solutions, where RSs of candidate cells are given within their respective candidate cell configurations, it would incur additional processing overhead at the UE because the UE would need read and process multiple candidate cell configurations to extract the RS configuration information before the cell switch.

Another option (option 4) where Layer 3 (L3) like measurement framework is used for L1 measurements, the UE may not be provided specific RSs and cells to be measured. This may require a significant change to L1 measurement and reporting framework which would be significantly different from Rel-15/16/17 beam management, CSI acquisition framework. Also, in such a setting, reporting overhead may be significant since the UE has to include additional information (e.g., cell identifier) which is undesirable for L1 measurements since the L1 measurements are low layer messages (intended to be small in size) and reported more frequently then L3 measurements (leads significant signalling overhead in the air interface if the message size is increased).

Observation 6: For L3 measurement based framework, significant changes would be required to existing L1 measurements and reporting used for BM and CSI acquisition. Also, the L1 reporting overhead would be much higher in such solution.

Rel-17 ICBM based framework (option-1) allows to configure RSs from both serving cell and up to 7 neighbouring cells within the serving cell configuration. In such configuration, since the maximum number of RSs (from up to 8 cells) configured for a single reporting configuration is up to 64, further mechanisms may be needed to update the UE with a specific set of reported RSs, e.g., remove/or add RS indices, due to UE movements. Continuous update of the CSI resource set by means of RRC Reconfiguration is costly in terms of signalling overhead and delay. This may be resolved by configuring multiple CSI reporting configurations (e.g., semi-persistent CSI reporting configs), each configuring up to 64 RSs, to cover a large set of RSs for LTM, and a different report config may be activated at different times. This kind of framework may work for Rel-18 LTM; however, one main issue with such framework (and also with Option-2 mentioned above) is that since the whole L1 measurement and reporting configuration is given within the serving cell configuration, a cell change would require a complete reconfiguration of L1 measurement and reporting for the new serving cell which would incur significant UE processing cost in case of fast cell changes, i.e., every time when a cell change occurs, the UE may need to read and process a large set of configuration associated with the new serving cell. Also, the current Rel-17 ICBM based framework does not support inter-frequency measurements, therefore, enhancements would be needed for that as well.

Observation 7: Rel-17 ICBM based framework may work for Rel-18 LTM with necessary enhancements for inter-frequency measurements; however, in such framework (and also with Option-2 mentioned above), every time when a cell change occurs, the UE may need to read and process a large set of configuration associated with the new serving cell, which may not be optimal in case of frequent cell changes.

Based on the above analysis, we can say that for LTM, a L1 measurement and reporting configuration needs to be considered which can satisfy the key LTM requirements, i.e., intra and inter-frequency measurements along with the intra-DU and inter-DU measurements from up to 8 cells, fast dynamic switching with minimal configuration change and without incurring any significant additional reporting overhead. In order to achieve all these objectives, one potential approach can be the following.

- In the preparation phase, the UE is provided with a common set of CSI resources consolidated over a set of candidate cells which may be specific for LTM (e.g., LTM-CSI-Resource-set), and it is maintained as a part of the common RRC configuration retained across LTM cell switching (over multiple LTM neighbouring cells). This set may contain detail parameters for each RS in terms of its timing, frequency, and power configuration. The resources from different cells may be differentiated by an identifier (PCI or a shorter local index to save some bit, e.g., LTM-Config-Index, which may also be reused later in the cell switch command to provide target cell identifier).

- For each cell (serving and candidate cells), a cell specific configuration (e.g., delta configuration compared to the common RRC configuration) may contain reporting configuration in terms of uplink resources to be used for reporting, measurement gap configuration to enable inter-frequency measurements, and an initial set of activated RSs (e.g., a bitmap of LTM-Config-Indices) from the common LTM-CSI-Resource-set to consider for measurements

- The initial set of activated RSs may dynamically be changed later (e.g., via a MAC-CE from the serving cell) based on the UE mobility; however, there is no change required to the LTM-CSI-Resource-set, and the reporting configuration.

With the above approach, even when a cell switch occurs, the UE keeps using the same common set of RSs (LTM-CSI-Resource-set), but switches to new serving cell’s specific configuration in terms of reporting, and measurement gap configuration and determines the measurement RSs from the LTM-CSI-Resource-set based on the default activated set of RSs. This approach would require minimal RRC configuration processing at the UE side when cell switches occur, and at the same time enables dynamic activation/deactivation of appropriate set measurements RSs from candidate cells for LTM L1 measurements. Note that if needed the CU may always change the common set of RSs (LTM-CSI-Resource-set) via RRC reconfiguration when for example, a new cell needs to be added or removed, but that is not expected to be done too frequently.

To show the overall picture, an example of RRC configuration for LTM specific to L1 measurement and reporting is shown in Figure 2.

Figure 2 An example of LTM RRC Configuration specific to L1 measurement and reporting

Observation 8: For LTM, a L1 measurement and reporting configuration needs to be considered which can satisfy the LTM requirements, i.e., intra and inter-frequency measurements along with the intra-DU and inter-DU measurements from up to 8 cells, fast dynamic switching with minimal configuration change and without incurring any significant additional reporting overhead.

Proposal 6: For LTM L1 measurement configuration, a common set of RSs across neighbouring cells specific for LTM measurements is provided, where

- detail parameters for each RS in terms of its timing, frequency, and power configuration are given

- the resources from different cells may be differentiated by an identifier (LTM-Config-Index, FFS: which may also be reused later in the cell switch command to provide target cell identifier)

- this configuration may retain across LTM cell switching; however, may be updated by the CU if needed (e.g., adding/removing a cell/RS)

Proposal 7: For LTM L1 measurement configuration, along with a common set of measurement RSs, a cell specific RRC configuration is provided, which includes:

- reporting configuration providing the uplink resource configuration

- measurement gap configuration for inter-frequency measurements

- a set of activated RSs/cells (e.g., a bitmap of LTM-Config-Indices) from the common LTM-CSI-Resource-set to consider for measurements

Proposal 8: A set of activated RSs (e.g., a bitmap of LTM-Config-Indices) from the common LTM-CSI-Resource-se can dynamically be changed later (e.g., via a MAC-CE from the serving cell).

## L1 Measurement Reporting

### Legacy reporting

RAN1 has made the following agreement in RAN1 #111 [3] for the reporting mechanism

- For gNB scheduled L1 measurement report for Rel-18 LTM, report as UCI is supported

  - Semi-persistent report on PUSCH, and aperiodic report on PUSCH are supported

    - FFS: periodic and semi-persistent PUCCH

  - In a single report instance, report for serving cell and candidate cell(s) for intra-frequency and/or inter-frequency can be included.

In Rel-17, RAN1 had a lengthy discussion whether the max number of reported RS in a single reporting instance, should be increased due to the increase of number of PCIs associated with the measured RS but settled on 4 (candidate values being discussed was up to 16). In practice, the increase of maximum number of reported RS may not provide additional benefit since the network typically decides on cell switch/handover based on the highest reported RSs of a cell.

Proposal 9: As a starting point, the maximum number of reported RS in a single reporting instance is 4.

Since the size of a CSI report would be of similar order of Rel-15/16/17 CSI reports, all types of legacy CSI reporting of CSI including the reporting via PUCCH can be supported for Rel-18 LTM, since it uses lower layer mechanism to determine whether to switch the UE to new target cell. It would be up to network implementation to determine which type to configure.

Proposal 10: Support periodic and semi-persistent CSI reporting using PUCCH for Rel-18 LTM.

For inter-frequency measurements, RS configuration with additional frequency information and measurement gaps would be configured in the CSI resource configuration given to the UE where each configured RS would have its dedicated identifier. Therefore, additional identifier in the report to indicate inter-frequency or frequency of the measurement may not be needed.

Observation 9: To support L1 reporting of inter-frequency measurements, additional frequency identifier may not be needed in the report as the associated RS which needs to be measured would be configured with its dedicated identifier in the CSI resource configuration.

In terms of reporting format, it was agreed in Rel-17 to reuse the Rel-15 L1-RSRP reporting format using one absolute 7-bit RSRP and remaining 4-bit differential RSRP value (relative to the absolute value). In order to avoid any new specification changes, the same format can be reused for Rel-18 LTM.

Proposal 11: Rel-17 ICBM and inter-cell mTRP L1 reporting format with one absolute 7-bit RSRP and remaining 4-bit differential RSRP value (relative to the absolute value) can be reused for Rel-18 LTM.

Although it has not been agreed, but a scenario, when a mTRP configuration is allowed to be enabled/triggered within the target cell immediately after the cell switch in order to maintain the high reliability and robustness can be a potential use case for LTM. For such a scenario, it would be useful to allow group-based beam reporting (as in Rel-17) for such candidate cells. However, given the limited TU, detailed enhancements to enable such scenarios can be considered later once the basic framework for LTM is completed.

Observation 10: Enabling Rel-17 group-based beam reporting for a candidate cell in order to trigger a mTRP operation may be useful.

### L1 event triggered reporting

RAN1 has made the following agreement in RAN1 #111 [3] for event-based reporting mechanism:

- For L1 measurement report for Rel-18 L1/L2 mobility, if UE event triggered report for L1 measurement is supported based on further study

  - At least the following aspects may be considered

    - How to define UE event and exact definition of events,

    - Report container

    - Resource allocation/assignment for UE event triggered report

    - Necessity of indication to gNB when the condition UE event is met, and how

    - Necessity to define the condition to start/stop the reporting,

    - Contents of the report/reporting format, PCI, RS ID, measurement result etc.

    - The interaction with filtered L1 measurement results (if supported)

    - Support of simultaneous configuration of both UE event triggered and any of periodic/semi-persistence/aperiodic reporting, and solutions when both of them are configured.

    - Report destination, whether the report is sent to serving cell only or can be sent to one or more candidate cell(s).

    - Benefit when L3 measurement is involved

On the UE Event Definition

For the event definition, at least following high level events could be considered:

Based on the L3 measurements, UE could determine to trigger indication to network that specific reporting configuration (e.g., specific to a certain candidate cell) should be activated. The L3 measurements are performed periodically, thus could be used for the event definition. Alternatively, L1 measurements may also be used for this purpose. The indication of the successful event triggering could be provided using lower layer signalling (e.g., MAC CE) where the UE indicates that specific reporting configuration could be activated. Alternatively, after the event triggering, the UE may start (e.g., autonomously) performing the relevant L1 measurements and reporting, and any indication would be provided implicitly in the L1 measurement report based on the reporting configuration e.g., when UE reports the top N-beams to network.

Proposal 12: Define event based on L3 or L1 measurements where UE determines to start performing L1 measurement on pre-configured L1 measurement RS (e.g., associated with a certain candidate cell) and reporting using the currently active reporting configuration for which the measurement RS are associated.

Report container/ resource allocation for UE triggered event

The resources for event-based reporting would potentially have high overhead, e.g., in case periodic resources are allocated. However, in some cases, depending on the agreed event, legacy L1 containers can be used to provision information if the content of the L1 report format is not changed. As an example, if the UE is configured to initiate/activate a reporting of configured DL RSs of specific candidate LTM cells associated with a reporting instance, e.g., based on observed signal quality of those candidate LTM cells (e.g., using L3 measurements), such event would not require changes to L1 reporting.

Observation 11: Depending on the definition of an event, the existing L1 signaling (L1 reporting container) can carry reported content if the L1 reporting format is not changed.

Observation 12: In case event based measurement results different from legacy L1 report format would need to be provided, a new L1 reporting format may be needed.

Although L1 signaling could be used, for event based reporting, a MAC CE based solution would provide more flexible approach in terms of container size and provision to network. MAC CE based solution would be similar to aperiodic reporting where it would not require periodic / preconfigured resource allocation, and instead, mechanism to request resources (SR /CBRA) may be used. Also, in case UE has UL grant available, it can be used to send the MAC CE.

Observation 13: MAC CE based reporting will provide most flexible container option for event based reporting.

Observation 14: MAC CE based reporting can be carried out using the available UL grant or resources can be requested by currently specified means i.e. SR/CBRA.

Proposal 13: For the UE event-based reporting, support at least MAC CE as a reporting container.

Indication of the triggered event

As discussed above, if the UE is configured to initiate reporting according to current L1 reporting configuration (but based on the event such as consider a set of DL RS of certain candidate cells “valid” for reporting) the NW may not need any additional explicit indication upon event triggering. For such event, the indication would be implicit if a candidate cell DL RS is started being reported as one of the top N-beams in the L1 report.

Observation 15: If the event triggers UE to perform L1 measurements for active/existing L1 reporting configuration, the indication of event is not needed.

In an alternative option, an L1 signal (+ MAC CE) could be assigned for indicating network that specific event has occurred. As an example, a dedicated SR could be assigned for indicating that a criterion for an event (e.g., that candidate cell or at least one RS for a candidate cell configured for reporting is above a quality threshold) has been fulfilled. The detailed information could be provided in the MAC CE. In case UL grant is available, the MAC CE could be provisioned on the available grant.

Observation 16: Dedicated L1 signalling (SR) could be used to indicate that UE intents to provide more information for a specific event.

Proposal 14: In event-based reporting, UE provides an indication to network on which reporting configurations should be activated.

Proposal 15: In event-based reporting, a dedicated L1 signal, SR, can be used to indicate that reporting event has occurred in case UL resources are not available for MAC CE.

Reporting content

The reporting content depend on the agreed event. For example, if the UE is configured to indicate a cell for which a reporting configuration is to be activated, or indicate cells according to the configured event, the cell ID (or the additional cell / LTM cell ID) may be provided.

Alternatively, if for LTM candidate cells, the periodic/S-P reporting is not used, and the reporting is based on the event-based reporting, the content may require measurement results as well.

Simultaneous configuration of event based reporting and L1 beam reporting

Both periodic and event-based reporting should be supported to be simultaneously configured, i.e., UE may be configured with events to indicate whether a specific LTM candidate is detected/eligible for periodic L1 reporting (based on, e.g., L3 measurements) or if some cells would not need to be reported/measured using L1.

Proposal 16: Support simultaneous configuration of any time type L1 reporting and event based reporting. It is up to NW to configure whether to use L1 reporting (e.g., periodic) or event based or both.

Reporting destination

The reporting destination of the event triggered report should the serving cell of the UE. The serving cell is able to configure the UE with appropriate configuration (i.e., activate/deactivate specific measurements).

Proposal 17: The reporting destination of the event triggered report is the serving cell of the UE.

## Beam Indication of Candidate Cell(s)

RAN1 has made the following agreements in RAN1 #111 [3] for beam indication mechanism

Agreement

- The beam indication of candidate cell(s) for Rel-18 LTM should be designed based on the following:

  - Beam indication for Rel-18 LTM is designed based on Rel-17 unified TCI framework, if both serving cell and candidate cell support Rel-17 unified TCI framework

  - FFS: whether/how to design mechanism for Beam indication for Rel-18 LTM when at least one from serving cell and candidate cell supports only Rel-15 TCI framework.

  - Note: How and whether to indicate the new serving cell(s) and timing for beam indication are separately discussed

Agreement

- For beam indication timing for Rel-18 LTM,

  - Support Scenario 2: Beam indication together with cell switch command,

    - For Rel-17 unified TCI framework,

      - Beam indication indicates TCI state for each target serving cell

  - FFS: Scenario 1: Beam indication before cell switch command

  - FFS: Scenario 3: Beam indication after cell switch command

- FFS: Activation of TCI state(s) of target serving and/or candidate cell(s).

Further in terms of serving cell’s awareness of candidate cells’ TCI states, the following assumptions were made by RAN2 [5] and RAN3 [6]:

- RAN2 assumption [5]:

RAN2 assumes that LTM (intra DU and inter DU) is network-controlled mobility where the control is from the source, i.e., measurements (L1 measurements) are configured in the UE from the source Cell, and the decision to switch cell is by the source cell, and enhancements considered for LTM before cell switch, e.g. pre-synchronization, TA handling, target beam management (to the extent it is supported) may be made by the source cell. RAN2 understands that this may require cooperation among source DU, CU, target DU, and/or OAM. RAN2 don’t see any blocking issue to share information between DUs but the support of this is in RAN3 domain. RAN2 see no necessity for a direct inter-DU-interface to support this.

- RAN3 assumption [6]:

Regarding Q3 about L1 measurement and TCI state configurations in the LS, RAN3 agreed that based on the current specification, the serving DU cannot know the measurement RS configuration and TCI state configuration of cells served by another DU.

Any possible consideration for Rel-18 on the coordination over F1 among serving DU, target DU, and CU would need clearly identified requirements from other groups.

It may be common to configure different cells with different TCI state framework (Rel-15 or Rel-17), e.g., the serving cell and a candidate cell may use different TCI state frameworks. One single TCI framework may not be able to be used for beam indication, e.g., in case the target cell (i.e., selected candidate cell for handover) tries to interpret the indicated TCI state based on its own (target cell) configured TCI state pools/lists. For example, a TCI state indicated from the serving cell configuration may not be valid or applicable with respect to the target cell configuration once the UE switches to that cell. Therefore, a pragmatic approach would be to develop a beam indication for Rel-18 LTM which can work for any kind of TCI frameworks.

Observation 17: Different cells may be configured with different TCI frameworks (Rel-15 or Rel-17), e.g., the serving cell and a candidate cell may use a different TCI state framework.

Proposal 18: Beam indication for candidate cell(s) in Rel-18 LTM should be designed such that it can work for any TCI framework, including the scenarios when serving cell and target cell support the same TCI framework (Rel-15 or Rel-17) or the different TCI framework (e.g., one Rel-15 and other one Rel-17).

To enable that one of the potential simple approaches is the UE may be configured such that, once it receives a mobility trigger relating to that target cell, it may consider the TCI State of that target cell to be essentially unknown (e.g., not indicated), or not specifically set, yet still leverage the information provided by the serving cell in relation to that target cell. Such an approach has benefits in relation to support of mobility scenarios since the beam management framework for the target cell does not need to be the same as in the source cell (i.e., the beam indication framework can be R17 in source cell and R15 in target cell and the mobility still works). Furthermore, a UE does not need to be specifically indicated a TCI state that is applied for the target cell (in target cell configuration) before the mobility trigger, thus allowing a reduction in latency in relation to a mobility event.

Before the cell switch, the UE may be provided at least a TCI state associated with the target cell which the UE can use to determine the DL RS indicated by the TCI state or the QCL source (i.e., SSB) of the RS indicated by the TCI state, without interpreting/mapping the TCI state ID to the target cell’s beam management configuration. Alternatively, simply a QCL source RS associated with the target cell can be provided as a beam indication directly to the UE.

The determined RS (e.g., DL RS), via the provided TCI state or QCL source RS, can be used by the UE after the cell switch for monitoring at least one DL channel and/or transmission of at least one UL channel of the target cell. The UE can be configured to consider that once it has completed the RRC reconfiguration (i.e. the handover is complete), it has not been provided with an indication of a TCI state or has not received activation for any TCI state in relation to the target cell (BWP), but it can keep using the determined QCL assumption for monitoring/transmitting the DL/UL channels until it receives a new activation/beam indication from the new target cell. In one option, the UE could carry out the cell change related signalling using the QCL assumption associated with the mobility trigger. With this UE may be able to assume the QCL information of a TCI State for a target cell, before the target cell actually indicates UE to apply a TCI state. In other words, the UE does not need to be pre-activated with TCI States that it applies upon handover/cell switch, i.e., the beam indication does not have to refer to a candidate RRC configuration that is not yet active (before the handover). This could relax UE implementation requirements and simplify network signalling.

Proposal 19: In Rel-18 LTM, at least a TCI state or a QCL source RS associated with the target cell can be provided as a beam indication before the cell switch.

Proposal 20: UE can determine a DL RS via the provided TCI state or the QCL source RS for monitoring at least one DL channel and/or transmission of at least one UL channel of the target cell after the cell switch until it receives a new activation/beam indication from the new target cell.

Proposal 21: The UE can be configured to consider that once it has completed the RRC reconfiguration (i.e. the handover is complete), it has not been provided with an indication of a TCI state or has not received activation for any TCI state in relation to the target cell.

The above approach would work for both scenarios, when serving cell and target cell support Rel-17 TCI framework and when at least one from the serving cell and the target cell supports Rel-15 TCI framework.

In terms of timing of the beam indication, the scenario 2, i.e., beam indication within the cell switch command has been agreed. However, it would be beneficial to support additional scenarios (scenario 1 and 3). For example, if the UE is in a Rel-17 ICBM scenario before cell switch communicating with a non-serving cell for UE-dedicated signals/channels, and if cell switch occurs where the target cell is same as the ICBM non-serving cell, the UE can be configured to use the indicated TCI state of non-serving cell used in the ICBM operation also for the QCL assumption for the target cell (and assume that it has not been indicated to apply a TCI State for the target cell) after the cell switch until it receives a new activation/beam indication from the target cell. This is scenario 1 in terms of timing of the beam indication. Therefore, it makes sense to not rule out scenario 1 to cover all different scenarios.

Observation 18: Scenarios when beam indication is provided before the cell switch command would be beneficial when the UE is in a Rel-17 ICBM scenario before cell switch, and the target cell is same as the ICBM non-serving cell. The UE can be configured to use the indicated TCI state of non-serving cell used in the ICBM operation also for the QCL assumption for the target cell after the cell switch until it receives it receives a new activation/beam indication from the target cell.

Proposal 22: Scenario 1 for timing of beam indication is also supported.

With scenario 3, the network may have more flexibility. In scenario 3, the network may decide to not provide any beam indication to the UE. This may be possible when the network does not have sufficient information for providing the beam indication. In such scenario, after the cell switch command is received, the UE may be configured to trigger a RACH procedure with the target cell to obtain the initial QCL assumption for the target cell’s DL/UL signals/channels. Therefore, it would be beneficial to support scenario 3 as well. For better handling of this scenario 3 along with the scenario 2, the cell switch command could include an indication whether there is a beam indication (a TCI state or a DL RS) is present or not in the command.

Observation 19: Scenarios when the beam indication is not provided the UE would be beneficial to enable more network flexibility. In such scenarios, the UE can be configured to trigger a RACH procedure with the target cell to obtain the initial QCL assumption.

Proposal 23: Scenario 3 for timing of beam indication is also supported. In such scenarios, the UE can be indicated whether there is a beam indication (a TCI state or a DL RS) is present or not in the cell switch command.

## Candidate Cell Procedures before Cell Switch

RAN1 has made the following agreement in RAN1 #111 [3] regarding the preparation for handover before reception of cell switch command:

- Regarding the potential RAN1 enhancements to reduce the handover delay / interruption for Rel-18 LTM

  - Support at least DL synchronization for candidate cell(s) based on at least SSB before cell switch command

    - Further study the necessary mechanism, e.g. signaling and UE capability

In addition to DL synchronization, there can be additional procedures for a candidate cell such as TRS measurements and CSI acquisition which can be beneficial to be performed before the cell switch to minimize the interruption due to cell switch.

In terms of TRS measurement(s), since the QCL-type A source reference signal for PDCCH/PDSCH is typically a TRS, support of early TRS tracking for candidate cell(s) would be beneficial. Besides that, it is also useful for fine time-frequency tracking of the candidate cell(s) (as compared to via SSB measurements). Similarly, since one of the motivations of Rel-18 LTM is to maintain high data transmission efficiency and reliability in the process of fast cell switch, early CSI acquisition for the candidate cell(s) would be beneficial so that the UE would be able to use high data link with the target cell immediately after the cell switch.

Proposal 24: Support early TRS tracking and CSI acquisition of candidate cell(s).

As discussed in the section 2.1.2 for the configurations of CSI-RSs for BM of candidate cells, similarly RAN1 can study mechanisms to efficiently configure the TRS and CSI-RS for CSI of candidate cell(s) relevant to the UE; otherwise, TRS and CSI-RS configuration given by the candidate cell may be in the directions not relevant to the UE or may be configured in all possible directions which would incur unnecessary network overhead, and also UE may need to perform measurements on a larger set of TRSs and CSI-RSs than needed. As mentioned in the section 2.1.2, similar mechanism(s) using SSB measurements for configuring the appropriate TRS can be used.

Moreover, for the CSI acquisition, the derived CSI at the UE for a candidate cell should be fed back to the candidate cell so that the candidate cell can use that information after the cell switch in case the UE is hand overed to this candidate cell. Since before the cell switch there is no direct communication link established between the UE and the candidate cell, the acquired CSI by the UE should be sent to the candidate cell via the serving cell which can further require coordination via CU in case the candidate cell is from another DU.

Proposal 25: For the early TRS and CSI acquisition, RAN1 to study mechanism(s) to configure TRSs or/and CSI-RS of a candidate cell relevant to the measuring UE. For the CSI acquisition, the derived CSI of a candidate cell is sent to the serving cell which will be forwarded to the candidate cell (via CU in case of inter-DU scenarios).

# Conclusion

In this contribution, we discuss the details of different components of Rel-18 LTM including L1 measurement and reporting configuration, beam indication of target cell, and candidate cell procedures which can be performed before cell switch. The following observations and proposal have been made:

Proposal 1: For inter-frequency measurements, additional parameters containing the frequency information of the RS such as ARFCN, frequency band indicator, and measurement gaps to enable inter-frequency measurements need to be provided along with the RS information.

Proposal 2: For Rel-18 LTM, L1-RSRP using CSI-RS for beam management can be supported L1 measurement of a candidate cell as starting point.

Observation 1: Depending on the RAN1 agreements on supporting L1-RSRP using CSI-RS for intra- or/and inter-frequency measurements, RAN4 would need to define the relevant requirements.

Observation 2: Without any additional information of the UE (e.g., location), a candidate cell’s CSI-RS configuration may not contain CSI-RSs relevant for the measuring UE.

Proposal 3: If CSI-RS measurements with a candidate cell are supported, RAN1 to study mechanism(s) for the configuration of CSI-RSs of a candidate cell relevant to the measuring UE with minimal overhead.

Proposal 4: Support of L1-SINR can be discussed later after the basic overall framework for L1 measurement configuration is in place for L1-RSRP.

Observation 3: Filtering the L1 beam measurements can decrease the number of ping-pongs substantially for LTM.

Observation 4: The cost of ping-pong is higher for inter-DU than intra-DU case in terms of signalling overhead and latency in re-transmitting the pending packets.

Proposal 5: Different L1 filtering configurations are supported for different groups of cells, where one group can contain the candidate cells belong to serving DU and another group can contain the candidate cells belong to different DU than the serving DU.

Observation 5: For L1 measurement and reporting solutions, where RSs of candidate cells are given within their respective candidate cell configurations, it would incur additional processing overhead at the UE because the UE would need read and process multiple candidate cell configurations to extract the RS configuration information before the cell switch.

Observation 6: For L3 measurement based framework, significant changes would be required to existing L1 measurements and reporting used for BM and CSI acquisition. Also, the L1 reporting overhead would be much higher in such a solution.

Observation 7: Rel-17 ICBM based framework may work for Rel-18 LTM with necessary enhancements for inter-frequency measurements; however, in such framework (and also with Option-2 mentioned above), every time when a cell change occurs, the UE may need to read and process a large set of configuration associated with the new serving cell, which may not be optimal in case of frequent cell changes.

Observation 8: For LTM, a L1 measurement and reporting configuration needs to be considered which can satisfy the LTM requirements, i.e., intra and inter-frequency measurements along with the intra-DU and inter-DU measurements from up to 8 cells, fast dynamic switching with minimal configuration change and without incurring any significant additional reporting overhead.

Proposal 6: For LTM L1 measurement configuration, a common set of RSs across neighbouring cells specific for LTM measurements is provided, where

- detail parameters for each RS in terms of its timing, frequency, and power configuration are given

- the resources from different cells may be differentiated by an identifier (LTM-Config-Index, FFS: which may also be reused later in the cell switch command to provide target cell identifier)

- this configuration may retain across LTM cell switching; however, may be updated by the CU if needed (e.g., adding/removing a cell/RS)

Proposal 7: For LTM L1 measurement configuration, along with a common set of measurement RSs, a cell specific RRC configuration is provided, which includes:

- reporting configuration providing the uplink resource configuration

- measurement gap configuration for inter-frequency measurements

- a set of activated RSs/cells (e.g., a bitmap of LTM-Config-Indices) from the common LTM-CSI-Resource-set to consider for measurements

Proposal 8: A set of activated RSs (e.g., a bitmap of LTM-Config-Indices) from the common LTM-CSI-Resource-se can dynamically be changed later (e.g., via a MAC-CE from the serving cell).

Proposal 9: As a starting point, the maximum number of reported RS in a single reporting instance is 4.

Proposal 10: Support periodic and semi-persistent CSI reporting using PUCCH for Rel-18 LTM.

Observation 9: To support L1 reporting of inter-frequency measurements, additional frequency identifier may not be needed in the report as the associated RS which needs to be measured would be configured with its dedicated identifier in the CSI resource configuration.

Proposal 11: Rel-17 ICBM and inter-cell mTRP L1 reporting format with one absolute 7-bit RSRP and remaining 4-bit differential RSRP value (relative to the absolute value) can be reused for Rel-18 LTM.

Observation 10: Enabling Rel-17 group-based beam reporting for a candidate cell in order to trigger a mTRP operation may be useful.

Proposal 12: Define event based on L3 or L1 measurements where UE determines to start performing L1 measurement on pre-configured L1 measurement RS (e.g., associated with a certain candidate cell) and reporting using the currently active reporting configuration for which the measurement RS are associated.

Observation 11: Depending on the definition of an event, the existing L1 signaling (L1 reporting container) can carry reported content if the L1 reporting format is not changed.

Observation 12: In case event based measurement results different from legacy L1 report format would need to be provided, a new L1 reporting format may be needed.

Observation 13: MAC CE based reporting will provide most flexible container option for event based reporting.

Observation 14: MAC CE based reporting can be carried out using the available UL grant or resources can be requested by currently specified means i.e. SR/CBRA.

Proposal 13: For the UE event-based reporting, support at least MAC CE as a reporting container.

Observation 15: If the event triggers UE to perform L1 measurements for active/existing L1 reporting configuration, the indication of event is not needed.

Observation 16: Dedicated L1 signalling (SR) could be used to indicate that UE intents to provide more information for a specific event.

Proposal 14: In event-based reporting, UE provides an indication to network on which reporting configurations should be activated.

Proposal 15: In event-based reporting, a dedicated L1 signal, SR, can be used to indicate that reporting event has occurred in case UL resources are not available for MAC CE.

Proposal 16: Support simultaneous configuration of any time type L1 reporting and event based reporting. It is up to NW to configure whether to use L1 reporting (e.g., periodic) or event based or both.

Proposal 17: The reporting destination of the event triggered report is the serving cell of the UE.

Observation 17: Different cells may be configured with different TCI frameworks (Rel-15 or Rel-17), e.g., the serving cell and a candidate cell may use a different TCI state framework.

Proposal 18: Beam indication for candidate cell(s) in Rel-18 LTM should be designed such that it can work for any TCI framework, including the scenarios when serving cell and target cell support the same TCI framework (Rel-15 or Rel-17) or the different TCI framework (e.g., one Rel-15 and other one Rel-17).

Proposal 19: In Rel-18 LTM, at least a TCI state or a QCL source RS associated with the target cell can be provided as a beam indication before the cell switch.

Proposal 20: UE can determine a DL RS via the provided TCI state or the QCL source RS for monitoring at least one DL channel and/or transmission of at least one UL channel of the target cell after the cell switch until it receives a new activation/beam indication from the new target cell.

Proposal 21: The UE can be configured to consider that once it has completed the RRC reconfiguration (i.e. the handover is complete), it has not been provided with an indication of a TCI state or has not received activation for any TCI state in relation to the target cell.

Observation 18: Scenarios when beam indication is provided before the cell switch command would be beneficial when the UE is in a Rel-17 ICBM scenario before cell switch, and the target cell is same as the ICBM non-serving cell. The UE can be configured to use the indicated TCI state of non-serving cell used in the ICBM operation also for the QCL assumption for the target cell after the cell switch until it receives it receives a new activation/beam indication from the target cell.

Proposal 22: Scenario 1 for timing of beam indication is also supported.

Observation 19: Scenarios when the beam indication is not provided the UE would be beneficial to enable more network flexibility. In such scenarios, the UE can be configured to trigger a RACH procedure with the target cell to obtain the initial QCL assumption.

Proposal 23: Scenario 3 for timing of beam indication is also supported. In such scenarios, the UE can be indicated whether there is a beam indication (a TCI state or a DL RS) is present or not in the cell switch command.

Proposal 24: Support early TRS tracking and CSI acquisition of candidate cell(s).

Proposal 25: For the early TRS and CSI acquisition, RAN1 to study mechanism(s) to configure TRSs or/and CSI-RS of a candidate cell relevant to the measuring UE. For the CSI acquisition, the derived CSI of a candidate cell is sent to the serving cell which will be forwarded to the candidate cell (via CU in case of inter-DU scenarios).

# References

[1] RP-222332, Revised WID on Further NR mobility enhancements, 3GPP TSG RAN Meeting #97-e, Online, September 12-16, 2022

[2] R4-2220733, Reply LS on L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility, 3GPP TSG RAN WG4 Meeting #105, November 14-18, 2022

[3] R1-2212946, FL final summary on L1 enhancements for inter-cell beam management, 3GPP TSG RAN WG1 Meeting #111, November 14-18, 2022

[4] R1-2210725, FL final summary on L1 enhancements for inter-cell beam management, 3GPP TSG RAN WG1 Meeting #110bis-e, October 10-19, 2022

[5] R2-2212988, Reply LS on L1 measurement and configurations for LTM, 3GPP TSG RAN WG2 Meeting #120, November 14-18, 2022

[6] R3-226829, Reply LS on L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility, 3GPP TSG RAN WG3 Meeting #118, November 14-18, 2022

- [7] R2-2208212, Prerequisites and benefits of Lower Layer Mobility, 3GPP TSG RAN WG2 Meeting #119-e, August 17 – 26, 2022

- [8] R1-2007151, Moderator summary for multi-beam enhancement: EVM, 3GPP TSG RAN WG1 Meeting #102-e, August 17-28, 2020