3GPP TSG RAN WG1 #112                                                                                                                           		                                                           R1- 2300432

Athens, Greece, February 27th – March 3rd,2023

Source:	vivo

Title:	Remaining issues on Rel-17 MBS UE features

Agenda Item:	8.16.1

Document for:	Discussion and Decision

- Introduction

In last meeting, Rel-17 NR UE features list has been discussed and updated [1]. In this contribution, our views on MBS remaining issues are further provided based on newly updated version [1].

- Discussion

  - 33-1-2 FDMed unicast PDSCH and group-common PDSCH for broadcast

Regrading FDMed unicast PDSCH and group-common PDSCH, the following agreements were achieved in previous meeting that the unicast PDSCH and group-common PDSCH can only be dynamically scheduled [1]. Thus, for 33-1-2 for FDM-ed unicast PDSCH and group-common PDSCH for broadcast, it should be clarified that the unicast PDSCH should be dynamically scheduled.

Agreement It is clarified that for the FDMed unicast PDSCH and group-common PDSCH capability of RRC_CONNECTED UE, the unicast PDSCH and group-common PDSCH can only be dynamically scheduled. Further down-selection between the following: - Opt 1: the unicast/multicast SPS retransmission PDSCH is included in the dynamically scheduled PDSCH for FDMed scheduling - FFS: whether new UE capability is needed - Opt 2: the unicast/multicast SPS retransmission PDSCH is NOT included in the dynamically scheduled PDSCH for lFDMed scheduling

Proposal 1: Update 33-1-2 to include the UE capability for FDMed unicast dynamically scheduled PDSCH and group-common PDSCH for broadcast.

| 33. NR_MBS | 33-1-2 | FDM-ed unicast PDSCH and group-common PDSCH for broadcast | 1. Support FDM between one unicast dynamically scheduled PDSCH and one group-common PDSCH for broadcast in RRC CONNECTED mode in a slot. | 33-1 | Yes |  |  | Per FSPC | N/A | N/A |  | NOTE: SPS PDSCH retransmission is included in dynamically scheduled PDSCH | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - 33-3-2 FDMed unicast PDSCH and group-common PDSCH for multicast

Regrading FDMed unicast PDSCH and multicast PDSCH, the following agreements were achieved in previous meeting that the unicast PDSCH and group-common PDSCH can only be dynamically scheduled [1]. One remaining issue is for SPS PDSCH retransmission, whether it is included in the dynamically scheduled PDSCH for FDMed scheduling.

Agreement It is clarified that for the FDMed unicast PDSCH and group-common PDSCH capability of RRC_CONNECTED UE, the unicast PDSCH and group-common PDSCH can only be dynamically scheduled. Further down-selection between the following: - Opt 1: the unicast/multicast SPS retransmission PDSCH is included in the dynamically scheduled PDSCH for FDMed scheduling - FFS: whether new UE capability is needed - Opt 2: the unicast/multicast SPS retransmission PDSCH is NOT included in the dynamically scheduled PDSCH for lFDMed scheduling

Regarding SPS PDSCH retransmission, it is usually deemed as dynamically scheduled PDSCH in current specification for unicast scheduling. For example, when determining number of PDSCHs supported in a slot for TDM case, or determining HARQ-ACK timing/PUCCH resource, or PDSCH aggregation factor, etc., same mechanism is used for SPS PDSCH retransmission and dynamically scheduled PDSCH. SPS PDSCH retransmission is scheduled by a certain DCI, it is natural to include it in the dynamically scheduled PDSCH for FDMed scheduling, i.e. Opt 1 should be adopted.

Proposal 2: Update 33-3-2 to include the UE capability for unicast/multicast SPS retransmission PDSCH for FDMed unicast PDSCH and group-common PDSCH for multicast.

| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and one group-common PDSCH for multicast | 1. Support FDM between one dynamically scheduled unicast PDSCH and one dynamically scheduled group-common PDSCH for multicast in RRC CONNECTED mode in a slot. | 33-2, or at least one of {33-5-1a, 33-5-1f} | Yes |  |  | Per FSPC | N/A | N/A |  | Note: this FG does not support FDMed SPS. SPS PDSCH retransmission is included in dynamically scheduled PDSCH | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - 33-4a NACK-only based HARQ-ACK feedback for multicast with ACK/NACK transforming

For NACK-only based HARQ-ACK feedback for dynamically scheduled group-common PDSCH for multicast, RRC-based enabling/disabling NACK-only based feedback is supported according to RAN1’s agreements, which should be clarified in 33-4.

Proposal 3: Update 33-4 to include the UE capability for NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling with ACK/NACK transforming.

| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling with ACK/NACK transforming | 1. Support NACK-only based HARQ-ACK feedback and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits 2. Support of shared PUCCH resource configurations with unicast 3. One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits 4. One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI | 33-2a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - 33-5-1a,33-5-1f and 33-5-1j NACK-only based HARQ-ACK feedback for SPS group-common PDSCH for multicast

For SPS group-common PDSCH for multicast activation and release, the following agreement was achieved in previous RAN1 meeting. One reason to make such agreement includes whether there can be no HARQ-ACK feedback for multicast SPS activation/deactivation. It is clear that UE should feedback HARQ-ACK information for multicast SPS activation/deactivation based on the agreement, i.e., disabled HARQ-ACK is not applicable for multicast SPS activation/deactivation, which should be included in 33-5-1a.

Agreement: @RAN1-107

For multicast SPS activation/deactivation, only ACK/NACK based feedback is supported

Proposal 4: Update 33-5-1a to include the UE capability for enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling.

| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | 1. Support of ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH and SPS release PDCCH, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 2. Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3. Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4. Support of shared SPS-PUCCH-AN-List configuration from unicast SPS | 33-5-1 | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For NACK-only based HARQ-ACK feedback for dynamically scheduled group-common PDSCH for multicast 33-4 and 33-4a are defined for NACK-only mode 1 and mode 2 respectively.  For SPS group-common PDSCH for multicast, similar UE capability should be defined to support NACK-only feedback for multiple TBs.

Proposal 5: Update 33-5-1f to include NACK-only based HARQ-ACK feedback for SPS group-common PDSCH for multicast with ACK/NACK transforming.

| 33. NR_MBS | 33-5-1f | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast with ACK/NACK transforming | 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling including: a) A single TB with NACK-only feedback transmitted in PUCCH b) One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits 2) Support of shared PUCCH resource configurations with unicast 3) One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits 4) One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI | 33-5-1a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Considering that at most 4 PUCCH resources can be configured by sps-PUCCH-AN-ListMulticast, it cannot support the case of more than 2 HARQ-ACK information bits in a slot corresponding to a specific sequence or a PUCCH transmission in FG 33-5-1j, which should be clarified.

Proposal 6: Update 33-5-1j to include NACK-only based HARQ-ACK feedback for SPS group-common PDSCH with only up to 2 TBs

| 33. NR_MBS | 33-5-1j | NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission for SPS group-common PDSCH for multicast | 1. Support NACK-only based HARQ-ACK feedback for SPS PDSCH for multicast, including: a) Up to 2 TBs with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of separate SPS-PUCCH-AN-List from unicast 3. Single TB with NACK-only feedback transmitted in PUCCH 4. Up to 2TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits | 33-5-1f | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Conclusion

In this contribution, we provide our further considerations on MBS remaining part in the following proposals and the updated FGs in tracked changes:

Proposal 1: Update 33-1-2 to include the UE capability for FDMed unicast dynamically scheduled PDSCH and group-common PDSCH for broadcast.

| 33. NR_MBS | 33-1-2 | FDM-ed unicast PDSCH and group-common PDSCH for broadcast | 1. Support FDM between one unicast dynamically scheduled PDSCH and one group-common PDSCH for broadcast in RRC CONNECTED mode in a slot. | 33-1 | Yes |  |  | Per FSPC | N/A | N/A |  | NOTE: SPS PDSCH retransmission is included in dynamically scheduled PDSCH | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: Update 33-3-2 to include the UE capability for unicast/multicast SPS retransmission PDSCH for FDMed unicast PDSCH and group-common PDSCH for multicast.

| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and one group-common PDSCH for multicast | 1. Support FDM between one dynamically scheduled unicast PDSCH and one dynamically scheduled group-common PDSCH for multicast in RRC CONNECTED mode in a slot. | 33-2, or at least one of {33-5-1a, 33-5-1f} | Yes |  |  | Per FSPC | N/A | N/A |  | Note: this FG does not support FDMed SPS. SPS PDSCH retransmission is included in dynamically scheduled PDSCH | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3: Update 33-4 to include the UE capability for NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling with ACK/NACK transforming.

| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling with ACK/NACK transforming | 1. Support NACK-only based HARQ-ACK feedback and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits 2. Support of shared PUCCH resource configurations with unicast 3. One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits 4. One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI | 33-2a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4: Update 33-5-1a to include the UE capability for enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling.

| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | 1. Support of ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH and SPS release PDCCH, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 2. Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3. Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4. Support of shared SPS-PUCCH-AN-List configuration from unicast SPS | 33-5-1 | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5: Update 33-5-1f to include NACK-only based HARQ-ACK feedback for SPS group-common PDSCH for multicast with ACK/NACK transforming.

| 33. NR_MBS | 33-5-1f | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast with ACK/NACK transforming | 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling including: a) A single TB with NACK-only feedback transmitted in PUCCH b) One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits 2) Support of shared PUCCH resource configurations with unicast 3) One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits 4) One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI | 33-5-1a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 6: Update 33-5-1j to include NACK-only based HARQ-ACK feedback for SPS group-common PDSCH with only up to 2 TBs

| 33. NR_MBS | 33-5-1j | NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission for SPS group-common PDSCH for multicast | 1. Support NACK-only based HARQ-ACK feedback for SPS PDSCH for multicast, including: a) Up to 2 TBs with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of separate SPS-PUCCH-AN-List from unicast 3. Single TB with NACK-only feedback transmitted in PUCCH 4. Up to 2TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits | 33-5-1f | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- References

- R1-2212895, Updated RAN1 UE features list for Rel-17 NR after RAN1 #111, Moderators (AT&T, NTT DOCOMO, INC.)