3GPP TSG RAN WG1 #112		R1-2300464

Athens, Greece, February 27th – March 3rd, 2023

Source:	vivo

Title:                     Discussion on further UE complexity reduction

Agenda Item:	9.6.1

Document for:	Discussion and Decision

- Introduction

In RAN#98-e meeting, the revised WID on Enhanced support of reduced capability NR devices was approved as following [1]:

Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2, CT1 and CT4 involvement Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. Check in RAN#99 regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

In addition, RAN1 #111 meeting, following agreements were made for UE complexity reduction [2]:

UE post-FFT buffer size Conclusion: For UE BB complexity reduction, for broadcast and unicast PDSCH, RAN1 does not assume that the UE post-FFT buffer size per slot is smaller than 20 MHz. Unicast PDSCH bandwidth Agreement: - For UE BB complexity reduction, a UE is able to receive a DL assignment in a DCI with a unicast PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The number of PRB scheduled in DCI is not larger than the maximum number of PRB agreed in previous agreement from 110b-e SIB1/OSI Conclusion: For UE BB complexity reduction, broadcast of separate SIB1/OSI (PDSCH) to Rel-18 RedCap UEs is not supported. Paging bandwidth Agreement: From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). RAR bandwidth Agreement: For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH Msg3 bandwidth Agreement: For UE BB complexity reduction, a UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Number of PRBs Agreement: For UE BB bandwidth reduction, for PUSCH, down-select between the following options for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), down-select between the following options for the maximum number of PRBs that the UE can process per slot: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS Same option will be selected for both PDSCH and PUSCH. UE peak data rate reduction Agreement: - The minimum DL peak rate target (for FD-FDD) is [10] Mbps based on peak data rate calculation according to 38.306. - The same value for X is used for DL and UL

In this contribution, we provide our views on UE BB bandwidth reduction, separate initial BWP, UE peak data reduction and support of additional separate early indication(s) for Rel-18 RedCap UE.

- UE BB bandwidth reduction

About UE bandwidth reduction to 5MHz, it was concluded that RAN1 does not assume that the UE post-FFT buffer size per slot is smaller than 20 MHz for both broadcast and unicast PDSCH. Based on this assumption, lots of agreements were made for unicast PDSCH and broadcast PDSCH bandwidth such as SIB1/OSI, paging and RAR. In the following, we discuss remaining issues for the resource allocation for RAR, simultaneous reception of multiple DL shared channels and separate initial DL/UL BWP for Rel-18 eRedCap UEs.

  - RAR Bandwidth

For RAR PDSCH, the agreements made in the last meeting give network scheduling full flexibility, so that the scheduled number of PRBs can be within or exceed 25 PRBs for 15KHz SCS or 11/12 PRBs for 30KHz SCS. In case the scheduling of RAR PDSCH is larger than 25 PRBs for 15KHz SCS or 11/12 PRBs for 30KHz SCS, if the time indicated by RAR UL grant between the RAR and MSG3 is not smaller than NT,1 + NT,2 + 0.5 + X ms, the eRedCap UE can transmit the MSG3. One remaining issue is to determine the value(s) of X. For Rel-18 eRedCap, given the maximum number of PRBs that the UE can process per slot is 25 PRBs for 15KHz SCS or 11/12 PRBs for 30KHz SCS, then X can be simply determined as NRB×0.18×2μ-55×2-μ ms or NRB×0.18×2μ-55 slot(s), where NRB is the number of PRBs scheduled for RAR PDSCH, μ =0 or 1 for 15KHz or 30KHz SCS respectively. Assuming the maximum scheduled bandwidth for RAR PDSCH is 20MHz, the maximum value of X can be considered as 3 slots corresponding to 3ms and 1.5ms for 15KHz and 30KHz SCS.

Proposal 1: When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the Rel-18 eRedCap UE can process per slot, the UE transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms, where X can be 1, 2 and 3.

According to TS 38.213 clause 8.3, for Rel-17 RedCap and non-RedCap UEs, the minimum processing time for decoding RAR PDSCH and preparing MSG3 transmission is NT,1 + NT,2 + 0.5 msec. While the actual MSG3 transmission timing is in slot n+k2+Δ, when a UE receives a PDSCH with a RAR message ending in slot 𝑛, where k2 and Δ are provided in default TDRA table for PUSCH and additional SCS specific slot delay value in TS 38.214 as following.

For Rel-18 eRedCap, depending on the maximum value(s) of X, the number of Row index(es) in above Table 6.1.2.1.1-2 that can meet the minimum processing timeline may be quite limited. Take X=3 slot as example, Table 1 summarizes the value(s) of k2 that can be used to meet the timeline requirement.

Table 1: K2 values and row index satisfying the minimum processing timeline requirement assuming X=3 slots

| SCS | (NT,1 + NT,2 + 0.5 + X msec) [unit: slot] | Δ | k2 |
| --- | --- | --- | --- |
| 15KHz | Ceil [(14+10+7)/14] + 3 = 6 (slots) | 2 | k2≥4, (Useable row index: 15 and 16) |
| 30KHz | Ceil [(13+12+14)/14] + 3 = 6 (slots) | 3 | k2 ≥3, (Useable row index: 11, 12, 13, 15 and 16) |

Observation 1: Assuming X=3 slots, the number of usable row index(es) in default PUSCH TDRA Table 6.1.2.1.1-2 in TS 38.214 is only two: 15 and 16 for 15KHz SCS; and five: 11, 12, 13, 15 and 16 for 30KHz SCS.

Based on above, to maintain the default PUSCH TDRA table unchanged and improve the number of row index(es) that can be used, larger Δ value can be considered for Rel-18 eRedCap in case the scheduled bandwidth for RAR PDSCH is larger than 5MHz.

Proposal 2: larger Δ value(s) can be considered for Rel-18 eRedCap in case the scheduled bandwidth for RAR PDSCH is larger than 5MHz to increase the usable number of row index(es) in the default PUSCH TDRA Table 6.1.2.1.1-2 in TS 38.214.

  - Simultaneous receptions of DL shared channels

Rel-17 RedCap and non-RedCap UEs are capable of decoding two broadcast PDSCHs in RRC_IDLE and RRC_INACTIVE modes and one unicast PDSCH and one broadcast PDSCH carrying system information for FR1 that partially or fully overlapping in time in non-overlapping PRBs as captured following in TS 38.214

[…] The UE in RRC_IDLE and RRC_INACTIVE modes shall be able to decode two PDSCHs each scheduled with SI-RNTI, P-RNTI, RA-RNTI or TC-RNTI, with the two PDSCHs partially or fully overlapping in time in non-overlapping PRBs. […] On a frequency range 1 cell, the UE shall be able to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI and, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI that partially or fully overlap in time in non-overlapping PRBs, unless the PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI requires Capability 2 processing time according to clause 5.3 in which case the UE may skip decoding of the scheduled PDSCH with C-RNTI, MCS-C-RNTI, or CS-RNTI. On a frequency range 2 cell, the UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if in the same cell, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI partially or fully overlap in time. […]

For Rel-18 eRedCap, given the maximum number of PRBs that the UE can process per slot is corresponding to 5MHz, further discussion is necessary on how to handle the two DL shared channels with the total transmission bandwidth exceeding 5MHz that partially or fully overlapping in time in non-overlapping PRBs.

For Rel-18 eRedCap UE in RRC_IDLE and RRC_INACTIVE modes, when two PDSCHs each scheduled with SI-RNTI, P-RNTI, RA-RNTI, MsgB-RNTI or TC-RNTI that partially or fully overlapping in time in non-overlapping PRBs, considering the timeline requirement between the RAR PDSCH that scheduled by PDCCH scrambled with RA-RNTI and MSG3 transmission, MSGB PDSCH or PDSCH that scheduled by PDCCH scrambled with MsgB-RNTI or with TC-RNTI and corresponding PUCCH carrying HARQ-ACK transmission, it is reasonable and beneficial to prioritize the decoding of PDSCH that scheduled with RA-RNTI,  MsgB-RNTI or TC-RNTI. As for the simultaneous reception of paging PDSCH and SI PDSCH, if it happens, it is reasonable to prioritize paging PDSCH over SI PDSCH. Therefore, following proposal is made:

Proposal 3: For Rel-18 eRedCap UE in RRC_IDLE and RRC_INACTIVE modes, when two PDSCHs each scheduled with SI-RNTI, P-RNTI, RA-RNTI, MsgB-RNTI or TC-RNTI that partially or fully overlapping in time in non-overlapping PRBs,

- If the total number of the non-overlapping PRBs for the two PDSCH is not larger than 5MHz, the UE shall be able to decode two PDSCHs.

- Otherwise, the UE shall decode the PDSCH according to the following priority order (in descending order):

  - PDSCH scheduled by RA-RNTI, MsgB-RNTI or TC-RNTI

  - PDSCH scheduled by P-RNTI

  - PDSCH scheduled by SI-RNTI

For Rel-18 eRedCap UE in RRC_CONNECTED mode on a frequency range 1 cell, in case the unicast PDSCH and the PDSCH scheduled with SI-RNTI that partially or fully overlap in time in non-overlapping PRBs and the total number of the non-overlapping PRBs for the two PDSCHs are larger than 5MHz, following two options can be considered.

- Option 1: define decoding priority, for example UE prioritizes decoding the PDSCH scheduled with SI-RNTI and the UE may skip decoding PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI.

- Option 2: similar as UE behavior on frequency range 2 cell that a Rel-18 RedCap UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if in the same cell, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI partially or fully overlap in time.

Option 1 put less scheduling restriction at the NW side compared to Option 2. The skipped unicast PDSCH reception can be recovered by retransmission. While Option 2 simplifies UE behavior.

Proposal 4: For Rel-18 eRedCap UE in RRC_CONNECTED mode, when the PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI and another PDSCH scheduled with SI-RNTI partially or fully overlap in time in non-overlapping PRBs during a process of P-RNTI triggered SI acquisition,

- If the total number of the non-overlapping PRBs for the two PDSCH is not larger than 5MHz, the UE shall be able to decode two PDSCHs.

- Otherwise, following options can be down-selected

  - Option 1: the UE shall prioritize decoding the PDSCH scheduled with SI-RNTI and the UE may skip decoding PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI.C-RNTI, MCS-C-RNTI, or CS-RNTI.

  - Option 2: error case that the UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if in the same cell, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI partially or fully overlap in time.

  - Separate initial DL/UL BWP

In RAN1#110bis-e meeting, it was agreed that the Rel-18 RedCap UEs can share the same separate initial DL/UL BWP as the Rel-17 RedCap UEs and FFS whether an additional separate initial DL/UL BWP specific to Rel-18 RedCap UE can be supported [3]. In RAN1#111 meeting, this issue was further discussed and following FL5 Proposal 2.4-3b in [4] was made with the most support. However, this proposal overlooked one important point that is “this up to one separate (RedCap-specific) initial DL/UL BWP” is from the UE perspective or from the “cell” perspective.

Proposal 2.4-3b:

- For a cell supporting Rel-17 and/or Rel-18 RedCap UEs, up to one separate (RedCap-specific) initial DL/UL BWP can be configured.

During the discussion, most companies share the views that if one separate RedCap-specific initial DL/UL BWP is configured on the cell, the separate RedCap-specific initial DL/UL BWP that applicable to RedCap UEs should be the same one. It does not mean different RedCap UEs can have different but up to one separate (RedCap-specific) initial DL/UL BWP. Therefore, the proposal 2.4-3b should be made from the “cell” perspective. However, it is still not accurate to just formulate from the cell perspective because a NR cell from gNB perspective can contain many CD-SSBs for a wide bandwidth. It cannot preclude from a cell perspective that many initial DL/UL BWP exist. To reflect the intention correctly, following modification can be considered:

Proposal 5: For a cell supporting Rel-17 and/or Rel-18 RedCap UEs, up to one separate (RedCap-specific) initial DL/UL BWP can be configured by the same SIB1 message.

For a cell supporting both Rel-17 and Rel-18 RedCap UEs, when one separate (RedCap-specific) initial DL/UL BWP, at least following cases can be supported:

- The separate (RedCap-specific) initial DL/UL BWP is configured Only for Rel-18 RedCap UEs;

- The separate (RedCap-specific) initial DL/UL BWP is configured for both Rel-17 and Rel-18 RedCap UEs.

However, depending on the market share and to provide NW sufficient configuration flexibility, the case that the separate (RedCap-specific) initial DL/UL BWP is configured Only for Rel-17 RedCap UEs can also be supported. The detailed signaling design for supporting above cases can be up to RAN2.

- UE peak data rate reduction

Based on the agreement, RAN1 supports UE peak data rate reduction at least as an add-on to UE BB bandwidth reduction, the remaining issue is to determine the relaxed constraint value. For 5MHz UE BB bandwidth, assuming the maximum number of PRBs is 25 for 15KHz SCS and 11 for 30KHz SCS with 64QAM and 1Rx/1layer, if the supported peak data rate for Rel-18 RedCap targets to 10Mbps based on [1],

- If the constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ 3.4, the DL/UL peak data rate can be achieved for 15KHz SCS is 11.4 Mbps/12.2Mbps, for 30KHz SCS is 10 Mbps/10.7Mbps.

- If the constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ 3.2, the DL/UL peak data rate can be achieved for 15KHz SCS is 10.7 Mbps/11.4Mbps, for 30KHz SCS is 9.4 Mbps/10 Mbps.

- If the constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ 3, the DL/UL peak data rate can be achieved for 15KHz SCS is 10Mbps/10.7Mbps, for 30KHz SCS is 8.8 Mbps/9.4Mbps.

Proposal 6: For UE peak data rate reduction added on to UE BB bandwidth reduction, if the target peak data rate is 10Mbps, down-select from the values of {3.4, 3.2, 3} to further relax the constraint vLayers·Qm·f ≥ 4.

If UE peak data rate reduction is supported as a standalone feature, the constraint vLayers·Qm·f ≥ 4 should be relaxed to vLayers·Qm·f ≥ 1 for 10Mbps target peak data rate.

Proposal 7: If UE peak data rate reduction is supported as a standalone feature, the constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ 1 for 10Mbps target peak data rate.

- Additional Separate early indication(s)

In RAN#98-e meeting, the support of additional separate early indication(s) was added as one objective under the UE BB bandwidth reduction [1]. The main discussion point is whether the additional separate early indication is supported by MSG3 only or the additional separate early indication is also supported by MSG1 in addition to by MSG3.

For Rel-18 RedCap UE early indication by MSG1, we think the same early indication as Rel-17 can be reused. About to additional separate early indication by MSG1 for Rel-18 eRedCap, the necessity, usage and benefits to distinguish Rel-17 RedCap UE and Rel-18 RedCap UE by MSG1 should be discussed. From our understanding, allowing such distinguish by MSG1 is mainly motivated by transmission bandwidth restriction for MSG2 and MSG3, e.g. NW can ensure that the scheduled number of PRBs for MSG2 and MSG3 for Rel-18 eRedCap UE do not exceed 5MHz.  However, based on the frequency domain resource allocation agreements for RAR, it is observed that there is no restriction put at the gNB side and gNB has the full flexibility to decide whether the number of PRBs for RAR is within or exceeds 5MHz. From UE side, proper UE behavior is defined in case the number of scheduled PRBs for RAR exceeds 5MHz. Therefore, no strong motivation is found to identify Rel-18 RedCap UEs from Rel-17 RedCap from RAR resource allocation perspective. For MSG3, the common understanding is the TBS for MSG3 is small e.g. 56 bits and the MSG3 with 56-bit TBS is also assumed in the SI evaluation for both non-RedCap and RedCap UEs. For such small TBS, resource allocation up to 5MHz is sufficient for MSG3 for all UEs in the cell. Therefore, it is also not necessary to have separate early indication by MSG1 for MSG3 resource allocation.

In summary, no strong justification is found to support the additional separate early indication for Rel-18 RedCap UEs by MSG1. On the other hand, the introduced separate early indication by MSG1 makes the gNB scheduling more complex and PRACH resources be more fragmented.

Proposal 8: Separate early indication by MSG3/MSGA PUSCH is supported for Rel-18 RedCap UEs.

Proposal 9: Separate early indication by MSG1/MSGA PRACH is not supported for Rel-18 RedCap UEs.

- Conclusion

This contribution discusses complexity reduction features including UE BB bandwidth reduction, peak data rate reduction and early identification aspect for Rel-18 enhanced RedCap devices. The observation and proposals are summarized as following:

Observation:

Observation 1: Assuming X=3 slots, the number of usable row index(es) in default PUSCH TDRA Table 6.1.2.1.1-2 in TS 38.214 is only two: 15 and 16 for 15KHz SCS; and five: 11, 12, 13, 15 and 16 for 30KHz SCS.

Proposals:

Proposal 1: When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the Rel-18 eRedCap UE can process per slot, the UE transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms, where X can be 1, 2 and 3.

Proposal 2: larger Δ value(s) can be considered for Rel-18 eRedCap in case the scheduled bandwidth for RAR PDSCH is larger than 5MHz to increase the usable number of row index(es) in the default PUSCH TDRA Table 6.1.2.1.1-2 in TS 38.214.

Proposal 3: For Rel-18 eRedCap UE in RRC_IDLE and RRC_INACTIVE modes, when two PDSCHs each scheduled with SI-RNTI, P-RNTI, RA-RNTI, MsgB-RNTI or TC-RNTI that partially or fully overlapping in time in non-overlapping PRBs,

- If the total number of the non-overlapping PRBs for the two PDSCH is not larger than 5MHz, the UE shall be able to decode two PDSCHs.

- Otherwise, the UE shall decode the PDSCH according to the following priority order (in descending order):

  - PDSCH scheduled by RA-RNTI, MsgB-RNTI or TC-RNTI

  - PDSCH scheduled by P-RNTI

  - PDSCH scheduled by SI-RNTI

Proposal 4: For Rel-18 eRedCap UE in RRC_CONNECTED mode, when the PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI and another PDSCH scheduled with SI-RNTI partially or fully overlap in time in non-overlapping PRBs during a process of P-RNTI triggered SI acquisition,

- If the total number of the non-overlapping PRBs for the two PDSCH is not larger than 5MHz, the UE shall be able to decode two PDSCHs.

- Otherwise, following options can be down-selected

  - Option 1: the UE shall prioritize decoding the PDSCH scheduled with SI-RNTI and the UE may skip decoding PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI.C-RNTI, MCS-C-RNTI, or CS-RNTI.

  - Option 2: error case that the UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if in the same cell, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI partially or fully overlap in time.

Proposal 5: For a cell supporting Rel-17 and/or Rel-18 RedCap UEs, up to one separate (RedCap-specific) initial DL/UL BWP can be configured by the same SIB1 message.

Proposal 6: For UE peak data rate reduction added on to UE BB bandwidth reduction, if the target peak data rate is 10Mbps, down-select from the values of {3.4, 3.2, 3} to further relax the constraint vLayers·Qm·f ≥ 4.

Proposal 7: If UE peak data rate reduction is supported as a standalone feature, the constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ 1 for 10Mbps target peak data rate.

Proposal 8: Separate early indication by MSG3/MSGA PUSCH is supported for Rel-18 RedCap UEs.

Proposal 9: Separate early indication by MSG1/MSGA PRACH is not supported for Rel-18 RedCap UEs.

6 	References

- RP-223544, Revised WID on Enhanced support of reduced capability NR devices, Ericsson

- RAN1#111 meeting, chairman’s notes

- RAN1#110bis-e meeting, chairman’s notes

- R1-2212536, FL summary #4 on Rel-18 RedCap UE complexity reduction, Moderator (Ericsson)