3GPP TSG RAN WG1 #112	                                              R1-2300465

Athens, Greece, February 27th – March 3rd, 2023

Source:	vivo

Title:	Discussions on techniques in spatial and power domains

Agenda Item:	9.7.1

Document for:	Discussion and Decision

- Introduction

In RAN#98-e meeting, a new WI on Network energy savings for NR was approved. The scope of WI in spatial and power domains are as follows. In this contribution, we will discuss the techniques for CSI measurement/report enhancements in spatial and power domains, including design on adaptation of spatial element and power offset, signaling design, and CSI processing issues [1].

- Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

- Discussion on NW ES for spatial and power domains

  - Design principle

During the SI phase of R18 NW ES study, it is identified that dynamical adaptation of the number of spatial elements in spatial domain can achieve NW energy saving. In terms of spatial element adaptation at gNB, how the antenna elements or TxRUs are adapted is transparent to the UE. However, the change of antenna ports would result in different behaviours at UE. Hence, the adaptation of spatial element will impact CSI measurement and/or report. Besides, since currently the CSI measurement resources are configured semi-statically, potential enhancement to CSI measurement procedures may be introduced to support the dynamic spatial element adaptation.

On the other hand, adaptation of power offset values between PDSCH and CSI-RS is intended to enable gNB to efficiently adjust the transmission power of PDSCH transmission. Therefore, it is important for gNB to obtain the CSI with different power offset values between PDSCH and CSI-RS. In this regard, adaptation of power offset values between PDSCH and CSI-RS will also have impact on CSI measurement and/or report.

Based on the above analysis, we think a unified framework for adaptation of spatial element and adaptation of power offsets between PDSCH and CSI-RS can be used.

Proposal 1: Design a unified framework for spatial elements adaptation and power offset adaptation.

In addition, the number of CSI-reportConfig is up to UE capability and is limited to up to 4 P/SP/A-CSI reportconfig per BWP and 8 simultaneous reports per CC so far. To enable efficient spatial element adaptation, multiple CSI reports configs can be triggered to report CSIs for different port muting pattern. However, introducing multiple CSI configs means that a portion of the amount of supported CSI report configs reported by the UE is taken up. And Simultaneous activation of multiple CSIs also poses a great challenge to the UE's CSI processing units. Finally, and to cater to the note in WID: ‘Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements’, we prefer to make further enhancements in the framework of one CSI report.

Proposal 2: Consider enhancement for one CSI report configuration to support adaptation of spatial element and/or power offsets between PDSCH and CSI-RS.

Before designing enhancements for spatial domain and power domain, we need to understand the relationship between TxRU, antenna elements and ports. As shown in the Fig. 1, the signal processing of MIMO is composed of the analog domain (called TxRU virtualization) and the digital domain (called port virtualization). Fig. 2(a) and Fig. 2 (b) give two possible structures for port virtualization. In Fig. 2(a), each port is mapped with a subset of TxRUs and therefore some ports can be unaffected during spatial element adaptation. While in Fig. 2 (b), each port is mapped with all of TxRUs and the port virtualization will definitely be affected by TxRU on/off. Both of Fig. 2(a) and Fig. 2 (b) are possible in practice, and therefore the potential enhancements designed to address the adverse effects of spatial element adaptation and power offset adaptation need to be applicable to both of these structures.

| Fig. 1: Hybrid beamforming | Fig. 2(a): Type-1 virtualization (a subset of TxRUs is mapped to a logical antenna port) | Fig.2(b): Type-2 virtualization (all TxRUs is mapped to a logical antenna port) |
| --- | --- | --- |

Proposal 3: Both type-1 and type-2 port virtualization need to be considered when designing the framework for adaptation of spatial elements.

  - CSI measurement enhancement

It is necessary to ensure the accuracy of CSI measurement in order for gNB to perform efficient scheduling when dynamic adaptation of spatial element or power offsets is enabled at gNB. Dynamic adaptation of the spatial elements and power offset may result that the already configured CSI-RS resource(set) is not the resource(set) that the BS wants the UE to measure. In order for the BS to get more accurate information, the UE needs to be notified of the updated resources to do the measurement, so the UE needs to be notified of the resource change. Therefore, the CSI measurement enhancement may focus on the signaling to update the active CSI-RS. The key point is how to configure CSI measurement resources for the adaptation of spatial elements or power offset and how to indicate the adaptation. We list several options below for comparison:

- Option 1: ONE resource can be configured with multiple antenna port configurations/ power offsets:

  - L1\L2 signaling indicates the activated antenna port configurations/ power offsets, and whether multiple antenna port configurations/ power offsets can be activated at the same time needed to be further discussed.

  - Spec impacts: one CSI-RS resource can only be configured by RRC with one kind of number of ports and one power offset.

  - For example, gNB transmits one CSI-RS resource with 32ports, and UE can report multiple CSIs (of 8ports/ 16ports) based on one resource.

    - There are some limitations with this case. If UE needs to measure both 32ports CSI and 16ports CSI from one CSI-RS resource with 32ports, at least two conditions need to be satisfied: Firstly, the REs corresponding to the target 16 ports in the CSI-RS resource are orthogonal with that corresponding to the other 16 ports, otherwise there will be an impact on the CSI measurement of the target 16 ports. Secondly, the port mapping of the target 16 ports shouldn’t be affected by the TxRUs that are to be shut down, so this option is only applicable to  Fig. 2(a).

- Option 2: multiple CSI-RS resource groups are associated with different antenna port configurations/ power offsets, and these resource groups belong to the same CSI-RS Resource set:

  - L1\L2 signaling indicates the activated group of resources, whether multiple groups of resources can be activated at the same time needed to be further discussed.

  - Spec impacts: In current spec, all CSI-RS resources in a CSI-RS resource set must be configured with the same density and number of ports, except for the NZP-CSI-RS used for interference measurements.

  - Fig. 3 shows an example for Option 2, where multiple resources in the same resource set can be grouped into several resource groups and different groups have different port configurations. L1 and L2 signaling can activate one or more resource groups at a time for CSI measurements.

Fig. 3: Example for Option 2                                         Fig. 4: Two implementations of option3

- Option 3: multiple CSI-RS resource sets belonging to one CSI reports are associated to different antenna port configurations/ power offsets as shown in Fig.4:

  - L1\L2 signaling indicates the activated resource set and whether multiple resource sets can be activated at the same time needed to be further discussed.

  - Spec impacts: In current spec, for periodic and semi-persistent CSI Resource Settings, when the UE is configured with groupBasedBeamReporting-r17, the number of CSI Resource Sets configured is S=2, otherwise the number of CSI-RS Resource Sets configured is limited to S=1

Fig. 5: one example for Option 3

  - Fig. 5 illustrates an example for Option 3, resource set1 and set2 and set3 all belong to the same CSI report and different resource sets have different measurement assumptions. L1/L2 signaling can activate one or more resource sets at one time for CSI measurement.

Regarding spec impacts, Opt1 and Opt2 may have much more spec impacts than Opt3 since the limitations listed above. In addition, Opt 3 is more flexible compared with Opt1 and Opt2. In Opt3, different CSI-RS resource sets (which have different port configurations or different power offset) in one report config can be activated/deactivated by L1/L2 signaling. And that means the UE can report the corresponding CSI with the spatial element adaptation and power offset adaptation based on the activated CSI-RS resource set. Opt1 is actually a framework for limited parameter adaptation and this framework has a backward compatibility problem. For example, new parameters will also need to be added if there are new NES technologies that require dynamic adaptation of certain parameters. Opt3 may be the best choice to assist spatial element adaptation and power offset adaptation, since Opt3 has minor changes to the existing CSI framework and easier to extend in the future.

Proposal 4: Support dynamic adaptation of CSI resource setting for adaptation of spatial element or power offset between PDSCH and CSI-RS (option 3).

  - Signaling design

Indication for potential enhancements related to spatial element adaptation may help the UEs to adapt the already configured CSI-RS configuration such as dynamic/semi-persistent ON-OFF of CSI-RS or to reconfigure the CSI-RS configuration, with respect to adapted number of spatial elements/ports [1]. Several signaling format can be considered for the indication:

- dedicated PDCCH

- Group common PDCCH

- MAC CE

The adjustment of number of ports and power offsets is cell-specific behaviour, which is common to all the UEs within the cell. If the notification of the adjustment is transmitted to each UE, signalling overhead would significantly increase. To reduce the signalling overhead, group-common L1 signalling can be considered for the spatial element adaptation. Similarly, the power offset can also be adapted in the same way by group common DCI. If the DCI is adopted to indicate the spatial element adaptation and power offset adaptation, the final application time needed to be further discussed.

Regarding what configurable fields the indication message may have, we list the following alternatives:

- Alt1: Directly indicate the CSI-RS Resource/ Resource Set/CSI report to be updated soon

- Alt2: RRC configure the antenna port configuration/ power offset combination list and DCI indicate the activated entry

Obviously, Alt1 is a more direct way, in which the UE does not need to know the change of the spatial element on the network side, it only needs to know which configuration to measure and when to report the CSI. In Alt2, RRC configure the CSI-RS Resource set/ antenna port configuration/ power offsets combination list in advance, and DCI can dynamically indicate the activated entry to facilitate the spatial element adaptation and power offset adaptation. It is important to note that the existing resource set granularity allows more flexibility in indicating the corresponding changes in spatial domain and power domain.

Proposal 5 : Support group common signaling to indicate dynamic adaptation of CSI resource setting.

  - CSI report enhancement

Before the start of CSI report enhancement, we have to first identify the hardware requirements for dynamic switching. One thing that must be made clear is how fast dynamic adaptation can be achieved. In Fig. 6(a), TxRUs are adapted per slot and is it possible to dynamically switch TxRU/ power offset per slot? In this case, the dynamic adaptation of spatial elements and power offsets makes a relatively fluctuation of CSI, if the network can know more CSI information, it may be beneficial to the BS's scheduling. But the challenges brought by multi-CSI processing for the UE also needs to be considered. In contrast, in Fig. 6(b), the spatial element and power offset adapt per several slots, for example, 10ms. And in such a case, the benefit of multi-CSI may not be very obvious.

Fig. 6: Illustration for dynamic adaptation

In contrast to such a dynamic adaptation in Fig. 6(a), we believe that the designed framework should be able to support at least Fig. 6(b). Activating the UE to report the corresponding CSI by indicating the changes in resource set for measurement is a feasible approach.

Proposal 6: The designed framework should at least support dynamic adaptation per several slots in Fig. 6(b).

Table.1 presents the system level simulation results of dynamic port adaptation (i.e., dynamic switching between 32ports and 8ports) for both multi-CSI and single-CSI. The criteria for the adjustment of the number of ports between 32 ports and 8 ports is the traffic load and the RSRP of UEs. And the UPT loss and energy saving gain are derived by comparing the results to the baseline where 32 ports are always on.

It is obviously shown from Table.1 that although the UPT loss of multi-CSI is slightly smaller than that of single-CSI, the energy saving gain of both is actually similar. Therefore, the motivation for reporting multiple CSI still needs further clarification.

|  | Dynamic adaptation between 32ports and 8ports（per slot） |  |
| --- | --- | --- |
|  | UPT Loss | Energy saving gain |
| Multi-CSI | 0.02% | 12.8% |
| Single-CSI | 0.36% | 9.4% |

Table.1 System level simulation results for multi-CSI and single-CSI

Observation 1: Compared to single-CSI, multi-CSI has marginal energy saving gain.

The current spec only allows one CSI in one CSI report with one measurement hypothesis and UE can report multiple CSI reports for different measurement hypothesises. In R18  NES SI, some companys propose CSI report enhancements in which multiple CSIs can be included in one CSI report. Whether these CSIs belong to the same report or to multiple reports, they both pose a challenge to the UE’s CSI processing units. So further clarification is still needed on the motivation for reporting multiple CSIs.

Proposal 7: Regarding the CSI report enhancement of multi-CSI, whether multiple CSIs belong to one report or not, the research motivation of multi-CSI has to be further clarified. And the measurement and computational complexity that multi-CSI brings to the UE also needs to be further clarified.

  - Beam management

The Dynamic adaptation of spatial element will undoubtedly have an impact on the CSI-RS transmission. Regardless of whether the ultimate effect is the reduction of the number of beams or the decrease of the RS power, it all will result in the redefinition of UE behaviours.

TxRU ON/OFF means that certain RSs are deactivated and the calculation of the UL power control is related to the measurements of the DL RSs, it is inaccurate to estimate the path loss based on these deactivated signals. In addition, there is a common problem that the decrease of the measured RSRP due to spatial element adaptation may lead the UE to mistakenly believe that a beam failure or BFR failure has occurred.

In conclusion, the above potential effects on UE can be listed as follows:

- L1-RSRP/L3-RSRP measurement

- UL power control

- Beam management, beam failure recovery, radio link monitoring

If the antenna ports used for RSRP measurement are mapped with all of TxRUs, TxRU adaptation definitely impact the RSRP computation. If the antenna ports used for RSRP measurement are mapped with a subset of TxRUs, RSRP computation can be independent of TxRU adaptation if associated TxRUs remain unchanged.

In view of the above-mentioned effects, a convenient way is that the adaptation of spatial element and power offset won’t impact the antenna ports which are used for L1-RSRP/L3-RSRP measurement, and which means the corresponding CSI-RS Resources may not be affected.

Proposal 8: Spatial element adaptation and power offset adaptation are not applicable to the CSI-RS Resources for L1-RSRP/L3-RSRP measurement/ beam management

  - CSI processing criteria

In current spec, the number of occupied CPUs is related to the number of CSI-RS resources in the CSI-RS resource set for channel measurement and CSI-RS resources are configured by RRC. If multiple CSI-RS Resource sets are configured by RRC or the CSI-RS Resources(sets) are changed dynamically, the rule of CPU occupation number needs to be modified accordingly. For example, the CPU occupation number is related to the number of the activated CSI-RS resources in the activated CSI-RS resource set for channel measurement.

In addition, the timeline for CPU calculation is also needed to be clarified. Take the Opt3 in section 2.2 as an example, the occupied CPU number varies with the number of CSI-RS resources in the activated CSI-RS Resource set for channel measurement and when this change in CPU number becomes effective still needs further discussion.

Proposal 9: For CSI measurement/report in case of enabling dynamic adaptation of spatial elements or power offsets, CSI processing criteria needs further study.

- Conclusion

In this contribution, network energy saving techniques in spatial and power domain are discussed and the following observations and proposals are summarized as follows:

Observation 1: Compared to single-CSI, multi-CSI has marginal energy saving gain.

Proposal 1: Design a unified framework for spatial elements adaptation and power offset adaptation.

Proposal 2: Consider enhancement for one CSI report configuration to support adaptation of spatial element and/or power offsets between PDSCH and CSI-RS.

Proposal 3: Both type-1 and type-2 port virtualization need to be considered when designing the framework for adaptation of spatial elements.

Proposal 4: Support dynamic adaptation of CSI resource setting for adaptation of spatial element or power offset between PDSCH and CSI-RS (option 3).

Proposal 5: Support group common signaling to indicate dynamic adaptation of CSI resource setting.

Proposal 6: The designed framework should at least support dynamic adaptation per several slots in Fig. 6(b).

Proposal 7: Regarding the CSI report enhancement of multi-CSI, whether multiple CSIs belong to one report or not, the research motivation of multi-CSI has to be further clarified. And the measurement and computational complexity that multi-CSI brings to the UE also needs to be further clarified.

Proposal 8: Spatial element adaptation and power offset adaptation are not applicable to the CSI-RS Resources for L1-RSRP/L3-RSRP measurement/ beam management

Proposal 9: For CSI measurement/report in case of enabling dynamic adaptation of spatial elements or power offsets, CSI processing criteria needs further study.

- Appendix

Table I. Simulation assumption for FR1 Urban Macro scenario

| Parameter | value |
| --- | --- |
| Scenarios | Urban Macro hexagonal layout with 7, 3 Sectors |
| Channel model | Uma |
| Inter-BS distance | 500m |
| Carrier frequency | 4GHz |
| Bandwidth | 100 MHz, 2.08% Guard Band |
| Subcarrier spacing | 30 kHz |
| Frame structure | DDDSU (S: 10D:2G:2U) |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 64T: (8,8,2,1,1;4,8) (dH, dV) = (0.5, 0.8) λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A) λ |
| BS antenna pattern | 3-TRxP pattern, 8 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| BS Power | 55dBm, EIRP should not exceed 73 dBm |
| UE max Power | 23 dBm, EIRP should not exceed 43 dBm |
| BS height | 25m |
| UE height | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl~uniform (1, Nfl) where Nfl~uniform (4,8) |
| Noise Figure | BS:5 dB, UE:9 dB |
| Max MCS | 256QAM |
| Down-tilt | 6 degrees |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| UE distribution | For evaluation of enhanced BS with TxRU muting: 20% outdoor (30km/h), 80% indoor (3km/h) For evaluation of enhanced BS with UE WUS: 100% outdoor (60km/h) |
| Traffic model | FTP3 (0.5MB as packet size, 200ms as mean inter-arrival time) |

- Reference

- RP-223540, “New WID: Network energy savings for NR”, RAN#98e, Huawei.

- TR38.864, “Study on network energy savings for NR”, (Release 18).