3GPP TSG RAN WG1 #112                                                                                            R1-2300472

Athens, Greece, February 27th – March 3rd, 2023

Source:	vivo

Title:	Discussions on coverage enhancements for NR NTN

Agenda Item:	9.11.1

Document for:	Discussion and Decision

- Introduction

In RAN#98-e, since no RAN2 related items are identified to be specified in the study, the objectives for coverage enhancement topic have been updated as following meaning that only RAN1/4 related items will be specified in this topic in Rel-18.

The detailed objectives are for NTN: - To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4] - To specify if necessary, enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) [RAN1]

The following conclusions and working assumption are reached in RAN1#111 as captured in [1] which mainly focuses on enhancements of PUCCH repetition for msg4 HARQ-ACK and DMRS bundling for PUSCH.

Conclusion For the study of NTN-specific PUSCH DMRS bundling, RAN1’s understanding is that Phase variation due to constant frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1 does not have impact on the phase continuity requirement for two adjacent slots specified as Table 6.4.2.5-1 in 38.101-1, according to annex F.9 and F.4 of 38.101-1. Conclusion RAN1 concluded that PUSCH DMRS bundling with sufficient TDW size should be applicable in NTN to meet the performance requirement for VoIP - FFS: How to determine TDW size, including UE capability. - Note: The above does not mean the performance requirements will be satisfied with DMRS bundling Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - One or more repetition factors may be configured via SIB - If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor - FFS: whether UE requests repetition or indicates repetition capability - If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB - FFS: whether UE requests repetition or indicates repetition capability - FFS: whether repetition factor is indicated by UE - FFS: UE behavior when repetition factor is not configured via SIB - FFS: whether one or more UE capabilities are needed for the above is for further discussion

In this contribution, according to above, we discuss the following aspects for coverage enhancement in NR NTN:

- PUCCH repetition for Msg4 HARQ-ACK,

- DMRS bundling of PUSCH,

- Other coverage enhancement techniques.

- Discussions

  - PUCCH repetition for Msg4 HARQ-ACK

Procedures and signaling

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - One or more repetition factors may be configured via SIB - If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor - FFS: whether UE requests repetition or indicates repetition capability - If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB - FFS: whether UE requests repetition or indicates repetition capability - FFS: whether repetition factor is indicated by UE - FFS: UE behavior when repetition factor is not configured via SIB - FFS: whether one or more UE capabilities are needed for the above is for further discussion

Based on the working assumption made in RAN1#111 meeting, it was agreed that repetition factors could be configured via SIB if necessary. In general, we can confirm the working assumption.

As for procedures and signaling, when repetition factor is not configured via SIB, legacy behaviour for Msg4 HARQ-ACK feedback on PUCCH can be reused and no enhancement is necessary.

Proposal 1: Legacy behaviour for Msg4 HARQ-ACK feedback on PUCCH applies when repetition factor is not configured via SIB.

If only one repetition factor is configured via SIB, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the configured repetition factor. Even though the single repetition factor equal to 1 means no repetition, UE should perform PUCCH repetition with factor 1 based on the PUCCH repetition procedures and signaling introduced in this WI. While for UE incapable of PUCCH repetition for Msg4 HARQ-ACK, PUCCH repetition cannot be performed and the legacy behaviour for Msg4 HARQ-ACK feedback on PUCCH can reused, which is different from PUCCH repetition with factor equal to 1.

Proposal 2: If only one repetition factor is configured via SIB, support 1 PUCCH repetition for Msg4 HARQ-ACK.

If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB. However, some UEs may not have the capability of performing PUCCH repetition for Msg4 HARQ-ACK, which means gNB would not know whether a dynamic repetition factor should be signaled to the UE if the UE repetition capability is not indicated or reported to gNB. Therefore, it is necessary to indicate or report UE repetition capability before UE is indicated to perform PUCCH repetition with the dynamically signaled factor. In this case, gNB will only indicate PUCCH repetition to UEs capable of PUCCH repetition for Msg4 HARQ-ACK.

Proposal 3: If multiple repetition factors are configured via SIB, UE repetition capability should be reported or indicated before one of the PUCCH repetition factors for Msg4 HARQ-ACK is dynamically indicated.

Regarding the indication or report of repetition capability, the mechanism of msg3 repetition capability reporting could be reused and specified if some NTN specific enhancements are necessary. For example, the indication or report of repetition capability on PUCCH could be bundled with the report of msg3 repetition capability using the same RSRP threshold and PRACH resource. Or, it could also be considered that UE being capable of msg3 repetition is mandatory to support repetition capability on PUCCH repetition for Msg4 HARQ-ACK, which requires minimum spec. impact. With the knowledge of UE repetition capability, gNB could dynamically indicate UEs to perform PUCCH repetition for Msg4 HARQ-ACK, e.g. via scheduling DCI corresponding to the msg4 PDSCH.

Proposal 4: For the indication or report of PUCCH repetition capability, support to reuse mechanism of msg3 repetition capability reporting.

From above, when multiple factors are configured via SIB, UE repetition capability is necessary to report or indicate, in order that only UEs capable of PUCCH repetition for Msg4 HARQ-ACK can be scheduled to perform repetition via dynamical indication. However, when only one repetition factor is configured, it’s not necessary for UE to request repetition or indicate another repetition capability which may require additional PRACH partitioning. In this case, it can be up to UE to determine whether repetition is actually performed, and gNB could assume PUCCH is repeated as long the single repetition factor is configured in SIB.

Proposal 5: If single repetition factor is configured via SIB, it’s up to UE to determine whether to perform PUCCH repetition for Msg4 HARQ-ACK.

Repetition factor

According to the coverage evaluation provided in our contribution [2], we first looked at the link budget results for Msg4 PUCCH without repetition which is shown in Table 1. It can be observed that 4.39 dB performance enhancements are required in LOS condition for parameter set-1 for LEO-1200 satellite which is the target scenario agreed in RAN1 to investigate the coverage gap for each channel.

Table 1. Link budget analysis for PUCCH for Msg4 HARQ-ACK

| Results | Satellite set/Channel/Comparison |  | LEO-600 | LEO-1200 |
| --- | --- | --- | --- | --- |
| Link budget results | Set-1 |  | -2.22 | -7.61 |
|  | Set-2 |  | -8.22 | -13.61 |
| LLS results | NTN-TDL-C | HARQ-ACK for Msg4 | -3.39 | -3.22 |
| Performance gap | NTN-TDL-C vs Set-1 | HARQ-ACK for Msg4 | -1.17 | 4.39 |
|  | NTN-TDL-C vs Set-2 | HARQ-ACK for Msg4 | 4.83 | 10.39 |

In order to compensate the performance gap observed, the support of PUCCH repetition for msg4 HARQ-ACK is considered. The performance results with different repetition numbers are illustrated in Figure 1, and the evaluation assumptions are provided in table A1 in Appendix A.

Figure 1. Performance of PUCCH repetition for msg4 HARQ-ACK

It can be observed that the performance of PUCCH for Msg4 HARQ-ACK with 4 or 8 repetitions could compensate the gap and meet the requirement of the CNR calculated in link budget calculation for LEO-1200 set-1 satellite. Furthermore, the performance of PUCCH for Msg4 HARQ-ACK with 8 repetitions can match with the performance of PUCCH repetition for HARQ ACK feedback on dedicated PUCCH resource.

Observation 1:

- The performance of PUCCH for Msg4 HARQ-ACK with 4 repetitions can meet the requirement of the CNR calculated in link budget calculation for LEO-1200 set-1 satellite.

- The performance of PUCCH for Msg4 HARQ-ACK with 8 repetitions can match with the performance of PUCCH repetition for HARQ ACK feedback on dedicated PUCCH resource.

With the observation, supporting a number of PUCCH repetition larger than 8 for msg4 HARQ-ACK is unnecessary. And we have following proposal.

Proposal 6: Do not support number of PUCCH repetitions larger than 8 for msg4 HARQ-ACK feedback.

Frequency hopping

In order to verify the performance of inter-slot frequency hopping, link level simulations are performed. The simulation assumptions are provided in table A1 in Appendix A, and the simulation results are depicted in Figure 2, considering a channel model of NTN-TDL-C with 30ns delay spread.

Figure 2. Performance of PUCCH repetition for msg4 HARQ-ACK with inter-slot/intra-slot frequency hopping

It can be observed that the performance gain with inter-slot FH and intra-slot FH decreases with the number of repetitions increased, and around 0.2dB and 0.4dB additional gain can be obtained under repetitions with inter-slot FH and intra-slot FH, respectively, compared to repetitions without frequency hopping.

Observation 2:

- The performance gain with inter-slot FH and intra-slot FH decreases with the larger repetition number.

- Compared to PUCCH repetitions for msg4 HARQ-ACK without FH, around 0.2dB and 0.4dB additional performance gain from PUCCH repetitions with inter-slot FH and intra-slot FH can be obtained, respectively.

According the performance gain observed and the flexibility of either configure intra-slot or inter-slot frequency hopping, inter-slot frequency hopping for PUCCH repetition for msg4 HARQ-ACK can also be considered, as intra-slot frequency hopping is already supported. In some scenarios with larger delay spread, the performance gain would be more significant. In msg3 repetition mechanism, inter-slot frequency hopping is supported and the frequency hopping flag information field in UL RAR grant or DCI format 0_0 with CRC scrambled by TC-RNTI is reused. Similarly, the frequency hopping flag information field in UL RAR grant could also be reused to enable/disable inter-slot frequency hopping for PUCCH repetition for msg4 HARQ-ACK when repetition number is configured to be more than 1. On the other hand, repurpose some fields in scheduling DCI corresponding to msg4 PDSCH could also be considered to enable/disable inter-slot frequency hopping for PUCCH repetition for msg4 HARQ-ACK.

Proposal 7: Support to enable/disable inter-slot frequency hopping for PUCCH repetition for msg4 HARQ-ACK considering the following alternatives

- Alt 1: reusing the frequency hopping flag information field in UL RAR grant when the repetition number is configured to be more than 1

- Alt 2: repurpose some fields in scheduling DCI corresponding to msg4 PDSCH

DMRS bundling for PUCCH repetition for msg4 HARQ-ACK

According to the simulation results of PUCCH repetition for msg4 HARQ-ACK with different repetition factors above, the performance of PUCCH for Msg4 HARQ-ACK with 4 or 8 repetitions can already compensate the gap and meet the requirement of the CNR calculated in link budget calculation for the target LEO-1200 set-1 satellite. Over-optimizing the performance of PUCCH repetition for msg4 HARQ-ACK is unnecessary. Furthermore, the discussion on DMRS bundling for PUCCH repetition for msg4 HARQ-ACK would be complicated and cumbersome, which would increase the work load not allowed by the limited TU allocated for this topic. The aspects that should be at least discussed and determined are e.g. the disabling/enabling of DMRS bundling, the configuration of time domain window (TDW) size, the events which violates phase continuity and power consistency, the maximum duration, and the UE capability on restarting DMRS bundling.

According to above, the support of DMRS bundling for PUCCH repetition for msg4 HARQ-ACK should be avoided in this release.

Proposal 8: Do not support the DMRS bundling for PUCCH repetition for msg4 HARQ-ACK.

  - DMRS bundling of PUSCH

Phase difference due to timing drift and/or doppler shift

In NTN, due to the movement of satellite and UE location, UE may need to pre-compensate the timing and frequency offset, via UE implementation. However, in order to support DMRS bundling, UE is required to maintain phase continuity and power consistency within an actual TDW, which implies that UE is not expected to perform pre-compensation within an actual TDW when DMRS bundling is enabled.

Proposal 9: RAN1 to conclude UE is not expected to perform pre-compensation on timing and frequency offset within an actual TDW when DMRS bundling is enabled.

Furthermore, when DMRS bundling is enabled, delay drift/doppler shift/timing drift after an actual TDW determined by events or a configured TDW defined for terrestrial network should also be considered, although perfect pre-compensation on timing and frequency offset is assumed to be performed before the first symbol of an actual TDW or a configured TDW.

For LEO, the distance between UE and satellite changes with fast movement of satellites, and the propagation time or round trip time varies as a result, which can be regarded as delay drift. Actually, the distance variation also depends on the satellite orbits and the elevation angle variation. For simplicity, the distance variation can be directly derived from the velocity and duration. Then, the delay drift with different durations are shown in table 2 under LEO-600 scenarios and LEO-1200 scenarios with the linear velocity of 7.5622km/s and 7.1172km/s, respectively.

Table 2. Delay drift with different durations

| Duration | 1ms | 2ms | 4ms | 8ms | 16ms | 32ms |
| --- | --- | --- | --- | --- | --- | --- |
| Delay drift for LEO-600 | 0.0504us | 0.101us | 0.202us | 0.403us | 0.807us | 1.613us |
| Delay drift for LEO-1200 | 0.0474us | 0.0949us | 0.19us | 0.38us | 0.759us | 1.518us |

According the timing error requirement specified in Table 7.1C.2-1 of 38.133, the delay drift is upper bounded by 29*64*Tc ≈0.928us with the SCS of 15kHz. It can be observed that the maximum duration of DMRS bundling due to delay drift or timing error is limited by 16ms.

Observation 3:

- For LEO-600 and LEO-1200, the maximum duration of DMRS bundling due to delay drift or timing error requirement is limited by 16ms for SCS of 15kHz.

Similarly, for LEO, the frequency error due to doppler shift varies with the satellite movement. In addition, doppler shift is varying with the changing angle between the velocity vector and the direction of propagation of the signal between UE and satellite. According to Table 4.2-2 in TR38.821, the maximum doppler shift variation are 0.13 ppm/s for LEO-1200 scenarios and 0.27 ppm/s for LEO-600 scenarios. Then, the doppler shift with different durations are shown in table 3.

Table 3. Doppler shift with different durations

| Duration | 1ms | 2ms | 4ms | 8ms | 16ms | 32ms |
| --- | --- | --- | --- | --- | --- | --- |
| Doppler shift for LEO-600 | 0.00027ppm @2GHz: 0.54Hz | 0.00054ppm @2GHz:1.08Hz | 0.00108ppm @2GHz:2.16Hz | 0.00216ppm @2GHz:4.32Hz | 0.00432ppm @2GHz:8.64Hz | 0.00864ppm @2GHz:17.28Hz |
| Doppler shift for LEO-1200 | 0.00013ppm @2GHz: 0.26Hz | 0.00026ppm @2GHz:0.52Hz | 0.00052ppm @2GHz:1.04Hz | 0.00104ppm @2GHz:2.08Hz | 0.00208ppm @2GHz:4.16Hz | 0.00416ppm @2GHz:8.32Hz |

It can be observed that the maximum doppler shift variation for LEO-600 scenarios within 32ms duration of DMRS bundling is 0.00864ppm, which is much smaller than ±0.1ppm the frequency error requirement specified in section 6.4.1 of 38.101-5.

Observation 4:

- For DMRS bundling with 32ms duration, the maximum doppler shift variation is much smaller than ±0.1ppm the frequency error requirement.

Furthermore, phase difference with different durations can be calculated based on such frequency error in table 3, according to ΔΦ=360 degree*frequency error*duration. The calculated results of phase difference with different durations are listed in table 4.

Table 4. Phase difference with different durations

| Duration | 1ms | 2ms | 4ms | 8ms | 16ms | 32ms |
| --- | --- | --- | --- | --- | --- | --- |
| Phase difference for LEO-600 | 0.1944degrees @0.54Hz | 0.7776degrees @1.08Hz | 3.1104degrees @2.16Hz | 12.4416degrees @4.32Hz | 49.7664degrees @8.64Hz | 199.0656degrees @17.28Hz |
| Phase difference for LEO-1200 | 0.0936degrees @0.26Hz | 0.3744degrees @0.52Hz | 1.4976degrees @1.04Hz | 5.9904degrees @2.08Hz | 23.9616degrees @4.16Hz | 95.8464degrees @8.32Hz |

It can be observed that phase difference with 16ms/32ms duration at LEO-600 scenarios and with 32ms duration at LEO-1200 scenarios is larger than the maximum allowable phase difference of 30 degrees for slot “0” and any slot “p” wherein DMRS bundling is applied, which is specified in 6.4.2.5-1 in TS38.101-1. Thus, the maximum duration of DMRS bundling due to phase difference is limited by 8ms for LEO-600 scenarios and 16ms for LEO-1200 scenarios.

Observation 5:

- For DMRS bundling, the maximum duration due to phase difference is limited by 8ms for LEO-600 scenarios and 16ms for LEO-1200 scenarios.

According to above, when DMRS bundling is enabled, considering the requirements of phase continuity/frequency error /timing error, the maximum duration of DMRS bundling should be limited as 8ms for LEO-600 scenarios and 16ms for LEO-1200 scenarios. Then, the maximum duration can be up to network implementation via the configuration of TDW length/size, and no spec impact is expected.

Proposal 10: Support the maximum duration of DMRS bundling in NTN as 8ms for LEO-600 scenarios and 16ms for LEO-1200 scenarios, which is up to network configuration without spec impact.

Event which causes power consistency and phase continuity not to be maintained

According to [2], DMRS bundling, also called joint channel estimation, could provide around 2dB gain for repetition transmissions. When DMRS bundling is enabled, the transmitter shall maintain power consistency and phase continuity across repetitions. Then the receiver could perform joint channel estimation within these repetitions.

Based on UE procedure for determining TDWs for DMRS bundling, UE determines one or multiple nominal TDWs within the duration of transmission, where the duration of each nominal TDW except the last nominal TDW is given by TDW length configured in number of consecutive slots. Furthermore, a nominal TDW consists of one or multiple actual TDWs. And the actual TDW is determined by events which cause power consistency and phase continuity not to be maintained across repetitions within a nominal TDW. Thus, when DMRS bundling is enabled, the transmitter shall maintain power consistency and phase continuity within an actual TDW. The events are categorized as dynamic events and semi-static events. UE is mandatory to restart an actual TDW after a semi-static event, while whether to restart an actual TDW after a dynamic event subjects to UE capability.

In order to support DMRS bundling in NTN scenarios, some NTN specific events which may cause power consistency and phase continuity not to be maintained should also be discussed and specified if needed. As discussed in Rel-17 coverage enhancement, TA adjustment could cause power consistency and phase continuity not to be maintained. On top of TA itself in TN, in Rel-17 NTN, the calculation of TA applied by NTN UE has been enhanced which is one of main enhancements on UL synchronization as described in section 4.3.1 of TS38.211 v17.3.0.

Uplink frame number for transmission from the UE shall start TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc before the start of the corresponding downlink frame at the UE where - NTA and NTA,offset are given by clause 4.2 of [5, TS 38.213], except for msgA transmission on PUSCH where NTA=0 shall be used; - NTA,adjcommon given by clause 4.2 of [5, TS 38.213] is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation if configured, otherwise NTA,adjcommon=0; - NTA,adjUE given by clause 4.2 of [5, TS 38.213] is computed by the UE based on UE position and serving-satellite-ephemeris-related higher-layers parameters if configured, otherwise NTA,adjUE=0. Figure 4.3.1-1: Uplink-downlink timing relation.

As can be seen, when common TA or UE specific TA takes effect, power consistency and phase continuity cannot be maintained, which should be regarded as events.

Although gNB can try to not allow the common TA to take effect within a nominal TDW by implementation when DMRS bundling is enabled, TACommonDrift and TACommonDriftVariation if configured could still result in the update of common TA. When the common TA is updated, it would be hard to control the repetition transmissions supporting DMRS bundling. In current spec., when and how to the updated common TA is up to UE implementation. In order to keep this UE behavior, it is preferred to regard the update of a common TA as an event. The application time of the updated common TA could be indicated or configured together with TACommonDrift and TACommonDriftVariation, if DMRS bundling is enabled.

Furthermore, UE specific TA is calculated by the UE based on UE position and serving-satellite-ephemeris-related higher-layers parameters if configured. Such UE specific TA could be reported to gNB, which is described in section 5.4.8 of TS 38.321. As can be seen, UL-SCH resource is needed for time advance report (TAR).  Similar to the discussions on common TA, the application of UE specific TA should also be regarded as an event. The application time of the UE specific TA should be known by gNB, which could be further discussed.

According to above, we have following proposal.

Proposal 11: The application of common TA and UE specific TA should be regarded as events which cause power consistency and phase continuity not to be maintained, when DMRS bundling is enabled.

In Rel-17 NR NTN, a single validity duration for both serving satellite ephemeris and common TA related parameters is defined, which determines the maximum time during which the UE can apply the satellite ephemeris without having acquired new satellite ephemeris. Since serving satellite ephemeris and common TA related parameters are necessary to maintain phase continuity and power consistency in NTN, DMRS bundling could be performed within the validity duration. Out of the validity duration, DMRS bundling should be disabled or cannot be performed.

Observation 6:

- DMRS bundling should be performed within the validity duration.

And the validity timer should be started/restarted with configured timer validity duration at the epoch time. That means, the serving satellite ephemeris and common TA related parameters share the same epoch time. If the validity timer is restarted due to the update of epoch time within a TDW, the phase continuity and/or power consistency would be violated. Thus,  the reception of epoch time or epoch time itself, depending on the understandings of the application time of the updated the serving satellite ephemeris and common TA related parameters as discussed in the LS from RAN2 [3], should be regarded as events.

Proposal 12: The indication or update of epoch time should be regarded as events which cause power consistency and phase continuity not to be maintained, when DMRS bundling is enabled.

  - Other enhancements

Compared to TN, one of the NTN-specific characteristics should be polarization. Since signals from LHCP and RHCP antennas are orthogonal, frequency re-use scheme with circular polarization can be utilized to mitigate inter-cell interference. Furthermore, both LHCP and RHCP signals can be transmitted simultaneously on the same frequency band. Thus, circular polarization can be regard as another spatial domain on top of antenna ports and circular polarization enhancement on Tx diversity could be further studied.

As is known, smart phones can only support linear polarization. To support circular polarization in smart phone especially for low band, it will be quite complex and challenging to the UE RF design. Therefore, circular polarization should be focused on only for downlink if needed.

Observation 7:

- Circular polarization can be regard as another spatial domain on top of antenna ports.

Proposal 13: Circular polarization enhancement on Tx diversity could be studied for downlink coverage enhancements in NR NTN if needed.

- Conclusion

In this contribution, we discuss the aspects related to coverage enhancement of NR NTN, and have following  observations and proposals:

Observation 1:

- The performance of PUCCH for Msg4 HARQ-ACK with 4 repetitions can meet the requirement of the CNR calculated in link budget calculation for LEO-1200 set-1 satellite.

- The performance of PUCCH for Msg4 HARQ-ACK with 8 repetitions can match with the performance of PUCCH repetition for HARQ ACK feedback on dedicated PUCCH resource.

Observation 2:

- The performance gain with inter-slot FH and intra-slot FH decreases with the larger repetition number.

- Compared to PUCCH repetitions for msg4 HARQ-ACK without FH, around 0.2dB and 0.4dB additional performance gain from PUCCH repetitions with inter-slot FH and intra-slot FH can be obtained, respectively.

Observation 3:

- For LEO-600 and LEO-1200, the maximum duration of DMRS bundling due to delay drift or timing error requirement is limited by 16ms for SCS of 15kHz.

Observation 4:

- For DMRS bundling with 32ms duration, the maximum doppler shift variation is much smaller than ±0.1ppm the frequency error requirement.

Observation 5:

- For DMRS bundling, the maximum duration due to phase difference is limited by 8ms for LEO-600 scenarios and 16ms for LEO-1200 scenarios.

Observation 6:

- DMRS bundling should be performed within the validity duration.

Proposal 1: Legacy behaviour for Msg4 HARQ-ACK feedback on PUCCH applies when repetition factor is not configured via SIB.

Proposal 2: If only one repetition factor is configured via SIB, support 1 PUCCH repetition for Msg4 HARQ-ACK.

Proposal 3: If multiple repetition factors are configured via SIB, UE repetition capability should be reported or indicated before one of the PUCCH repetition factors for Msg4 HARQ-ACK is dynamically indicated.

Proposal 4: For the indication or report of PUCCH repetition capability, support to reuse mechanism of msg3 repetition capability reporting.

Proposal 5: If single repetition factor is configured via SIB, it’s up to UE to determine whether to perform PUCCH repetition for Msg4 HARQ-ACK.

Proposal 6: Do not support number of PUCCH repetitions larger than 8 for msg4 HARQ-ACK feedback.

Proposal 7: Support to enable/disable inter-slot frequency hopping for PUCCH repetition for msg4 HARQ-ACK considering the following alternatives

- Alt 1: reusing the frequency hopping flag information field in UL RAR grant when the repetition number is configured to be more than 1

- Alt 2: repurpose some fields in scheduling DCI corresponding to msg4 PDSCH

Proposal 8: Do not support the DMRS bundling for PUCCH repetition for msg4 HARQ-ACK.

Proposal 9: RAN1 to conclude UE is not expected to perform pre-compensation on timing and frequency offset within an actual TDW when DMRS bundling is enabled.

Proposal 10: Support the maximum duration of DMRS bundling in NTN as 8ms for LEO-600 scenarios and 16ms for LEO-1200 scenarios, which is up to network configuration without spec impact.

Proposal 11: The application of common TA and UE specific TA should be regarded as events which cause power consistency and phase continuity not to be maintained, when DMRS bundling is enabled.

Proposal 12: The indication or update of epoch time should be regarded as events which cause power consistency and phase continuity not to be maintained, when DMRS bundling is enabled.

Proposal 13: Circular polarization enhancement on Tx diversity could be studied for downlink coverage enhancements in NR NTN if needed.

References

- Chairman notes for 3GPP TSG RAN WG1 #111, November 14-18, 2022.

- R1-2206063, Discussions on coverage enhancement for NR NTN, vivo, 3GPP TSG RAN WG1 #110, August 22-26, 2022.

- R2-2211048, LS on validity of assistance information, 3GPP TSG RAN WG2 #119bis-e, October 10-19, 2022.

Appendixes

Appendix A. Evaluation assumptions for PUCCH repetition

Table A1. Link-level simulation assumptions for PUCCH transmission

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 2bits UCI or 1-bit for HARQ-ACK for Msg4. |
| Carrier frequency | 2 GHz |
| SCS | 15kHz |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. |
| Antenna configuration | 1T1R |
| Repetitions | w/ repetition. The maximum number of repetitions is 8 for 2bits UCI; {1, 2, 4, 8} repetitions for 1-bit Msg4 HARQ-ACK. |
| Symbol duration | 14 OS |
| Number of PRBs | 1 PRB |