3GPP TSG RAN WG1 #112	R1-2300475

Athens, Greece, February 27th –March 3rd, 2023

Source:	vivo

Title:	Discussion on TA management for L1/L2 Mobility

Agenda Item:	9.12.2

Document for:	Discussion and Decision

Introduction

In the RAN1#111 meeting, TA management-related issues, e.g., TA acquisition, TA maintenance, and the association between TA and candidate cell, were discussed and the following agreements were achieved [1].

Agreement On mechanism to acquire TA of the candidate cell(s) in Rel-18 LTM, at least support PDCCH ordered RACH. - The PDCCH order is only triggered by source cell - FFS: the details including content of DCI, RACH resource configuration, RAR transmission mechanism, etc. - Note: any other RACH-based solutions are for discussion separately Agreement For PDCCH ordered RACH in LTM, at least the following enhancements are supported - Introduce indication of candidate cell and/or RO of candidate cell in DCI - configuration of RACH resource for candidate cell(s) is provided prior to the PDCCH order - FFS: whether/how to transmit RAR Agreement On whether RAR is needed for PDCCH ordered RACH for a candidate cell in LTM, the following alternatives are considered for further study - Alt 1: RAR is needed - Alt 2: RAR is not needed - Note: If Alt 2 is supported, TA value of candidate cell is indicated in cell switch command - Alt 3: whether RAR is needed can be configured Agreement - TA updating (i.e. re-acquisition of TA) for candidate cell can be triggered by NW. - same triggering mechanism reuse the initial TA acquisition, i.e., PDCCH order triggered RACH in a candidate cell

Based on the above agreements and discussion in the last meeting, this contribution would be divided into three parts to discuss potential solutions and remaining issues as follows:

- TA acquisition for candidate cells, e.g., TA acquisition methods and TA indication.

- TA maintenance for candidate cells, e.g., the number of TAs and the procedure of relative timing adjustment.

- TA application, e.g., the association between TA and candidate cell(s) and TA switch.

TA acquisition

In RAN1#110bis-e meeting, TA acquisition of candidate cell(s) before receiving cell switch command had been agreed to reduce the handover latency, except for the case that the candidate cell is a deactivated SCell, which may need further study on its necessity.  In our view, it should be supported if the time alignment timer corresponding to a TAG of the deactivated SCell involved has expired, TA acquisition should be performed for the deactivated SCell before cell switch command is received. While for the case that the time alignment timer is not expired, which means UE maintains uplink synchronization with the deactivated SCell, TA acquisition of the deactivated SCell is not needed.

TA acquisition should be performed for the deactivated SCell before receiving cell switch command when the candidate cell is a deactivated SCell.

For the TA acquisition methods, many potential solutions discussed in the previous meetings can be divided into two categories, i.e., RACH-based solutions and RACH-less solutions.

RACH-based TA acquisition

In the last meeting, early RACH triggered by the PDCCH order to achieve initial time alignment for candidate cells before handover was agreed, and related details including the content of DCI, RACH resource configuration, and the need for RAR transmission were discussed without reaching consensus.

In Rel-16, random Access Preamble index, SS/PBCH index and PRACH Mask index are included in PDCCH order to establish secondary TAG or to achieve uplink synchronization. After receiving PDCCH order, the UE would transmit corresponding preamble to the serving cell which sent the PDCCH order. For Rel-18 LTM, the functionality of PDCCH order is to aquire TA for candidate cell(s) before handover. Therefore, besides the contents mentioned above, the candidate cell identity should be added in PDCCH order to inform UE which candidate cell to send the preamble to.

Besides the contents included in PDCCH order in Rel-16, candidate cell identity should be introduced.

For the RAR transmission, we think it should be discussed separately for different scenarios. If there is no uplink transmission between candidate cell and UE before cell switch command is received, it is meaningless for UE to monitor the RAR to obtain the TA value for the candidate cell before handover. And the absolute TA value could be carried in the cell switch command. If uplink transmission between candidate cell and UE has been supported before handover, like Rel-17 inter-cell beam management or inter-cell mTRP, when UE receives the RAR, it can transmit uplink signals based on the initial TA value carried in the RAR at once.  To minimize the spec impact and workload, we only support CFRA and the RAR is transmitted from the source cell. With the above cases, we think whether RAR transmission is required or not can be determined by the network configuration or indication.

For PDCCH order-based early RACH,

- Only support CFRA.

- Support the network to configure whether the RAR is needed or not.

  - If needed, the RAR is transmitted from the source cell.

  - If not needed, the absolute TAC is carried in the cell switch command.

To support early RACH triggered by the PDCCH order to achieve initial time alignment for candidate cells before handover, the configuration of candidate cell(s), e.g., RACH and SSB, needs to be applied before handover. Besides, considering scheduling restrictions between RACH procedure and signals from the serving cell, especially in inter-frequency scenarios, the maximum number of candidate cell(s) that UE can achieve time alignment before handover should also be discussed and may be indicated in UE capability.

To support early RACH triggered by the PDCCH order to achieve initial time alignment for candidate cells before handover.

The configuration of RACH for candidate cell(s) needs to be applied before handover.

The maximum number of candidate cell(s) that UE can achieve time alignment before handover should be discussed and may be indicated in UE capability.

RACH-less TA acquisition

Except for early RACH triggered by PDCCH order, RACH-less solutions can also be considered to reduce handover latency and UE complexity in certain cases.

SRS based TA acquisition

For intra-frequency scenarios, initial time alignment can be achieved by other UL signals, e.g., SRS. In this case, UE transmits SRS based on the downlink receive timing of the source gNB, and candidate gNBs measure the SRS and forward the measurement results to source gNB. Meanwhile, candidate gNBs also can calculate their own timing advance value based on the SRS resource and then forward it to the source gNB respectively. As shown in Figure 2, in synchronous scenarios, the timing offset calculated by candidate gNB(s) is , and the timing offset calculated by the source gNB(s) is . Source gNB can obtain correct TA values of the candidate gNB(s) as .  In asynchronous scenarios, the downlink timing difference between source gNB and candidate gNB(s) can be obtained by interaction in advance, thus the TA value of the candidate gNB(s) also can be acquired similarly.

An example of timing advance adjustment procedure

Support UL measurement based on SRS to achieve initial time alignment for candidate cells before handover in intra-frequency scenarios.

After the serving gNB obtains absolute TA values, UE can be informed by the absolute TAC MAC CE as shown in Figure 2.  To avoid ambiguity in the relationship between TACs and cells, cell identity needs to be added in the absolute timing advance command MAC CE.

Absolute Timing Advance Command MAC CE

Support to introduce cell identity in the absolute timing advance command MAC CE to avoid ambiguity of the relationship between absolute TACs and cells.

UE-based TA measurement

UE-based TA measurement with one TAC from the serving cell

For synchronized scenarios, the initial TA of the candidate cell(s) can be acquired by UE-based measurement with one TAC from the source cell. For example, as shown in Figure 3, the TA value of an absolute or relative TAC from the source gNB is 2*s1, the downlink receives timing difference between the source gNB and the candidate gNB is s2-s1, and based on them, if the downlink timing difference between the gNBs is negligible, the TA value for the candidate gNB can be calculated as 2*s1+2*(s2-s1) =2*s2.

Support UE-based TA measurement to acquire initial TA for candidate cell in the synchronized scenario.

TA maintenance

UE capability on TA maintenance

After achieving initial time alignment for candidate cells, multiple TAs for candidate cells should be maintained by UE before handover, especially when the source cell is configured with Rel-17 ICBM or inter-cell mTRP. Thus, when receiving the cell switch command, UE can transmit UL signals based on the maintained timing advance corresponding to the target cell at once.  However, maintenance of multiple TAs requires corresponding UE capability, since UE needs to monitor and maintain the synchronization of multiple uplinks simultaneously. And, in AI 9.1.1.2, two TAs is being discussed in MTRP scenarios and UE capability on maintaining TAs for two TRPs had been agreed. Similarly, a UE capability on multiple TAs maintenance for candidate cells also should be supported in L1/L2 mobility. As for the maximum number of TAs UE can maintain simultaneously, it needs further discussion.

Maintenance of multiple TAs for candidate cell(s) and serving cell shall be a UE capability, and the maximum number of the TAs is FFS.

If maintenance on multiple TAs for candidate cell(s) and serving cell is not supported by UE, the most straightforward method is that UE only maintains the TA of the source gNB before handover and maintains the TA of the target gNB after handover. Thus, multiple TAs for candidate cells would be maintained at the gNB side based on Msg1 or SRS. And, when the network determines to perform handover, the TAC for the target gNB could be carried in cell switch command for the UE.

If the UE is not capable of maintaining multiple TAs for candidate cell(s) and serving cell, the TAC for the target cell could be carried in cell switch command for the UE.

If maintenance of multiple TAs for candidate cells is supported by UE, besides initial time adjustment in the RACH procedure, UL timing can also be maintained by relative time adjustment using the relative TAC received from the gNB.

Relative time adjustment

In this section, we discuss how UE acquires cell-specific TAC for cell-specific relative time adjustment. The following two alternatives can be considered.

- Alt1: UE receives multiple relative TACs associated with candidate cell(s) and serving cell.

- Alt2: UE receives one relative TAC associated with serving cell, and TACs associated with candidate cell(s) are determined by the received TAC together with DL receive timing difference.

For Alt1, cell identity also needs to be added in the timing advance command MAC CE in Figure 4, or the TAG ID may be extended to indicate TAG association with a specific candidate cell. Because TAG configuration is provided per CG and TAG id is numbered within the CG, thus TAG id in source CG and target CG may be the same. To avoid this ambiguity, introducing the cell identity in the relative TAC command is a necessity.

Timing Advance Command MAC CE

When UE maintains multiple TAs based on multiple TACs, cell identity also needs to be added in the timing advance command MAC CE to avoid ambiguity of the relationship between relative TACs and cells.

For Alt2, as shown in Figure 5, one TAC received by UE is determined based on the source gNB, then UE calculates the TA for target cell by utilizing the timing offset between DL receive timing from source cell and target cell. Considering source gNB and target gNB may not be synchronized, the additional error will be introduced for target gNB, because the timing offset may be unfortunately compensated along with propagation delay. The offset can be calculated by utilizing two TACs received from two RACH procedures and the DL timing difference from two TRPs measured by UE as shown in Figure 5.

An example of time alignment in two RACH procedures

Timing offset p between non-synchronized TRPs can be obtained through two RACH procedures.

Assuming timing offset p between non-synchronized gNBs is obtained through two RACH procedures, Table 1 provides an analysis of Rx timing offset for each gNB where TAC is calculated by DL timing difference whereas different propagation delays and timing offsets between gNBs are considered. It can be seen that UL timing for the target gNB is aligned well for all cases.

Calculation on Rx timing offset for each gNB where TAC is calculated by DL timing difference

|  | Source gNB | Candidate gNB#1 | Candidate gNB#3 | Candidate gNB#3 |
| --- | --- | --- | --- | --- |
| DL Tx timing | T | T | T+p | T+p |
| Propagation delay | t | t+d | t | t+d |
| UE Rx timing | T+t | T+t+d | T+t+p | T+t+d+p |
| RX timing difference Δ | Δ=0 | Δ=d | Δ=p | Δ=d+p |
| TA offset (s=2t) | s | s+2Δ | s+2Δ-2p | s+2Δ-2p |
| DL timing reference (based on each gNB) | T+t | T+t+d | T+t+p | T+t+d+p |
| UE Tx timing | T+t-s | T+t+d-s-2Δ | T+t+p-s-2Δ+2p | T+t+d+p-s-2Δ+2p |
| gNB Rx timing | T+2t-s | T+2t+2d-s-2Δ | T+2t-s-2Δ+3p | T+2t+2d-s-2Δ+3p |
| gNB Rx timing offset | 2t-s | 2t+2d-s-2Δ | 2t-s-2Δ+2p | 2t+2d-s -2Δ+2p |
| gNB Rx timing offset | 0 | 0 | 0 | 0 |

Alt 2 has minimum spec impact because we only need to obtain the TAC for the source gNB in advance and other TACs for candidate cells can be acquired by UE implementation. And compared to the network-initiated RACH to update TA for candidate cell(s)  agreed in the last meeting, Alt-2 could save significant RACH overhead, especially when the number of candidate cell is large. Therefore, for the case of initial time alignment achieved by the RACH procedure, we propose to support maintaining UL timing by using the TAC from the source gNB and the DL timing difference between the candidate gNB(s) and source gNB. However, if initial time alignment is achieved by SRS, the timing offset p between non-synchronized gNBs cannot be obtained. Therefore, in this case, Alt 1 is more appropriate.

For relative time adjustment when maintaining multiple TAs,

If the initial time alignment is achieved by SRS, support relative time adjustment by multiple TACs;

If the initial time alignment for candidate cell(s) is achieved by RACH procedure, support only one TAC is explicitly indicated and associated with the source gNB, the TA values for candidate cell(s) are determined by the received TAC together with DL receiving timing difference between the source cell and candidate cell(s) respectively.

TAT-related issues

If maintenance of multiple TAs for candidate cells is supported by UE, the TAT-related issues should be further discussed, e.g., TAT configuration, and the condition of TAT to be started/restarted. To simplify multiple TA maintenance procedure, independent TAT configuration should be supported. As shown in Figure 6, serving cell and candidate cells are associated with 4 TAGs, and each TAG is configured with TAT. When UE receives RAR or absolute TAC or relative TAC, the corresponding TAT would be started/restarted.

Support independent TAT configuration for candidate cell(s), and when UE receives absolute TAC, corresponding TAT would be started.

TAG configutaion for candidate cell(s)

When UE receives a PDCCH order from the PCell, it would send a preamble to candidate cell#2. After that, UE would monitor RAR from the PCell with the same QCL-type assumption as the SSB indicated in the PDCCH order. If the RAR is received by UE within a pre-configured window, the TAT associated with TAG#2 would be started. In the procedure of TA maintenance, if multiple TAs are maintained by multiple TACs, when UE receives the TAC associated with a specific TAG, e.g., TAG#1, only the TA value for TAG#1 will be updated and the corresponding TAT will be restarted. If multiple TAs are maintained by UE-based TA measurement with one TAC from the serving cell, once receiving one TAC associated with TAG#1 from the serving cell, UE would calculate and update the TA value for TAG#1, TAG#2, TAG#3 and TAG#4  respectively. Due to the TA updating,  the TATs associated with TAG#1, TAG#2, TAG#3 and TAG#4 need to be restarted.

If multiple TAs are maintained by UE-based TA measurement with one TAC from the serving cell, all TATs that UE maintains need to be restarted.

TA application

The relationship between TA(s)/TAG(s) and candidate cell(s)

In the RAN1#110bis-e meeting, the relationship between TA(s)/TAG(s) and candidate cell(s) had been discussed, and potential schemes are listed below:

- Alt1: Associate TA/TAG and candidate cell implicitly, e.g.,

- the association between TA/TAG and TCI states can be configured

- Alt2: Associate TA/TAG and candidate cell explicitly, e.g.,

- Alt2-1:  the association is provided as a part of candidate cell(s) configuration

- Alt2-2: the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration

In legacy TAG configuration in Rel-15/16, each serving cell is associated with a TAG indicated by TAG id in ServingCellConfig, and all uplink reference signals and channels configured in the serving cell, except PRACH, are transmitted based on the time advance corresponding to the TAG. Therefore, we think it can be extended directly for L1/L2 mobility, that is a TA/TAG id is introduced in each candidate cell configuration, i.e., Alt 2-1, and the timing advance of all uplink reference signals and channels configured in the candidate cell, except PRACH, follow the indicated TA/TAG. Compared to Alt1 and Alt2-2, which introduce additional association between TCI states or RS groups between TA/TAG complicating the specification design, Alt2-1 is the simplest one and has less specification impact.

Support introducing a TA/TAG id in each candidate cell configuration explicitly.

TA switch

Besides the association between TA(s)/TAG(s) and candidate cell(s), another issue related to TA application is TA switch. Regarding TA switch, the related issue is when to apply TA. In AI 9.12.1, following scenarios were listed based on the beam indication time in RAN1#110bis-e and scenario 2 had been agreed in the last meeting.

  - Scenario 1： beam indication is before cell switch command.

  - Scenario 2： beam indication is together with cell switch command

  - Scenario 3： beam indication is after cell switch command

In our view, for scenario 2 where tx/rx to/from candidate cells does not happen before cell switch command is received, TA switch before cell switch is meaningless. After UE receiving cell switch command, UL signals need to be transmitted to the target cell based on the TA values for the target cell. Therefore, the TA switch is preferred together with the cell switch for scenario 2. Contrary to scenario 2, tx/rx to/from candidate cells may happen before cell switch command is received in scenario 1. Therefore, TA switch is applied together with beam switch, where the new beam is associated with a cell identity different from that of the old beam and the TAGs associated with the cell identities are different. For scenario 3, beam indication is after cell switch command, which means data transmission based on the indicated TCI state comes later than that of scenario 1 and 2. From the perspective of UE experience on cell switch, scenario 3 is not a good choice.

For the timing of the TA switch,

- It is applied together with the beam switch in scenario 1 when the new beam is associated with a cell identity different from that of the old beam and the cell identities are associated with different TAs/TAGs.

- It is applied together with the cell switch in scenario 2 when the target cell and the source cell associate different TAs/TAGs.

Conclusions

- Timing offset p between non-synchronized TRPs can be obtained through two RACH procedures.

Proposal 1:	TA acquisition should be performed for the deactivated SCell before receiving cell switch command when the candidate cell is a deactivated SCell.

Proposal 2:	Besides the contents included in PDCCH order in Rel-16, candidate cell identity should be introduced.

Proposal 3:	For PDCCH order-based early RACH,

- Only support CFRA.

- Support the network to configure whether the RAR is needed or not.

  - If needed, the RAR is transmitted from the source cell.

  - If not needed, the absolute TAC is carried in the cell switch command.

Proposal 4:	To support early RACH triggered by the PDCCH order to achieve initial time alignment for candidate cells before handover.

- The configuration of RACH for candidate cell(s) needs to be applied before handover.

- The maximum number of candidate cell(s) that UE can achieve time alignment before handover should be discussed and may be indicated in UE capability.

Proposal 5:	Support UL measurement based on SRS to achieve initial time alignment for candidate cells before handover in intra-frequency scenarios.

Proposal 6:	Support to introduce cell identity in the absolute timing advance command MAC CE to avoid ambiguity of the relationship between absolute TACs and cells.

Proposal 7:	Support UE-based TA measurement to acquire initial TA for candidate cell in the synchronized scenario.

Proposal 8:	Maintenance of multiple TAs for candidate cell(s) and serving cell shall be a UE capability, and the maximum number of the TAs is FFS.

Proposal 9:	If the UE is not capable of maintaining multiple TAs for candidate cell(s) and serving cell, the TAC for the target cell could be carried in cell switch command for the UE.

Proposal 10:	When UE maintains multiple TAs based on multiple TACs, cell identity also needs to be added in the timing advance command MAC CE to avoid ambiguity of the relationship between relative TACs and cells.

Proposal 11:	For relative time adjustment when maintaining multiple TAs,

- If the initial time alignment is achieved by SRS, support relative time adjustment by multiple TACs;

- If the initial time alignment for candidate cell(s) is achieved by RACH procedure, support only one TAC is explicitly indicated and associated with the source gNB, the TA values for candidate cell(s) are determined by the received TAC together with DL receiving timing difference between the source cell and candidate cell(s) respectively.

Proposal 12:	Support independent TAT configuration for candidate cell(s), and when UE receives absolute TAC, corresponding TAT would be started.

Proposal 13:	If multiple TAs are maintained by UE-based TA measurement with one TAC from the serving cell, all TATs that UE maintains need to be restarted.

Proposal 14:	Support introducing a TA/TAG id in each candidate cell configuration explicitly.

Proposal 16:	For the timing of the TA switch,

- It is applied together with the beam switch in scenario 1 when the new beam is associated with a cell identity different from that of the old beam and the cell identities are associated with different TAs/TAGs.

- It is applied together with the cell switch in scenario 2 when the target cell and the source cell associate different TAs/TAGs.

References

- Chairman’s notes, RAN1#111.