3GPP TSG-RAN WG1 Meeting #112	R1-2300489

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.1.1.2

Source:	FGI

Title:	Discussion on two TAs for multi-DCI

Document for:	Discussion and Decision

Introduction

In RAN#98-e meeting, the Rel-18 WID on NR MIMO evolution for downlink and uplink was approved [1], which includes the following objective corresponding to the TA enhancement for multi-DCI multi-TRP:

- Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

In the previous meetings, some agreements are achieved to make progress for TA enhancement of the UL multi-DCI multi-TRP transmission. However, there are still some pending issues that need to be discussed. In this contribution, we will provide our views on the pending issues.

Discussion

Association between TAs and UL Channels/Signals

In RAN1#110bis-e, the following agreement was made:

Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, the four options agreed in RAN1#110bis-e are refined as below (down-selection of one or a combination of the options to be performed in RAN1#111): - Option 1: Associate TAG to TCI-state/spatial relation - Configure TAG ID as part of UL/joint TCI state or spatial relation - for UL transmission, the TAG ID associated with the UL/joint TCI state or spatial relation is utilized - Option 2: Associate TAG to CORESETPoolIndex - for dynamically scheduled/activated PUSCH, TAG associated with the CORESET pool index of the CORESET carrying the scheduling/activating PDCCH is utilized for UL transmission - for Type 1 CG, P/SP-SRS, and P/SP-PUCCH, coresetPoolIndex is RRC-configured. - FFS: Other signals/channels: AP-SRS, and dynamic HARQ-ACK - Option 3: Associate TAG to SSB group (if such an association is agreed in agenda 9.1.1.2). For a UL transmission, UE adopts the TAG associated with the SSB group such that - if the PL RS is an SSB, then the UE adopts the TAG associated with the SSB group which the PL RS of the UL transmission belongs to - if the PL RS is a CSI-RS, then the UE adopts the TAG associated with the SSB group which the QCL source SSB of the PL RS belongs to - Option 4: TAG association performed as follows: - for dynamically scheduled/activated channels/signals, TAG associated with the CORESET pool index of the CORESET carrying the scheduling PDCCH is utilized for UL transmission - for P/SP UL channels / signals (not scheduled or activated by DCI), TAG ID is RRC-configured.

The issue corresponding to how to associate TAs and UL channels/signals has been discussed for a long time. However, it is hard to achieve one compromise solution. For Option1, it is a simple solution since each UL channel/signal needs to be associated with one or more TCI state(s)/spatial relation information for UL transmission. Moreover, it provides more flexibility to the gNB for scheduling. Even though such flexibility may result in some error cases (e.g., one TRP is configured with multiple TAGs or two TRPs are configured with one TAG), the gNB can avoid these error cases by a proper configuration. In addition, Option 1 can be applied to other scenarios (e.g., single-DCI-based multi-TRP) easily for future release. The main problem with Option 1 is that it cannot be applied for FR1 since the spatial relation cannot be configured for FR1 for now. However, this problem can be solved simply by introducing spatial relations for FR1.

Proposal 1: Support Option 1 to associate TAs and UL channels/signals.

Proposal 2: RAN1 to Introduce spatial relations for FR1

Additional Type1 CSS for Receiving RAR from a TRP Corresponding to an Additional PCI

In RAN1#110bis-e, the following agreement was made:

Agreement For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, support one of the alternatives (down selection to be done in RAN1#111): - Alt 1: PDCCH scheduling RAR will always be received from serving cell è there is no need for additional type 1 CSS configuration per additional PCI - Alt 2: In addition to PDCCH scheduling RAR being received from serving cell, reception of PDCCH scheduling RAR from a TRP corresponding to an additional PCI for a RACH procedure associated to the additional PCI is supported è additional type 1 CSS configuration per additional PCI needs to be supported

The issue that whether the PDCCH scheduling RAR can be received from a TRP corresponding to a non-serving cell (or additional PCI) has been discussed for a long time. Regarding this issue, the problem is that it may result in large UE complexity for PDCCH reception/monitoring if enabling to receive PDCCH scheduling RAR from a TRP corresponding to a non-serving cell. From our perspective, it may not be worth supporting this feature.

Proposal 3: Support Alt 1 since Alt 2 may result in large UE complexity.

Conclusions

In this contribution, we have the following proposals:

Proposal 1: Support Option 1 to associate TAs and UL channels/signals.

Proposal 2: RAN1 to Introduce spatial relations for FR1

Proposal 3: Support Alt 1 since Alt 2 may result in large UE complexity.

References

- [1] RP-223276, WID Update: MIMO Evolution for Downlink and Uplink, Samsung

- [2] Chair’s notes RAN1#111, 3GPP TSG RAN WG1 #111, November 14th – 18th, 2022

- [3] Chair’s notes RAN1#110bis-e, 3GPP TSG RAN WG1 #110bis-e, October 10th – 19th, 2022