3GPP TSG RAN WG1 #112	R1-2300512

Athens, Greece, Feb. 27th – March 3rd, 2023

Source:	Lenovo

Title:	CSI enhancements for high mobility and coherent JT

Agenda Item:	9.1.2

Document for:	Discussion and Decision

# Introduction

In RAN#94-e [1], it was agreed to study, and, if applicable, specify CSI enhancements for CJT under FR1, CSI reporting enhancements for high mobility, in addition to time-domain channel property reporting enhancements. In this contribution we provide our views on different aspects of these three issues.

# CSI enhancement for coherent joint transmission

In RAN1#111 [2], the following was agreed for CSI enhancements for CJT under MIMO

Agreement For the Type-II codebook refinement for CJT mTRP, the values of the following codebook parameters are gNB-configured via higher-layer (RRC) signaling and common across all N CSI-RS resources: - N1, N2 - Note: Since (O1, O2) are fixed for a given (N1, N2), (O1, O2) are also common across all N CSI-RS resources - R,  - Note: Since N3 is inferred from the number of CQI subbands and R, N3 is also common across all N CSI-RS resources - Note: Since per-layer K0 is defined jointly across all N CSI-RS resources, so is . Conclusion On the Type-II codebook refinement for CJT mTRP, for mode-1 and mode-2, there is no consensus on introducing additional/explicit per-CSI-RS-resource amplitude scaling and/or co-phase (with separate alphabet set(s)) as additional PMI component(s). - Note: This conclusion has no impact on the Working Assumption reached in RAN1#110bis-e regarding W2 quantization group Agreement On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter R, the supported value(s) from the legacy specification are reused. - FFS: whether additional value 4 can also be added Agreement On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter , introduce as a candidate value  = 1/8 in addition to the supported value(s) from the legacy specification. - FFS (by RAN1#111): whether additional value 1 can also be added Agreement On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter pv, in addition to the supported value(s) from the legacy specification for Rel-16 regular eType-II codebook, introduce as a candidate value - pv = 1/8 for v=1,2 (hence 1/16 for v=3,4) FFS (by RAN1#111): whether additional value pv = 1/2 for v=1,2,3,4 can also be added Agreement On the Type-II codebook refinement for CJT mTRP: - The maximum value of 2NN1N2 is 128 - The support of 2NN1N2 >32 is UE optional for UEs supporting Rel-18 CJT CSI enhancement - Note: Following the legacy specification on the maximum number of NZP CSI-RS ports per CSI-RS resource, the maximum value of 2N1N2 is 32. Agreement On the Type-II codebook refinement for CJT mTRP, on the L parameter, down select from the following alternatives (by RAN1#111): - Alt1. Each of the {Ln, n=1, ..., N} is gNB-configured via higher-layer (RRC) signaling - Alt4. Lmax is gNB-configured via higher-layer (RRC) signaling and the relative value(s) of {Ln, n=1, ..., N} are reported by the UE - The relative value(s) of {Ln, n=1, ..., N} are reported by the UE, such that n=1NLn≤Lmax - TBD: Whether the value(s) of {Ln, n=1, ..., N} or the total value of n=1NLn are reported implicitly or explicitly FFS (by RAN1#111): - Whether the supported candidate values for Ln follow the legacy candidate values for L, or some additional value(s) are also supported - If Alt4 is supported, whether the candidate values for Ln are gNB-configured via higher-layer signaling Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, study and down select (no later than RAN1#112) only one from the following schemes: - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources - Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) - Alt3. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf,n where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf,n is independently selected across N CSI-RS resources For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FS basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources Conclusion: On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter , there is no consensus in supporting the additional value of 1. Agreement On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter pv, support the additional value of pv=1/2 for v=1,2,3,4 with the following condition: - Only to be used in combination with other parameter value(s) to limit the increase in PMI overhead comparable to the maximum overhead of the legacy Rel-16/17 Type-II codebooks (exact parameter combination(s) FFS). Agreement On the Type-II codebook refinement for CJT mTRP, regarding the bitmap(s) for indicating the locations of NZCs, reuse the legacy design. This implies that the size of the bitmap for selected CSI-RS resource n (Bn) is, Bn=2LnMv - FFS: additional mechanism to reduce bitmap overhead for larger N values, e.g. including via Parameter Combination. Agreement On the W2 coefficient quantization scheme for the Type-II codebook refinement for CJT mTRP, for N=3 and N=4, just as N=2, reuse the following components of the legacy Rel-16/17 per-coefficient quantization scheme: - Alphabets for amplitude and phase - Quantization of phase and quantization of differential amplitude relative to a reference, reference amplitude (with SCI determining the location of one reference amplitude), where the reference is defined for each layer and each “group” of coefficients Agreement On the Type-II codebook refinement for CJT mTRP, regarding the SD basis selection, for a configured value of NTRP, a set of NL combinations of values for {L1, ..., LNTRP} is gNB-configured via higher-layer (RRC) signaling - When NL>1, the selected combination of values for {L1, ..., LNTRP} is reported in CSI part 1 using an indicator, selected from the NL configured combinations - NL =1 is one of the supported candidate values - FFS: Other supported value(s) of NL, and its respective UE capability - FFS: The supported combinations of values for {L1, ..., LNTRP} - Following the legacy design, the SD basis selection for the n-th (n=1,...,N) selected CSI-RS resource is indicated in CSI part 2 using a combinatorial indicator selected from a set of PCSI-RS/2Ln codepoints where, for Rel-16-based refinement PCSI-RS = 2*N1N2. - The supported candidate values for each of the Ln parameters include the legacy candidate values, i.e. {2,4,6} for Rel-16-based refinement, and - for Rel-17-based refinement, the gNB configures a set of N_L combinations for {alpha1, ..., alphaNTRP} Ln=αnPCSI-RS/2 where αn=12,34, 1 FFS: Whether the set of NL combinations of values for {L1, ..., LNTRP} can be implicitly derived Following the legacy design, for all the selected N CSI-RS resources, the SD basis oversampling group for each CSI-RS resource is indicated in CSI part 2 using an indicator selected from a set of O1O2 codepoints.

In the remainder of this section, we provide our views on codebook CSI enhancement for CJT, in light of the agreements and discussions in the previous Rel-18 RAN WG1 meetings.

## Codebook design

One aspect of potential Rel-18 CJT codebook design is reporting the total number of non-zero coefficients. In our opinion, two alternatives exist:

- Alt1. Reporting the total number of non-zero coefficients across all TRPs in Part 1 of the CSI report, whereas the number of non-zero coefficients per TRP can be derived from the per TRP coefficients bitmap.

- Alt2. Reporting the number of non-zero coefficients per TRP separately in the CSI report.

- FFS: whether/how the multiple indicators of the number of non-zero coefficients are reported across the two parts of the CSI report

While Alt1 may result in less overall CSI feedback overhead, Alt2 has the advantage of having straightforward mapping between the reported number of non-zero coefficients and the respective PMI. Further study and down selection of both alternatives is needed.

Further study whether/how the number of non-zero coefficients is reported in the CSI report, e.g., separately for each TRP or jointly for all TRPs

In RAN1 #111, it was agreed that a set of NL combinations of values for L1,…, LNTRP can be configured by the network corresponding to the number of SD basis indices per TRP, where the UE feeds back an index of the selected combination of values for L1,…, LNTRP in Part 1 of the CSI report, in case of NL>1. The supported candidate values for each of the Ln parameters include the legacy candidate values, whereas the supported combinations of values for L1,…, LNTRP as well as the number of supported combinations NL need further discussion. In principle, the larger the supported value of NL, the higher the complexity of the beam combination selection by the UE, and the larger the number of additional UCI bits needed in Part 1 of the CSI report for indicating the selected beam combination. Moreover, given that it was agreed in [3] that the UE reports a sub-selection of N TRPs from the configured set of NTRP TRPs for CJT, the number of beam selection hypotheses would grow up to NL∙2NTRP-1 corresponding to all TRP combinations in conjunction with all the beam selection combinations, leading to significantly higher complexity of selecting the TRP-beam combination. From another aspect, the number of bits needed for reporting the indices of the selected beams based on the beam combination value is log2PCSI-RS/2Ln bits per TRP. Considering the aspects above, we propose the following potential solution alternatives:

- Alt1. All permutations of a length NTRP beam combination vector are supported as candidate beam combinations, e.g., for NTRP=3, the configured beam combination (2,6,4) also implies the following beam combinations (2,4,6), (4,2,6), (4,6,2), (6,2,4) and (6,4,2) are candidate beam combinations. An additional indicator of the selected permutations may need to be reported

- Alt2. The total number of beams Ltot per beam combination is restricted with a maximum value, e.g., for NTRP=3, the beam combination (2,2,4) implies a constraint on the total number of beams Ltot≤2+2+4=8, where the possible combinations of values are {2, 2}, {2, 4}, {4, 2}, {4, 4}. For 3 TRPs, the possible combinations of values are {2, 2, 2}, {2, 2, 4}, {2, 4, 2}, {4, 2, 2}. For 4 TRPs, the only possible combination of values would then be {2, 2, 2, 2}.

For the number-of-beam combination reporting, consider the following alternatives for down selection:

- Alt1. Permutations of a configured combination of the number of beams are supported as additional candidates of number-of-beam combinations

- Alt2. A constraint on the aggregate number of beams is implied from the configured number-of-beam combinations

- In RAN1 #110 [4], it was also agreed that two modes are supported for Rel-18 Type-II codebook for CJT corresponding to whether a common FD basis transformation matrix is supported for the selected N TRPs in CJT. Furthermore, in RAN1 #111, three alternatives were agreed for study and down selection for the basis vectors of CJT codebook for mode-1, as follows:

- Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources.

  - Example formulation: Wf,n=diag1 ej2πN3φn…. ej2πN3(N3-1)φnWf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources

- Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset)

- Alt3. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources.

  - Example formulation: Wf,n=diag1 ej2πN3φn…. ej2πN3(N3-1)φnWf,n where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf,n is independently selected across N CSI-RS resources

Note that mode 1 is designed for inter-site scenario in which the channel is expected to be frequency selective corresponding to distributed TRPs. Clearly, Alt1 is the most restricted alternative, since the N TRPs share a common FD basis matrix with a per-TRP offset. Under Alt2, each TRP is allocated an independent FD basis transformation matrix, Wf,n, whereas Alt3 improves on Alt2 via introducing independent FD basis transformation matrix per TRP in addition to a per-TRP offset. While Alt3 introduces more freedom for FD basis selection, the phase offset can be implicitly included as part of the differential phase coefficient values in coefficient matrices W2,n. Therefore, we prefer Alt2.

Support Alt2 for CJT FD basis selection with independent FD basis selection per TRP for mode-1 of CJT codebook

## Quantization scheme of the non-zero coefficients

In RAN1 #110bis, Alt1 was agreed for W2 quantization group, whereas Alt3 was set as a working assumption that is to be confirmed in RAN1#111. For Alt3, one group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N). The precision for amplitude quantization is improved by quantization per TRP. It can be used in inter-site CJT scenario where large RSRP difference may exist between TRPs. Moreover, supporting 2N amplitude coefficient groups helps ensure that at least one non-zero coefficient is reported per TRP per polarization, which avoids power amplifier inefficiencies that may be caused if no non-zero coefficient is selected for a given polarization per TRP. Therefore, we support to confirm the work assumption Alt3.

Confirm the working assumption Alt3 for W2 quantization group and strongest coefficient indicator design

For Alt3, for each of the 2N–1 amplitude groups (other than the group associated with the SCI), the reference amplitude is reported. Here, one amplitude group may correspond to one polarization of a TRP/TRP group. For the reference amplitude reporting, differential quantization can be used between the reference amplitude for one amplitude group and the largest reference amplitude across all amplitude groups, where the amplitude coefficient of the largest reference amplitude across all amplitude groups is set to ‘1’. The Rel-16 eType-II reference amplitude quantization for parameter i2,3,l provided in Table 5.2.2.2.5-2, Clause 5.2.2.5 of TS 38.214 [5], can be reused for differential quantization of reference amplitude of one amplitude group since amplitude groups are defined corresponding to both TRP and polarization. Due to the larger number of reference amplitude values, a reduction in the quantization resolution of the 2N–1 reference amplitude values should be further studied

Reuse Rel-16 eType-II reference amplitude quantization for differential quantization of the strongest coefficients for each polarization per TRP

- FFS: whether the number of reference amplitude quantization bits can be reduced

## NZC bitmap design

For CJT codebook design, the CSI feedback overhead corresponding to reporting the bitmap for indicating the locations of the non-zero coefficients is expected to be proportional with the number of TRPs N. In RAN1#111, it was agreed to support the legacy bitmap design Bn=2LnMv,n (Mv,n=Mv for mode 2), whereas further enhancements to reduce the CSI feedback overhead corresponding to bitmap reporting is to be further discussed. While one advantage of the legacy design is its straightforward design based on the eType-II and FeType-II codebooks, it may not be efficient for scenarios with a sparse number of ones in the bitmaps, e.g., at smaller β values. In our understanding, one enhancement for the legacy bitmap design at large N values is to report only a subset of the 2LnMv,n bitmap corresponding to each TRP. For instance, assume CJT with Mode 2, with N=2, L1=L2=4, and M=4. Under Alt1, the bitmap overhead is 64 bits. For ease of exposition, we consider the following example:

B1=10000010101100100000001010000000 ,  B2=00000101100011010010000000001000

Based on the example above, only two and three columns of B1, B2, respectively, comprise one-valued entries. If the UE can indicate the column indices with at least one non-zero entry for each bitmap, e.g., via two length-4 bitmaps for both CSI-RS resources, e.g., b1=[1 0 1 0], and b2=[1 1 0 1], the bitmaps B1, B2 overhead can be reduced by reporting B1', B2', as follows

B1'=1001111001010100 ,  B2'=001001100110010010001000

Note that the total UCI overhead corresponding to the bitmaps b1, b2, B1' and B2' is 48 bits, which is a 25% reduction of the CSI overhead corresponding to reporting legacy B1, B2 bitmaps. Note that the same idea can be applied to bitmap rows, i.e., beams. We therefore have the following proposal

For CJT codebook design with N >2, support a two-stage bitmap design, where a first stage bitmap for each CSI-RS resource indicates the indices of rows and/or columns with at least one non-zero coefficient, and a second stage bitmap whose size is based on the indicated rows and/or columns in the first stage bitmap per CSI-RS resource

## Mapping order of CSI fields and CSI omission

For CJT-based CSI report, the CSI report fields may include CSI for multiple TRPs including a maximum of N = 4 TRPs. The potential CSI report may include a distinct CSI Part 1 and CSI Part 2 for each TRP or a joint CSI Part 1 and CSI Part 2 for the cooperative TRPs, with each jointly designed CSI part comprising jointly encoded parameters across TRPs, e.g., joint basis selection and/or number of non-zero coefficients reporting. For CSI Part 1, it may possibly include fields corresponding to RI, CQI, total number of non-zero coefficients across layers, selection of N out of NTRP CSI-RS resources, selected combinations of values for {L1, ..., LNTRP} for SD basis selection. For CSI Part 2, it may include PMI for multiple cooperative TRPs, where some PMI parameters may be separately or jointly determined for multiple cooperative TRPs for codebook mode 1 or mode 2, respectively. The mapping order of the bits corresponding to the CSI fields within the CJT CSI report should be carefully designed, given that some fields may exist for multiple cooperative TRPs. For instance, the mapping order of CSI fields may be based on concatenating N groups of bit sequences, each group comprising the CSI fields corresponding to a given TRP. Alternatively, the CSI fields may be grouped by concatenating different groups corresponding to different CSI report quantities, with each CSI report quantity comprising parameters that are ordered with respect to the TRP index.

Study the mapping order of CSI fields corresponding to CJT-based CSI reporting.

Note that in the case of uplink resource scarcity, or in case the uplink channel quality is low, the resources allocated for UCI may not suffice to carry all CSI fields corresponding to the CJT-based CSI report, and hence the UE may need to omit a portion of the CSI report to adjust to the available uplink resources, e.g., via omitting a subset of CSI Part 2. The conventional CSI omission is defined based on priority level for CSI reports for single TRP transmission, which is defined as Table 5.2.3-1 in TS 38.214 [5], in which CSI Part 2 is omitted level by level, beginning with the lowest priority level until the target data requirement is met, i.e., the maximum number of levels of Part 2 CSI are transmitted given the constraints on the available UL resources and the target rate. When CSI Part 2 fields corresponding to a particular priority level is omitted, the UE shall omit all of the fields corresponding to that priority level. For CJT-based CSI report, the CSI report fields may include CSI for multiple TRPs including a maximum of N=4 cooperative TRPs. The CSI omission scheme may need specific design corresponding to CJT CSI reporting, based on the reported CSI quantities as well as the number of cooperating TRPs. In detail, the CSI omission granularity, such as per TRP omission or joint TRP omission can be further studied for CJT-based CSI reporting. For per TRP omission, the finer omission granularity is introduced. The priority among priority levels corresponding different TRPs can be further studied. Similarly, for joint TRP omission, whether supporting partial reporting of a CSI quantity in one priority reporting level can be discussed to achieve finer omission granularity. When joint TRP omission is made based on defined bit groups, bit priority for determining CSI bits in group 1 or group 2 of a CJT CSI report would need to be further studied. In detail, the TRP index, i.e., the index of NZP CSI-RS for channel measurement, can be introduced for determining priority value Pril,i,f,n, where n is newly introduced TRP index. The priority formula can be extended such as Pril,i,f,n=2⋅N⋅L⋅υ⋅πf+N⋅υ⋅i+N⋅(l-1)+n or Pril,i,f,n=2⋅n⋅L⋅υ⋅Mυ+2⋅L⋅υ⋅πf+υ⋅i+l based on existed bit priority formula for eType2 reports, where N is the number of configured or UE selected TRPs for CJT.

Study whether/how to define a new priority level with finer omission granularity defined per TRP

Study whether/how to define a new bit priority formula including TRP index for joint TRP omission

# CSI enhancement for high/medium UE velocities

In RAN1#111 [2], the following, was agreed for CSI enhancements for high/medium velocities:

Conclusion For the Rel-18 Type-II codebook refinement for high/medium velocities, there is no consensus on applying DD unit for CQI. Therefore, DD unit (of size d slots) is applied only to PMI. - Note: This conclusion has no impact on the number of CQIs included in one CSI reporting instance (a separate issue to be decided separately) Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, - For PMI, DD unit duration of d (in slots) is the duration associated with each of the N4 W2 matrices (combining coefficients before DD compression at the UE, or after DD de-compression at the gNB). - TBD (by RAN1#111): The time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), and the number of CQI(s) X included in a CSI report. Agreement For the Type-II codebook refinement for high/medium velocities, for N4>1, regarding the parameter Q, at least Q=2 is supported. - FFS: Whether Q=3 and/or Q=4 are also supported as other candidate value(s), as well as the supported Parameter Combination(s) Agreement For the Type-II codebook refinement for high/medium velocities, the parameter WCSI (in slots) is determined as follows: WCSI = dN4 Agreement For the Type-II codebook refinement for high/medium velocities, the parameter N4 (length of DFT vector, unit-less) is gNB-configured via higher-layer (RRC) signalling at least from the following set of candidate values: {1, 2, 4} - FFS: If additional candidate value(s) of N4 are supported, e.g. 3, 5, 6, 8, 10, 16, 32, as well as the supported Parameter Combination(s) Agreement For the Type-II codebook refinement for high/medium velocities, the parameter K (the number of AP-CSI-RS resources for the CMR) is gNB-configured via higher-layer (RRC) signalling at least from the following set of candidate values: {4, 8} - FFS: If additional candidate value(s) of K are supported, e.g. 5, 12, 16, also taking into account other use cases (e.g. for training filter coefficients, prediction or performance monitoring) and TDD Agreement For the Type-II codebook refinement for high/medium velocities, the parameter m (offset between two AP-CSI-RS resources for the CMR, in slots) is gNB-configured via higher-layer (RRC) signalling from the following set of candidate values: {1, 2} - FFS: Whether 4, 5, 8, 12, and/or 16 are also supported as other candidate value(s) Agreement For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, support the following: - Q different 2-dimensional bitmaps are introduced for indicating the location of the NZCs, where the qth (q=1,…., Q) 2-dimensional bitmap corresponds to qth selected DD basis vector - The number of selected DD basis vectors is denoted as Q - This implies that for each layer, the location of NZCs in SD-FD can be different for different selected DD basis vectors. FFS: Further overhead reduction on bitmap(s) FFS: Whether the number of NZCs is upper bounded across all DD basis vectors or per DD basis vector Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, - The definition and supported values for each of the SD/FD codebook parameters follow the legacy specification. - FFS: The supported parameter combinations considering SD, FD, and DD codebook parameters - For N4=1, the legacy quantization is fully reused. Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, for N4=1, one CSI reporting instance includes a single W2 per layer, a single W1, and a single Wf. Conclusion For the Rel-18 Type-II codebook refinement for high/medium velocities, when N4>1, there is no consensus in introducing rotation factor (selected for each SD basis vector) as an additional PMI component. Agreement For the Type-II codebook refinement for high/medium velocities, in one CSI reporting instance, for a given CQI sub-band, at least support including one CQI - FFS: The association of the CQI with PMI(s) and/or slot(s) within one duration of CSI reporting window WCSI - FFS: The support for including more than one CQIs Agreement For the Type-II codebook refinement for high/medium velocities, the parameter δ (in slots) is gNB-configured via higher-layer (RRC) signalling from a set of the following candidate values: - First candidate value: δ=0, - 2 additional non-zero values of parameter δ are supported - FFS: the non-zero value(s), to be selected from 1, 2, 3, 4, 5, 6, and 8 Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), assuming 1 CQI in one sub-band and one CSI reporting instance, down-select (by RAN1#112) one from the following alternatives: - Alt1. The CQI is associated with the entire duration of the CSI reporting window and all the N4 W2 matrices - Alt2A. The CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Alt2B. The CQI is associated with the first/earliest d slots of the CSI reporting window and the first/earliest one of the N4 W2 matrices Note: The N4 W2 matrices represent the combining coefficients before DD compression at the UE, or after DD de-compression at the gNB. Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, decide by RAN1#112 whether including X>1 CQIs in one sub-band and one CSI reporting instance are supported. - If supported, also decide the value(s) of X and the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots). Agreement For the Type-II codebook refinement for high/medium velocities, for N4>1, regarding parameter Q, decide in RAN1#112 whether to support the additional values of 3 and/or 4. Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter δ (in slots), support the additional value of 2. - FFS (by RAN1#112): For the last supported additional value, down select between 1, 3, 4, and 5. Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter N4 (length of DFT vector, unit-less), support 8 as an additional candidate value. - FFS (by RAN1#112): Whether any of the following additional candidate values are supported: 3, 5, 16, 32. - The candidate values supported by UE are reported via UE capability (details can be discussed in UE feature). Conclusion: For the Type-II codebook refinement for high/medium velocities, regarding the parameter m (offset between two AP-CSI-RS resources for the CMR, there is no consensus in supporting additional candidate value(s). Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter d (in slots), - Support at least the following candidate value: - If the configured CMR is AP-CSI-RS, this candidate value is the configured value of m parameter - FFS: If the configured CMR is P/SP-CSI-RS - FFS: Whether in the above two cases, the number of slots between the last CSI-RS occasion no later than the legacy reference resource and the starting of WCSI window shall be integer multiples of d slots. - FFS: Whether additional candidate value(s) of d are supported, e.g. d<m, d>m, d=1 If more than one candidate values of d are supported, the value of d is gNB-configured via higher-layer (RRC) signalling

In this section, we discuss the outline of CSI reporting enhancements for high-speed UEs and provide our views on the different aspects of the codebook design that are still open for discussion.

## Codebook design

In RAN1#109-e [6], it was agreed to use Rel-16 eType-II regular codebook and Rel-17 FeType-II PS codebook as baseline for Rel-18 potential codebook design, with the possibility to prioritize/down-select from the two codebooks. Since Rel-17 FeType-II PS codebook is primarily designed for scenarios in which partial UL-DL channel reciprocity is assumed, it is preferred to prioritize Rel-16 eType-II codebook as a baseline codebook for potential Rel-18 codebook, since it is not restricted by any UL-DL channel reciprocity condition. Therefore, we support prioritizing Rel-16 eType-II codebook as a baseline for potential Rel-18 codebook design

Only Rel-16 eType-II codebook is selected as a baseline for Rel-18 codebook

Moreover, it was agreed in RAN1#110bis-e [3] to support the following codebook structure for Rel-18 codebook:

- Alt2-A. Doppler-domain basis commonly selected for all SD/FD bases, e.g. W1W2(Wf⨂Wd)H, where Wd is an N4xQ matrix whose columns are selected from a size N4 DFT matrix.

For ease of exposition, our preference is to restructure the codebook to the following format:

W(l)=W1W2(l)Wz(l)H

- Where Wz(l)=Wf(l)⨂Wd(l). Here, the 1D DFT-based transformation matrix Wf(l) is replaced with a 2D DFT-based transformation matrix Wz(l) of size N3N4xMQ corresponding to a joint time/frequency domain transformation, where N4 corresponds to the number of time samples. This approach would simplify the description of the precoder, since the structure of Wz(l) would resemble that of the spatial-domain transformation matrix W1, which is also based on a sub-selection of columns of a 2D DFT matrix.

Delay and Doppler domain basis matrices are reported in the form of a 2D DFT matrix Wz(l), where Wz(l)=Wf(l)⨂Wd(l)

Also, from the codebook structure defined under Alt2-A, the size of W2(l) is 2LxMQ. One alternative is to report W2(l) in the form of Q concatenated matrices W2,1(l), …, W2,Q(l),  as follows:

W2(l)=W2,1(l)…W2,Q(l).

Under this format, the legacy definition and format of the non-zero coefficient matrices can be reused. We therefore have the following proposal:

The codebook coefficients are reported in the form of Q non-zero coefficient matrices W2,q(l), where q=1,…,Q

One other parameter that needs to be designed is the parameter d, corresponding to the duration (in slots) in which each of the N4 precoders is valid. In our opinion, the CSI prediction must be associated with a CSI-RS transmission occasion, and hence the parameter value d should take on a value that is at least equal to the CSI-RS transmission periodicity for periodic and semi-persistent CSI-RS resources, or at least equal to the value m corresponding to the spacing between two consecutive transmissions of aperiodic CSI-RS resources, i.e., d≥m.

The parameter value d is configured to be at least equal to the periodicity of a periodic or semi-persistent CSI-RS resource, or at least equal to the value m corresponding to the spacing (in slots) between two consecutive transmissions of aperiodic CSI-RS resources, i.e., d ≥ m

In RAN1#111, it was agreed to support Q=2 coefficient matrices W2 across the WCSI slots corresponding to the CSI duration, whereas the support of Q=3, 4 was left for further study. Our preference is to at least support Q=4 so as to accommodate larger CSI report durations WCSI and/or lower channel coherence times at high speed.

Support at least Q = 4 corresponding to reporting four coefficient matrices W2,q

## Coefficient quantization

For CSI reporting under Rel-18 Type-II codebook, Q coefficient matrices W2 need to be reported corresponding to the different PMI reporting instances. The first issue that needs to be discussed is whether the quantization of differential amplitude and differential phase coefficients need to follow legacy codebook design, or a new quantization approach is needed for Rel-18 Type-II codebook. In our assessment, there is no clear motivation to modify the quantization scheme corresponding to the differential amplitude and phase coefficients, and hence legacy amplitude and phase quantization codebooks can be reused.

Reuse Rel-16 eType-II legacy design for differential amplitude and differential phase coefficient codebook quantization values

One other aspect that needs to be discussed is the number of strongest coefficient indicator and reference amplitude values, as follows:

- Alt1. A single SCI value and one reference amplitude value, i.e., a common reference amplitude value for the same polarization across all Q of the W2 matrices

- Alt2. Q SCI values and Q reference amplitude values, i.e., separate SCI and reference amplitude value per W2 matrix

- Alt3. A single SCI value and 2Q-1 reference amplitude values, i.e., one reference amplitude value per W2 matrix per polarization

Both Alt2 and Alt3 have similar CSI feedback overhead, since the overhead of reporting the SCI and the reference amplitude are similar. On the other hand, Alt1 has less overhead since only one SCI and one reference amplitude value needs to be reported. Since the precoding vector is normalized for each transmission, Alt2 is more reasonable compared with Alt3. Hence, down selection between Alt1 and Alt2 is proposed, as follows

For SCI and reference amplitude reporting, down select between the following alternatives:

- Alt1. A single SCI value and one reference amplitude value are reported for the two polarizations with common values across all Q of the W2 matrices

- Alt2. Q SCI values and Q reference amplitude values, i.e., separate SCI and reference amplitude value per W2 matrix

## CQI reporting

- For UE-based CSI prediction, both UE and network need to have a common understanding of the correspondence between the CSI quantities and their corresponding time intervals. Given the codebook design agreed in RAN1#110bis-e with the dimensions of matrix Wd being N4xQ, the UE reports Q sets of W2 coefficients, which expand to generate N4 precoders, where N4≥Q. In our opinion, for high-speed CSI reporting, a degradation in channel quality is to some extent inevitable due to the inverse proportionality between the channel coherence time and the UE speed. Given that, conventional CQI reported for the measured channel may be mismatched with the quality of the channel at the time of PDSCH transmission using the codebook-based precoder, i.e., CQI value(s) corresponding to estimated precoder at UE at time t may not match the CQI value(s) corresponding to the estimated precoder during PDSCH transmission at time t + t0. Given that, a few alternatives should be considered for CQI reporting, as follows:

- Alt1-A: A single CQI value corresponding to the entire WCSI CSI report duration

- Alt1-B: A single CQI value corresponding to a reference slot within the WCSI slots

- Alt2: Two CQI values corresponding to two reference slots within the WCSI slots

  - FFS: whether the reference slot indices are network configured or UE selected

- Alt1-A can be considered as a baseline corresponding to legacy CSI reporting, whereas Alt1-B is a variation in which one CQI is reported, however a reference slot is indicated that enables the network to derive CQI values for the remainder WCSI-1 slots. For Alt2, two CQI values are reported corresponding to two reference slots, e.g., the first and last slots of the WCSI slots corresponding to the CSI report, and hence the network can use extrapolation to estimate the CQI value corresponding to other slots. Clearly, the alternatives above vary in terms of CQI accuracy and reporting overhead. One issue with Alt1-A is that the same CQI value is assumed for the entire CSI duration, which is not compatible with the moderate/high UE speed scenario that is targeted by the codebook design. While Alt1-A partially resolves this issue by indicating a reference slot corresponding to the reported CQI value, extrapolation of the actual channel quality corresponding to the remainder of slots would be challenging, especially under aperiodic CSI reporting. The issue of extrapolation is resolved under Alt2, thanks to reporting two CQI values with different reference slots to enable the extrapolation of the channel quality. One open question for Alt2 is whether the reference slots are network configured or UE reported. While UE-based reporting of the reference slot indices incurs additional CSI feedback overhead, it enables better extrapolation via selecting the two reference slots that improve the channel quality extrapolation accuracy. Additionally, more discussion is needed on whether reporting multiple subband CQI values is supported if a subband CQI format is configured. Further details are FFS

On the CSI measurement and reporting for Rel-18 Type-II codebook refinement for high/medium velocities, support Alt2 for CQI reporting with two CQI values corresponding to two reference slots of the CSI interval

- FFS: whether the two reference slot indices are network configured or UE selected

- FFS: Whether/How subband CQI reporting is supported

## NZC bitmap design

One important design aspect that needs discussion is the NZC bitmap design for Rel-18 codebook for high speed. Given that Q sets of W2 coefficients are fed back by the UE, three alternatives exist for the codebook design, as follows:

- Alt1. Q bitmaps of size 2LxM each are reported corresponding to the Q sets of W2 coefficients matrices

- Alt2. One bitmap of size 2LxM is reported that is common for all Q sets of W2 coefficients matrices

- Alt3. Two bitmaps, where a first bitmap of size MQ indicates the selected S pairs of FD basis vector and DD basis vector and a second bitmap of size 2LS for indicating the location of the non-zero coefficients across all Q sets of W2 coefficients matrices

Note that the bitmap overhead for Q>1 can be significantly large, e.g., 448 bits for Alt1 assuming L=4, M=7, Q=2 and RI=4, and hence bitmap overhead reductions are crucial to ensure a reasonable overhead. An example of the CSI feedback overhead corresponding to different combinations of L, M, Q and RI values for Rel-18 high speed codebook design are provided in Table 1.

| L | M | Q | RI | Overhead |
| --- | --- | --- | --- | --- |
| 2 | 4 | 2 | 2 | 64 |
| 4 | 4 | 2 | 4 | 256 |
| 4 | 7 | 2 | 4 | 448 |
| 6 | 4 | 2 | 3 | 288 |

Table 1. An example of NZC bitmap overhead corresponding to different combinations of L, M, Q and RI values for Rel-18 high speed codebook design

Given the alternatives of NZC bitmap reporting provided above, it is clear that for Q=1, both Alt1 and Alt2 are equivalent, whereas the overhead of Alt3 is larger than that of Alt1 at large values of S, i.e., most FD basis indices include non-zero coefficients. For Q>1, Alt3 can achieve significant gains due to overhead reduction, especially at smaller β values. While the bitmap reporting overhead under Alt2 bitmap design is smaller than that of Alt1 bitmap design, the overall CSI feedback overhead corresponding to Alt1 may be smaller than that of Alt2, since the per W2 matrix bitmap reduces the CSI feedback overhead corresponding to reporting the amplitude and phase values of the non-zero coefficients, whose CSI overhead saving can dominate the additional bitmap overhead. A comparison of the NZC bitmap reporting overhead corresponding to Alt1 and Alt3 for different values of S, assuming L=4, M=7, Q=2 and RI=4 for Rel-18 high speed codebook design is provided in Table 2, which illustrate that significant CSI feedback overhead savings that can be achieved when S<MQ.

| S | Alt1 | Alt3 | Overhead Reduction |
| --- | --- | --- | --- |
| 6 | 448 | 248 | 45% |
| 8 | 448 | 312 | 30% |
| 10 | 448 | 384 | 14% |
| 12 | 448 | 440 | 2% |
| 14 | 448 | 504 | -12% |

Table 2. An example of NZC bitmap reporting overhead in bits corresponding to Alt1 and Alt3 for different values of S, assuming L=4, M=7, Q=2 and RI=4 for Rel-18 high speed codebook design

Analytically, for a parameter combination of L=4, and Q=2, the NZC bitmap overhead corresponding to Alt3 would exceed that of Alt1 only if the total number of FD basis indices comprising non-zero coefficients exceeds 87% of the overall FD basis indices across the Q reporting occasions. Given that, our preference is to support Alt3 for NZC bitmap reporting.

For bitmap reporting under Rel-18 Type-II codebook for high speed, support Alt3 with two bitmaps, where a first bitmap of size MQ indicates the selected S pairs of FD basis vector and DD basis vector and a second bitmap of size 2LS for indicating the location of the non-zero coefficients across all Q sets of W2 coefficients matrices

# TDCP reporting

- In RAN1#111 [2], the following agreements were made for TDCP reporting

Agreement For the Rel-18 TRS-based TDCP reporting, down select only one of the following alternatives by RAN1#112: - AltA.1 (Doppler spread) as described in R1-2210523 - AltA.2 (Doppler shift): A UE is configured to report the Doppler shifts corresponding to the M strongest peaks of the wideband Doppler spectrum, for each of the N≥1 configured TRS resources - A TDCP report can be configured with N periodic TRS resources (e.g., N=2 with one TRS resource per TRP) - Parameter M is RRC configured with candidate values TBD, e.g. M=1,2,3,… - Wideband Doppler spectrum is calculated from the wideband time correlation function, given, as an example, by At,τ=ct,τct,0 , where ct,τ=n=0N-1hnt+τ∙hn*t and hn is the channel for subcarrier n. - AltB (TD correlation profile) as described in R1-2210523 Down-selection is to done based on, at least, the (single-)user throughput (LLS) performance comparison among the alternatives assuming: - Three special cases of an agreed use case (companies can select only one or more): aiding gNB to determine switching between Type-I and Rel-16 eType-II codebooks, or to determine SRS periodicity in the UL-SRS reciprocity-based precoding scheme; or aiding the gNB implementation in CSI prediction for TDD - In their simulations on switching between Type-I and Rel-16 eType-II codebooks, companies should state how to calculate the metric for the determination and how to set the threshold, and what the UE reports. - In their simulations on UL-SRS reciprocity-based precoding scheme, companies should state how to set the SRS periodicity based on the reported metrics, and what the UE reports; and the results should be displayed in terms of user throughput vs SRS overhead - In their simulations on CSI prediction for TDD, the results should be the correlation between real channel and predicted channel, and what the UE reports; aided by the reported metric. - Other scenarios of the agreed use cases can optionally be simulated - Based on the agreed EVM for sTRP and mTRP Note: Different alternatives may or may not apply to different use cases FFS: The need for a measure of confidence level in the TDCP report, and/or UE behaviour when the quality of TDCP measurement is not sufficiently high FFS: TDCP parameter(s) signalled with respect to each alternative

## PUSCH reporting time-domain behavior

In RAN1#110 [4], it was agreed to support standalone TDCP reporting based on TRS signaling with aperiodic reporting, and furthermore, it was concluded in RAN1#110bis-e [3] that no consensus was reached to support periodic, semi-persistent, or event-triggered/UE-initiated TDCP reporting. In our opinion, aperiodic TDCP reporting should follow the same behavior as conventional aperiodic CSI reporting, in terms of being reported only over PUSCH. Aperiodic TDCP reporting over PUCCH should not be supported.

Aperiodic TDCP reporting is supported only over PUSCH

## TDCP report fields

In RAN1#111, it was agreed to down-select from the following options corresponding to the fields reported within a TDCP report, as shown in Table 3.

| TDCP report | What to report | How to calculate: examples, possible UE implementation for evaluation |
| --- | --- | --- |
| A1. Doppler spread | One Doppler spread value, i.e. fd,max-fd,min. (see column 3 of A1) | The normalized channel correlation for each delay τ can be calculated as follows ri,jτ=hiτhj*τhiτhjτ where hi(τ) indicates the time domain estimated channel at delay τ in symbol i within a TRS burst. Further averaging for multiple TRS busts is necessary. The UE can optionally perform interference/noise reduction to get ri,jτ. The Doppler for each delay can be calculated as ri,jτ=J0(2πfd(τ)(j-i)γ) Where γ is the time duration for an OFDM symbol. To calculate fd(τ), the UE can search candidate fd to minimize the error of e(k)(τ)=(i,j)ϵSri,jτ-J0(2πfd(k)(j-i)γ)2 fdτ=argmin(e(k)(τ)) UE does not need to calculate fd(τ) for the each tap, but it only needs to calculate it for top N tops. From multiple fd(τ), the UE can calculate the following: fd,max=max{fdτ1,…,fd(τN)} fd,min=min{fdτ1,…,fd(τN)} |
| A2. Doppler shift | With M identified peaks in measured CIR: (1) N Doppler shifts; (2) M values of delay shift in CIR | A UE is configured to report the Doppler shifts corresponding to the M strongest peaks of the wideband Doppler spectrum, for each of the N≥1 configured TRS resources - A TDCP report can be configured with N periodic TRS resources (e.g., N=2 with one TRS resource per TRP) - Parameter M is RRC configured with candidate values TBD, e.g. M=1,2,3,… - Wideband Doppler spectrum is calculated from the wideband time correlation function, given, as an example, by At,τ=ct,τct,0, where ct,τ=n=0N-1hnt+τ∙hn*t and hn is the channel for subcarrier n. |
| B. Time-domain correlation profile | Amplitude At,τ vs. delay value , e.g., non-zero quantized version of amplitude At,τ for a number of delay values τ (quantized amplitude vs delay) Example equation At,τ=ct,τct,0 where ct,τ=n=0N-1hnt+τ∙hn*t and hn is the channel for subcarrier n. | - Normalized auto-correlation of a time series measured from a TRS resource. - Multiple auto-correlation values can be calculated from different lags of the same resource or different resources - The autocorrelation can be estimated by replacing the channel hn for subcarrier n in the defining formula in column 2, with the matched filter subcarrier components Xn=Rn∙Sn* of the received signal Rn where Sn* is the complex conjugate of the known transmitted TRS signal. For ct,0 one can use the arithmetic average over the two TRS symbols separated by the time τ , i.e. At,τ≈n=0N-1Xnt+τ∙Xn*t12∙n=0N-1Xnt∙Xn*t+Xnt+τ∙Xn*t+τ Or, alternatively, one may use the geometric average for ct,0, i.e. At,τ≈n=0N-1Xnt+τ∙Xn*tnXnt+τ2nXnt2 Further methods to remove noise bias and to suppress noise can be used. |

Table 3. Different Alternatives for TDCP reporting with respect to measurement and reported parameters

In the following, we provide our views on each of the alternatives above, as follows.

- Alt-A: In general, one issue with all Alt-A approaches based on Doppler shift reporting is whether/how the Doppler shift can be differentiated from CFO, since they both cause a frequency shift with the same order of values. Even if the process of inferring the Doppler shift from the aggregate frequency shift can be handled in a spec-transparent manner based on UE implementation, it is important that the proponents of Alt-A explain the process so we can assess the feasibility and efficiency of Alt-A before. In the sequel, we provide further views on each of the five variants of Alt-A, as follows

- Alt-A1: In our opinion, Alt-A1 is a variant of Alt-B, since the channel autocorrelation is computed as an intermediate step before computing the Doppler spread. Moreover, Alt-A1 would require significant specification impact, and hence is not preferred.

- Alt-A2: This alternative resembles Alt-A1, with the difference stemming from the relative Doppler shift in Alt-A2 being matched with the SRS differential delay shifts, rather than matching with Bessel’s function of the first type. One issue with Alt-A2 is that it presumes the power-delay profile (PDP) computed based on the SRS symbols is similar to the PDP corresponding to the TRS symbols. If this assumption holds in reality, the TDCP can be inferred from SRS signaling without the need of TDCP reporting.

- Alt-B: One advantage of Alt-B is that it is the most straightforward approach, since the channel autocorrelation needs to be computed as an intermediate step for most other Alt-A variants. Two autocorrelation functions are proposed for Alt-B, based on either arithmetic or geometric scaling of the channel correlation. One example of TDCP reporting under Alt-B is via introducing a codebook of quantized amplitude values corresponding to the autocorrelation function At,τ, however, for a meaningful mapping between the quantized amplitude values and the autocorrelation value, the autocorrelation function needs to be specified. For a small codebook of values, e.g., with two values corresponding to either a large or small autocorrelation value, specifying the channel autocorrelation function is not needed. Note that instead of reporting the quantized correlation amplitude for a fixed lag, alternatively the lag is reported (in terms of a symbol index or TRS occasion index) with respect to a fixed correlation amplitude. The fixed correlation amplitudes can be configured from a small set of values, e.g., two values corresponding to strong, weak correlation, so that the process is less dependent on the underlying autocorrelation function as much as possible. In light of that, we support Alt-B, with the TDCP report indicating a correlation lag value corresponding to a large channel autocorrelation value, i.e., channel coherence time, in terms of an index of a TRS symbol at high speed, or an index of a TRS transmission occasion over periodic TRS transmissions at low/moderate speed. Hence, TDCP reporting can include indices of TRS symbols/TRS occasions with respect to a reference TRS/TRS symbol corresponding to a period of strong correlation, rather than reporting exact time-domain correlation values and/or Doppler-profile parameters, e.g., Doppler spread. Given that reporting TRS/TRS symbol indices corresponds to time-domain reporting, we prefer Alt-B over Alt-A.

Alt-B is preferred over Alt-A variants, since the channel correlation in time can be measured in terms of TRS/TRS symbol indices corresponding to a period of strong channel correlation with respect to a reference TRS/TRS symbol, compared with reporting absolute Doppler-domain parameters for all TRSs/TRS symbols

- Given that, our preference is to support Alt-B for TDCP reporting of channel correlation in terms of indices of TRS/TRS symbols with respect to a reference TRS/TRS symbol.

For TDCP reporting format, support Alt-B with channel correlation reported in terms of TRS/TRS symbol indices with respect to a reference TRS/TRS symbol corresponding to a channel coherence interval

# Conclusion

This contribution addressed CSI enhancements for NR Rel. 18, including enhancements for high mobility scenarios, TDCP reporting enhancements, as well as CSI enhancements for CJT.

- For CSI enhancements for CJT, we have the following proposals:

- Further study whether/how the number of non-zero coefficients is reported in the CSI report, e.g., separately for each TRP or jointly for all TRPs

For the number-of-beam combination reporting, consider the following alternatives for down selection:

- Alt1. Permutations of a configured combination of the number of beams are supported as additional candidates of number-of-beam combination

- Alt2. A constraint on the aggregate number of beams is implied from the configured number-of-beam combinations

Support Alt2 for CJT FD basis selection with independent FD basis selection per TRP for mode-1 of CJT codebook

Confirm the working assumption Alt3 for W2 quantization group and strongest coefficient indicator design

Reuse Rel-16 eType-II reference amplitude quantization for differential quantization of the strongest coefficients for each polarization per TRP

- FFS: whether the number of reference amplitude quantization bits can be reduced

For CJT codebook design with N >2, support a two-stage bitmap design, where a first stage bitmap for each CSI-RS resource indicates the indices of rows and/or columns with at least one non-zero coefficient, and a second stage bitmap whose size is based on the indicated rows and/or columns in the first stage bitmap per CSI-RS resource

- Study the mapping order of CSI fields corresponding to CJT-based CSI reporting

- Study whether/how to define a new priority level with finer omission granularity defined per TRP

- Study whether/how to define a new bit priority formula including TRP index for joint TRP omission

For CSI enhancements for high mobility, we have the following proposals:

- Only Rel-16 eType-II codebook is selected as a baseline for Rel-18 codebook

- Delay and Doppler domain basis matrices are reported in the form of a 2D DFT matrix Wz(l), where Wz(l)=Wf(l)⨂Wd(l)

- The codebook coefficients are reported in the form of Q non-zero coefficient matrices W2,q(l), where q=1,…,Q

- The parameter value d is configured to be at least equal to the periodicity of a periodic or semi-persistent CSI-RS resource, or at least equal to the value m corresponding to the spacing (in slots) between two consecutive transmissions of aperiodic CSI-RS resources, i.e., d ≥ m

- Support at least Q = 4 corresponding to reporting four coefficient matrices W2,q

- Reuse Rel-16 eType-II legacy design for differential amplitude and differential phase coefficient codebook quantization values

- For SCI and reference amplitude reporting, down select between the following alternatives:

- Alt1. A single SCI value and one reference amplitude value are reported for the two polarizations with common values across all Q of the W2 matrices

- Alt2. Q SCI values and Q reference amplitude values, i.e., separate SCI and reference amplitude value per W2 matrix

- On the CSI measurement and reporting for Rel-18 Type-II codebook refinement for high/medium velocities, support Alt2 for CQI reporting with two CQI values corresponding to two reference slots of the CSI interval

- FFS: whether the two reference slot indices are network configured or UE selected

- FFS: Whether/How subband CQI reporting is supported

- For bitmap reporting under Rel-18 Type-II codebook for high speed, support Alt3 with two bitmaps, where a first bitmap of size MQ indicates the selected S pairs of FD basis vector and DD basis vector and a second bitmap of size 2LS for indicating the location of the non-zero coefficients across all Q sets of W2 coefficients matrices

- For TDCP reporting enhancements, we have the following proposals:

- Aperiodic TDCP reporting is supported only over PUSCH

- Alt-B is preferred over Alt-A variants, since the channel correlation in time can be measured in terms of TRS/TRS symbol indices corresponding to a period of strong channel correlation with respect to a reference TRS/TRS symbol, compared with reporting absolute Doppler-domain parameters for all TRSs/TRS symbols

- For TDCP reporting format, support Alt-B with channel correlation reported in terms of TRS/TRS symbol indices with respect to a reference TRS/TRS symbol corresponding to a channel coherence interval

# References

| [1] | RP-213598, Samsung, "New WID: MIMO Evlution for Downlink and Uplink," Online, Dec. 6-17, 2021. |
| --- | --- |
| [2] | 3GPP TSG RAN WG1 #111, "Draft Meeting Report," Nov. 14-18, 2022. |
| [3] | 3GPP TSG RAN WG1 #110bis-e, "Draft Meeting Report," Online, Oct. 10-19, 2022. |
| [4] | 3GPP TSG RAN WG1 #110, "Draft Meeting Report," Toulouse, France, Aug. 22-26, 2022. |
| [5] | 3GPP TS 38.214, "Physical layer procedures for data," Dec. 2022. |
| [6] | 3GPP TSG RAN WG1 #109-e, "Draft Meeting Report," Online, May 9-20, 2022. |