3GPP TSG RAN WG1#112                                          	                    R1- 2300515

Athens, Greece, Feb. 27th –Mar. 3rd, 2023

Agenda Item:	9.1.4.1

Source:	Lenovo

Title:	UL precoding indication for multi-panel transmission

Document for:	Discussion

Introduction

One of Rel-18 MIMO evolution WID scops is to support simultaneous multi-panel UL transmission as highlighted in the approved WID[1]:

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In RAN1#109e, the following agreements were reached[3]:

Agreement For single-DCI based STxMP PUSCH SDM scheme, support the layer combinations of {1+1, 1+2, 2+1 and 2+2}

In RAN1#110bis, the following agreements were further reached [3]:

Agreement - Reuse the DCI field ‘Antenna Ports’ in DCI format 0_1 and 0_2 to indicate DMRS ports for SDM scheme of single-DCI based STxMP PUSCH: - The total numbers of layers, L, indicated by two TPMI fields of CB PUSCH or two SRI fields of NCB PUSCH is used to determine the DMRS port indication table. - L1 of the indicated DMRS ports are associated with the L1 PUSCH layers which are indicated by the first TPMI field for CB PUSCH or the first SRI field for NCB PUSCH and the rest L- L1 of the indicated DMRS ports are associated with the L-L1 PUSCH layers which are indicated by the second TPMI field for CB PUSCH or the second SRI field for the NCB PUSCH. - FFS: how to partition the indicated DMRS ports among the PUSCH layers. Agreement Support STxMP PUSCH+PUSCH transmission in multi-DCI based system in Rel-18. - Two independent PUSCHs associated with different TRPs can be transmitted by a UE simultaneously in same active BWP. - The total number of layers of these two PUSCHs is up to 4. - FFS: whether the number of layers of each of these two PUSCHs is up to 2. Agreement For SDM scheme of single-DCI based STxMP PUSCH - Configure two SRS resource sets for CB or NCB . - FFS : These two SRS resource sets can have different number of SRS resources for codebook -based or non-codebook based. - For codebook -based PUSCH , DCI indicates two TPMI fields, and each TPMI field separately indicates the precoding information and the number of layers conveyed over the SRS ports of the indicated SRS resource in each SRS resource set. - For non-codebook based PUSCH and codebook -based PUSCH , DCI indicates two SRI fields and each field indicates SRS resource(s) for each SRS resource set separately. - FFS : For codebook -based PUSCH , the two SRS resources indicated by the two SRI fields can have different number of SRS ports

The following agreements were reached in RAN1#110bis

Agreement Regarding the TPMI/SRI indication for multi-DCI based STxMP PUSCH+PUSCH: - Configure two SRS resource sets for CB or NCB. - FFS: Whether/how to associate coresetPoolIndex with SRS resource set implicitly or explicitly. - FFS: the maximal number of configured/indicated SRS resources in each set for NCB/CB - FFS: the maximal number of SRS ports in each set for CB. - FFS: Separate codebooks and separate maxRanks are configured for different SRS resource sets. - For type 1 CG-PUSCH (if supported), FFS how to associate the PUSCH with one TRP - e.g., configure a coresetPoolIndex value in a type 1 CG-PUSCH - e.g., use a single CG to configure two type 1 CG PUSCHs for STxMP PUSCH+PUSCH Agreement Support dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission - FFS the indication of dynamic switching - FFS: max number of layers when switching to sTRP transmission Agreement Support SFN-based transmission scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18. Agreement Support to configure up to 2 PTRS ports for SDM scheme of single-DCI based STxMP PUSCH transmission: - For 2 PTRS ports, study how to use the ‘PTRS-DMRS association’ field in DCI format 0_1 and 0_2 to indicate the PTRS-DMRS association for SDM scheme. Agreement For the switching between SDM scheme of single-DCI based STxMP PUSCH and Rel-17 mTRP PUSCH TDM scheme, Alt2 is supported. FFS whether Alt1 is supported in addition to Alt2. - Alt1: Support dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and Rel-17 mTRP PUSCH TDM scheme - FFS: how to support dynamic switching, e.g., using the indicated PUSCH repetition number - Note: It is up to gNB implementation to configure SDM scheme of single-DCI based STxMP PUSCH or Rel-17 mTRP PUSCH TDM scheme or both of them in RRC. Dynamic switching between them is only when both schemes are configured in RRC. - Alt2: Support RRC-based switching between SDM scheme of single-DCI based STxMP PUSCH and Rel-17 mTRP PUSCH TDM scheme Agreement The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are fully/partially overlapped in time domain (Depending on RAN4’s input on Pcmax requirements). - Note: No symbol-level power adjustment within a PUSCH transmission occasion in the case of fully/partially overlapping in time domain Agreement Multi-DCI based STxMP PUSCH+PUSCH transmission at least supports the following PUSCH combinations: - DG-PUSCH + DG-PUSCH - CG-PUSCH + DG-PUSCH

The following agreements were reached in RAN1#111

Agreement For the DMRS port indication for SDM scheme single-DCI based STxMP transmission, support Alt2: - Alt2: the DMRS ports associated with two TPMI/SRI fields can be in same or different CDM groups. Agreement Support CG PUSCH + CG PUSCH in multi-DCI based STxMP PUSCH+PUSCH transmission. Agreement Support the SFN scheme for single-DCI based STxMP PUCCH transmission. Conclusion There is no consensus on the support STxMP PUCCH+PUCCH in multi-DCI based mTRP system. R1-2212580 Summary #2 on Rel-18 STxMP Moderator (OPPO) From Nov 16th session Agreement - For multi-DCI based STxMP, to schedule a PUSCH for STxMP PUSCH+PUSCH transmission, - Alt1: The first SRS resource set is associated with coresetPoolIndex value 0 and the other SRS resource set is associated with coresetPoolIndex value 1 - The PUSCH is associated with SRS resource set with the same value of coresetPoolIndex - FFS: Which is the first SRS resource set, e.g., the set with lower set ID. - Regarding how to interpret the SRI/TPMI field in DCI: - For DG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where scheduling DCI format 0_1 or 0_2 is received - For Type 2 CG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where activation DCI is received. - For Type 1 CG-PUSCH, one SRS_resource_set_index value is configured in RRC in ConfiguredGrantConfig and the srs-ResourceIndicator/precodingAndNumberOfLayers correspond to the SRS resource set Agreement - Support DCI-based dynamic switching between SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission - The DCI field “SRS resource set indicator” is used to indicate the switching between SFN scheme and sTRP transmission. - (Conclusion) There is no consensus to support DCI-based dynamic switching between SFN scheme and SDM scheme R1-2212581 Summary #3 on Rel-18 STxMP Moderator (OPPO) From Nov 17th session Agreement For the SFN scheme of single-DCI based STxMP PUSCH: - Configure two SRS resource sets for CB or NCB. - FFS: Number of SRS resources of SRS resource set, and number of SRS ports of SRS resource - The DCI indicates two SRI fields and TPMI fields for SFN transmission, - On the indication of number of layers for CB and NCB PUSCH: - Alt1: Similar to rel-17 mTRP TDM scheme, the number of layers is indicated by the first SRI field (for NCB PUSCH) or the first TPMI field (for CB PUSCH); Agreement For dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission: - Use the 2-bit “SRS resource set indicator” DCI field to dynamically indicate the sTRP or SDM transmission. - FFS: how to interpret each codepoint of “SRS resource set indicator”. - For the maximal number of layers for sTRP transmission, down-select: - Option1: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec - Option2: Configuring one additional maximal numbers of layers for sTRP transmission, in addition to maxRank in current spec. - Down-select one from the following for maximal number of layers of SDM transmission: - Alt1: Configure one single maximal number of layers (separate from the maximal number(s) of layers for sTRP), that is applied to the first SRS resource set and the second SRS resource set, separately. - Alt1a: The maxRank (or Lmax) in current spec is also applied to the first SRS resource set and the second SRS resource set, separately. - Alt2: Configure separate maximal numbers of layers for the first SRS resource set and the second SRS resource set - Alt3: no dedicated configuration for SDM. The maximal number(s) of layers of sTRP and the UE capability reporting for SDM are used to determine the maximal number of layers of SDM transmission. - Alt4: The maximal number(s) of layers in above Option1/2 also applied to the first SRS resource set and the second SRS resource set, separately. - FFS: To ensure the same size of DCI for sTRP and SDM cases, how to use/interpret the TPMI/SRI field(s) and whether to do reserved bit or zero padding. Agreement For SDM scheme single-DCI based STxMP transmission, when L1 and L2 layers are indicated/determined by two TPMI fields of CB PUSCH or two SRI fields of NCB PUSCH respectively: - The first L1 indicated DMRS ports correspond to the L1 layers indicated by the first TPMI or SRI field - The remaining L2 indicated DMRS ports correspond to the L2 layers indicated by the second TMPI or SRI field - Support at least one of the following options for indication of layer combination {1+2}: - Option 1: new entry is added to DMRS table, e.g., {0, 2, 3}, {2, 0, 1}. - Option 2: use DCI field (e.g., SRS resource set indicator) to indicate that for layer combination {1+2}, the first two indicated DMRS ports correspond to the 2 layers indicated by the second TPMI or SRI field and the rest one indicated DMRS port correspond to the layer indicated by the first TPMI or SRI field. - Option 3: For layer combination of {1+2}, the DMRS port in the CDM group with only one port is mapped to the SRI/TPMI field indicating one layer, and the DMRS ports in the CDM group with 2 ports are mapped to the SRI/TPMI field indicating 2 layers - Other options are not precluded Conclusion There is no consensus to support layer combinations {1+3} and {3+1} in SDM scheme of single-DCI based STxMP PUSCH.

In this contribution, we provide our views on the UL precoding indication mechanism for multi-panel transmission.

Discussion

Single-DCI based multi-panel UL transmission

Figure 1 Illustration of multi-panel PUSCH mode 1

An STxMP mode 1 is illustrated in Figure 1, where two panels are equipped for the UE for simultaneous UL transmission and each panel is assumed to have four antenna ports, i.e., antenna port 1000, 1001, 1002, 1003 of a panel. A single-DCI based multi-panel/TRP PUSCH scheme is provided in Figure 1, a PUSCH with 3 layers is scheduled to be transmitted by both panel#0 and panel#1, where the first and the second PUSCH layers are transmitted by PUSCH antenna port 1000, 1001, 1002, 1003 of panel#0 to TRP#0 according to the first TCI state, and the third PUSCH layer is transmitted by PUSCH antenna port 1000, 1001, 1002, 1003 of panel#1 to TRP#1 according to the second TCI state. To better support codebook based PUSCH transmission, two panels should have the same number of antenna ports and the same coherent capability. This requirement also applies to multi-DCI based multi-panel/TRP PUSCH transmission.

The same number of antenna ports and the coherent capability for each panel is assumed for both single-DCI and multi-DCI simultaneous multi-panel UL transmission.

As stated in WID, Rel-18 MIMO enhancement should be discussed based on unified TCI framework. One issue is how to support PUSCH transmission scheduled by DCI format 0_0 for the UE supporting simultaneous multi-panel UL transmission. For the PUSCH scheduled by DCI format 0_0 in the scenario illustrated in Figure 1, when only one UL TCI state is indicated, the PUSCH scheduled by DCI format 0_0 is transmitted by single port by applying the UL TCI state, the panel used for the PUSCH transmission is based on the applied UL TCI state. For example, if the indicated UL TCI state is associated with panel#1, the UE shall transmit the PUSCH scheduled by DCI format 0_0 based single antenna port of panel#1.

When two UL TCI states are indicated, the PUSCH scheduled by DCI format 0_0 is transmitted based on single antenna port with the first UL TCI state as default.

In S-DCI MTRP, PUSCH scheduled by DCI format 0_0 is transmitted based on single antenna port using the first indicated UL TCI state when two UL TCI states are indicated.

SRS resource and SES resource set indication

It has been agreed to configured two SRS resource sets for SDCI SDM PUSCH transmission and 2-bits “SRS resource set indicator” DCI field to dynamically indicate sTRP or SDM transmission. First, we proposed to have a fix association between the two SRS resource sets and the two indicated SRI/TPMI field as that for MDCI M-TRP scenario. When two TPMI/SRI fields are indicated by the scheduling DCI, the first SRS resource set is associated with the first TPMI/SRI field and the second SRS resource set is associated with the second TPMI/SRI field. To support the dynamic switching between sTRP and SDM PUSCH transmission, the following “SRS resource set indicator” field codepoint interpretation can be considered:

- ’00’: sTRP PUSCH is scheduled with the first SRI/TPMI field and the first SRS resource set

    - The second SRI/TPMI field is ignored by the UE

- ’01’: sTRP PUSCH is scheduled with the second SRI/TPMI field and the second SRS resource set

    - The first SRI/TPMI field is ignored by the UE

- ’10’: SDM PUSCH is scheduled with both SRI/TPMI fields and both SRS resource sets

- ’11’ : reserved

When a UE is configured with two SRS resource sets in SDCI MTRP mode and the UL grant is configured to contain two TPMI/SRI fields, the first SRS resource set is associated with the first TPMI/SRI field and the second SRS resource set is associated with the second TPMI/SRI field.

Considering the following SRS resource set indicator field interpretation to support dynamic sTRP and SDM transmission

- ’00’: sTRP PUSCH is scheduled with the first SRI/TPMI field and the first SRS resource set

  - The second SRI/TPMI field is ignored by the UE

- ’01’: sTRP PUSCH is scheduled with the second SRI/TPMI field and the second SRS resource set

  - The first SRI/TPMI field is ignored by the UE

- ’10’: SDM PUSCH is scheduled with both SRI/TPMI fields and both SRS resource sets

- ’11’ : reserved

Another open issue is how to configure the maximum number of layers for sTRP transmission as well as for SDM transmission. Several options were provided for different mode as follows:

- For the maximal number of layers for sTRP transmission, down-select:

  - Option1: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec

  - Option2:  Configuring one additional maximal numbers of layers for sTRP transmission, in addition to maxRank in current spec.

- Down-select one from the following for maximal number of layers of SDM transmission:

  - Alt1: Configure one single maximal number of layers (separate from the maximal number(s) of layers for sTRP), that is applied to the first SRS resource set and the second SRS resource set, separately.

  - Alt1a: The maxRank (or Lmax) in current spec is also applied to the first SRS resource set and the second SRS resource set, separately.

  - Alt2: Configure separate maximal numbers of layers for the first SRS resource set and the second SRS resource set

  - Alt3: no dedicated configuration for SDM. The maximal number(s) of layers of sTRP and the UE capability reporting for SDM are used to determine the maximal number of layers of SDM transmission.

  - Alt4: The maximal number(s) of layers in above Option1/2 also applied to the first SRS resource set and the second SRS resource set, separately.

As discussed before, we prefer a symmetric design for the STxMP PUSCH, e.g., the panels used for STxMP PUSCH transmission should have the same number of antenna ports. Therefore, for sTRP transmission, we prefer option 1, i.e., maxRank (or Lmax) in current spec is used to configure the maximum number of layers of sTRP transmission. For SDM based STxMP PUSCH transmission, we prefer alt3, i.e., The maximal number(s) of layers of sTRP and the UE capability reporting for SDM are used to determine the maximal number of layers of SDM transmission.

For the maximal number of layers for sTRP transmission, the maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec.

For maximal number of layers of SDM transmission, the maximal number(s) of layers of sTRP and the UE capability reporting for SDM are used to determine the maximal number of layers of SDM transmission.

Codewords to layer mapping for SDM based STxMP PUSCH

Another issue is the codeword-to-layer mapping rule for this scenario. In Rel-15/16/17, only one codeword is supported, since each PUSCH transmission occasion is transmitted by a panel to a TRP. However, at least for SDM based multi-panel PUSCH transmission as illustrated in Figure 1, different PUSCH layers are transmitted by different panels to different TRPs. Given that the WID restricts the total number of PUSCH layers transmitted from both panels is no more than 4, two alternatives exist for the codeword-to-layer mapping, as follows:

Alt-1. A single codeword is mapped to both panels.

This alternative is aligned with Rel. 15 PUSCH transmission chain with up to 4 layers mapped to the same codeword. One issue that needs further discussion under this alternative is whether/how restrictions are imposed on the number of layer pairs transmitted from both panels.

Alt-2. Two codewords are mapped separately to two panels.

One other alternative is map the PUSCH layers from each panel to a same codeword, where each codeword has independent MCS conformed to the respective panel-to-gNB channel condition for better performance. Similarly, whether/how restrictions are imposed on the allowed rank pair values across both codewords should be discussed.

Figure 3 illustrates the performance comparation between two alternatives. In the simulation, each panel transmits two layers. For single CW scheme, all the 4 layers transmitted by both panels are from a same CW with one MCS scheme, while independent TPMI are indicated for each panel. For dual-CW scheme, the two layers from each panel are from independent CWs with independent MCS and TPMI. The detail simulation assumptions are provided in appendix. It can be observed that single-CW scheme is marginally better than dual-CW scheme in lower SINR region because single CW can provide larger channel coding gain with lower coding rate. However, dual-CW scheme outperforms single CW scheme in media and higher SINR region since the scheduled MCS corresponding to the CW for each panel are well matched with the independent channel conditions between each panel and the TRP link.

Figure 3 Throughput of STxMP PUSCH transmission with different CW-to-Layer mapping scheme

Support two CW for STxMP PUSCH transmission.

DMRS for SDM STxMP PUSCH

It was agreed to use a single antenna port(s) field to indicate the DMRS ports for SDM based PUSCH scheduling with two TPMI/SRI field. The first L1 indicated DMRS ports correspond to the L1 layers indicated by the first TPMI or SRI field, and the remaining L2 indicated DMRS ports correspond to the L2 layers indicated by the second TMPI or SRI field. Some options were provided in RAN1#111 to support the layer combination {1+2} as follows:

  - Option 1: new entry is added to DMRS table, e.g., {0, 2, 3}, {2, 0, 1}.

  - Option 2: use DCI field (e.g., SRS resource set indicator) to indicate that for layer combination {1+2}, the first two indicated DMRS ports correspond to the 2 layers indicated by the second TPMI or SRI field and the rest one indicated DMRS port correspond to the layer indicated by the first TPMI or SRI field.

  - Option 3: For layer combination of {1+2}, the DMRS port in the CDM group with only one port is mapped to the SRI/TPMI field indicating one layer, and the DMRS ports in the CDM group with 2 ports are mapped to the SRI/TPMI field indicating 2 layers

We understand the motivation of all those is to ensure the DMRS port for the layers corresponding to a same TRP are from a same CDM group. For example, the 1st DMRS port is within a CDM group while the last two DMRS ports are within another CDM group. However, we have agreed another agreement “the DMRS ports associated with two TPMI/SRI fields can be in same or different CDM groups”, it means that {0, 1, 2} can be used for both {1+2} and {2+1} by considering Rel-15 DMRS principle. Based on this analysis, we think all the options are not needed.

Do not introduce additional options to support layer combination {1+2}.

PTRS for SDM STxMP PUSCH

Up to 2 PTRS ports for SDM scheme of single-DCI based STxMP PUSCH can be configured according to RAN1#110bis agreement. It means that when SDM STxMP PUSCH is scheduled, all the PUSCH layers corresponding to a panel share a same PTRS ports. Further the one of the DMRS ports associated with each panel should be associated with one of the PTRS ports. For example, PTRS port 0 is associated with one DMRS port for the first panel and PTRS port 1 is associated with one DMRS port for the second panel. Considering the supported rank combination, up to 2 layers can be scheduled for each panel, then 1 bit is sufficient to indicate the DMRS port associated with each PT-RS port. Thus, 2bits PTRS-DMRS association’ field in DCI format 0_1 and 0_2  is used to indicate the DMRS port associated with each PT-RS port, where the MSB indicates the DMRS port associated with PT-RS port 0 and the LSB indicates the DMRS port associated with PT-RS port 1.

When SDM based STxMP PUSCH is scheduled with two PTRS ports, 2bits 'PTRS-DMRS association’ field in scheduling DCI is used to indicate the DMRS port associated with each PTRS port, where the MSB indicates the DMRS port associated with PT-RS port 0 and the LSB indicates the DMRS port associated with PTRS port 1.

For non-codebook based PUSCH in Rel-15, PTRS port index is configured for each SRS resource for nCB. When 2 PTRS ports and 2 SRS resource sets for nCB are configured, PTRS port 0 can be configured for all SRS resources within one SRS resource set, while PTRS port 1 can be configured for all SRS resources within the other SRS resource set. However, when dynamic switch between SDM of S-DCI based STxMP PUSCH and sTRP is supported, it’s better to configure 2 PTRS ports for each of two SRS resource sets for partial and non-coherent UE. With such configuration, when SDM based S-DCI STxMP PUSCH is scheduled, the associated PTRS port for the indicated SRS resources of each SRS resource set should be same and the different PTRS ports are expected to be associated to  the indicated SRS resources from different SRS resource sets. When sTRP PUSCH is scheduled, one or two PTRS ports can still be scheduled according to Rel-15. Based on the analysis above, we propose that:

Introduce a restriction on the indicated SRI(s) for non-codebook based SDM STxMP PUSCH transmission such that the associated PTRS port of all indicated SRS resources for each SRS resource set is the same and the associated PTRS ports of any two indicated SRS resources from two SRS resource sets are different.

Multi-DCI based multi-panel UL transmission

For multi-DCI based STxMP, it was agreed that the first SRS resource set is associated with coresetPoolIndex value 0 and the other SRS resource set is associated with coresetPoolIndex value 1. On the determination of the first and the second SRS resource set, we think the same mechanism used in Rel-17 MTRP PUSCH can be reused, i.e., the first and the second SRS resource set corresponding to the SRS resource with lower and larger SRS resource set ID, respectively.

The first and the second SRS resource sets correspond to the SRS resources with the lower and the larger SRS resource set ID, respectively

In Rel-17, only one PUSCH or one PUCCH can be transmitted in a time instance, as a result, only two non-overlapped PUCCH resources can be transmitted in a slot for UCI transmission. For example, when separate ACK feedback is configured for multi-DCI MTRP, the PUCCH resources carry the HARQ-ACK codebook for different TRPs should be transmitted with TDM manner. With STxMP, two overlapped PUSCHs can be transmitted in a time instance. New UCI multiplexing rule is required for high efficient UCI multiplexing considering STxMP PUSCH transmission.

Study the UCI multiplexing rule in STxMP PUSCH and/or PUCCH transmission.

Conclusion

In this contribution, we provide the following proposals:

Proposal 1	The same number of antenna ports and the coherent capability for each panel is assumed for both single-DCI and multi-DCI simultaneous multi-panel UL transmission.

Proposal 2	In S-DCI MTRP, PUSCH scheduled by DCI format 0_0 is transmitted based on single antenna port using the first indicated UL TCI state when two UL TCI states are indicated.

Proposal 3	When a UE is configured with two SRS resource sets in SDCI MTRP mode and the UL grant is configured to contain two TPMI/SRI fields, the first SRS resource set is associated with the first TPMI/SRI field and the second SRS resource set is associated with the second TPMI/SRI field.

Proposal 4	Considering the following SRS resource set indicator field interpretation to support dynamic sTRP and SDM transmission

- ’00’: sTRP PUSCH is scheduled with the first SRI/TPMI field and the first SRS resource set

  - The second SRI/TPMI field is ignored by the UE

- ’01’: sTRP PUSCH is scheduled with the second SRI/TPMI field and the second SRS resource set

  - The first SRI/TPMI field is ignored by the UE

- ’10’: SDM PUSCH is scheduled with both SRI/TPMI fields and both SRS resource sets

- ’11’ : reserved

Proposal 5	For the maximal number of layers for sTRP transmission, the maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec.

Proposal 6	For maximal number of layers of SDM transmission, the maximal number(s) of layers of sTRP and the UE capability reporting for SDM are used to determine the maximal number of layers of SDM transmission.

Proposal 7	Support two CW for STxMP PUSCH transmission.

Proposal 8	Do not introduce additional options to support layer combination {1+2}.

Proposal 9	When SDM based STxMP PUSCH is scheduled with two PTRS ports, 2bits 'PTRS-DMRS association’ field in scheduling DCI is used to indicate the DMRS port associated with each PTRS port, where the MSB indicates the DMRS port associated with PT-RS port 0 and the LSB indicates the DMRS port associated with PTRS port 1.

Proposal 10	Introduce a restriction on the indicated SRI(s) for non-codebook based SDM STxMP PUSCH transmission such that the associated PTRS port of all indicated SRS resources for each SRS resource set is the same and the associated PTRS ports of any two indicated SRS resources from two SRS resource sets are different.

Proposal 11	The first and the second SRS resource sets correspond to the SRS resources with the lower and the larger SRS resource set ID, respectively

Proposal 12	Study the UCI multiplexing rule in STxMP PUSCH and/or PUCCH transmission.

References

- RP-213598 New WID: MIMO Evolution for Downlink and Uplink

- Chairman Notes of RAN1#109e

- Chairman Notes of RAN1#110

- Chairman Notes of RAN1#110bis-e

- Chairman Notes of RAN1#111

Appendix

Tab1. Link-level simulation parameters

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | CP-OFDM |
| Carrier Frequency | 4 GHz |
| Subcarrier spacing | 30kHz |
| Channel Model | CDL-C in TR 38.901 with 30ns delay spread |
| Delay spread | 30ns |
| UE velocity | 3km/h |
| Allocation bandwidth | 20MHz |
| BS antenna configuration | 4RX: (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| UE antenna configuration | 4TX: (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| MIMO Rank | Rank1/ rank2 per panel (rank fixed) |
| Feedback delay for precoding | 5ms |
| DMRS type | R15 Type 1 DMRS |
| DMRS configurations | Single symbol DMRS without additional DMRS symbols |
| DMRS mapping type | Mapping type A for PUSCH. |
| Link adaptation | PMI and MCS are adaptive |
| Number of CWs | 1 CW/ 2CWs |
| HARQ | Off |
| Channel estimation | Ideal channel estimation |
| Receiver type | MMSE |
| EVM | No radio impairments |