3GPP TSG RAN WG1 #112                                R1-2300563

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.14.3

Source:	Xiaomi

Title:	Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM

Document for:	Discussion and Decision

- Introduction

The Rel-18 WID for Further NR coverage enhancements is approved [1], which includes the following objective:

The objective of this work item is to specify further uplink coverage enhancements for PRACH, power domain and DFT-S-OFDM.

The detailed objectives of the work item are as follows:

- Specify following PRACH coverage enhancements (RAN1, RAN2)

  - Multiple PRACH transmissions with same beams for 4-step RACH procedure

  - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure

  - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable.

  - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable.

- Study and if necessary specify following power domain enhancements

  - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1)

  - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

- Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

Coverage enhancement is one of the key factors for communication networks. At present in real deployments, uplink transmission and coverage have always been the bottleneck of the system performance.

2	Discussion

In NR, two waveforms of DFT-s-OFDM and CP-OFDM can be applied for the uplink. Unlike LTE, a consistent CP-OFDM waveform for the uplink same as downlink is preferred which can benefit the system design and implementation. Considering the better PAPR/CM characteristics of DFT-s-OFDM waveform, it is more suitable for cell-edge UEs to achieve a better coverage and performance. In Rel-16, some PAPR enhancements was also introduced for the DMRS with Pi/2 BPSK modulation for the DFT-s-OFDM waveform to maintain a low PAPR/CM performance. Currently, the waveform used in the uplink is RRC-configured and the switching between CP-OFDM and DFT-s-OFDM needs RRC reconfiguration. This limitation imposes a large barrier to switch over to DFT-s-OFDM waveform for cell-edge UEs. Thus we agree the flexible assessment of DFT-s-OFDM should be pursued.

A lot of debates were held at the stage of the WID, in our understanding of the scope, the dynamic switching is to enhance the switching between CP-OFDM and RANK-1 DFT-s-OFDM.

In RAN1#110bis meeting, the following agreement has been made on DSW [2]:

Agreement

- Dynamic waveform switching enhancement in R18 is only applicable to PUSCH channel.

For PUSCH, this switching flexibility would be beneficial if it can be applicable to both DG PUSCH and CG PUSCH because of the same demands for coverage enhancements, especially when considering URLLC traffic data requires even more robustness.

Proposal 1: For PUSCH, the dynamic switching between CP-OFDM and DFT-s-OFDM supports both DG PUSCH and CG PUSCH, at least CG-PUSCH Type 2.

In RAN1#110bis meeting, the following agreement has been made on mechanism of DSW [2]:

Working Assumption

Support at least one of the following options for the dynamic waveform indication in R18:

Alt 1: Indication from an UL scheduling DCI

- Alt 1-A: New field in scheduling DCI

- Alt 1-B: Reuse existing field in scheduling DCI

  - Alt 1-B-1: Explicit indication by repurposing field, e.g.

    - Add one column to TDRA table

    - Add one column to MCS table(s)

    - Other solutions not precluded

  - Alt 1-B-2: Implicit determination from condition(s) on scheduling information, e.g.

    - RA type, MSB of RA

    - Number of RBs (below threshold or multiple of 2,3,5)

    - Location of RB allocation within carrier and the associated MPR

    - MCS below threshold

    - Number of PUSCH repetitions (or whether PUSCH repetition is used) and/or TBoMS

    - Number of DMRS CDM group(s) without data

    - Precoding information and number of layers

    - SRI

    - Condition over multiple types of scheduling information

    - Other types of scheduling information not precluded

- Indicated waveform applies at least to the scheduled PUSCH transmission

  - FFS: Whether it also applies to subsequent transmissions, and of which type

- FFS: DCI formats can contain the indication

- FFS: Indication applies only if condition(s) are satisfied (e.g. PDCCH occasion, /RNTI, /Search space of the scheduling DCI, latest PHR reported by the UE, etc.)

Alt 2: Indication from a non-UL scheduling DCI

- FFS: DCI formats that can provide the indication (e.g. Downlink DCI, UE-group common DCI)

- FFS: Types of subsequent transmissions to which indication is applicable

In RAN1#111 meeting, the following agreement has been made on assistance information [3]:

Agreement

For DCI based solution,

- For supported dynamically scheduled PUSCH, support dynamic waveform switching indication from UL scheduling DCI

  - Note: “Supported dynamically scheduled PUSCH” is to be confirmed in further discussion

  - Note: It does not imply that the waveform switching indication applies to other transmission or not

- Indication from non-UL scheduling DCI is not supported.

Note: the working assumption made in RAN1#110b-e for “Support at least one of the following options for the dynamic waveform indication in R18” does not need to be confirmed

Working Assumption

Support new 1-bit field for dynamic waveform indication from UL scheduling DCI

- Note: no change of the current size alignment procedure between UL DCI and DL DCI

From previous discussions on the candidate solutions, we prefer Alt.1-A for the simplicity. 1-bit overhead increase is acceptable compared to the increase of the overhead alignment between different waveforms. For Alt.2, it is also beneficial to allow using DL scheduling DCI or GC-DCI for the indication of the waveform to be used for the UL. Then no enhancements is needed for the UL scheduling DCI.

In the last meeting, Alt.1-A was agreed as a working assumption for the dynamic waveform indication for UL scheduling DCI.

Proposal 2: Confirm the Working Assumption.

In RAN1#110bis meeting, the following agreement has been made on assistance information [2]:

Agreement

To study and if necessary, specify, enhancements to assist the scheduler in determining waveform switching, such as:

- Reporting power headroom related information

- Other solutions are not precluded

In RAN1#111 meeting, the following agreement has been made on assistance information [3]:

Agreement

Study the necessity of the following potential enhancements to assist the scheduler in determining waveform switching:

- Reporting power headroom related information based on PCMAX,f,c applicable to a target waveform

  - Target waveform can be same or different from waveform of an actual PUSCH transmission

  - FFS target RB allocation and/or target modulation order can be same or different from respective properties of an actual PUSCH transmission

  - FFS determination of target waveform, target RB allocation, target modulation order

  - FFS details, e.g. report PCMAX,f,c or Type 1 power headroom for a waveform, or difference thereof between waveforms

- PHR triggering enhancements, e.g.

  - Network-triggered PHR

  - PH becomes lower (higher) than a threshold

  - PHR triggered by waveform switching

- Reporting of recommended waveform or request to switch waveform

- Other solutions not precluded

We think it is necessary for the gNB to obtain certain information on power boosting related information, and that can be achieved by enhancing PHR reporting. Instead, UE can also report the proposed waveform to the gNB.

Proposal 3: Support UE reporting of the proposed waveform to the gNB.

Proposal 4: Support to study the PHR triggering and reporting mechanism to help determine a waveform switching.

3	Conclusion

In this contribution, we provide our views of the enhancements on the dynamic switching between uplink waveforms, our proposals are as follows:

Proposal 1: For PUSCH, the dynamic switching between CP-OFDM and DFT-s-OFDM supports both DG PUSCH and CG PUSCH, at least CG-PUSCH Type 2.

Proposal 2: Confirm the Working Assumption.

Proposal 3: Support UE reporting of the proposed waveform to the gNB.

Proposal 4: Support to study the PHR triggering and reporting mechanism to help determine a waveform switching.

4	References

RP-221858, New WID: Revised WID on Further NR coverage enhancements, China Telecom, RAN#96, June 2022

RAN 1#110bis-e Meeting chairman notes, October 2022

RAN 1#111 Meeting chairman notes, November 2022