3GPP TSG RAN WG1 #112	                                                 	R1-2300575

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.3.3

Source:	Xiaomi

Title:	Discussion on potential enhancements on dynamic/flexible TDD

Document for:	Discussion

Introduction

In RAN1#111 meeting [1], the following agreements were achieved for the dynamic TDD:

Agreement For gNB-to-gNB co-channel CLI measurement and/or channel measurement, at least periodic NZP CSI-RS/SSB is the baseline in RAN1 study. - FFS: Whether SSB is CD-SSB or NCD-SSB In the study RAN1 assumes that exchange of configuration for NZP CSI-RS /SSB can be an enabler for gNB-to-gNB CLI measurement and/or channel measurement. Agreement For the purpose of UE-to-UE CLI mitigation, consider the following potential enhancements: - For L1/L2 UE-to-UE CLI reporting, periodic, semi-persistent, aperiodic reporting. - FFS: Event triggered reporting. - For L1/L2 UE-to-UE CLI measurement, periodic, semi-persistent, or aperiodic measurement resource. Companies are encouraged to bring additional details and evaluation results to determine the benefit of the above potential enhancements. Agreement For spatial domain coordination, the exchange of beam related information among gNB(s) (e.g., victim gNB(s) and aggressor gNB(s)) can be an enabler for inter-gNB co-channel CLI management. - For example 1 (from aggressor gNB to victim gNB), DL beam indication from aggressor gNB(s) - For example 2 (from victim gNB to aggressor gNB), preferred/restricted DL beam and associated resource configuration, beam based inter-gNB co-channel CLI measurement result from victim gNB - FFS: how to define DL beam indication - FFS: how to define DL beam Note: The above examples are only provided as starting point for further discussions Agreement For gNB-to-gNB co-channel CLI handling, beam level (i.e., based on measurement result per SSB resource and/or per CSI-RS resource) CLI measurement can be considered for study. Agreement For UE-to-UE co-channel CLI handling, study whether/how to enhance UL power control mechanism. - Existing UL power control mechanism is baseline.

In this contribution, we discuss the possible enhancements on dynamic/flexible TDD.

Discussion

UE-to-UE CLI handling schemes

UE-to-UE CLI measurement and reporting

In previous meeting, it was agreed that L1/L2 UE-to-UE CLI measurement, including periodic, semi-persistent and aperiodic measurement, can be considered as the potential enhancements for the purpose of UE-to-UE CLI mitigation. However, which kind of measurement types should be employed has not been determined yet. In our view, at least the periodic measurement should be supported. Actually, the periodic measurement for L3 UE-to-UE CLI measurement has been supported in Rel-16. Regarding to L1/L2 UE-to-UE CLI measurement, the periodic measurement can be used to offer timely CLI measurement results if proper measurement periodicity is configured. On the other hand, the aperiodic measurement is typically used for the dynamic interference estimation. For instance, gNB can trigger aperiodic measurement in addition to periodic measurement for more accurate assessment on interference level. In this case, the aperiodic measurement can be adopted as a supplement of the periodic measurement.

On the other hand, it was agreed that exiting measurement resources, i.e., SRS resources defined in Rel-16 for SRS-RSRP measurement, CLI-RSSI resources defined in Rel-16 for CLI-RSSI measurement are considered to be a baseline in previous meeting. To support the periodic and aperiodic CLI measurement as aforementioned, the periodic and aperiodic CLI resources, i.e., SRS resources and CLI-RSSI resources can be considered.

Proposal 1: At least L1/L2 based periodic and aperiodic CLI measurement can be supported for UE-to-UE CLI mitigation.

As mentioned above, both periodic CLI measurement and aperiodic CLI measurement can be considered. Considering the aperiodic CSI-RS resources can only be associated with aperiodic CSI-RS reporting in current specification, it is natural that aperiodic CLI reporting should be considered as well. As for the periodic reporting, it can offer the results for the periodic CLI measurement. Besides, the periodic reporting is semi-statically configured and doesn’t need dynamic trigger signaling. In this case, only semi-static information is exchanged between gNBs for periodic reporting, which is beneficial for the reduction of information exchange overhead.

Proposal 2: At least L1/L2 based periodic and aperiodic CLI reporting can be supported for UE-to-UE CLI mitigation.

- CLI measurement metrics

RSRP and RSSI have been adopted for L3 based UE-UE CLI measurement. For L1 based UE-UE CLI measurement, RSRP and RSSI can be reused as they can be used to reflect the level of UE-UE CLI in nature. In addition, CSI and CQI are also supported by several companies [2]. CSI/CQI offers more accurate channel information. Nevertheless, high calculation complexity may be needed for CSI/CQI as they may need non-linear operations.

Proposal 3: The L1 based RSRP and RSSI can be considered as baseline metrics for L1 based UE-to-UE CLI measurement.

- L1/L2 based CLI reporting

As for L1/L2 based CLI reporting, there could be different approaches for the reporting signaling:

- Option 1: Defining separate CLI reporting procedure.

- Option 2: Reused CSI reporting procedure with extension of CSI reporting, i.e. UE-to-UE CLI information is regarded as CSI.

CLI reporting separate from CSI reporting framework is needed if option 1 is supported. For instance, signaling of a CLI reporting config, e.g., CLI-ReportConfig including CLI related reporting quantities, time and frequency resources for CLI reporting, and resources for CLI measurement, etc, should be introduced. The UE performs CLI measurement and reporting based on the configuration of CLI-ReportConfig.  On the other hand, if option 2 is supported, we don’t need to define a new RRC signaling for CLI reporting (e.g., CLI-ReportConfig). Basically, the CSI reporting framework is extended by including the CLI-related measurement metrics, e.g., update reportQuantity with including CLI-RSRP and CLI-RSSI, update CLI-ReportConfig with including resources for CLI measurement. In this case, the UE performs CLI measurement and reporting based on the configuration of CSI-ReportConfig.

Proposal 4: The L1/L2 based CLI reporting signaling can be designed by following approaches:

- Option 1: Defining separate CLI reporting procedure.

- Option 2: Reused CSI reporting procedure with extension of CSI reporting, i.e. UE-to-UE CLI information is regarded as CSI.

Generally speaking, if option 1 is adopted, we may need to consider the how procedure of CLI reporting, at least includes CLI reporting framework design, processing delay. There may be or may not be specification updates for the aforementioned aspects. Hence, it can be imagined that more work load is on the table. On the other hand, everything related to CSI reporting can be almost reused for option 2. Therefore, option 2 is preferred and CLI information can be regarded as a CSI by defining a new reporting type.

Proposal 5: The L1/L2 based CLI reporting should be regarded as a CSI reporting with a new type.

- Subband CLI reporting

The wideband measurement is supported in Rel-16 CLI mechanism, i.e. CLI measurement result is averaged on the CLI resource. The L3 based wideband measurement can reflect the overall interference level related to the CLI resource. However, the CLI level may be varied across different subbands due to the situation of resource occupancy. For instance, the UE may suffer more severe interference within the subband wherein uplink transmission is on-going for another UE. Furthermore, the uplink transmission can be within the UL subband in SBFD scenario and the DL resources close the UL subband suffers more interference. Hence, the subband CLI reporting should be considered for the UE-to-UE CLI measurement and reporting enhancement in order to obtain more accurate measurement.

Proposal 6:  Subband CLI reporting can be considered for UE-UE CLI mitigation.

Time alignment of UE-to-UE handling

Figure 1 The time offset for the reception of CLI RS.

The victim UE needs to receive CLI-RS transmitted by the aggressor UE for CLI measurement, e.g., SRS. As shown in Figure 1, there is a time offset between DL reception timing and CLI-RS arrival timing at UE side. In Rel-16, the time offset is handled by UE implementation. However, the basic assumption is that victim UE and aggressor UE have similar UL timing [2]. On the other hand, the UL timing of Macro UE and indoor UE have significant difference in HetNet scenario, which is the most promising scenario for DTDD.  Hence, the time alignment scheme for UE-to-UE CLI handling needs further study at least for HetNet scenario.

The time offset between the CLI-RS arriving time and DL frame boundary at victim UE side is equal to: NTA,offsetTc+ T1+T2- T3 [4], wherein NTA,offset is TA-related information of aggressor UE, and T1, T2 and T3 are the propagation delay between gNB2 and aggressor UE, the delay between gNB 1 and victim UE, the delay between UE1 and UE2, respectively.  NTA,offset is designed to provide sufficient time for gNB UL/DL transition, which is identical among gNBs within the same TDD band by default [5]. Therefore, it can be assumed that the NTA,offset of the aggressor UE is equal to that of victim UE (NTA, offset' for distinguish).

As for the propagation delay between gNB2 and aggressor UE, considering that the aggressor UE locates under the Micro cell in HetNet scenario as shown in Figure 1, T2 is negligible due to the small cell radius of Micro cell. Taking 50m of cell radius as an example, the propagation delay T2 is equal to 0.17us. Similarly, the propagation delay between UE1 and UE2 is also negligible as they are close to each other once UE-to-UE CLI is an issue. Accordingly, the time offset mentioned above is approximately equal to: NTA,offsetTc+ T1 = (NTA, offset' +NTA/2)Tc , where NTA, offset' and NTA are TA-related information of victim UE.

As for other scenarios where victim UE and aggressor UE locate at same type of serving cells, the UL timing of victim UE and aggressor UE can be considered approximately same, i.e., T1≈T2. Recalling that  NTA, offset'= NTA,offset, and T3 is negligible as analyzed above, the time offset in this scenario is approximately equal to NTA,offsetTc+ T1+T2 ≈ (NTA, offset' +NTA)Tc, wherein NTA, offset' and NTA are TA-related information of victim UE.

Observation 1：The time offset between DL reception timing and CLI-RS arrival timing at UE side can be determined by the UE’s TA information.

Based on the aforementioned observation, the time alignment offset can be realized by UE’s implementation. No more enhancement is needed.

Proposal 7: The misalignment between CLI-RS arrival timing and DL timing at victim UE side can be handled by UE implementation.

- The reception window for multiple aggressors

Multiple aggressor UEs may transmit the CLI-RS simultaneously. Based on the aforementioned analysis, the time offset between DL reception timing and CLI-RS arriving timing at victim UE side can be determined by the TA information of victim UE. Hence, the arriving time of CLI-RSs from multiple aggressor UEs should be approximately same. Accordingly, the misalignment on arrival timing of multiple aggressor UEs may not exceed the CP of DL symbol at victim UE.

Observation 2：In most scenarios, the arriving time of CLI-RSs are within the same CP duration even the CLI-RSs are from multiple aggressor UEs.

The aforementioned observation is obtained based on the ideal situation that the time offset between DL reception timing and CLI-RS arriving timing is totally determined by the victim UE’s TA information. Actually, the above ideal situation may be not true as it is achieved based on some approximate assumption. For example, the time offset between CLI-RS arrival timing and victim UE’s DL timing may exceed CP duration, as shown in Figure 2. The simultaneous reception of multiples CLI-RS will suffer severe inter-symbol interference (ISI). In our view, that problem can be avoided by gNB scheduling, i.e., only the aggressor UEs with similar NTA value can be scheduled to transmit CLI-RS on the same OFDM symbol.

Figure 2 The reception of SRSs from multiple aggressors.

Proposal 8: The simultaneous reception of multiple SRSs from different aggressor UEs from same serving cell for CLI measurement can be realized by gNB scheduling.

gNB-to-gNB CLI handling

The gNB-gNB CLI measurement and reporting scheme is an on-going discussion issue in dynamic/flexible TDD topic, which is not able to be realized by current standard. In our view, the overall procedure for the gNB- gNB measurement and reporting framework can be described as below:

- Step1: Exchange the information of CLI measurement related configuration between CUs if the victim gNB and aggressor gNB belong to different CUs. For instance, the information can be exchanged via Xn;

- Step2: Transmit the CLI-RS by the aggressor gNB based on the measurement configuration provided by CU2; Generally speaking, the RSSI and RSRP are recommended as the measurement metrics for gNB-to-gNB CLI measurement.

- Step3:  Receive the CLI-RS and perform CLI measurement by the victim gNB based on the measurement configuration provided by CU1.

- Step4: Victim gNB reports the measurement results.

- Step5: The victim gNB and aggressor gNB perform the CLI-related operations to mitigate CLI. For instance, the CLI can be mitigated by power, beam related enhancement.

Observation 3：The above procedure of the gNB-gNB measurement and reporting can be a starting point.

gNB-to-gNB measurement and reporting

Regarding the resources for gNB-to-gNB CLI measurement, it has been agreed that the periodic NZP CSI-RS/SSB can be studied as a baseline in previous meeting. If the periodic CLI-RS is supported for periodic gNB-to-gNB CLI measurement, periodic reporting can also be used.  Actually, the periodic reporting can provide timely measurement results with less overhead for information exchange, which is beneficial for the gNB-gNB CLI measurement mitigation.

Proposal 9:  Support periodic reporting for gNB-gNB CLI mitigation.

Another issue needs to be discussed is what kind of SSB can be employed for CLI measurement, i.e., CD-SSB or NCD-SSB. If the NCD-SSB is configured within the active DL BWP, the UE uses that SSB to perform measurement which otherwise should be performed using CD-SSB. Generally speaking, either CD-SSB or NCD-SSB can be used for time synchronization, Radio Link monitoring (RLM), beam failure recovery, etc. What kind of SSB that UE adopts for the above measurement depends on what kind of SSB exists within the active DL BWP. An example is shown in Figure 3, the UE will conduct measurement with CD-SSB if the UE locates on BWP2. With the NCD-SSB, the UE does not need retuning RF and receive CD-SSB if its active DL BWP doesn’t contain CD-SSB. From the perspective of victim gNB, the functionality and structure of CD-SSB and NCD-SSB transmitted by aggressor gNB are same. The only difference is frequency location. Therefore, both CD-SSB and NCD-SSB can be used for gNB-to-gNB CLI measurement.

Proposal 10:  Both CD-SSB and NCD-SSB are sufficient for gNB-to-gNB CLI measurement.

Figure 3 CD-SSB and NCD-SSBs of the aggressor gNB

UE and gNB transmission and reception time

Another issue is how to handle the misalignment between CLI-RS arrival timing and UL timing at victim gNB side. If the aggressor gNB transmits the CLI RS based on its DL symbol timing and the UEs served by victim gNB transmit the UL data based on its UL symbol timing, the time gap between the CLI RS and UL data is equal to NTA, offsetTc+Tdelay, wherein  Tdelay is the propagation delay between aggressor gNB and victim gNB. An example is shown in Figure 4, wherein TTA, offset =NTA, offsetTc.

Figure 4 Timing of CLI-RS and UL data at victim gNB

Currently, the default value of  TTA, offset for FR1 TDD are 13us and 20.3us, and 7us for FR2. Obviously, the arriving time gap shown in Figure 4 exceeds the CP duration of one OFDM symbol. Accordingly, simultaneous reception of CLI RS of UL data may suffer severe ISI.

Observation 4: There is severe ISI between CLI RS and UL data at victim gNB side.

One possible solution is that gNB drops uplink data from served UE and only receives the CLI RS on configured CLI symbols. In this sense, the CLI RS can be received by gNB implementation. No additional specification work is needed. However, gNB may receive the CLI RS across two UL symbols. At least two symbols are not available for UL transmission for victim gNB, as shown in Figure 5. Besides, except the timing for UL data reception (Timing 1 in Figure 6), the gNB has to determine another timing for CLI RS (Timing 2 in Figure 6). It brings additional complexity at gNB side.

Figure 5 The reception of CLI RS with existing TA schemes

Observation 5: One CLI RS symbol may results two UL symbol unavailable at victim gNB side due to the misalignment of timing between CLI-RS arrival and UL timing.

Another solution is to configure the UE served by victim gNB with the zero NTA, offset. With zero NTA, offset, the arriving time gap between the CLI RS and UL data decreases to Tdelay, as shown in Figure 6. Taking 500m cell radium for Macro cell as an example, the propagation delay is equal to 1.67 us, which is within the CP duration assuming 15kHz and 30kHz SCS. As for the scenario with higher SCS, the cell radius will decease correspondingly. To summarize, the zero NTA, offset is sufficient for the timing alignment of UL data and CLI RS reception. The gNB is able to reuse existing timing for both UL data and CLI RS reception. On the other hand, the  NTA, offset  provides guard period for gNB UL/DL transition [5]. If zero  NTA, offset is configured, then the time for guard period disappears. At least one OFDM symbol should be reserved for UL/DL transition, as shown in Figure 6.

Figure 6 Timing of CLI-RS and UL data at victim gNB

Observation 6: For each UL/DL transition at victim gNB, at least one OFDM symbol is not available for the victim gNB if zero NTA, offset is configured.

Coordinated scheduling

Agreement For gNB-to-gNB co-channel CLI measurement, the potential benefit of uplink resources muting can be studied further. Note: Proponents of uplink resource muting are encouraged to provide evaluation result for comparison of performance between two cases when uplink resource muting based gNB-gNB CLI handling schemes including both UE transparent and non-UE transparent schemes is applied or not.

The coordination-based solutions is based on the long term measurement results, the gNB can make proper scheduling decisions, e.g. choose clean time/frequency resources for data transmission to avoid severe interference. The victim gNB can also request the required time or frequency domain resource or TDD configuration to avoid the gNB-gNB CLI.

For the time domain resource coordination, it is possible that the victim gNB can request aggressor gNB to reserve time resource to protect its UL transmission. For example, some slots can be reserved as “UL” only resource to protect its transmission of the important information. Then the aggressor gNB can set the protected slots as the UL slots to avoid the CLI.

For the frequency domain resource coordination, the victim gNB can request to reserve some frequency resource to protect its UL transmission. For example, some UL RBs/subbands/RB sets can be reserved to protect its transmission of the important information. Then the aggressor gNB can hold its DL transmission on the reserved UL resources.

Another issue that needs to be discussed is uplink resources muting scheme. To be specific, the resources for gNB-to-gNB CLI measurement are muted for UL transmission. The scheme may be implemented by gNB scheduling, but such methods will reduce the scheduling flexibility, especially for the periodic UL transmission scheduling. In this case, the indication of UL muting resources to UE may be a better choice. The UE can abandon the UL transmission on the UL muting resources indicated by gNB to reduce the interference to gNB-gNB CLI measurement. In this case, the measurement accuracy can be improved.  Similar mechanism with rate matching patterns (RMP) can be re-used with potential enhancement for the indication of reserved resources.

Proposal 11: The RMP can be considered with potential enhancement to support UL reserved resource indication.

Power control

For the gNB-gNB CLI mitigation, aggressor gNB can ramp down its DL transmission power to reduce the interference level and the victim gNB ramp up UE’s UL transmission power to improve its signal strength.

For the case the aggressor gNB adjust its DL transmission power to reduce the interference level, based on the CLI measurement result, the victim gNB can request the wanted DL power level from the aggressor gNB on intended resources..

For the case the victim UE adjust its UL transmission power to boost its signal strength, based on the CLI measurement result, the victim gNB configures different power parameters on different sets of resources, the UEs can then adjust its UL transmission power based on the configurations. The power control scheme by taking into account the CLI interference level from the aggressor may need to be studied.

Proposal 12: The power adaptation schemes to alleviate the CLI issue can be further studied.

Conclusions

In this contribution, we provide the potential scenario and candidate solutions for the dynamic/flexible TDD, the following proposals are made based on our analysis:

For the UE-UE CLI handling:

Observation 1：The time offset between DL reception timing and CLI-RS arrival timing at UE side can be determined by the UE’s TA information.

Observation 2：In most scenarios, the arriving time of CLI-RSs are within the same CP duration even the CLI-RSs are from multiple aggressor UEs.

Proposal 1: At least L1/L2 based periodic and aperiodic CLI measurement can be supported for UE-to-UE CLI mitigation.

Proposal 2: At least L1/L2 based periodic and aperiodic CLI reporting can be supported for UE-to-UE CLI mitigation.

Proposal 3: The L1 based RSRP and RSSI can be considered as baseline metrics for L1 based UE-to-UE CLI measurement.

Proposal 4: The L1/L2 based CLI reporting signaling can be designed by following approaches:

- Option 1: Defining separate CLI reporting procedure.

- Option 2: Reused CSI reporting procedure with extension of CSI reporting, i.e. UE-to-UE CLI information is regarded as CSI.

Proposal 5: The L1/L2 based CLI reporting should be regarded as a CSI reporting with a new type.

Proposal 6:  Subband CLI reporting can be considered for UE-UE CLI mitigation.

Proposal 7: The misalignment between CLI-RS arrival timing and DL timing at victim UE side can be handled by UE implementation.

Proposal 8: The simultaneous reception of multiple SRSs from different aggressor UEs from same serving cell for CLI measurement can be realized by gNB scheduling.

For gNB-gNB CLI handling:

Observation 3：The above procedure of the gNB-gNB measurement and reporting can be a starting point.

Observation 4: There is severe ISI between CLI RS and UL data at victim gNB side.

Observation 5: One CLI RS symbol may results two UL symbol unavailable at victim gNB side due to the misalignment of timing between CLI-RS arrival and UL timing.

Observation 6: For each UL/DL transition at victim gNB, at least one OFDM symbol is not available for the victim gNB if zero NTA, offset is configured.

Proposal 9:  Support periodic reporting for gNB-gNB CLI mitigation.

Proposal 10:  Both CD-SSB and NCD-SSB are sufficient for gNB-to-gNB CLI measurement.

Proposal 11: The RMP can be considered with potential enhancement to support UL reserved resource indication.

Proposal 12: The power adaptation schemes to alleviate the CLI issue can be further studied.

References

- 3GPP RAN1#111 chairman-note, 3GPP TSG RAN WG1 #111, November 14th – 18th, 2022.

- R1-2211399, Intel Corporation, Potential enhancements on dynamic/flexible TDD, 3GPP TSG RAN WG1 Meeting #111, Toulouse, France, November 14th – 18th, 2022.

- R1-2212116, Qualcomm Incorporated, On potential enhancement on dynamic/flexible TDD, 3GPP TSG RAN WG1, Meeting #111, Toulouse, France, November 14th – 18th, 2022.

- R1-1902292, Samsung, Cross-link interference measurements and reporting at a UE, 3GPP TSG RAN WG1 #96, Athens, Greece, 25th February – 1st March, 2019.

- R4-1804119, Intel Corporation, On NR NTA offset, 3GPP TSG-RAN WG4 Meeting #86bits, Melbourne, Australia, April 16 – April 20, 2018.

- 3GPP TS 38.133, “Requirements for support of radio resource management (Release 16)”.