3GPP TSG RAN WG1 #112		                                                                 R1-2300613

Athens, Greece, February 27th – March 3rd, 2023

Title:	Draft Reply LS on PSFCH configured power with multiple resource pools configuration

Response to:	R1-2300030/R4-2220553

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	[RAN1], CATT, GOHIGH

To:	RAN4

Cc:	RAN2

Attachment:	-

Contact Person:

Name:	Rui Zhao

E-mail Address:	zhaorui@gohigh.com.cn

Overall description

RAN1 thanks RAN4 for the LS on PSFCH configured power with multiple resource pools configuration. RAN1 would provide response to the following question from RAN4.

Question 1: RAN4 agreed to define PEMAX,c for PSFCH configured Tx Power in TS38.101-1 as sum of IEs sl-maxTransPower when multiple resource pools configured are transmitted simultaneously and would like to know if this can have any impact to RAN1.

Answer:

In the RAN1 specification, only the PCMAX is used while the definition is referred to the RAN4 specification. Thus, the current definition of RAN4 has no impacts to RAN1 specification.

Question 2: RAN4’s understanding is that the total transmit power is shared equally between simultaneously transmitted PSFCHs regardless of whether they are from a single or multiple resource pools. RAN4 would like to confirm with RAN1 that this understanding is correct.

Answer:

According to RAN1’s specification, the transmit power of PSFCH is defined in resource pool specific manner. It can only ensure that the transmit power is equally between simultaneously transmitted PSFCHs when they are from a single resource pools.

- 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions for HARQ-ACK information and conflict information, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i on active SL BWP b of carrier f as

Actions:

To RAN2:

RAN1 respectfully requests RAN4 to take the above responses into account in the future work.

Date of Next TSG-RAN1 Meetings:

RAN1#112bis-e			17th -26th April 2023				E-Meeting

RAN1#113			22th -26th May 2023 				Incheon, Korea