3GPP TSG RAN WG1 #112                                                                                   R1-2300670

Athens, Greece, February  27th – March 3rd, 2023

Source:	CATT

Title:	Discussion on general aspects of AI/ML framework

Agenda Item:	9.2.1

Document for:	Discussion and Decision

Introduction

Artificial Intelligence (AI) and Machine Learning (ML) are powerful in solving non-linear issues, and become one of the most popular research directions around the world. For wireless communication, AI/ML also attracts strong interest from academic circle, and already shows its capability in improving performance in many fields. 3GPP also finished an RAN3-led AI/ML study in Rel-17, in which AI/ML models are applied for better data collections in several typical use cases, including network energy saving, load balancing, and mobility optimization [1]. In Rel-18, a study item on AI/ML in RAN1 was approved [2], to investigate the support of AI/ML in physical layer other than implementation-based approaches.  Substantial progress was achieved on life cycle management (LCM), common KPIs, generalization and a few terminologies in RAN1 meetings [3][4][5][6].

Further investigation is needed to complete the study. In this contribution, we share our views on the general aspects of AI/ML-based approaches for air interface, including the terminology, collaboration levels, life cycle management and UE capabilities. We also provide our views on the common evaluation methodology and KPIs.

Discussion

Terminology

As required by the SID, the study shall identify common notation and terminology for AI/ML related functions, procedures and interface [2]. An initial terminology list was built up as Working Assumption for RAN1 discussion in RAN1#109-e [3]. In RAN1#110, new terms on online training, offline training and model delivery were agreed to be captured in the terminology list [4].

Model transfer

In RAN1#109-e, it was agreed to differentiate Level y and Level z by whether model transfer is applied or not. In addition, the following terminology for model transfer was agreed in the same meeting [3].

| AI/ML model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| --- | --- |

In RAN1#110, RAN1 group agreed to incorporate the term ‘model delivery’ in the terminology list [4]. Note that the definition of model delivery is very inclusive since it can be any way to deliver a model. It is convenient to use ‘model delivery’ to describe the delivery of an AI/ML model from one entity to another, even including methods outside 3GPP, i.e. WIFI.

The following WA was achieved on the boundary between collaboration Level y and Level z [5]:

Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not. - Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z - Clarifying note: Level y includes cases without model delivery.

Unlike ‘model delivery’, the term ‘model transfer’ should serve the purpose of restricting the model delivery within 3GPP’s scope. Based on the WA above, the definition of model transfer shall be revised to reflect that the model is delivered via 3GPP signaling in collaboration Level z. Otherwise, the definition is not consistent with the working assumption. The revised part is marked in red in the following proposal.

Proposal 1: The definition of ‘AI/ML model transfer’ is revised (marked in red) as follows:

| AI/ML model transfer | Delivery of an AI/ML model over the air interface via 3GPP signaling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| --- | --- |

Model selection

In RAN1#111, the following definition for model selection was discussed, but not agreed due to limited online time [7]:

| Model selection | Selection of an AI/ML model for activation among multiple models for the same functionality. Note: Model selection may or may not be carried out simultaneously with model activation |
| --- | --- |

There are some similarities between model activation, model selection and model switching. But they can still be distinguished by:

- Model activation focuses on making an AI/ML model ‘usable’ from inactive state. It does not imply multiple AI/ML models with the same functionality are deployed. The action of activation may need some time, which may be non-negligible.

- Model selection, as clarified in the intended definition, hints choosing an AI/ML model from ‘multiple’ models for the same functionality. The key point may be the determination of which model should be selected, e.g. based on model monitoring. Logically, an AI/ML model will be selected before activated. But from view of specification they may be combined together.

- Model switching emphasizes activating one model while deactivating another model, from multiple models with the same functionality. It is more like a compound procedure, which is subjected to the limited device capability that only one model can be activated concurrently.

Based on the above observations, we think the previous terminology of model selection can be directly agreed as working assumption. This does not imply that model selection will have specification impact in the future.

Proposal 2: Capture the definition of model selection in the working list of terminologies.

| Model selection | Selection of an AI/ML model for activation among multiple models for the same functionality. Note: Model selection may or may not be carried out simultaneously with model activation. |
| --- | --- |

General AI/ML framework

Functional framework

In our view, the framework of AI/ML application in air interface should embody how AI/ML model is trained, deployed, monitored and interactive with other modules for wireless communication. Thus the framework shall at least include function blocks as: data collection, model training, model management (e.g. model selection/switching/monitoring etc.), AI/ML model and actor. Figure 1 illustrates how these function blocks are interactive with each other. Potential data feedback from actor to data collection block is marked in dash line.

Figure 1 Functional framework of AI/ML in NR air interface

Compared to RAN3 framework, model management becomes much more complicated. Specifically, in RAN3, all AI/ML models are deployed within network. Thus all LCM operations are always up to network implementation and may not need to present in the functional framework. However, in RAN1, collaboration between UE and network is introduced for AI/ML based approaches. A model deployed in one node may also be (fully or partially) managed by the other node. Finer illusion is needed to demonstrate the difference. For example, the purpose of data collection becomes more complicated, e.g. training, inference and monitoring, which lead to different input and output directions. Such new features/functions require careful study than before.

Proposal 3: Compared to RAN3 framework, life cycle management becomes much more complicated and should be considered carefully in RAN1 framework.

In last meetings, good consensus was reached on deferring functional framework discussion until sufficient progress is made on LCM. From the view of study progress, we agree that sufficient process on LCM should be achieved firstly. Then we can consider how to define RAN1 framework for AI/ML-based approach.

Proposal 4: Wait until sufficient progress is made on LCM before deciding how to capture it into functional framework.

Collaboration levels

As required by the SID, several collaboration levels between UE and gNB should be identified. According to the interaction degree in terms on signaling between UE and gNB, three collaboration levels were agreed as Level x, Level y and Level z [3]. Collaboration Level x is a pure implementation based method, while Level y and Level z require 3GPP signaling interaction to manage the AI/ML model. With clear boundary between Level x/y/z, we should further align the understanding on the relationship between collaboration levels and LCM.

For LCM of an AI/ML model in NR air interface, it was agreed that two categories can be applied, i.e. model identification and functionality identification [6]. For functionality identification, the model of one side (e.g. UE side) will not be identified by the other side (e.g. NW side). Obviously, functionality identification can only enable collaboration level y, but may not support some LCM procedures, e.g. model switching requested by NW. Model identification may enable collaboration level y or level z. If the UE only reports its model(s) to NW but does not support model transfer, then it supports collaboration level y. Most LCM procedures will be supported, except model transfer. On the contrary, if the UE supports model transfer, it supports collaboration level z. When transferred from NW to UE (not transparently from view of 3GPP OTA signaling), an AI/ML model can be considered as identified by the UE and NW.

The following table summarizes the relationship between collaboration levels, functional/model identification and LCM procedures.

Table 1 Relationship between collaboration levels, functional/model identification and LCM procedures

| Identification | Model transfer | Collaboration level | LCM procedure within 3GPP |
| --- | --- | --- | --- |
| No identification (implementation based) | N/A | Level x | No |
| Functionality identification | Not support | Level y | Yes, partially, e.g. AI/ML function enabling, fallback to non-AI/ML based approach, performance monitoring. |
| Model identification | Not support | Level y | Yes, mostly, e.g. AI/ML model activation/deactivation, model selection, model switching, model monitoring, etc. |
|  | Support | Level z | Yes, all LCM procedures can be supported. |

Observation 1: From UE’s perspective,

- If a UE only supports functionality identification, collaboration level y can be enabled. Partial standardized LCM procedure within 3GPP network can be supported, e.g. AI/ML function enabling, fallback to non-AI/ML based approach, performance monitoring.

- If a UE supports model identification but not model transfer, collaboration level y can be enabled. Most standardized LCM procedures within 3GPP network can be supported, e.g. AI/ML model activation/deactivation, model selection, model switching, model monitoring, fallback.

- If a UE supports model identification and model transfer, collaboration level z can be enabled. All standardized LCM procedures can be supported.

Common evaluation methodology and KPIs

Model Generalization

The following three approaches were proposed to achieve good performance across different scenarios, configurations or sites in RAN1#110bis-e [5]:

- Model generalization, i.e., using one model that is generalizable to different scenarios/configurations/sites

- Model switching, i.e., switching among a group of models where each model is for a particular scenario/configuration/site

- Model update, e.g. model fine-tuning.

Table 2 gives a high-level overview of the complexity, feasibility and requirement for these three approaches.

Table 2 Comparison of different approaches to deal with different scenarios/configurations/sites

| Approaches | Model generalization | Model switching | Model update |
| --- | --- | --- | --- |
| Storage requirement | Relatively larger | Relatively larger | Relatively smaller |
| Computation complexity | Relatively higher | Relatively lower | Relatively lower |
| Typical LCM signaling other than activation, deactivation and fallback | None | Selection, Switching | Update, [Configuration], (Re-)training |
| Model delivery/transfer after initial deployment | No strong need | No strong need | Possible |
| Model re-training (incl. online training) | No strong need | No strong need | Possible |
| Model monitoring metric related to KPIs | Yes | Yes | Yes |
| Model monitoring based on data distribution or applicable condition | May not be distinguishable | Yes | Yes |
| Performance | FFS, depends on the outcome per use case |  |  |

The above analysis is based on the empirical assumption that: model generalization requires a relatively larger size model, while model switching relies on a set of smaller size models. Model update only needs smaller size model too. Model update may be based on online training or offline training, with model delivery/transfer or developed by itself (i.e. w/o model delivery/transfer).

However, for all approaches, further study is required in each use case to provide more information on performance. Conclusion/prioritization should not be made in rush before we have clear understanding on the trade-off of these approaches.

Proposal 5: For comparison between model generalization, model switching and model update, further study these approaches in each use case. No need to draw conclusion/prioritization before sufficient investigation.

Common KPI for AI/ML-based approach

Several agreements on initial list of common KPI were achieved in past few meetings [4][5], including:

- Performance

- Over-the-air Overhead

- Inference complexity

- Training complexity

- LCM related complexity and storage overhead

- FFS: Power consumption, latency (e.g., Inference latency)

Most of the agreed common KPIs are clear, which are intuitively computable. Performance related KPIs are varying among different use cases, but consensus is already achieved in each agenda. OTA related KPIs focus on size and possibly periodicity of the signaling for model management. Inference complexity can be reflected by the complexity of computation and model size. Nevertheless, some KPIs may need further clarification.

Training complexity is affected by many aspects and uneasy to quantize. To train a specific AI/ML model, there is roughly a most suitable size of training data, which avoids overfitting but acquires considerable generalization ability. Empirically, the more complex problem is targeted, the larger AI/ML model should be used. And subsequently, more training data should be collected. Hence, training complexity can be reflected by model complexity, to a certain degree.

Observation 2: Training complexity can be reflected by model complexity.

To solve different problems in different scenarios by AI/ML-based approach, various strategies may be considered, e.g. supervised learning, unsupervised learning, reinforcement learning and transfer learning. Under different training strategies, model performances, converge latencies, required memories, computation operations are different. It is vendors’ freedom to design and choose their training strategies. Companies may share how their AI/ML models are trained to contribute the study, but it seems not meaningful to set up training related KPIs for comparison, especially for offline training. For online training, since physical layer is sensitive to latency and traffic load which can be affected by training complexity, we can further discuss this case later.

Proposal 6: At least for offline training, companies can voluntarily share their training strategies, but no need to set up training complexity KPIs for comparison.

It is FFS whether latency shall also be a common KPI. It is understandable that latency is sensitive in wireless communication system. However, there is some difficulty to set a KPI of latency for an AI/ML model. For model complexity itself, an exact number can be calculated in terms of, e.g. FLOPs or number of parameters. But when it turns to latency, we can foresee that:

- The inference latency not only depends on model complexity, but also computation power that the deployed node can offer. Even for the same model at the same node, the inference latency may be varying if the offered computation power is varying.

- The inference latency depends on not only model complexity, but also optimization in software/hardware/firmware. For the same model at different nodes, the inference latency may also be varying.

Hence, inference latency is a comprehensive result. It is highly related to the capability of deployed node, rather than the model itself. It is more proper to discuss latency in UE/network capability, e.g. as part of computation power.

Proposal 7: It is more proper to discuss inference latency under UE/network capability, rather than common KPI for AI/ML model.

It is also questionable whether power consumption should be a common KPI. We understand that power may be important especially for UE side, but it is also true that:

- It is difficult to define feasible EVM for power consumption for both AI/ML-based and non-AI/ML-based approaches.

- Even for the same AI/ML model, power consumption may be varying among different UEs/platforms due to different implementation methods in hardware and firmware.

To balance the interest and difficulty, we have the following proposal.

Proposal 8: Power consumption is not considered as a common KPI for AI/ML-based approach.

- It is up to companies’ interest to report their results of power consumption.

Life cycle management

Model training

From the view of simplicity, we think offline training should be a starting point. This is due to the fact that offline training is more robust since more data and sufficient time are allowed for validation and test. In some cases, when AI/ML models are updated/fine-tuned periodically, they are still classified as offline training based on the relative time scale between inference and update. It is more suitable to assume offline training when we start the analysis of LCM.

In previous meetings, some companies’ interest on online training is observed. Although we should aware that the cost of computation and information exchange will be increased by online training, we are fine to consider it as the supplementary investigation. Anyway, RAN1 may need to provide analysis/conclusion on specification impact, complexity, and performance for both approaches at the end of the study.

Observation 3: RAN1 may need to provide analysis/conclusion on complexity, performance and spec impact for both offline and online training at the end of SI. Parallel study on both approaches is necessary.

Model/functionality identification

To align the understanding of AI/ML-based approaches between UE and NW, two methods can be applied, i.e. model identification and functionality identification [6]:

Working Assumption Terminology; Description. Model identification; A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification.. Terminology; Description. Functionality identification; A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality. Note: whether and how to indicate Functionality will be discussed separately.

It is FFS the granularity of functionality. To us, it is quite clear that at least sub-use case level should be supported. Otherwise, the other side still cannot figure out what’s the AI/ML-based functionality that can be provided. Whether finer granularity is needed can be further studied.

Proposal 9: For functionality identification, the granularity of functionality should be at least sub use case level. Whether finer granularity is needed can be FFS.

One remaining issue is the dependence between functionality identification and model identification. From NW’s view, indeed, if a UE performs model identification to NW, the functionality of the model(s) will also be identified by NW, e.g. via model description information. But anyway, both approaches should be studied during the SI. Whether functionality identification is the prerequisite of model identification can be left to normative phase, when UE capability needs to be designed.

Proposal 10: The functionality of an AI/ML model can be identified by model identification. Whether functionality identification is the prerequisite of model identification can be left to normative phase.

For model identification, model description information is the core part. This may also be known as metadata. It will be impossible to perform LCM within 3GPP without sufficient model description information of the identified model. As a starting point, the following information can be considered for model description information when provided from UE to NW:

- Model functionality

  - The basic information that should be provided, e.g. the model is for CSI compression, beam prediction, positioning. The indication should be sub-use case level or even finer level. Forward capacity should be considered for other functionalities agreed in future.

- Model input/model normative input

  - This information is useful at least for model inference and model monitoring, e.g. whether the input is raw channel or eigenvector, for intermediate KPI calculation in CSI compression. It is also useful if UE asks network to provide some dataset collected in local site for model fine-tuning.

  - The input may just be normative from specification point of view, while the actual input can be deduced from normative input but up to UE’s implementation.

  - If UE use some additional proprietary input that does not need network’s involvement, the UE may not need to inform such additional proprietary to network, e.g. compensation for UE’s sampling frequency offset.

- Model output/model normative output

  - Network need to understand the output type of the model, e.g. whether it is a spatial domain or time domain prediction for beam management, whether the output is predicted RSRP and/or predicted beam ID for beam prediction.

  - The output may just be normative from specification point of view. Normative output is deduced/post-processed from the actual output, which is up to UE’s implementation.,

  - Output format, e.g. the dimension of the output.

- Assistance information for inference

  - This may include, for example, quantization related information for CSI compression. For another example, the beam shape related information for beam prediction. The UE may require some assistance information/signal from network. The required assistance information should be discussed per use case, and should not disclose the unavailable private information.

- Model performance

  - This may include, e.g. performance of inference accuracy or system performance, inference latency, etc., which can help network’s decision on whether and when to activate the model.

  - The performance may be hypothetic or predicted.

- Concurrent use with other AI/ML models and/or non-AI/ML features

  - This information is useful for network to plan the LCM of AI/ML model, when multiple AI/ML models (e.g. with different functionality) cannot be activated simultaneously. Note that multiple AI/ML models share the limited storage and computation power.

  - Whether an active AI/ML model has no impact on all non-AI/ML features is still questionable. This is desired, but currently this is not guaranteed. It may impact model monitoring when some legacy mechanism is involved.

- Applicable conditions

  - The condition may be, e.g. scenarios, configurations or sites. This information may be useful for model switching/selection/fallback, e.g. supported payload sizes for CSI compression, supported perdition time for beam prediction, or indoor/outdoor requirement for positioning. It may also enable model monitoring based on applicable conditions.

- Pairing information for two-sided model

  - This will be useful when multiple network parts of two-sided models are deployed at network, for the case of, e.g. separate training. But the feasibility for UE to be aware of network’s model(s) needs further discussion. One possible way may be that UE and network train their models based on a common dataset, then the UE only needs to indicate the training dataset to network. Network can be aware of the paired model based on the indicated dataset.

Proposal 11: For model identification, the following information can be considered as the starting point for model description information when provided from UE to NW:

- Information on model functionality,

- Information on model input/model normative input,

- Information on model output/model normative output,

- Information on assistance information for inference,

- Information on model performance,

- Information on concurrent use with other AI/ML models and/or non-AI/ML features,

- Information on applicable conditions,

- Information on pairing information for two-sided model.

Note that, model description information is not only critical in model identification, but also in model transfer via 3GPP signaling, which will be discussed in Section 2.4.5. We think model description information for model identification can be reused for model transfer.

Proposal 12: Model description information for model identification can be reused for model transfer.

For an identified AI/ML model, a unique model ID, from UE’s or network’s point of view, is needed for model management, e.g. indication of the model. Note that this is aligned with RAN2’s assumption. For a model identified from UE to network, it is expected that such unique model ID will be assigned to the AI/ML model by network, or reported by UE. Whether the unique model ID is reported by UE or assigned by network can be further studied. Also, whether the model ID is global or not can be further discussed. Moreover, the model ID can be explicit or implicit indication, which can also be further studied.

Proposal 13: For an identified AI/ML model, further study the following aspects on the model ID:

- Whether the model ID is reported by UE or assigned by network,

- Whether the model ID is global or not,

- Whether the model ID is explicit or implicit.

Model monitoring

The following agreement was reached in RAN1#110bis-e, which gives a high level guidance for model monitoring metrics/methods [5]:

Agreement Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case: - Monitoring based on inference accuracy, including metrics related to intermediate KPIs - Monitoring based on system performance, including metrics related to system peformance KPIs - Other monitoring solutions, at least following 2 options. - Monitoring based on data distribution - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc. - Output-based: e.g., drift detection of output data - Monitoring based on applicable condition Note: Model monitoring metric calculation may be done at NW or UE

Detailed definition of each possible metrics can be found under different agendas for different use cases. Note that further processing can be considered to improve the robustness of model monitoring, e.g. filtering/averaging in time domain or acquiring statistical result within certain duration. Whether criterions need to be specified can be further discussed.

One key motivation of model monitoring is to determine the most suitable AI/ML model in current time. Ideally, UE or NW can always switch to the selected AI/ML model to achieve best performance among all AI/ML models. This means not only the activated AI/ML model but also the inactive model(s) should be monitored, As a result, it is important to study how to monitor the performance of inactive AI/ML models.

One possible way to estimate the inactive AI/ML model may be, e.g. input data distribution or applicable condition. However, whether these are the only solutions and the monitoring accuracy/effectiveness need more study.

Proposal 14: For model monitoring, further study how to monitor the performance of inactive AI/ML models for better LCM..

Model selection, activation, deactivation, switching and fallback operation

The following agreement was achieved in RAN1#110bis-e in agenda 9.2.1 [5]:

Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models FFS: other mechanisms

Meanwhile, another agreement was achieved in agenda 9.2.2.2 [5], especially for two-sided model:

Agreement In CSI compression using two-sided model use case, study potential specification impact for performance monitoring including: - NW-side AI model performance monitoring: NW monitors the performance metrics and make decisions of model activation/ deactivation/updating/switching - UE-side AI model performance monitoring: UE monitors the performance metrics and reports to Network, NW will further makes decisions of model activation/ deactivation/updating/switching

Combining the agreements above, we can see that:

- If UE can make the decision of model activation/deactivation/updating/switching, the model can only be UE-sided model, but not two-sided model, at least from potential specification point of view.

- If UE manages its one-sided model by itself and not reported to network, it should belong to collaboration Level x. There is no specification impact at all, and out of 3GPP study.

Proposal 15: For model selection, activation, deactivation, switching and fallback for single UE sided models, if the UE makes decision autonomously without reporting to network, the case belongs to collaboration Level x and has no specification impact.

Proposal 16: For model selection, activation, deactivation, switching, and fallback for UE part of two-sided models, network makes decisions and informs to UE. It belongs to collaboration Level y or Level z.

It is FFS for network-sided model. For network-sided model, we can see that:

- For single-sided model, model selection, activation, deactivation, switching, and fallback can be handled within network without UE’s involvement. This can be collaboration Level x without specification impact. Meanwhile, collaboration Level y or Level z is also possible, if UE’s assistance on data collection or model monitoring is needed for network’s AI/ML model.

- For network part of a two-sided model, model selection, activation, deactivation, switching, and fallback shall not be decided by a UE. It is expected that network serves multiple UEs with one network part of two-sided model, so the decision shall be made by network jointly considering performance of all UEs. The decision may or may not inform to UE, which may need more study.

Proposal 17: For model selection, activation, deactivation, switching, and fallback for single network sided models, network makes decisions and may or may not inform to UE:

- If the decision is not informed to UE, it belongs to collaboration Level x,

- If the decision is informed to UE, it belongs to collaboration Level y or Level z.

Proposal 18: For model selection, activation, deactivation, switching, and fallback for network part of two-sided models, network makes decisions and may or may not inform UE. It belongs to collaboration Level y or Level z.

- FFS whether/when network’s decision needs to be informed to UE.

Table 3 summarizes the handling of the above operations.

Table 3 Overview of model selection, activation, deactivation, switching, and fallback

| Model location | Decided by which entity | Triggering | Applicable collaboration | Applicable model type |
| --- | --- | --- | --- | --- |
| UE side | By network | Network-initiated | Level y or Level z | One-sided, two-sided |
|  |  | UE-initiated, requested to the network | Level y or Level z | One-sided, two-sided |
|  | By UE | Event-triggered as configured by the network, UE’s decision is reported to network | Level y or Level z | One-sided |
|  |  | UE-autonomous, UE’s decision is reported to the network | Level y or Level z | One-sided |
|  |  | UE-autonomous, UE’s decision is not reported to the network | Level x | One-sided |
| Network side | By network | Network-initiated, network’s decision is not informed to UE | Level x | One-sided |
|  |  | Network-initiated, network’s decision is informed to UE | Level y or Level z | One-sided |
|  |  | Network-initiated, network’s decision may be (FFS: whether/when) informed to UE | Level y or Level z | Two-sided |
|  |  | UE-initiated, requested to the network | Level y or Level z | Two-sided |

Among all the options, we can see that:

- Level x options does not have specification impact and out of 3GPP study.

- UE-autonomous decision is possible, but the decision usually counts only one UE’s performance only. Such decision is likely to be sub-optimal and even erratic to the network.

- Network decision can take into account both network-initiated and UE-initiated request. This is the safest method to guarantee the network performance.

Therefore, we suggest prioritizing network decision method in the study.

Proposal 19: For model selection, activation, deactivation, switching, and fallback, prioritize the mechanism where the decision is made by network.

Model transfer

As discussed in Section 2.1, ‘model transfer’ can be defined as model delivery via 3GPP signaling. When performed, network and UE shall be aware of that one or more AI/ML models are transferred with certain purpose, thanks to the model description information along with the delivered model.

Usually, network has higher capability of computation and storage than UE. It is also easier for network to collect a large number of data with integrity and generalization. Hence, it is more convenient for network to perform AI/ML model training than UE, especially for initial training. In addition, site-specific AI/ML model may achieve better performance across different cells. As a result, model transfer from network to UE seems more usual.

Observation 4: It is more usual to perform model transfer from network to UE.

As part of LCM, RAN2 may design signaling for model transfer, which needs RAN1’s input as baseline. Take model transfer from network to UE as example. The following aspects can be further studied:

- Full or partial model

Usually, for initial model deployment, full model should be transferred. But for model update, partial model may be enough. For example, only the parameters are transferred while the model structure is unchanged. For another example, only partial parameters are transferred. This may impact the size of model transfer.

- Periodicity/trigger

Whether a model is valid or not depends on the similarity of distribution between the input data and the training dataset. An AI/ML model may be outdated and become invalid. In different use cases, the periodicity of model transfer (e.g. for model update) may be different, which needs further study. It is also possible that no obvious periodicity is found, so model transfer will be triggered dynamically.

- Latency and reliability requirements

The latency and reliability of model transfer may impact the overall performance of AI/ML-based approach. In general, smaller latency and higher reliability will facilitate faster model deployment. Suggestions from RAN1 will be helpful for RAN2 to consider related signaling.

- Model delivery format

As agreed in RAN1#111, proprietary format model and open format model are considered in RAN1 study. Both of them may be used during model transfer. For either way, RAN1 should study the feasibility, effort, pros and cons of different solutions on acquiring available model delivery format between UE and network.

Some other aspects may also be considered. For example, whether the model is transfer via User Plane or Control Plane. These can be up to RAN2.

Proposal 20: For model transfer, the following aspects can be further studied in RAN1:

- Full or partial model transfer.

- Periodicity/trigger.

- Latency and reliability requirement.

- Model delivery format.

Model delivery format

The following working assumption on model format is achieved in RAN1#111 [6]:

Working Assumption Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion, Proprietary-format models; ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format. Open-format models; ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspective. From RAN1 discussion viewpoint, RAN1 may assume that: - Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared. - Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared

Note that the above definition is from view of 3GPP. Hence, open format include both open MRF (e.g. ONNX) and 3GPP-standadized MRF. Some pros and cons of these formats are summarized in Table 4.

Table 4 Comparison between different model delivery formats

| Formats | Pros | Cons |
| --- | --- | --- |
| Proprietary format | - Allow customized optimization for hardware of a specific vendor - Will not disclose proprietary information across vendors - No compiling issue when binary image is used | - Need additional effort for cooperated agreement involving multi-vendors, especially for network side - Higher implementation efforts to support multiple proprietary formats - Hard to support flexible partial model update |
| Open format | - Friendly to international cooperation and independent development - Allow flexible partial model update - Can be directly applied in the near future (for open MRF like ONNX) - Maintenance and evolution of MRF is under 3GPP control (for 3GPP-standadized format) | - May not allow customized optimization for hardware of a specific vendor - Proprietary information may be disclosed - May require compiling capability in target node - May have license/authorization issue(for open MRF like ONNX) - May take huge effort to develop the format (for 3GPP-standadized format) |

It can be observed that different model formats have different advantages and disadvantages. Despite the engineering difficulty, it may be realistic to consider proprietary format or open format at present. However, 3GPP-standadized MRF should be a right direction in a long-term study. This is the only way that can guarantee both international cooperation and 3GPP control. We can have parallel study for all these formats.

Proposal 21: Further study proprietary format and open format in a parallel way.

No matter proprietary format or open format is used, a well-designed model description information is important for model transfer and model identification. Even for open format, the target node cannot understand the functionality, input and output without proper information. Therefore, model description information should be provided regardless the model format. An example is given in Figure 2 for model transfer in collaboration Level z, i.e. using 3GPP signaling over the air interface.

Figure 2 One possible structure for model transfer via 3GPP OTA signaling

Proposal 22: Model description information should be provided during model transfer, regardless proprietary format or open format is used.

UE capabilities

AI/ML-based approaches are data-driven and rely on huge computation power at least for training. In general, deploying AI/ML model at network side should be simpler. Meanwhile, deploying AI/ML model at UE side is also possible and promising, but we need to be very careful in terms of UE capability.

In case a UE supports AI/ML-based approaches, several levels of UE capabilities should be defined. The following aspects should be considered as a starting point:

- Hardware related capability

Hardware related capability mainly refers to the storage and computation power. They may be changed dynamically along with the time.

- Storage/buffering size

All the AI/ML models at UE side share the storage of UE hardware. The capability of storage/buffering size may impact: (1) the number of AI models that can be supported/configured to the UE, and (2) the size of each AI model that can be supported/configured.

- Computation power

All the AI/ML models at UE side share the computation power of UE hardware. The capability of computation power may impact: (1) the number of simultaneous activated AI/ML models, and (2) the inferring/training latency subject to a specific size of AI/ML model.

- LCM related capability

Some LCM procedures may be more ‘essential’ and should be supported anyway, e.g. model activation/deactivation. However, some other LCM operations may be more ‘optional’ and raise higher requirement to UE.

- Capability of online training

Online training requires frequent update of deployed AI/ML model in real-time or near-real-time. This brings non-negligible burden to the UE in regard of computation and power consumption. It is more realistic to consider online training as an ‘optional capability’ for a UE supporting AI/ML-based approach.

- Capability of data collection for model training

At the initial phase, AI/ML model may be developed in offline manner, in which the training data is collected in advance. It is desired that a deployed AI/ML model can be updated/fine-tuned based on fresh data. It will be very beneficial if a UE supporting AI/ML-based approach has the capability of data collection, including, e.g. measurement, pro-/post-possessing, storage and reporting.

- Capability of implementing downloaded AI/ML model (i.e. collaboration Level z)

Due to the higher requirement storage and computation, AI/ML-based approach may need more hardware optimization than usual. If the downloaded model is based on some MRF rather than runtime image, it needs to be compiled before loaded into hardware. It is possible that a UE can only supports a proprietary model, but not the one downloaded from the network, even if the sizes and computation power between them are similar. Still, implementing downloaded AI/ML model from network may be important in some use cases, e.g. the network transfers an AI/ML-based encoder for CSI feedback to the UE to implement.

Although it may be a little too early to consider UE capability for now, we should keep it in mind throughout this study to strive for a practical AI/ML-based approach.

Proposal 23: For support of AI/ML, consider defining several levels of UE capabilities based on one or more following aspects:

- Hardware related capability

  - Storage,

  - Computation power,

- LCM related capability

  - Capability of online training,

  - Capability of data collection for model training,

  - Capability of implementing downloaded AI/ML model (i.e. collaboration Level z).

LS to RAN2

It was proposed to send LS to RAN2 (and possibly SA2) to give them a rough idea of model sizes considered in RAN1 evaluations. Our input is provided below.

- The following table lists the model sizes for the UE-side part (i.e., CSI generation part) of CSI compression models.

| Company | Sub-use-case | Model description | Model size (#Number of model parameters, e.g., number of weights in a neural network ) | Model size (Mbytes) |
| --- | --- | --- | --- | --- |
| CATT | CSI compression | The model is designed based on transformer model. | 2.51 M | 10 Mbytes |

- The following table lists the model sizes for the UE-side CSI prediction models.

| Company | Sub-use-case | Model description | Model size (#Number of model parameters, e.g., number of weights in a neural network ) | Model size (Mbytes) |
| --- | --- | --- | --- | --- |
| CATT | CSI prediction | The model is designed based on Conv-LSTM model. | 41 k | 0.16 Mbytes |

- The following table lists the model sizes for the UE-side spatial beam prediction (BM-Case1) models.

| Company | Sub-use-case | Model description | Model size (#Number of model parameters, e.g., number of weights in a neural network ) | Model size (Mbytes) |
| --- | --- | --- | --- | --- |
| CATT | BM-Case1 | The model is designed based on ResNet18 model. | 206 k | 0.8 Mbytes |

- The following table lists the model sizes for the UE-side temporal beam prediction (BM-Case2) models.

| Company | Sub-use-case | Model description | Model size (#Number of model parameters, e.g., number of weights in a neural network ) | Model size (Mbytes) |
| --- | --- | --- | --- | --- |
| CATT | BM-Case2 | The model is designed based on Conv-LSTM model. | 87 k | 0.35 Mbytes |

- The following table lists the model sizes for the UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning (Positioning Case 1)

| Company | Sub-use-case | Model description | Model size (#Number of model parameters, e.g., number of weights in a neural network ) | Model size (Mbytes) |
| --- | --- | --- | --- | --- |
| CATT | Positioning Case 1 | For direct AI/ML positioning, the model is designed based on ResNet18 model. | 11.2 M | 44 Mbytes |
| CATT | Positioning Case 1 | For AI/ML assisted positioning, ToA is estimated. The model is designed based on ResNet18 model. | 0.7 M | 2.8 Mbytes |

- The following table lists the model sizes for the UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning (Positioning Case 2a)

| Company | Sub-use-case | Model description | Model size (#Number of model parameters, e.g., number of weights in a neural network ) | Model size (Mbytes) |
| --- | --- | --- | --- | --- |
| CATT | Positioning Case 2a | For AI/ML assisted positioning, ToA is estimated. The model is designed based on ResNet18 model. | 0.7 M | 2.8 Mbytes |
| CATT | Positioning Case 2a | For AI/ML assisted positioning, LOS/NLOS is identified. The model is designed based on ResNet18 model. | 0.7 M | 2.8 Mbytes |

Conclusion

In this contribution, we provide our views on general aspects of AI/ML framework for NR air interface. The observations and proposals are summarized as follows:

Observation 1: From UE’s perspective,

- If a UE only supports functionality identification, collaboration level y can be enabled. Partial standardized LCM procedure within 3GPP network can be supported, e.g. AI/ML function enabling, fallback to non-AI/ML based approach, performance monitoring.

- If a UE supports model identification but not model transfer, collaboration level y can be enabled. Most standardized LCM procedures within 3GPP network can be supported, e.g. AI/ML model activation/deactivation, model selection, model switching, model monitoring, fallback.

- If a UE supports model identification and model transfer, collaboration level z can be enabled. All standardized LCM procedures can be supported.

Observation 2: Training complexity can be reflected by model complexity.

Observation 3: RAN1 may need to provide analysis/conclusion on complexity, performance and spec impact for both offline and online training at the end of SI. Parallel study on both approaches is necessary.

Observation 4: It is more usual to perform model transfer from network to UE.

Proposal 1: The definition of ‘AI/ML model transfer’ is revised (marked in red) as follows:

| AI/ML model transfer | Delivery of an AI/ML model over the air interface via 3GPP signaling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| --- | --- |

Proposal 2: Capture the definition of model selection in the working list of terminologies.

| Model selection | Selection of an AI/ML model for activation among multiple models for the same functionality. Note: Model selection may or may not be carried out simultaneously with model activation. |
| --- | --- |

Proposal 3: Compared to RAN3 framework, life cycle management becomes much more complicated and should be considered carefully in RAN1 framework.

Proposal 4: Wait until sufficient progress is made on LCM before deciding how to capture it into functional framework.

Proposal 5: For comparison between model generalization, model switching and model update, further study these approaches in each use case. No need to draw conclusion/prioritization before sufficient investigation.

Proposal 6: At least for offline training, companies can voluntarily share their training strategies, but no need to set up training complexity KPIs for comparison.

Proposal 7: It is more proper to discuss inference latency under UE/network capability, rather than common KPI for AI/ML model.

Proposal 8: Power consumption is not considered as a common KPI for AI/ML-based approach.

- It is up to companies’ interest to report their results of power consumption.

Proposal 9: For functionality identification, the granularity of functionality should be at least sub use case level. Whether finer granularity is needed can be FFS.

Proposal 10: The functionality of an AI/ML model can be identified by model identification. Whether functionality identification is the prerequisite of model identification can be left to normative phase.

Proposal 11: For model identification, the following information can be considered as the starting point for model description information when provided from UE to NW:

- Information on model functionality,

- Information on model input/model normative input,

- Information on model output/model normative output,

- Information on assistance information for inference,

- Information on model performance,

- Information on concurrent use with other AI/ML models and/or non-AI/ML features,

- Information on applicable conditions,

- Information on pairing information for two-sided model.

Proposal 12: Model description information for model identification can be reused for model transfer.

Proposal 13: For an identified AI/ML model, further study the following aspects on the model ID:

- Whether the model ID is reported by UE or assigned by network,

- Whether the model ID is global or not,

- Whether the model ID is explicit or implicit.

Proposal 14: For model monitoring, further study how to monitor the performance of inactive AI/ML models for better LCM.

Proposal 15: For model selection, activation, deactivation, switching and fallback for single UE sided models, if the UE makes decision autonomously without reporting to network, the case belongs to collaboration Level x and has no specification impact.

Proposal 16: For model selection, activation, deactivation, switching, and fallback for UE part of two-sided models, network makes decisions and informs to UE. It belongs to collaboration Level y or Level z.

Proposal 17: For model selection, activation, deactivation, switching, and fallback for single network sided models, network makes decisions and may or may not inform to UE:

- If the decision is not informed to UE, it belongs to collaboration Level x,

- If the decision is informed to UE, it belongs to collaboration Level y or Level z.

Proposal 18: For model selection, activation, deactivation, switching, and fallback for network part of two-sided models, network makes decisions and may or may not inform UE. It belongs to collaboration Level y or Level z.

- FFS whether/when network’s decision needs to be informed to UE.

Proposal 19: For model selection, activation, deactivation, switching, and fallback, prioritize the mechanism where the decision is made by network.

Proposal 20: For model transfer, the following aspects can be further studied in RAN1:

- Full or partial model transfer.

- Periodicity/trigger.

- Latency and reliability requirement.

- Model delivery format.

Proposal 21: Further study proprietary format and open format in a parallel way.

Proposal 22: Model description information should be provided during model transfer, regardless proprietary format or open format is used.

Proposal 23: For support of AI/ML, consider defining several levels of UE capabilities based on one or more following aspects:

- Hardware related capability

  - Storage,

  - Computation power,

- LCM related capability

  - Capability of online training,

  - Capability of data collection for model training,

  - Capability of implementing downloaded AI/ML model (i.e. collaboration Level z).

References

- TR 37.817, Study on enhancement for Data Collection for NR and EN-DC, v17.0.0.

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#94-e, e-Meeting, December 6th – 17rd, 2021.

- Chairman’s Notes RAN1#109-e, RAN1#109-e, e-Meeting, May 9th - 20th, 2022.

- Chairman’s Notes RAN1#110, RAN1#110, Toulouse, France, September 22nd - 26th, 2022.

- Chairman’s Notes RAN1#110bis-e, RAN1#110bis-e, e-Meeting, October 10th - 19th, 2022.

- Chairman’s Notes RAN1#111, RAN1#111, Toulouse, France, November 14th - 18th, 2022.

- R1-2213003, Summary#1 of General Aspects of AI/ML Framework, Moderator (Qualcomm), RAN1#111, Toulouse, France, November 14th - 18th, 2022.