3GPP TSG RAN WG1 #112		                                                     R1-2300680

Athens, Greece, February 27th – March 3rd, 2023

Source:	CATT, GOHIGH

Title:	Discussion on channel access mechanism for sidelink on unlicensed spectrum

Agenda Item:	9.4.1.1

Document for:	Discussion and Decision

Introduction

In RAN#94-e meeting, a new work item NR sidelink evolution was approved and further revision was updated in [1]. The objective of this work item is to satisfy identified requirements of commercial sidelink applications, including increased sidelink data rate and support of new carrier frequencies for sidelink. One of the main objective is to study and specify sidelink operation on unlicensed spectrum.

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102). - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

In this contribution, issues related to channel access mechanism for sidelink on unlicensed spectrum are discussed.

Channel access mechanism

In NR sidelink, four sidelink physical channels are defined, including PSCCH, PSSCH, PSFCH and PSBCH. According to the WID[1], these physical channels shall be reused as the baseline in SL-U. However, due to the requirements such as regulation limitation, some changes of sidelink physical channel structures may be needed in SL-U. The detailed design of physical channel structures can be found in our contribution[2].

In this section, the following channel access procedures for dynamic channel occupancy are discussed:

- Channel access priority class (CAPC) for SL

- Contention window adjustment procedures

- Channel access procedures for S-SSB and PSFCH transmission

- CP extension (CPE)

- Remaining issues on Type 2 channel access procedures

Channel access priority class (CAPC) for SL

In RAN1 #110b-e meeting, the following agreement was achieved regarding CAPC for SL[3]:

Agreement - In Type 1 SL channel access procedure, the following table is adopted for channel access priority class (CAPC) for SL. - FFS: the applicability and usage of NOTE1 in the table - FFS: whether mp=1 can be used with p=1, and applicable cases Channel Access Priority Class (p); mp; CWmin,p; CWmax,p; Tslmcot,p; allowed CWp sizes. 1; 2; 3; 7; 2 ms; {3,7}. 2; 2; 7; 15; 4 ms; {7,15}. 3; 3; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. 4; 7; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. [NOTE1: Forp=3,4, Tslmcot,p=10ms if the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided, otherwise,Tslmcot,p=6ms.] NOTE 2: When Tslmcot,p=6ms it may be increased to 8ms by inserting one or more gaps. The minimum duration of a gap shall be 100μs. The maximum duration before including any such gap shall be 6ms..

Although it is concluded that UL CAPC table from NR-U is adopted for SL-U, there are still some remaining issues that need to be further discussed.

The first issue is whether Note 1 in the table is applicable to SL-U. In NR-U, Tslmcot,p=10ms is used for p=3,4, if the environment can be guaranteed to be absent of other technologies. While in SL-U, it is unclear whether this environment is valid since sidelink transmission at least may be coexistent with Uu transmission. And if it is not a valid environment in SL-U, only Tslmcot,p=6ms should be used for p=3,4.

Proposal 1: RAN1 to discuss whether absence of other technologies is a valid environment in SL-U.

Another issue is whether mp=1 can be used with p=1 for such as S-SSB and PSFCH transmission. Considering that Type 1 channel access duration is highly depending on the value of mp, the smaller the value of mp adopts, the easier the Type 1 channel access is to succeed. Therefore, for S-SSB and PSFCH transmission whose transmission occasion is determined, the minimum value of mp, i.e. mp=1, can be used with p=1.

Proposal 2: Regarding CAPC for SL, mp=1 can be additionally used with p=1 for such as S-SSB and PSFCH transmission.

In RAN2#120 meeting, the following agreement was already achieved[4]. A corresponding LS is sent to RAN1 in[5].

Agreements on SL CAPC for SBCCH and PSFCH

1: 	The highest priority SL CAPC is used for SBCCH SDU transmission (if SL CAPC is applied to SBCCH SDU).

2:	SL CAPC for PSFCH is left to RAN1.

Considering that CAPC value of SBCCH SDU transmission is the same as S-SSB transmission, p = 1 should also be used for S-SSB transmission in order to follow RAN2’s agreement.

Proposal 3: Regarding CAPC for SL, p=1 is always used for S-SSB transmission.

Regarding SL CAPC for PSFCH, RAN2’s agreement is to leave the decision to RAN1. While in RAN1, the following proposal was discussed in the last meeting [6], but no consensus was achieved.

Proposal 2-2 (II): - For PSFCH transmission in SL-U, one of the following options is supported for the CAPC level (p) - Option 1: Any CAPC level can be used (up to UE implementation) - Option 2: CAPC level (p=1) is always used - Option 3: Use same CAPC level as the associated PSSCH – same as in R16 for selecting PSFCH(s) to be transmitted due to limited UE capability - Note, the CAPC level should be indicated in SCI - FFS when UE transmit multiple PFSCH corresponding to different PSSCHs with different CAPC levels - FFS when LBT sensing time longer than one GP symbol

The value of CAPC level has an impact on the channel access duration of a UE’s transmission, which actually has no related to the content of the transmission. In the case, using the same CAPC level as PSSCH transmission for the associated PSFCH seems unnecessary. Considering that PSFCH is some kind of control information, NR-U principle of PUCCH should be reused. That is, CAPC level p=1 is always used for PSFCH transmission. This is also beneficial for quick PSFCH transmission and resolve the issue of TX/RX misalign accordingly.

Proposal 4: Regarding CAPC for SL, p=1 is always used for PSFCH transmission.

Contention window adjustment procedures

In RAN1#111 meeting, the following agreement was achieved regarding the definition of “SL reference duration” [7]:

Agreement SL reference duration is defined as a duration corresponding to a channel occupancy initiated by the UE including transmission of PSSCH(s), starting from the beginning of the channel occupancy initiated by the UE including transmission of PSSCH(s), until either (one option to be selected later): - Option 1a: - the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted - Note, SL reference duration is not used if PSSCH with ACK/NACK HARQ-ACK enabled cannot be found in the latest COT - FFS: Whether to support another ending timing is FFS, e.g for MCSt if needed - Option 1b: - the end of the first slot where at least one PSSCH with HARQ-ACK enabled is transmitted - Note, SL reference duration is not used if PSSCH with HARQ-ACK enabled cannot be found in the latest COT - FFS: Whether to support another ending timing is FFS, e.g for MCSt if needed - Option 2a: - the end of the first slot where at least one PSSCH with HARQ-ACK enabled if it is transmitted, otherwise until the end of the channel occupancy - FFS: Whether to support another ending timing is FFS, e.g for MCSt if needed - Option 2b: - the end of the first slot where at least one PSSCH with HARQ-ACK enabled if it is transmitted, otherwise until the time when UE updates the CW - FFS: Whether to support another ending timing is FFS, e.g for MCSt if needed Agreement - If CWp=CWmax,p, the next higher allowed value for adjusting CWp is CWmax,p. - If the CWp=CWmax,p is consecutively used K times for generation of Ninit, CWp is reset to CWmin,p only for that priority class p for which CWp=CWmax,p is consecutively used K times for generation of Ninit. K is selected by UE from the set of values {1, 2, …,8} for each priority class p∈1,2,3,4.

Furthermore, the following proposal was discussed in the last meeting regarding the detailed contention window adjustment procedures[6], but no consensus was achieved.

Proposal 3-2 (II): Contention window adjustment procedures (to further down-select between options in each case): - SL HARQ-ACK feedback disabled in SCI (i.e., all cast types) - Option 1: For every priority class p∈1,2,3,4, use the latest CWp. - Option 2: CW is adjusted according to number blind retransmissions of the TBs within a COT. - Option 3: CW is adjusted according to CR/CBR measurement, if CR/CBR is supported in SL-U. - Only unicast (ACK and NACK) within SL reference duration - Option 2: If at least one ‘ACK’ is received, CWp=CWmin,p for each priority class p∈1,2,3,4; otherwise, increase. - Only groupcast option 2 (ACK and NACK) within SL reference duration - Option 1: Based on a (pre-)configurable ratio of received SL HARQ-ACK feedbacks. - Option 2: If at least one ‘ACK’ is received, CWp=CWmin,p for each priority class p∈1,2,3,4; otherwise, increase. - FFS whether the at least one ‘ACK’ is from just one UE or every UE - Only groupcast option 1 (NACK-only) within SL reference duration - Option 1: For every priority class p∈1,2,3,4, use the latest CWp. - Option 2: If ‘NACK’ or a collision indicator (IUC scheme 2) is received, increase CWp; Otherwise, CWp=CWmin,p or use the latest CWp (FFS which). - Option 6: GC option 1 (NACK-only) is not supported in SL-U - FFS: the case when SL HARQ-ACK feedback is not available after the last update of CWp.

According to the agreements and FL proposal, the following issues still need to be discussed:

- The definition of a “SL reference duration”

- CW adjustment when SL-HARQ feedback is disabled

- CW adjustment for groupcast option 2 with SL-HARQ feedback enabled

- Whether groupcast option 1 (NACK-only) can be supported for SL-U

Issue 1: The definition of a “SL reference duration”

In the last meeting, it has been agreed that SL reference duration starts from the beginning of the channel occupancy initiated by the UE including transmission of PSSCH(s), but the ending time is still requiring further down-selection. In NR-U, the ending time is defined as the end of the first slot where at least one unicast PDSCH is transmitted over all the resources allocated for the PDSCH, or the end of the first transmission burst by the gNB that contains unicast PDSCH(s) transmitted over all the resources allocated for the PDSCH, whichever occurs earlier[8]. A similar definition should be considered in SL-U. Since it is still unclear how to support NACK-only HARQ feedback in SL-U, Option 1a is preferred for the definition of ending time of SL reference duration.

Proposal 5: Support Option 1a for the definition of ending time of SL reference duration.

- Option 1a: the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted

Multi-consecutive slots transmission (MCSt), which can be considered as SL transmission burst, is supported in SL-U. Following NR-U principle, the ending time of the first transmission burst where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted should also be considered as another ending time of SL reference duration.

Proposal 6: The ending time of the first transmission burst, where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted, should also be considered as another ending time of SL reference duration.

Issue 2: CW adjustment when SL-HARQ feedback is disabled

For the case when all transmissions within the latest SL reference duration have SL-HARQ feedback disabled, no available HARQ feedback information can be used. In this case, the NR-U principle should be reused. That is, for every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p.

Proposal 7: When all transmissions within the latest SL reference duration have SL-HARQ feedback disabled, Option 1 in RAN1#111 meeting is supported.

- Option 1: For every priority class p∈1,2,3,4, use the latest CWp.

Issue 3: CW adjustment for groupcast option 2 with SL-HARQ feedback enabled

According to the NR-U spec, available HARQ-ACK feedback(s) corresponding to PDSCH(s) in the reference duration for the latest DL channel occupancy is used for CW adjustment. If at least one HARQ-ACK feedback is 'ACK' for PDSCH(s) with transport block based feedback, CWp is reset to CWmin,p for every priority class p∈1,2,3,4. Otherwise, CWp is increased to the next higher allowed value for every priority class. While in sidelink groupcast option 2, only one HARQ-ACK feedback is “ACK” cannot represent that the situation of channel occupancy is ideal, resetting CWp to CWmin,p is thus unreasonable. Therefore, a (pre-)configured ratio should be introduced for groupcast option 2 and both ACK feedback and NACK feedback should be considered.

Proposal 8: In case only groupcast option 2 PSSCH(s) is (are) transmitted within the latest SL reference duration, Option 1 in RAN1#111 meeting is supported with the consideration of both ACK feedback and NACK feedback.

- Option 1: Based on a (pre-)configurable ratio of received SL HARQ-ACK feedbacks.

Issue 4: Whether groupcast option 1 (NACK-only) can be supported for SL-U

In SL-U, transmission loss may occur due to the competition with other systems. If HARQ NACK-based feedback is applied, the Tx UE cannot distinguish whether the Rx UE successfully receives the data or fails to send NACK. In case that the NACK was lost or delayed due to channel access failure, the Tx UE may assume that the data was successfully received and will no longer send retransmission(s), which will lead to reliability decrease. Therefore, in order to guarantee the performance of sidelink transmissions in unlicensed spectrum, NACK-based feedback is not supported for SL-U.

Proposal 9: Groupcast option 1, i.e. NACK-based HARQ feedback, is not supported for SL-U.

Channel access procedure for S-SSB and PSFCH transmission

In RAN1 #111 meeting, the following agreement was achieved regarding channel access mechanism for S-SSB and PSFCH [7]:

Agreement - Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met: - Time duration is at most 1ms per transmission - The duty cycle of the S-SSB transmissions is at most 1/20 - FFS: details of EDT - FFS: whether/how to define observation period, including whether or not observation period would be captured in the specifications if defined - FFS: Type 2A applicability for PSFCH without a shared channel occupancy and further limitations for combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure

Although it was agreed that Type 2A channel access is applicable for S-SSB transmissions under the condition which is similar to the condition adopted in NR-U for transmissions with discovery burst, there are still some issues that need to be discussed on this topic.

The first issue is whether/how to define observation period. This came from the consideration of duty cycle ambiguity, that is, how to determine the time interval when evaluating the duty cycle restriction. If the time interval is defined as S-SSB period, i.e., 160ms, S-SSB transmission(s) will always occur periodically within the time interval. In this case, the duty cycle can be calculated as (S-SSB transmission(s) within a S-SSB period) divide (S-SSB period). If the time interval is defined as another determined value (e.g. 10ms), the calculation of duty cycle may be impacted by the distribution of S-SSB occasions. When the S-SSB occasions are concentrated within a certain time interval, the duty cycle requirement may not be met in that time interval, but can be met in another time interval. Therefore, in order to simplify the calculation of duty cycle, it is proposed to define the time interval of duty cycle as S-SSB period.

Proposal 10: When Type 2A channel access procedure is applied to S-SSB transmission as discovery burst in NR-U, the total S-SSB transmission duration per UE is at most 1ms and the duty cycle is at most 1/20 with the time interval of duty cycle defined as S-SSB period, i.e., 160ms.

Another issue is about channel access procedure for PSFCH transmission. In NR sidelink, if PSFCH resource is configured in the resource pool, the Rx UE can transmit feedback information on the next available PSFCH resource after receiving data. PSFCH resources are always configured periodically, with the period of 1, 2, or 4. Before each PSFCH resource, there is a guard symbol, whose length is based on the subcarrier spacing. Therefore, Type 2 channel access procedures can be performed before PSFCH transmissions within a COT based on the length of guard symbol.

Proposal 11: Within a shared channel occupancy, Type 2 channel access procedures are applicable to PSFCH transmission in SL-U.

When there is no available COT that can be used for PSFCH transmission, PSFCH may also be transmitted as discovery burst in NR-U. The time interval for evaluating the duty cycle should be the same as S-SSB transmission, i.e., 160ms. Whether the duty cycle of 1/20 can be satisfied is based on the PSFCH period and the frequency of PSFCH transmission.

Proposal 12: Type 2A channel access procedure is applicable for PSFCH transmissions for a UE without a shared channel occupancy, with the total PSFCH transmission duration per UE is at most 1ms and the duty cycle is at most 1/20 with the time interval of duty cycle defined as S-SSB period, i.e., 160ms.

The last issue is whether to introduce short control signaling transmission (SCSt) in SL-U. This issue has been discussed since the first RAN1 meeting of Rel-18. However, due to different ideas of SCSt, the group hasn’t reached a consensus on this issue. The most controversial point is the difference between transmission with discovery burst (using Type 2A channel access) and short control signaling transmission.

Type 2A channel access procedures, which is defined in NR-U, is mainly applicable to the transmission(s) within a COT by sensing the channel to be idle for at least a sensing interval [8]. Short control signaling is adopted in European regulation [9] as well as Chinese regulation [10].  SCSt is defined as transmissions used by the equipment to send management and control signals without sensing the channel for the presence of other signals. That is to say, even if the channel is sensed to be busy, SCSt can also be adopted.

Things are also different regarding the limitations of Type 2A channel access and SCSt. For discovery burst transmitted by a gNB, the transmission(s) duration is restricted to be at most 1ms, and the duty cycle is at most 1/20. For SCSt, the following limitations are applied in European regulation:

- within an observation period of 50 ms, the number of SCS transmissions by the equipment shall be equal to or less than 50; and

- the total duration of the equipment's SCS transmissions shall be less than 2 500 µs within said observation period.

While in China, the following limitations are applied:

- within an observation period of 50 ms, the number of SCS transmissions by the equipment shall be equal to or less than 50; and

- the total duration of the equipment's SCS transmissions shall be less than 2 500 µs within said observation period, or the duty cycle shall be no more than 1/10.

It can be observed that the definition and limitations are both different between Type 2A channel access and SCSt. Therefore, these channel access procedures should be discussed separately for S-SSB and PSFCH transmission.

Observation 1: The definition and limitations are both different between Type 2A channel access and SCSt.

Proposal 13: Type 2A channel access procedures and SCSt should be discussed separately for S-SSB and PSFCH transmission.

Considering that both S-SSB and PSFCH transmission can be identified as some kind of management and control information, short control signaling can also be used for S-SSB and PSFCH transmissions. Furthermore, since different countries may have different regulation requirements, the supportive of SCS transmission shall meet each country’s regulation.

Proposal 14: UE may perform S-SSB or PSFCH transmission as SCSt without sensing the channel when the transmission meets the regulation for SCSt in each country.

CP extension (CPE)

In RAN1 #111 meeting, the following agreement was achieved regarding CPE for SL-U [7]:

Agreement - A CPE is transmitted from a CPE starting position before SL transmission within a COT, select one or both of the two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: within at most 1, 2 or 4 symbols just before the next AGC symbol for 15, 30 or 60 kHz SCS, respectively - FFS: whether Option 1 and Option 2 are both applicable and the conditions (e.g., Option 1 in case of COT sharing and Option 2 in case of initiating a COT) - FFS: which channel access type(s) is applicable for option 1 and option 2 - FFS: other details - A single CPE starting position for PSFCH - FFS CPE starting position and whether it should be (pre-)configured in each RP, pre-defined or indicated - FFS other details (e.g., indication granularity) - Note: value 0 is a candidate - At least one CPE starting position for S-SSB - FFS CPE starting position should be (pre-)configured, pre-defined or indicated - FFS: Whether multiple CPE starting positions should be (pre-)configured, pre-defined or indicated - FFS CPE starting positions for the R16 S-SSB and the additional S-SSBs - Note: value 0 is a candidate - One or multiple CPE starting positions can be (pre-)configured in each resource pool for PSSCH/PSCCH - When multiple CPE starting positions are (pre-)configured, - FFS whether/how to define a criteria for selecting a default CPE starting position (e.g., according to partial/full RB set allocation, resource reservation information, within or outside of a COT, etc.) - FFS criteria for selecting one of the multiple CPE starting positions (e.g., according to priority level (e.g., CAPC or L1), selected randomly by UE from the (pre-)configured set of CPEs, selected by the UE based on channel access result, determined based on indication from the COT initiating UE, etc.) - FFS other details

According to the proposal, the following issues still need to be considered:

- Down selection of CPE starting position

- CPE starting position for PSFCH

- CPE starting position for S-SSB

- CPE starting position for PSCCH/PSSCH

Issue 1: Down selection of CPE starting position

In NR-U, channel access type and CP extension are combined indicated by DCI according to pre-configuration or higher layer configuration. The length of CPE is also related to the subcarrier spacing and channel access duration as follows[11]:

Text=k=12μTsymb,  l-kmod 7∙2μ μ-Δi

In NR SL, slot-level transmissions are performed. A guard symbol defined in each slot is for the purpose such as TX/RX switching. In order to prevent other RATs from accessing the channel on unlicensed spectrum, CPE can be used to fill the guard symbol in each slot at least for transmissions within a COT, as shown in Figure 1. That is, the stating position of each CPE can be within the symbol just before the next AGC symbol, i.e., within the guard symbol.

For transmissions without a COT, sidelink transmissions may not consecutive. In this case, CPE starting position can also be pre-configured taking NR-U principle into consideration. That is, a CPE starting position can be within at most 1, 2 or 4 symbols just before the next AGC symbol for 15, 30 or 60 kHz SCS, respectively.

Figure 1: CPE starting position for transmissions within a COT

Proposal 15: For transmissions within a COT, CPE starting position can be within the guard symbol, i.e., within the symbol just before the next AGC symbol.

Proposal 16: For transmissions without a COT, NR-U principle can be considered as baseline, i.e., CPE starting position can be within at most 1, 2 or 4 symbols just before the next AGC symbol for 15, 30 or 60 kHz SCS, respectively.

Issue 2: CPE starting position for PSFCH

According to the design in NR SL, PSFCH symbols are determined by resource pool configuration. That is, PSFCH transmission occasions are determined in each resource pool. Therefore, the CPE starting position of PSFCH should also be (pre-)configured in each RP. Considering that there is also a guard symbol before PSFCH symbol, CPE starting position can be within the guard symbol, i.e., within the symbol just before the next PSFCH symbol.

Proposal 17: The CPE starting position for PSFCH should be (pre-)configured in each RP within the symbol just before the next PSFCH symbol.

Issue 3: CPE starting position for S-SSB

In NR SL, S-SSB resource set within each S-SSB period is pre-configured. Each UE who is triggered to transmit S-SSB should perform S-SSB transmission on the determined resource within a S-SSB resource set. This principle should be reused for S-SSB transmission in SL-U. Each UE who has S-SSB to transmit should not be blocked by other UE’s CPE. That is, a single CPE starting position should be adopted for S-SSB transmission and the determined starting position should be (pre-)configured within the symbol just before the next AGC symbol.

Proposal 18: A single CPE starting position is adopted for S-SSB transmission. The CPE starting position should be (pre-)configured within the symbol just before the next AGC symbol.

Issue 4: CPE starting position for PSCCH/PSSCH

In the last meeting, it has been agreed that one or multiple CPE starting positions can be (pre-)configured in each resource pool for PSSCH/PSCCH. However, how to select one CPE when multiple CPE starting positions are pre-configured is still under discussion. Some companies propose to determine the CPE starting position according to priority level, in order to guarantee the transmission reliability of high priority traffic. For example, set the CPE starting position of the low priority traffic later than the high priority traffic. In this case, high priority traffic can not be blocked by the low priority traffic, since it can occupy the channel earlier by transmitting longer CPE. This consideration is based on an assumption that different sidelink transmissions can not be performed in the same slot which against the principle of NR sidelink design. In fact, NR sidelink has already considered the transmission mechanism of different priority traffics, i.e., pre-emption mechanism. The reserved resources should be evaluated before transmission. If it is pre-empted by a higher priority traffic, resource reselection will be triggered. If it is not pre-empted, it can be transmitted FDMed with a higher priority traffic even if the reserved resource is at the same slot with the high priority traffic. Therefore, the existing mechanism can already guarantee the transmission reliability of high priority traffic and no further optimization is required regarding the impact of priority level on CPE starting positions.

Proposal 19: No further optimization is required regarding the impact of priority level on CPE starting positions.

Except considering the priority level, there is no reason to select different CPE starting positions before the same slot. As shown in Figure 2, UE 1 and UE 2 reserve resources at the same slot. After re-evaluation/pre-emption checking, these resource remains reserved. Then, both of the transmissions can be transmitted FDMed at the slot. In this case, if different CPE starting position is adopted, one of them may not be transmitted due to LBT failure. Therefore, even if multiple CPE starting positons are pre-configured in the resource pool, a single CPE starting position should be adopted before transmissions at the same slot. Similar as NR-U, the determined CPE starting position can be indicated by the SCI from the COT initiating UE.

Figure 2: A single CPE starting position before transmissions at the same slot

Proposal 20: For PSCCH/PSSCH, a single CPE starting position should be adopted before transmissions at the same slot. The determined CPE starting position can be indicated by the SCI from the COT initiating UE.

Remaining issues on Type 2 channel access procedures

In RAN1 #110 meeting, the following agreement was achieved regarding Type 2 channel access procedures[13]:

Agreement - Type 2A/2B/2C SL channel access procedures - Type 2A channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE for a gap ≥ 25μs in a shared channel occupancy - FFS any other transmission by a UE (e.g., other than COT sharing) - FFS whether Type 2A is used also for the case of short control signalling transmission - Type 2B channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE at least when the gap is 16μs in a shared channel occupancy - FFS the case when the gap is between 16 and 25us - FFS any other transmission by a UE (e.g., other than COT sharing) - Type 2C channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE for a gap ≤ 16μs in a shared channel occupancy and the duration of the corresponding transmission is at most 584us. - FFS any other transmission by a UE (e.g., other than COT sharing) - FFS whether Type 2C is used also for the case of short control signalling transmission - FFS under which conditions (other than the gap) UEs can apply the Type 2A/2B/2C SL channel access procedures - FFS under which conditions Type 2B or Type 2C is applied in case of a gap of 16 μs

One remaining issue is that for the case when the gap is between 16 and 25us, whether Type 2B channel access can be used. In NR-U, Type 2B channel access procedure is performed by sensing the channel within a duration of Tf =16μs. After sensing the channel to be idle within Tf, the UE or the gNB may transmit the transmission immediately. For sidelink transmission within a shared channel occupancy, the transmission gap between two UEs cannot always equal to 16μs. Therefore, for the case when the transmission gap is between 16 and 25μs, Type 2B channel access procedure is also applicable by sensing the channel within a duration of 16μs.

Proposal 21: For the case when the transmission gap is between 16 and 25μs, Type 2B channel access procedure is also applicable.

Another issue is how to determine the starting time of Type 2 channel access. Type 2 channel access only requires tens of microseconds of channel sensing time, which is smaller than the duration of a symbol for some SCS configurations. Therefore, it can be performed in the previous slot of the selected PSCCH/PSSCH transmission resource. As shown in Figure 3, slot n is the slot that UE will perform PSCCH/PSSCH transmission, and the starting time of Type 2 channel access can be located in the last symbol of the previous slot n-1. Firstly, UE should perform Type 2 channel access which requires 16us or at least 25us channel sensing. Then, an RX/TX switching time should also be considered to guarantee that UE can transmit PSCCH/PSSCH at the boundary of slot n. After RX/TX switching, if there is still a gap, CPE can be transmitted until the starting time of the next AGC symbol. In a word, Type 2 channel access can start at the previous slot of PSCCH/PSSCH transmission with the considerations of Type 2 channel access duration and RX/TX switching.

Figure 3: Type 2 channel access starting at the previous slot of PSCCH/PSSCH transmission

Proposal 22: Type 2 channel access can be performed in the previous slot of the corresponding PSCCH/PSSCH transmission.

Proposal 23: The starting time of Type 2 channel access should take channel access duration and RX/TX switching time into account.

Sharing of channel occupancy time

In RAN1 #111 meeting, the following proposal was achieved regarding UE-to-UE COT sharing[7].

Agreement For UE-to-UE COT sharing, - When performing S-SSB transmission(s), a responding UE can utilize a COT shared by a COT initiating UE (using type 1 channel access) when the responding UE is intended to transmit S-SSB within RB set(s) corresponding to the shared COT. - When performing PSFCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE. - FFS: whether a responding UE can transmit PSFCH(s) to UE(s) other than the initiator - When performing PSSCH/PSCCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when the responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to the shared COT is intended for the COT initiating UE - FFS whether to support the case if a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information. - FFS: how to determine / what are the restrictions to the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) to utilize the COT shared by the initiating UE. - FFS whether the responding UE can utilize the COT when at least the responding UE’s PSCCH transmission in the reserved resources within the shared COT or MCSt is intended for the COT initiating UE and what are the restrictions (e.g., priority, etc.) and indication to the responding UE. - FFS: UE forwarding/relaying information about a COT initiated by another UE.

Based on the proposal, the following issues should be further studied:

- Further restrictions on responding UE’s transmission

- Contents of COT sharing information

- Container for COT sharing information

- Conditions for transmitting COT sharing information

- UE behavior after receiving COT sharing information

Issue 1: Further restrictions on responding UE’s transmission

In the last meeting, restrictions on responding UE’s transmission was fully discussed and preliminary agreement was achieved for UE performing S-SSB, PSFCH or PSCCH/PSSCH transmissions. However, there are still some remaining issues that need to be further discussed, such as further restrictions on responding UE’s transmission.

For PSFCH transmission, the period can be as long as four slots, which may be longer than the maximum COT sharing duration. That is, PSFCH transmission may not be transmitted within the COT that indicated by the COT initiating UE. In this case, allowing a responding UE to transmit PSFCH to the UE other than the initiator can provide more opportunity to the responding UE to transmit PSFCH. Otherwise, Type 1 channel access will be used which suffer even longer channel access duration.

Proposal 24: A responding UE is allowed to transmit PSFCH(s) to UE(s) other than the initiator.

For PSCCH/PSSCH transmission, allowing a responding UE to transmit PSSCH/PSCCH to destination other than the COT initiating UE is also beneficial to the whole COT sharing mechanism within the SL-U system. After a COT is initialized, it can be aware by any UE who has received the COT sharing information. There may be some UEs that have PSSCH/PSCCH transmissions towards the COT initiating UE, but the transmissions may not consecutive. Therefore, in order to maintain the COT, it is supported for a responding UE transmitting PSSCH/PSCCH to destination ID other than the source/destination ID of the COT initiating transmission.

Proposal 25: A responding UE can transmit PSSCH/PSCCH to destination ID other than the source/destination ID of the COT initiating transmission.

Issue 2: Contents of COT sharing information

In SL-U, a distributed COT sharing mechanism should be introduced where the COT initiating UE should indicate the remaining COT or the deadline of COT to inform the available channel occupancy time. Then, the COT shared UE can judge whether Type 2 channel access can be performed before its transmission based on the remaining COT or the deadline of COT. Besides, the index of LBT sub-bands where Type 1 channel access is performed should also be indicated to COT shared UE. And some other indispensable information such as source ID, destination ID, and the channel access priory should also be carried accordingly.

Proposal 26: COT sharing information should at least include the remaining COT, the sensed LBT sub-bands, source ID, destination ID, and the channel access priority.

Issue 3: Container for COT sharing information

The container for COT sharing information is related to the overhead of COT sharing information. Considering that only some ID information as well as other short information such as CAPC level and remaining COT duration is required, SCI can be the qualified container for transmitting COT sharing information. Furthermore, since source/destination ID information is carried by 2nd stage SCI, at least 2nd stage SCI should be considered for COT sharing information transmission.

Proposal 27: SCI is qualified container for transmitting COT sharing information and at least 2nd stage SCI should be considered.

Issue 4: Conditions for transmitting COT sharing information

This issue is to define the conditions under which UE can transmit COT sharing information, and the following principles should be considered:

- The first condition is that UE has data to transmit. Since COT sharing information should be carried by SCI, only when the UE has SCI to transmit, the COT sharing information can be transmitted. From the perspective of complexity reduction, standalone SCI should not be introduced in SL-U. That is, SCI can only be transmitted when UE has corresponding data information to transmit.

- Another condition is that the remaining COT is larger than a (pre-)configured threshold, so that more UEs can utilize COT to perform Type 2 channel access. Considering the maximum COT depends on the channel access priority, channel access priority could also be another metric, i.e., UE can perform COT sharing only when the channel access priority value is larger than a (pre-)configured value.

Proposal 28: The following conditions should be introduced under which UE can perform COT sharing:

- UE has data to transmit.

- The remaining COT is larger than a (pre-)configured threshold or the channel access priority value is larger than a (pre-)configured value.

Since standalone SCI is not supported in SL-U, when a UE performs Type 1 channel access for S-SSB or PSFCH transmission, a UE-to-UE COT sharing should not be started. Furthermore, S-SSB and PSFCH transmission occasions are determined by (pre-)configuration. If a certain transmission occasion is missed, the corresponding transmission may be dropped, leading to system performance decrease. Therefore, when there is no COT that can be used for S-SSB or PSFCH transmission, short control signaling is the preferred transmission method than Type 1 channel access. In a word, no matter whether Type 1 channel access is used or not for S-SSB or PSFCH transmission, UE-to-UE COT sharing started with S-SSB or PSFCH transmission is not supported.

Proposal 29: UE-to-UE COT sharing started with S-SSB or PSFCH transmission is not supported.

Issue 5: UE behavior after receiving COT sharing information

In NR-U, COT sharing information is indicated by the gNB, while each UE sharing the COT can be distributed in the COT uniformly. UE can perform transmission within TCO=Tmcot,p+Tg, where Tmcot,p is the maximum channel occupancy time based on the channel access priority class and Tg is the total duration of all gaps of duration greater than 25μs that occur between two transmissions within the COT. It can be observed that the COT sharing duration in NR-U actually is a relative time, and the absolute duration of the COT depends on how many gaps of duration greater than 25μs within the COT. The restriction of the absolute duration of the COT can be up to the regulation of each country, such as 20ms in Chinese regulation.

Observation 2: In NR-U, the COT sharing duration is a relative time, and the absolute duration of the COT depends on how many gaps of duration greater than 25μs are within the COT.

For sidelink UE without the coordination by the gNB, it should first identify the availability of COT after receiving COT sharing information. The method can be different considering the cast type of COT sharing information transmission. For unicast transmission, the COT will only be used by single destination UE, the unicast pair can use the COT as that in NR-U, i.e. the COT is calculated by the transmission duration of unicast pair as shown in Figure 4. And the restriction of the absolute duration of the COT can be up to the regulation of each country. The unicast pair can perform Type 2 channel access as long as the duration of transmission(s) is less than the received COT duration.

Figure 4: Calculation of channel occupancy time for unicast

Things are the same for connection-based groupcast. Since UEs within the same group have unified COT sharing information, the total transmission duration is also aware. In this case, the COT can be calculated as the transmission duration of the group.

For connectionless groupcast and broadcast, the COT can be shared by multiple UEs who may not aware of each other. Due to the impact of distributed resource allocation mechanism, the UE receiving the COT sharing information cannot know whether the shared COT is used by other UEs or not, and do not have sufficient information to determine the remaining available COT. One solution for this issue is to use the absolute time to determine the remaining available COT, i.e. the COT duration is calculated by the absolute duration from the starting occasion of COT sharing, as shown in Figure 5. The COT sharing UEs can perform Type 2 channel access within this absolute COT sharing duration.

Figure 5: Calculation of channel occupancy time for groupcast/broadcast

Proposal 30: The cast type should be considered for COT sharing operation:

- For unicast and connection-based groupcast, the COT sharing duration can be determined as that in NR-U, and the restriction of the absolute duration of the COT can be up to the regulation of each country.

- For connectionless groupcast and broadcast, the COT sharing ending time for all the COT sharing UEs is an absolute time, i.e. determined by the absolute duration from the starting occasion of COT sharing.

Multiple channel access procedure

In RAN1 #111 meeting, the following agreement was achieved regarding multiple channel access procedure[7]:

Agreement For dynamic channel access mode with multi-channel case in SL-U, use NR-U DL (Type A or Type B) multi-channel access procedure as the baseline for multiple PSFCH transmissions on multiple channels, where each PSFCH transmission is confined within one LBT channel - FFS: the case for S-SSB if agreed to transmit S-SSB (or S-SSB can be (pre-)configured) in more than one RB set - FFS: whether type A or type B or both will be supported for this case for PSFCH - FFS: whether multiple PSFCH transmissions on multiple channels after performing the multi-channel access procedure is limited to contiguous RB sets

According to the agreement, one remaining issue is whether type A or type B or both will be supported for DL channel access procedure for PSFCH.

In NR-U, if Type A multi-channel access is adopted, an eNB/gNB shall perform Type 1 channel access on each channel  ci∈C, where C is a set of channels on which the eNB/gNB intends to transmit, and i=0,1,…q-1, and q is the number of channels on which the eNB/gNB intends to transmit. If Type B multi-channel access is adopted, the eNB/gNB only perform Type 1channel access on channel cj, where cj is uniformly randomly selected from C before each transmission on multiple channels ci∈C, or selected no more frequently than once every 1 second. For each channel ci≠cj, ci∈C, the eNB/gNB shall sense the channel ci for at least a sensing interval Tmc=25μs immediately before transmitting on channel cj, and the eNB/gNB may transmit on channel ci immediately after sensing the channel ci to be idle for at least the sensing interval Tmc. The channel ci is considered to be idle for Tmc if the channel is sensed to be idle during all the time durations in which such idle sensing is performed on the channel cj in given interval Tmc. Therefore, it can be observed that NR-U Type B multi-channel access can provide quick transmission opportunity and transmissions on multiple channels may be performed at the same time.

Observation 3: NR-U Type B multi-channel access can provide quick transmission opportunity and transmissions on multiple channels may be performed at the same time.

For multiple PSFCH transmissions on multiple channels, each PSFCH transmission should be performed on the PSFCH symbol. UE can perform Type 1 channel access on a selected channel, while for other channels on which the UE intends to transmit PSFCH, only a sensing interval is required for the UE to sense. That is, following the principle of NR-U Type B channel access procedure. In this case, PSFCH transmissions on other channels can be performed without going through the whole Type 1 channel access procedure, saving additional power consumption for energy detection and improving the success probability of channel access as well.

Proposal 31: NR-U DL Type B multi-channel access procedure is supported for multiple PSFCH transmissions on multiple channels.

Another issue is multiple channel access for S-SSB transmissions. Considering that at most 11 RBs will be used for SLSS and PSBCH transmission in NR sidelink, one RB set is enough for the transmission of S-SSB. In order to minimum the complexity of synchronous search, multiple channel access procedure is not applicable to S-SSB transmission.

Proposal 32: Multiple channel access procedure is not applicable to S-SSB transmission.

Furthermore, for larger packets such as FTP model traffic, PSCCH/PSSCH transmissions may also need more than 20MHz bandwidth, i.e., the transmission resource involves multiple LBT sub-bands and multiple channel access procedure should be performed. In RAN1#110b-e meeting, the following agreement was achieved regarding multi-channel access[3]:

Agreement For dynamic channel access mode with multi-channel case in SL-U, NR-U UL channel access procedure is considered as baseline for transmission on multiple channels - FFS: whether transmission of PSFCH and/or S-SSB on a subset of RB sets is supported (using the NR-U DL channel access procedure as baseline) - FFS any necessary enhancement and modification for the SL-U operation

Considering that it is agreed to adopt NR-U UL multi-channel access procedure as baseline for transmission on multiple channels, the following issues should be further studied for PSCCH/PSSCH transmission based on the operations defined in TS 37.213[8]:

- How to identify initial contention window counter Ninit

According to the procedure of Type 1 channel access procedure, the first step is to identify Ninit according to the channel access priority which is randomly selected from [0, CWp]. For the case that one transmission occupied multiple LBT sub-bands, the same CWp should be identified for each sub-band since the same channel access priority should be adopted for a certain transmission. Then, the remaining issue is how to identify Ninit for different sub-bands. For example, Ninit is randomly selected from [0, CWp] for each Ninit, i.e., Ninit is identified independently for each sub-bands and the values may be different.

- How to perform COT sharing

If the transmission occurs on multiple channels, then UE should perform channel access on each channel. Since the channel occupation of different channels are different, the ending time of Type 1 channel access procedure may be different as shown in Figure 6. Then, the remaining issue is how to perform COT sharing. The candidate options are proposed as follows:

- Option 1: COT sharing is performed independently for each LBT sub-band. In this way, UE can immediately share the COT information of a channel when the LBT of corresponding channel is successful. And UE only shares COT sharing information of the successfully accessed channel.

- Option 2: COT sharing is performed jointly for all LBT sub-bands. In this way, UE can only share the COT sharing information after the channel access of all channels is successful. And UE shares COT sharing information of all channels jointly.

Figure 6: Different type 1 channel access ending time of two sub-bands

- The impact of half duplex

Channel access is a procedure that UE senses whether the detected energy is larger than threshold and can be treated as a reception operation. Considering the restrictions of half duplex, UE cannot continue channel access procedure when performing other transmissions including sidelink or uplink transmission. The raised issue is how to process channel access when performing other transmissions. For example, the channel can be treated as busy within UL transmission duration, UE continues performing Type 1 channel access procedure afterwards, and the value N is maintained as the same value before the transmission duration.

Proposal 33: For multiple channel access procedure, the following issues should be further studied:

- How to identify initial contention window counter Ninit

- How to perform COT sharing

- The impact of half duplex

The impact on resource allocation

In RAN1 #109-e meeting, the following agreement was achieved regarding resource allocation mechanisms for SL-U[12].

Agreement - The existing sidelink mode 1 RA including dynamic grant, Type 1 and Type 2 configured grants are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the allocated resource(s), in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213. - FFS whether/how mode 1 resource allocation selection procedure needs to be updated / enhanced due to shared spectrum channel access - The existing sidelink mode 2 RA schemes are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the selected and/or reserved resources, in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213. - FFS whether/how mode 2 resource selection procedure needs to be updated / enhanced due to shared spectrum channel access - FFS whether/how multi-consecutive slots transmission can be supported for NR sidelink operation in unlicensed spectrum, including the following aspects - channel access, resource allocation and PHY channel design - FFS whether/how enhancement is needed between the end of the LBT procedure and the start of the SL transmission to retain channel access - RAN1 to strive for a common solution for channel access for Mode 1 and Mode 2

In this section, the following issues relating to the impact of channel access procedures on resource allocation mechanisms are discussed:

- Multi-consecutive slots transmission (MCSt)

- Mode-2 resource allocation enhancements

Multi-consecutive slots transmission (MCSt)

In RAN1 #110 meeting, it is agreed to support multi-consecutive slots transmission (MCSt) in SL-U for Mode 1 and Mode 2 resource allocation schemes, in order to maintain a COT from being occupied by other systems. In RAN1 #110b-e meeting, the following agreement was further achieved regarding the detailed design of multi-consecutive slots transmission (MCSt) for SL-U[3].

Agreement On the support of MCSt operation in SL-U, following options are to be further studied and one or more of the following options will be selected in future meetings. - When L1 is triggered for reporting a subset of candidate resources for MCSt, - Option 1: Only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1 - Note, this is applicable for transmission of a single TB and multiple TBs - FFS: whether this is the same or different than Rel-16 - Option 2: one or multiple sets of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) are provided for the resource selection procedure in L1 - FFS: any further information needs to be provided to L1 for MCSt - When L1 reports a subset of candidate resources for MCSt, - Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time - FFS whether the set of single-slot resources within a candidate multi-slot resource can have different LsubCH sizes - Option B: L1 reports candidate single-slot resources in (SA) as in Rel-16 - It is up to the higher (MAC) layer to select a set of single-slot resources that are consecutive in logical slots - Option C: L1 reports consecutive single-slot candidate resources in SA - FFS whether the consecutive single-slot candidate resources can have different LsubCH sizes - FFS: any further information needs to be reported to MAC layer, provided to L1 or utilized for MCSt - FFS: whether/how to consider the additional LBT time in SL resource allocation

According to the agreement, there are still some remaining issues that need to be discussed regarding multi-consecutive slots transmission (MCSt) for SL-U.

The first is which information needs to be provided to L1 for MCSt when L1 is triggered for reporting a subset of candidate resources for MCSt. Since the current NR sidelink design has already support transmitting transmissions in addition to the parameters provided by the MAC layer, only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is required for the resource selection procedure in L1, i.e., reuse the same design as Rel-16.

Furthermore, when L1 is triggered for reporting a subset of candidate resources for MCSt, the length of consecutive slots should also be provided by the MAC layer for the purpose of initial candidate resource determination and corresponding resource exclusion.

In Rel-17, SL DRX is introduced considering the power saving requirement of some UEs. If one or multiple SL DRX(s) is configured in the RX UE, the MAC entity of the TX UE should indicate SL DRX active time of the RX UE to the physical layer when resource selection is triggered. Similarly, for MCSt, SL DRX active time of RX UE should also be provided by the MAC layer.

Proposal 34: When L1 is triggered for reporting a subset of candidate resources for MCSt, only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1. Additional information such as the length of consecutive slots and SL DRX active time should also be provided to L1.

For mode-2, the single-slot candidate resources may be discrete after the resource exclusion procedure, leading to no consecutive transmission resources can be selected. Therefore, resource exclusion enhancements are required in mode 2 resource allocation, such as supporting multi-slot candidate resources. Regarding the  LsubCH size, since the resource allocation method has never considered the multi-process transmission, different  LsubCH sizes within a candidate multi-slot resource should not be supported.

Proposal 35: When L1 reports a subset of candidate resources for MCSt, support Option A. The LsubCH size should be the same within a candidate multi-slot resource.

- Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time

Another issue for supporting MCSt is how to support HARQ feedback transmission. For broadcast, multi-consecutive slots transmission can be easily implemented by selecting consecutive resources in time domain during the resource selection procedure. In unicast and groupcast, HARQ feedback should also be considered in order to guarantee the reliability of sidelink transmission.

Proposal 36: The supportive of HARQ feedback in unicast and groupcast should be further considered for MCSt.

Mode-2 Resource allocation enhancements

For sidelink mode-2 resource allocation, the existing mechanism is applied for the scenario that UEs are working on dedicated spectrum, that is, sidelink UE can ensure that transmissions can be completed on reserved resources without considering the channel being occupied by other technologies. However, for the unlicensed spectrum, UE needs to perform channel access before transmission and can only occupy the channel continuously within a maximum channel occupancy time (MCOT). If an interruption occurs between transmissions, the channel may be occupied by other technologies and subsequent transmission(s) may be delayed or dropped. Therefore, considering the impact of channel access mechanism on sidelink mode-2 resource allocation mechanism, the following potential enhancements should be considered for resource allocation mechanism:

- The relationship between resource selection and channel access

- The number of selected resources for a TB

- Restrictions on the selected resources

- Combined sensing and LBT procedures

Issue 1: The relationship between resource selection and channel access

The first issue is to identify either resource selection procedure or LBT procedure should be performed firstly after triggering resource selection. From our understanding, UE should perform resource selection procedure to determine the corresponding PSCCH/PSSCH transmission resources and then perform LBT procedure. The reasons are provided as follows:

- Firstly, LBT should be carried out at identified resource(s). Otherwise, UE need to perform LBT for all LBT sub-bands, which will cause higher workload for channel access operation especially when multiple LBT sub-bands are configured, such as 100MHz bandwidth.

- Secondly, the sensing processing time (Tproc,0) and Tx processing time (Tproc,1) should also be considered. If UE starts to perform resource selection after the successful LBT procedure, other RAT can access and occupy the channel due to the gap of sensing and Tx processing time. Then, the previous LBT success will be useless.

Based on the above considerations, the starting time of channel access should be after the resource selection time of the PSCCH/PSSCH transmission.

Proposal 37: Considering the complexity and efficiency of SL-U channel access, it is preferred that UE should perform resource selection procedure first and then perform channel access procedure.

Issue 2: The number of selected resources for a TB

Type 1 channel access is a channel access mechanism based on contention window and random back-off, so the duration of Type 1 channel access is uncertain and may be longer, especially when the channels are busy. Thus, an issue that should be considered is that the selected resources may become unavailable because of Type 1 channel access. As shown in Figure 7, four resources are actually needed for the TB, and the first two resources cannot be used because they are located in the duration of Type 1 channel access. If no further enhancement for resource selection, i.e., four resources are selected, then only the two remaining resources can be used, which will reduce the probability of successful decoding for other UEs. So in order to avoid reducing the number of transmissions for a TB caused by Type 1 channel access, UE can select more resources than TB transmission actually needs, such as n+m, where n is the actual transmission number and m is the number of additional selected candidate resources. As shown in Figure 7, the two green blocks are two additional selected resources and UE can still perform four transmissions for this TB even though two resources cannot be used. Considering that the unavailable resources caused by Type 1 channel access and additional selected candidate resources which are not really used will not indicated in SCI, so it will not affect the sensing and resource exclusion operations for other UEs.

Figure 7: More resources than TB transmission actually needs are selected

Proposal 38: How to alleviate the resources unavailability caused by Type 1 channel access should be further studied, such as selecting more candidate transmission resources than configured TB transmission times.

Issue 3: Restrictions on the selected resources

In Rel-16/17, UE performs random selection from final available resource set with the limitation of the maximum gap that SCI can indicate and/or HARQ RTT. But for SL-U, UE should perform Type 1 channel access or Type 2 channel access before transmissions, so in order to reduce the operations of channel access, resource allocation mechanism can be enhanced from the perspective of both time domain and frequency domain.

Regarding time domain, besides consecutive slots transmission, another enhancement is considering received COT in resource selection procedure. Especially, the received COT can be used to identify resource selection window or be treated as the restrictions for resource selection. UE will select resources within the received remaining COT so that this UE only needs to perform Type 2 channel access whose duration is determinate and much smaller than Type 1 channel access.

Regarding frequency domain, UE can firstly select resources located within one sub-bands to increase the probability of successful channel access. And if UE cannot select resources successfully from any single sub-band, then it can select resources from the union of available resources of all sub-bands.

Proposal 39: The following enhancements for resource selection procedure can be further studied:

- Selecting resources with the limitation of COT in time domain

- Selecting resources within one sub-band in frequency domain

Issue 4: Combined sensing and LBT procedures

As mentioned before, resource selection procedure should be performed firstly and then channel access procedure can be performed. However, when channel access failure occurs, UE may not finish the transmission on the reserved resource. On the other hand, sensing based resource selection method, which was designed for the sidelink UE performing autonomous resource selection, should be reused in SL-U. In this case, UE may perform RSRP measurement as in sidelink mode-2 and energy detection as in NR-U at the same time. How to consider the relationship between sensing and LBT is an essential issue that should be discussed.

On the one hand, sensing and LBT procedures can be considered as independent mechanisms, where LBT is performed by detecting energy in one or multiple sensing slots before transmission and sensing is performed by decoding the SCI as well as measuring the SL-RSRP. In this case, energy detection result is always compared with the energy detection threshold no matter whether there are other sidelink UEs transmitting on the channel. That is, even though there are only sidelink UEs transmitting on unlicensed spectrum, channel access failure still may occur due to the undifferentiated detection of the sidelink UEs and other RAT UEs. Although this is a simple way to implement sensing and LBT, it may lead to reliability performance decrease or transmission delay and the sidelink FDM transmission design is thus invalid.

On the other hand, sensing and LBT procedures can also be considered as combined procedures, in order to reduce the impact of other sidelink UEs on channel access. In this case, energy detection result should take both LBT energy detection and RSRP measurement into account. The impact of other sidelink UEs can be avoided by excluding the detected energy of sidelink UEs from the total energy detection threshold. Some enhancements may be required to achieve this effect.

An example of combined sensing and LBT procedure is given in Figure 8. Given that a transmission resource in slot 4 is selected by UE-A, based on the sensing result before the resource selection triggered time n. UE-A then continues performing sensing, i.e., RSRP measurement and SCI decoding, as well as Type 1 channel access before transmission.  During the Type 1 channel access procedure, if UE-A detected a resource is occupied by another SL UE (e.g. UE-B), it should exclude the detected energy of UE-B when performing energy comparison between the detected power and the energy detection threshold.

Figure 8: Combined sensing and LBT procedure

Proposal 40: Combined sensing and LBT procedures should be further studied for NR sidelink operation in unlicensed spectrum.

Conclusion

In this contribution, issues of channel access mechanism for sidelink on unlicensed spectrum are discussed. Observations and proposals are given as follows:

Proposal 1: RAN1 to discuss whether absence of other technologies is a valid environment in SL-U.

Proposal 2: Regarding CAPC for SL, mp=1 can be additionally used with p=1 for such as S-SSB and PSFCH transmission.

Proposal 3: Regarding CAPC for SL, p=1 is always used for S-SSB transmission.

Proposal 4: Regarding CAPC for SL, p=1 is always used for PSFCH transmission.

Proposal 5: Support Option 1a for the definition of ending time of SL reference duration.

- Option 1a: the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted

Proposal 6: The ending time of the first transmission burst, where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted, should also be considered as another ending time of SL reference duration.

Proposal 7: When all transmissions within the latest SL reference duration have SL-HARQ feedback disabled, Option 1 in RAN1#111 meeting is supported.

- Option 1: For every priority class p∈1,2,3,4, use the latest CWp.

Proposal 8: In case only groupcast option 2 PSSCH(s) is (are) transmitted within the latest SL reference duration, Option 1 in RAN1#111 meeting is supported with the consideration of both ACK feedback and NACK feedback.

- Option 1: Based on a (pre-)configurable ratio of received SL HARQ-ACK feedbacks.

Proposal 9: Groupcast option 1, i.e. NACK-based HARQ feedback, is not supported for SL-U.

Proposal 10: When Type 2A channel access procedure is applied to S-SSB transmission as discovery burst in NR-U, the total S-SSB transmission duration per UE is at most 1ms and the duty cycle is at most 1/20 with the time interval of duty cycle defined as S-SSB period, i.e., 160ms.

Proposal 11: Within a shared channel occupancy, Type 2 channel access procedures are applicable to PSFCH transmission in SL-U.

Proposal 12: Type 2A channel access procedure is applicable for PSFCH transmissions for a UE without a shared channel occupancy, with the total PSFCH transmission duration per UE is at most 1ms and the duty cycle is at most 1/20 with the time interval of duty cycle defined as S-SSB period, i.e., 160ms.

Observation 1: The definition and limitations are both different between Type 2A channel access and SCSt.

Proposal 13: Type 2A channel access procedures and SCSt should be discussed separately for S-SSB and PSFCH transmission.

Proposal 14: UE may perform S-SSB or PSFCH transmission as SCSt without sensing the channel when the transmission meets the regulation for SCSt in each country.

Proposal 15: For transmissions within a COT, CPE starting position can be within the guard symbol, i.e., within the symbol just before the next AGC symbol.

Proposal 16: For transmissions without a COT, NR-U principle can be considered as baseline, i.e., CPE starting position can be within at most 1, 2 or 4 symbols just before the next AGC symbol for 15, 30 or 60 kHz SCS, respectively.

Proposal 17: The CPE starting position for PSFCH should be (pre-)configured in each RP within the symbol just before the next PSFCH symbol.

Proposal 18: A single CPE starting position is adopted for S-SSB transmission. The CPE starting position should be (pre-)configured within the symbol just before the next AGC symbol.

Proposal 19: No further optimization is required regarding the impact of priority level on CPE starting positions.

Proposal 20: For PSCCH/PSSCH, a single CPE starting position should be adopted before transmissions at the same slot. The determined CPE starting position can be indicated by the SCI from the COT initiating UE.

Proposal 21: For the case when the transmission gap is between 16 and 25μs, Type 2B channel access procedure is also applicable.

Proposal 22: Type 2 channel access can be performed in the previous slot of the corresponding PSCCH/PSSCH transmission.

Proposal 23: The starting time of Type 2 channel access should take channel access duration and RX/TX switching time into account.

Proposal 24: A responding UE is allowed to transmit PSFCH(s) to UE(s) other than the initiator.

Proposal 25: A responding UE can transmit PSSCH/PSCCH to destination ID other than the source/destination ID of the COT initiating transmission.

Proposal 26: COT sharing information should at least include the remaining COT, the sensed LBT sub-bands, source ID, destination ID, and the channel access priority.

Proposal 27: SCI is qualified container for transmitting COT sharing information and at least 2nd stage SCI should be considered.

Proposal 28: The following conditions should be introduced under which UE can perform COT sharing:

- UE has data to transmit.

- The remaining COT is larger than a (pre-)configured threshold or the channel access priority value is larger than a (pre-)configured value.

Proposal 29: UE-to-UE COT sharing started with S-SSB or PSFCH transmission is not supported.

Observation 2: In NR-U, the COT sharing duration is a relative time, and the absolute duration of the COT depends on how many gaps of duration greater than 25μs are within the COT.

Proposal 30: The cast type should be considered for COT sharing operation:

- For unicast and connection-based groupcast, the COT sharing duration can be determined as that in NR-U, and the restriction of the absolute duration of the COT can be up to the regulation of each country.

- For connectionless groupcast and broadcast, the COT sharing ending time for all the COT sharing UEs is an absolute time, i.e. determined by the absolute duration from the starting occasion of COT sharing.

Observation 3: NR-U Type B multi-channel access can provide quick transmission opportunity and transmissions on multiple channels may be performed at the same time.

Proposal 31: NR-U DL Type B multi-channel access procedure is supported for multiple PSFCH transmissions on multiple channels.

Proposal 32: Multiple channel access procedure is not applicable to S-SSB transmission.

Proposal 33: For multiple channel access procedure, the following issues should be further studied:

- How to identify initial contention window counter Ninit

- How to perform COT sharing

- The impact of half duplex

Proposal 34: When L1 is triggered for reporting a subset of candidate resources for MCSt, only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1. Additional information such as the length of consecutive slots and SL DRX active time should also be provided to L1.

Proposal 35: When L1 reports a subset of candidate resources for MCSt, support option A. The LsubCH size should be the same within a candidate multi-slot resource.

- Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time

Proposal 36: The supportive of HARQ feedback in unicast and groupcast should be further considered for MCSt.

Proposal 37: Considering the complexity and efficiency of SL-U channel access, it is preferred that UE should perform resource selection procedure first and then perform channel access procedure.

Proposal 38: How to alleviate the resources unavailability caused by Type 1 channel access should be further studied, such as selecting more candidate transmission resources than configured TB transmission times.

Proposal 39: The following enhancements for resource selection procedure can be further studied:

- Selecting resources with the limitation of COT in time domain

- Selecting resources within one sub-band in frequency domain

Proposal 40: Combined sensing and LBT procedures should be further studied for NR sidelink operation in unlicensed spectrum.

References

  - RP-222806, “WID revision: NR sidelink evolution”, RAN #98-e meeting.

  - R1-2300681, “Discussion on physical channel design framework for sidelink on unlicensed spectrum”, RAN1 #112 meeting.

  - 3GPP TSG RAN WG1 Meeting RAN1 #110bis-e Chairman's Notes, Oct. 2022.

  - R2-2213005, “Report from session on NR SL”, RAN2 #120 meeting.

  - R1-2300017, “LS to RAN1 on CAPC for SL-U”, RAN2 #120 meeting.

  - R1-2212686, “FL summary for AI 9.4.1.1: SL-U channel access mechanism (EOM)”, RAN1 #111 meeting.

  - 3GPP TSG RAN WG1 Meeting RAN1 #111 Chairman's Notes, Nov. 2022.

  - 3GPP TS 37.213, “Physical layer procedures for shared spectrum channel access”, V17.4.0, 2022-12.

  - ETSI EN 301 893, “5 GHz WAS/RLAN; Harmonised Standard for access to radio spectrum”, V2.1.48, 2022-06.

  - MIIT〔2021〕No.129, “2400MHz, 5100MHz and 5800MHz bands technical requirements for interference avoidance of radio transmitting equipment”, 2021-09.

  - 3GPP TS 38.211, “NR; Physical channels and modulation”, V17.4.0, 2022-12.

  - 3GPP TSG RAN WG1 Meeting RAN1 #109-e Chairman's Notes, May. 2022.

  - 3GPP TSG RAN WG1 Meeting RAN1 #110 Chairman's Notes, Aug. 2022.