3GPP TSG RAN WG1 #112	R1-2300699

Athens, Greece, 27th February – 3rd March, 2023

Agenda item:		9.13.2

Title:	Low Power WUS Receiver Architectures

Source:	Nokia, Nokia Shanghai Bell

Document for:		Discussion and Decision

# Introduction

At the RAN#94-e meeting, it was agreed to study Wake Up Signal and Receivers designs.  These designs are to be primarily targeted at delay and power-sensitive, small form-factor devices, such as industrial sensors, controllers and wearables. Unlike previous power saving study items, the objectives for this study encompasses new signals and new receiver architectures.

In this contribution, we provide our thoughts on the following candidate LP-WUS architectures:

- RF envelope detection LP-WUS receiver

- Homodyne/zero-IF LP-WUS receiver

- Heterodyne architecture with IF envelope detection

- FSK Architectures

# Discussion

The design of the low power wakeup receiver is a trade-off between performance and power saving. This trade-off will vary depending on the use-case; hence we envisage different LP-WUR architectures being supported depending on the use-case.

Observation 1:      The optimum LP-WUR architecture may vary depending on the use-case.

Irrespective of the use case, the following factors will impact the design of the wake-up receiver and the trade-off between performance and power saving:

- WUS bit rate

    - The desired bit rate helps define the bandwidth and the number of NR symbols. The bitrate can either be fixed for all carriers or, depending on the NR configuration, for the particular band.

    - Example:

      - A WUS signal could contain 40 bits and be transmitted within 4 NR symbols (for this example). For a Manchester encoded WUS, this requires a data rate of approximately 300Kbs for OFDM symbol with subcarrier spacing of 15KHz and a data rate of approximately 600Kbs for OFDM symbol with sub carrier spacing of 30KHz.

      - This will require dynamic BW depending on the data rate/sub-carrier spacing and require a different sampling rate for the decoding.

    - Alternatively, the bit rate, bandwidth and time duration are fixed for the WUS and independent of the sub-carrier spacing. This will increase complexity at gNB as the number of bits per NR symbol will change depending on the sub-carrier spacing.

- Coverage

    - The required coverage is highly use-case dependent and can greatly influence the LP-WUR architecture.  For example, a simpler low power architecture for the WUR can be chosen for use-cases with stationery devices, whereas for use cases with some degree of mobility, a more advanced LP-WUR would be preferred.

- Multi-band support

    - A device made for worldwide operation supporting multiple bands, can be expected to need to support LP-WUS functionality in different bands. This will require a more advanced and also more power-hungry LP-WUR architectures.

- LP-WUS duration

    - The LP-WUS duration needs to be investigated. It would be preferable to aim to be less that a slot duration, to avoid the additional complexity incurred attempting to support inter-slot and/or multiple transmission reception point (TRP) transmisions. This will indirectly affect the bit rate, bandwidth, required gain and receiver performance.

- Stability – keep time and frequency tracking

    - For a LP-WUR operating in a DRX mode, it is essential to wake-up at correct time for WUS decoding. To be able to do this reliably, a robust synchronisation sequence/method is needed to handle the time drift.

    - With respect to FR2 tight frequency adjustment/ requirement should be considered to minimize the required bandwidth for the WUS.

  - Reuse of main radio HW components

    - For LP WUR integrated in a device with restricted size and volume and to keep the price as low as possible it would be beneficial to reuse especially the antennas and frontend (BPF Filters etc) and depending on the HW architecture optional other compentat in the main radio.

Based on afore discussion we make following observations and proposals:-

Observation 2:    	To support simpler beam mapping the time duration of the WUS should be less than 1 time slot.

Proposal 1:       	The LP-WUS duration and bandwidth scale with different symbol and sub-carrier numerology.

Proposal 2:       	RAN1 define a target LP-WUS symbol duration and bit rate for further architecture evaluations.

Proposal 3: 	The receiver gain/noise evaluation should be based on the required sensitivity, performance and coverage.

Proposal 4:	There should be support for WUS in all NR bands and the location of the WUS within the NR band.

Proposal 5:	Select a LP-WUR architecture with high reuse og HW components form the Main Radio to obtain low cost and minimal influence of the size and volume of the device.

Proposal 6: 		Evaluate techniques to maintain timing synchronisation when duty cycle operation is applied.

## RF envelope detection LP-WUS receiver

At the last RAN1#111 meeting, the following observation was agreed for the Homodyne/zero-IF LP-WUS receiver.

Agreement

The following observation to be captured in TR38.869:

For the architecture with RF envelope detection,

- It can achieve relatively low power consumption due to the removal of LO/PLL.

- Interference suppression for adjacent channel interference requires very high-Q matching network and/or RF BPF, which is challenging due to the high Q values and may require off-chip components.

- Interference suppression for interference from legacy NR signals and/or other LP WUS on adjacent subcarriers, if performed in RF, requires very high-Q matching network and/or RF BPF, which is challenging due to the high Q values and may require off-chip components.

- The support of multiple bands and/or carriers may require multiple high-Q matching networks and/or RF BPFs or multiple off-chip components.

- RF LNA can be applied to improve sensitivity, with the cost of additional power consumption.

- The noise figure can be relatively high.

The rf envelope detection receiver architecture is mainly aimed for single band usage, that is to say, bands within the bandwidth of the rf envelope detector. The application of a tuneable RF filter in the RF path will affect the the NR Main Radio performance (insertion loss, higher NF etc) in case of reuse of antenna and RF frontend.. If the RF envelope detector LP-WUS is prefered it would be beneficial to have full separation (no reuse) of the NR Main radio and the LP-WUR.

To obtain support for the WUS to be used in various supported bands by a device, the antennas and the tuneable RF filter should be designed to support frequency areas and not be limited to support NR bands.

To design a tuneable filter for the FR2 frequency band is expected to be challenging due to tolerances and accuracy of the tuneable section in full temperature range.

For further evaluation here, the following assumptions and observations for the receiver architecture are made:

- In order to support more than one band, the RF envelope detection LP-WUS receiver is expected to require a wideband LNA and wideband RF envelope detector. This will set high requirements for the matched filter selectivity to have a good suppression of the NR OFDM signal within the same band as the LP-WUS.

  - The required gain of the LNA can be determined when the evaluation of the required SNR, bandwidth, coverage and selectivity has been completed.

- The WUS is assumed to have flexible location within a carrier.

  - The degree of flexibility within a carrier is open for investigation.

- As the bandwidth of the WUS signal is expected to be scaled according to the sub carrier spacing, the requirements for the match filter frequency selection will change accordingly.

  - This can lead to challenges due to the change in Q for different bandwidth configuration depending on the sub-carrier spacing.

Figure 1: Illustration of a MC-OOK receiver architecture with RF envelope detector.

Pros:

- It is possible to obtain very low power consumption due to no LO/XO being required and the optional LNA.

- Very fast enable time due to only very little settle time for the tuneable filter

Cons:

- In case of variable bandwidth of the WUS depending on the subcarrier spacing a solution for dynamic bandwidth might be needed

- To obtain correct center frequency for the WUS over full temperature range, calibration or special handling of the tuneable filter might be needed.

Observation 3:    For the RF Envelope Detection architecture to support LP-WUS for more than one NR band and with flexible positioning within a NR band, will be more challenging compared to other architectures, due to the need to support very high selectivity requirements for the (expected) wideband LNA and wideband RF envelope detector.

Proposal 7: 	The evaluation of the design of the tuneable filter for the RF Envelope Detection architecture, considers:

- a bandwidth allocation that allows multiple carriers to be supported

- LP-WUS configured for different sub-carrier/symbol numerologies

## Homodyne/zero-IF LP-WUS receiver

The existing NR OFDM based main radio, can be relatively easily adapted to be reused to act as a LP-WUS zero-IF receiver. In this section, we describe the,

- How certain main radio components can either be reused, modified and/or optimised for the zero-IF LP-WUS receiver architecture to support lower power consumption and/or flexible LP-WUS placement in the frequency domain.

- The relationship between the zero-IF LP-WUS receiver architecture frequency offset tolerance and the WUS bandwidth,

At the last RAN1#111 meeting, the following observation was agreed for the Homodyne/zero-IF LP-WUS receiver.

Agreement

The following observation to be captured in TR38.869:

For homodyne/zero-IF architecture with baseband envelope detection,

- For the support of band and/or carrier tuning, the band and/or carrier tuning can be achieved via tuning the LO frequency.

- The matching network and RF BPF for LP WUR may or may not reuse those of the main radio.

- It is more effective and less complex to use BB BPF/LPF instead of high-Q matching network and/or RF BPF to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- Using FLL instead of PLL consumes less power, but it may result in larger frequency error.

- It can suffer from LO leakage (DC offset) and flicker (1/f) noise. The impact may be alleviated by using BB BPF in some cases.

- RF LNA can be applied to improve sensitivity, with the cost of additional power consumption.

- The baseband envelope detection can be done in either analog domain (before ADC) or digital domain (after ADC).

A Zero-IF receiver can be relatively easily modified to reuse many of the existing NR OFDM based main radio components, including the frontend, antennas, LNA, mixer ADC and parts of the uP/ statemachine as illustrated in the figure below:

Figure 2: Illustration of OFDM/ Zero-IF Receiver architecture

One benefit of reusing the NR main radio parts, is that it would readily support flexible bandwidth and placement of the LP-WUS in and within different bands.

- To support more than one band, the receiver could either use a single wideband LNA or multiple LNAs supporting smaller frequency areas.

  - The LF filter is expected to provide good suppression of the NR OFDM signal within the same band as the WUS.

  - The required gain of the LNA can be determined when the evaluation of the required SNR, bandwidth, mobility and selectivity have been completed.

- As the bandwidth of the WUS signal is expected to be scaled according to the sub carrier spacing, the LP filter will most likely be required to have different cut off frequencies, e.g. one configuration for each sub-band spacing configuration.

- To reduce the current consumption of the receiver, a duty cycle mode of operation is recommended.

- The reference clock for the receiver could be a low precision clock with an accuracy of 200ppm. (Typical spec for the RTC 32KHz reference clock). To derive the required maximum accuracy of the reference clock – the guard band, and the required suppression of NR OFDM signal need to be defined.

In addition to operating this main radio derived LP-WUS receiver in a DRX/duty cycle manner around predefined WUS occasions to reduce overall power consumption, specific components of the main radio that can reused but in a reduced power consumption mode, include:

- The multi-bit ADC.

  - This can be reused for LP-WUS detection, but at a lower resolution to reduce power consumption.

- The reference clock for the main radio deived LP-WUS receiver.

  - This could be a low precision clock with an accuracy of 200ppm. (Typical spec for the RTC 32KHz reference clock).

- The FFT.

  - For the example for a MC-OOK modulation scheme, supporting a 8 bit LP-WUS for each NR symbol, it is possible to reduce the complexity of the FFT algorithm by >85 times using a smaller FFT (Based on Matlab simulations, where the FFT is reduced from 256 to 8. The reduction will be higher compared to higher order FFT configurations).

- The statemachine/ uP functionality.

  - This can be optimized compared to OFDM receiver implementation, due to the much simpler structure of the LP-WUS signal.

Observation 4:    	The existing NR OFDM based main radio, can be relatively easily reused to act as a LP-WUS zero-IF receiver, capable of supporting a wide range of LP-WUS deployment options.

Proposal 8: 	Perform an evaluation of the required functionality of the statemachine/uP and compare the optimization with existing NR radio implementation

To derive the required minimum accuracy of the reference clock for the LP-WUS zero-IF receiver architecture, the guard band and the required suppression of NR OFDM signal needs to be defined. The Figure 3 illustrates an example of OOK-modulated OFDM modulation scheme for an LP-WUS signal requiring a total of 240 KHz (8x30KHz given the 30KHz subcarrier spacing) when there is no frequency offset uncertainty, i.e. ideal frequency conditions.

Figure 3:  Illustration of 8 bit OOK-modulated OFDM LP-WUS for each NR symbol. 4x8 bits are illustrated

The required LP-WUS bandwidth increases as the tolerance of the reference oscillator used is relaxed. This increased bandwidth will impact the required sample rate of the ADC. The higher the sample rate of the ADC, the higher the power consumption of the receiver.  Assuming the minimum sample rate of the ADC is 2x the sample rate for a 8 bit WUS signal, then for a 30KHz subcarrier spacing configuration on a 1 GHz carrier and a 20ppm uncertainty tolerance, the bandwidth goes from 2x240KHz= 480KHz to 2x1680KHz= 3360KHz for the same configuration but with a 200ppm frequency tolerance.

Observation 5:		For a MC-OOK modulated signal, the required LP-WUS bandwidth increases as the tolerance of the reference oscillator used is relaxed .

A narrower bandwidth of the LP-WUS signal is beneficial for the overall NR network data capacity but can make LP-WUS performance sensitive especially in fading conditions if there is only one antenna.

By keeping the bandwidth of the LP-WUS relatively low,  the ADC sampling rate can also be kept relatively low and thereby save power. A method to keep the frequency accuracy high could be studied.

Proposal 9: 		Evaluate techniques to allow lower powered but less accurate reference osciallators to be used. Techniques that could be studied include:

- Including a preamble to the LP-WUS signal for frequency (and possibly time) synchronisation.

- Using a separate beacon signal.

- Other techniques are not precluded.

Observation 6:  	The required accuracy of the reference clock is partly determined by the required guard band suppression characteristics.

Observation 7:	The LO/Mixer can be relaxed in tolerance and thereby lower the current consumption when running in LP-WUS mode.

Proposal 10:  	The evaluation of the design of the Zero-IF architecture, considers:

- The accuracy of the reference clock and it’s impact on the guard band.

## Heterodyne architecture with IF envelope detection

At the last RAN1#111 meeting, the following observation was agreed for the Homodyne/zero-IF LP-WUS receiver.

Agreement

The following observation to be captured in TR38.869:

For heterodyne architecture with IF envelope detection,

- For the support of band and/or carrier tuning, the band and/or carrier tuning can be achieved via tuning the LO frequency.

- The matching network and RF BPF for LP WUR may or may not reuse those of the main radio.

- It is more effective and less complex to use IF BPF instead of high-Q matching network and/or RF BPF to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- Using FLL instead of PLL consumes less power, but it may result in larger frequency error.

- The IF frequency can be properly selected to avoid LO leakage (DC offset) and flicker (1/f) noise.

- Image rejection can be done via either image rejection filter or image rejection mixer.

  - Image rejection filter can be done in either RF or IF, which may require high-Q filter.

  - Image rejection mixer requires two-branch (I/Q) mixing with good matching in gain and phase, which consumes additional power.

- RF LNA and/or IF AMP can be applied to improve sensitivity, with the cost of additional power consumption.

The IF envelope detection Heterodyne receiver architecture is aimed for multi band usage. This architecture allows for a  easy re-use of the main radio elements, such as the NR frontend, antennas, LNA and potentially also the mixer.

For further evaluation here, the following assumptions and observations for this receiver architecture are made:

- The Modulation is assumed to be Manchester encoded MC-OOK.

- To support more than one band the receiver could use a wideband LNA and wideband IF envelope detector.

  - This will require the IF filter to provide good suppression of the NR OFDM signal within the same band as the WUS.

  - The required gain of the LNA can be determined when the evaluation of the required SNR, bandwidth, coverage and selectivity have been completed.

- The ADC is expected to be at least 1 bit ADC.

- The sample rate depends on the sub-carrier spacing, bandwidth and data rate.

- The WUS is assumed to have flexible location within a carrier.

  - The degree of flexibility within a carrier is open for investigation.

- As the bandwidth of the WUS signal is expected to be scaled according the the sub carrier spacing, the IF filter is expected to support different passbands, with one configuration for each sub-band spacing configuration.

- To minimize the power consumption the receiver should exploit a duty cycle mode of operation. To ensure reliable time tracking, the LP-WUS, or periodic beacon style signal, should be present sufficiently often to avoid timing drift and loss of synchronization.

- The architecture can support FR2 but will add stability requirements to the LO to ensure the frequency tolerances are as low as possible. Ie 200ppm in 28GHz will yield a frequency error of 5.6MHz. As this tolerance is larger than the preferred bandwidth of 5MHz for the WUS including guardband, methods to keep the frequency error low may need to be evaluated.

Figure 4: Illustration of the Heterodyne receiver architecture with IF Envelope detection

Observation 8:    The Heterodyne receiver based architecture can most easily support LP-WUS being configured on different carriers and in different positions within a carrier.

Observation 9:    The Heterodyne receiver based architecture can most easily reuse elements of the main radio architecture.

Observation 10:    To reduce the relatively higher power consumption of the Heterodyne receiver based architecture, a duty cycle mode of operation is beneficial, for which methods to maintain timing synchronisation may be useful.

Observation 11:	To reduce the power consumption of the MR ADC, it should support reconfiguration to operate at lower sample frequences and/or lower numbers of bit resolution. This might require HW changes to the MR

## FSK architectures

In this sub-section, we provide our thoughts on the FSK architectures tentatively discussed (but not agreed) in the draft proposal below below at the last RAN1#111 meeting,.

#### Draft Proposal 4-3: (FSK)

Study the receiver architectures for FSK based on the following diagrams:

- Parallel homodyne receiver

- Parallel heterodyne receiver

- Homodyne receiver with frequency to amplitude conversion

- Heterodyne receiver with frequency to amplitude conversion

- Note: Other architectures are not precluded.

### Parallel homodyne FSK receiver

Figure 5:  Parallel homodyne BFSK receiver

This architecture is based on 2 zero-IF LP-WUS receivers working in parallel to detect the BFSK “mark” and “space” bits.  As expected, the pros and cons of this design are similar to the OOK variant, hence the following observations.

Observation 12:    The parallel homodyne BFSK receiver shares the same characteristics as the homodyne (zero-IF) OOK receiver.

When compared against the other 3 BFSK architectures described in this proposal, the following observations can be made:

Observation 13:    Compared to other BFSK detector architectures, the parallel homodyne BFSK receiver architecture is expected to:

- Have better selectivity compared to than the frequency-to-amplitude based reciever designs, given the separate filtering per FSK tone, reducing the impact of spurious noise between the tones.

- Require more power than the frequency-to-amplitude based reciever designs, given the separate parallel processing chains applied per FSK tone.

- Be more complex to design to incorporate retunable elements, to support alternative frequency placements of the LP-WUS.

### Parallel heterodyne receiver

Figure 5:  Parallel heterodyne BFSK receiver

This architecture is based on 2 heterodyne architectures with IF envelope detection receivers working in parallel to detect the BFSK “mark” and “space” bits.  As expected, the pros and cons of this design are similar to the OOK variant, hence the following observations.

Observation 14:    The parallel heterodyne BFSK receiver shares the same characteristics as the heterodyne architecture with IF envelope detection OOK receiver.

When compared against the other 3 BFSK architectures described in this proposal, the following observations can be made:

Observation 15:    Compared to other BFSK detector architectures, the parallel heterodyne BFSK receiver architecture is expected to:

- Have better selectivity compared to than the frequency-to-amplitude based reciever designs, given the separate filtering per FSK tone, reducing the impact of spurious noise between the tones.

- Require more power than the frequency-to-amplitude based reciever designs, given the separate parallel processing chains applied per FSK tone.

- Require more power than the comparable homodyne architecture but also support improved selectivity.

- Be less complex to incorporate support retunable elements, to support alternative frequency placements of the LP-WUS.

### BFSK Quadrature demodulator

Figure 6:  BFSK Quadrature demodulator

Whilst in the draft proposal, this receiver is described as a “Homodyne receiver with frequency to amplitude conversion”, we feel that a better title is a BFSK Quadrature demodulator.  Our understanding of this type of demodulator, is that it requires a single local oscillator with a carrier frequency half-way between the FSK tones to create a downshifted baseband complex IQ form of the FSK signal.  Then by detecting the the change in phase of the resultant complex baseband IQ signal, the presence of a positive or negative frequency tone can be determined.

Observation 16:    A BFSK Quadrature demodulator uses one filter chain, and is therefore less complex, and less power cinsumming than parallel matched filter based BPSK recievers.

Observation 17:    A BFSK Quadrature demodulator uses one main input filter for both tones, and therefore suffers more from any spurious noise between the tones, leading to poorer selectivity compared to other BPSK designs.

Proposal 11:	Frequency tolerances for any FSK receiver design should be evaluated, as the allowed frequency offset will affect the required frequency spacing between the FSK signals and between the FSK signals and the NR data.

### Heterodyne receiver with frequency to amplitude conversion

Figure 7:  Heterodyne receiver with frequency to amplitude conversion

This demodulator design is composed of a phase shifter and a mixer, that work together to provide FM-to-AM conversion. After mixing the received FSK signals and the phaseshifted received FSK signals by a mixer, a positive or negative voltage at output of the mixer generates the decoded BFSK signal of 1’s or 0’s.

Observation 18:    The Heterodyne BFSK receiver with frequency to amplitude conversion, uses one filter chain, and is therefore less complex than parallel matched filter based BPSK recievers.

Observation 19:    The Heterodyne BFSK receiver with frequency to amplitude conversion, uses one filter chain,  and therefore suffers more from any spurious noise between the tones, and generally has poorer selectivity compared to other BPSK designs.

Observation 20:    The Heterodyne BFSK receiver with frequency to amplitude conversion, incorporates an IF stage that should be less complex to adapt to support retunable elements, to support alternative frequency placements of the LP-WUS

### General considerations for further study into FSK demodulator designs

Each of these FSK designs, have their relative pros and cons.  Ultimately, the preferred architecture, will depend on the weighting of multiple criteria, including:

- Cost

- Power consumption

- Selectivity

- Tunability/Flexibility

- Payload Capacity

Note in the list above, we have mentioned Payload Capacity.  Whilst we have discussed the 4 designs assuming BFSK only, we feel that in order to support larger LP-WUS payloads given the constraints of NR OFDM sub-carrier and symbol numerology, M-FSK should be considered.   When M-FSK is considered, the parallel matched filters detector designs should have a clear advantage in terms of noise suppression and therefore selectivity, over the single chain designs relying on some form of FM-to-AM conversion.

Proposal 12:    Further study of FSK demodulators consider the M-FSK variants.

# Conclusion

In this contribution, we have discussed various candidate LP-WUS architectures, and from those discussions we have the following observations and proposals.:

Observation 1:      The optimum LP-WUR architecture may vary depending on the use-case.

Observation 2:    	To support simpler beam mapping the time duration of the WUS should be less than 1 time slot.

Observation 3:    For the RF Envelope Detection architecture to support LP-WUS for more than one NR band and with flexible positioning within a NR band, will be more challenging compared to other architectures, due to the need to support very high selectivity requirements for the (expected) wideband LNA and wideband RF envelope detector.

Observation 4:    	The existing NR OFDM based main radio, can be relatively easily reused to act as a LP-WUS zero-IF receiver, capable of supporting a wide range of LP-WUS deployment options.

Observation 5:		For a MC-OOK modulated signal, the required LP-WUS bandwidth increases as the tolerance of the reference oscillator used is relaxed .

Observation 6:  	The required accuracy of the reference clock is partly determined by the required guard band suppression characteristics.

Observation 7:	The LO/Mixer can be relaxed in tolerance and thereby lower the current consumption when running in LP-WUS mode.

Observation 8:    The Heterodyne receiver based architecture can most easily support LP-WUS being configured on different carriers and in different positions within a carrier.

Observation 9:    The Heterodyne receiver based architecture can most easily reuse elements of the main radio architecture.

Observation 10:    To reduce the relatively higher power consumption of the Heterodyne receiver based architecture, a duty cycle mode of operation is beneficial, for which methods to maintain timing synchronisation may be useful.

Observation 11:	To reduce the power consumption of the MR ADC, it should support reconfiguration to operate at lower sample frequences and/or lower numbers of bit resolution. This might require HW changes to the MR

Observation 12:    The parallel homodyne BFSK receiver shares the same characteristics as the homodyne (zero-IF) OOK receiver.

Observation 13:    Compared to other BFSK detector architectures, the parallel homodyne BFSK receiver architecture is expected to:

- Have better selectivity compared to than the frequency-to-amplitude based reciever designs, given the separate filtering per FSK tone, reducing the impact of spurious noise between the tones.

- Require more power than the frequency-to-amplitude based reciever designs, given the separate parallel processing chains applied per FSK tone.

- Be more complex to design to incorporate retunable elements, to support alternative frequency placements of the LP-WUS.

Observation 14:    The parallel heterodyne BFSK receiver shares the same characteristics as the heterodyne architecture with IF envelope detection OOK receiver.

Observation 15:    Compared to other BFSK detector architectures, the parallel heterodyne BFSK receiver architecture is expected to:

- Have better selectivity compared to than the frequency-to-amplitude based reciever designs, given the separate filtering per FSK tone, reducing the impact of spurious noise between the tones.

- Require more power than the frequency-to-amplitude based reciever designs, given the separate parallel processing chains applied per FSK tone.

- Require more power than the comparable homodyne architecture but also support improved selectivity.

- Be less complex to incorporate support retunable elements, to support alternative frequency placements of the LP-WUS.

Observation 16:    A BFSK Quadrature demodulator uses one filter chain, and is therefore less complex, and less power cinsumming than parallel matched filter based BPSK recievers.

Observation 17:    A BFSK Quadrature demodulator uses one main input filter for both tones, and therefore suffers more from any spurious noise between the tones, leading to poorer selectivity compared to other BPSK designs.

Observation 18:    The Heterodyne BFSK receiver with frequency to amplitude conversion, uses one filter chain, and is therefore less complex than parallel matched filter based BPSK recievers.

Observation 19:    The Heterodyne BFSK receiver with frequency to amplitude conversion, uses one filter chain,  and therefore suffers more from any spurious noise between the tones, and generally has poorer selectivity compared to other BPSK designs.

Observation 20:    The Heterodyne BFSK receiver with frequency to amplitude conversion, incorporates an IF stage that should be less complex to adapt to support retunable elements, to support alternative frequency placements of the LP-WUS

Proposal 1:       	The LP-WUS duration and bandwidth scale with different symbol and sub-carrier numerology.

Proposal 2:       	RAN1 define a target LP-WUS symbol duration and bit rate for further architecture evaluations.

Proposal 3: 	The receiver gain/noise evaluation should be based on the required sensitivity, performance and coverage.

Proposal 4:	There should be support for WUS in all NR bands and the location of the WUS within the NR band.

Proposal 5:	Select a LP-WUR architecture with high reuse og HW components form the Main Radio to obtain low cost and minimal influence of the size and volume of the device.

Proposal 6: 		Evaluate techniques to maintain timing synchronisation when duty cycle operation is applied.

Proposal 7: 	The evaluation of the design of the tuneable filter for the RF Envelope Detection architecture, considers:

- a bandwidth allocation that allows multiple carriers to be supported

- LP-WUS configured for different sub-carrier/symbol numerologies

Proposal 8: 	Perform an evaluation of the required functionality of the statemachine/uP and compare the optimization with existing NR radio implementation

Proposal 9: 		Evaluate techniques to allow lower powered but less accurate reference osciallators to be used. Techniques that could be studied include:

- Including a preamble to the LP-WUS signal for frequency (and possibly time) synchronisation.

- Using a separate beacon signal.

- Other techniques are not precluded.

Proposal 10:  	The evaluation of the design of the Zero-IF architecture, considers:

- The accuracy of the reference clock and it’s impact on the guard band.

Proposal 11:	Frequency tolerances for any FSK receiver design should be evaluated, as the allowed frequency offset will affect the required frequency spacing between the FSK signals and between the FSK signals and the NR data.

Proposal 12:    Further study of FSK demodulators consider the M-FSK variants.

# References

- RP-222644    Revised SID: Study on low-power Wake-up Signal and Receiver for NR       Vivo

- R1-2300700   L1 signal design and procedures for low power WUS					  Nokia, Nokia Shanghai Bell

- R1-2300698   Low power WUS Evaluation Methodology			             Nokia, Nokia Shanghai Bell