3GPP TSG RAN WG1 #112	R1-2300705

Athens, Greece, February 27th – March 3rd, 2023

Source:	ZTE

Title:	Discussion on network verified UE location for NR NTN

Agenda Item:	9.11.2

Document for: Discussion

Introduction

In RAN1#111 [1], the observations on the location verification methods were obtained. Based on RAN1 observations, it has been agreed that WI phase in RAN#98e is started and the WID is updated [2]:

Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied.

Note 1: Enhancements assume reuse of the RAT dependent positioning framework

Note 2: The specification of DL-TDOA enhancements will be subject to the study of the impact of realistic UE clock drift onto DL-TDOA performance

Note 3: The target accuracy for position verification purposes is as documented in clause « recommendations » of the 3GPP TR 38.882 (i.e. 10 km granularity)

Note 4: Multiple satellite in view by the UE may be considered if time allows

Note 5: The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved

Note 6: The enhancements should take into account the mirror-image ambiguity

Note 7: Network verified UE location is an optional UE feature

In this contribution, the solutions for position verification in NTN based on updated WID is discussed.

Enhancement on multi-RTT method with single satellite

              - In RAN#98e, the WID is updated and multi-RTT to support the network verified UE location in NTN assuming a single satellite in view is prioritized. And reuse of the RAT dependent positioning framework should be considered.

              - When only a single satellite is in view, the mobility of satellite should be utilized as shown in Figure 1. In such case, the satellite positions at different time instance are regarded as multiple anchor points. The multiple RTTs corresponding to these anchor points are obtained and utilized for multi-RTT method.

Figure 1 Illustration of multi-RTT method with single satellite

              - In legacy TN, multi-RTT method is to obtain RTTs corresponding to stationary TRPs at different places simultaneously. While in NTN with single satellite in view as shown in Figure 1, RTTs corresponding same moving satellite at different time instance are obtained. Due to the difference, adaptions are needed to support multi-RTT method in NTN with single satellite. In RAN1#111, following conclusion is achieved, which shows the potential enhancements:

Conclusion For network verification of UE location in NR NTN based on multi-RTT using UE RX-TX time difference report, if the UE reports needed to perform multi-RTT can be assumed to be trusted, existing multi-RTT framework may be reused with potential enhancements to adapt it to NTN context. This may include, but not limited to: - If justified: NTN-specific definition of UE RX-TX time difference, including as an example, potential modifications to UE Rx – Tx time difference to enable network verification of UE location without introducing any additional measurements at the UE (with respect to Rel-17 NTN) - The following is not precluded: the UE Rx – Tx time difference is defined as TUE-RX – TUE-TX, where TUE-RX – TUE-TX is directly derived from the timing advance TTA applied by the UE at a given subframe. - Above does not imply that the relevant work is prioritized. - Other assistance data (e.g. ephemeris) to be transferred from gNB to the LMF. - If justified: Other assistance data (e.g. to resolve ambiguity on mirror position issue) to be transferred from UE to LMF - If justified: Adaptations enabling Rx-TX measurements for Multi-RTT involving multiple cells within the same satellite For network verification of UE location in NR NTN based on DL-TDOA positioning, if the UE reports needed to perform DL-TDOA positioning can be assumed to be trusted, existing DL-TDOA positioning framework may be reused with potential enhancements to adapt it to NTN context.

              - In following, the enhancements on multi-RTT are discussed.

2.1 Derivation of RTT from TA report

              - In Rel-17 NTN, TA report has been supported to accommodate the update of Koffset. In fact, the RTT between satellite and UE is equal to the UE specific TA for service link. That is, network is able to obtain the RTT based on TA report from UE and perform positioning using the multi-RTT method.

              - In legacy TN, the RTT is calculated based on the UE RX-TX time difference and gNB RX-TX time difference. Originally, the RX-TX time difference is to measure the time difference of transmit and received timing of same subframe in order to measure the round-trip delay. However, in NR, since DL and UL transmission may be based on timing of difference cells, following definition is captured in 38.215 [3]:

| Definition | The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP) [18], defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP. Multiple DL PRS or CSI-RS for tracking resources, as instructed by higher layers, can be used to determine the start of one subframe of the first arrival path of the TP. For frequency range 1, the reference point for TUE-RX measurement shall be the Rx antenna connector of the UE and the reference point for TUE-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TUE-RX measurement shall be the Rx antenna of the UE and the reference point for TUE-TX measurement shall be the Tx antenna of the UE. |
| --- | --- |

              - In NTN, the propagation delay can be longer than one subframe. As a result, the legacy Rx – Tx definition no longer works since it can only measure the time difference of the closest uplink subframe to the time of received subframe, which is shorter than one subframe. To solve this issue, the definition of UE RX-TX time difference can be updated as TUE-RX – TUE-TX= TTA, where TTA is the overall TA value applied at the time instance of TA report. The satellite RX-TX time difference can be calculated based on the common TA TTA,common corresponding to that time instance. And the RTT between UE and satellite can be finally obtained as RTT = TTA - TTA,common. Moreover, compared with legacy RTT measurement based on PRS and SRS, the quality of TA report is not affected by SNR. Therefore, it is more suitable to NTN scenario where link budget is limited.

              - However, the TA is reported with a granularity of slot in Rel-17, which may lead to significant quantization error. In 38.321 [4], it has been specified that the Timing Advance field of Timing Advance Report MAC CE indicates the least integer number of slots greater than or equal to the Timing Advance value. Hence, the quantization error of TA can be at most te=1 ms when SCS=15kHz, which corresponds to c*te/2=150km estimation error of distance between UE and anchor point. With such large error, the target accuracy for position verification purposes, i.e., 10km granularity, cannot be satisfied. Hence, defining higher TA report granularity should be supported in Rel-18 for better position verification performance.

              - W.r.t to the reliability of reported TA, it should be noted that UE should at least have a valid GNSS information at physical layer used for pre-compensation. Otherwise, the UL synchronization will be lost and UE cannot access the network. This GNSS information used for UL synchronization should be accurate and trustable. And of course, the TA derived based on this GNSS information should also be trustable. It should be noted that the TA used for UL synchronization is based on a valid GNSS instead of reported GNSS. Hence, the reliability of TA does not depend on the reliability of reported GNSS. The only possible case where reported TA is not trustable is that UE can fake the reported TA, i.e., report a TA not equal to the actual TA used in UL synchronization. However, if a UE is able to fake the reported TA applied in physical layer, which means physical layer is not secure, UE should also be able to fake other reported physical layer parameters including the RTT obtained based on measurement. Therefore, as long as the RTT obtained based on measurement can be trustable, the TA report should also be trustable and no more verification on the reported TA is needed.

              - Observation 1: TA report supported in Rel-17 NTN can be used for RTT estimation. The granularity may need to be enhanced for better location verification performance.

              - Observation 2: TA reported by UE can be considered to have similar reliability as other RAT dependent parameters since it is a physical layer parameter related to UL synchronization.

Proposal 1: TA report based RTT estimation should be supported in multi-RTT method in NTN location verification.

Proposal 2: TA report with higher granularity can be investigated to improve the location verification performance.

2.2 Assistance data for location verification

              - According to above discussion, the location verification in NTN with single satellite in view should utilize the mobility of satellite. Additional assistance data should be transferred to the location verification module.

              - Firstly, the ephemeris of satellite should be known by the location verification module to obtain the satellite positions at difference time instance. However, ephemeris is generated by NCC. If NCC directly indicate the ephemeris to gNB or gateway, the location verification module at CN may not know this information. Hence, unless new interface between NCC and CN is defined, the ephemeris should be transferred to CN by gNB or UE.

Proposal 3: Ephemeris report to CN from gNB or UE should be supported for location verification.

              - Moreover, if TA report based multi-RTT method is supported, the TA will be reported to gNB instead of CN since it is originally reported for scheduling. If the TA is also used for location verification, it should also be transferred to CN. Generally, the location verification procedure is better to be transparent to UE to avoid UE malicious faking. Hence, the TA values for location verification should be firstly reported gNB similar to other TA values reported for scheduling. Then, gNB will transfer the TA values for location verification to CN.

Proposal 4: TA report to CN from gNB should be supported for location verification.

Solving mirror image ambiguity

              - When only single satellite is in view, the anchor points will be in the same plane. As a result, the multi-RTT method cannot resolve the actual position and the mirror point. Other methods may be combined to solve the mirror image ambiguity.

              - In our view, the mirror image ambiguity can be solved by implementation and additional enhancement is not needed. When the actual UE position is far from the orbit plane, the distance between actual position and mirror point will be large. They are not likely to be in the same beam/cell. Hence, network is easy to solve the mirror image ambiguity through the beam ID or cell ID, which is solely based on implementation. When the actual UE position is close to the orbit plane, the actual position and mirror point may be in same beam. However, by proper allocation of the beam, e.g., avoiding setting a beam across the orbit plane, the probability that actual position and mirror points locate within same beam can be minimized. Even if they are in same beam, the network can distinguish them based on UL-AoA at satellite since they are from different direction with respect to the orbit plane. Moreover, UL-AoA is measured at network side and there is no need of spec enhancement. Overall, the mirror image ambiguity issue can be solved based on implementation and no spec enhancement is needed.

Proposal 5: Mirror image ambiguity when single satellite is in view can be solved through implementation. No spec enhancement is needed.

Conclusions

In this contribution, analysis on network verified UE location for NR-NTN is conducted with following proposals and observations:

              - Observation 1: TA report supported in Rel-17 NTN can be used for RTT estimation. The granularity may need to be enhanced for better location verification performance.

              - Observation 2: TA reported by UE can be considered to have similar reliability as other RAT dependent parameters since it is a physical layer parameter related to UL synchronization.

Proposal 1: TA report based RTT estimation should be supported in multi-RTT method in NTN location verification.

Proposal 2: TA report with higher granularity can be investigated to improve the location verification performance.

Proposal 3: Ephemeris report to CN from gNB or UE should be supported for location verification.

Proposal 4: TA report to CN from gNB should be supported for location verification.

Proposal 5: Mirror image ambiguity when single satellite is in view can be solved through implementation. No spec enhancement is needed.

- Reference

- R1-2210952, FL Summary #4: Network verified UE location for NR NTN, RAN1#111

- RP-223534, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, RAN#98e

- 3GPP TS 38.215, Physical layer measurements (Release 17), V17.2.0

- 3GPP TS 38.321, Medium Access Control (MAC) protocol specification (Release 17), V17.2.0