3GPP TSG RAN WG1 Meeting #112	R1-2300760

Athens, Greece, Feb 27th – Mar 3rd , 2023

Agenda Item:	9.5.1.2  Measurements and reporting for SL positioning

Source:	Locaila, Inc

Title:	Proposed RTT measurement method using Carrier Phase Positioning

Document for:	Discussion & Decision

Introduction

In the last 111th meeting, participating companies in TR38.859 presented simulation results showing the superior accuracy of the Carrier Phase Positioning method. The results, even though some were based on limited assumptions, showed that the carrier phase positioning can achieve accuracy within a centimeter range. Applying the technology to V2X measurements is expected to provide the high precision required by the vehicle industry, even with a regulatory bandwidth of only tens of MHz in the FR1 range.

This contribution presents one method of applying Carrier Phase technology to measure the relative distance between vehicles by utilizing the phase of the reciprocal signal. This approach is similar to RTT (Return Trip Time) but we propose to use the new terminology RTP (Return Trip Phase) as it calculates the signal phase in the frequency domain to determine the distance and latency.

(Proposal 1) Introduce new terminology RTP (Return Trip Phase) to avoid any confusion with RTT when discussing the frequency domain method such as the carrier phase positioning.

Solution for the RTP (Return Trip Phase) measurement

To measure precise distance or time delay information using the phase information of the reference signal, a new reference signal structure optimized for phase measurement is required. Of course, phase measurement is possible to some extent using the legacy com-N PRS structure of Rel-17, however, the use of a continuous signal (e.g. unstaggered comb pattern) proposed in [6] offers more advantages for sidelink positioning between vehicles. We propose to study the potential benefit of introducing a new PRS signal efficient for accurate phase measurement.

(Proposal 2) Study new PRS signal (e.g. unstaggered pattern) efficient for phase-based sidelink positioning methods such as carrier phase positioning.

Figure 1 below illustrates a single/double-sided RTP method in which the target UE-B communicates a returning signal with anchor UE-A and calculates the signal delay based on the relative distance.

Figure 1. Single/Double-sided RTP method between two vehicles

In Figure 1, the internal clock signal of anchor UE-A and target UE-B are generated by unsynchronized local oscillators, which results in their local time being represented as (t-ψA) and (t-ψB) respectively. While the local clock errors ψA and ψB can be changed slightly over time, for convenience, instantaneous time errors are assumed to be fixed constants.

The RTP procedure begins with anchor UE-A periodically broadcasting the PRS signal generated by Equation (1) below. This PRS signal is the same reference signal used in methods such as TDoA or AoA.

(1)

PRSAt=A1ejω1t-ψA+A2ejω2t-ψA+…+Amejωmt-ψA=q=1mAqejωqt-ψA

In Equation (1), w1, w2, … wm represent the angular frequencies of subcarriers that are equally spaced in the frequency domain. The legacy comb-N structure can also be viewed as a form of equal spacing of subcarriers. However, in this contribution, the PRS waveform structure proposed in [6][8] is preferred, rather than the staggered comb-N arrangement. A1, A2, … Am represent the initial phase sequence of each subcarrier that makes up the PRS signal. This initial phase sequence can be applied for purposes such as avoiding PAPR and can be removed if the receiver knows this sequence.

The PRS signal can be transmitted periodically at k times of a slot interval. If the initial phase sequences A1, A2, … Am for each iteration cycle remain unchanged, we can see that the same PRS waveform repeats every k times the slot length L, as shown in Equation (2).

(2)

PRSAt=PRSAt+kL ,   L=slot length

In the above (2), L denotes the length of a slot, and k is the PRS iteration cycle. The implication is that the phase value of the PRS signal measured at the time (t+kL) for any k value is equal to the phase value measured at time t. In other words, when applying the phase-based measurement method, which operates on a slot basis, all signals measured at different slot times can be considered to be measured at the same time t, hence the phase comparison can be made regardless of the slot location. Therefore, relative time can be measured more flexibly and simply than the RTT method.

The baseband PRS signal in Equation (1) above is mixed with the carrier signal of frequency wc at the RF frontend of the transmitter, and modulated into the passband signal in Equation (3) below.

(3)

PRSApasst=ejωct-ψA∙PRSAt=q=1mAqejωc+ωqt-ψA

The passband signal in (3) above reaches target UE-B after the tAB delay. This can be represented by Equation (4) below. To simplify the description, Equation (4) does not include a multipath channel expression, but just shows a single transmission delay on a line-of-sight path.

(4)

PRSAcht=e-jωc+ωqτAB∙PRSApasst=q=1mAqejωc+ωqt-ψA-τAB

At target UE-B, the received passband PRS signal is mixed with a carrier signal of reverse complex exponents to perform down-conversion, as shown in Equation (5). This process takes into account the local clock error ψB of target UE-B.

(5)

yBt=e-jωct-ψB∙PRSAcht=qAqejωcψB-ψA-τAB+ωqt-ψA-τAB

Target UE-B collects the down-converted baseband signal samples of Equation (5) above and decomposes them using the FFT process. Hence the PRS subcarriers of w1, w2, … wm is recovered. The recovered PRS subcarriers are shown in the vector form of Equation (6).

(6)

YB=N∙A1ejωc+ω1ψB-ψA-τABA2ejωc+ω2ψB-ψA-τAB⋮Amejωc+ωmψB-ψA-τAB+σB

The subcarrier vector YB in Equation (6) above contains both local clock error  (ψB-ψA) and path delay tAB in the exponent part of the complex number.  σB represents the residual error that remains after the FFT computation.

Note that the subcarrier vector YB includes the initial phase sequence A1, A2, … Am of the PRS signal sent by anchor UE-A. It can be removed if the UE-B knows the sequence. Equation (7) shows the calculated carrier phase value, obtained by removing the initial phase sequence value of A and then summing each complex value of the YB vector elements, and applying the arc-tangent function (i.e. angle function).

(7)

argsumYB≅argejωc+ω∆ψB-ψA-τAB=ωc+ω∆ψB-ψA-τAB-2πNcB

εAB=ψB-ψA-τAB=argsumYB+2πNcBωc+ω∆

In Equation (7) above, wc is the carrier frequency and wΔ is the median of the subcarrier array w1, w2, … wm.  (wc + wΔ) is a very large number (>108), but the arg(..) function returns only values between the range -π ~ +π. To fully recover the lost carrier phase value, it is necessary to estimate and compensate for the unknown integer number NcB. This is a well-known Integer Ambiguity problem [5].   [7][8] proposed a method of estimating an integer cycle number using a subcarrier wavelength that is much longer than the carrier wavelength. This can be calculated by applying the conjugate calculation to the subcarrier vector YB as in Equation (8) below. By applying the subcarrier phase measurement proposed in [7][8], the estimated integer number NcB can be applied to calculate the accurate εAB  value of Equation (7) above.

(8)

NcB=roundωc+ω∆ωgargsumYBYB*2π-argsumYB2π

Target UE-B applies Equation (7) above to inform anchor UE-A of the measured εAB value using a single-sided RTP message. A new reference signal, UL-RTPB, used for returning the phase value εAB  can be configured as shown in Equation (9) below.

(9)

UL_RTPBt=ejωcεAB∙q=1mBqejωqt-ψB+εAB

The UL-RTPB signal in Equation (9) above is a phase-rotated PRSB signal by the amount εAB value. This gives the effect of copying the phase of the received PRSA signal and returning it to anchor UE-A.

The UL-RTPB signal in Equation (9) reaches anchor UE device A through a carrier modulation process as described in Equations (3) through  (5). Therefore, the baseband signal reaches anchor UE-A after the (t+kL) slot time and is downconverted as shown in Equation (10).

(10)

yAt+kL=yAt=q=1mBqej-2τABωc+ωqt-ψA-2τAB

The subcarrier vector YA in Equation (11) below is obtained by collecting the demodulated baseband signal samples of Equation (10), applying the FFT, and extracting m subcarrier elements in the frequency domain.

Similar to the case of the PRS reference signal, if the anchor UE-A knows the initial phase sequence B1, B2, … Bm of the UL-RTPB signal, it can be removed.

(11)

YA=N∙B1e-jωc+ω12τABB2e-jωc+ω22τAB⋮Bme-jωc+ωm2τAB+σA

After removing the initial phase sequence value in the vector YA, the carrier phase value is calculated by summing the complex value of each element and applying the arc-tangent function (i.e. angle function) as  (12) below.

(12)

argsumYA≅arge-jωc+ω∆2τAB=-ωc+ω∆2τAB-2πNcA

∴ τAB=-argsumYA+2πNcA2ωc+ω∆

Similar to the case of Equation (7), Equation (12) above also contains an unknown integer number NcA.

It can also be resolved using the subcarrier method proposed in [7][8]. As a result, the anchor UE-A can calculate the path delay tAB with high accuracy.

Anchor UE-A may further implement a double-sided RTP procedure by encoding the calculated time delay information tAB into a DL-RTPA signal similar to Equation (9) and delivering it to target UE-B.

Figure 2 below illustrates a timing diagram of the single-sided RTP procedure between anchor UE-A and target UE-B.

Figure. 2 Timing  Diagram of Sing-Sided RTP Process Between Anchor UE and  Target UE

In Figure 2 above, the PRSA signal transmitted by anchor UE-A reaches target UE-B after the tAB time delay.  target UE-B calculates the phase information εAB of  the PRSA signal through the above equations (5) to (7) and uses it to configure the UL-RTPB reference signal.  The UL-RTPB signal is delivered to anchor UE-A. By applying equations (10) through (12),  anchor UE-A can finally obtain a time delay tAB that eliminates the local clock error.

Conclusion

This contribution briefly shows that the carrier phase method can successfully be applied for the sidelink positioning and achieves high accuracy path delay by removing the initial clock error.

The RTP procedure introduced herein is only one example to apply the carrier phase method to the V2X environment. There are other carrier phase applications, and solutions to explore for other use cases and environments such as V2V, V2I location finding, synchronization between Nodes, and for direction finding as with AoA, AoD, etc.

Based on this discussion, we suggest studying further on the feasibility of the carrier phase positioning for sidelink applications.

(Proposal 3) RAN1 should study the feasibility of carrier phase-based positioning for application to sidelink use cases.

Proposals

(Proposal 1) Introduce new terminology RTP (Return Trip Phase) to avoid any confusion with RTT when discussing the frequency domain method such as the carrier phase positioning.

(Proposal 2) Study new PRS signal efficiency for phase-based sidelink positioning methods such as carrier phase positioning.

(Proposal 3) RAN1 should study the feasibility of carrier phase-based positioning for application to sidelink use cases.

References

| [1] | draft TR38.859 v0.2.0, Study on expanded and improved NR positioning (Release 18), RAN1, December, 2022 |
| --- | --- |
| [2] | 3GPP R1-2104880 , "Carrier/Subcarrier Phase Based Enhancement for 5G NR Positioning " , Dankook University , RAN WG1 #105-e e-Meeting, May 10th – 27th, 2021 |
| [3] | 3GPP R1-2104844 , "Carrier Phase Based Downlink Angle of Departure Measurement" , Dankook University , RAN WG1 #105-e e-Meeting, May 10th – 27th, 2021 |
| [4] | 3GPP R1-2203634 , "Use cases and applications on Carrier Phase Based Positioning for NR" , Locaila, RAN WG1 #109-e e-Meeting, May 9 – 20, 2022 |
| [5] | Paul de Jonge, Christian Tiberius, “The LAMBDA method for integer ambiguity estimation : implementation aspects,” LGR-Series, Publications of the Delft Geodetic Computing Centre, August, 1996 |
| [6] | 3GPP R1-2203635, "Continuous PRS for improved carrier phase measurement" , Dankook University , RAN WG1 #109-e e-Meeting, May 9th – 20th, 2022 |
| [7] | 3GPP R1-2206227, "Solution for Integer Ambiguity, TRP synchronization and Vertical Positioning" , Locaila, RAN WG1 #110, Aug 22 – Aug 26, 2022 |
| [8] | 3GPP R1-2211259, "Experiment and simulation result on carrier phase based positioning" , Locaila, RAN WG1 #111, Nov 14 – Nov 18, 2022 |