3GPP TSG RAN WG1 #112                                                                            R1-2300766

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.11.4

Source:	NEC

Title:	On Improved GNSS Operations for IoT NTN

Document for:	Discussion

# Introduction

The release 18 work item on IoT support of non-terrestrial networks [1] includes the following objective:

Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed.

Regarding the possible enhancements, several agreements have been made in RAN1. However, some issues still require further study and, in this contribution, we provide our views on some of those aspects related to improved GNSS operations for IoT-NTN.

# Discussion

## Closed loop time and frequency correction

It is considered that closed loop time and frequency correction, with potential enhancements, for IoT-NTN can reduce the need for UE to update GNSS position fix in long connection time and, therefore, reduce UE power consumption. Unlike closed loop timing correction, closed loop frequency correction is not supported by legacy specifications. Previously, RAN1 discussed whether to support closed loop frequency correction via MAC CE command signalling. Based on the analysis provided by some companies in the previous meetings, it can be observed that the frequency offset error may not exceed tolerable range for low speed IoT UEs. Therefore, it is not necessary to spend standardisation efforts on specifying enhancements for closed loop frequency correction. Moreover, it is not possible to receive signalling timely when long UL repetition is configured. Therefore, closed loop correction mechanism is not needed for IoT NTN.

Proposal 1: Closed-loop correction during long UL repetition is not needed for IoT NTN.

## GNSS measurement triggering

On the issue of how to start a GNSS measurement in connected mode, following agreements have been made in RAN1 on GNSS measurement triggering:

Agreement (RAN1 109e)

Further study on whether there is a need for potential enhancements on the following for long connection time

- UE triggered GNSS measurement.

- Network triggered GNSS measurement.

Agreement (RAN1 110)

When eNB triggers UE to make GNSS measurements, UE re-acquires GNSS position fix

- FFS details of signalling

- FFS how UE reports GNSS assistance information after eNB trigger and the detailed content

Note: further discuss whether a UE is expected to handle all eNB triggers

In RAN1#110-bis meeting, following agreements were made for network triggered GNSS measurement signalling:

Agreement (RAN1 110-bis)

Support eNB to at least aperiodically trigger UE to make GNSS measurement.

Agreement (RAN1 110-bis)

If eNB aperiodically triggers UE to make GNSS measurement, a MAC CE is used.

In our understanding, eNB trigger for GNSS measurement must arrive at the UE before the expiry of GNSS validity duration. Since UE cannot receive downlink signalling during long uplink repetitions, which may last for several seconds or tens of seconds, periodic GNSS measurement can be useful in this case. Triggering periodic GNSS measurement may also be useful to reduce signalling overhead for UEs which may require frequent GNSS position fix e.g., high speed UEs.

Proposal 2: Support periodic GNSS measurement to avoid expiry of GNSS validity duration during long UL repetition and to reduce signalling overhead where possible.

Regarding signalling to trigger GNSS measurement, it is already agreed that MAC CE is used to trigger GNSS measurement aperiodically. Other options such as PDCCH order/DCI-based signalling were discussed in the last meeting. However, we think since MAC CE is already agreed, DCI-based solutions will involve larger specification impact. We think single MAC CE can be designed such that it can trigger both periodic and aperiodic GNSS measurement.

Proposal 3: Support MAC CE design such that it can trigger both aperiodic and periodic GNSS measurements.

We also think UE triggered GNSS measurement must be supported if the current GNSS validity duration changes and the UE has not received signalling from the network to acquire new GNSS position fix. Since simultaneous GNSS measurement and NTN NB-IoT/eMTC operation is not assumed, it is important that gNB and UE have a common understanding of when and for how long the UE is performing GNSS measurement so that the gNB does not transmit any RS, data or control information to the target UE. Therefore, UE should indicate to the gNB when it starts GNSS measurement. It has already been agreed that UE reports GNSS position fix time duration for measurement at least during the initial access stage. Although measurement time is not expected to change significantly, if it changes due to any reason (e.g. change in UE location) UE can indicate this along with the GNSS measurement trigger signalling.

Proposal 4: Support UE triggered GNSS measurement to cater for any changes in GNSS validity duration.

## Connected mode GNSS acquisition

In RAN1, following agreements have been made for potential enhancements of GNSS operation during a long connection.

Agreement

At least the following options can be considered on GNSS measurement in connected for potential enhancements for improved GNSS operations:

- Option 1: UE re-acquires GNSS position fix during RLF procedure

- Option 2: UE re-acquires GNSS position fix with a new gap

Note: this does not imply that a Rel-18 IoT NTN UE is mandated to support one or both of the options.

The issue of GNSS measurement in connected mode was also discussed extensively in the previous meetings. While most companies support configuring a measurement gap to perform GNSS position fix, some think that a timer based mechanism can be used as a fallback option if no trigger signalling is received after GNSS validity duration expires. Following was agreed in RAN1#111:

Agreement

For GNSS measurement in RRC connected, if eNB aperiodically triggers connected UE to make GNSS measurement, UE can re-acquire GNSS position fix with a gap

- FFS details of gap configuration

The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement

- FFS based on configured timing

In our understanding, the intention of configuring new timing is to prevent a UE from going to IDLE mode in case it doesn’t receive a command in time, and/or misses a command and GNSS validity has expired. We also think it is useful to define a fallback procedure if gNB driven procedure fails due to any reason or if it is unable to adapt to changing conditions (as mentioned in the previous section). Defining timer(s) can be one way for GNSS measurements to be autonomously triggered by the UE.

Proposal 5: The UE may re-acquire GNSS autonomously based on configured timing if it does not receive eNB trigger to make GNSS measurement.

However, it is also important that gNB is made aware when UE is performing GNSS measurements. Therefore, UE must send signalling to the gNB for common understanding between the UE and the network about GNSS measurement. Hence, timer(s) need to be configured such that the UE does not lose UL synchronization before it sends the relevant signalling to the gNB.

Proposal 6: Support signalling transmission to the gNB to inform about UE triggered GNSS measurement.

# Conclusion

From the discussion, we have following proposals.

Proposal 1: Closed-loop correction during long UL repetition is not needed for IoT NTN.

Proposal 2: Support periodic GNSS measurement to avoid expiry of GNSS validity duration during long UL repetition and to reduce signalling overhead where possible.

Proposal 3: Support MAC CE design such that it can trigger both aperiodic and periodic GNSS measurements.

Proposal 4: Support UE triggered GNSS measurement to cater for any changes in GNSS validity duration.

Proposal 5: The UE may re-acquire GNSS autonomously based on configured timing if it does not receive eNB trigger to make GNSS measurement.

Proposal 6: Support signalling transmission to the gNB to inform about UE triggered GNSS measurement.

# References

[1] RP-221806, Revised WID on IoT NTN enhancements, 3GPP TSG RAN Meeting #96, June 6-9, 2022.