3GPP TSG RAN WG1 #112                                         R1-2300804

Athens, Greece, February 27th – March 3rd, 2023

Source:         ZTE

Title:            Discussion on resource allocation for SL positioning reference signal

Agenda item:    9.5.1.3

Document for:	 Discussion and Decision

Introduction

In RAN#98-e meeting, various features have been recommended for normative work [1] for support of sidelink (SL) ranging/positioning. The following shows the expected enhancements for SL-PRS resource allocation in the WID:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2].

In this contribution, we provide our views and solutions for SL-PRS resource allocation including SL-PRS resource pool, SL-PRS configuration, SL positioning resource allocation and cast type of SL-PRS transmission.

Discussion

## 2.1 SL-PRS resource pool

For SL-PRS resources pool, according to the agreements made in RAN1#111, either dedicated resource pool and/or a shared resource pool with SL communication can be (pre-)configured for SL-PRS. In other words, companies are encouraged to study mechanisms and provide solutions for the support of both dedicated resource pool and shared resource pool.

Agreement in RAN1#111 With regards to the SL Positioning resource allocation, support - Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS. - Note: this does not imply that the design is the same for both types of resources pools - Note: shared resources pool(s) should be supported with backward compatibility

For SL-PRS resources pool, it is well known that large bandwidth is required for high-accuracy positioning if timing-based positioning methods are used, the larger the bandwidth, the higher the positioning accuracy. However, current sidelink RS in Rel-15/16 is always configured and limited within the bandwidth of PSSCH/SL-data resource pool. If the SL-PRS is configured similarly as current sidelink RS within the frequency range of PSSCH, it is difficult to satisfy the positioning requirement. Therefore, we need to consider solutions for large bandwidth SL-PRS configuration.

### 2.1.1 Dedicated SL-PRS resource pool

In Rel-15/16, NR sidelink resource pool is a set of time and frequency resources that can be used for sidelink transmission and/or reception. A UE can be (pre)configured by higher layer signaling with one or more sidelink resource pool. The hierarchical sidelink resource configuration is SL Frequency --> SL BWP --> SL resource pool according to TS 38.331 [2]. Moreover, in order to avoid the situation that one UE has to transmit or receive sidelink data on multiple BWPs simultaneously since groupcast and broadcast is supported, only one BWP is allowed to be configured on one carrier frequency for NR sidelink communication and this particular one BWP is used for both transmission and reception.

For dedicated SL-PRS resource pool design, we believe a carrier frequency or a BWP dedicated for SL-PRS can provide sufficient time-frequency resources for SL-PRS and thus results in high-accuracy positioning. However, As mentioned early, only one BWP and one carrier frequency is defined in Rel-16/17 SL communication. We may consider to separately specify frequency or BWP configuration for SL-PRS after completing specifying SL CA operation in NR sidelink evolution. Therefore, In Rel-18, we prefer dedicated SL-PRS resource pool associated with the BWP of a carrier that is the same as that of the SL communication resource pool. For resource pool level configuration as shown in Figure 2.1-1, the SL-PRS and SL-data belong to the same BWP but different resource pool configuration. Each pool/configuration can occupy a part of the BWP resource including time and frequency resources.

Figure 2.1-1: Dedicated SL-PRS configuration - resource pool level

A dedicated SL-PRS resource pool can be configured for reception of SL-PRS, or for transmission of SL-PRS which can be associated with either SL-PRS resource allocation scheme 1 or SL-PRS resource allocation scheme 2.

- Scheme 1 SL-PRS resource pool: indicate the resources by which the UE is allowed to transmit SL-PRS based on network scheduling on the configured BWP.

- Scheme 2 SL-PRS resource pool: indicates the resources by which the UE is allowed to transmit SL-PRS by UE autonomous resource selection on the configured BWP.

Dedicated SL-PRS resource pool configured by higher layer should at least include the following parameters:

- SL-PRS resource pool level configuration: time resource of resource pool, frequency resource of resource pool

- SL-PRS configuration: contains SL PRS resource set/ SL PRS resource configuration, for example: the period of SL PRS resource/SL PRS resource set, number of symbols that a SL PRS occupied in a slot, start symbol of SL PRS in a slot, SL PRS resource frequency location and bandwidth, SCS, comb size, muting pattern, SL-PRS QCL information, periodic/semi-persistent/aperiodic SL PRS, priority of SL PRS resource/SL PRS resource set

- Synchronization configuration: Indicates the allowed synchronization reference(s) which is (are) allowed to use the configured SL-PRS resource pool.

- Resource allocation scheme 2 related configuration: CBR config and its association relationship with SL-PRS transmission parameter, SL-PRS resource reservation period, sensing window, selection window, SL-PRS RSRP threshold

- Power control configuration: e.g., maximum SL-PRS transmission power, alpha value and P0 value for either sidelink pathloss based power control or downlink pathloss based power control.

One of the discussion points in previous RAN1 meetings is: can dedicated SL-PRS resource pool be multiplexed in a TDM manner with the resource pool for sidelink communication? In our understanding, the relationship between different resource pools is totally up to high layer configuration as in Rel-16/17. If a dedicated resource pool for SL-PRS multiplexed in a TDM manner with the resource pool for sidelink communication is applied, the positioning latency may be high. The distribution principle of reserved slots is discretely and evenly distributed in SFN/DFN cycles (10240 ms), the interval between two reserved slots might be quite large. Moreover, if SL-PRS resource pool is not overlapped with any SL communication resource pools to make sure pool level TDM, the available slots for SL-PRS are limited. Therefore, this type of SL-PRS resource pool design is more suitable for services that do not require high positioning latency or for UEs with low velocity. Moreover, UE can be (pre-)configured by high layers with one or more SL-PRS resource pools, and multiple UEs can be involved in SL communication. To make sure pool level TDM, SL-PRS resource pool should not be overlapped with any SL communication resource pool, which makes SL-PRS available resources even more restricted. As usual, even though resource pools are configured in non-TDMed manner by RRC signaling, the actual resource allocations can still be TDMed based on the proper network scheduling or UE sensing.

Proposal 1: For the support of dedicated SL-PRS resource pool:

- A dedicated SL-PRS resource pool can be associated with either SL-PRS resource allocation scheme 1 or SL-PRS resource allocation scheme 2

- A dedicated SL-PRS resource pool should at least include the following configuration: SL-PRS resource pool level configuration, SL-PRS configuration, synchronization configuration, resource allocation scheme 2 related configuration, power control configuration

- Pool level TDMed restriction by RRC signaling between SL-PRS resource pool and SL communication pool should be avoided.

The key point is to firstly discuss multiplexing between SL-PRS real transmission/reception and other SL signal rather than resource pool configuration. Due to the half-duplex restriction, the UE cannot transmit and receive sidelink resource at the same time. Furthermore, like Rel-16/17 design, it is better not to support SL-PRS transmitted or received with SL-data (sidelink data) at the same time for the purpose of low latency, low UE complexity and high measurement accuracy. Figure 2.1-2 shows one example of FDM between SL-PRS and SL-data which we do not prefer in this Release, we suggest to only consider TDM between SL-PRS and SL-data.

Figure 2.1-2 FDM between SL-PRS and SL-data

Proposal 2: In Rel-18, only TDM between SL-PRS transmission and SL-data transmission is supported.

Agreement in RAN1#110b-e At least for a dedicated resource pool for positioning, - With regards to the bandwidth of SL-PRS transmission, downselect from the following alternatives: - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool - Note: Companies are encouraged to provide their analysis and views on the above alternatives - FFS: Bandwidth of SL-PRS transmission for shared resource pool (if supported)

The bandwidth of SL-PRS should be able to flexibly configured depending on different positioning requirements and different positioning methods. Even though larger bandwidth brings higher positioning accuracy for timing-based positioning methods, different positioning scenarios may have different accuracy requirements. When the positioning accuracy requirement is not urgent, a smaller bandwidth compared to that of the resource pool can be applied to save frequency resource and further get power boosting gain. Moreover, for angle-based positioning methods, bandwidth is not a critical influence factor of accuracy. Therefore, we prefer Alt.1 in the above agreement, that is, the bandwidth of SL-PRS can be the same or smaller than that of the resource pool.

Proposal 3: At least for a dedicated resource pool for positioning,

- With regards to the bandwidth of SL-PRS transmission, the bandwidth of SL-PRS can be same or smaller than that of the resource pool for flexibility

Agreement in RAN1#110b-e For a dedicated resource pool for SL positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, consider the following options: - Opt. 1: No other channel can be included beyond SL-PRS - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included - FFS: Details - FFS: definition of PSSCH associated with SL-PRS transmission(s) - Note: Companies are encouraged to provide their analysis and views on the above

As shown in the above agreement, we discussed which channels can be included in the dedicated SL-PRS resource pool in RAN1#110b-e. For Option 2, dedicated SL-PRS resource pool includes PSCCH which carries SCI and for Option 3, dedicated SL-PRS resource pool includes both PSCCH and PSSCH associated with SL-PRS transmission(s). In our understanding, the choice between Option 2 and Option 3 depends on whether two-stage SCI is supported for SL positioning. Both options can cause a waste of resources since SCI also occupy a large bandwidth same as SL-PRS. Moreover, when number M of SL-PRS symbols within a slot excluding the symbol(s) used for AGC training / RxTx Turnaround is equal to or larger than 12, instead of reusing the slot structure of SL communication, the location of SCI/PSCCH for SL-PRS cannot be fixed in the beginning of a slot.

For Option 2, some companies raised that multiple SCIs from different UEs can be efficiently multiplexed in a FDMed manner in the OFDM symbol(s), and the SL-PRSs can be multiplexed with different comb offsets or even sequences which is also FDMed. However, we should note that UE is not able to support full duplex, i.e. based on this kind of FDM multiplexing, a UE cannot sense/measure the SCI and SL-PRS from another UE B since their SCI and PRS are transmitted simultaneously. Hence, this kind of design will cause resource waste and low measurement efficiency.

Observation 1: In option 2, multiple SCIs from different UEs cannot be multiplexed in FDMed manner because of half-duplexed restriction, i.e. UE cannot transmit SCI and sense/measure SCI from another UE B simultaneously. Hence, Option 2 will cause large SCI overhead.

Among those three Options, we prefer Option 1 for the purpose of low-overhead/saving-resource and support of 12 symbol of SL-PRS.

Proposal 4: For a dedicated resource pool for SL positioning, no other channel can be included beyond SL-PRS in the resource pool for the purpose of low-overhead/saving-resource and support of 12 symbol of SL-PRS.

### 2.1.2 Shared resource pool(s) with sidelink communication for SL-PRS

The following agreement related to shared SL-PRS resource pool was reached in RAN1#110b-e:

Agreement in RAN1#110b-e With regards to the SL Positioning resource allocation, for SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication (if supported), backward compatibility with legacy Rel-16/17 UEs should be ensured.

SL-PRS resources can be configured in SL communication resource pool, in other words, SL-PRS and SL-data transmission/reception share the same resource pools and same logic slots. However, as we clarified at the beginning of this section, large bandwidth is required for high-accuracy positioning. Therefore, extending SL-PRS outside the SL resource pool in terms of frequency domain as shown in Figure 2.1-3 should be considered to satisfy the positioning accuracy requirement.

Figure 2.1-3: Extending SL-PRS outside the shared resource pool in terms of frequency domain

In current SL specification, 1st stage SCI is carried in PSCCH and used for the scheduling of PSSCH and 2nd stage SCI on PSSCH. In order to support SL positioning, SCI can also be used to reserve and indicate SL-PRS resource(s). At least the following information is transmitted by means of SCI: priority of SL-PRS, SL-PRS resource time and frequency indication (e.g. reservation period), source ID, destination ID, cast type indicator. One Tx UE may transmit SL data to UE1 but transmit SL-PRS to UE2. In this case, destination ID, cast type or other configurations for SL communication and SL positioning may be different and a new 2nd stage SCI can be introduced.

Proposal 5: If SL-PRS shares resource pool with SL communication, extending SL-PRS outside the SL resource pool in terms of frequency domain without impact on backward compatibility should be supported to satisfy the positioning accuracy requirement.

2.2 SL-PRS configuration/triggering/activation/reservation

With regards to the configuration/activation/deactivation/triggering/reservation of SL-PRS, the following agreements were achieved in the previous RAN1 meetings.

Agreement in RAN1#109-e With regards to the configuration/activation/deactivation/triggering of SL-PRS, study the following options: - Option 1: High-layer-only signaling involvement in the SL-PRS configuration - No Lower layer involvement, e.g., SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL-PRS. - Based on the study, this option may correspond to - A SL-PRS configuration that is a single-shot or multiple shots - A high-layer configuration that may be received from an LMF, a gNB, or a UE - Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - For example, high layer signaling can may be used for SL-PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS. - Option 3: Only lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - Note 1: Include aspects in the study related to flexibility, overhead, latency, and reliability as/if needed. Agreement in RAN1#110 - With regards to the configuration/activation/deactivation/triggering of SL-PRS, Option 3 from the previous corresponding RAN1 #109 agreement will not be considered further. - With regards to reservation of SL-PRS, it can be considered based on the Option 1 or Option 2 from the previous corresponding RAN1 #109 agreement.

### 2.2.1 Periodic, semi-persistent or aperiodic SL-PRS

Agreement in RAN1#110b-e With regards to the SL-PRS time domain behavior, at least study the following behaviors from Tx UE perspective: - Periodic SL-PRS - SL-PRS is transmitted periodically with a transmission periodicity - FFS: any additional details, including whether or not higher layers can start/stop transmission. - Semi-persistent SL-PRS - SL-PRS is transmitted periodically with a transmission periodicity after activation and until deactivation - FFS: any additional details - Aperiodic SL-PRS - SL-PRS is transmitted at least once after [triggering/request] - Note: the brackets in the above means that companies are encouraged to study further whether “triggering” and/or “request” should be used and provide their definitions. - FFS: any additional details - FFS: Applicability of the above time behaviors for scheme 1 & scheme 2 - FFS: Rx UE behavior is separately discussed. - FFS: What mechanism(s) are used for activation/deactivation/triggering is part of the study

In our view, RAN1 should first study whether to support periodic SL-PRS, semi-persistent SL-PRS or aperiodic SL-PRS before discussing high-layer and/or lower-layer signalling involvement in SL-PRS configuration. Different SL-PRS time domain behaviour may result in different control signaling transfer procedures between UE, gNB and LMF. For periodic SL-PRS, we think it is workable for high-layer only signalling (e.g. LMF via LPP signaling or gNB via RRC signaling) for configuring and delivering periodic SL-PRS configuration to UE, this is more suitable for UEs in in-coverage or partial-coverage scenarios. For aperiodic SL-PRS, DCI/SCI can be used to trigger the SL-PRS transmission for more resource efficiency, this is more suitable for high traffic load scenarios.

For the brackets “[triggering/request]” in the above agreement, we think “triggering” is more suitable. Normally we say certain procedure (e.g., SL-PRS transmission, IUC report) is triggered by a request or other signaling or when certain conditions are met.

Proposal 6: RAN1 should at least support periodic SL-PRS and aperiodic SL-PRS.

  - Periodic SL-PRS

    - SL-PRS is transmitted periodically with a transmission periodicity

  - Semi-persistent SL-PRS

    - SL-PRS is transmitted periodically with a transmission periodicity after activation and until deactivation

  - Aperiodic SL-PRS

    - SL-PRS is transmitted at least once after triggering

### 2.2.2 High-layer and lower-layer signaling involvement

Our first priority is to support high-layer and lower-layer signalling involvement in the configuration/activation /triggering/reservation of SL-PRS. It can be used for aperiodic and semi-persistent SL-PRS where high layer signaling can be used for SL-PRS configuration and lower layer signaling can be used for SL-PRS triggering/activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS.

If sidelink resource collision happens, UEs need to be aware of other UE’s SL-PRS configuration via lower-layer signaling (e.g. SCI). In our understanding, using SCI to schedule/reserve/indicate SL-PRS resource(s) is preferred. A UE (e.g., UE-A) can use SCI to request or transmit SL-PRS resource reservation/configuration to other UEs (e.g., UE-B), then UE-B can detect SCI to further obtain the SL-PRS related information. For SCI configuration, there are two options: (1) each slot includes SCI and SL-PRS similar as SL communication slot design as shown in Figure 2.2-1. (2) one SCI may schedule multiple SL-PRS occasions for overhead reduction as shown in Figure 2.2-2. To be specific:

Option 1:

As shown in Figure 2.2-1, the SL-PRS is always transmitted along with SCI in the same slot. The characteristics of this option can be summarized as follows:

- A standalone/new SCI design is needed, which includes the time and frequency resource of SCI, information carried in SCI and the association between SCI and SL-PRS resource assignment.

- Bandwidth aspect: In a slot, SCI and SL-PRS is always transmitted in the same slot and apply a TDM manner. Large bandwidth is required for positioning accuracy and SL-PRS may occupy the whole bandwidth within one resource pool. In such case, SCI may have the same bandwidth with that of SL-PRS. However, this may result in a waste of frequency resources.

- Extra AGC or Rx-Tx turnaround symbol: Normally, the transmission power of SL-PRS is different from that of SCI, extra Rx-Tx turnaround or AGC symbol might be needed between the symbol of SCI and that of SL-PRS.

Figure 2.2-1 Each slot includes SCI and SL-PRS

Option 2:

As shown in Figure 2.2-2, SCI and SL-PRS are not always bounded in the same slot, wherein one SCI can trigger/reserve multiple SL-PRS occasions/samples. The characteristics of this option can be summarized as follows:

- Overhead reduction: As we described in Option 1, SCI and SL-PRS fixed and bounded in every slot can cause resource waste. Moreover, in order to support 1/2/4 samples for SL positioning and increase positioning accuracy, the 1/2/4 SL-PRS samples can share the same SCI. There is no need to have SCI for each sample for overhead reduction.

- High flexibility design of SCI: Either new SCI for SL positioning only or Rel-17 SCI design can be used. Single PSCCH/PSSCH may schedule multiple occasions of SL-PRS transmission. In a dedication SL-PRS resource pool, the SL-PRS could be indicated to a receiving UE using an SCI received in the same resource pool or some other resource pool. And the other resource pool could be another dedicated resource pool, or dedicated resource pool for SCI, or a SL communication resource pool or a shared resource pool.

- AGC or Rx-Tx turnaround symbol: we do not need to design AGC or Rx-Tx turnaround symbol in each slot between SCI and SL-PRS, thus more resources can be saved and used for SL positioning.

Figure 2.2-2 SCI triggers 4 SL-PRS samples

To sum up, one SCI scheduling multiple SL-PRS occasions is able to not only avoid resource waste, but also achieve high flexibility design of SCI. Moreover, as we discussed in Section 2.1.1, for the purpose of low-overhead/saving-resource and support of 12 symbol of SL-PRS, we prefer no other channel (e.g., PSCCH, PSSCH) can be included beyond SL-PRS in a dedicated SL-PRS resource pool. Therefore, SCI can be from other resource pools which could be dedicated resource pool for SCI or a SL communication resource pool, instead of the same resource pool as SL-PRS. We have the following proposal:

Proposal 7: At least support using SCI to reserve/indicate SL-PRS resource(s)

- One SCI may schedule multiple SL-PRS occasions for overhead reduction

- In a dedication SL-PRS resource pool, the SL-PRS could be indicated to a receiving UE using an SCI.

  - The SCI is from another resource pool which could be dedicated resource pool for SCI, or a SL communication resource pool.

### 2.2.3 High-layer-only signaling involvement

High-layer-only signaling involvement in the configuration/activation/triggering/reservation of SL-PRS is more suitable for periodic SL-PRS. However, it may cause resource waste and resource collision especially for sidelink resource selection scheme 2. If sidelink resource collision happens, UEs need to be aware of other UE’s SL-PRS configuration and how to inform the information without SCI need to be further studied.

Uu based positioning only supports periodic DL-PRS configuration, we may consider similar design for periodic SL-PRS or high-layer-only signaling involvement of SL-PRS in SL-PRS resource allocation scheme 1. For periodic SL-PRS configuration, since the LMF cannot decide the radio resources, the SL-PRS configuration is generated by each TRP in each NG-RAN node, and LMF gathers the SL-PRS configuration of all the TRPs (from all the NG-RAN nodes), then LMF may distribute the (probably revised) SL-PRS configuration to gNB or UE. Different TRPs within one NG-RAN node can resolve SL-PRS resource overlapping via muting pattern.

Observation 2: High-layer-only signaling involvement in the configuration/activation/triggering/reservation of SL-PRS is more suitable for periodic SL-PRS.

Proposal 8: If periodic SL-PRS or high-layer-only signaling involvement of SL-PRS is supported, UEs need to be aware of other UE’s SL-PRS configuration without SCI’s involvement to avoid resource collision.

## 2.3 SL positioning resource allocation

According to the agreements made in Rel-18 SI phase, both Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) and UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) are supported. For in-coverage and partial-coverage scenarios, at least one UE is inside the network coverage so both scheme 1 and scheme 2 resource allocation can be applied. For out-of-coverage scenario, the network cannot provide/determine the set of resources used for SL-PRS transmission, then only resource allocation scheme 2 can be applied.

Agreement in RAN1#110 Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging: - Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS. - Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS Agreement in RAN1#110b-e - Study further the granularity of time-domain resource allocation for SL-PRS transmission.

In current Rel-16/17 SL communication, the granularity of time-domain resource allocation for SL-data transmission is slot-based allocation. However, applying the same slot-based allocation for SL-PRS transmission may not be very resource efficient. In our companion TDoc [4], we discussed the frequency and time domain pattern of a SL-PRS resource. Specifically, regarding the value N (comb size) and the number M of SL-PRS symbols of a SL-PRS resource within a slot excluding the symbol(s) used for AGC training / RxTx Turnaround, an agreement was reached. With regards to the granularity of time-domain resource allocation for SL-PRS transmission, we suggest to use SL-PRS-resource-based allocation, where each SL-PRS resource includes M <=14 symbols. Furthermore, compared with slot-based SL-PRS allocation, resource-based allocation is more extensible to FR2 in which multiple resources with different beams may be multiplexed in the same slot or across some slots.

Moreover, AGC training symbol and Rx/Tx turnaround symbol should also be considered for SL-PRS resource allocation if one AGC training symbol and/or one Rx/Tx turnaround symbol is allocated per SL-PRS resource.

Proposal 9: With regards to the granularity of time-domain resource allocation for SL-PRS transmission

- Support SL-PRS-resource-based allocation, i.e. M <=14 symbols are configured for each SL-PRS resource

### 2.3.1 Scheme 1 SL-PRS resource allocation

The following agreement was achieved regarding scheme 1 SL-PRS resource allocation:

Agreement in RAN1#110b-e Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from the network. Consider one or more of the following options: - Opt. 1: through higher layers from the LMF - Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from gNB - Up to further discussion which one or more of these shall be applicable

For the design of NG-RAN UE SL positioning architecture for scheme 1 SL-PRS resource allocation, LMF should also be part of sidelink positioning to manage the support of different location services for target UEs. Especially when UEs in one positioning session have more than one serving NG-RAN nodes (e.g. gNBs), it is necessary to use LMF to interact with multiple NG-RAN nodes for assistance data information transfer, the capability of NG-RAN nodes transfer and location information transfer. However, current sidelink communication does not need LMF’s participation and clearly LMF does not have any sidelink related information. In order to support LMF’s involvement for sidelink positioning and hybrid positioning, sidelink related information transfer between LMF and gNB should be supported (Figure 2.3-1 shows one possible signaling procedure).

Figure 2.3-1: Sidelink related information transfer between LMF and gNB

For the periodic SL-PRS configuration (with shares the significant part of current LPP), there exists an issue. Different UEs may belong to different gNB’s reach, when serving gNB provides SL-PRS configuration to scheme 1 UE for transmitting, the scheme 1 UEs with different serving gNBs may possibly acquire SL-PRS configuration with resource conflict. This will lead to an issue that, UEs will have interface with each other when sending the SL-PRS according to the overlapping SL-PRS configuration.

For example, in Figure 2.3-2, UE 1 to UE m are all in coverage and in the same sidelink positioning session, in which, UE 1 and UE 2 have the common serving gNB as gNB 1, UE 3 has the serving gNB as gNB 2, and so on. UE 1 and UE 2 are configured to adopt scheme 1 and can receive the SL-PRS configuration from gNB 1, UE 3 is configured to adopt scheme 1 and can receive the SL-PRS configuration from gNB 2. UE 1 and UE 2 will not have resource conflict due to the serving gNB 1 can allocate well; however, UE 1/UE 2 and UE 3 will have SL-PRS resource conflict since gNB 1 and gNB 2 will not coordinate SL-PRS configuration of these three UEs. Under this situation, it is straightforward to support LMF to resolve the SL-PRS resource conflict between different scheme 1 UEs in the same sidelink positioning session.

Figure 2.3-2. Example of different scheme 1 UEs in the same sidelink positioning session under different gNB’s coverage

Observation 3: Different UEs may belong to different gNB’s reach, when serving gNB provides periodic SL-PRS configuration to scheme 1 UE for transmitting, the scheme 1 UEs with different serving gNBs may possibly acquire periodic SL-PRS configuration with some resource conflict.

Specifically, regarding how to resolve the SL-PRS resource conflict between different scheme 1 UEs in the same sidelink positioning session, we have the following solutions and corresponding analyses as shown in Table 2.3-1:

Table 2.3-1: LMF resolving SL-PRS resource conflict between different scheme 1 UEs

| Solution | Details |
| --- | --- |
| Solution 1: LMF to coordinate SL-PRS configuration of different gNBs | (1) LMF needs to send a request message via NRPPa to trigger each gNB to provide the SL-PRS configuration for scheme 1 UE (2) Each gNB provides the response message to LMF containing the configured SL-PRS configuration of each scheme 1 UE. (3a) LMF resolves the interference by LMF’s implementation, for example LMF deletes some of the overlapping SL-PRS resources of some of the gNBs. LMF provides the non-overlapped SL-PRS configuration to mode 1 UEs via LPP. (3b) LMF sends the feedback (e.g. collected all gNBs’ SL-PRS configuration, or muting pattern) to each gNB, gNB distribute the modified SL-PRS configuration to each scheme 1 UE. |
| Solution 2: LMF ask gNBs to coordinate with each other | (1) LMF sends a request message to gNB(s) and the request message is to request gNB(s) to coordinate SL-PRS configuration with other gNB (s) (2) The gNB sets up Xn interface with the other gNB(s) as requested by LMF, and the gNB sends the SL-PRS configuration of itself to other gNB via Xn interface. (3) After receiving one gNB’s SL-PRS configuration, the other gNB(s) can configure non-overlapped SL-PRS configuration. |

Among solution 1 and solution 2, we prefer solution 1 since solution 2 includes some Xn interface design where even the legacy Rel-16/17 positioning do not require the positioning information interaction between gNBs. Solution 1 only need to reuse the NRPPa procedure between LMF and gNB. For SL-PRS configuration delivery, there is an FFS in the RAN1#110 agreement saying that if NW is to allocate SL-PRS configuration to scheme 1 UE, whether the SL-PRS configuration should come from gNB and/or LMF. Considering the UE may usually be configured with periodic, aperiodic or semi-persistent SL-PRS together, we think a unified design is desired, that is, regardless of periodic/aperiodic/semi-persistent SL-PRS (SL-PRS time-domain behavior), serving gNB of the UE should configure the SL-PRS configuration for UE. In other words, among procedure (3a) and (3b), we prefer (3b).

In sidelink resource allocation mode 1 according to TS 38.214 [3], for SL data transmission, dynamic grant, configured grant type 1 and configured grant type 2 are supported.

- For sidelink dynamic grant, the PSSCH transmission is scheduled by a DCI format 3_0.

- For sidelink configured grant type 2, the configured grant is activated by a DCI format 3_0.

- For sidelink configured grant type 1, the PSSCH transmission follows the higher layer configuration.

Dynamic grant and CG type2, CG type1 in SL communication correspond to aperiodic SL-PRS, semi-persistent SL-PRS and periodic SL-PRS. In order to support mode 1 resource allocation for SL positioning, DCI can be used to dynamically or semi-persistently schedule the SL-PRS resource(s). We should consider whether a new DCI format is needed: (1) DCI format 3_0 can be reused and extra fields related to SL-PRS transmission should be added to DCI format 3_0. (2) A new DCI format exclusively for SL-PRS can be introduced, the new DCI (e.g., with a CRC scrambled by ‘SL-PRS-RNTI’) can be designed to indicate/select several SL-PRS resource configurations.

Proposal 10: Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from the network.

- Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from gNB

  - Support using DCI to dynamically or semi-persistently schedule SL-PRS and consider whether a new DCI format is needed.

  - Support LMF to resolve the SL-PRS resource conflict between different scheme 1 UEs in the same sidelink positioning session, downselect among the following solutions:

    - Solution 1: LMF coordinates SL-PRS configuration of different gNBs

    - Solution 2: LMF asks gNBs to coordinate with each other

### 2.3.2 Scheme 2 SL-PRS resource allocation

In SL-PRS resource allocation mode 2, UE selects the candidate resource(s) for SL-PRS transmission in the selection window based on the sensing results. The following agreements were approved in RAN1#110 and RAN1#111, wherein companies are encouraged to study resource selection mechanism for SL-PRS, Intel-UE coordination (IUC) and SL-PRS congestion control mechanisms.

Agreement in RAN1#110 Regarding Scheme 2 SL-PRS resource allocation, study at least the following aspects: - Resource selection mechanism for SL-PRS - Inter-UE coordination - Aspects for congestion control mechanisms for SL-PRS Agreement in RAN1#111 For Scheme 2, with regards to Resource allocation mechanism for SL-PRS, pick one or both of the following options: - Option 1: A sensing based resource allocation should be introduced - Option 2: A random resource selection should be introduced - In either option 1 or 2, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed.

For SL-PRS resource allocation mechanism in scheme 2, a sensing-based resource selection should be the baseline. A UE in scheme 2 should first sensing before transmitting SL-PRS in the selection window. Sensing is based on reservation information contained in the corresponding SCI.

Proposal 11: For Scheme 2, with regards to Resource selection mechanism for SL-PRS, at least a sensing-based resource selection should be introduced.

- Sensing is based on SCI.

If SCI trigged SL-PRS is applied, problems may exist when there are several SL-PRS occasions transmitting in the sensing window but the scheduling PSCCH/PSSCH is outside the sensing window. In such case, UE is not able to detect the PSCCH/PSSCH transmitted from other UE(s) and cannot obtain other UEs’ SL-PRS configuration and reserved resources in selection window. The UE may further transmit SL-PRS in the resources overlapping with SL-PRS#3 and #4 from the other UE(s). Consequently, the transmission collision happens. So, the situation shown in Figure 2.3-3 should be avoided where SCI triggering SL-PRS is outside the sensing window.

Hence, the sensing window size should be no less than the interval between two SCIs triggering SL-PRS to make sure at least one SCI is present in the sensing window.

Figure 2.3-3: No PSCCH/PSSCH in the sensing window

Proposal 12: If SCI trigged multiple SL-PRS occasions is applied, the sensing window size should be no less than the interval between two SCIs triggering SL-PRS.

Moreover, for scheme 2 UE autonomous selection of SL-PRS resources, whether the UE performs resource exclusion or not is not only related to the SL-PRS RSRP measurement and Tx priority (both UEs), but also related to whether the two UEs have positioning signaling interaction. For example, as long as two UEs have no positioning signaling interaction, UEs with different comb offset may transmit SL-PRS resources in the same slot/time. However, if two UEs have positioning signaling interaction, due to the half-duplex restriction, UEs with different comb offset cannot transmit SL-PRS resources in the same slot/time.

As shown in Figure 2.3-4, suppose the SL-PRS comb offset of UE 1 is 0, if UE1 detects in the sensing window that UE 2 (whose SL-PRS comb offset is 3) have time domain resource conflict, UE 1 and UE 2 cannot support an FDM manner for SL-PRS transmission in the selection window if they have to measure SCI/SL-PRS from each other.

Figure 2.3-4: SL-PRS resources with different comb offset multiplexed in the same slot

Proposal 13: FDMed PRS multiplexing between UEs having positioning serving interaction should NOT be supported.

For UE autonomous SL-PRS resource allocation, inter-UE coordination (IUC) information transfer for sidelink positioning among a number of UEs may be beneficial for maximum resource utilization and minimum resource conflicts. As shown in Figure 2.3-5, UE-A can determine (one or more) resource conflict or determine whether a set of resources are recommended for UE-B’s SL-PRS transmission. Then, the UE-B can take the SL-PRS IUC information for sidelink positioning into consideration before its transmission. Basically, Rel-17 Scheme 1 and 2 IUC procedure can be used as a starting point.

Figure 2.5-5: An example procedure for SL-PRS IUC

Proposal 14: Support SL-PRS IUC to maximize resource utilization and minimize resource conflicts.

- Use Rel-17 Scheme 1 and 2 IUC procedure as a starting point

[LOW] Feature Lead Proposal 5.2.2.D-v0 For Scheme 2, at least for the dedicated resource pool for positioning, with regards to congestion control mechanisms for SL-PRS - Study congestion metrics definition for SL-PRS (e.g. CBR and CR) - Study which parameters of a SL-PRS configuration could be impacted by the congestion control mechanism - Note: Companies are encouraged to study the above in conjunction to the resource allocation mechanisms for SL-PRS.

In the current specification for SL communication, sidelink congestion control is used in sidelink resource allocation mode 2. There are mapping relationship between PSSCH transmission parameter (such as MCS, PRB number, retransmission number, MaxTxPower, CR limit), CBR ranges and priority ranges. Similarly, congestion control mechanisms for SL-PRS can be studied after the design of SL-PRS resource allocation and configuration is determined. For example, during UE SL-PRS resource selection procedure, UE can measure SL-PRS CBR (definition can be further determined). Combining the CBR measurement result and the SL-PRS transmission priority, the corresponding transmission parameter for SL-PRS can be further determined.

Proposal 15: For Scheme 2, specify the congestion control mechanisms for SL-PRS after the design of SL-PRS resource allocation and configuration is determined.

- Congestion metrics definition for SL-PRS (e.g. CBR and CR)

- The mapping between congestion measurements, SL-PRS priority and SL-PRS transmission parameters.

2.4 SL-PRS cast type

Agreement in RAN1#110b-e From RAN1 perspective, the following cast types of SL-PRS transmission can be introduced for SL positioning: Unicast, Groupcast (not including many to one) - Broadcast (as a working assumption). - FFS: Applicability of the above cast types Agreement in RAN1#111 - Unicast, Groupcast (not including many to one) and Broadcast of SL-PRS transmission are recommended for normative work.

According to the discussion during RAN1#110b-e and RAN1#111, unicast, groupcast and broadcast of SL-PRS transmission are supported for SL-PRS transmission. Similar as the design in SL communication, we support all three cast types including unicast, groupcast and broadcast for SL positioning.

Unicast, which is a basic cast type for SL positioning. Two UEs can establish a one-to-one unicast connection with each other and the source ID and destination ID should be reported correspondingly. Unicast is more suitable for relative positioning, e.g., SL-RTT.

Groupcast, which is a more signaling-efficient way for SL positioning compared to unicast. For example, a target UE can groupcast its SL-PRS transmission to multiple anchor UEs within a group (e.g., SL-TDOA method). In this way, the target UE can avoid transmitting SL-PRS one by one and thus save overhead and increase efficiency.

Broadcast: More suitable for RSUs’ periodic SL-PRS transmission. RSUs can broadcast SL-PRS and other neighboring UEs can use the corresponding measurements to calculate its location.

For SL-PRS resource pool in Rel-18, SCI which can contain cast type and source/destination UE information is used to trigger/reserve SL-PRS resource. For example, if the cast type indicated in SCI is unicast, that means the SL-PRS reserved by this SCI is only expected to be decoded by a particular UE. The cast type can also be indicated as groupcast or broadcast, in such case, one source UE is associated with multiple destination UEs and multiple destination UEs is expected to correctly decode and measure the SL-PRS transmitted from the source UE.

For example, as shown in Figure 2.4-1, if SL-PRS is configured in a dedicated SL-PRS resource pool, if the cast type indicated in Tx UE’s SCI is unicast, Rx UE should determine if its destination ID is equal to Tx UE’s source ID, (and/or determine if its source ID is equal to Tx UE’s destination ID), if the condition can be met, then Rx UE can further process the SL-PRS resources associated with Tx UE’s SCI.

Figure 2.4-1: Filtering for unicast SL-PRS transmission

Alternatively, as shown in Figure 2.4-2, if SL-PRS is configured in a dedicated SL-PRS resource pool, if the cast type indicated in Tx UE’s SCI is groupcast and/or broadcast, Rx UE should determine if its destination ID is equal to Tx UE’s destination ID, if the condition can be met, then Rx UE can further process the SL-PRS resources associated with Tx UE’s SCI.

Figure 2.4-2: Filtering for groupcast/broadcast SL-PRS transmission

Proposal 16: Support using SCI to indicate the cast type of SL-PRS transmission:

  - For example, Rx UE uses the cast type and corresponding source/destination ID filtering to decide whether to process the SL-PRS associated with Tx UE’s SCI.

Conclusion

In this contribution, we discuss potential solutions for SL positioning, and we have the following observations and proposals:

Observation 1: In option 2, multiple SCIs from different UEs cannot be multiplexed in FDMed manner because of half-duplexed restriction, i.e. UE cannot transmit SCI and sense/measure SCI from another UE B simultaneously. Hence, Option 2 will cause large SCI overhead.

Observation 2: High-layer-only signaling involvement in the configuration/activation/triggering/reservation of SL-PRS is more suitable for periodic SL-PRS.

Observation 3: Different UEs may belong to different gNB’s reach, when serving gNB provides periodic SL-PRS configuration to scheme 1 UE for transmitting, the scheme 1 UEs with different serving gNBs may possibly acquire periodic SL-PRS configuration with some resource conflict.

Proposal 1: For the support of dedicated SL-PRS resource pool:

- A dedicated SL-PRS resource pool can be associated with either SL-PRS resource allocation scheme 1 or SL-PRS resource allocation scheme 2

- A dedicated SL-PRS resource pool should at least include the following configuration: SL-PRS resource pool level configuration, SL-PRS configuration, synchronization configuration, resource allocation scheme 2 related configuration, power control configuration

- Pool level TDMed restriction by RRC signaling between SL-PRS resource pool and SL communication pool should be avoided.

Proposal 2: In Rel-18, only TDM between SL-PRS transmission and SL-data transmission is supported.

Proposal 3: At least for a dedicated resource pool for positioning,

- With regards to the bandwidth of SL-PRS transmission, the bandwidth of SL-PRS can be same or smaller than that of the resource pool for flexibility

Proposal 4: For a dedicated resource pool for SL positioning, no other channel can be included beyond SL-PRS in the resource pool for the purpose of low-overhead/saving-resource and support of 12 symbol of SL-PRS.

Proposal 5: If SL-PRS shares resource pool with SL communication, extending SL-PRS outside the SL resource pool in terms of frequency domain without impact on backward compatibility should be supported to satisfy the positioning accuracy requirement.

Proposal 6: RAN1 should at least support periodic SL-PRS and aperiodic SL-PRS.

  - Periodic SL-PRS

    - SL-PRS is transmitted periodically with a transmission periodicity

  - Semi-persistent SL-PRS

    - SL-PRS is transmitted periodically with a transmission periodicity after activation and until deactivation

  - Aperiodic SL-PRS

    - SL-PRS is transmitted at least once after triggering

Proposal 7: At least support using SCI to reserve/indicate SL-PRS resource(s)

- One SCI may schedule multiple SL-PRS occasions for overhead reduction

- In a dedication SL-PRS resource pool, the SL-PRS could be indicated to a receiving UE using an SCI.

  - The SCI is from another resource pool which could be dedicated resource pool for SCI, or a SL communication resource pool.

Proposal 8: If periodic SL-PRS or high-layer-only signaling involvement of SL-PRS is supported, UEs need to be aware of other UE’s SL-PRS configuration without SCI’s involvement to avoid resource collision.

Proposal 9: With regards to the granularity of time-domain resource allocation for SL-PRS transmission

- Support SL-PRS-resource-based allocation, i.e. M <=14 symbols are configured for each SL-PRS resource

Proposal 10: Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from the network.

- Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from gNB

  - Support using DCI to dynamically or semi-persistently schedule SL-PRS and consider whether a new DCI format is needed.

  - Support LMF to resolve the SL-PRS resource conflict between different scheme 1 UEs in the same sidelink positioning session, downselect among the following solutions:

    - Solution 1: LMF coordinates SL-PRS configuration of different gNBs

    - Solution 2: LMF asks gNBs to coordinate with each other

Proposal 11: For Scheme 2, with regards to Resource selection mechanism for SL-PRS, at least a sensing-based resource selection should be introduced.

- Sensing is based on SCI.

Proposal 12: If SCI trigged multiple SL-PRS occasions is applied, the sensing window size should be no less than the interval between two SCIs triggering SL-PRS.

Proposal 13: FDMed PRS multiplexing between UEs having positioning serving interaction should NOT be supported.

Proposal 14: Support SL-PRS IUC to maximize resource utilization and minimize resource conflicts.

- Use Rel-17 Scheme 1 and 2 IUC procedure as a starting point

Proposal 15: For Scheme 2, specify the congestion control mechanisms for SL-PRS after the design of SL-PRS resource allocation and configuration is determined.

- Congestion metrics definition for SL-PRS (e.g. CBR and CR)

- The mapping between congestion measurements, SL-PRS priority and SL-PRS transmission parameters.

Proposal 16: Support using SCI to indicate the cast type of SL-PRS transmission:

  - For example, Rx UE uses the cast type and corresponding source/destination ID filtering to decide whether to process the SL-PRS associated with Tx UE’s SCI.

Reference

- RP-223549 New WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, Ericsson, 3GPP TSG RAN Meeting #98-e.

- 3GPP TS 38.331 h30: "Radio Resource Control (RRC) protocol specification (Release 17)".

- 3GPP TS 38.214 h40: "NR; Physical layer procedures for data (Release 17)".

- R1-2300802 Discussion on SL positioning reference signal