3GPP TSG RAN WG1 #112		                                R1-2301020

Athens, Greece, February 27th – March 3rd, 2023

Source: 	CMCC

Title:	Discussion on XR-specific capacity enhancements

Agenda item:	9.10.1

Document for:	Discussion & Decision

- Introduction

In the new WID on XR enhancements for NR approved in the last RAN #98-e meeting [1], the objectives of Rel-18 XR WI are given as the following:

Specify the enhancements related to power saving:

-	DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2).

Specify the enhancements related to capacity:

-	Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2);

-	Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE (RAN1);

-	BSR enhancements including at least new BS Table(s); (RAN2);

-	Delay reporting of buffered data in uplink; (RAN2);

-	Provision of XR traffic assistance information for DL and UL (e.g. periodicity); (RAN2);

-	Discard operation of PDU Sets (RAN2);

Specify the enhancements for XR Awareness (RAN2, RAN3): TBD (detailed objectives will be further clarified at RAN#99 based on the conclusions of TR38.835, and work to be started only after RAN#99)

In this contribution, we will further discuss the XR enhancements related to capacity from RAN1’s perspective, including discussions on multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration and discussions on the dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE.

- Discussion

2.1 Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration

Due to the quasi-periodic characteristic and low latency requirement of XR traffic, configured grant (CG) can be adopted to schedule the UL XR traffic. Compared with dynamic scheduling/grant (DG), CG can reduce the dynamic signaling overhead and transmission delay introduced by SR and/or BSR. Considering the large frame size of UL XR video traffic, it may require multiple TBs to transmit a video frame and one of the Rel-18 XR enhancement scopes is to  support multiple PUSCH transmission occasions in a CG period. Similar feature has been supported in Rel-16 NR-U and the corresponding description in TS 38.214 is copied as below.

6.1.2.3 Resource allocation for uplink transmission with configured grant … A set of allowed periodicities P are defined in [12, TS 38.331]. The higher layer parameter cg-nrofSlots, provides the number of consecutive slots allocated within a configured grant period. The higher layer parameter cg-nrofPUSCH-InSlot provides the number of consecutive PUSCH allocations within a slot, where the first PUSCH allocation follows the higher layer parameter timeDomainAllocation for Type 1 PUSCH transmission or the higher layer configuration according to [10, TS 38.321], and UL grant received on the DCI for Type 2 PUSCH transmissions, and the remaining PUSCH allocations have the same length and PUSCH mapping type, and are appended following the previous allocations without any gaps. The same combination of start symbol and length and PUSCH mapping type repeats over the consecutively allocated slots.

As shown in the above paragraph, the current specification allows the allocation of multiple consecutive slots within a CG period. However, in the scenario of TDD transmission with typical DL/UL slot configurations, e.g., DDDSU, the UL slots are non-consecutive in most cases. Therefore, to transmit the XR video traffic with large frame sizes in the TDD scenario, it should be proposed to allocate multiple non-consecutive slots within a CG period. As illustrated in Figure 1, there are 3 non-consecutive CG slots allocated within a period of a single CG configuration.

Figure 1. Illustration of multiple non-consecutive CG-PUSCH slots allocated in a CG period

Proposal 1. Support to configure multiple non-consecutive PUSCH slots/occasions within a CG period.

2.2 Dynamic indication of unused CG PUSCH occasion(s)

According to the outcome in clause 5.1 of TR 38.838 [2], the XR frame size of a single stream follows truncated Gaussian distribution where the average size is determined by data rate and frame generation rate, and the maximum size is 3 times the minimum size. While in the case of multiple streams of XR traffic, the ratio between the sizes of two different XR frames can be even larger. As provided in section 2.1, multiple CG PUSCH transmission occasions can be allocated in a period of a single CG PUSCH configuration. Considering the variable frame size of the XR video frame, the number of CG PUSCH occasions is usually semi-statically configured based on a relatively large frame size to avoid requiring additional dynamic scheduling to satisfy the PDB requirement, which will cause a waste of resources when XR frames with small sizes are transmitted. To handle this issue, dynamic indication of unused CG PUSCH occasions based on UCI by the UE is one of the scopes of Rel-18 XR enhancement WI.

To support dynamic indication of the unused CG PUSCH occasions, some issues need to be considered, e.g., how to align the UCI periodicity and XR traffic periodicity, how to guarantee the reliability of the transmission of UCI, etc. We will discuss these issues in the following sections.

2.1.1 Un-alignment issue of UCI occasion and XR periodicity/jitter

Whatever using UCI or CG-UCI to indicate the unused CG-PUSCH occasion(s), there will be a misalignment issue between the non-integer periodicity of XR traffic (e.g., 16.667ms) with the presence of jitter and the integer periodicity of a CG period/UCI transmission period. Thus, it’s possible that the UL XR video frame has not been prepared by UE on the UCI transmission occasion. In this case, UE may have no knowledge about the usage of PUSCH occasions in the associated CG period and the UCI sent by UE could not precisely indicate the unused CG PUSCH occasions. As illustrated in Figure 2 with system configuration of 30kHz SCS, DDDSU TDD configuration, and 15ms CG period, the UCI indicating the unused PUSCH occasions is periodically transmitted with the same periodicity with CG configuration, and the frame generation rate of XR traffic is 60fps, i.e., the packet arrival periodicity is about 16.67ms. Obviously, the CG periodicity or UCI transmission periodicity doesn’t align with the XR traffic periodicity. In some cases, UE has not yet acquired the actual data size of XR video frame at the UL slot when the UCI is transmitted.

One solution is that UE only sends UCI when the XR video frame is generated, however, gNB needs to detect UCI on all the possible UL slots which increases the gNB detection complexity. Another simple solution to indicate the usage of CG PUSCH occasions precisely and reduce the gNB detection overhead is that UE can re-send the UCI to override the inaccurate indication information in the previous UCI. For example, in Figure 2, the second UCI conveys the accurate information of the used CG-PUSCH occasions which can override the UCI transmitted in the first slot of the CG period. Another issue that needs further study is how to inform gNB of the “last UCI”, there are some methods, e.g., explicitly indicate the “last” in UCI content or define a time window of the “last UCI”.

Figure 2. Illustration of UCI overriding mechanism

Therefore, it is necessary to introduce a UCI overriding mechanism, which allows UE to send UCI again to override the inaccurate indication of the unused CG PUSCH occasions.

Proposal 2. It is necessary to introduce a UCI overriding mechanism to solve the misalignment issue of UCI occasion and XR periodicity/jitter, which allows UE to re-transmit UCI to override the inaccurate indication of the unused CG PUSCH occasions in previous UCI.

2.1.2 Reliability issue of UCI transmission

The above section discusses the situation where UE does not know the actual usage of CG PUSCH occasions when the UCI is transmitted. In this section, we will discuss the reliability of the transmission of UCI, so it is assumed that UE has known the accurate CG PUSCH occasions used to transmit the XR video frame and transmits the credible UCI content, i.e., the “last UCI”.

We assume a bitmap is used in the UCI to indicate which CG PUSCH occasions will be (or not be) used. However, when a detection error occurs in the UCI transmission, the incorrect indication to the gNB will lead to capacity degradation. Specifically, if the UCI indicates some of the CG PUSCH occasions will not be used by the UE while gNB fails to correctly detect the UCI, then these unused CG PUSCH occasions will be wasted. On the contrary, if the UCI indicates some of the CG PUSCH occasions will be used by the UE while gNB fails to correctly detect the UCI and releases the corresponding CG PUSCH occasions, then the transmission of this XR video frame would fail, which results in severe capacity loss.

To enhance the transmission reliability of UCI, the UCI could be endowed with the capability of self-check. For example, the UCI sent by UE can include two parts of indication information, where the first part indicates to the gNB whether there is at least one unused CG PUSCH occasion, and the second part indicates the specific usage of CG PUSCH occasions. In this way, after gNB detects the UCI sent by UE, it can determine the reliability of this UCI by the means of checking the two parts of indication information. For example, if the first part indicates that there is no unused CG PUSCH occasion while the second part indicates some CG PUSCH occasions are not used, that is, the two parts of indication information contradict each other, the gNB will be aware of the fact that there is detection error of UCI and discard the UCI. However, there are some cases that the detection error of UCI could not be distinguished by this method. Therefore, to further improve the transmission reliability of UCI, the UCI sent by UE can include one more part of indication information, which indicates the number of unused CG PUSCH occasions in the associated CG period. The three parts of indication information contained in the UCI would provide more a powerful capability of self-check.

Figure 3 illustrates an example of the dynamic indication of unused CG PUSCH occasions based on the UCI that contains two parts of indication information. It is assumed that the number of CG PUSCH occasions allocated in a CG period is six and the first four of them will be used to transmit the XR video frame, which means that two unused CG PUSCH occasions should be indicated by the UE. So, the first part of UCI indicates to the gNB that there exist unused CG PUSCH occasions and the second part uses a bitmap to indicate that the last two CG PUSCH occasions will not be used by the UE, where the bit “1” means “UE uses” while “0” means “UE does not use ”.

Figure 4 illustrates another example of the dynamic indication of unused CG PUSCH occasions based on UCI, where the UCI contains three parts of indication information. Specifically, the first part of UCI indicates to the gNB that there exist unused CG PUSCH occasions, the second part indicates that the number of unused CG PUSCH occasions is two, and the third part uses a bitmap to indicate that the last two CG PUSCH occasions will not be used by the UE.

Figure 3 Illustration of UCI that contains two parts of indication information

Figure 4 Illustration of UCI that contains three parts of indication information

Proposal 3 The UCI for dynamic indication of unused CG PUSCH occasion(s) could be endowed with the capability of self-check to improve its transmission reliability.

- Conclusions

In this contribution, the XR enhancements related to capacity are discussed from RAN1’s perspective, and the following proposals are made.

Proposal 1. Support to configure multiple non-consecutive PUSCH slots/occasions within a within a CG period.

Proposal 2. It is necessary to introduce a UCI overriding mechanism to solve the alignment issue of UCI occasion and XR periodicity/jitter, which allows UE to re-transmit UCI to override the inaccurate indication of the unused CG PUSCH occasions in previous UCI.

Proposal 3 The UCI for dynamic indication of unused CG PUSCH occasion(s) could be endowed with the capability of self-check to improve its transmission reliability.

- References

- RP-223502, “New WID on XR Enhancements for NR”, RAN#98e, Dec. 12-16, 2022.

- TR 38.838, “Study on XR (Extended Reality) Evaluations for NR”.