3GPP TSG RAN WG1 #112                                          R1-2301048

Athens, Greece, February 27th – March 3rd, 2023

Source: 	ETRI

Title:	Enhancements on cell DTX/DRX mechanism

Agenda Item:	9.7.2   Enhancements on cell DTX/DRX mechanism

Document for:	Discussion/Decision

Introduction

In RAN#98e, a new WI aiming network energy savings for NR was approved. The objective of the WI includes developing cell DTX/DRX mechanism which was identified as an efficient time-domain technique for NES during the SI as follows [1]:

- Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3]

- Note: No change for SSB transmission due to cell DTX/DRX.

- Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

RAN2 already has made some progress during the SI phase. They reached the following agreements in the last meeting [2]:

Agreements (RAN2#120) - Clarify previous agreement to: periodic cell DTX/DRX pattern is configured by UE-specific RRC. Periodic cell DTX/DRX can be activated/deactivated by L1/L2 signalling and UE-specific RRC signaling. - Capture in TR 38.864 that both UE specific and common L1/L2 signalling can be considered for at least activating/deactivating the cell DTX/DRX pattern, per the agreement in 119b-e. - Cell DTX and Cell DRX modes can be configured and operated separately (e.g. one RRC configuration set for DL and the other set for UL). Cell DTX/DRX can also be configured and operated together. - It is up to NW whether legacy UEs can access cells with Cell DTX/DRX - Cell DTX/DRX can be configured per serving cell and can be applicable for different cells in CA. No additional RAN2 impacts or enhancements are foreseen. - Whether to support multiple Cell DTX/DRX configurations can be discussed later in the normative phase. - At least the following parameters can be configured per cell DTX/DRX configuration: periodicity, start slot/offset, on duration. Details related to UE behaviour can be discussed during WI phase - From RAN2 perspective DTX/DRX is feasible

In this contribution, we discuss our initial view on issues in support of cell DTX/DRX mechanism for RRC connected UEs potentially related to RAN1.

Discussion

As one way to realize the network DTX/DRX operation, it was argued by some companies that it may be sufficient to make alignment of the legacy C-DRX configuration across UEs within a serving cell. By making all UEs to wake up and go to sleep based on a common ON-OFF duty cycle, gNB would sleep longer during the common OFF durations (i.e., inactive times). However, as the C-DRX is mainly oriented to optimizing PDCCH monitoring behaviours of RRC connected UEs, one of its limitations is that it cannot entirely control UE’s transmission and reception operation outside active time, for example, configured PUSCH and DL HARQ feedback. Those potential transmissions outside active time will prevent gNB to sleep a long time, which will diminish the energy saving gains. It is also not realistic to configure all DL/UL signals of every UE to be fall inside common DRX active times. Furthermore, the C-DRX is less flexible in view that UE doesn’t perform CSI-related transmission/reception behaviours outside active time.

To overcome limitations of the existing framework, the new Rel-18 cell DTX/DRX should be designed as a generalized mechanism to cover various operation scenarios as captured in the TR for examples [3]:

-  Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX/DRX non-active periods.

-	Example 2: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals)

-	Example 3: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS).

-	Example 4: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

Pursuing to provide high flexibility and controllability, RAN2 is discussing several issues including separation of cell DTX and cell DRX, multiple cell DTX/DRX modes, multiple cell DTX/DRX configurations, etc.

According to the RAN2 agreements, similar to the C-DRX, it is expected that cell DTX/DRX cycle will be provided by RRC, and the on-duration or the active time will periodically occur at each cycle. It was also agreed in RAN2 that UE-specific and common L1/L2 signalling can be considered for activation and deactivation of cell DTX/DRX patterns. We think that this part is what RAN1 can focus during the normative work.

Activation/deactivation signalling for cell DTX/DRX

In light of the extensive study results conducted during the Rel-16 WUS design, the group-common DCI is a good candidate signal for indicating dynamic activation or deactivation of cell DTX/DRX to a group of UEs. For clarity of discussion, it is important to share a common understanding what the activation/deactivation DCI means. For example, at least for cell DTX the following clarification can be considered as baseline.

- Activation DCI (i.e., Wake-Up Signal)

  - Indicates whether or not to wake-up at the next occurrence of cell DTX on-duration

  - Monitored outside active time (before on-duration)

- Deactivation DCI (i.e., Go-To-Sleep signal)

  - Indicates whether or not to go-to-sleep at the next occurrence of cell DTX off-duration

  - Monitored within active time (before off-duration)

Proposal 1: For cell DTX operation, support dynamic activation indication based on a group-common DCI. The activation DCI

- Indicates whether or not to wake-up at the next occurrence of DTX on-duration

- Is monitored outside DTX active time (before on-duration)

Proposal 2: For cell DTX operation, support dynamic deactivation indication based on a group-common DCI. The deactivation DCI

- Indicates whether or not to go-to-sleep at the next occurrence of DTX off-duration

- Is monitored within DTX active time (before off-duration)

In designing the activation DCI, the principles applied to the Rel-16 WUS can be reused to a large extent. Regarding the DCI format, one can introduce a new group-common DCI format, or refine the existing DCI format 2_6. We expect that this issue is highly related to whether to allow simultaneous cell DTX operation and legacy C-DRX operation from a single UE perspective.

On the contrary, there seems no L1 signalling that exactly matches the deactivation DCI. Although the DCI-based PDCCH monitoring skipping indication was introduced in Rel-17, the behaviours during PDCCH skipping duration and inactive time are quite different and in principle the PDCCH skipping operation is performed independently with the DRX on-off patterns. Rather, the deactivation DCI could be very similar to the DRX command MAC CE except that it is a L2 message and a unicast transmission.

There may exist commonalities between the activation DCI and the deactivation DCI for cell DTX, for example, target receiving UE group, payload size, search space set to be monitored except time domain location, etc. In such case, it may be efficient to use a single DCI format for both the activation and the deactivation purposes.

Another related aspect is whether/how to support cell DRX activation/deactivation. If supported, similar clarification may be brought for the activation DCI and the deactivation DCI as discussed above on the cell DTX case. How UE distinguishes the cell DTX (de-)activation DCI and the cell DRX (de-)activation DCI needs also to be studied. For example, different DCI formats, indicator field in the DCI, different RNTIs, different search space sets, etc. can be used to differentiate the DCIs for the cell DTX and the cell DRX.

Proposal 3: Further study the following aspects for dynamic activation and deactivation of cell DTX/DRX.

- Whether a new DCI format needs to be introduced (against reusing DCI format 2_6).

- Whether/how to support L1-based activation/deactivation for cell DRX.

- Total number of DCI formats for indicating cell DTX activation and deactivation, and potentially cell DRX activation and deactivation.

As aforementioned, it is desirable to support multiple UE transmission/reception behaviours during cell DTX/DRX inactive time (i.e., cell DTX/DRX modes). Each cell DTX mode can be associated with a set of DL signals UE receives/monitors (or, equivalently does not receive/monitor). Similarly, each cell DRX mode can be associated with a set of UL signals UE transmits (or, equivalently does not transmit). Dynamic switching between cell DTX modes or between cell DRX modes is a key feature to be distinguished from the legacy operation, and needs to be supported as well. For example, multiple cell DTX modes can be configured by RRC, and one of the cell DTX modes can be dynamically indicated by DCI. The cell DTX mode indication information can be included in the deactivation DCI.

Proposal 4: UE can adapt transmission/reception behaviours during cell DTX/DRX inactive time based on configuration of multiple cell DTX/DRX modes.

Proposal 5: Each cell DTX(/DRX) mode is associated with a set of DL(/UL) signals UE receives(/transmits) (or, equivalently does not receive(/transmit)).

Proposal 6: Deactivation DCI can indicate one of the cell DTX(/DRX) modes to apply during the next DTX(/DRX) inactive time.

Active time extension for cell DTX/DRX

In the UE C-DRX operation, UE extends the active time when additional transmissions are expected to occur. For example, the active time is extended when UE receives a DCI scheduling a new DL or UL transmission. In addition, UE regards time slots the DL/UL HARQ retransmission timer is running on as the active time. The same functionality should be guaranteed for Rel-18 UEs performing cell DTX/DRX operations. Two high-level approaches can be considered.

- Alt. 1: Specify the behaviour of extending the active time for cell DTX/DRX.

- Alt. 2: Configure the legacy C-DRX together with cell DTX/DRX. The active time is extensible based on the C-DRX framework.

Proposal 7: For cell DTX/DRX operation, discuss how to support active time extension when additional transmissions are expected.

DL HARQ-ACK feedback

It was agreed in RAN2 that cell DTX and cell DRX modes can be configured and operated separately. One implication of this agreement is that there may not be an available UL resource when UE should report PDSCH HARQ-ACK. In other words, the PUCCH resource for a PDSCH HARQ-ACK may fall outside the cell DRX active time.

One useful solution is to apply Type 3 HARQ-ACK codebook and enhanced Type 2 HARQ-ACK codebook introduced in Rel-16 NR-U. In NR-U, UE can be instructed to withhold a HARQ-ACK feedback by the non-numerical K1 value indication, and later can be triggered to report the pending HARQ-ACK(s). Those functionalities would work for the cell DRX operation as well. One difference is that in NR-U, gNB and UE do not know when the next channel occupancy will be available. However, in the cell DRX operation, the location of the next available UL resource (i.e., the next cell DRX active time) is known a priori based on configuration. It would be worth to study potential enhancements to increase the efficiency.

Proposal 8: Discuss how to prevent DL HARQ-ACK feedback during the cell DRX inactive time.

Conclusion

In this contribution, we discussed potential RAN1 issues on the introduction of cell DTX/DRX mechanism for NR network energy saving, from which we note the following proposals.

Proposal 1: For cell DTX operation, support dynamic activation indication based on a group-common DCI. The activation DCI

- Indicates whether or not to wake-up at the next occurrence of DTX on-duration

- Is monitored outside DTX active time (before on-duration)

Proposal 2: For cell DTX operation, support dynamic deactivation indication based on a group-common DCI. The deactivation DCI

- Indicates whether or not to go-to-sleep at the next occurrence of DTX off-duration

- Is monitored within DTX active time (before off-duration)

Proposal 3: Further study the following aspects for dynamic activation and deactivation of cell DTX/DRX.

- Whether a new DCI format needs to be introduced (against reusing DCI format 2_6).

- Whether/how to support L1-based activation/deactivation for cell DRX.

- Total number of DCI formats for indicating cell DTX activation and deactivation, and potentially cell DRX activation and deactivation.

Proposal 4: UE can adapt transmission/reception behaviours during cell DTX/DRX inactive time based on configuration of multiple cell DTX/DRX modes.

Proposal 5: Each cell DTX(/DRX) mode is associated with a set of DL(/UL) signals UE receives(/transmits) (or, equivalently does not receive(/transmit)).

Proposal 6: Deactivation DCI can indicate one of the cell DTX(/DRX) modes to apply during the next DTX(/DRX) inactive time.

Proposal 7: For cell DTX/DRX operation, discuss how to support active time extension when additional transmissions are expected.

Proposal 8: Discuss how to prevent DL HARQ-ACK feedback during the cell DRX inactive time.

References

[1] RP-223540, “New WID: Network energy savings for NR,” Huawei, RAN#98-e.

[2] Chair Notes RAN2#120.

[3] 3GPP TR 38.864, “Study on network energy savings for NR (Release 18),” V18.0.0.