3GPP TSG RAN WG1 #112	R1-2301050

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.8.2

Source: 	ETRI

Title:	Discussion on NCR signaling and procedures

Document for:	Discussion/Decision

Introduction

The following agreements on NR network-controlled repeater (NCR) signalling and procedure had been agreed in [1] and [2]:

Agreement For NCR-MT which can support adaptive beams in C link, - Rel-15 beam indication framework can be reused. - Rel-17 beam indication framework (i.e., the unified TCI) can be reused as well. The gNB can configure the unified TCI for the NCR-MT, if the NCR-MT supports. Agreement To support CSI measurement/reporting mechanisms for NCR-MT in C-link - The necessary legacy mechanism for receiving CSI-RS is reused for NCR-MT. - The necessary legacy mechanism for reporting CSI is reused for NCR-MT. - FFS: The details of the necessary mechanisms will be further discussed and decided. - Note: this does not mean all the legacy procedures for receiving CSI-RS and reporting CSI will be supported. Agreement HARQ-ACK feedback for PDSCH carrying the side control information from higher layer (e.g., MAC-CE, RRC) is supported. The legacy HARQ-ACK feedback mechanism is reused. - FFS: Whether HARQ-ACK feedback for PDCCH carrying side control information is supported - Note: This does not mean all legacy HARQ-ACK feedback mechanism will be supported. Agreement PUCCH and PUSCH are supported for NCR-MT. Agreement The TA adjustment mechanism of legacy UEs is supported for NCR-MT in C link. Agreement To support the sounding procedure for NCR-MT in C link, the necessary mechanism of legacy UE sounding procedure is supported. - FFS: The details of the necessary mechanism of legacy UE sounding procedure. - Note: This does not mean all legacy UE sounding procedure will be supported. Agreement As optional functionalities for the NCR-MT, at least Rel-15 legacy BFD/BFR/RLM mechanisms are supported - FFS: The behavior of NCR-Fwd when BFR/RLF happen in C link. Agreement - Legacy Rel-15 initial/random access procedure is supported for NCR-MTs in C link. - Note: No additional enhancement is necessary from RAN1 point of view. - The CRC bits of the PDCCHs carrying side control information are scrambled by a new dedicated RNTI - Applicable only for NCR-MT

This contribution presents ETRI’s views on remaining issues for NCR signalling and procedures.

Discussion

Beam indication

In RAN2#120 meeting, it was agreed that whether the NCR-Fwd can forward other cells is up to implementation as below [1].

Agreements gNB cell that NCR-Fwd is forwarding is the same cell the NCR-MT is connected to. Whether the NCR-Fwd can forward other cells is up to implementation

Given the situation, we think RAN1 have to clarify the level of frequency resources sharing the same beam indication from a cell the NCR-MT is connected to. From our understanding, the following options are available:

- Option 1: Beam indication from an SCI is applied to all passbands of the NCR-Fwd.

- Option 2: Beam indication from an SCI is applied to a subset of passbands of the NCR-Fwd.

  - Option 2-1: A passband in the subset of passbands is in the same band with the band containing the SCI.

  - Option 2-2: A passband in the subset of passbands is in the same cell group with the cell group containing the SCI.

  - Option 2-3: A passband in the subset of passbands is in the set of cells configured by gNB.

  - Option 2-4: A passband in the subset of passbands is in the frequency resources declared by repeater.

  - Option 2-5: A passband in the subset of passbands is in the frequency resources reported by repeater.

Proposal 1. RAN1 to down-select one of the following options to clarify the frequency resource applying the beam indication:

- Option 1: Beam indication from an SCI is applied to all passbands of the NCR-Fwd.

- Option 2: Beam indication from an SCI is applied to a subset of passbands of the NCR-Fwd.

  - Option 2-1: A passband in the subset of passbands is in the same band with the band containing the SCI.

  - Option 2-2: A passband in the subset of passbands is in the same cell group with the cell group containing the SCI.

  - Option 2-3: A passband in the subset of passbands is in the set of cells configured by gNB.

  - Option 2-4: A passband in the subset of passbands is in the frequency resources declared by repeater.

  - Option 2-5: A passband in the subset of passbands is in the frequency resources reported by repeater.

Power control

In RAN1#110b-e meeting, it was agreed to support the sounding procedure for NCR-MT in C link. Since it was also agreed that simultaneous UL transmission of C-link and backhaul link is NCR capability (i.e., it will be supported per the NCR capability signalling), we suggest to differentiate the NCR-MT’s power control on SRS. For instance, two different values of PCMAX, i.e., one for TDMed transmission of C-link and backhaul link and the other one for the non-TDMed (e.g., FDMed or SDMed) transmission of C-link and backhaul link, can be determined/configured by two different power classes considering the multiplexing modes. The same approaches can be applied to other uplink channels/signals, including PUCCH, PUSCH, etc.

Proposal 2. Support two different values of PCMAX, one for simultaneous backhaul link and C-link UL transmissions and the other one for TDMed C-link UL transmission.

Conclusion

In this contribution, ETRI’s views on L1/L2 signalling to carry the SCI were shown and the following proposals were made:

Proposal 1. RAN1 to down-select one of the following options to clarify the frequency resource applying the beam indication:

- Option 1: Beam indication from an SCI is applied to all passbands of the NCR-Fwd.

- Option 2: Beam indication from an SCI is applied to a subset of passbands of the NCR-Fwd.

  - Option 2-1: A passband in the subset of passbands is in the same band with the band containing the SCI.

  - Option 2-2: A passband in the subset of passbands is in the same cell group with the cell group containing the SCI.

  - Option 2-3: A passband in the subset of passbands is in the set of cells configured by gNB.

  - Option 2-4: A passband in the subset of passbands is in the frequency resources declared by repeater.

  - Option 2-5: A passband in the subset of passbands is in the frequency resources reported by repeater.

Proposal 2. Support two different values of PCMAX, one for simultaneous backhaul link and C-link UL transmissions and the other one for TDMed C-link UL transmission.

- References

- RAN1#110b-e Chair’s notes

- RAN1#111 Chair’s notes

- RAN2#120 Chair’s notes