3GPP TSG RAN WG1 #112	R1-2301071

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.8.2

Source: 	LG Electronics

Title: 	Discussion on other aspects for NCR

Document for:	Discussion and decision

Introduction

In this contribution, we discuss remaining other aspects of NCR including control plane signaling and procedures relevant to RAN1.

Support of legacy UE features for NCR-MT

In the RAN1#110bis meeting, it was agreed to support CSI measurement/reporting and sounding procedure for NCR-MT in C-link as follows [1]. However, the legacy procedures for CSI measurement/reporting and sounding procedures, which feature NCR-MT supports remain as FFS.

Agreement To support CSI measurement/reporting mechanisms for NCR-MT in C-link - The necessary legacy mechanism for receiving CSI-RS is reused for NCR-MT. - The necessary legacy mechanism for reporting CSI is reused for NCR-MT. - FFS: The details of the necessary mechanisms will be further discussed and decided. - Note: this does not mean all the legacy procedures for receiving CSI-RS and reporting CSI will be supported. Agreement To support the sounding procedure for NCR-MT in C link, the necessary mechanism of legacy UE sounding procedure is supported. - FFS: The details of the necessary mechanism of legacy UE sounding procedure. - Note: This does not mean all legacy UE sounding procedure will be supported.

In our view, the basic L1 feature required for the operation of NCR-MT may not be much different from that of IAB-MT. The discussion on the L1 mandatory features to be supported for IAB-MT was conducted in Rel-16 IAB WI, and Table 4.2.15.1-1 of TS38.306 shows the determined L1 mandatory features of the IAB-MT. Table 1 summarizes the feature groups of the L1 mandatory features for IAB-MT.

In order to discuss the support of legacy UE features for NCR-MT, we propose to consider these L1 mandatory features of IAB-MT as mandatory features in NCR-MT as a baseline.

From this point of view, in the case of CSI measurement/reporting and sounding procedure for NCR-MT, Feature group indices 2-32 (Basic CSI feedback) and 2-52 (Basic SRS) can be supported as mandatory features for NCR-MT.

Table 1. Layer-1 mandatory features for IAB-MT

| - Features | Index | Feature group |
| --- | --- | --- |
| 0. Waveform, modulation, subcarrier spacings, and CP | 0-1 | CP-OFDM waveform for DL and UL |
|  | 0-3 | DL modulation scheme |
|  | 0-4 | UL modulation scheme |
| 1. Initial access and mobility | 1-1 | Basic initial access channels and procedures |
|  | 1-3 | SS block based RLM |
| 2. MIMO | 2-1 | Basic PDSCH reception |
|  | 2-5 | Basic downlink DMRS for scheduling type A |
|  | 2-6 | Basic downlink DMRS for scheduling type B |
|  | 2-12 | Basic PUSCH transmission |
|  | 2-16 | Basic uplink DMRS (uplink) for scheduling type A |
|  | 2-16a | Basic uplink DMRS for scheduling type B |
|  | 2-22 | Aperiodic beam report |
|  | 2-32 | Basic CSI feedback |
|  | 2-50 | Basic TRS |
|  | 2-52 | Basic SRS |
| 3. DL control channel and procedure | 3-1 | Basic DL control channel |
| 4. UL control channel and procedure | 4-1 | Basic UL control channel |
|  | 4-10 | Dynamic HARQ-ACK codebook |
| 5. Scheduling/HARQ operation | 5-1 | Basic scheduling/HARQ operation |
| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation with restriction |
| 7. Channel coding | 7-1 | Channel coding |
| 8. UL TPC | 8-3 | Basic power control operation |

Proposal 1: For CSI measurement/reporting procedure for NCR-MT, feature group index 2-32 (Basic CSI feedback) is supported for NCR-MT.

Proposal 2: For the sounding procedure for NCR-MT, feature group index 2-52 (Basic SRS) is supported for NCR-MT.

Proposal 3: To support legacy UE features for NCR-MT, L1 mandatory features for IAB-MT are considered mandatory features for NCR-MT as a baseline.

The behavior of NCR-Fwd in the case of BFR/RLF/RLM

In terms of the BFD/BFR/RLM mechanism, it was agreed that at least Rel-15 legacy mechanisms are supported for NCR-MT [2]. According to the agreement, the behavior of NCR-Fwd when BFR/RLF happens in C-link remains as FFS.

Agreement As optional functionalities for the NCR-MT, at least Rel-15 legacy BFD/BFR/RLM mechanisms are supported - FFS: The behavior of NCR-Fwd when BFR/RLF happen in C link.

Meanwhile, following agreements were made in RAN2#120 regarding NCR-MT RLF. According to the agreements, NCR-Fwd is assumed to be OFF when RLF is happed in C-link. Therefore, it is no longer necessary to discuss the operation of NCR-Fwd in relation to the RLF operation of NCR-MT in RAN1.

NCR-MT RLF: - After RLF is declared by NCR-MT, NCR-MT performs cell selection and trigger RRC re-establishment; - If NCR-MT enters RRC_IDLE due to no suitable cell is find, NCR-Fwd is OFF; - During RRC re-establishment procedure, NCR-Fwd is OFF.

Regarding NCR-Fwd status when BFD is detected, one approach is that NCR-Fwd is OFF upon BFD detection by NCR-MT. Another approach is to leave it to NCR-Fwd implementation. The related discussion is underway in RAN2, so the decision can be made by RAN2

Proposal 4: NCR-Fwd behaviour when BFD is detected by NCR-MT can be decided by RAN2.

DCI format to carrying side control information

In terms of the DCI format for carrying side control information, it is not appropriate to reuse the legacy DCI format used for DL/UL data scheduling. Side control information is not information for data scheduling and is for a purpose completely different from the legacy DL/UL grant. In addition, there are lots of redundant fields that are not necessary to carry side control information in the DL/UL grant. Therefore, proposes that a new DCI format for carrying side control information is introduced.

Proposal 5: New DCI format for carrying aperiodic side control information is introduced.

According to the results of the discussion in our companion contribution, the following fields are required for the DCI that carries aperiodic side control information [3].

- Forwarding frequency resource index

This field indicates the index of forwarding frequency resource to which side control information is applied. As suggested in [5], to apply different side control information for each forwarding frequency resource of NCR-Fwd, information of a forwarding frequency resource to which a corresponding DCI is applied can be indicated in DCI carrying side control information. Forwarding frequency resources may be divided into carrier or carrier group units.

- Beam information field

A beam information field is used to indicate a beam index applied to the access link. To indicate multiple beam indices applied to different time resources, multiple beam information fields (i.e., Lmax beam information fields) can exist in a DCI.

Additionally, as suggested in our companion contribution, an indication of OFF state for NCR-Fwd can be combined to this field. For example, as in Table 2, a field value of ‘0’ may mean an OFF state, and other values may mean a specific beam index respectively.

Table 2. An example of a Beam/OFF indication field

| value | description |
| --- | --- |
| 0 | OFF |
| 1 | Physical access link beam index n0 |
| 2 | Physical access link beam index n1 |
| … | … |
| N | Physical access link beam index N-1 |

- Time domain resource field

A time resource indication field is used to indicate a time resource to which the indication by beam information field is applied. Lmax time resource fields are included in a DCI where Lmax is the number of beam information fields. Then, beam indication of the i-th beam information field is applied to the time resource indicated by the i-th time domain resource field.

Proposal 6: At least the following fields can be included in a DCI for carrying side control information.

- Forwarding frequency resource index

- Lmax Beam information fields

- Lmax Time resource fields

In a DCI for carrying side control information, Lmax beam information fields and Lmax time resource fields can be included. In this case, the DCI size may vary according to the value of Lmax. For NCR-MT to monitor PDCCH, it is necessary to know the exact DCI size.

The number of beam information fields and time resource fields to be included in a DCI may vary depending on the number of physical access link beams, or the number or traffic load of UEs supported by NCR. Therefore, rather than fixing the value of Lmax, it is preferable to make it configurable by the network.

Proposal 7: To determine DCI size for side control information, the value of Lmax can be configured by the network.

When there are Lmax beam information fields and time resource fields in a DCI, not all Lmax fields may be used for actual beam indication. When only L < Lmax beam indication fields are used for actual beam indication, the NCR-MT should be able to determine that the remaining Lmax - L fields are unused fields. For this purpose, it is necessary to make the time resource field can indicate ‘no time resource’. If it is indicated by a time resource field, the beam information field corresponding to the time resource field can be determined as an unused field.

Proposal 8: To indicate L < Lmax beam informations by a DCI, ‘no time resource’ can be indicated by the time resource field.

Summary

In this contribution, we discuss other aspects of NCR and obtained the following proposals.

Proposal 1: For CSI measurement/reporting procedure for NCR-MT, feature group index 2-32 (Basic CSI feedback) is supported for NCR-MT.

Proposal 2: For the sounding procedure for NCR-MT, feature group index 2-52 (Basic SRS) is supported for NCR-MT.

Proposal 3: To support legacy UE features for NCR-MT, L1 mandatory features for IAB-MT are considered mandatory features for NCR-MT as a baseline.

Proposal 4: NCR-Fwd behaviour when BFD is detected by NCR-MT can be decided by RAN2.

Proposal 5: New DCI format for carrying aperiodic side control information is introduced.

Proposal 6: At least the following fields can be included in a DCI for carrying side control information.

- Forwarding frequency resource index

- Lmax Beam information fields

- Lmax Time resource fields

Proposal 7: To determine DCI size for side control information, the value of Lmax can be configured by the network.

Proposal 8: To indicate L < Lmax beam informations by a DCI, ‘no time resource’ can be indicated by the time resource field.

Reference

- Chair’s note in RAN1#110-e meeting

- Chair’s note in RAN1#111 meeting

- LG Electronics, “Discussion on side control information and NCR behavior,” R1-2301070, Athens, Greece, February 27 – March 3, 2023