3GPP TSG-RAN WG1 Meeting #112	Tdoc R1- 2301076

Athens, Greece, February 27h – March 3rd, 2023

Agenda Item:	9.1.4.1

Source:	Ericsson

Title:	UL precoding indication for multi-panel transmission

Document for:	Discussion

# 1	Introduction

During RAN#94-e, a new WID for Rel-18 MIMO evolution for DL and UL was agreed. [1]. The objective of the work item concerning precoding for multi-panel UL transmission reads:

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

- UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission.

  - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

- UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation.

  - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

For completeness, the full objective is included. Parts that are not applicable for this AI are written in Italics.

In this contribution, we will provide a high-level overview of how the objective should be approached. We will also provide simulation results on the STxMP performance using the agreed EVMs. We will also discuss design aspects of an STxMP solution.

# 2	Discussion

UEs operating in FR2 are commonly equipped with multiple antenna panels, where each antenna panel can perform analog beamforming. The panel layout is up to UE design, and the selection of which panel to use for transmission is left to UE implementation. The UE selects one beam for reception and transmission in a panel of its choice. The NW must inform the UE from which direction it will receive the UL signals and configures the UE with a spatial relation or an UL TCI state.

Fundamentally, it would be beneficial if the UE could transmit using more than one panel, and there is nothing in the standard that prevents the UE from doing that: it could transmit identical signals from the two panels.

Simultaneous multi-panel transmission (STxMP) was discussed in Rel-16. The motivation was to investigate if it is possible to improve the performance compared to single-panel transmission or transparent transmit diversity. However, a UE that is capable of simultaneous transmission over multiple panels has high complexity, and STxMP was discarded early in the Rel-16 discussion and was never really considered due to the increased complexity of the UE implementation. Still in Rel-18, there are complexity concerns, and the gains for the feature must motivate that increased complexity.

RAN1 will specify the beam indication aspects in AI 9.1.1.1. The beam indication aspects would include a method to provide the UE with up to two UL TCI states. The mapping between an UL TCI state and a PUSCH transmission will also be part of the beam indication design.

The precoding aspects will be defined in AI 9.1.4.1. For codebook-based UL transmission, the precoder is selected from a codebook, and the signaled TPMI indicates which of the precoders in the codebook that the UE should use. For non-codebook-based UL transmission, the gNB determines the beams that the UE uses through SRI signaling.

## 2.1	Evaluation results

A remaining issue is whether two CWs will be supported for STxMP PUSCH in addition to one CW. The assumption was that performance would be better with two CWs, one per TRP/panel, since the MCS could be separately adjusted.

### 2.1.1	System-level simulation setup

To evaluate the performance of STxMP, system-level simulations have been performed according to the agreed EVM (the details are provided in the Appendix), a single-carrier FR2 system is considered. Two scenarios have been selected for the simulations: Indoor hotspot (InH) and dense urban.

The UE is equipped with two panels each with four dual-polarized antenna elements arranged in a ULA. The UE transmit power is 23 dBm per panel for the scenario. We’ve also simulated a total UE power of 23 dBm. We note that both values are unrealistically high.

The UE uses per-panel (it is assumed that coherency cannot be maintained over UE panels) SVD precoding to transmit up to two layers from each panel to each TRP.

### 2.1.2	System-level simulation results

The InH scenario is favorable for STxMP where low inter-site distance and pathloss allow transmission from both panels with high MCS to the receiving TRPs enabling up to rank-4 UL transmissions. The results comparing the performance with one and two CWs are depicted in Figure 1. Here, each UE panel has two ports (one per polarization) and per-UE power control is used.

Figure 1	Mean and cell-edge user throughput with STxMP for the InH scenario, one and two CWs, and with joint power control over the UE panels.

From Figure 1, we note that two CWs yields limited gains in terms of mean user throughput and that cell-edge user throughput gains are insignificant.

With per-UE power control, the throughput gains with two CWs compared to one CW is marginal.

Note that the simulations above are for per-UE power control. This means that the two panels use the same transmit power, which in turn means that the quality of the transmissions differ. In the RAN1#110bis-e meeting, it was agreed that two SRS resource sets (one per UE panel) is supported for STxMP, which could enable separate/independent power control for each panel. With per-panel power control, it is expected that the difference in performance between one and two CWs will be even less pronounced as SNR differences can be equalized (e.g., it is more likely that the same MCS is used over two CWs).

Figure 2	Mean and cell-edge user throughput with STxMP for the InH scenario, one and two CWs, and with separate power control per UE panel.

In Figure 2, we repeat the one versus two CWs comparison for the case when per-panel power control is used. We observe that 1) per-panel power control results in higher throughput compared to per-UE power control and 2) the performance difference between one and two CWs is insignificant in terms of both mean and cell-edge user throughput.

With per-panel power control, the benefits of two CWs compared to one CW is smaller than with per-UE power control.

We have also compared the performance of one and two CWs for the dense urban scenario, for the case of per-panel power control, and the results are shown in Figure 3.

Figure 3	Mean and cell-edge user throughput with STxMP for the dense urban scenario, one and two CWs, and with separate power control per UE panel.

As for the InH scenario, the performance gain with two CWs compared to one CW is not substantial for the dense urban scenario.

### 2.1.3	Concluding remarks

Currently in NR, one CW is used for up to four layers. In general, staying with a single-CW design is quite beneficial from a higher-layer point of view since there are no impacts to HARQ. The overhead also increases with an increased number of CWs. Finally, it is good to have a consistent rule when it comes to the relation between the number of layers and CWs, to limit the number of options. Based on this discussion, and the simulation results, we propose the following:

Specify only single-CW transmission for STxMP.

## 2.2	Design considerations for single-DCI based STxMP

The WID states that precoding for both single-DCI and multi-DCI mTRP should be considered. In this section, we discuss design cosideration for single-DCI based STxMP.

We have the following working assumption from RAN1#110:

Working assumption (RAN1#110)

Support the following scheme for STxMP PUSCH transmission in single-DCI based mTRP system:

- SDM scheme

- In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e.

During the RAN1#110bis-e meeting, it was agreed that SFN will be supported:

Agreement (RAN1#110bis-e)

Support SFN-based transmission scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18.

Therefore, with Proposal 1 in mind, we propose to:

Confirm the above working assumption with the following updates:

Support the following schemes for STxMP PUSCH transmission in single-DCI based mTRP system:

- SDM scheme,

- SFN scheme. In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e.

For both schemes, the number of CWs is 1.

The remainder of this section is organized as follows. in Section 2.2.1, we discuss the configuration of SRS resources over two SRS resource sets. In Section 2.2.2, we discuss dynamic switching between sTRP and mTRP transmission. In Section 2.2.3, we discuss the maximum number of layers for STxMP. Finally, in Section 2.2.4, we discuss aspects related to DMRS and PTRS configuation.

### 2.2.1	SRS resource/resource set configuration

Up to Rel-16, a single SRS resource set is used to select which precoder to use for the scheduled PUSCH. In Rel-17, an additional SRS resource set was introduced to handle PUSCH repetition over two TRPs.

#### 2.2.1.1	SDM STxMP

For SDM STxMP, in the RAN1#110bis-e meeting, the following was agreed:

Agreement (RAN1#110bis-e)

For SDM scheme of single-DCI based STxMP PUSCH.

- Configure two SRS resource sets for CB or NCB.

  - FFS: These two SRS resource sets can have different number of SRS resources for codebook-based or non-codebook based.

- For codebook-based PUSCH, DCI indicates two TPMI fields, and each TPMI field separately indicates the precoding information and the number of layers conveyed over the SRS ports of the indicated SRS resource in each SRS resource set.

- For non-codebook based PUSCH and codebook-based PUSCH, DCI indicates two SRI fields, and each field indicates SRS resource(s) for each SRS resource set separately.

  - FFS: For codebook-based PUSCH, the two SRS resources indicated by the two SRI fields can have different number of SRS ports.

According to the above agreement, two open questions are whether SRS resource sets can have different number of SRS resources and if SRS resources can have different number of ports.

Multi-panel UEs may use panels with a different number of transmit chains. For this reason, in Rel-17, new UE-capability signaling was added to report up to four UE capability values, where each value comprises a number (1, 2, or 4) of SRS ports. A pointer to a UE capability value can be included for each reported beam in a beam report, which informs the NW about the number of SRS ports that is associated with that beam. Such UEs should be supported for SDM STxMP as well, which entails that number of SRS ports (and, hence, the maximum number of layers) can vary over SRS resource sets.

UE panels with different capabilities should be supported for codebook-based PUSCH, which implies that the number of SRS ports in SRS resources belonging to different SRS resource sets can be different. Hence, we propose:

For two SRS resource sets with usage ‘codebook’, support SRS resources belonging to different SRS resource sets being configured with different number of SRS ports.

Regarding the number of SRS resources per SRS resource set, for non-codebook-based PUSCH, only single-port SRS resources are supported. Furthermore, the number of SRS ports per SRS resource set is determined by the number of (single-port) SRS resources in that SRS resource set. Hence, supporting UE panels with different number of transmit chains implies that the number of SRS resources per SRS resource set can be different.

For codebook-based PUSCH, it is already supported in the standard to configure up to two SRS resources, with an equal number of SRS ports, per SRS resource set (except for full-power mode 2, for which up to four SRS resources with different number of SRS ports per SRS resource set is possible):

Excerpt from 3GPP TS 38.214, Clause 6.1.1.1

Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', when multiple SRS resources are configured by SRS-ResourceSet with usage set to 'codebook', the UE shall expect that higher layer parameters nrofSRS-Ports in SRS-Resource in SRS-ResourceSet shall be configured with the same value for all these SRS resources.

This allows the NW to measure the channel via up to two different virtualizations per SRS resource set and to indicate a preferred virtualization, which should be supported for STxMP as well.

Based on the above, we propose:

For SRS resource sets with usage ‘codebook’ or ‘nonCodebook’, support different number of SRS resources per SRS resource set.

An additional reason for supporting SRS resource sets with different number of ports/resources is that different TRPs may want to schedule different number of SRS ports.

#### 2.2.1.2	SFN STxMP

For SFN STxMP, there is a similar discussion regarding the number of ports per SRS resource and on the number of SRS resources in different SRS resource sets. In the RAN1#110bis-e meeting, the following was agreed:

Agreement (RAN1#111)

For the SFN scheme of single-DCI based STxMP PUSCH:

- Configure two SRS resource sets for CB or NCB.

  - FFS: Number of SRS resources of SRS resource set, and number of SRS ports of SRS resource

- The DCI indicates two SRI fields and TPMI fields for SFN transmission,

- On the indication of number of layers for CB and NCB PUSCH:

  - Alt1: Similar to Rel-17 mTRP TDM scheme, the number of layers is indicated by the first SRI field (for NCB PUSCH) or the first TPMI field (for CB PUSCH)

To support UE panels with different capabilities, SRS resource sets with different number of SRS ports (for codebook) and different number of SRS resources (for codebook and non-codebook) should be supported, not only for SDM STxMP, but for SFN STxMP as well:

For SFN, the number of SRS ports per SRS resource and the number of SRS resources per SRS resource set is the same as for SDM and can vary over SRS resource sets.

It follows that the maximum number of layers for SFN is upper bounded the smallest number of SRS ports over the two SRS resource sets.

### 2.2.2	Dynamic switching between transmission modes

For Rel-17 mTRP PUSCH repetition, dynamic switching between mTRP and sTRP PUSCH operation is supported by two SRS resource sets and through the introduction of a 2-bit SRS resource set indicator field in DCI, for which the first two codepoints indicate sTRP transmission and the last two codepoints indicate mTRP transmission:

- The difference between the first two codepoints is which of the two SRS resource sets that is associated with a PUSCH transmission, i.e., if the transmission is to a first or a second TRP.

- The difference between the last two codepoints is towards which TRP a first PUSCH repetition is transmitted, i.e., if the first or second SRS resource set is associated with a first slot.

It has been agreed that the SRS resource set indicator field will be used also for dynamic switching between SDM STxMP and sTRP PUSCH and between SFN STxMP and sTRP PUSCH. However, according to the above agreements, switching between SDM STxMP and SFN STxMP is not supported:

Agreement (RAN1#111)

- Support DCI-based dynamic switching between SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission.

  - The DCI field “SRS resource set indicator” is used to indicate the switching between SFN scheme and sTRP transmission.

- (Conclusion) There is no consensus to support DCI-based dynamic switching between SFN scheme and SDM scheme.

It was discussed in RAN1#110bis-e whether dynamic switching between Rel-18 STxMP and Rel-17 mTRP PUSCH repetition should be supported. With STxMP, it is possible to transmit to both TRPs at the same time. SFN STxMP can increase reliability by transmitting simultaneously the same layer(s) to multiple TRPs. Rel-17 mTRP TDM, on the other hand, is designed to increase reliability for UEs that cannot transmit simultaneously on two panels. Hence, it is not clear what is the advantage of supporting dynamic indication of Rel-17 mTRP TDM repetition in addition to dynamic indication of SFN STxMP:

Do not support dynamic switching between STxMP and Rel-17 mTRP PUSCH repetition.

Since dynamic switching between SDM STxMP, SFN STxMP, and Rel-17 mTRP repetition, according to Proposal 6, will not be supported, and since all these schemes are configured with two SRS resource sets. RRC signaling is required to switch between these schemes, and the fields present in DCI Format 0_1 and 0_2 will depend on this RRC signaling, as discussed next.

For mTRP PUSCH repetition, as described above, two codepoints for mTRP operation are needed because it matters in which order PUSCH is associated with the SRS ports belonging to the first and second SRS resource set. For STxMP operation, PUSCH is transmitted over the SRS ports associated with both SRS resource sets at the same time and, hence, the ordering is irrelevant and one codepoint is enough to indicate SDM STxMP operation (i.e., three codepoints in total are needed for dynamic switching between SDM STxMP and sTRP PUSCH). The same is true for SFN STxMP: three codepoints in total are needed for dynamic switching between SFN STxMP and sTRP PUSCH.

One codepoint in SRS resource set indicator field is sufficient to indicate SDM STxMP or SFN STxMP.

For SFN STxMP, the number of layers indicated for the first SRS resource set determines the number of layers for the second SRS resource set. Hence, the legacy “Second precoding information” and “Second SRS resource indicator” field in DCI Format 0_1 and 0_2 can be used can be used for both Rel-17 mTRP repetition and SFN STxMP.

For SDM STxMP, on the other hand, the rank should be separately indicated for the two SRS resource sets, which requires a new field in the scheduling DCI:

Introduce “Second precoding information and number of layers” field in DCI Format 0_1 and 0_2 for indicating both precoder and rank for codebook-based SDM STxMP.

Furthermore, when SDM STxMP is configured, the “SRS resource set indicator” field in the scheduling DCI associates one or both of “Precoding Information and number of layers” and “Second precoding Information and number of layers” fields with one or both SRS resource set(s).

For non-codebook-based SDM STxMP, the bit width of “Second SRS resource indicator” field in DCI Format 0_1 and 0_2 needs to be increased to support indication of both precoder and rank.

### 2.2.3	Supported number of layers

During the RAN1#111 meeting, the following was discussed regarding the maximum number of layers for SDM STxMP and the maximum number of layers when switching from SDM STxMP to sTRP PUSCH:

Agreement (RAN1#111)

For dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission:

- Use the 2-bit “SRS resource set indicator” DCI field to dynamically indicate the sTRP or SDM transmission.

- FFS: how to interpret each codepoint of “SRS resource set indicator”.

- For the maximal number of layers for sTRP transmission, down-select:

  - Option1: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec.

  - Option2:  Configuring one additional maximal numbers of layers for sTRP transmission, in addition to maxRank in current spec.

- Down-select one from the following for maximal number of layers of SDM transmission:

  - Alt1: Configure one single maximal number of layers (separate from the maximal number(s) of layers for sTRP), that is applied to the first SRS resource set and the second SRS resource set, separately.

  - Alt1a: The maxRank (or Lmax) in current spec is also applied to the first SRS resource set and the second SRS resource set, separately.

  - Alt2: Configure separate maximal numbers of layers for the first SRS resource set and the second SRS resource set.

  - Alt3: no dedicated configuration for SDM. The maximal number(s) of layers of sTRP and the UE capability reporting for SDM are used to determine the maximal number of layers of SDM transmission.

  - Alt4: The maximal number(s) of layers in above Option1/2 also applied to the first SRS resource set and the second SRS resource set, separately.

- FFS:  To ensure the same size of DCI for sTRP and SDM cases, how to use/interpret the TPMI/SRI field(s) and whether to do reserved bit or zero padding.

The parameter maxRank in current specification is used for determining the maximum PUSCH rank for sTRP transmission. We do not see the need for separate requirements on maximum number of PUSCH layers for sTRP transmission depending on whether the sTRP transmission is part of legacy sTRP configuration or part of STxMP configuration. Hence, we propose:

Support Option1 for sTRP transmission, i.e., the maximal number of layers for sTRP transmission is configured by the maxRank (or Lmax) in current specification.

For SDM STxMP transmission, the maximum rank per TRP can differ from the maximum rank during sTRP transmission. One reason for this is that it has been agreed that maximum two layers per TRP will be supported for SDM STxMP transmission, while sTRP transmission may support up to 4 layers to a single TRP. Another reason is that, for SDM STxMP, fewer number of layers might be required per TRP compared to sTRP transmission. For example, it could be that NW configures a maximum rank of 2 for sTRP transmission, while the maximum rank per TRP for SDM STxMP is set to 1. Hence, there is a need to configure a separate rank restriction for SDM STxMP and, therefore, we propose

Support Alt1 for SDM STxMP transmission, i.e., configure one single maximal number of layers (separate from the maximal number(s) of layers for sTRP), that is applied to the first SRS resource set and the second SRS resource set, separately.

For SFN STxMP, if up to rank 4 is supported, the maximum rank for sTRP and SFN STxMP is the same and the legacy maxRank parameter can be used. If up to rank 2 is supported, the same parameter that is used to configure the maximum rank per TRP (i.e., per SRS resource set) for SDM STxMP can be re-used.

### 2.2.4	DMRS port indication

On DMRS port indication, the following was agreed in RAN1#111:

Agreement (RAN1#111)

For SDM scheme single-DCI based STxMP transmission, when L1 and L2 layers are indicated/determined by two TPMI fields of CB PUSCH or two SRI fields of NCB PUSCH respectively:

- The first L1 indicated DMRS ports correspond to the L1 layers indicated by the first TPMI or SRI field

- The remaining L2 indicated DMRS ports correspond to the L2 layers indicated by the second TMPI or SRI field

- Support at least one of the following options for indication of layer combination {1+2}:

  - Option 1: new entry is added to DMRS table, e.g., {0, 2, 3}, {2, 0, 1}.

  - Option 2: use DCI field (e.g., SRS resource set indicator) to indicate that for layer combination {1+2}, the first two indicated DMRS ports correspond to the 2 layers indicated by the second TPMI or SRI field and the rest one indicated DMRS port correspond to the layer indicated by the first TPMI or SRI field.

  - Option 3: For layer combination of {1+2}, the DMRS port in the CDM group with only one port is mapped to the SRI/TPMI field indicating one layer, and the DMRS ports in the CDM group with 2 ports are mapped to the SRI/TPMI field indicating 2 layers

  - Other options are not precluded

Furthermore, it was agreed that DMRS ports associated with two SRS resource sets can be in the same of different CDM groups:

Agreement (RAN1#111)

For the DMRS port indication for SDM scheme single-DCI based STxMP transmission, support Alt2:

- Alt2: the DMRS ports associated with two TPMI/SRI fields can be in same or different CDM groups.

#### 2.2.4.1	DMRS ports for rank-3 CP-OFDM

An open question is how to indicate DMRS ports for layer combination {1+2} to support that DMRS ports  associated with different SRS resource sets can be mapped to different CDM groups. As we shall discuss next, it is preferred that DMRS port indication supports a mapping from DMRS ports associated with an SRS resource set to CDM groups that does not vary under rank adaptation.

In Figure 4, we show an STxMP UE configured with rank {2+2} that is co-scheduled with legacy UEs in a mTRP system for which the rank per TRP is 4. For single-symbol type-1 DMRS the DMRS ports for the STxMP UE are {0, 1, 2, 3} where ports {0, 1}, which belong to CDM group 0, are transmitted from the first UE panel to the first TRP and ports {2, 3}, which belongs to CDM group 1, are transmitted from the second UE panel to the second TRP. Hence, CDM group 1 is available to the first TRP and CDM group 0 is available to second TRP. Thus, for each serving TRP, there is an empty CDM group for each TRP that can be used to serve other UEs (configured with same/different DMRS sequence) such that all DMRS ports (both CDM groups) are occupied for both TRPs.

Figure 4	DMRS ports for rank-4 STxMP UE co-scheduled with non-STxMP UEs.

In Figure 5, we show the same mTRP system for the case when the STxMP UE, e.g., due to varying channel conditions, has received a DCI from the first TRP indicating rank {1+2}. In this case, with legacy “Antenna ports” table, the indicated DMRS ports are {0, 1, 2} with DMRS port {0} being transmitted from the first UE panel and port {1, 2} being transmitted from the second UE panel. As shown in the left part of Figure 5, this causes DMRS interference (reuse of a DMRS port) for the second TRP (i.e., the TRP that did not send the DCI). This problem can be resolved, as shown in the right part of Figure 5, by introducing a new entry in the “Antenna ports” table, such that the indicated DMRS ports are {0, 2, 3}.

Figure 5	DMRS ports after rank adaptation from rank 4 to rank 3 for STxMP UE co-scheduled with non-STxMP UEs. In the left part of the figure, STxMP UE use DMRS port {0, 1, 2}, which causes interference. In the right part of the figure, STxMP UE use DMRS ports {0, 2, 3}, which avoid interference.

Based on the above, we propose:

Add additional entries to “Antenna ports” tables to support that DMRS ports associated with different SRI/TPMI fields can be in different CDM groups.

This proposal is in line with Option 1 above. Note that there is no additional DCI overhead associated with this solution, as only one out of eight codepoints in legacy 3-bit “Antenna ports” table is in use. Hence, DMRS ports {0, 2, 3} can be indicated by repurposing one of the seven reserved codepoints.

It is worth pointing out that the other listed options, i.e., Option 2 and Option 3, also enable that DMRS ports associated with different SRS resource sets can be mapped to different CDM groups. However, these solutions cause mapping from DMRS ports associated with an SRS resource set to CDM groups to vary under rank adaptation, which, as shown above, is problematic. In fact, both Option 2 and Option 3 would cause not 1, but 3 DMRS port collisions in the above example.

#### 2.2.4.2	DMRS ports for DFT-S-OFDM

To improve UL coverage for cell-edge UEs, DFT-S-OFDM should be supported for STxMP.

For multi-DCI STxMP scheduled by a DG, DFT-S-OFDM can be supported by configuring transform precoding for PUSCH and by indicating separate and orthogonal DMRS ports in the DCI corresponding to the first and second TRP, respectively.

For single-DCI SFN STxMP, DFT-S-OFDM can be straightforwardly supported: An indicated DMRS port (corresponding to a single layer) will be mapped to both SRS resource sets.

For single-DCI SDM STxMP, DMRS ports associated with different SRS resource sets correspond to different layers. However, in current specification, when transform precoding is enabled, it is not possible to indicate more than one orthogonal DMRS port with existing “Antenna ports” tables. To solve this issue, it should be possible to indicate two orthogonal DMRS ports for transform precoding, one per SRS resource set, and, hence, we propose:

Add additional “Antenna ports” tables to support indication of two DMRS ports for SDM STxMP when transform precoding is configured.

It should be possible to indicate entries of such tables in the DCI scheduling PUSCH for the case when both SRS resource sets are indicated in the “SRS resource set indicator” field.

## 2.3	Design considerations for multi-DCI based STxMP

Most of the design considerations for multi-DCI based STxMP have been resolved:

Agreement (RAN1#111)

- For multi-DCI based STxMP, to schedule a PUSCH for STxMP PUSCH+PUSCH transmission,

  - Alt1: The first SRS resource set is associated with coresetPoolIndex value 0 and the other SRS resource set is associated with coresetPoolIndex value 1

    - The PUSCH is associated with SRS resource set with the same value of coresetPoolIndex.

    - FFS: Which is the first SRS resource set, e.g., the set with lower set ID.

- Regarding how to interpret the SRI/TPMI field in DCI:

  - For DG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where scheduling DCI format 0_1 or 0_2 is received

  - For Type 2 CG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where activation DCI is received.

- For Type 1 CG-PUSCH, one SRS_resource_set_index value is configured in RRC in ConfiguredGrantConfig and the srs-ResourceIndicator/precodingAndNumberOfLayers correspond to the SRS resource set.

For the FFS, we support that the first SRS resource set is the one with the lower set ID.

In RAN1#110bis-e, the following was agreed:

Agreement (RAN1#110bis-e)

Support STxMP PUSCH+PUSCH transmission in multi-DCI based system in Rel-18.

- Two independent PUSCHs associated with different TRPs can be transmitted by a UE simultaneously in same active BWP.

- The total number of layers of these two PUSCHs is up to 4.

- FFS: whether the number of layers of each of these two PUSCHs is up to 2.

For the FFS, we propose to reuse whatever limitations we agree for single-DCI:

Support the same layer combinations for multi-DCI STxMP PUSCH as for single-DCI STxMP PUSCH.

## 2.4	Enhancements to group-based reporting

One vital building block in an STxMP solution is to determine if the UE can transmit two signals simultaneously, or rather under what circumstances this is possible. This issue is illustrated in Figure 6.

Figure 6	Communication using both panels is not always possible.

To facilitate PUSCH scheduling using STxMP, the NW must be made aware if the UE can communicate with the target TRP(s) using both panels. This problem has been studied before, but in the context of simultaneous reception on different panels. For reception, the problem is solved using a special type of report, the group-based report. In a group-based report, the UE reports a pair of DL RS, and if the NW transmits two DL signals, where the QCL Type D of the first DL signal is given by the first DL RS in the reported pair, and the QCL Type D of the second DL signal is given by the second DL RS in the reported pair, the UE can receive both signals at the same time.

It seems that the group-based reporting could be extended to also inform the NW if the UE could simultaneously transmit two UL signals. Therefore, we propose

Study enhancements to group-based reporting as a tool for the NW to schedule STxMP PUSCH.

We note that this topic has been addressed in contributions to AI 9.1.1.1, but in our view, the discussion belongs in AI 9.1.4.1.

For single-DCI based mTRP, the scheduling decisions are sent in one PDCCH, and it is then reasonable to assume that the scheduling decisions are also made in one node. It is then sufficient that the report that describes the possibilities for STxMP also reaches that node. It is then relatively easy for the NW to make an educated decision on whether to use STxMP or not.

For multi-DCI based mTRP, the situation is somewhat different. There is an imprecise assumption on “non-ideal backhaul”, which makes it somewhat blurred what information is available in each “TRP”. If the schedulers operate “independently”, it becomes more difficult for the NW to adjust the scheduling. At least, we need to assume that the report reaches both TRPs:

For multi-DCI, a report that describes if STxMP is possible must reach both TRPs.

Observation 4 states the minimum level of coordination that is needed to make STxMP work for multi-DCI based mTRP. Hence, it is not enough to rely on only semi-static coordination between the TRPs.

# Conclusion

In the previous sections we made the following observations:

Observation 1	With per-UE power control, the throughput gains with two CWs compared to one CW is marginal.

Observation 2	With per-panel power control, the benefits of two CWs compared to one CW is smaller than with per-UE power control.

Observation 3	One codepoint in SRS resource set indicator field is sufficient to indicate SDM STxMP or SFN STxMP.

Observation 4	For multi-DCI, a report that describes if STxMP is possible must reach both TRPs.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Specify only single-CW transmission for STxMP.

Proposal 2	Confirm the above working assumption with the following updates:

Proposal 3	For two SRS resource sets with usage ‘codebook’, support SRS resources belonging to different SRS resource sets being configured with different number of SRS ports.

Proposal 4	For SRS resource sets with usage ‘codebook’ or ‘nonCodebook’, support different number of SRS resources per SRS resource set.

Proposal 5	For SFN, the number of SRS ports per SRS resource and the number of SRS resources per SRS resource set is the same as for SDM and can vary over SRS resource sets.

Proposal 6	Do not support dynamic switching between STxMP and Rel-17 mTRP PUSCH repetition.

Proposal 7	Introduce “Second precoding information and number of layers” field in DCI Format 0_1 and 0_2 for indicating both precoder and rank for codebook-based SDM STxMP.

Proposal 8	Support Option1 for sTRP transmission, i.e., the maximal number of layers for sTRP transmission is configured by the maxRank (or Lmax) in current specification.

Proposal 9	Support Alt1 for SDM STxMP transmission, i.e., configure one single maximal number of layers (separate from the maximal number(s) of layers for sTRP), that is applied to the first SRS resource set and the second SRS resource set, separately.

Proposal 10	Add additional entries to “Antenna ports” tables to support that DMRS ports associated with different SRI/TPMI fields can be in different CDM groups.

Proposal 11	Add additional “Antenna ports” tables to support indication of two DMRS ports for SDM STxMP when transform precoding is configured.

Proposal 12	Support the same layer combinations for multi-DCI STxMP PUSCH as for single-DCI STxMP PUSCH.

Proposal 13	Study enhancements to group-based reporting as a tool for the NW to schedule STxMP PUSCH.

# References

RP-213598, Revised WID: MIMO evolution for downlink and uplink, Samsung, RAN#94-e, Dec 2021.

# Appendix: Simulation parameters

| Parameters | Values |
| --- | --- |
| Carrier frequency | 30 GHz |
| Subcarrier spacing | 120 kHz |
| Bandwidth | 80 MHz |
| Scenarios | 1: Dense urban (TR 38.913) 2: Indoor hotspot (TR 38.901/802) |
| Scheduler | Proportional fair |
| Traffic model | FTP model 1with 500 kB packet size |
| Number of BSs | 1: 7 (3 sectors/cell) 2: 12 (1 sector/cell) |
| Number of UEs | 2000 |
| Handover margin | 3 dB |
| MIMO scheme | SU-MIMO (up to rank 4 per UE, with up to rank 2 per panel). |
| Modulation | Up to 256QAM |
| Power control | P0= -80, α=0.8 |
| BS Antenna Configuration | 1: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), [dH, dV] = (0.5, 0.5) λ 2: (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), [dH, dV] = (0.5, 0.5) λ |
| BS antenna radiation pattern | TR 38.802, Table A.2.1-7 |
| BS antenna height | Dense urban: 25 m Indoor hotspot: 3 m |
| BS noise figure | 7 dB |
| UE antenna configuration | (M, N, P) = (1, 4, 2), dH, = 0.5 λ, with 2 back-to-back panels |
| UE antenna radiation pattern | TR 38.802, Table A.2.1-8 |
| UE distribution | 1: 100% outdoor 2: 100% indoor |
| UE antenna height | 1.5 m |
| UE speed | 3 km/h |
| UE and panel orientation | Vertical but random in azimuth |
| UE transmit power | 23 dBm (up to 23 dBm per panel) |