3GPP TSG RAN WG1 Meeting #112			R1-2301098

Athens, Greece, February 27 – March 03, 2023

Agenda item:	9.2.2.2

Title:	Joint CSI estimation and compression with AI/ML

Source:	Beijing Jiaotong University

Document for:	Discussion

1. Introduction

In RAN 94-e meeting, new study item on AI/ML for NR Air Interface was approved [1]. In the study item scope, the initial set of use cases were identified:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In this paper, a joint channel estimation and CSI feedback for enhancing CSI feedback quality and reducing computational complexity is proposed, and some potential specification impacts are discussed.

2. Joint CSI estimation and feedback with AI/ML

In the 5G system, the acquisition of channel state information (CSI) is a very critical step. Based on the modular design architecture, the CSI is first estimated at the user equipment (UE) and then fed back to the base station (BS) to complete the acquisition of CSI in the BS side. Hence CSI feedback quality in the BS side is correlated with both the CSI compression quality and the CSI estimation quality. However, the existing CSI compression method [2,3] assumes that the channel estimation stage is perfect. A large number of pilots should be used to achieve this prerequisite, which reduces the downlink transmission efficiency. Or else, the imperfect channel estimation would affect the CSI compression performance.

Here, we consider an FDD massive MIMO system, where a BS with Nt antennas serves a single-antenna user utilizing orthogonal frequency division multiplexing (OFDM) over  Nc subcarriers. The uplink channel is devoted by Hu=[hu,1,hu,2,⋯,hu,Nc]∈CNt×Nc, and the downlink channel is devoted by Hd=[hd,1,hd,2,⋯,hd,Nc]∈CNt×Nc. Due to similar propagation paths, gains and AoDs/AoAs, CSIs among nearby sub-carriers and antennas are correlated.

Figure 1: The time-frequency resource grid structure

To reduce the channel estimation overhead and improve the uplink transmission efficiency, we use the pilot pattern in a time-frequency grid as shown in Fig.1. Assuming the coherence time is much larger than T, the channel can be regarded constant with the time over the time-frequency grid. The pilot interval in frequency domain is Ni. The pilot length in time domain is L≤T. The amount of pilot in the resource grid is NcNi×L. This pilot pattern is adopted for each antenna in the BS.

Figure 1: The joint CSI estimation and compression architecture

The proposed joint CSI estimation and compression method is shown as Fig.2. According to the aforementioned pilot pattern, downlink pilot signals are transmitted by the BS over the mth subcarrier is Pm∈CL×Nt. The received signals at the UE are given by:

ym=Pmhm+nm, m=1,2,⋯,NcNi,

where nm~CN(0,σd2) is the complex additive Gaussian noise over the mth subcarrier, and σd2 represent the downlink noise power. The received signals Y=[y1,y2,⋯,ym]∈CL×NcNi are then encoded as:

x=EncoderY∈CNc',

where Nc' represents the number of subcarriers used in the uplink, and Nc'<Nc. The UE transmits the encoded x over Nc' subcarriers on the uplink channel. At the BS, the received encoded signals are expressed as:

Yu=Hu'x+Nu,

Where Nu represents the complex additive Gaussian noise over the antennas and subcarriers. Maximum ratio combining is then employed at the BS to convert Yu∈CNt×Nc' to yu∈CNc'. Finally, the reconstruction of  Hd is recovered by decoding yu:

Hd=Decoderyu∈CNt×Nc.

Different from separation based channel estimation and CSI compression methods which recover the downlink channel and then compress it to reduce the feedback overhead, the proposed method directly maps the received signal of the downlink to the transmitted signal of the uplink. The encoder at the UE and decoder at the BS can be learned by using deep learning (DL) based end-to-end training strategy. The evaluation results for the proposed is shown in [4].

Proposal #1: Study DL-based joint CSI estimation and compression as a use case for CSI feedback enhancement.

3. Conclusion

In this paper, the CSI feedback enhancement use case is discussed. Based on the discussion above, we propose the following proposal:

Proposal #1: Study DL-based joint CSI estimation and compression as a use case for CSI feedback enhancement.

4. References

[1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

[2] R1-2204057, CSI compression with AI/ML

[3] J. Xu, B. Ai, N. Wang N and W. Chen, “Deep Joint Source-Channel Coding for CSI Feedback: An End-to-End Approach”, IEEE Journal on Selected Areas in Communications, vol. 41, no. 1, pp. 260-273, 2023.

[4] R1-220XXX(申请之后填写), Evaluation of joint CSI estimation and compression with AI/ML