3GPP TSG RAN WG1 #112			R1-2301165

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.1.1.1

Source:	Google

Title:	Discussion on unified TCI framework extension for multi-TRP

Document for: 	Discussion

- Introduction

In this contribution, we discuss some issues related to unified TCI framework extension for multiple-TRP scenario. In Rel-18 MIMO WID [1], RAN Plenary has agreed to specify extension of Rel-17 unified TCI to indicate multiple DL and UL TCI states. The target use case is multi-TRP scenario. In previous RAN1 meetings [2] [3], RAN1 has made some agreements with respect to beam indication and beam association with channels/RSs, as shown in the followings. In subsequent sections, we discuss and provide our opinions on these related issues.

RAN1 #110b(e) Conclusion On unified TCI framework extension in Rel-18, there is no consensus to support simultaneous configuration of both joint and separate DL/UL TCI modes in a serving cell Conclusion On unified TCI framework extension in Rel-18, there is no consensus to support separate RRC-configured TCI state list(s) for each of TRPs Agreement On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: - Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported) Agreement On unified TCI framework extension for M-DCI based MTRP: - For a serving cell configured with joint DL/UL TCI mode, one joint TCI state can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - For a serving cell configured with separate DL/UL TCI mode, a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) Agreement On unified TCI framework extension, up to 2 joint TCI states can be indicated by MAC-CE/DCI and applied to CJT-based PDSCH reception (PDSCH-CJT) in a BWP/CC configured with joint DL/UL TCI mode - Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability - FFS: QCL type(s)/assumption(s) of the indicated joint TCI state(s) applied to PDSCH-CJT - Note: On how to inform UE to apply which indicated joint TCI state(s) to target channel(s)/signal(s) in the BWP/CC, it is discussed individually in AI 9.1.1.1 Agreement On unified TCI framework extension for M-DCI based MTRP: - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET that is associated with the same coresetPoolIndex value - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDSCH scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value - FFS: Other channel(s)/signal(s) that has explicit or implicit association with a coresetPoolIndex value - FFS: Other channel(s)/signal(s) that doesn’t have association with a coresetPoolIndex value Above are applicable to the CORESET(s) that is configured/allowed to follow the indicated joint/DL TCI state FFS: The configuration/rule to configure/allow CORESET(s) to follow the indicated joint/DL TCI state, including the option to reuse the same configuration/rule as in Rel-17 unified TCI framework Agreement On unified TCI framework extension, study the following enhancements for TRP-specific BFR: - Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP - Enhancement to beam update after NW response to TRP-specific BFR request RAN1 #111 Agreement On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design Agreement On unified TCI framework extension, PDSCH-CJT is supported as a S-DCI based MTRP scheme Note: Above does not preclude discussions specific to PDSCH-CJT design in the unified TCI framework Agreement On unified TCI framework extension, down-select at least one of the following alternatives for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT): - Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state - Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB Agreement On unified TCI framework extension for S-DCI based MTRP, use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 Agreement On unified TCI framework extension for M-DCI based MTRP, the same configuration/rule used in Rel-17 unified TCI framework (for determining whether the UE shall apply the indicated joint/DL TCI state to PDCCH on a CORESET and respective PDSCH) is reused to determine whether the UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET associated with the same coresetPoolIndex value and PDSCH scheduled/activated by the PDCCH. Agreement On unified TCI framework extension for M-DCI based MTRP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH transmission scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value

- Discussion

In previous RAN1 meetings, companies have discussed a lot on possible combinations of indicated unified TCI states. In addition, how to associate indicated unified TCI states with each channel and RS is also a hot issue on the table. In our views, before concluding these issues, there is one essential issue that RAN1 should first conclude. That is, what is the meaning that when only one unified TCI state is indicated in S-DCI mode?

In details, in S-DCI mode, if the beam indication DCI/MAC-CE indicates only one TCI state for a serving cell, which following case does it stand for?

- Case 1: It may be still M-TRP mode now in the serving cell; network just wants to update the common beam for one TRP

- Case 2: It is now S-TRP mode in the serving cell. No association is needed to indicate for each channel and RS

For Case 1, how to switch M-TRP and S-TRP mode, and whether to switch it dynamically can be further discussed. For Case 2, it may mean that switching between M-TRP and S-TRP is done dynamically by number of indicated TCI states. Also, network is not able to indicate one TCI just for updating one beam of a TRP when operating in M-TRP mode. Since this general issue would have an impact on future detailed design, we suggest that RAN1 should tackle this issue first.

In our views, Case 1 should be supported; otherwise, when NW operates M-TRP mode and would like to update one beam for just one TRP, NW needs to indicate two beams simultaneously. Such behavior is not quite efficient from signaling overhead perspective.

Proposal 1: RAN1 to discuss, in S-DCI mode, whether M-TRP mode is operated in a serving cell when only one unified TCI for a TRP is indicated for the serving cell.

Proposal 2: Support that NW can update beams for one TRP by just indicating unified TCIs for the TRP while still maintaining M-TRP mode.

RAN1 has had several debates on how to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception. In RAN1#111, we finally achieved the following agreement.

RAN1 #111 Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. Agreement On unified TCI framework extension for S-DCI based MTRP, a new indicator field is supported as the DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - FFS: Detail design of the new indicator field

From this agreement, NW has the flexibility to decide whether to enable DCI signalling for dynamic indication for beam association. If the new indicator field is configured, it can indicate which indicated TCI state to apply for PDSCH reception. However, an open issue is that how to determine the default indicated TCI state when the new indicator field is absent. In our views, the default beam issue can be addressed by RRC signalling. Another solution of determining the default beam issue can actually be performed by a rule-based method.

Another one open issue is how to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. We observe that DCI format 1_0 is also a case that the new indicator field is not present. Hence, we should work on a unified solution for DCI format 1_1/1_2 with the new indicator field absent and for DCI format 1_0.

Proposal 3: Down-select one of the followings to tackle default beam issue for indicating PDSCH beam association when the new indicator field is not present in DCI format 1_1/1_2,

- Alt 1: RRC-based method

- Alt 2: Rule-based method

Proposal 4: Strive to have a unified solution for determining default indicated TCI state for DCI format 1_1/1_2 with the new indicator field absent and for DCI format 1_0.

In RAN1#111, RAN1 had an agreement on how to indicate unified TCI by DCI for S-DCI based M-TRP in the following.

RAN1 #111 Agreement On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Increase on the size of the TCI field - Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS

We recall that, in RAN1 #109(e), RAN1 has agreed to support that, for M-TRP S-DCI mode, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate multiple joint/DL/UL TCI states in a CC/BWP or a set of CCs/BWPs in a CC list. However, the details are still unclear. Considering impact brought by multiple TRP, current 3-bit TCI field (i.e., 8 TCI field codepoints) may not be enough to indicate all possible combinations. Not to mention more combinations would emerge from subsequent discussions when coming to separate TCI mode. Based on these factors, we suggest increasing the size of the TCI field. In such way, there are more TCI codepoints to indicate possible combinations. According to above discussion, we propose the following proposals.

Proposal 5: Increase the size of the TCI field for indicating unified TCI at least for S-DCI based MTRP.

Another one critical issue is how UE understands which TRP is associated with an indicated TCI. UE needs to understand this information for updating all channels/RSs, which are associated with the TRP and suitable for applying unified TCI.

Proposal 6: FFS on how to indicate UE that which TRP is associated with an indicated TCI.

- Conclusion

According to the above discussion(s), we have the following observation(s) and proposal(s).

Proposal 1: RAN1 to discuss, in S-DCI mode, whether M-TRP mode is operated in a serving cell when only one unified TCI for a TRP is indicated for the serving cell.

Proposal 2: Support that NW can update beams for one TRP by just indicating unified TCIs for the TRP while still maintaining M-TRP mode.

Proposal 3: Down-select one of the followings to tackle default beam issue for indicating PDSCH beam association when the new indicator field is not present in DCI format 1_1/1_2,

- Alt 1: RRC-based method

- Alt 2: Rule-based method

Proposal 4: Strive to have a unified solution for determining default indicated TCI state for DCI format 1_1/1_2 with the new indicator field absent and for DCI format 1_0.

Proposal 5: Increase the size of the TCI field for indicating unified TCI at least for S-DCI based MTRP.

Proposal 6: FFS on how to indicate UE that which TRP is associated with an indicated TCI.

# Reference

- RP-213598 New WID: MIMO Evolution for Downlink and Uplink, Samsung

- Draft Report of 3GPP TSG RAN WG1 #110bis-e v0.1.0 (Online, 10th – 19th October 2022)

- Draft Report of 3GPP TSG RAN WG1 #111 v0.2.0 (Toulouse, France, 14th – 18th November 2022)