3GPP TSG RAN WG1 #112								      R1-2301171

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.14.1

Source:	InterDigital, Inc.

Title:	Discussion on PRACH coverage enhancements

Document for:	Discussion and Decision

# Introduction

The RAN-approved work item to further enhance the NR coverage and objectives are included in [1]. One of the objectives of the WI is to specify multiple PRACH transmissions for 4-step RACH. In the last RAN1 meeting, different aspects of multiple PRACH transmission were discussed and the agreements are captured in the Appendix. In this contribution, we discuss when to enable multiple PRACH transmissions, supporting multiple values of multiple PRACH transmission and RAR monitoring for multiple PRACH transmission.

# Discussion

## Enabling multiple PRACH transmissions

Single vs multiple PRACH transmissions:

Depending on the UE’s conditions (e.g., UE in bad coverage situation or UE in cell center), the UE can select multiple PRACH transmissions or single PRACH transmission. One condition to determine whether multiple PRACH transmissions is needed could be based on the SSB measurements during initial access procedure. For example, when the RSRP of all SSBs is below RSRP threshold, the UE can use multiple PRACH transmissions. The UE then selects the best beam on which PRACH repetition will be performed.

Proposal 1: The UE can use multiple PRACH transmissions when the RSRP of the SSBs is below a threshold.

The number of repetitions per multiple PRACH transmission:

In the RAN1#111 meeting, supporting multiple values for the number of multiple PRACH transmissions when using the same beam were discussed and the following were agreed:

Agreement For multiple PRACH transmissions with same Tx beam, down-select one option from the following options. - Option 1: gNB can only configure one value for the number of multiple PRACH transmissions. - Option 2: gNB can configure one or multiple values for the number of multiple PRACH transmissions. - FFS: details

UEs can be in different coverage situation and not all the UEs will need the same number of repetitions. Some UEs may need fewer repetitions and others may need larger number of repetitions. With only one configured value of repetitions, the issue is how to set this value. If a large number is configured, a UE may end up using more repetitions than what is needed and thus wasting PRACH resources. In case a small number is configured, for some UEs the coverage of PRACH transmissions will still be limited because of the insufficient number of repetitions. Option 2 enables UEs with different coverage situation to benefit from multiple PRACH transmission without wasting PRACH resources. The UE can pick a value of the number of multiple PRACH transmission based on the SSB-RSRP threshold for the first RACH attempt. If the UE does not receive a random-access response, the UE then can increase the number of multiple PRACH transmission in the next attempt.

Proposal 2: The UE can be configured with one or multiple values for the number of multiple PRACH transmissions (Option 2).

Proposal 3: The UE can increase the number of multiple PRACH transmissions if RAR is not received.

## PRACH repetition using different Tx beams

As per work item objective, RAN1 agreed to study the following case for multiple PRACH transmissions using different Tx beams:

Agreement - Study at least the following case for multiple PRACH transmissions with different Tx beams. - UE uses different TX beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS - FFS: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs /CSI-RSs, where the different SSBs/CSI-RSs are not associated with the same RO. - Note: not related to decision on CFRA Note: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs/CSI-RSs, where the different SSBs/CSI-RSs are associated with the same RO is not considered.

The purpose of supporting multiple PRACH transmissions is to enhance the coverage of the PRACH transmission. When the UE is in a coverage limited situation, using different SSBs/CSI-RSs to repeat the PRACH transmission does not help to increase the coverage. The UE should pick the best SSB/CSI-RS and repeat the PRACH transmission over ROs associated with the same SSB/CSI-RS. Therefore, we think that RAN1 should focus only on studying using different Tx beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS.

Proposal 4: RAN1 to focus on studying UE using different Tx beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS.

## Resource configuration for PRACH transmissions using the same Tx beam

In the RAN1#111 meeting, 3 options regarding resource configuration for multiple PRACH transmission using the same beam are agreed to be considered. The options are covering the possibility of transmitting multiple PRACH transmission with separate preamble and/or separate ROs:

Agreement For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, consider one or multiple of the following options. - Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs. - Option 2: Multiple PRACH are transmitted on separate ROs. - Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs. - Other options are not precluded. - Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

Option 1 uses classical preamble partitioning; whereby reserved preambles can be used for R18. In Option 2, multiple PRACH are transmitted on separate ROs and thus there will be no shared preamble with legacy UEs. Such option can be more appropriate when gNB can provision more ROs with less overhead, e.g., in FR2. Both options can provide resource partitioning to allow the gNB to differentiate between multiple PRACH transmission and legacy single PRACH transmission. We think both options can be supported. Whether to use one or both options can be left for network configuration.

Proposal 5: For resource configuration of multiple PRACH transmissions, the following options are supported:

- Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs.

- Option 2: Multiple PRACH are transmitted on separate ROs.

## RAR monitoring

In the current NR specifications, after transmitting PRACH, the UE starts monitoring random access response within the RAR window. Only one RAR window that starts after transmitting the single PRACH was supported. With the support of multiple PRACH transmission, the question is whether the UE should monitor single RAR window or multiple monitoring RAR windows corresponding to each PRACH transmission. In RAN1#110b-e meeting, the following two options were agreed for further consideration:

Agreement For multiple PRACH transmissions with same beam, for RAR monitoring, consider the following options. - Option 1: One RAR window per each PRACH transmission, the RAR window follows the legacy design. - FFS: RA-RNTI. - Option 2: Only one RAR window for all of the multiple PRACH transmissions. - FFS: the start position of the RAR window. - FFS: RA-RNTI.

With the agreement made in the RAN1#111 meeting regarding the support of differentiating between multiple PRACH transmissions and single PRACH transmission, the gNB will be able to combine the repeated PRACH transmissions from one UE. There will be no need to have one RAR window per each PRACH transmission as the gNB will know if PRACH corresponds to multiple or single PRACH. Furthermore, option 1 increases the blind decoding efforts because the UE needs to monitor multiple RAR windows without clear benefit.

Proposal 6: For multiple PRACH transmissions with same beam, one RAR window is configured for all of the multiple PRACH transmissions.

The start time of RAR window can be after completing all the PRACH transmissions of multiple PRACH transmissions. For each value of the number of multiple PRACH transmissions, the start time of RAR window can be different. Larger number of repetitions would require late starting time of RAR window whereas for a smaller number of repetitions, RAR can start earlier.

Proposal 7:  The start of the RAR window is separately configured for each value of the number of multiple PRACH transmissions.

# Conclusion

In this contribution, we have discussed and proposed the following:

Proposal 1: The UE can use multiple PRACH transmissions when the RSRP of the SSBs are below a threshold.

Proposal 2: The UE can be configured with one or multiple values for the number of multiple PRACH transmissions (Option 2).

Proposal 3: The UE can increase the number of multiple PRACH transmissions if RAR is not received.

Proposal 4: RAN1 to focus on studying UE using different Tx beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS.

Proposal 5: For resource configuration of multiple PRACH transmissions, the following options are supported:

- Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs.

- Option 2: Multiple PRACH are transmitted on separate ROs.

Proposal 6: For multiple PRACH transmissions with same beam, one RAR window is configured for all of the multiple PRACH transmissions.

Proposal 7:  The start of the RAR window is separately configured for each value of the number of multiple PRACH transmissions.

# References

RP-221858, “Revised WID on Further NR coverage enhancements”, China Telecom.

# Appendix

## RAN1#111 agreements

Agreement

For multiple PRACH transmissions with same Tx beam, support to differentiate at least between multiple PRACH transmissions and single PRACH transmissions.

Agreement

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, consider one or multiple of the following options.

- Option 1: Multiple PRACH are transmitted with separate preamble on shared ROs.

- Option 2: Multiple PRACH are transmitted on separate ROs.

- Option 3: Partial of multiple PRACHs are transmitted with separate preamble on shared ROs, while the other multiple PRACHs are transmitted on separate ROs.

- Other options are not precluded.

- Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

Agreement

- Study at least the following case for multiple PRACH transmissions with different Tx beams.

  - UE uses different TX beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS

  - FFS: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs /CSI-RSs, where the different SSBs/CSI-RSs are not associated with the same RO.

  - Note: not related to decision on CFRA

Note: UE uses different TX beams to transmit the multiple PRACH over ROs associated with different SSBs/CSI-RSs, where the different SSBs/CSI-RSs are associated with the same RO is not considered.

- Working Assumption

- Simulation results for multiple PRACH transmissions with different beam(s) and same beam(s) (baseline) to be discussed in the next meeting.

- Simulation assumptions in TR 38.830 are used as the starting point for the simulation.

    - Focus on FR2.

      - UE antenna configuration 2-2-2(baseline), 1-4-1(optional)

    - Performance metric: 0.1% false alarm, 1% miss-detection

    - Companies report the number of beams, the beam widths, beam correspondence assumption, and the boresights.

  - Channel model for link-level simulation: CDL-A defined in table 7.7.1-1 in TR 38.901.

  - Both that UE fulfills beamCorrespondence requirements Without UL-BeamSweeping and UE fulfils beamCorrespondence requirements with UL-BeamSweeping can be considered in the simulation are used as starting point for simulation.

Agreement

For multiple PRACH transmissions with same Tx beam, down-select one option from the following options.

- Option 1: gNB can only configure one value for the number of multiple PRACH transmissions.

- Option 2: gNB can configure one or multiple values for the number of multiple PRACH transmissions.

  - FFS: details

Agreement

- For multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt.

  - Note: whether to support multiple numbers of PRACH transmissions is separately discussed.

## RAN1#110b-e agreements

Agreement

- For multiple PRACH transmissions with same beam, at least support to use same PRACH preamble during the multiple PRACH transmissions in one RACH attempt.

  - FFS: whether different preambles can be utilized in different PRACH transmissions during the multiple PRACH transmissions in one RACH attempt.

Agreement

- For multiple PRACH transmissions with same beam, at least ROs located at different time instances can be utilized for the transmissions.

- FFS: whether/how the starting RB of ROs can be different at different time instances for multiple PRACH transmissions.

- FFS: whether/how multiple PRACH transmissions located in the same time instance, e.g., for UEs with multiple Tx chains.

Agreement

For multiple PRACH transmissions with same beam, for RAR monitoring, consider the following options.

- Option 1: One RAR window per each PRACH transmission, the RAR window follows the legacy design.

  - FFS: RA-RNTI.

- Option 2: Only one RAR window for all of the multiple PRACH transmissions.

  - FFS: the start position of the RAR window.

  - FFS: RA-RNTI.