3GPP TSG RAN WG1 #112	R1-2301199

Athens, Greece, Feb. 27th – Mar. 3rd, 2023

Source:	Lenovo

Title:	Evaluation on AI/ML for CSI feedback enhancement

Agenda Item:	9.2.2.1

Document for:	Discussion and Decision

# Introduction

In RAN1#111, agreement was reached corresponding to one use case of AI/ML for spatial-frequency domain CSI compression. Concretely, the following was agreed in RAN1#111 [1].

Agreement For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following evaluation cases for sequential training are considered for multi-vendors - Case 1 (baseline): Type 3 training between one NW part model and one UE part model - Note 1: Case 1 can be naturally applied to the NW-first training case where 1 NW part model to M>1 separate UE part models - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training UE part model is the same or a subset of the dataset for training NW part model - Note 2: Case 1 can be naturally applied to the UE-first training case where 1 UE part model to N>1 separate NW part models - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training NW part model is the same or a subset of the dataset for training UE part model - Companies to report the AI/ML structures for the combination(s) of UE part model and NW part model, which can be the same or different - FFS: different quantization methods between NW side and UE side - Case 2: For UE-first training, Type 3 training between one NW part model and M>1 separate UE part models - Note: Case 2 can be also applied to the M>1 UE part models to N>1 NW part models - Companies to report the AI/ML structures for the M>1 UE part models and the NW part model - Companies to report the dataset used at UE part models, e.g., same or different dataset(s) among M UE part models - Case 3: For NW-first training, Type 3 training between one UE part model and N>1 separate NW part models - Note: Case 3 can be also applied to the N>1 NW part models to M>1 UE part models - Companies to report the AI/ML structures for the UE part model and the N>1 NW part models - Companies to report the dataset used at NW part models, e.g., same or different dataset(s) among N NW part models - FFS: whether/how to report overhead of dataset Agreement For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input dimensions of CSI generation part (e.g., different bandwidths/frequency granularities, or different antenna ports), the generalization cases of are elaborated as follows - Case 1: The AI/ML model is trained based on training dataset from a fixed dimension X1 (e.g., a fixed bandwidth/frequency granularity, and/or number of antenna ports), and then the AI/ML model performs inference/test on a dataset from the same dimension X1. - Case 2: The AI/ML model is trained based on training dataset from a single dimension X1, and then the AI/ML model performs inference/test on a dataset from a different dimension X2. - Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of X1, X2,..., Xn, and then the AI/ML model performs inference/test on a single dataset subject to the dimension of X1, or X2,…, or Xn. - Note: For Case 2/3, the solutions to achieve the scalability between Xi and Xj, are reported by companies, including, e.g., pre-processing to angle-delay domain, padding, additional adaptation layer in AI/ML model, etc. - FFS the verification of fine-tuning - FFS other additional cases Agreement For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different output dimensions of CSI generation part (e.g., different generated CSI feedback dimensions), the generalization cases of are elaborated as follows - Case 1: The AI/ML model is trained based on training dataset from a fixed output dimension Y1 (e.g., a fixed CSI feedback dimension), and then the AI/ML model performs inference/test on a dataset from the same output dimension Y1. - Case 2: The AI/ML model is trained based on training dataset from a single output dimension Y1, and then the AI/ML model performs inference/test on a dataset from a different output dimension Y2. - Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of Y1, Y2,..., Yn, and then the AI/ML model performs inference/test on a single dataset of Y1, or Y2,…, or Yn. - Note: For Case 1/2/3, companies to report whether the output of the CSI generation part is before quantization or after quantization. - Note: For Case 2/3, the solutions to achieve the scalability between Yi and Yj, are reported by companies, including, e.g., truncation, additional adaptation layer in AI/ML model, etc. - FFS the verification of fine-tuning - FFS other additional cases Agreement For the evaluation of the high resolution quantization of the ground-truth CSI in the CSI compression, Float32 is adopted as the baseline/upper-bound of performance comparison. Agreement For the evaluation of quantization aware/non-aware training, the following cases are considered and reported by companies: - Case 1: Quantization non-aware training, where the float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training - Fixed/pre-configured quantization method/parameters is applied for the inference phase - Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc. - Case 2: Quantization aware training, where quantization/dequantization is involved in the training process - Case 2-1: Fixed/pre-configured quantization method/parameters are applied during the training phase; the same quantization codebook is applied for the inference phase - Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc. - Case 2-2: The quantization method/parameters are updated in together with the AI/ML models during the training; when training is finished, the final quantization codebook is applied for the inference phase - Companies to report how to update the quantization method/parameters during the training - Note: the above cases apply for training Type 1/2/3 - Others are not precluded. Agreement For the evaluation of an example of Type 3 (Separate training at NW side and UE side) with sequential training, companies to report the set of information (e.g., dataset) shared in Step 2 - For NW-first training - Dataset construction, e.g., the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only, or other information if applicable. - Quantization behavior, e.g., whether the shared output of the Network side CSI generation part is before or after quantization. - For UE-first training - Dataset construction, e.g., the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only, or other information if applicable. - Quantization behavior, e.g., whether the shared inputof the UE side CSI reconstruction part is before or after quantization. Agreement For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input/output dimensions, companies to report which case(s) in the following are evaluated - Case 0 (benchmark for comparison): One CSI generation part with fixed input and output dimensions to 1 CSI reconstruction part with fixed input and output dimensions for each of the different input and/or output dimensions. - Case 1: One CSI generation part with scalable input and/or output dimensions to N>1 separate CSI reconstruction parts each with fixed and different output and/or input dimensions - Case 2: M>1 separate CSI generation parts each with fixed and different input and/or output dimensions to one CSI reconstruction part with scalable output and/or input dimensions - Case 3: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions Agreement For the evaluation of the high resolution quantization of the ground-truth CSI in the CSI compression, if R16 Type II-like method is considered, companies to report the R16 Type II parameters with specified or new/larger values to achieve higher resolution of the ground-truth CSI labels, e.g., L, pv, β, reference amplitude, differential amplitude, phase, etc.

In this contribution document, we further discuss our views on the evaluation methodology for CSI feedback.

# Performance of an AI/ML model for different UE link-types

In RAN#109-e [3], we have agreed on parameters of EVM (also specified in appendix ‎11.1) which are used for generation of simulated training data and evaluation of the model. We have also agreed to report average throughput or intermediate KPI for evaluation of the proposed model. Some companies also suggested to present CDF of throughput or intermediate KPIs.

In the following we discuss:

- Due to the EVM assumption, there are different types of links in the environment

- The number of samples from different link-types are very different (e.g., based on assumption on UE distribution).

- Average and CDF of the metrics mask the performance for UEs with less likely link-types.

Simulation assumption:

- We simulate users using the proposed EVM, [3] (appendix ‎11.1) with 32 and 4 antennas at the eNB and the UE.

- We have assumed that a single two-sided model (Me for UE-part and Md for gNB-part) is already trained using this dataset. The Me and Md parts are already deployed at the UE and gNB respectively.

- We have simulated the model in 2400 drops and each drop we have simulated 300 UEs with bandwidth of 10MHz and 13 sub-bands.

- We have used the samples of the last 85 drops (Total of 25500 samples) exclusively for testing.

- Single layer transmission.

Simulation results:

- Average intermediate KPI: Considering all UEs, the following is the performance of the AI/ML model with respect to Type2. Rel-16 codebook.

Table 1. AI/ML Intermediate KPI Performance.

| Test Set | Model | # of feedback | SGCS |
| --- | --- | --- | --- |
| AI/ML Model | M | 272 bits | 85.6 |
| Type2. Rel16 | -- | 272 bits | 81.4 |

- CDF of the intermediate KPI: Considering all UEs, the following is the performance of the AI/ML mode with respect to Type2. Rel16 codebook.

: CDF of SGCS for different UEs under test

- Number of UEs in different link-types: In our EVM, we have used UMa scenario which has different link characteristic of LOS, NLOS, and O2I and for UE distributions, we have used UE deployments of 80% indoor (3km/h), 20% outdoor (30km/h). With these assumptions, there are at least three different UE link-types in the network. In our test samples size of 25500 samples, we counted the number of UEs in each link-type. The results are as the following.

Table 2. UE Link-Type distribution.

|  | Link Characteristic | UE deployment type | Number of UEs |
| --- | --- | --- | --- |
| UE Link-Type 1 | O2I | Indoor | 20400 |
| UE Link-Type 2 | NLOS | Outdoor Car | 3080 |
| UE Link-Type 3 | LOS | Outdoor Car | 2020 |

As can be seen, the dataset is highly imbalanced. So, the average/CDF test results mainly represent the results of the UE link-type-1. To see how UEs in the different link-types are benefiting from the AI/ML model, it is important to evaluate the performance of individual UE link-types separately.

- Now, instead of reporting the aggregate performance for all UEs, we have computed the performance of the AI/ML model for each link-type. The results are as the following in Table 3:

Table 3. AI/ML Intermediate KPI Performance.

|  |  | Performance of the Model Structure 1 |
| --- | --- | --- |
| CSI generation part | AL/ML model backbone | Conv net |
|  | Pre-processing | SVD decomposition Normalization |
|  | Post-processing | NA |
|  | FLOPs/M |  |
|  | Number of parameters/M | ~500.000 |
|  | [Storage /Mbytes] | ~2MB |
| CSI reconstruction part | AL/ML model backbone |  |
|  | [Pre-processing] | NA |
|  | [Post-processing] | Normalization |
|  | FLOPs/M |  |
|  | Number of parameters/M | ~675.000 |
|  | [Storage /Mbytes] | ~2.5MB |
| Common description | Input type | Eigen Vector |
|  | Output type | Eigen Vector |
|  | Quantization /dequantization method | Vector Quantization |
| Dataset description | Train/k | ~590K |
|  | Test/k | 25 k |
|  | Ground-truth CSI quantization method | No Quantization |
| Benchmark |  | Type-2 Rel.16 |
| SGCS, benchmark, layer 1 272 bits feedback | All UEs | 81.4 |
|  | UE Link-Type 1 | 81.3 |
|  | UE Link-Type 2 | 75.0 |
|  | UE Link-Type 3 | 91.9 |
| SGCS, AI/ML, layer 1 272 bits feedback | All UEs | 85.6 |
|  | UE Link-Type 1 | 85.6 |
|  | UE Link-Type 2 | 79.2 |
|  | UE Link-Type 3 | 95.6 |

Some observations below:

- The trained model in our simulation always has better performance compared to the Rel.16 codebook.

- The “average result” is mainly dominated by the result of UE link-Type 1

- The results of only UE link-Type2 and UE link-Type3 are very different. Note that such difference cannot be observed in the average or CDF of the KPI. So, it seems essential to report the performance of the AI/ML model for different UE types.

Note: What is the best way to construct UE-link type still need further studies, i.e., for now we have divided the UEs links based on the combination of the link Characteristic and UE deployment type. This can be extended further if needed.

Due to the non-uniform nature of number of UEs link-types in EVM, the average performance is biased towards one particular UE link-type.

CDF of throughput/intermediate KPIs cannot effectively show the performance of all link-types.

Other than the average performance, companies are encouraged to report the performance of the AI/ML model for each link-types (UE groups)

- At least initially, UE link-types (UE groups) can be defined as below. Other link-types are for FFS

- link-Type1: Indoor/O2I

- link-Type2: Outdoor-car/NLOS

- link-Type3: Outdoor-car/LOS

# Performance of an AI/ML model for different Model Sizes

In the section 2, we have shown the achievable gain of using AI/ML for generating the CSI feedback. Here we analyse the performance of model which is about %75 smaller, i.e., instead of about 1.2M parameters, it has about 0.3M parameters. The results are presented in the Table 4.

Table 4. AI/ML Intermediate KPI Performance – Compare different model sizes.

|  |  | Model Structure 1 | Model Structure 2 |
| --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone | Conv net | Conv net |
|  | Pre-processing | SVD decomposition Normalization | SVD decomposition Normalization |
|  | Post-processing | NA | NA |
|  | FLOPs/M |  |  |
|  | Number of parameters/M | ~500.000 | ~120.000 |
|  | [Storage /Mbytes] | ~2MB | ~0.6MB |
| CSI reconstruction part | AL/ML model backbone |  |  |
|  | [Pre-processing] | NA | NA |
|  | [Post-processing] | Normalization | Normalization |
|  | FLOPs/M |  |  |
|  | Number of parameters/M | ~675.000 | ~210.000 |
|  | [Storage /Mbytes] | ~2.5MB | ~0.8MB |
| Common description | Input type | Eigen Vector | Eigen Vector |
|  | Output type | Eigen Vector | Eigen Vector |
|  | Quantization /dequantization method | Vector Quantization | Vector Quantization |
| Dataset description | Train/k | ~590K | ~590K |
|  | Test/k | 25 k | 25 k |
|  | Ground-truth CSI quantization method | No Quantization | No Quantization |
| Benchmark |  | Type-2 Rel.16 | Type-2 Rel.16 |
| SGCS, benchmark, layer 1 272 bit feedback | All UEs | 81.4 | 81.4 |
|  | UE Link-Type 1 | 81.3 | 81.3 |
|  | UE Link-Type 2 | 75.0 | 75.0 |
|  | UE Link-Type 3 | 91.9 | 91.9 |
| SGCS, AI/ML, layer 1 272 bit feedback | All UEs | 85.6 | 79.2 |
|  | UE Link-Type 1 | 85.6 | 78.9 |
|  | UE Link-Type 2 | 79.2 | 71.7 |
|  | UE Link-Type 3 | 95.6 | 92.4 |

As can be seen from Table 4, the reduced size model has inferior performance compared to the larger model (at least when we compare the structures that we have simulated for structure 1 and structure 2). Even in some UE-Link types (UE-Link type1 and UE-Link type 2) the performance of the AI/ML model is inferior to the baseline Type-2 feedback.

One possible explanation could be that the smaller models are not able to completely capture the statistics of all different channel models. In other words, as the training data presented to this small-size-model have different channel types, the model tries to learn all of them (as much as possible) to have a good average performance (i.e., to have a good generalization property).  However, this generalization ability results in having lower performance.

So, one possible scheme to improve the performance of small-size-models might be to present training data with less variation, i.e., to have more specialized model (different models for different channel/UE types) instead of having one model for all UEs.

We investigate the performance of such model in the next section.

Although smaller AI/ML models have lower overhead, they may experience performance loss when they are trained for all different UE-link types.

Study the performance of AI/ML models for more restrictive scenarios/configurations, e.g., different UE-link-types (UE groups).

# Model Updating and associated cost

Use of smaller models can help reducing overheads for example for model delivery, and training time of the model; however, as we have shown in results of Section ‎3,  it will reduce the achievable performance of the model. The following Table 5 is extracted form table 4.

Table 5. AI/ML Intermediate KPI Performance – Compare different model sizes.

| SGCS layer 1 |  | Model Structure 1 | Model Structure 2 |
| --- | --- | --- | --- |
| Benchmark |  | Type-2 Rel.16 | Type-2 Rel.16 |
| Benchmark | UE Link-Type 3 | 91.9 | 91.9 |
| AI/ML | UE Link-Type 3 | 95.6 | 92.4 |

As can be seen, the AI/ML in Model structure-1 is about 3.7% better than the baseline schemes, while Model structure-2 is only about 0.8% better that the baseline solution.

As we discussed in the previous section, this might be due to the lower capability of small-size-models. So, to improve the performance of Model structure-2, we have trained it only using the data of UE Link-Type 3.

Note: Since we already have a model trained for all types of UE Link-Types. Instead of retraining the model from scratch we have adapted the model (fine-tuned) when this time it only observes UE Link-Type 3 during training phase.

The results are presented in the following Table 6.

Table 6. AI/ML Intermediate KPI Performance – Model Update.

| SGCS layer 1 |  | Model Structure 1 | Model Structure 2 | Model Structure 2 After model adaptation |
| --- | --- | --- | --- | --- |
| Benchmark |  | Type-2 Rel.16 | Type-2 Rel.16 | Type-2 Rel.16 |
| Benchmark | UE Link-Type 3 | 91.9 | 91.9 | 91.9 |
| AI/ML | UE Link-Type 3 | 95.6 | 92.4 | 93.3 |

Note 1: The results shows that the fine-tuning step is able to recover about 30% of the loss that the smaller model experience.

Note 2: The time/complexity required for updating the model is much less than the time needed for initial training. In this experiment,

- The time elapsed for initial training of the model structure-2 was 2900sec (about 50 min)

- The time elapsed for updating of the trained model was 300sec (about 5 min)

Of course, since both steps have been executed on the same device, we can see of almost 10 times reduction in time needed for model update.

This observation also verifies the lower complexity of the model-update which may make it a feasible solution to be performed over-the-air, in a single loop or in sequential manner, i.e., type 2 or type 3.

Having UEs with different link-types, model adaptation (e.g., fine-tuning, model update) may improve the performance of the AI/ML especially for small-size models. Of course, the gain and the cost associated with such model-adaptation should be evaluated.

Model adaptation (e.g., fine-tuning, model update) for example for UEs of different link-types should be considered especially for small-size models.

Study the over-the-air training of the models, especially for model update and model adaptation, which needs less amount of data exchange.

Performance and cost associated with model adaptation (e.g., fine-tuning, model update) should be evaluated for different UE link-types.

# Template for reporting the evaluation results of AI/ML-based CSI compression

In RAN1#111 we have agreed on a table to report the evaluation results of AI/ML-based CSI compression without generalization/scalability verification.

The content of this table is good for initial common evaluation metrics but we believe the “important KPI/metrics” can be different in a) different training types and b) different stages of the LCM, c) where the training happens (one the node which uses the model, on a RAN node, on cloud node or another node), d) how the training/update data is collected (simulated data/ offline data/ near-real time data).

For example,

- Type 3 for initial training when on two nodes on the cloud using offline data

  - No need to evaluate training complexity

  - No need to evaluate the time needed for collecting the training data.

  - Need to evaluate the model transfer overhead

- Type 1 for initial training when at the UE using recently collected data

  - Need to evaluate training complexity

  - No need to evaluate the time needed for collecting the training data.

  - No Need to evaluate the model transfer overhead

- Type 3 for model update when on two nodes on the cloud using recently collected data [offline data does not make sense that much for model update]

  - No need to evaluate training complexity

  - Need to evaluate the time needed for collecting the training data.

  - Need to evaluate the model transfer overhead

Additionally, we note that one particular design may use different training types during different LCM stages, like, use training type 3 during initial training and using Training type 2 during model update (to have lower overhead and less latency).

A single AI/ML design may use different training scheme in different stages of LCM. e.g., initial training, model update, model fine-tuning.

The KPIs that are important for evaluation of the training scheme may be different depending on:

- Lifecycle stage during which training is performed (e.g., initial training, model update, …)

- Where the training is performed (e.g., UE, on gNB, on other node(s) in the cloud)

- If training needs only simulated data or offline field dataset or online (e.g., (near) real-time) dataset is needed.

When discussing Template for reporting the evaluation results, consider requirements for all stages of the LCM (not only the inference stage), and all different deployment types. As one example, important combinations of the following quadruple should be considered when we discuss evaluation template:

- (Training scheme, Lifecycle stage, Training entity, Dataset type) where

- Training scheme: Type 1, Type 2, Type 3

- Lifecycle stage: Initial Training, Model update, Fine-tuning

- Training entity: UE, gNB, both UE and gNB, cloud node(s)

- Dataset type: Simulated dataset, Offline field dataset, Online (near-real time)

Baes on the above discussions we suggest adding a few sections to the templates for evaluation of different schemes. Note that, evaluation of the performance only during the inference stage is not sufficient for evaluation of an AI/ML model without considering all stages of the LCM.

We suggest adding the following rows to the template we use for evaluation of the model. Note that, similar to some other rows as “Pre-processing” and “Post-processing”, we can have NA for some of the KPIs if they are not applicable to the proposed scheme. To shorten the table, we have removed some of the rows which are already agreed upon. Then new additions are marked with different color.

Table 7. Template for evaluation of the performance of AI/ML models

|  |  | Source 1 |
| --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |
|  | Pre-processing |  |
|  | Post-processing |  |
|  | FLOPs/M |  |
|  | Number of parameters/M |  |
|  | [Storage /Mbytes] |  |
| CSI reconstruction part | AL/ML model backbone |  |
|  | [Pre-processing] |  |
|  | [Post-processing] |  |
|  | FLOPs/M |  |
|  | Number of parameters/M |  |
|  | [Storage /Mbytes] |  |
| Dataset description | Train/k |  |
|  | Test/k |  |
|  | Ground-truth CSI quantization method |  |
| Benchmark |  |  |
| Intermediate KPI I#1 of benchmark, [layer 1] | CSI feedback payload X |  |
|  | CSI feedback payload Y |  |
|  | CSI feedback payload Z |  |
| Gain for intermediate KPI I#1, [layer 1] | CSI feedback payload X |  |
|  | CSI feedback payload Y |  |
|  | CSI feedback payload Z |  |
| Gain for 5% UPT | CSI feedback payload X |  |
|  | CSI feedback payload Y |  |
|  | CSI feedback payload Z |  |
| Data collection overhead | Initial training |  |
|  | Model monitoring |  |
|  | Model adaptation (update/switching/…) |  |
| Data collection latency | Initial training |  |
|  | Model monitoring |  |
|  | Model adaptation (update/switching/…) |  |
| Model Delivery overhead | Initial model |  |
|  | After Model adaptation (update/switching/…) |  |
| Model Delivery latency | Initial model |  |
|  | After Model adaptation (update/switching/…) |  |
| Model Training | Training type |  |
|  | Training overhead (e.g., type2 gradient exchange) |  |
|  | … |  |
| … |  |  |
| FFS others |  |  |

# CQI reporting

For two-sided AI models under Type-3 training collaboration, separate training at network side and UE side is assumed, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained separately at the UE and network sides, respectively. One advantage of this collaboration type is that it ensures model privacy and does not require model parameter sharing across network and UE, it may cause mismatch between the target (nominal) precoding vector(s) assumed at the UE side and the actual precoding vector(s) computed at the network side. To elaborate more, at the UE side, the UE develops a nominal decoder D1 to compute the nominal precoding vector v1, and based on that the UE computes a nominal CQI, e.g., CQI 1, based on the actual channel H and the nominal precoding vector v1, where the CQI value is reported to the network side, as shown in .

- : Computation of nominal precoding vector based on UE-side nominal decoder

On the other hand, at the network side, the network develops an actual decoder D2 to compute the actual precoding vector v2, as shown in .

: Computation of actual precoding vector based on NW-side actual decoder

Since D1 and D2 are not necessarily identical, v1 ≠ v2 and hence the actual CQI, e.g., CQI 2, is not equal to the nominal CQI fed back by the UE. Given that, the nominal CQI may not meet the target BLER for DL transmission. This may lead to a mismatch between the nominal CQI value reported by the UE and the actual CQI value that meets the target BLER. Considering that, further enhancements are needed for two-sided AI-based CSI compression under training collaboration Type 3 to ensure precise CQI characterization.

Assuming two-sided AI models for CSI compression under training collaboration Type 3, further enhancements are needed to ensure precise CQI characterization in the presence of mismatch between the nominal decoder (at UE side) and the actual decoder (at network side).

# Data collection

One prerequisite of supporting AI-based CSI feedback compression is the availability of sufficient, relevant dataset points to enable model training. Under FDD mode, the CSI-based training dataset points can be collected at the UE side based on received CSI-RS, however the UE may be unable to save the dataset points and create large datasets due to memory and complexity limitations at the UE side. Furthermore, for Type 1 training collaboration with network-based training as well as Type 3 training collaboration, the transfer of the dataset to the network may be needed. Given that, signaling the CSI dataset points from the UE to the network may be required for FDD networks under NW-based Type 1 training as well as Type 3 training collaboration

For FDD systems with network-based model training as well as Type-3 training collaboration, siginaling the CSI training data from the UE to the network is needed.

## 7.1 Training data signaling

Different alternatives exist for training data signaling between the UE and the network. In this section, we focus on the different alternatives for signaling the CSI-based training dataset from the UE to the network, as follows:

- Alt1. Proprietary signaling. The CSI-dataset is transferred without specification impact using non-3GPP technologies

- Alt2. Legacy CSI-dataset feedback. The training dataset is inferred from a collection of CSI feedback occasions based on legacy NR codebook types

- Alt3.  Explicit CSI-dataset feedback. The training dataset is signaled via enhanced 3GPP-based dedicated signaling over an UL channel.

Note that more than one technique for training dataset signaling may be needed based on the requirements on the dataset size and the latency.

For instance, Alt1 (proprietary signaling) of the training dataset may be more reasonable for initial training phases, however it may not be suitable for other stages of the LCM, e.g., model adaptation (update) due to latency requirements which may be impossible to meet using proprietary signaling.

Alt2, on the other hand, can help provide training dataset without additional specification impact, however the data resolution based on legacy codebooks may be insufficient to build/train AI models with good performance, and can only be used as auxiliary/side information for acquisition of large-scale channel parameters that are less sensitive to the CSI feedback resolution, e.g., statistical channel delay and/or Doppler characteristics.

Alt3 can be supported as part of LCM of the AI model as part of dataset update and/or validation. Similar to AP CSI reporting over PUSCH under DCI format 0_2, the training dataset can be carried over a dedicated PUSCH that is dedicated for training data feedback, i.e., carries no UL data, which can be supported during periods of low network load.

Evaluate schemes related to transfer of CSI-dataset for different stages of the LCM.

Evaluate the following CSI training data signaling techniques:

- Alt1. Proprietary signaling via non-3GPP techniques

- Alt2. Legacy CSI-dataset feedback where the NR codebook-based CSI is utilized as CSI training data

- Alt3. Explicit CSI-dataset feedback via enhanced 3GPP-based signaling of the CSI training data

## 7.2 Training data format

One important aspect of training data signaling is the format of the training dataset points, which corresponds to the input of the AI model. The following alternatives are provided for CSI data format:

- Alt-A. Legacy codebook-based dataset points. The training dataset is inferred from a collection of CSI feedback occasions based on legacy NR codebook types

- Alt-B. High-resolution codebook-based dataset points. The training dataset corresponds to codebook-based CSI feedback with improved resolution, e.g., a variant of Rel-16 eType-II codebook with new/larger parameter values to achieve higher resolution of the CSI dataset labels, e.g., L, pv, β, amplitude and phase quantization levels

- Alt-C. Floating point representation of raw CSI data. The training dataset corresponds to raw CSI, e.g., raw channel matrix or channel eigenvectors that are characterized based on floating point representation.

As discussed in the previous section, Alt-A can help provide CSI training data without additional specification impact, however the data resolution based on legacy codebooks may be insufficient to build/train AI models with good performance and can only be used as auxiliary/side information for acquisition of large-scale channel parameters that are less sensitive to the CSI feedback resolution, e.g., statistical channel delay and/or Doppler characteristics. Alt-C provides the best CSI representation where the CSI mismatch between actual CSI values and training dataset can be made as small as possible, however Alt-B is easier to implement due to the similarity of the corresponding dataset generation to the codebook-based CSI compression techniques that are supported in today’s chipsets. Therefore, further evaluation of the training data format is needed.

Evaluate the following CSI training data formats:

- Alt-A. Legacy codebook-based dataset points generated via multiple occasions of NR codebook-based CSI feedback

- Alt-B. High-resolution codebook-based dataset points generated via high-resolution variants of NR-based CSI codebooks

- Alt-C. Floating point representation of raw CSI data, e.g., raw channel matrices or sets of channel eigenvectors

# Datasets used for Multivendor simulations

There are a few proposals, e.g., Type3 training, in which the aim is to study the design an AI/ML model that  can be trained for multiple vendors. For examples, there are a few evaluations on the argument that type-3 can be used to train a CSI-feedback model when we have multiple Encoder-parts (one for each vendors) and single decoder part.

For example,

- First, Vendor-1 to Vendor-k train their respective Encoder-1 to Encoder-k using their own dataset D1 to Dk.

- Then Vendor-1 to Vendor-k send the associated datasets to the a single node who want to train the decoder part. For example, lets define datasets  B1 to Bk where Bi={zji,yji} where zji represents a sample that the ith vendor have as the output of the encoder-part and  yji represents the expected output of the decoder-part

- After receiving all datasets B1 to Bk,  this gNB node can train the decoder part.

Some evaluation results show the good/acceptable performance of the model trained above. However, in those simulation, it seems that datasets  D1 to Dk are all sampled from the EVM setup that we have defined.

We wanted to point out, sampling from the same EVM for dataset generation is not appropriate for generating dataset for simulating multi-vendor cases, as the generated dataset (from the same EVM) will have similar statistics and cannot be a correct representative of having multiple-vendors. As we expect, different vendors have some difference between the measurements that they have from the channel.

Study methods for generating representative datasets for evaluation of multi-vendor scenarios.

# Metrics for intermediate performance evaluation

## Relative Achievable Rate

Consider an eNB and a UE equipped with M and N antennas respectively. We have collected L samples of the channel Hij where i=1,2,⋯,L shows different time samples and j=1,2,⋯,B show the number of frequency measurements we have over different subcarriers.  Hij, itself, is a complex-valued matrix of size M×N, i.e.,  Hij∈CM×N. The corresponding Tx-side eigenvectors and eigenvalues of H are denoted by vijk and λijk where k=1,2,⋯,K and K is the number of eigenvectors. Also, Vij=vij1 ,⋯,vijK . Furthermore, assume that the estimated Tx-side eigenvectors and eigenvalues are denoted by vijk and λijk, respectively. Also, Vij=vij1 ,⋯,vijK

As we discussed in last meetings, GCS based metrics are vulnerable to eigenvector mismatch. In addition GCS is not directly associated with the achievable throughput of the network.

To remove eigenvector mismatch problem and also having a closer metric to the network throughput, we propose to use relative achievable rate (RAR) defined as:

RARγ=EHi1Bj=1Blog2detI+γVij Hij HijTVijEHi1Bj=1Blog2detI+γVij Hij HijTVij

where γ is the SNR. This can be simplified as

RARγ=EHi1Bj=1Bk=1Klog21+γHijTvijk2 EHi1Bj=1Bk=1Klog21+γHijTvijk2

Note that RAR varies in different γ. So, in general it can be evaluated for different values of γ.

Also, to have an even a simpler evaluation metric, we can report RAR(1), i.e.,:

RAR1=EHi1Bj=1Bk=1Klog21+HijTvijk2 EHi1Bj=1Bk=1Klog21+HijTvijk2

As one intermediate KPI, to evaluate the efficiency of the estimated precoders, we suggest to use relative achievable rate (RAR) defined as:

- RARγ=EHi1Bj=1Bk=1Klog21+γHijTvijk2 EHi1Bj=1Bk=1Klog21+γHijTvijk2

- where γ is the SNR and channels are normalized, i.e.,  E|Hi|=1 is assumed to be normalized. For simplification, RARγ can report at γ=1, i.e.,

- RAR1=EHi1Bj=1Bk=1Klog21+HijTvijk2 EHi1Bj=1Bk=1Klog21+HijTvijk2

- Note also, if the metric is intended for comparison between different implementations which use the same “H”, then the denominator can be removed (as it will be the same between different models).

# Conclusions

This contribution addressed AI/ML-based CSI feedback enhancements. We have the following observations and proposals:

Observation 1:	Due to the non-uniform nature of number of UEs link-types in EVM, the average performance is biased towards one particular UE link-type.

Observation 2:	CDF of throughput/intermediate KPIs cannot effectively show the performance of all link-types.

Observation 3:	Although smaller AI/ML models have lower overhead, they may experience performance loss when they are trained for all different UE-link types.

Observation 4:	Having UEs with different link-types, model adaptation (e.g., fine-tuning, model update) may improve the performance of the AI/ML especially for small-size models. Of course, the gain and the cost associated with such model-adaptation should be evaluated.

Observation 5:	A single AI/ML design may use different training scheme in different stages of LCM. e.g., initial training, model update, model fine-tuning.

Observation 6:	The KPIs that are important for evaluation of the training scheme may be different depending on:

		Lifecycle stage during which training is performed (e.g., initial training, model update, …)

		Where the training is performed (e.g., UE, on gNB, on other node(s) in the cloud)

	If training needs only simulated data or offline field dataset or online (e.g., (near) real-time) dataset is needed.

Proposal 1	Other than the average performance, companies are encouraged to report the performance of the AI/ML model for each link-types (UE groups)

At least initially, UE link-types (UE groups) can be defined as below. Other link-types are for FFS

	link-Type1: Indoor/O2I

	link-Type2: Outdoor-car/NLOS

	link-Type3: Outdoor-car/LOS

Proposal 2	Study the performance of AI/ML models for more restrictive scenarios/configurations, e.g., different UE-link-types (UE groups).

Proposal 3	Model adaptation (e.g., fine-tuning, model update) for example for UEs of different link-types should be considered especially for small-size models.

Proposal 4	Study the over-the-air training of the models, especially for model update and model adaptation, which needs less amount of data exchange.

Proposal 5	Performance and cost associated with model adaptation (e.g., fine-tuning, model update) should be evaluated for different UE link-types.

Proposal 6	When discussing Template for reporting the evaluation results, consider requirements for all stages of the LCM (not only the inference stage), and all different deployment types. As one example, important combinations of the following quadruple should be considered when we discuss evaluation template:

(Training scheme, Lifecycle stage, Training entity, Dataset type) where

Training scheme: Type 1, Type 2, Type 3

Lifecycle stage: Initial Training, Model update, Fine-tuning

Training entity: UE, gNB, both UE and gNB, cloud node(s)

Dataset type: Simulated dataset, Offline field dataset, Online (near-real time)

Proposal 7	We suggest adding the following rows to the template we use for evaluation of the model. Note that, similar to some other rows as “Pre-processing” and “Post-processing”, we can have NA for some of the KPIs if they are not applicable to the proposed scheme. To shorten the table, we have removed some of the rows which are already agreed upon. Then new additions are marked with different color.

Template for evaluation of the performance of AI/ML models

|  |  | Source 1 |
| --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |
|  | Pre-processing |  |
|  | Post-processing |  |
|  | FLOPs/M |  |
|  | Number of parameters/M |  |
|  | [Storage /Mbytes] |  |
| CSI reconstruction part | AL/ML model backbone |  |
|  | [Pre-processing] |  |
|  | [Post-processing] |  |
|  | FLOPs/M |  |
|  | Number of parameters/M |  |
|  | [Storage /Mbytes] |  |
| Dataset description | Train/k |  |
|  | Test/k |  |
|  | Ground-truth CSI quantization method |  |
| Benchmark |  |  |
| Intermediate KPI I#1 of benchmark, [layer 1] | CSI feedback payload X |  |
|  | CSI feedback payload Y |  |
|  | CSI feedback payload Z |  |
| Gain for intermediate KPI I#1, [layer 1] | CSI feedback payload X |  |
|  | CSI feedback payload Y |  |
|  | CSI feedback payload Z |  |
| Gain for 5% UPT | CSI feedback payload X |  |
|  | CSI feedback payload Y |  |
|  | CSI feedback payload Z |  |
| Data collection overhead | Initial training |  |
|  | Model monitoring |  |
|  | Model adaptation (update/switching/…) |  |
| Data collection latency | Initial training |  |
|  | Model monitoring |  |
|  | Model adaptation (update/switching/…) |  |
| Model Delivery overhead | Initial model |  |
|  | After Model adaptation (update/switching/…) |  |
| Model Delivery latency | Initial model |  |
|  | After Model adaptation (update/switching/…) |  |
| Model Training | Training type |  |
|  | Training overhead (e.g., type2 gradient exchange) |  |
|  | … |  |
| … |  |  |
| FFS others |  |  |

Proposal 8	Assuming two-sided AI models for CSI compression under training collaboration Type 3, further enhancements are needed to ensure precise CQI characterization in the presence of mismatch between the nominal decoder (at UE side) and the actual decoder (at network side).

Proposal 9	For FDD systems with network-based model training as well as Type-3 training collaboration, siginaling the CSI training data from the UE to the network is needed.

Proposal 10	Evaluate schemes related to transfer of CSI-dataset for different stages of the LCM.

Proposal 11	Evaluate the following CSI training data signaling techniques:

	Alt1. Proprietary signaling via non-3GPP techniques

	Alt2. Legacy CSI-dataset feedback where the NR codebook-based CSI is utilized as CSI training data

	Alt3. Explicit CSI-dataset feedback via enhanced 3GPP-based signaling of the CSI training data

Proposal 12	Evaluate the following CSI training data formats:

	Alt-A. Legacy codebook-based dataset points generated via multiple occasions of NR codebook-based CSI feedback

	Alt-B. High-resolution codebook-based dataset points generated via high-resolution variants of NR-based CSI codebooks

	Alt-C. Floating point representation of raw CSI data, e.g., raw channel matrices or sets of channel eigenvectors

Proposal 13	Study methods for generating representative datasets for evaluation of multi-vendor scenarios.

Proposal 14	As one intermediate KPI, to evaluate the efficiency of the estimated precoders, we suggest to use relative achievable rate (RAR) defined as:

RARγ=EHi1Bj=1Bk=1Klog21+γHijTvijk2 EHi1Bj=1Bk=1Klog21+γHijTvijk2

where γ is the SNR and channels are normalized, i.e.,  E|Hi|=1 is assumed to be normalized. For simplification, RARγ can report at γ=1, i.e.,

RAR1=EHi1Bj=1Bk=1Klog21+HijTvijk2 EHi1Bj=1Bk=1Klog21+HijTvijk2

Note also, if the metric is intended for comparison between different implementations which use the same “H”, then the denominator can be removed (as it will be the same between different models).

# Appendix 1

## Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 2GHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 41 dBm for 10MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| Overhead |  | Only CSI-feedback overhead |
| Traffic model |  | Full Buffer |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |

# References

| [1] | RAN1 Chair, RAN1 Chair’s Notes, 3GPP RAN1#111, Toulouse, November 14th – 18th, 2022. |
| --- | --- |
| [2] | RAN1 Chair, "RAN1 Chair’s Notes," 3GPP RAN1#110bis-e, e-Meeting, October 10th – 19th, 2022. |
| [3] | RAN1 Chair, "RAN1 Chair’s Notes," 3GPP RAN1#109-e, May 9-20, 2022. |
| [4] | RP-213599, "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface," RAN#94e, Dec 2021. |
| [5] | R1-2207836, Moderator, "Summary#1 for CSI evaluation of [110-R18-AI/ML]," Auguest 22-26, 2022. |
| [6] | RAN1 Chair, "RAN1 Chair’s Notes," RAN1#110, Toulouse, France, August 22nd – 26th, 2022. |
| [7] | 3GPP TSG RAN WG1 #110bis-e, "Draft Meeting Report," Online, Oct. 10-19, 2022. |