3GPP TSG- RAN WG1 Meeting #112                                             R1-2301340

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.2.3.2

Source:	Apple Inc.

Title:	Discussion on other aspects of AI/ML for beam management

Document for:	Discussion/Decision

# Introduction

At RAN1 #109-e, the following agreements were reached:

Agreement

For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations

- BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams

- BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams

- FFS: details of BM-Case1 and BM-Case2

- FFS: other sub use cases

Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range

Agreement

For the sub use case BM-Case1, consider both Alt.1 and Alt.2 for further study:

- Alt.1: AI/ML inference at NW side

- Alt.2: AI/ML inference at UE side

Agreement

For the sub use case BM-Case2, consider both Alt.1 and Alt.2 for further study:

- Alt.1: AI/ML inference at NW side

- Alt.2: AI/ML inference at UE side

Agreement

Regarding the sub use case BM-Case2, the measurement results of K (K>=1) latest measurement instances are used for AI/ML model input:

- The value of K is up to companies

Agreement

Regarding the sub use case BM-Case2, AI/ML model output should be F predictions for F future time instances, where each prediction is for each time instance.

- At least F = 1

- The other value(s) of F is up to companies

Also the conclusions were drawn at RAN1 #109-e for use case 1:

Conclusion

For the sub use case BM-Case1, consider the following alternatives for further study:

- Alt.1: Set B is a subset of Set A

  - FFS: the number of beams in Set A and B

  - FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …)

- Alt.2: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams)

  - FFS: the number of beams in Set A and B

  - FFS: QCL relation between beams in Set A and beams in Set B

  - FFS: construction of Set B (e.g., regular pre-defined codebook, codebook other than regular pre-defined one)

- Note1: Set A is for DL beam prediction and Set B is for DL beam measurement.

- Note2: The narrow and wide beam terminology is for SI discussion only and have no specification impact

- Note3: The codebook constructions of Set A and Set B can be clarified by the companies.

Conclusion

Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input:

- Alt.1: Only L1-RSRP measurement based on Set B

- Alt.2: L1-RSRP measurement based on Set B and assistance information

  - FFS: Assistance information. The following were mentioned by companions in the discussion:  Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc.

    - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side.

- Alt.3: CIR based on Set B

- Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID

- Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives

- Note2: All the inputs are “nominal” and only for discussion purpose.

And conclusions for use case 2 were drawn at RAN1 #109-e:

Conclusion

For the sub use case BM-Case2, further study the following alternatives with potential down-selection:

- Alt.1: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams)

  - FFS: QCL relation between beams in Set A and beams in Set B

- Alt.2: Set B is a subset of Set A (Set A and Set B are not the same)

  - FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …)

- Alt.3: Set A and Set B are the same

- Note1: Predicted beam(s) are selected from Set A and measured beams used as input are selected from Set B.

- Note2: It is up to companies to provide other alternative(s)

- Note3: The narrow and wide beam terminology is for SI discussion only and have no specification impact

Conclusion

Regarding the sub use case BM-Case2, further study the following alternatives of measurement results for AI/ML input (for each past measurement instance):

- Alt.1: Only L1-RSRP measurement based on Set B

- Alt 2: L1-RSRP measurement based on Set B and assistance information

  - FFS: Assistance information. The following were mentioned by companies in the discussion:, Tx and/or Rx beam angle, position information, UE direction information, positioning-related measurement (such as Multi-RTT), expected Tx and/or Rx beam/occasion for the prediction (e.g., expected Tx and/or Rx beam angle for the prediction, expected occasions of the prediction), Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight directions (azimuth and elevation), 3dB beamwidth, etc.) , increase ratio of L1-RSRP for best N beams, UE orientation information

    - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side.

- Alt.3: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID

- Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives

- Note2: All the inputs are “nominal” and only for discussion purpose.

At RAN1 #110, the following were reached:

Agreement

At least for the sub use case BM-Case1 and BM-Case2, support both Alt.1 and Alt.2 for the study of AI/ML model training:

- Alt.1: AI/ML model training at NW side;

- Alt.2: AI/ML model training at UE side.

Note: Whether it is online or offline training is a separate discussion.

Agreement

For the sub use case BM-Case1 and BM-Case2, further study the following alternatives for the predicted beams:

- Alt.1: DL Tx beam prediction

- Alt.2: DL Rx beam prediction

- Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam)

- Note1: DL Rx beam prediction may or may not have spec impact

Agreement

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

    - FFS on the beams of Set B

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each report/measurement during training and/or inference)

    - FFS on fixed or variable number of beams (pairs)

    - FFS on the details

  - Other options are not precluded.

  - FFS on the number of beams (pairs) in Set B

  - Note: This does not preclude the alternative that Set B is different from Set A.

Agreement

- To evaluate the performance of AI/ML in beam management at least for NW side beam prediction, UCI report overhead can be further studied as one of KPI options.

  - FFS: number of UCI reports and UCI payload size

Agreement

For the sub use case BM-Case1, support the following alternatives for further study:

- Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A)

- Alt.2: Set B is a subset of Set A

- Note1: Set A is for DL beam prediction and Set B is for DL beam measurement.

- Note2: The beam patterns of Set A and Set B can be clarified by the companies.

Agreement

For the sub use case BM-Case2, further study the following alternatives:

- Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A)

- Alt.2: Set B is a subset of Set A (Set A and Set B are not the same)

- Alt.3: Set A and Set B are the same

- Note1: The beam pattern of Set A and Set B can be clarified by the companies.

Agreement

Regarding the sub use case BM-Case1 and BM-Case2, study the following alternatives for AI/ML output:

- Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

  - E.g., N predicted beams can be the top-N predicted beams

- Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and  other information

  - FFS: other information (e.g., probability for the beam to be the best beam, the associated confidence, beam application time/dwelling time, Predicted Beam failure)

  - E.g., N predicted beams can be the top-N predicted beams

- Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

  - E.g., N predicted beams can be the top-N predicted beams

  - FFS: details of Beam angle(s)

At RAN1 #111, the following were agreed:

Agreement

For the sub use case BM-Case1 and BM-Case2, at least support Alt.1 and Alt.2 for AI/ML model training and inference for further study:

- Alt.1. AI/ML model training and inference at NW side

- Alt.2. AI/ML model training and inference at UE side

- The discussion on Alt.3 for BM-Case1 and BM-Case2 is dependent on the conclusion/agreement of Agenda item 9.2.1 of RAN1 and/or RAN2 on whether to support model transfer for UE-side AI/ML model or not

Alt.3. AI/ML model training at NW side, AI/ML model inference at UE side

In this contribution, we provide our views on enhancement use cases and potential spec impact for AI based beam management.

# Discussion on generalization

## Discussion on model generalization

The AI model’s generalization performance plays a central role from dataset collection, inference, and performance monitoring. Broadly there can be two fundamental approaches to construct AI/ML models for beam management:

- 1) With at least beam measurement at UE as input, the entity performing AI/ML inference (UE or gNB) estimates key parameters in the wireless channel and identifies the best Tx beam. With that, there is not much difference in principle between the estimator implicitly or explicitly embedded in the AI/ML model and an estimator developed by a systems engineer. Just by the universal approximation theorem, the AI/ML model performing the estimation does not need to exhibit how it is actually done, and perhaps even the estimation problem does not need explicitly formulation. With that, anything particular to a specific cell or terrain type may be secondary inputs or does not constitute as inputs to the AI/ML model at all.

- 2) Looking from another angle, if the radio condition between network and a UE does not change or changes slowly, the best Tx beam at a location for one UE remains the best Tx beam at the same location for another UE (we ignore UE orientation here to simplify discussion).  In this case, there is no need to build a universal beam estimator or channel parameter estimator, rather a neural network with good memory about a particular geographical area serves the beam management purpose well. Beam measurement and assistance information such as UE position are used as a key to make an inquiry to the Tx beam database embodied by the neural network. With that, ideally for each cell a neural network model is customized. As the AI model embodies the terrain information implicitly and each cell can be different from terrain perspective, intuitively models for cells will be different. If model generalizes at all, that may just be a coincidence. In Section 4.1.6 of “AI/ML performance with per-cell (region) model” of [2], an intriguing evaluation is provided. It shows that very high fidelity precoder compression (for CSI feedback) can be achieved by leveraging cell-specific AI/ML model; that suggests wireless channel information can be represented and captured in a highly efficient way when its representation and capture (or feedback) is cell-specific. As beam management requires coarser wireless channel information than CSI feedback, perhaps even an AI/ML model simpler than that for CSI feedback can be utilized for beam management in a cell-specific way. Then it can be expected the more a model is customized/particular to a cell, the less it generalizes. While a model which does not generalize well does have its problem such as logistically how network and UE would coordinate their use, it may still merit discussion if it provides superior performance.

We have

Observation 1: AI/ML models can be crafted as a universal channel parameter estimator with good generalization or as a beam management database and associated query mechanism customized for a specific cell which is not expected to generalize well.

Four generalization cases (Generalization Case 1/2/2A/3) were agreed at RAN1 #111:

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

      - Note: Companies to report the ratio for dataset mixing

      - Note: number of the multiple scenarios/configurations can be larger than two

- FFS the detailed set of scenarios/configurations

- The following case for generalization verification, can be optionally considered by companies:

  - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

    - Company to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance

    - FFS: Investigate of the feasibility the fine-tuning on the UE/Network side

As for details of scenarios and configurations in the generalization study, details are also agreed at RAN1 #111:

Agreement

- For generalization performance verification, consider the following

  - Scenarios

    - Various deployment scenarios,

      - e.g., UMa, UMi and others,

      - e.g., 200m ISD or 500m ISD and others

      - e.g., same deployment, different cells with different configuration/assumption

      - e.g., gNB height and UE height

      - FFS: e.g., Carrier frequencies

    - Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others

    - Various UE mobility,

      - e.g., 3km/h, 30km/h, 60km/h and others

  - Configurations (parameters and settings)

    - Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions)

    - Various gNB settings, e.g., DL Tx beam codebook (including various Set A of beam(pairs) and gNB antenna array dimensions)

    - Various Set B of beam (pairs)

    - T1 for measurement /T2 for prediction for BM-Case2

  - Other scenarios/configurations(parameters and settings) are not precluded and can be reported by companies.

Regarding the evaluation of Set B in generalization study, also at RAN1 #111, we have

Agreement

For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, additionally considering

- Various Set B of beam(pairs)

Based on evaluation provided in [[4]],  it noted for Case 1 (matched training/test datasets), the AI based method’s performance tends to be much better than the conventional method’s given the same number of beams for measurement, which can motivates further work on AI enabled beam management at least for Case 1. For Case 2 (mismatched training/test datasets), the AI based method’s performance tends to be much worse than that for Case 1 matching test dataset or matching the training dataset. Case 3 provides a remedy wherein mixture of datasets is used to train an AI model.

## Discussion on model training and model inference

From the agreements reached at RAN1 #109-e and RAN1 #110:

Agreement

For the sub use case BM-Case1, consider both Alt.1 and Alt.2 for further study:

- Alt.1: AI/ML inference at NW side

- Alt.2: AI/ML inference at UE side

Agreement

For the sub use case BM-Case2, consider both Alt.1 and Alt.2 for further study:

- Alt.1: AI/ML inference at NW side

- Alt.2: AI/ML inference at UE side

Agreement

At least for the sub use case BM-Case1 and BM-Case2, support both Alt.1 and Alt.2 for the study of AI/ML model training:

- Alt.1: AI/ML model training at NW side;

- Alt.2: AI/ML model training at UE side.

Note: Whether it is online or offline training is a separate discussion.

At RAN1 #111, the following was agreed:

Agreement

For the sub use case BM-Case1 and BM-Case2, at least support Alt.1 and Alt.2 for AI/ML model training and inference for further study:

- Alt.1. AI/ML model training and inference at NW side

- Alt.2. AI/ML model training and inference at UE side

- The discussion on Alt.3 for BM-Case1 and BM-Case2 is dependent on the conclusion/agreement of Agenda item 9.2.1 of RAN1 and/or RAN2 on whether to support model transfer for UE-side AI/ML model or not

Alt.3. AI/ML model training at NW side, AI/ML model inference at UE side

Regarding these alternatives, we observe

- Alt. 1. Model training at the NW side & inference at the NW side

  - Data collection:

    - As network is aware of the RRC configuration of CSI-RS and CSI/BM reporting configuration, even though CSI-RS and CSI/BM reporting are configured on a UE-specific way, that does not prevent network from aggregating data from CSI/BM reporting and utilize them for model training.

  - Model generalization problem can be circumvented as whether a single AI model or multiple AI models are needed for inference is transparent to the UE.

  - Specifical impact for this combination can include enhanced beam reporting, e.g., more efficient beam reporting. It can be expected the feedback overhead to support this combination will be more than those for other combinations, and it is likely it will be more than that for the conventional beam management related feedback overhead. As shown in Figure 1, the current NR beam reporting targets a small number of beams, inheriting the same design for beam reporting for AI/ML may not be resource-efficient. There are at least 3 alternatives for set B design:

- Set-B-alt-1: Set B is fixed in both number of beams and the beam constellations (analog beam design)

- Set-B-alt-2: The number of beams in Set B is fixed, but the beam constellations can be changed with time.

- Set-B-alt-3: Both the number of beams and beam constellations in Set B can change.

For both alternatives Set-B-alt-2 and Set-B-alt-3, whether set B indication or beam indication should be used is not so clear yet. In general beam indication provides a finer granularity and full flexibility, yet that also comes with the accompanied feedback overhead. When RSRPs and/or CIRs (Channel Impulse Responses) are fed back to network, a careful re-examination on the Rel-15 design is needed.

- Alt. 3. Model training at the NW side & inference at the UE side

  - Specific impact for this combination can include model transfer or model delivery if superior beam management performance with cell-specific AI models is the target as discussed in the generalization section.

  - Even with non-cell specific AI models, whether a single AI model or multiple AI models are needed should be discussed. In our companion paper[3], we have investigated model mismatch with training data set and test data set as follows:

- Set A design (Column first vs Row first)

- Set B design (Column shift = 0 and 1 for Set A at 16 beams, and Column shift = 0, 1, 2 and 3 for Set A at 4/8 beams)

- Antenna element spacing ((dV=0.5,dH=0.5) vs (dV=0.8,dH=0.4))

And AI model performance degrades compared with the case without model mismatch.

  - Model generalization performance is key for this combination: if model generalizes well across different scenarios/configurations, it can be expected model delivery takes place infrequently. On the other hand, if model generalization performance for beam management is not so good, then frequent model delivery may be needed, e.g., model 1 is used by network in Cell 1 while model 2 is used by network in Cell 2. Further depending on the amount of memory and the number of neural network engines available at the UE which moves from one cell to another cell, the time when a new AI/ML model is activated may vary.

- Model training at the UE side, model inference at the NW side

  - Specific impact for this combination can include model delivery, with similar consideration as above.

- Alt. 2 Model training at the UE side, and inference at the UE side

  - For this combination, inference and performance monitoring are left to UE implementation. However, data collection still needs discussion, e.g., whether cell-specific signals are supported so the data for model training can be readily collected; otherwise each UE has a siloed view on the Tx beams from the network as CSI-RS is currently configured in a UE-specific way, data collection then will take a prolonged time. in an idealized setup, UE or UE side server has all the freedom to optimize its model(s), it offers opportunities for UE performance differentiation; realistically it is not clear how many AI models the UE needs to store, and by what criteria model switching/selection can be done by the UE itself.

In summary, we have

Proposal 1:

- For Model training at the NW side & inference at the NW side (Alt. 1), study efficient signalling of set B selection or beam selection and RSRP representation.

- For Model training at the NW side & inference at the UE side (Alt. 3), study model generalization performance, study model transfer/model delivery for cell-specific AI models and non cell-specific AI models.

- For Model training at the UE side, and inference at the UE side (Alt. 2), study cell-specific signals to facilitate data collection.

Figure 1 Feedback overhead for beam reporting in NR design

# Discussion on assistance information

Following the agreements at RN1-109-e, AI/ML inference at UE side and AI/ML inference at network side are to be further studied. In our view, the further study consists of several parts:

- Study the input(s) to the AIML inference model, which may include assistance information in some cases or for some specific models, and at the same time no assistance information may be needed for in some cases or for some specific models.

- The acquisition of assistance information. As captured in the conclusions from RAN1-109-e, “The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side”. Another key factor to consider is user privacy, sacrificing user privacy for better beam management performance is not an acceptable, and how to maintain user privacy and achieve better beam management performance should be explored if assistance information is deemed truly useful.

As the assistance information for use case 1 and assistance information for use case 2 largely overlap, we can discuss assistance information for both jointly:

  - Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.),

  - Tx and/or Rx beam angle

  - expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction)

  - UE position information

  - positioning-related measurement (such as Multi-RTT)

  - UE orientation information

  - UE direction information,

  - Tx beam usage information,

  - increase ratio of L1-RSRP for best N beams

We have several observations:

For Rx beam related information which can be on beam shape (Rx beam pattern including its 3dB beamwidth, Rx beam boresight direction, etc., it may be available to UE vendors/UE chipset vendors.

- If UE side inference is used, it seems it may be assumed the assistance information can be available, but it should be justified to define them explicitly as inputs to the AI/ML inference model. In our understanding, the AI/ML inference model for training/inference Alt. 2 reside on the UE side, and each UE vendor should have freedom to decide what or any assistance information is used.

- The assumption that such information can be conveyed explicitly to the NW side for NW side inference may not be valid, e.g., they may bear proprietary information. We note conventionally the Rx beam operation is purely up to UE implementation. In the P3 operation, with repetition on for CSI-RS, UE can test different RX beams but there is no CSI reporting on RSRP out of that operation. In our view, departing from the conventional practice for RX beam management is not desirable or warranted.

- Further it can be assumed different UE vendors may take different designs in managing Rx beams, a generic Rx beam information is neither desirable nor feasible. As shown in [1], it says “A model-based beam codebook design that assumes ideal omni-directional antenna pattern, and neglects the impact of terminal housing around the antenna, does not work well because the radiation pattern of a practical mmWave antenna combined with the impact of terminal housing is highly irregular. In this paper, we propose a novel and efficient data-driven method to generate a beam codebook to boost the spherical coverage of mmWave terminals”. If a data-driven approach is used in designing the Rx beam or any of the practical issues in UE implementation is considered, it is not clear one can assume beam shape and beam direction remain useful terms to describe Rx beams.

We have

Observation 2: conventionally Rx beam design is transparent to network operation, AI/ML aided/enabled beam management should not depart from that. Whether additional information about Rx beams such as Rx beam shape and Rx beam angle can be useful, or concepts such as Rx beam shape and/or Rx beam orientation can be used in practice need further study.

As for the following assistance information:

- UE position information, or positioning-related measurement (such as Multi-RTT)

- UE orientation information

In general, disclosing UE location information to network raises severe concern on user privacy. To use UE position information for beam management, how to protect user privacy in the whole life cycle of AI/ML model should be studied.

Proposal 2: If UE position information is used AI/ML aided beam management, user privacy needs to be considered in data collection for model training and input for inference with UE position information.

# Conclusion

In this contribution, we provided some discussion on enhancement for AI based beam management. Based on the discussion, we have:

Observation 1: AI/ML models can be crafted as a universal channel parameter estimator with good generalization or as a beam management database and associated query mechanism customized for a specific cell which is not expected to generalize well.

Proposal 1:

- For Model training at the NW side & inference at the NW side (Alt. 1), study efficient signalling of set B selection or beam selection and RSRP representation.

- For Model training at the NW side & inference at the UE side (Alt. 3), study model generalization performance, study model transfer/model delivery for cell-specific AI models and non cell-specific AI models.

- For Model training at the UE side, and inference at the UE side (Alt. 2), study cell-specific signals to facilitate data collection.

Observation 2: conventionally Rx beam design is transparent to network operation, AI/ML aided/enabled beam management does not need to depart from that. Whether additional information about Rx beams such as Rx beam shape and Rx beam angle can be useful, or concepts such as Rx beam shape and/or Rx beam orientation can be used in practice need further study.

Proposal 2: If UE position information is used AI/ML aided beam management, user privacy needs to be considered in data collection for model training and input for inference with UE position information.

# References

- Beam Codebook Design for 5G mmWave Terminals,  Jianhua Mo, Boon Loong Ng, Sanghyun Chang, Pengda Huang, Mandar Kulkarni, Ahmad Alammouri, Jianzhong Charlie Zhang, Jeongheum Lee, Won-Joon Choi, https://arxiv.org/pdf/1908.01004.pdf

- R1-2206032, “Evaluation on AI/ML for CSI feedback enhancement”, vivo, RAN1 #110, August 2022.

- R1-2211807, “Evaluation on AI/ML for beam management”, Apple, RAN1 #111, November 2022

- R1-2301339, “Evaluation for AI/ML based beam management enhancements”, Apple, RAN1 #112, February  2023.