3GPP TSG RAN WG1 #112	                                         	           R1-2301372

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.13.2

Source:	Apple Inc.

Title:	On low power wake-up receiver architectures

Document for:	Discussion/Decision

# Introduction

In Rel-18, a study item was approved for low-power wake-up signal and receiver for NR (WID in RP-222644 [1]), and it includes the following objectives.

- Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

In this contribution, we focus on low-power wake-up receiver (LP WUR) architectures based on past agreements

# LP WUR architectures

In RAN1#110bis-e, a few receiver architectures have been agreed to be studied further for LP WUR, which can be used at least for OOK or adapted for FSK.

Agreement

Study at least the following three types of receiver architectures for LP-WUR:

- Architecture with RF envelope detection

- Heterodyne architecture with IF envelope detection

- Homodyne/zero-IF architecture with baseband envelope detection

- Note: The details of each type of receiver architecture are discussed separately.

- Note: Above receiver architectures are considered suitable for OOK modulation. Some of the architectures

can be applicable for other modulations such as FSK.

For the 3 types of receiver architectures agreed to be studied, we think the architecture with RF envelope detection can be deprioritized.

The following was agreed in RAN1#111:

Agreement

The following observation to be captured in TR38.869:

For the architecture with RF envelope detection,

- It can achieve relatively low power consumption due to the removal of LO/PLL.

- Interference suppression for adjacent channel interference requires very high-Q matching network and/or RF BPF, which is challenging due to the high Q values and may require off-chip components.

- Interference suppression for interference from legacy NR signals and/or other LP WUS on adjacent subcarriers, if performed in RF, requires very high-Q matching network and/or RF BPF, which is challenging due to the high Q values and may require off-chip components.

- The support of multiple bands and/or carriers may require multiple high-Q matching networks and/or RF BPFs or multiple off-chip components.

- RF LNA can be applied to improve sensitivity, with the cost of additional power consumption.

- The noise figure can be relatively high.

There are a few major drawbacks with this architecture:

- Difficulty to achieve good interference rejection performance

  - In this architecture, the interference rejection can only be done through the high-Q matching network and/or RF BPF. Given that this is done at RF frequency, and the bandwidth of the bandpass filter corresponds to the bandwidth of the WUS itself, this would result in an extremely high Q. For example, if the carrier frequency is 4 GHz and the WUS bandwidth is 4 MHz, the Q factor is 1000.

  - If we consider the ACS requirements, the adjacent channel interference can be 30dB+ stronger than the interested signal. This requires the filter to have very strong attenuation factor for the adjacent channels.

  - Overall it would be either impractical to build such a high-Q filter or extremely expensive, and it requires off-chip components.

- Difficulty in the support of multiple bands or different carriers within a band

  - Making the filter tunable on top of the high-Q requirements is even more challenging. This means that multiple filters may need to be built.

- The performance is very sensitive to the interference within the WUS bandwidth, e.g. inter-cell interference.

- Poor sensitivity compared to other receiver architectures

Due to these reasons, we propose to deprioritize the study based on this architecture even though it can achieve very low power consumption.

Proposal 1: De-prioritize the study on the architecture with RF envelope detection.

Comparing the heterodyne architecture and the homodyne/zero-IF architecture, a high-level comparison is summarized in Table 1.

|  | heterodyne architecture | homodyne/zero-IF architecture |
| --- | --- | --- |
| Interference rejection | IF BPF | BB LPF/BPF |
| Power consumption | Relatively higher | Relatively lower |
|  | Image rejection needed | Flicker (1/f) noise |
| Tunable band/carrier | Achieved by tunable LO. There could be multiple LOs to cover different frequency bands to achieve lower power consumption. |  |

When we investigate LP WUR architecture, it is very important to keep ACS requirements in mind, at least until RAN4 provides feedback that the current ACS requirements can be potentially relaxed or modified. With the current ACS requirements, the adjacent channel interference can be ~32 dB more than the wanted signal.

If we assume zero-IF architecture, the signal after down-converting to baseband typically goes to the ADC after an analog BB LPF/BPF, but this analog filter usually has limited attenuation on adjacent channel interference. Therefore, the ADC still needs to have very large dynamic range (potentially 50dB+) to handle the high interference level, which requires a large number of bits. This in turn will impact the power consumption of ADC.

Similarly, for heterodyne architecture, the IF BPF needs to be able to handle such stringent ACS requirements, potentially requiring large area and/or extra power consumption.

Even though we have sent an LS to RAN4, it is unclear when RAN4 will be able to provide feedback. Therefore, it is important that RAN1 still considers ACS requirements in the architecture evaluation. Otherwise, the observations/conclusions may not be realistic.

Proposal 2: ACS requirements should be considered in the architecture evaluation.

In addition, the following was agreed under AI 9.13.3, and legacy OFDM-based signals/channels is also listed as one of the candidate waveforms to be considered further.

Agreement

- Study generation and link performance of multi-carrier (MC)-ASK (including OOK) waveform

  - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS.

    - Note that above does not preclude DFT-S-OFDMA

- Study generation and link performance of multi-carrier (MC)-FSK waveforms

  - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS.

- Study link performance of OFDMA-based signals/channels considering at least the existing signal/channel structure (e.g. CSI-RS, SSS)

  - Other signal/channel structures are not precluded

- For next meeting, companies to provide input on aspects to consider that might impact link performance

For legacy OFDM-based signals/channels, an OFDM-based receiver, or a simplified version, can be used. Either the heterodyne architecture or the homodyne/zero-IF architecture can be considered. The more promising signal in our view is to use sequence-based signal, such as SSS-like signal or ZC sequence-based signal. The LP WUR would need to perform a correlation with the interested sequence. For SSS-like sequence, an FFT is still necessary as the sequence correlation needs to be performed in frequency domain. For ZC sequence, the correlation can be potentially done in time domain and FFT may be skipped. An example zero-IF receiver architecture is shown in Figure 1.

The main concern against OFDM-based receiver was the relatively higher power consumption compared to OOK/FSK-based receiver, due to the higher requirements on frequency/phase accuracy which leads to higher power consumption. However, in our companion contribution [2], our evaluation results show that the power saving gain is not very sensitive to the power consumption of LP WUR, as long as it is sufficiently low (e.g. 10 mW or less). In addition, for sequence-based non-coherent correlation detection, the high phase accuracy is no longer necessary. Therefore, OFDM-based receiver architecture should be also studied, because it has the potential benefit of reusing (part of) the existing MR receiver architecture, reusing legacy NR signals, and may be better at satisfying existing ACS requirements and more efficiently multiplexing with existing NR signals.

Figure 1 Example zero-IF receiver architecture for legacy OFDM-based sequence

Observation 1: OFDM-based receiver architecture has the potential benefit of maximally reusing the existing MR receiver architecture, reusing legacy NR signals, and may be better at satisfying existing ACS requirements and more efficiently multiplexing with existing NR signals.

In terms of how to reduce the power consumption compared to the MR, the following possibilities can be considered:

- For RF front-end,

  - Lower performance LNA/amplifier (e.g. lower power consumption with lower gain and/or higher noise figure)

  - Lower performance oscillator, the buffer amplifier and PLL

  - Lower power ADC (e.g. smaller bit width and possibly lower sampling rate with smaller bandwidth WUS)

- For baseband, separate circuits can be dedicated to WUR for signal processing, which would allow the optimization on power consumption.

We had a very preliminary analysis that showed it is possible to achieve up to 90% power reduction compared to MR by largely reusing the existing MR architecture at RF FE, with the cost of up to 20dB increase in noise figure. Further investigation is necessary to understand the achievable performance.

From the literature, there are only a few papers that provide some hint on the possibility of building low-power OFDM-based receivers for our purpose here. The most relevant ones are [3] and [4]. Both of them are for NB-IoT, meaning the bandwidth is 180 kHz. In [3], a full NB-IoT transceiver is built, with the power consumption of 11.8 mW for receiving. In [4], a NB-IoT WUR based on the standardized NWUS is presented, and the RF FE consumes 2.1 mW, and BB processing is expected to add some additional power consumption (but may not be significant due to simple processing of NWUS). These papers present the possibility of achieving the power consumption of ~10 mW or less with OFDM-based receiver.

|  | [3] NB-IoT | [4] NB-IoT WUR |
| --- | --- | --- |
| Frequency | 450 – 2200 MHz | 750 - 960 MHz |
| Power consumption | 11.8 mW for receiving | 2.1 mW (RF only, BB not considered) |
| Sensitivity | -140 dBm NRSRP, 164 dB max coupling loss | -109 dBm (no repetition) |
|  | 55 nm CMOS RF-SoC | 28 nm CMOS |
|  | -15 dBm out-of-band blocker @85MHz away a 10b noise shaping SAR ADC | PLL with a phase noise of -111 dBc/Hz @ 1MHz offset No off-chip SAW filter |

Proposal 3: OFDM-based receiver architecture, based on the heterodyne architecture or the homodyne/zero-IF architecture, is further studied for LP WUR.

# Conclusion

In this contribution, we have discussed LP WUR architectures, and have the following proposals:

Proposal 1: De-prioritize the study on the architecture with RF envelope detection.

Proposal 2: ACS requirements should be considered in the architecture evaluation.

Observation 1: OFDM-based receiver architecture has the potential benefit of maximally reusing the existing MR receiver architecture, reusing legacy NR signals, and may be better at satisfying existing ACS requirements and more efficiently multiplexing with existing NR signals.

Proposal 3: OFDM-based receiver architecture, based on the heterodyne architecture or the homodyne/zero-IF architecture, is further studied for LP WUR.

# Reference

- RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, RAN#97e, September 2021.

- R1-2301371, On performance evaluation for low power wake-up signal, Apple, RAN1#112, Feb. 2023.

- P.S. Tseng; W. Yang; M.J. Wu; L.M. Jin; D.P. Li; E.C. Low; C.H. Hsiao; H.T. Lin; K.H. Yang; S.C. Shen; C.M. Kuo; C.L. Heng; G.K. Dehng, “A 55nm SAW-Less NB-IoT CMOS Transceiver in an RF-SoC with Phase Coherent RX and Polar Modulation TX”, 2019 IEEE Radio Frequency Integrated Circuits Symposium (RFIC).

- Trevor J. Odelberg, Jaeho Im, David D. Wentzloff, “A 2.1mW -109dBm NB-IoT Wake-Up Receiver”, 2021 IEEE Radio Frequency Integrated Circuits Symposium (RFIC).