3GPP TSG RAN WG1 #112			R1-2301396

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.1.1.2

Source: 	Qualcomm Incorporated

Title: 	Supporting two TAs for multi-DCI based mTRP

Document for:	Discussion/Decision

# Introduction

The Rel-18 WID on NR MIMO evolution for downlink and uplink is approved [1], which includes the following objective:

- Study, and if justified, specify the following

  - Two TAs for UL multi-DCI for multi-TRP operation

  - …

For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In Rel-16/17 mTRP, there can be only one single TA per CC, and it is further assumed that the received signals from different TRPs are within the CP at the UE side. In this contribution, we discuss framework and possible enhancements to enable two TAs in a CC that is configured with multi-DCI based mTRP operation in Section 2, address the impact on random access procedures in Section 3, and discuss switching between intra-cell and inter-cell mTRP operations in Section 4.

# Framework, signaling, and other details for two TAs

In RAN1 #110-bis-e, the following was agreed with respect to reference timing of DL reception for TA:

Agreement

For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG

- baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length

- as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length

  - FFS: the maximum Rx timing difference (could be up to RAN4)

  - Other than UE capability details and relevant configuration, no additional RAN1 specification enhancement specific for this case is expected

One remaining issue is how to determine the two DL reference timing. In legacy, the reception of the first detected path (in time) in a reference CC is considered, where the reference CC is SpCell for PTAG, and any of the CCs in the same TAG for STAG. Similar rule can be followed in the case of multi-DCI except that reception of the first detected path (in time) in the reference CC is based on the DL-RS of the active TCI states associated with the corresponding coresetPoolIndex value.

Proposal 1: For multi-DCI multi-TRP operation with two TAs, the two DL reference timings are determined based on the following

- For a TAG associated with a coresetPoolIndex value, “the first detected path (in time) in the reference CC” is based on the DL-RS’s of the active TCI states associated with the same coresetPoolIndex value.

Regarding the association of a TAG with UL channels / signals, the following was agreed in RAN1 #110-bis-e:

Agreement

For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, the four options agreed in RAN1#110bis-e are refined as below (down-selection of one or a combination of the options to be performed in RAN1#111):

- Option 1: Associate TAG to TCI-state/spatial relation

  - Configure TAG ID as part of UL/joint TCI state or spatial relation

  - for UL transmission, the TAG ID associated with the UL/joint TCI state or spatial relation is utilized

- Option 2: Associate TAG to CORESETPoolIndex

  - for dynamically scheduled/activated PUSCH, TAG associated with the CORESET pool index of the CORESET carrying the scheduling/activating PDCCH is utilized for UL transmission

  - for Type 1 CG, P/SP-SRS, and P/SP-PUCCH, coresetPoolIndex is RRC-configured.

  - FFS:   Other signals/channels:  AP-SRS, and dynamic HARQ-ACK

- Option 3: Associate TAG to SSB group (if such an association is agreed in agenda 9.1.1.2). For a UL transmission, UE adopts the TAG associated with the SSB group such that

  - if the PL RS is an SSB, then the UE adopts the TAG associated with the SSB group which the PL RS of the UL transmission belongs to

  - if the PL RS is a CSI-RS, then the UE adopts the TAG associated with the SSB group which the QCL source SSB of the PL RS belongs to

- Option 4:  TAG association performed as follows:

  - for dynamically scheduled/activated channels/signals, TAG associated with the CORESET pool index of the CORESET carrying the scheduling PDCCH is utilized for UL transmission

  - for P/SP UL channels / signals (not scheduled or activated by DCI), TAG ID is RRC-configured.

As discussed in the previous meeting, Option 1 does not work for FR1 without support of unified TCI. Essentially, Option 1 requires three different solutions: One for unified TCI, another for spatial relation for FR2, and a third one for FR1 without support of unified TCI. Also, Option 3 requires either to specify association for each possible DL-RS or to specify multi-chain relationship (if SSBs are divided into two groups) since PL-RS may be CSI-RS and the QCL of the CSI-RS may be TRS, etc. Option 2 and Option 4 are essentially the same. One minor difference between them is that for semi-static UL channels/RSs, a coresetPoolIndex value is RRC-configured in Option 2, while in Option 4, a TAG ID is directly configured. Both approaches can work for the purpose of this sub-agenda. However, configuration of coresetPoolIndex value can be potentially beneficial for other sub-agenda’s as well such as unified TCI extension to multi-DCI based mTRP or for STxMP for multi-DCI.

In addition, Option 1 and 3 have the following issue: Considering the main use case of two non-co-located TRPs (two TAGs needed) with non-ideal backhaul, without additional restrictions, these options allows for PUCCH/PUSCH associated with same coresetPoolIndex to have different TAGs, and at the same time, PUCCH/PUSCH associated with different coresetPoolIndex to have same TAG. This means that we allow out-of-order operation / separate HARQ-Ack feedback (across different coresetPoolIndex) even though both transmissions are toward the same TRP (same TAG), but we do not allow out-of-order operation / separate HARQ-Ack feedback (within the same coresetPoolIndex) even though the two transmissions are toward different TRPs (different TAGs). In other words, one could redefine the Rel-16 operations of mDCI based mTRP with respect to TAG ID differentiation for the case that two TAGs are anyway needed for two TRPs, but such rules are already defined based on coresetPoolIndex in Rel-16. On the other hand, if restrictions are added to Option 1 or Option 3, they essentially become same as Option 2 or Option 4.

Proposal 2: For multi-DCI based multi-TRP operation with two TAs, support Option 2 to associate TAGs to target UL channels/signals.

Agreement

For multi-DCI based multi-TRP operation with two TAs, study how to handle overlapping part between two UL transmissions associated with two TAs, where the study includes:

- whether to introduce scheduling restriction in overlapping part

- whether to introduce dropping rules

- whether specification impact is needed, or if the issue can be handled via implementation

- whether to allow overlapped transmission in case the UE supports STxMP transmission (if STxMP feature is agreed in NR Rel-18)

Conclusion

For multi-DCI based Multi-TRP operation with two TA enhancement, it cannot always be assumed that both TRPs have knowledge of the overlapping region between transmissions corresponding to the two TAs.

- Note: This doesn’t prevent the network from applying scheduling restrictions even if the TRPs have no knowledge of the overlapping region

Regarding overlap between two UL transmissions due to different TAs, the following were agreed in RAN1 #110 and in RAN1 #110-bis-e, respectively:

In legacy, UCI multiplexing procedures and dropping rules are based on logical time. Given that all UL channels in a CC have the same TA, there is no overlap in actual time after resolving the overlap in logical time. However, this is no longer the case with two TAGs in the same CC. For example, if UL channel 1 and UL channel 2 do not overlaps in logical time (w/o considering TA) but they overlap in actual time due to different TA values.

Note that the issue above exists also in legacy but only across different slots and only due to a new TA command. The issue is addressed by reducing the duration of a later slot after applying a TA command. This is captured in 38.213 as “If two adjacent slots overlap due to a TA command, the latter slot is reduced in duration relative to the former slot”. In comparison, in the case of multi-TA in one CC:

- UE may be able to transmit two different UL signals/channels in a CC simultaneously,

- The overlap is not only across different slots but can be also within a given slot,

- The overlap is not only occasionally due to a new TA command but can be also frequent due to the fact that different UL signals/channels are associated with different TAs.

It is important to note that scheduling restrictions can solve the issue as leaving a symbol gap can be enough since MTTD is less than half a symbol or less than one symbol for most sub-carrier scaping (34.6usec in FR1 and 8.5usec in FR2 based on the RAN4 reply LS). It should be noted that for Rel-16 TDM based multi-DCI with non-ideal backhaul, TRPs need to coordinate semi statically to ensure that UL signals / channels do not overlap logically. Now, in addition to that, TRPs needs to leave a symbol margin as part of that semi-static coordination to ensure that UL signals / channels do not overlap physically.

Alternatively, if the issue is addressed by UE dropping rule, we can have two cases

- Case 1: If the dropped part is from the beginning of the later UL (legacy rule), DMRS is lost if UL transmission has front-loaded DMRS (typical case), and hence, the whole UL is lost.

- Case 2: If the dropped part is from the ending of the earlier UL (which is different than legacy rule), coding rate is impacted (as the last symbol is lost)

In both cases, at least one symbol is lost anyways (unless if the overlapping part is very small). Then, it seems that the scheduling restriction (leaving a symbol gap) is a better approach.

A third approach would be to leave this case to UE implementation. In current RAN4 spec, “transient periods” are defined in case of power / RB change. A similar approach may be followed in case of TA change.

Overall, we think that scheduling restriction may be the easiest approach w/o resulting in loosing the whole UL transmission (e.g., which is the case if UE drops part of the front-loaded DMRS symbol, i.e., Case 1 above similar to legacy rule in case of new TA command).

Proposal 3: For TDM multi-DCI based multi-TRP operation with two TAs, UE does not expect the two UL transmissions associated with the two TAs to overlap in actual (physical) time, even partially.

- This applies after UE determines the two UL channels/signals are to be transmitted, i.e., after legacy UCI multiplexing / UL dropping / limitations for UE transmission as described in clause 11.1 of 38.213.

Given that multiple TAGs are also used in legacy UL-CA operation (up to 4 TAGs can be configured for different CCs), some consistency for TAG configurations is needed across different CCs. Among multiple CCs, some may be configured with two coresetPoolIndex values (with multi-DCI based mTRP) while others may be configured with a single value of coresetPoolIndex. For example, for the purpose of HARQ-Ack (separate and joint), there is relationship between coresetPoolIndex value(s) of different CCs for the purpose of HARQ-Ack reporting and codebook construction. In the case of multi-TA in a CC, TAGs configured for different CCs need to be consistent across coresetPoolIndex values of different CCs. Figure 1 illustrates a few examples for consistent and inconsistent configurations, where it is assumed that UE can support up to 4 TAGs overall.  As it can be seen, the top-right and bottom-right examples are not consistent as a same TAG is used in the two CCs but are associated with different coresetPoolIndex values across the two CCs.

Figure 1: Consistency of TA configurations across different CCs.

Proposal 4: Study necessary conditions to ensure consistency of TA configurations across different CCs. Use case for each configuration needs to be justified.

# Impact on random access procedures

In legacy systems for connected mode UEs, PDCCH order can be used to trigger contention-free random access to acquire UL timing when it goes out-of-sync. The PDCCH order for this purpose can be used either for the PCell (for PTAG) or SCell (for STAG). In either case, UE transmits the PRACH in the cell for which the UL timing re-sync is required in response to the PDCCH order, and then RAR PDCCH/PDSCH is received on the PCell. The RAR PDSCH indicates the absolute TA value (12 bits) for the cell in which the PRACH is transmitted on. This is illustrated in Figure 2.

Figure 2: CFRA triggered by PDCCH order to acquire UL timing for PCell / SCell.

In RAN1 #111, CFRA triggered by PDCCH order was agreed for both intra-cell and inter-cell multi-DCI based mTRP with two TAGs:

Agreement

For multi-DCI based Multi-TRP operation with two TA enhancement, support CFRA triggered by PDCCH order for both intra-cell and inter-cell cases.

For intra-cell multi-DCI based mTRP, the following alternatives were agreed in RAN1 #110-bis-e:

Agreement

For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support at least one of the following alternatives (down selection to be done in RAN1#111):

- Alt 1:  indicate TAG ID as part of TA command in RAR

- Alt 2:  indicate TAG ID as part of PDCCH order

- Alt 3:  divide SSBs into two groups, one for each TRP.    If a SSB associated to a RACH procedure belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP.

- Alt 4:  divide RACH resources into two groups, where for a RACH procedure, if the corresponding RACH resource belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP.

- Alt 5:  divide preambles into two groups, where for a RACH procedure, if the corresponding preamble belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP

- Alt 6:   TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order

- Alt 7:  Each TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the TCI state used to receive the PDCCH order

Note: If Alt 1 or Alt 2 is downselected, then it does not preclude indication of two TAG IDs (if supported)

Regarding Alts 3, 4, and 5, such division is not necessary for intra-cell case given that association of SSBs / RACH resources / preambles with the two TRPs is up to the network and is transparent to the UE so far. In fact, the existing CFRA-based PDCCH order is able to trigger PRACH toward either of TRPs for intra-cell mTRP, and the main remaining issue is for the UE to know the absolute TA command in RAR corresponds to which of the two TAGs. Between Alt 6 and Alt 7, Alt 6 does not require additional grouping given that coresetPoolIndex already provides this functionality. Between Alt 2 and Alt 6, Alt2 is preferred given that it is more flexible (it can support cross-TRP PDCCH order even for intra-cell), and it allows for a unified solution between inter-cell and intra-cell cases.

On the other hand, the problem with Alts 2, 6, and 7 is that they are not applicable to UE-initiated CBRA. Given that RAN1 has already agreed that the decision related to UE-initiated CBRA is up to RAN2, selecting any of the Alts 2, 6, or 7 means potential misalignment between RAN1 and RAN2. Even w/o enhancing UE-initiated CBRA, the UE may select an SSB from either the first TRP or the second TRP for CBRA in which case Alt1 is necessary to avoid the confusion that TA command should be applied to which of the two TAGs unless if UE always falls back to single-TAG operation after any UE-initiated CBRA. This fallback behaviour may require some spec impacts and should be discussed in case Alts 2, 6, or 7 are agreed.

Given the discussions above, our first preference is Alt1. Our second preference is Alt2 (to have a unified solution with inter-cell), but in that case, a fallback behaviour as discussed above is needed.

Proposal 5: For CFRA-based PDCCH order in case of intra-cell multi-DCI based multi-TRP operation with two TA enhancement:

- First preference: Support to indicate TAG ID (first TAG or second TAG) as part of TA command in RAR (Alt1)

- Second preference:

  - Support to indicate TAG ID (first TAG or second TAG) as part of PDCCH order (Alt2).

  - UE always falls back to single-TAG operation after any UE-initiated CBRA.

    - FFS: Details.

For inter-cell multi-DCI based mTRP, the following were agreed in RAN1 #110-bis-e and RAN1 #111, respectively:

Agreement

For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, support a mechanism to determine which PRACH configuration (i.e., RACH configuration corresponding to serving cell PCI or an additional PCI) to be used in the RACH procedure triggered by PDCCH order

- FFS:  Explicit indication or implicit indication through PDCCH order

Working Assumption

For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI

- the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI

For determining which PRACH configuration to be used in the PRACH procedure triggered by PDCCH order, explicit indication in the PDCCH order DCI payload is needed given that 7 additional PCIs may be configured. It should be noted that there are already 10 reserved bits in PDCCH order DCI, which can be used for this purpose.

Proposal 6: For CFRA-based PDCCH order in case of inter-cell multi-DCI based multi-TRP operation with two TA enhancement, support PDCCH order DCI payload to indicate a RACH configuration corresponding to the serving cell PCI or one of the configured additional PCIs (explicit indication).

Agreement

For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, support one of the alternatives (down selection to be done in RAN1#111):

- Alt 1: PDCCH scheduling RAR will always be received from serving cell  there is no need for additional type 1 CSS configuration per additional PCI

- Alt 2: In addition to PDCCH scheduling RAR being received from serving cell, reception of PDCCH scheduling RAR from a TRP corresponding to an additional PCI for a RACH procedure associated to the additional PCI is supported  additional type 1 CSS configuration per additional PCI needs to be supported

Regarding PDCCH scheduling RAR in case of inter-cell multi-DCI based multi-TRP, the following was agreed  in RAN1 #110-bis-e:

The aspect above (PDCCH scheduling RAR) should be discussed separately for the active additional PCI and inactive additional PCIs (given that in Rel-17 inter-cell mTRP, up to 7 additional PCIs can be RRC-configured, and only one of them can be active). This is because when a PCI is not active, UE does not transmit any UL channel/signal toward that TRP until TCI state activation MAC-CE indicates a TCI state associated with that PCI. Hence, the indication of absolute TA in RAR PDSCH may not be necessary, but it may still be possible to trigger CFRA by PDCCH order (i.e., w/o the subsequent RACH procedures) for the inactive PCI to acquire the timing and get prepared before that PCI is activated.

With respect to Alt1/Alt2 of the agreement above, the following should be noted:

- If the multi-DCI based mTRP is defined in a SCell, Alt1 versus Alt2 is not relevant as the PDCCH scheduling RAR (Type 1 CSS) can be only transmitted in the PCell. PCell may not even be configured with multi-DCI based mTRP.

- For an inactive additional PCI, Alt2 is not possible as PDCCH cannot be transmitted from that TRP (because the TCI state(s) from that TRP is not activated yet). Even with Alt1, UE should not apply the absolute TA command of RAR since that PCI is still inactive. Hence, the following possibilities may be considered for inactive additional PCI:

  - Possibility 1: RAR is not transmitted to the UE (the random-access ends after PRACH transmission). The TRP associated with the inactive PCI can determine the TA based on measuring the PRACH, and can send TA commands to adjust the timing once it is activated.

  - Possibility 2: RAR is transmitted to the UE (e.g., from the serving cell in PCell) but the UE does not apply it. Instead, UE saves the absolute TA command and applies it after a TCI state associated with that PCI becomes active.

- For an active additional PCI, both Alt1 and Alt2 are possible at least for the PCell. However, in Rel-17 inter-cell mTRP, Type1 CSS cannot be from an additional PCI.

  - If this condition is kept, it means that Type-1 CSS still needs to come from the serving PCI of PCell (Alt1). Then, the following two aspects need to be addressed:

    - a) whether additional delay for starting the MSG2 response window would be needed to accommodate the non-ideal backhaul between two TRPs, and

    - b) whether the existing QCL relationship in SpCell between PDCCH order DCI and RAR PDCCH/PDSCH needs to be relaxed when PDCCH order is coming from additional PCI but Type1 CSS is coming from serving PCI.

  - If this condition is relaxed, how to receive Type 1 CSS from additional PCI requires some discussions (either TCI state of the CORESET associated with the Type1 CSS needs to change by MAC-CE, or a second Type1 CSS needs to be defined)

Considering the discussions above, and the fact that multi-DCI based mTRP may be even configured for SCell (where Type 1 CSS is not transmitted), we prefer the direction of Alt1 for active additional PCI, and we think more discussions would be needed for inactive additional PCI.

Proposal 7: For inter-cell multi-DCI based multi-TRP operation with two TAGs in a CC:

- If PDCCH order triggers PRACH transmission toward an active additional PCI in the CC, PDCCH scheduling RAR is received from serving cell PCI of the SpCell.

  - FFS: Whether additional delay for starting the RAR response window is needed to accommodate the non-ideal backhaul.

  - FFS: When the CC is SpCell, whether the existing QCL relationship in SpCell between PDCCH order DCI and RAR PDCCH/PDSCH needs to be relaxed when PDCCH order is from additional PCI but Type1 CSS is coming from serving PCI.

- If PDCCH order triggers PRACH transmission toward an inactive additional PCI in the CC, study the following possibilities:

  - Possibility 1: RAR is not transmitted to the UE.

  - Possibility 2: PDCCH scheduling RAR is received from serving cell PCI of the SpCell (same as active additional PCI), but the UE does not apply it until after a TCI state associated with the PCI becomes active.

Agreement

For multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP at least for inter-cell Multi-DCI.

- FFS: for intra-cell Multi-DCI

- FFS: whether there are any restrictions needed

- FFS: if cross TRP RACH triggering is an optional feature

In RAN1 #111, the following was agreed with respect to cross-TRP PDCCH order:

One remaining issue is the PRACH power control in case of cross-TRP PDCCH order. In existing spec, PL-RS and referenceSignalPower for PRACH power control are based on the DL-RS that the DMRS of PDCCH order is QCLed with. However, this rule is not applicable to cross-TRP PDCCH order. In order to understand how the issue should be resolved, RAN1 first needs to decide for which scenarios the cross-TRP PDCCH order should be allowed in case of inter-cell mTRP.

Let’s call this legacy power control rule for PRACH in case of CFRA-based PDCCH order as “Rule 1”. Let’s also call the QCL rule between PDCCH order and RAR in case of CFRA-based PDCCH order in PCell as “Rule 2”, and further assume that PDCCH scheduling RAR is received from serving cell PCI of the SpCell as discussed above. Furthermore, let’s assume coresetPoolIndex value 0 (TRP0) is associated with the serving cell PCI (PCI0), and coresetPoolIndex value 1 (TRP1) is associated with PCI1 (active additional PCI). Hence, RAR is always from TRP0. Considering the sequence of PDCCH order  PRACH  RAR, we have the following cases for the three different scenarios of triggering PRACH toward serving cell PCI (PCI0), active additional PCI (PCI1), and inactive additional PCI (PCI2):

| PRACH toward | PDCCH order PRACH RAR | Rule 1 | Rule 2 |
| --- | --- | --- | --- |
| PCI0 (serving PCI) | Case 1: TRP0 PCI0 TRP0 | Ok | Ok |
|  | Case 2: TRP1 PCI0 TRP0 | Violated | Violated |
| PCI1 (active additional PCI) | Case 1: TRP0 PCI1 TRP0 | Violated | Ok |
|  | Case 2: TRP1 PCI1 TRP0 | Ok | Violated |
| PCI2 (inactive additional PCI) | Case 1: TRP0 PCI2 TRP0 | Violated | Ok |
|  | Case 2: TRP1 PCI2 TRP0 | Violated | Violated |

As it can be seen from the table above:

- When PRACH is triggered for serving cell PCI, cross-TRP PDCCH order is not needed, and Case 1 is enough, which does not violate Rule 1 or Rule 2.

- When PRACH is triggered for active additional PCI, both Case 1 and Case 2 can be supported:

  - Case 1 corresponds to cross-TRP PDCCH order and has the benefit that Rule 2 is not violated. However, Rule 1 is violated and an alternative rule is needed.

  - Case 2 corresponds to same-TRP PDCCH order and should be supported as agreed before.

- When PRACH is triggered for inactive additional PCI, Case 1 can be enough. This assumes that RAR is transmitted. Otherwise, Rule 2 becomes irrelevant and Case 1 and Case 2 become similar. Note that whether RAR is transmitted for this scenario is a separate discussion (as discussed in detail before). In any case, Rule 1 is violated, and an alternative rule is needed.

Hence, cross-TRP PDCCH order should be supported for both active additional PCI and inactive additional PCI. For the case of active additional PCI and in Case 1 above, the PRACH power control can be based on the DL-RS of the TCI state of the CORESET with the lowest ID among the CORESETs associated with coresetPoolIndex value 1. For the case of inactive additional PCI, the PRACH power control can be based on the SSB index that is indicated by PDCCH order DCI. However, note that this assumes that the UE has already measured that SSB index from that inactive additional PCI. Hence, it should be configured for L1-RSRP report.

Proposal 8: For inter-cell multi-DCI based multi-TRP operation with two TAGs in a CC:

- If the serving cell PCI is indicated by PDCCH order DCI, the legacy procedure is applied for PRACH power control (cross-TRP PDCCH order is not allowed).

- If the active additional PCI is indicated by PDCCH order DCI:

  - If the PDCCH order DCI is associated with coresetPoolIndex value 1 (same-TRP PDCCH order): The legacy procedure is applied for PRACH power control.

  - If the PDCCH order DCI is associated with coresetPoolIndex value 0 (cross-TRP PDCCH order): The PRACH power control can be based on the DL-RS of the TCI state of the CORESET with the lowest ID among the CORESETs associated with coresetPoolIndex value 1.

- If an inactive additional PCI is indicated by the PDCCH order DCI (only cross-TRP PDCCH order is possible), the PRACH power control is based on the SSB index that is indicated by the PDCCH order DCI, and the UE expects the SSB index to be configured for L1-RSRP report.

In 2-step RACH, the “Absolute Timing Advance Command MAC CE” can be included in a PDSCH scheduled by a PDCCH in response to MSGA transmission (in case of C-RNTI). This MAC-CE currently has two octets with 12 bits for TA command and 4 reserved bits. In RAN1 #110-bis-e, the following was agreed

Agreement

For multi-DCI based Multi-TRP operation with two TA enhancement, support enhancements related to indicating TAG ID via absolute TA command:

- FFS: whether the indication is implicit or explicit

- Detailed indication schemes are FFS

- This does not preclude indication of two TAG IDs (if supported)

- Note: This applies at least to MSGB in case of C-RNTI

:

Our understanding is that the agreement above is specific to “Absolute Timing Advance Command MAC CE” for SpCell (given that 2-step RACH can be only configured for the SpCell). Hence, we propose the following:

Proposal 9: For SpCell configured with two TAGs, support using a reserved bit of the “Absolute Timing Advance Command MAC CE” to indicate whether the TA command corresponds to the first TAG or the second TAG.

So far, only one random-access procedure can be ongoing at any point in time in a MAC entity. However, with 2 TA enhancements for multi-DCI based mTRP, there is some motivation for allowing two parallel random-access procedures. Note that this does not mean that PRACH transmissions are simultaneous, but it means while one random-access procedure is ongoing, another random-access procedure can also start. This enhancement allows for reducing latency for UL synchronization when both TAs are out-of-sync. Furthermore, in the case of non-ideal backhaul, one TRP need to know exactly when an ongoing random-access procedure for the other TRP is finished before it can initiate another one, e.g., by sending a PDCCH order to acquire UL timing.

Proposal 10: For multi-DCI based mTRP with two TAs, support two parallel random-access procedures in a MAC entity.

# Switching between intra-cell and inter-cell mTRP

In Rel-17 inter-cell mTRP, up to 7 additional PCIs can be configured while one of them may be activated based on TCI state activation MAC-CE. The MAC-CE can also switch between intra-cell and inter-cell mTRP based on whether both coresetPoolIndex values have activated TCI states associated with the serving cell PCI, or one coresetPoolIndex value has activated TCI states associated with serving cell PCI and the other coresetPoolIndex value has activated TCI states associated with an additional PCI.

At the same time, the configuration of two TAGs for a CC configured with multi-DCI based mTRP and additional PCIs is RRC-based. Then, depending on the deployment scenario, network should be able to operate in one of the following assumptions:

- Assumption 1: UE maintains two TAGs irrespective of intra-cell or inter-cell mTRP.

  - With this assumption, CFRA-based PDCCH order should be able to acquire TA for serving cell PCI associated with coresetPoolIndex value 0, serving cell PCI associated with coresetPoolIndex value 1, additional PCI #1, additional PCI #2, …, additional PCI #7 (up to 9 possibilities).

  - Note that only two TAs out of the 9 possibilities above are active at any given time (which correspond to the two maintained TAGs) depending on intra-cell mTRP or inter-cell mTRP as well as which additional PCI is active.

- Assumption 2: UE maintains only one TAG if all active TCI states across both coresetPoolIndex values are associated with the serving cell PCI.

  - With this assumption, one TAG is associated with the serving cell PCI, and another TAG is associate with an additional PCI. Hence, UE switches between single-TAG and two-TAGs depending on whether the UE is in intra-cell or inter-cell mTRP mode.

Proposal 11: If a CC is RRC-configured with two TAGs and one or more additional PCIs with multi-DCI based mTRP operation, study the following assumptions when TCI state activation MAC-CE(s) result in switching between intra-cell and inter-cell multi-DCI based mTRP:

- Assumption 1: UE maintains two TAGs irrespective of intra-cell or inter-cell mTRP.

- Assumption 2: UE maintains only one TAG if all active TCI states across both coresetPoolIndex values are associated with the serving cell PCI.

# Conclusion

In this contribution, we proposed:

Proposal 1: For multi-DCI multi-TRP operation with two TAs, the two DL reference timings are determined based on the following

- For a TAG associated with a coresetPoolIndex value, “the first detected path (in time) in the reference CC” is based on the DL-RS’s of the active TCI states associated with the same coresetPoolIndex value.

Proposal 2: For multi-DCI based multi-TRP operation with two TAs, support Option 2 to associate TAGs to target UL channels/signals.

Proposal 3: For TDM multi-DCI based multi-TRP operation with two TAs, UE does not expect the two UL transmissions associated with the two TAs to overlap in actual (physical) time, even partially.

- This applies after UE determines the two UL channels/signals are to be transmitted, i.e., after legacy UCI multiplexing / UL dropping / limitations for UE transmission as described in clause 11.1 of 38.213.

Proposal 4: Study necessary conditions to ensure consistency of TA configurations across different CCs. Use case for each configuration needs to be justified.

Proposal 5: For CFRA-based PDCCH order in case of intra-cell multi-DCI based multi-TRP operation with two TA enhancement:

- First preference: Support to indicate TAG ID (first TAG or second TAG) as part of TA command in RAR (Alt1)

- Second preference:

  - Support to indicate TAG ID (first TAG or second TAG) as part of PDCCH order (Alt2).

  - UE always falls back to single-TAG operation after any UE-initiated CBRA.

    - FFS: Details.

Proposal 6: For CFRA-based PDCCH order in case of inter-cell multi-DCI based multi-TRP operation with two TA enhancement, support PDCCH order DCI payload to indicate a RACH configuration corresponding to the serving cell PCI or one of the configured additional PCIs (explicit indication).

Proposal 7: For inter-cell multi-DCI based multi-TRP operation with two TAGs in a CC:

- If PDCCH order triggers PRACH transmission toward an active additional PCI in the CC, PDCCH scheduling RAR is received from serving cell PCI of the SpCell.

  - FFS: Whether additional delay for starting the RAR response window is needed to accommodate the non-ideal backhaul.

  - FFS: When the CC is SpCell, whether the existing QCL relationship in SpCell between PDCCH order DCI and RAR PDCCH/PDSCH needs to be relaxed when PDCCH order is from additional PCI but Type1 CSS is coming from serving PCI.

- If PDCCH order triggers PRACH transmission toward an inactive additional PCI in the CC, study the following possibilities:

  - Possibility 1: RAR is not transmitted to the UE.

  - Possibility 2: PDCCH scheduling RAR is received from serving cell PCI of the SpCell (same as active additional PCI), but the UE does not apply it until after a TCI state associated with the PCI becomes active.

Proposal 8: For inter-cell multi-DCI based multi-TRP operation with two TAGs in a CC:

- If the serving cell PCI is indicated by PDCCH order DCI, the legacy procedure is applied for PRACH power control (cross-TRP PDCCH order is not allowed).

- If the active additional PCI is indicated by PDCCH order DCI:

  - If the PDCCH order DCI is associated with coresetPoolIndex value 1 (same-TRP PDCCH order): The legacy procedure is applied for PRACH power control.

  - If the PDCCH order DCI is associated with coresetPoolIndex value 0 (cross-TRP PDCCH order): The PRACH power control can be based on the DL-RS of the TCI state of the CORESET with the lowest ID among the CORESETs associated with coresetPoolIndex value 1.

- If an inactive additional PCI is indicated by the PDCCH order DCI (only cross-TRP PDCCH order is possible), the PRACH power control is based on the SSB index that is indicated by the PDCCH order DCI, and the UE expects the SSB index to be configured for L1-RSRP report.

Proposal 9: For SpCell configured with two TAGs, support using a reserved bit of the “Absolute Timing Advance Command MAC CE” to indicate whether the TA command corresponds to the first TAG or the second TAG.

Proposal 10: For multi-DCI based mTRP with two TAs, support two parallel random-access procedures in a MAC entity.

Proposal 11: If a CC is RRC-configured with two TAGs and one or more additional PCIs with multi-DCI based mTRP operation, study the following assumptions when TCI state activation MAC-CE(s) result in switching between intra-cell and inter-cell multi-DCI based mTRP:

- Assumption 1: UE maintains two TAGs irrespective of intra-cell or inter-cell mTRP.

- Assumption 2: UE maintains only one TAG if all active TCI states across both coresetPoolIndex values are associated with the serving cell PCI.

- References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung.