3GPP TSG RAN WG1 #112			R1-2301435

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.11.4

Source: 	Qualcomm Incorporated

Title: 	Improved GNSS Operations for IoT-NTN

Document for:	Discussion and Decision

# Background

In RAN#94-e, the following objectives with RAN1 impact were included in the WID for eMTC/NB-IoT over NTN in Release 18:

IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below:

- Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

- Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption [RAN1]

In RAN1#110b-e, the following was agreed:

Agreement

- Support eNB to at least aperiodically trigger UE to make GNSS measurement.

Agreement

- If eNB aperiodically triggers UE to make GNSS measurement, a MAC CE is used.

Agreement

UE reports GNSS position fix time duration for measurement at least during the initial access stage

- which message carries this information is up to RAN2

Agreement

In connected mode, UE may report GNSS validation duration with MAC CE.

In RAN1#111, the following was agreed:

Agreement

For GNSS measurement in RRC connected, if eNB aperiodically triggers connected UE to make GNSS measurement, UE can re-acquire GNSS position fix with a gap

- FFS details of gap configuration

The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement

- FFS based on configured timing

In this contribution, we provide our views on aspects related to improved GNSS operations for IoT-NTN.

# Baseline power consumption due to GNSS fixes

  - Parameters for evaluation of power consumption

To evaluate the power consumption impact of reading GNSS to maintain uplink synchronization (by appropriately pre-compensating the time and frequency offsets), we provide some reference parameters associated with the different transmit and receive operations that a UE may perform during establishing and maintaining a connection.

The key elements in evaluating power consumption associated with a transmit/receive operation at the UE are the current drawn by the UE (e.g., in milliamperes) and time-duration of the operation (e.g., in milliseconds). In Table 1 below, we report these numbers (with current values relative to a reference downlink reception current) for a typical NB-IoT over NTN scenario (e.g., a good coverage LEO satellite setting for Set 2 in [1]) corresponding to a downlink SNR (for 15 kHz 1 PRB reception) of 0 dB and an uplink SNR (for 15 kHz 1 PRB transmission) of -5 dB (with a PC5 UE transmitting at the max. power of 20 dBm).

Table 1: Parameters for evaluating power consumption in IoT over NTN.

| Operation | Current (Referenced to downlink current CDL) | Duration |
| --- | --- | --- |
| GNSS reception | CGNSS=1.1×CDL | 2000 ms |
| Downlink Reception | CDL | - PDCCH: 40 ms - PDSCH (RAR): 8 ms - PDSCH (Msg4): 8 ms - PDSCH (Conn. Release): 8 ms |
| Uplink Transmission | CUL=4.3×CDL | - PRACH: 1.6 ms - Msg3: 8 ms - PUSCH (data): 8 ms per ~1000 bits (simulated with 8000 bits per ON-duration) - HARQ-ACK: 8 ms |
| Sleep | Csleep=5.5×10-5×CDL | - PSM: 8 hrs - CDRX: ~10s |

  - Power consumption—short, sporadic connections

We model a short, sporadic connection according to Fig. 1 below. Essentially, the IoT UE is assumed to transmit its payload once every 8 hrs, once every 24 hrs, etc; after transmitting the payload, the UE’s connection is released, and it goes back into deep sleep mode, until before the next transmission occasion.

Figure 1: Short, sporadic transmissions for IoT over NTN.

In this setting, we observe that, acquiring a GNSS fix before every connection results in ~34% of the UE’s total power consumption resulting from GNSS alone. While there may be subtle differences depending on exact implementation, the general observation that GNSS consumes a significant chunk of the UE’s power—and thereby, directly reduces the expected battery life of the UE—is valid.

Observation 1: Under the studied scenario of short, sporadic connections, a GNSS fix before every connection consumes approximately 34% of the UE’s total available energy.

For UEs that are mobile, e.g., say tracking devices, etc., that are operating in this short, sporadic connection paradigm, this power penalty due to GNSS cannot be mitigated significantly, under the purview of Release 17 assumptions of GNSS-based uplink pre-compensation.

However, for UEs that are fixed, e.g., smart meters, etc, these UEs may be able to save power by having a much more relaxed (e.g., once a week, or once a month, depending on the setting) GNSS fixing schedule.

  - Power consumption—long connections (e.g., based on CDRX)

We model an example long connection according to Fig. 2 below. In this instance, the IoT UE may remain in connected mode for a significantly longer duration of time than the short, sporadic connections described above. These may be facilitated e.g., via connected mode DRX (CDRX), wherein much larger payloads are transmitted or received by the UE during the longer connection.

Figure 2: Long connection with connected mode DRX for IoT over NTN.

In this setting, we observe that a GNSS fix before every uplink transmission occasion (which, in the absence of other mechanisms, may end up being required to maintain uplink synchronization accuracy within specified limits) results in ~45% of the UE’s total power consumption resulting from GNSS fixes. While (as described above) we can mitigate this somewhat for fixed UEs, for mobile UEs (especially UEs moving at high speeds), without other enhancements to connected mode synchronization, we may not be able to avoid this hit to the UE’s power consumption on account of GNSS fixes.

Observation 2: Under the studied scenario of a long connection employing connected mode DRX (with a DRX cycle of ~10s), a GNSS fix before every uplink transmission consumes approximately 45% of the UE’s total available energy without additional enhancements w.r.t uplink synchronization.

- This is especially true for mobile UEs that cannot depend on a prior acquired GNSS fix

# General framework

In previous meetings, several aspects of GNSS operations have been discussed, including eNB-controlled GNSS re-acquisition, UE-autonomous GNSS re-acquistion, and usage of closed loop time and frequency commands.

In our view, RAN1 should design a framework such that the most efficient mechanism is used, depending on the situation. In particular, and as studied in Section 2 (also captured in TR 36.763), GNSS measurements require a large amount of power and, therefore should be avoided if there is an alternative.

Therefore, the overall framework for improved GNSS operations and power consumption should be as follows:

- GNSS measurements should be minimized as much as possible. As we study in Section 4, closed loop time and frequency commands can extend the connection duration of a UE with very small specification impact and signaling overhead.

- If the time and frequency drift degrades up to the point where closed loop commands are no longer effective, the network may indicate explicitly to the UE that it shall reacquire GNSS.

- In some cases, the network may not be able to send the indication in (2) successfully to the UE, due to e.g. network load or bad instantaneous channel conditions. In this case, there should be a mechanism under which the UE is able to perform autonomous GNSS measurements during a recovery procedure without the need to go to IDLE mode, similar to the one defined in Rel-17 for ephemeris.

Proposal 1: For the objective on improved GNSS operations and reduced power consumption, the following framework is followed by RAN1:

- Specify closed loop time and frequency enhancements to increase the time between consecutive GNSS re-acquisitions.

- Specify aperiodic eNB triggers to instruct the UE to re-acquire GNSS (already agreed).

- Specify a mechanism for the UE to re-acquire GNSS upon expiry of the GNSS validity (already agreed).

Regarding points 2 and 3, we propose to have a common framework to treat the GNSS validity. This framework would also enable other mechanisms that were previously discussed in RAN1 but not agreed (e.g. UE acquiring GNSS during C-DRX).

The framework for updating GNSS validity relies on the UE reporting a new validity via MAC CE (as agreed) after each GNSS fix, regardless of it being UE autonomous or eNB triggered. After the validity duration is updated, the UE and eNB will be in sync with respect to the remaining time. For this, the following specification support is needed:

- After a GNSS reacquisition the UE will inform of the new validity duration to the eNB via MAC CE. This behavior is the same for the different GNSS reacquisition techniques (UE autonomous, eNB tiggered, or even during C-DRX).

- The MAC CE indicates the “remaining validity duration”, i.e., the UE may report multiple times the same validity duration after subsequent fixes.

Proposal 2: After a GNSS reacquisition, the UE reports its new GNSS validity duration to the eNB:

- The validity duration indicates the “remaining validity duration”.

- The reporting is performed regardless of the method of GNSS reacquisition (UE autonomous, eNB triggered, etc.)

# Closed loop time and frequency correction

  - Power savings

We show, in this section, that the above penalty to UE power consumption (during long connections) from GNSS fixes can be mitigated significantly by facilitating closed-loop time and frequency corrections issued by the base-station.

The framework for closed loop time and frequency corrections would follow a similar principle as the one used today in NB-IoT for time corrections.

- Based on transmission of an uplink signal (e.g. (N)PUSCH, (N)PRACH), the network estimates the observed time and/or frequency error.

- The network issues closed loop commands for the UE to adjust its transmission time and/or frequency. With these closed loop corrections, the UE can skip a GNSS fix and keep operating with acceptable time and frequency errors.

Such a strategy is (approximately) schematically depicted in Fig. 3 below.

Figure 3: Relaxed GNSS fixing using NPRACH-based closed loop corrections.

In Table 2, we demonstrate how the power penalty due to GNSS fixes diminishes with an increase in the GNSS relaxation factor, which essentially denotes—relative to the baseline scheme in Fig. 2—the factor by which we reduce the total number of GNSS fixes required and replace it with adequate close-loop corrections.

Table 2: Reduction in power penalty due to GNSS, by relaxing required GNSS fixes and enabling NPRACH-driven closed-loop time and frequency corrections.

| GNSS Relaxation factor | Power penalty due to GNSS in connected mode for long connections |
| --- | --- |
| No Relaxation (Baseline, as shown in Fig. 4) | 45% |
| 2 | 29% |
| 4 | 17% |

Based on the results above, we make the following observations and proposals with regards to supporting long connections in eMTC and NB-IoT over NTN.

Observation 3: For long connections in eMTC and NB-IoT over NTN, closed-loop time and frequency corrections lowers the GNSS power penalty from 45% to 17% (with a GNSS relaxation factor of 4), w.r.t a baseline without closed-loop corrections.

Proposal 3: RAN1 to specify enhancements to closed-loop time and frequency corrections, to mitigate UE power consumption on account of GNSS fixes.

  - Modifications to base TA for PRACH transmission

Currently, a UE uses the following formula to determine the timing advance—i.e., the offset applied to an uplink frame w.r.t a corresponding downlink frame

TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs

where NTA denotes the TA component based on accumulating “TA commands” received from the base-station.

In response to NPRACH (received in a RAR), the TA command specifies the “entire NTA” (as opposed to “incremental” updates).

The implication of this is that any time a UE transmits a NPRACH, it uses a value of NTA=0 (i.e., it flushes any accumulated TA commands that it may have received, e.g., via MAC-CEs). Thus, as an example, an NPRACH triggered by a “PDCCH order” in connected mode also uses NTA=0.

Observation 4:  According to current specifications, any time a UE transmits a NPRACH, it uses a value of NTA=0 in calculating the TA for the NPRACH transmission.

In NTN, NTA, adjUE denotes the TA component specific to satellite communications and is determined by the UE (to “pre-compensate” large UE-satellite round-trip delays) based on the UE’s own location (e.g., by acquiring a “GNSS position fix”), the NTN serving satellite’s location (e.g., by reading the satellite ephemeris info broadcast on SIBs), and the common TA.

The accuracy (over time) of NTA, adjUE depends on the accuracy of location info. For example, if a considerable amount of time has passed since the last GNSS position fix, say for a mobile UE, the accuracy of this term can be thrown off.

Observation 5: If a considerable amount of time has passed since the last GNSS position fix, e.g., for a mobile UE, the accuracy of NTA, adjUE becomes progressively worse over time, i.e., the error in NTA, adjUE increases over time.

One solution to this problem is for the eNB to issue TA commands so that NTA compensates the error introduced in NTA, adjUE. This approach, however, has the drawback of not being usable for NPRACH, since NPRACH uses an NTA=0. Therefore, if the error introduced by NTA, adjUE is larger than the maximum correction capability of NPRACH, the UE would fail random access in connected mode (in case of, e.g., PDCCH order or SR). Therefore, NPRACH will be unusable in connected mode if no enhancements are introduced done.

Observation 6: Although the eNB can progressively correct (by issuing TA commands) the timing error due to a stale UE location, this correction is not applied when transmitting NPRACH (which currently uses NTA=0). This may cause the timing error to go beyond the NPRACH correction capability.

To mitigate this, several solutions can be considered, such as updating the NTA, adjUE term after each NPRACH transmission, so that it acts as a “proxy” for an updated GNSS fix, without needing to spend precious power in acquiring a GNSS fix every time. Other solutions may include—for NPRACH in connected mode for NTN—allowing “accumulation” in the NTA term, after every response (from the network) corresponding to a NPRACH transmission.

With these solutions, it is feasible to realize a “power-optimal” scenario, wherein, a UE only reads GNSS (ideally) once before a connection, and subsequently, with the above adjustments to the TA for NPRACH, receives “closed loop fixes” to keep its TA synchronized for uplink transmissions.

We also note that, the discussion above, in the context of TA, would also need to be extended to “(doppler) frequency correction”, for maintenance of uplink synchronization—which comprises both time and frequency pre-compensation.

In the light of the above discussion, we make the following proposals:

Proposal 4: RAN1 to specify solutions such that closed loop TA commands can also applied to NPRACH. Candidate solutions include:

- Updating the NTA, adjUE term with every TA command received in response to transmitting a NPRACH in connected mode

- Accumulating prior TA commands into the NTA term, as opposed to universally setting it to 0 for a NPRACH transmission

Proposal 5: Proposal 3—related to TA—shall also translate to an analogous solution for closed-loop residual (doppler) frequency correction.

  - Doppler frequency errors due to outdated GNSS

We consider a LEO satellite at an orbit of 1200 km, and depict below, the progression of the residual (uncorrected) Doppler frequency shift that may occur, as a function of time that has elapsed since the last GNSS fix. For this depiction, we assume that the starting position (the position at which the UE has a fresh GNSS fix) is at the nadir beam, where the satellite is directly above the UE (elevation angle 90 degree); thereafter, the satellite and the UE move in opposite directions with their respective velocities. The error (uncorrected Doppler shift) is then calculated by the difference between the “exact” Doppler shift (had the UE known the GNSS in real time) and the “estimated” Doppler shift—assuming the UE doesn’t update its GNSS location after the initial time point. Note here, we assume that we know the satellite’s ephemeris (e.g., PVT) “exactly” at all points for this depiction, which is optimistic.

We see above that, for high UE speeds, even within a timespan of 2 minutes, there can be an approximately 220 Hz frequency shift on account of an outdated GNSS, which would result in significant performance degradation for an NB-IoT UE with a subcarrier spacing of 3.75 kHz and may make it difficult for the UE to meet the requirements set by RAN4.

Observation 7: For mobile UEs, with a GNSS fix that is outdated by 2 minutes, there may be a residual doppler frequency error of 220 Hz, with LEO satellites at a 1200 km orbit operating in the S band.

- This error may be outside the synchronization requirements set by RAN4.

- This may cause significant interference among UEs in the uplink when a 3.75 kHz subcarrier spacing is used for NB-IoT.

- This may be mitigated by introducing a closed-loop frequency correction mechanism, as proposed in this contribution.

- Requiring fresh GNSS fixes within such short intervals (e.g., 2 minutes) severely degrades battery life of the UE.

# Conclusion

In this contribution we presented our views on enhancements to NB-IoT/eMTC over NTN as it relates to improved GNSS operations. We summarize our proposals below.

On power consumption due to GNSS fixes, and general framework for GNSS operation and reduce power consumption, we make the following observations and proposals:

Observation 1: Under the studied scenario of short, sporadic connections, a GNSS fix before every connection consumes approximately 34% of the UE’s total available energy.

Observation 2: Under the studied scenario of a long connection employing connected mode DRX (with a DRX cycle of ~10s), a GNSS fix before every uplink transmission consumes approximately 45% of the UE’s total available energy without additional enhancements w.r.t uplink synchronization.

- This is especially true for mobile UEs that cannot depend on a prior acquired GNSS fix

Proposal 1: For the objective on improved GNSS operations and reduced power consumption, the following framework is followed by RAN1:

- Specify closed loop time and frequency enhancements to increase the time between consecutive GNSS re-acquisitions.

- Specify aperiodic eNB triggers to instruct the UE to re-acquire GNSS (already agreed).

- Specify a mechanism for the UE to re-acquire GNSS upon expiry of the GNSS validity (already agreed).

Proposal 2: After a GNSS reacquisition, the UE reports its new GNSS validity duration to the eNB:

- The validity duration indicates the “remaining validity duration”.

- The reporting is performed regardless of the method of GNSS reacquisition (UE autonomous, eNB triggered, etc.)

On specifying, and facilitating closed-loop time and frequency corrections to minimize the number of GNSS fixes required, we make the following proposals and observations:

Observation 3: For long connections in eMTC and NB-IoT over NTN, closed-loop time and frequency corrections lowers the GNSS power penalty from 45% to 17% (with a GNSS relaxation factor of 4), w.r.t a baseline without closed-loop corrections.

Proposal 3: RAN1 to specify enhancements to closed-loop time and frequency corrections, to mitigate UE power consumption on account of GNSS fixes.

Observation 4:  According to current specifications, any time a UE transmits a NPRACH, it uses a value of NTA=0 in calculating the TA for the NPRACH transmission.

Observation 5: If a considerable amount of time has passed since the last GNSS position fix, e.g., for a mobile UE, the accuracy of NTA, adjUE becomes progressively worse over time, i.e., the error in NTA, adjUE increases over time.

Observation 6: Although the eNB can progressively correct (by issuing TA commands) the timing error due to a stale UE location, this correction is not applied when transmitting NPRACH (which currently uses NTA=0). This may cause the timing error to go beyond the NPRACH correction capability.

Proposal 4: RAN1 to specify solutions such that closed loop TA commands can also applied to NPRACH. Candidate solutions include:

- Updating the NTA, adjUE term with every TA command received in response to transmitting a NPRACH in connected mode

- Accumulating prior TA commands into the NTA term, as opposed to universally setting it to 0 for a NPRACH transmission

Proposal 5: Proposal 3—related to TA—shall also translate to an analogous solution for closed-loop residual (doppler) frequency correction.

Observation 7: For mobile UEs, with a GNSS fix that is outdated by 2 minutes, there may be a residual doppler frequency error of 220 Hz, with LEO satellites at a 1200 km orbit operating in the S band.

- This error may be outside the synchronization requirements set by RAN4.

- This may cause significant interference among UEs in the uplink when a 3.75 kHz subcarrier spacing is used for NB-IoT.

- This may be mitigated by introducing a closed-loop frequency correction mechanism, as proposed in this contribution.

- Requiring fresh GNSS fixes within such short intervals (e.g., 2 minutes) severely degrades battery life of the UE.

# References

[1] R1-2103070, “Scenarios applicable to NB-IoT/eMTC” by Qualcomm Incorporated, RAN1 104-bis-e.