3GPP TSG RAN WG1 #112			R1-2301484

Athens, Greece, February 27th – March 3rd, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on general aspects of AI/ML framework

Agenda Item:	9.2.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#94-e meeting, a new SID [1] on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” was approved. This SID captures the objective of SI in general aspects of AI/ML framework as following.

AI/ML model, terminology and description to identify common and specific characteristics for framework investigations:

- Characterize the defining stages of AI/ML related algorithms and associated complexity:

  - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable

  - Inference operation, e.g., input/output, pre-/post-process, as applicable

- Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g.,

  - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes]

  - Various levels of UE/gNB collaboration targeting at separate or joint ML operation.

- Characterize lifecycle management of AI/ML model: e.g.,  model training, model deployment , model inference, model monitoring, model updating

- Dataset(s) for training, validation, testing, and inference

- Identify common notation and terminology for AI/ML related functions, procedures and interfaces

- Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate

In this contribution, we discuss the general aspects of AI/ML framework.

- Discussion on general aspects of AI/ML framework

  - NW-UE collaboration level

At the RAN1#109-e and the RAN1#110bis-e meeting, the following agreements and the working assumption were made regarding NW-UE collaboration level [2] [3].

Agreement

Take the following network-UE collaboration levels as one aspect for defining collaboration levels

- Level x: No collaboration

- Level y: Signaling-based collaboration without model transfer

- Level z: Signaling-based collaboration with model transfer

Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings

FFS: Clarification is needed for Level x-y boundary

Agreement

Clarify Level x/y boundary as:

- Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE.
(Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.)

Working Assumption

- Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not.

- Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z

- Clarifying note: Level y includes cases without model delivery.

The signalling, e.g., model transfer, was agreed as one factor of collaboration levels, while other factors, such as one-sided model or two-sided model, are still not precluded for determining collaboration levels as shown in the above agreement [2]. In our view, it is difficult to order NW-UE collaboration levels if two factors are included in them. For example, it is difficult to compare the collaboration levels between two-sided model without model transfer and one-sided model with model transfer, i.e., which one is higher collaboration level. Hence, we prefer to classify NW-UE collaboration levels based on only one factor.

Proposal 1: Define NW-UE collaboration level based only on signalling perspective.

In addition, the boundary between each level was clarified at the RAN1#110bis-e meeting. If model delivery specified in 3GPP signalling is supported, NW-UE collaboration falls into level z, and the boundary between level x and y was agreed to be whether any dedicated AI/ML-specific enhancement collaboration is supported between NW and UE. In the last three meetings, the NW-UE collaboration level was sufficiently discussed, and the clear boundary was made. Therefore, the sub level of each NW-UE collaboration level is not necessarily further defined

Proposal 2: No need to define the sub-level of NW-UE collaboration level.

In NW-UE collaboration level z, the model transfer, in other words, the model delivery specified in 3GPP over the air interface, is expected. One important aspect in model transfer discussion is the format of model. At the RAN1#111 meeting, the concept of proprietary-format models and open-format models are defined as below [4].

Working Assumption

Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion,

| Proprietary-format models | ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format |
| --- | --- |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspective |

From RAN1 discussion viewpoint, RAN1 may assume that:

- Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared.

- Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared

In case of model transfer with the proprietary format models, model design is not mutually recognizable across vendors. On the other hand, model transfer with the open format models requires the disclosure of model design information. Supporting model transfer gives more flexibility of model management scenarios. However, the feasibility of model transfer is still questionable especially for open format models. One issue of model transfer with open format models is the hardware optimization. When AI model is deployed at UE, the hardware optimization on the device is necessary to work efficiently. In that case, the flexible determination of the model structure and model parameters could be difficult. Also, the model transfer is supposed to deliver model information from one entity to the other entity. If model information, such as model parameter / model structure / UE hardware information to determine applicable model structure or parameters on device, is considered as a proprietary asset, the information cannot be disclosed over the 3GPP air interface. Hence, some companies are negative on the further study of model transfer in Rel-18.

As the assumption on life cycle management (LCM) discussion largely depends on the support of model transfer, it is difficult to proceed the LCM discussion without concluding the further study of model transfer. Indeed, the progress of LCM discussion in RAN 1 is slow due to this reason. To get out of this deadlock situation, we think it is better for RAN1 to focus on LCM framework without open format model transfer first. Currently, RAN2 is working on the study of model transfer signalling mechanism. After the sufficient RAN2 study, RAN1 can come back to the discussion of model transfer feasibility including open format.

Proposal 3: Deprioritize model transfer with open format models in LCM discussion until the feasibility is confirmed.

  - Functionality and model ID

At the RAN1#111 meeting [4], it was agreed to study two LCM procedures: functionality-based LCM and model ID-based LCM.

Agreement

For UE-part/UE-side models, study the following mechanisms for LCM procedures:

- For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality

  - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality.

  - FFS: Whether or how to indicate Functionality

- For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs

Even though functionality-based LCM procedure and model-ID-based LCM were decided to study, the exact definition of functionality and model-ID have not been defined yet. In functionality-based LCM procedure, UE is expected to receive the indication regarding which functionality should be activated/deactivated/monitored. As there may be multiple models for one functionality, UE could control which exact model to be activated/deactivated/monitored within one functionality.

In some sub use cases, e.g., CSI compression or beam prediction at UE side, the information UE can report depends on which model is active or inactive. For example, the temporal beam prediction at UE side outputs beam quality at certain predicted time instance(s), where the predicted time instances could be different according to the active model. Likewise, the compression ratio is supposed to be different according to the active model in CSI compression. The active model determines what information can be obtained from model inference. As the desired information from model inference depends on the NW operation, NW should determine which model to be active according to what information is useful for the NW operation. Hence, at least the functionality should be different if the model output type is different so that NW could control what information can be obtained from model inference.

Proposal 4: Different functionalities should be assigned if the output type of models is different.

According to the model structure or dataset used for training, models with the same output type could provide different performance. Some models might work better in certain configuration/scenario, while other models might work in different configuration/scenario. To maximize the AI/ML gain, it is important to determine the appropriate model among the same output type models according to suitable configuration/scenario. When some configuration/scenario, e.g. surrounding environment or NW configuration, can be known only by NW not UE, NW should be able to determine which model to be activated within the same functionality. In our view, model ID can be used for that purpose. Fig. 1 illustrates the examples of functionality-based LCM and model-ID-based LCM. When functionality-based LCM is applied, UE needs to manage which model to be used among all models whose output type is the same. On the other hand, different model ID can be assigned within one functionality for model ID-based LCM. In the example of Fig. 1, different model IDs are assigned according to suitable configuration and scenario for each model or model set within functionality#1 and funcaitionality#2, respectively.

Proposal 5: Different Model ID can be assigned, even when the output type of models is the same.

Observation 1: Model-ID-based LCM can make it easy to manage scenario/configuration specific models especially when some scenario/configuration can be detected only by NW.

Figure 1. Example of functionality-based LCM and model ID-based LCM.

It is also notable that multiple models may be assigned to one model ID. If the management of certain models can be handled better by UE rather than NW, UE should be able to control it. Thus, model ID-based LCM could enjoy this flexibility by the proper model ID assignment.

Observation 2: When one model ID is assigned to multiple models, UE can handle the model management within model ID.

Considering these aspects, model ID-based model management is more flexible than functionality-based model management.  Furthermore, functionality-based model management can be viewed as one specific case of model ID-based model management, where one model ID is assigned per functionality. From the forward compatibility perspective, model-ID based model management should be assumed in the life cycle management (LCM) discussion.

Proposal 6: Prioritize model ID-based model management than functionality-based model management from the forward compatibility perspective.

  - LCM framework

Agreement

Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management

- Data collection

  - Note: This also includes associated assistance information, if applicable.

- Model training

- [Model registration]

- Model deployment

  - Note: Terminology is to be defined.

- [Model configuration]

- Model inference operation

- Model selection, activation, deactivation, switching, and fallback operation

- Model monitoring

- Model update

  - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training.

- Model transfer

- UE capability

Note: Some aspects in the list may not have specification impact.

Note: Aspects with square brackets are tentative and pending terminology definition.

Note: More aspects may be added as study progresses.

At the RAN1#110 meeting, it was agreed to study stages of LCM including the necessity discussion, as the above agreement [5]. The model selection was captured in this agreement as one of studying item without the clear definition. As can be seen in the above agreement, the model selection often shows up in discussion equivalently to model activation/deactivation/switching/fallback operation. Hence, it is reasonable to assume the model selection is the similar concept of these model operations. Considering it, the model selection should be defined as follows.

| Model selection | Activate one AI/ML model among multiple models within the same functionality. |
| --- | --- |

The definition of model deployment is also under discussion. Most companies assume that model deployment includes two aspects: model delivery and the process of converting a trained AI/ML model into an executable form for inference at a target device, which is often referred as model conversion. The controversial point is which process should be firstly conducted, model delivery or model conversion. Since terminologies are introduced just to facilitate the discussion, it is not reasonable to narrow down to one process order in the terminology discussion especially when it is not concluding. Hence, we prefer to define model conversion and model delivery as follows with FFS regarding the process order. After defining these terminologies, RAN1 could narrow down the order if necessary.

| Model conversion | Process of converting a trained AI/ML model into an executable form for inference at a target device. |
| --- | --- |
| Model delivery | Process of both model conversion and model delivery, where model conversion may or may not be performed before model delivery. |

Proposal 7: Define model selection, model conversion, and model delivery as follows.

| Model selection | Select one AI/ML model for activation among multiple registered models for the same functionality. |
| --- | --- |
| Model conversion | Process of converting a trained AI/ML model into an executable form for inference at a target device. |
| Model delivery | Process of both model conversion and model delivery, where model conversion may or may not be performed before model delivery. |

LCM framework is beneficial for studying each stage in detail, since it provides the common big picture of LCM among companies. Also, LCM framework is useful to discern the necessity of some LCM stages.

Working Assumption

| Terminology | Description |
| --- | --- |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |

Note: It is encouraged for the 3gpp discussion to proceed without waiting for online/offline training terminologies.

At the RAN1#110 meeting [5], online training is defined as the training process based on (near) real-time data, while in case of offline training the model is not updated based on the (near) real-time data. When the online training is considered, the model can be updated in LCM time scale. Thus, LCM framework could be different according to whether online training is supported or not.

Proposal 8: Consider the LCM framework with and without online training, separately.

Fig.2 illustrates the proposed LCM frameworks without online training and with online training, respectively. In the proposed LCM framework, the model identification is placed once the model gets trained. The detail of model identification is discussed in the section 2.6. After UE and NW have the common understanding on the available models, model selection, activation, deactivation, switching, and monitoring can also be managed by NW for the UE side models.

In the proposed LCM figures, the data collection is not included, because the data collection can be performed in parallel at any stage of the LCM framework.

Figure 2. Proposed LCM framework (a) LCM framework without online training and (b) LCM framework with online training.

As shown in Fig.2, LCM framework without online training is simpler.  Also, online training requires more implementation difficulty than offline training. Therefore, LCM without online training is more suitable as the first step of LCM discussion in Rel-18 AI/ML.

Observation 3: LCM without online training is more suitable as the first step of LCM discussion in Rel-18 AI/ML, considering the feasibility and simplicity.

  - AI/ML operations over different scenarios/configurations

At the RAN1#110bis-e meeting [3], it was agreed to study the three approaches achieving high performance over different scenarios/configurations: model generalization, model switching, and model update.

Agreement

Study various approaches for achieving good performance across different scenarios/configurations/sites, including

    - Model generalization, i.e., using one model that is generalizable to different scenarios/configurations/sites

    - Model switching, i.e., switching among a group of models where each model is for a particular scenario/configuration/site

      - [Models in a group of models may have varying model structures, share a common model structure, or partially share a common sub-structure. Models in a group of models may have different input/output format and/or different pre-/post-processing.]

    - Model update, i.e., using one model whose parameters are flexibly updated as the scenario/configuration/site that the device experiences changes over time. Fine-tuning is one example.

Model update is one potential solution to the various scenarios/configurations. This approach is enabled by updating model parameters according to scenarios or configurations, which requires model parameters to be flexibly updated as often as the scenario and the configuration are changed. However, some companies have concerns of the feasibility of the model update at the short time scale, such as online training, because model testing followed by model deployment could be needed after model update. Thus, the feasibility of model update at the short time scale is questionable.

Observation 4: The feasibility of model update at the short time scale is questionable.

Model generalization is another approach for models to work for different scenarios/configurations. If the model generalizable to all scenarios/configurations can be trained, any LCM operation may not be necessary after the model deployment. However, the several generalization evaluations in each sub use case shows the performance degradation when the datasets for model training and model inference are different. For example, different antenna/beam configurations are difficult for one unified model to be generalizable to. Hence, it is ideally better to train the model with dataset collected in the same scenarios/configurations as model inference. This could be achieved by model switching among multiple models per functionality. If multiple models are available to the inference entity where each model is specific to one scenario/configuration, high performance can be maintained by switching the model according to scenario/configuration. The drawback of this approach is the storage issue. However, the number of necessary models can be reduced by combining the model generalization techniques. Therefore, the combined solution of model switching and model generalization is a promising technique to adapt different scenarios/configurations at the short time scale.

Observation 5: Multiple available models are required for the model switching.

Observation 6: The combined solution of model switching and model generalization is a promising technique to adapt different scenarios/configurations at the short time scale.

To manage the configuration specific model operation, either UE or NW needs to know the configuration of the other entity or configuration applicable to the model. In our view, there are two approaches to enable it via assistance signalling and model identification. The detail procedure is described in the subsequent sections.

  - Data collection

Agreement

Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models.

Note: User data privacy needs to be preserved. The provision of assistance information may need to consider feasibility of disclosing proprietary information to the other side.

At the RAN1#110bis-e meeting [3], the above agreement related to the development of a set of scenario/configuration-specific models was made. The scenario/configuration-specific model can be trained with the data collected from the specific scenarios or/and configurations. To facilitate the data collection specific to scenarios or/and configuration, some enhancements of assistance signalling should be studied because such signalling indicating a set of configurations or scenarios has not been defined yet in current 5G NR framework.

As the note in the above agreement suggests, the proprietary/privacy aspects should be considered in the assistance signalling. For instance, beam/antenna configuration could be viewed as proprietary information. Such information needs to be transmitted in the form that the proprietary/privacy information would not be disclosed. In our view, one solution could be ID-base signaling. If NW reports beam/antenna configuration ID instead of the beam/antenna configuration itself, UE can identify whether it is the same configuration or not even without the detail information on beam/antenna configuration at gNB, and vice versa. In this manner, NW and UE can categorize the collected dataset, and train the model specific to certain scenario/configuration. In addition, the assistance signaling makes it possible for NW and UE to discern whether the trained model is applicable to the other entity’s configuration after model training.

Proposal 9: Assistance signalling including scenario/configuration ID should be assumed for dataset collection, model training, and model inference of scenario/configuration-specific AI models.

Fig. 3 shows each stage description of NW side models with UE configuration ID as assistance information, where the model is trained at NW side. If the UE configuration ID is reported to NW, NW can categorize the collected dataset according to UE configuration ID or/and scenarios around gNB. Then, it is possible to train NW side models specific to UE configurations and scenarios. In addition, NW can discern whether the trained model is appliable to the configuration of certain UE during the model inference, if the UE reports its configuration ID as assistance information. Thus, NW can activate NW side models only for applicable UE based on assistance information without disclosing the proprietary information.

Figure 3. Each stage of NW side models with UE configuration as assistance information, where the model is trained at NW side.

Fig. 4 illustrates each stage of UE side models with NW scenario/configuration ID as assistance information, where the model is trained at UE side. During the data collection, NW scenario/configuration ID helps categorizing collected data into the corresponding NW configuration and scenario. Based on the categorized dataset, UE side model specific to certain NW configuration or/and scenario can be trained. In the model identification, UE can report the model only applicable to the NW configuration and scenario. In this approach, the activation of the models inapplicable to the NW configurations or the scenarios can be avoided. Another approach is to report the applicable NW scenario/configuration ID associated with the models in the model identification. In this case, NW can activate or deactivate UE side models, considering the current NW configuration or scenario. These approaches to manage scenario/configuration specific models are illustrated in Fig. 5.

Figure 4. Each stage of UE side models with NW scenario/configuration ID as assistance information, where the model is trained at UE side.

Figure 5.  Two approaches to discern models applicable to the configuration/scenario: via UE check and via NW check.

Observation 7: It is possible for UE and NW to discern whether the model is applicable to the configuration of other entity by ID-based indication without disclosing proprietary information.

For UE side model training, the dataset can be collected by dataset delivery or UE measurements. Dataset delivery is studied in each AI/ML use case. However, the enhancement of UE measurements for the data collection is not sufficiently studied.

In the current 5G NR framework, DL RS is configured/indicated by NW. When UE side performs model training, the necessary UE measurements for model management are up to UE side. According to the functionality or model, desired DL RS configuration for UE measurement could be different. Hence, it is preferable for UE to be able to request specific DL RS transmission for UE side data collection. Fig. 6 shows the flow of dataset request. First, UE requests what DL RS is preferred for the data collection. Then, NW determines to accept or reject the DL RS request based on NW operation. If the request is acknowledged, UE can measure the desired DL RS for data collection.

Figure 6.  Study DL RS request for UE side data collection.

Proposal 10: Study DL RS request for UE side data collection.

  - Model identification

For the management of UE side model by NW, NW needs to know what model/functionality is available at UE side model. Also, the essential model description information associated with available models, i.e., required measurements and assistance information for the model inference input, is necessary for the proper decision of model activation/deactivation/fallback approach. For those purposes, model identification and functionality identification can be used.

Working Assumption

| Terminology | Description |
| --- | --- |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |

| Terminology | Description |
| --- | --- |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |

Note: whether and how to indicate Functionality will be discussed separately.

Since there is a concern about disclosing the proprietary model information, the model information should be informed to UE without exposing the proprietary information. However, the following information should be at least informed to NW for the management of UE side models by NW for both functionality identification and model identification.

- Model functionality: nominal information of post-processed model output

- Necessary measurements/assistance information to calculate model inputs

In addition, the following information could be shared between NW and UE in case of model identification.

- Model ID

- Applicable NW configuration/scenario identifier

Proposal 11: Study what information should be exchanged between UE and NW in model/functionality identification for model/functionality management. At least the following information should be shared.

- Model functionality: nominal information of post-processed model output

- Necessary measurements/assistance information to calculate model inputs

- Model ID (only for model identification)

- Applicable NW configuration/scenario identifier (only for model identification)

In case of NW side models, NW can manage models by model activation/deactivation/fallback approach by NW implementation. Also, the model monitoring of NW side model can be performed without UE awareness of NW side models. What UE is required to do for the NW side model monitoring would be the UE measurements and the corresponding UE reporting for model monitoring. These UE behaviors can be performed without model awareness. Thus, the UE does not need to be aware of NW side models.

Proposal 12: UE does not need to be aware of NW side models.

After UE and NW share the same understanding on model/functionality via model/functionality identification, the model/functionality may become unavailable due to the power consumption and computational resources associated with the model. As the availability could be changed based on the UE status, some frameworks to inform availability to NW might be necessary. In our view, there are two approaches to inform availability of functionality/model, as shown in Fig.7. First approach is reporting only available models in model identification, where identified functionality/model is the same as available functionality/model. In this manner, UE needs to trigger the model identification, whenever the availability is changed. The other approach is to report the availability of identified models. Both approaches are the same in a sense that UE reports availability when the available functionality/model is changed. The only difference is just whether model identification includes the availability notification or not.

Figure 7. Two approaches to inform availability.

Proposal 13: Study how to inform the availability of UE side model/functionality.

  - Model inference

There is a discussion about how to treat AI model/algorithm and data used for it. Nowadays, the collected data is regarded as an asset of companies due to its utility of AI operations for many purposes. Also, a lot of resources and money are devoted for developing the AI model/algorithm to provide better services. As a result, some companies prefer to treat collected data and AI model/algorithm as the proprietary asset of companies. Along this idea, the structure and mechanism of some AI models/algorithms should not be visible to other companies, e.g., even NW vendors or operators. However, this idea would make it difficult to operate NW with certain reliability. For example, even if AI could provide statistically higher performance than conventional schemes, AI-based operation might not work well in some scenarios with outlier data. If these unexpected results cannot be coped with, it is difficult to deploy AI for 5G NR in the practical system. Hence, when AI is deployed, it is better to have some fallback operations to guarantee the performance corresponding to each sub use case.

Proposal 14: Study the mechanism to apply fallback operations of each sub use case.

Also, the model monitoring should be supported to help the proper model operation decisions for the reliable model managements. At the RAN1#110bis-e meeting [3], the following agreement related to the model monitoring was made.

Agreement

Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training).

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

As shown in the above agreement, the model monitoring could be useful for the several types of model operation decisions: model activation, deactivation, switching, update and fallback operations. Since these model operations are different, the model monitoring should be optimized according to each model operation. Table 1 summarizes the expected time scale and the desired performance metric in each model operations. In our view, the model activation, deactivation, selection, switching and fallback operations (model operation A) are useful to quickly recover from the model failure. For example, if the instantaneous performance degradation due to the active model is detected in the model monitoring, the appropriate model operation A could immediately alleviate the performance. Hence, the performance metric reflecting the instantaneous performance degradation is useful for the model operation A. On the other hand, the model update (model operation B) should be less frequently than model operation A in the actual deployment, because the model update requires the more complex processing compared to just activation or deactivation of models. Especially, the expected time scale of model update becomes long when online training is not supported. Given that the model is updated less frequently, the decision of model update should be based on performance metrics less affected by the instantaneous performance degradation. Thus, the appropriate performance metrics is different according to the expected time scale of model operations. Considering it, the various performance metrics in model monitoring should be supported. Especially, (near) real time-scale performance metric should be provided to NW for reliable model activation/deactivation/switching/fallback in case of NW-controlled models.

Proposal 15: In model monitoring, (near) real time-scale performance metrics should be provided to NW for reliable model activation/deactivation/switching/fallback in case of NW-controlled models

Table 1. Model monitoring for several types of model operation decisions.

|  | Model operation A (Model activation, deactivation, selection, switching, and fallback operations) | Model operation B (Model update) |
| --- | --- | --- |
| Expected time scale of model operations | Short time scale to reduce the time from model failure to action | Long time scale due to complexity of model updating |
| Desired performance metric | Performance metric reflecting the instantaneous performance degradation (e.g., monitoring based on inference accuracy) | Performance metric less affected by instantaneous performance degradation (e.g., monitoring based on data distribution, system performance) |

Model switching or fallback operation could be used to handle the performance degradation due to the active model. For the proper decision of these model operations, the model monitoring of inactive models or/and fallback operation in addition to the active model should be supported. If the (near) real time performance of an active model, inactive models and the corresponding fallback operation is available, the model switching and the fallback operation can be performed properly by comparing each performance.

Proposal 16: Study the model monitoring mechanism for inactive models and fallback operation in addition to active models.

  - General views on KPI

Some KPIs in evaluation methodology are common among all sub use cases. At the RAN1#110bis-e meeting [3], the following agreement related to common KPIs was made.

Agreement

The following are additionally considered for the initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML

- Clarification on inference complexity

  - Note: Inference complexity includes complexity for pre- and post-processing.

- LCM related complexity and storage overhead

  - Storage/computation for training data collection.

  - Storage/computation for training and model update

  - Storage/computation for model monitoring.

  - Storage/computation for other LCM procedures, e.g., model activation, deactivation, selection, switching, fallback operation.

- FFS: Power consumption, latency (e.g., Inference latency)

Power consumption is one of common KPIs, and it is still controversial if it should be evaluated in each sub use case evaluation. From the commercial aspects, it is useful to evaluate power consumption. Also, the various approaches were proposed to evaluate the power consumption. However, it is difficult to reach consensus on the unified power consumption model, since the assumed implementation are different across companies. Hence, we prefer not to spend a time to make the unified power consumption model, while companies should be able to voluntarily provide their models estimating power consumption based on FLOPs with their expected implementations.

Proposal 17: Companies can voluntarily provide their models estimating power consumption model based on FLOPs with their expected implementations.

- Conclusion

In this contribution, we discussed the general aspects of AI/ML framework. Based on the discussion we made the following observations and proposals.

Observation 1: Model-ID-based LCM can make it easy to manage scenario/configuration specific models especially when some scenario/configuration can be detected only by NW.

Observation 2: When one model ID is assigned to multiple models, UE can handle the model management within model ID.

Observation 3: LCM without online training is more suitable as the first step of LCM discussion in Rel-18 AI/ML, considering the feasibility and simplicity.

Observation 4: The feasibility of model update at the short time scale is questionable.

Observation 5: Multiple available models are required for the model switching.

Observation 6: The combined solution of model switching and model generalization is a promising technique to adapt different scenarios/configurations at the short time scale.

Observation 7: It is possible for UE and NW to discern whether the model is applicable to the configuration of other entity by ID-based indication without disclosing proprietary information.

Proposal 1: Define NW-UE collaboration level based only on signalling perspective.

Proposal 2: No need to define the sub-level of NW-UE collaboration level.

Proposal 3: Deprioritize model transfer with open format models in LCM discussion until the feasibility is confirmed.

Proposal 4: Different functionalities should be assigned if the output type of models is different.

Proposal 5: Different Model ID can be assigned, even when the output type of models is the same.

Proposal 6: Prioritize model ID-based model management than functionality-based model management from the forward compatibility perspective.

Proposal 7: Define model selection, model conversion, and model delivery as follows.

| Model selection | Select one AI/ML model for activation among multiple registered models for the same functionality. |
| --- | --- |
| Model conversion | Process of converting a trained AI/ML model into an executable form for inference at a target device. |
| Model delivery | Process of both model conversion and model delivery, where model conversion may or may not be performed before model delivery. |

Proposal 8: Consider the LCM framework with and without online training, separately.

Proposal 9: Assistance signalling including scenario/configuration ID should be assumed for dataset collection, model training, and model inference of scenario/configuration-specific AI models.

Proposal 10: Study DL RS request for UE side data collection.

Proposal 11: Study what information should be exchanged between UE and NW in model/functionality identification for model/functionality management. At least the following information should be shared.

- Model functionality: nominal information of post-processed model output

- Necessary measurements/assistance information to calculate model inputs

- Model ID (only for model identification)

- Applicable NW configuration/scenario identifier (only for model identification)

Proposal 12: UE does not need to be aware of NW side models.

Proposal 13: Study how to inform the availability of UE side model/functionality.

Proposal 14: Study the mechanism to apply fallback operations of each sub use case.

Proposal 15: In model monitoring, (near) real time-scale performance metrics should be provided to NW for reliable model activation/deactivation/switching/fallback in case of NW-controlled models

Proposal 16: Study the model monitoring mechanism for inactive models and fallback operation in addition to active models.

Proposal 17: Companies can voluntarily provide their models estimating power consumption model based on FLOPs with their expected implementations.

References

- Moderator (Qualcomm), RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Dec. 2021.

- Moderator (Qualcomm), R1-2203280, “Summary of General aspects of AI/ML framework”, May. 2022.

- Moderator (Qualcomm), R1-2210661, “Summary#5 of General Aspects of AI/ML Framework”, Oct. 2022

- Moderator (Qualcomm), R1-2213003, “Summary#1 of General Aspects of AI/ML Framework”, Nov. 2022

- Moderator (Qualcomm), R1-2208178, “Summary#4 of General Aspects of AI/ML Framework”, Aug. 2022