3GPP TSG RAN WG1 #112		R1-2301513

Athens, Greece, February 27th – March 3rd, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on Network verified UE location for NR NTN

Agenda Item:	9.11.2

Document for: 	Discussion/Decision

- Introduction

The Rel-18 NR NTN WID has been updated as follows regarding network verified UE location [1].

- Network verified UE location Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied. Note 1: Enhancements assume reuse of the RAT dependent positioning framework Note 2: The specification of DL-TDOA enhancements will be subject to the study of the impact of realistic UE clock drift onto DL-TDOA performance Note 3: The target accuracy for position verification purposes is as documented in clause « recommendations » of the 3GPP TR 38.882 (i.e. 10 km granularity) Note 4: Multiple satellite in view by the UE may be considered if time allows Note 5: The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved Note 6: The enhancements should take into account the mirror-image ambiguity Note 7: Network verified UE location is an optional UE feature

The motivation of network verified UE location is that relying only on the GNSS based location information reported by the UE is not considered reliable as mentioned by SA3-LI. The UE reported location information (e.g. GNSS information), could be erroneous due to intentional (e.g. maliciously tampering by user or by 3rd party) or unintentional (e.g. interference) causes, hence it cannot be considered trusted by network operators. The target services include services which subject to national regulations or other operational constraints, e.g. Public Warning System (PWS), Lawful interception (LI), Emergency services (EMS), Charging and Tariff notifications. The UE location information is considered verified if the reported UE location is consistent with the network-based assessment to be within 5-10 km (similar to terrestrial network macro cell size). Such verification should enable country identification, selection of an appropriate core network, and the support of all the regulatory service.

In this contribution, our views on solutions for network to verify UE reported GNSS location information are provided.

- Discussion

As copied above from the WID, enhancements for multi-RTT positioning method to support network verify UE location in NTN assuming a single satellite in view is mainly considered while this document includes discussion for multi-satellite case.

  - UE location verification based on multi-RTT positioning methods

    - Information report

In our view, the verification for UE location in NTN is performed at LMF, and the same signaling as in TN NR positioning can be applied for the transfer of measurement results and assistance information. However, different from TN, in NTN, the reference signals (e.g., PRS, SRS) are transmitted between gNB and UE with relaying by satellite, hence the measurements (e.g., Rx-Tx time difference) indicate the propagation delay between UE and gNB with relaying by satellite. An example of applying multi-RTT method in NTN is illustrated in Figure 1.

Figure 1 Example of multi-RTT method in NTN

For multi-RTT based method with single satellite, multiple measurements will be performed between UE and one TRP/gNB via a single satellite at different timing. Each measurement timing is required to calculate UE location. Meanwhile it is noted that the measured quantity at UE can be the same as in legacy multi-RTT framework, i.e., time difference between received timing of DL subframe from TRP and UE transmission timing of UL subframe at UE.

In addition, for each measurement, as shown in Figure 1, if the conventional RTT definition for positioning is reused, the reported UE/gNB Rx-Tx time difference to LMF (i.e., ‘gNB Rx-Tx time difference’ – ‘UE Rx-Tx time difference’) corresponds to the sum of RTT between gNB and satellite and RTT between satellite and UE. However, the required measurement result at LMF to estimate UE location is RTT between satellite and UE, i.e., (‘gNB Rx-Tx time difference’ – ‘UE Rx-Tx time difference’) – ‘RTT between gNB and satellite’, which is different from TN positioning.

Hence, from these aspects, additional information needs to be reported to LMF. At least the satellite ephemeris should be additionally reported to LMF by gNB. By using this information, LMF can obtain satellite positions at multiple measurement timing and ‘RTT between gNB and satellite’ abovementioned. Considering satellite movement, report of additional timing information may be needed.

Proposal 1:

For network verification of UE location in NR NTN based on multi-RTT framework, report additional information to LMF, including at least satellite ephemeris.

    - Impact of satellite movement in a single measurement

Considering the high moving speed of satellite in LEO/MEO, the location of satellite varies a lot during a single measurement. The impact of movement of the satellite needs to be studied and identified for the verification of UE location. An example of multi-RTT method considering the varying positions of satellite in a single measurement is illustrated in Figure 2.

It can be observed that, at different time, the location of satellite changes a lot. The inaccurate location of the satellite will lead to inaccurate UE location estimation. The required Rx-Tx time difference should be the RTT/ propagation delay between UE and satellite at a specific time point, and this time point should be carefully selected. For example, the time when LMF calculates the RTT based on received measurements (e.g., t3 in Figure2) shouldn’t be the specific time point, for location of satellite already changes a lot during the period of RS transmission and measurement (e.g., from t0~t3). In legacy multi-RTT framework, UE and gNB report time stamp to LMF, which represent the time of measuring UE Rx-Tx time difference and gNB Rx-Tx time difference respectively. The satellite location at the time stamp of each measurement (e.g., t2/t2’ in Figure2) can be approximately regarded as the satellite location for calculation. To further reduce the inaccuracy of verification due to satellite movement, the satellite location at the time of satellite transmitting the reference signal (e.g., t1/t1’ in Figure 2) should be applied for UE location verification. Thus, to address the impact of satellite movement, legacy timing information report should be enhanced by reporting additional timing information to LMF, e.g., the timing information of satellite transmitting the reference signal.

Figure 2 Example of muli-RTT method: satellite position at difference times in a single measurement

Proposal 2:

For network verification of UE location in NR NTN based on time-based positioning methods, study and identify the impact on the movement of satellite in a single measurement.

- Legacy timing information report is enhanced by reporting additional timing information to LMF, e.g., the timing information of satellite transmitting the reference signal.

  - Consideration of the mirror image ambiguity

According to simulations and previous discussions, the time-based positioning methods, e.g., multi-RTT method cannot distinguish between the mirror positions on either side of the orbital plane. Following agreements were made in RAN#98 meeting on the issue of mirror-image ambiguity.

Agreement Note 6: The enhancements should take into account the mirror-image ambiguity

To address the mirror image ambiguity, some additional information, it was argued that such as low-resolution UL AoA information can be reported to LMF to assist verification based on multi-RTT method. However, we believe that the procedure to achieve UL AoA measurement with current NTN assumption is still not clear, e.g., whether/how satellite measures UL AoA, how the measurements are transferred to gNB, or whether/how gNB measures UL AoA. The measurement of UL AoA via satellite (at least the feasibility) should be clarified before applying it to address the issue of mirror image ambiguity.

Proposal 3:

The procedure of acquiring UL AoA information via satellite (at least the feasibility) should be firstly clarified.

- If clarified, low resolution UL AoA information can be reported to LMF to resolve the mirror image ambiguity.

  - Reliability of UE reported TA

Following agreements was made in RAN#98 meeting :

Agreement Note 5: The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved

The TA report has been considered to be combined with multi-RTT approach in previous discussion, which may save signaling overhead as it does not require the report of information related to UE/gNB Rx-Tx time difference. However, companies’ views are quite diverse on whether UE-specific TA reported by UE can be trusted. We think an LS should be sent to SA3 to conclude on this issue, and avoid unnecessary discussion.

Proposal 4:

Send LS to SA3 to conclude whether TA report can be trusted.

  - Study of multiple satellite case

It can be observed from companies’ evaluation results that the performance of verification via single satellite is much worse than that of multiple satellites case if the time gap between two measurements is small and it may not always satisfy the requirement on verification of 5-10km. Meanwhile, to provide accurate positioning, the time duration of the verification procedure via single satellite is also relative long, e.g., tens of or over 100 seconds. According to SA2 reply, the expected latency of verification is approximately 1 minute maximum and 30 seconds preferably.

Thus, verification via multiple satellites becomes important for some cases where e.g., single satellite cannot meet the accuracy or latency requirement for verification, less time duration of the verification is required (e.g., some emergency services), etc. For these cases, UE location verification with multiple satellites should be considered.

Observation:

Single satellite may not meet the accuracy requirement for verification, or not satisfy less time duration of the verification. In such cases, verification via multiple satellite is beneficial.

- Conclusion

Proposal 1:

For network verification of UE location in NR NTN based on multi-RTT framework, report additional information to LMF, including at least satellite ephemeris.

Proposal 2:

For network verification of UE location in NR NTN based on time-based positioning methods, study and identify the impact on the movement of satellite in a single measurement.

- Legacy timing information report is enhanced by reporting additional timing information to LMF, e.g., the timing information of satellite transmitting the reference signal.

Proposal 3:

The procedure of acquiring UL AoA information via satellite should be firstly clarified.

- If clarified, low resolution UL AoA information can be reported to LMF to resolve the mirror image ambiguity.

Proposal 4:

Send LS to SA3 to conclude whether TA report can be trusted.

Observation:

Single satellite may not meet the accuracy requirement for verification, or not satisfy less time duration of the verification. In such cases, verification via multiple satellite can be beneficial.

- References

[1] RP- 223534 , “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements”, Thales, Dec. 2022.

[2] R1-2210952 , FL Summary #4: Network verified UE location for NR NTN, Thales, Nov. 2022