3GPP TSG RAN WG1 #112               	          	      		                        R1-2301528

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:   	5

Source:   	LG Electronics

Title: 	Discussion on LS on PSFCH configured power with multiple resource pools

Document for:   	Discussion and decision

- Introduction

In this contribution, we will provide our view on the questions related to the parameter of PEMAX,c for the simultaneous PSFCH transmissions on the multiple SL pools in RAN4 LS [1].

- Discussion

The following two questions are asked to RAN1 from RAN4 on LS [1].

- Question 1: RAN4 agreed to define PEMAX,c for PSFCH configured Tx Power in TS38.101-1 as sum of IEs sl-maxTransPower when multiple resource pools configured are transmitted simultaneously and would like to know if this can have any impact to RAN1.

- Question 2: RAN4’s understanding is that the total transmit power is shared equally between simultaneously transmitted PSFCHs regardless of whether they are from a single or multiple resource pools. RAN4 would like to confirm with RAN1 that this understanding is correct.

Regarding the questions above, our understanding is that when the PSFCH transmissions are simultaneously performed on the multiple SL pools, RAN4 defines the PEMAX,c for the configured Tx power as the sum of sl-maxTransPower values set for the corresponding SL pools. On the other hand, according to the part marked in yellow below in Section 16.2.3 of TS 38.213, the power calculations of PSFCH transmissions are performed per SL pool. Note that the OLPC parameters for the PSFCH transmissions are also configured per SL pool. When applying the PEMAX,c defined by RAN4 to the RAN1 specification for the simultaneous PSFCH transmissions on the multiple SL pools, we found the following aspects that require further discussion/clarification.

- If the PEMAX,c of RAN4 is used when selecting the number of transmitted PSFCHs and calculating their power values for each SL pool according to the RAN1 specification, the sum of calculated power values of finally selected PSFCHs on the multiple SL pools could exceed the maximum power of the UE.

- According to the RAN1 specification, the UE capability of the maximum number of PSFCHs that can be simultaneously transmitted is considered individually for each SL pool, and the number of transmitted PSFCHs in each SL pool is determined based on this restriction. When following this principle, the total number of finally selected PSFCHs on the multiple SL pools could exceed this capability of the UE.

- According to the current specification, the OLPC parameters for the PSFCH transmissions are configured independently for each SL pool, and in this case, only PSFCHs transmitted in the same SL pool are guaranteed to have the same power value.

16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions for HARQ-ACK information and conflict information, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i on active SL BWP b of carrier f as - if dl-P0-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PO,PSFCH is a value of dl-P0-PSFCH-r17, if using the parameter is supported by the UE and the parameter is provided; else dl-P0-PSFCH-r16 if provided - αPSFCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCH=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - if Nsch,Tx,PSFCH≤Nmax,PSFCH - if PPSFCH,one+10log10Nsch,Tx,PSFCH≤PCMAX, where PCMAX is determined for Nsch,Tx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - UE autonomously determines NTx,PSFCH PSFCH transmissions first with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, for 1≤i≤ 8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, for i>8, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any - zero, otherwise and PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] where PCMAX is defined in [8-1, TS 38.101-1] and is determined for the NTx,PSFCH PSFCH transmissions - else - the UE autonomously selects Nmax,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 - if PPSFCH,one+10log10Nmax,PSFCH≤PCMAX, where PCMAX is determined for the Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nmax,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - the UE autonomously selects NTx,PSFCH PSFCH transmissions in ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, 1≤i≤8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, i>8, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any - zero, otherwise and PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] where PCMAX is determined for the NTx,PSFCH simultaneous PSFCH transmissions according to [8-1, TS 38.101-1] - else PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH) [dBm] where the UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥1 and where PCMAX is determined for the NTx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1].

Observation: When the PEMAX,c defined by RAN4 for the simultaneous PSFCH transmissions on the multiple SL pools applies to the RAN1 specification, further discussion/clarification are required on at least the following aspects:

- If the PEMAX,c of RAN4 is used when selecting the number of transmitted PSFCHs and calculating their power values for each SL pool according to the RAN1 specification, the sum of calculated power values of finally selected PSFCHs on the multiple SL pools could exceed the maximum power of the UE.

- According to the RAN1 specification, the UE capability of the maximum number of PSFCHs that can be simultaneously transmitted is considered individually for each SL pool, and the number of transmitted PSFCHs in each SL pool is determined based on this restriction. When following this principle, the total number of finally selected PSFCHs on the multiple SL pools could exceed this capability of the UE.

- According to the current specification, the OLPC parameters for the PSFCH transmissions are configured independently for each SL pool, and in this case, only PSFCHs transmitted in the same SL pool are guaranteed to have the same power value.

- Conclusion

In this contribution, we discussed how RAN1 provides feedbacks on RAN4’s questions in LS [1]. The following observation is given.

Observation: When the PEMAX,c defined by RAN4 for the simultaneous PSFCH transmissions on the multiple SL pools applies to the RAN1 specification, further discussion/clarification are required on at least the following aspects:

- If the PEMAX,c of RAN4 is used when selecting the number of transmitted PSFCHs and calculating their power values for each SL pool according to the RAN1 specification, the sum of calculated power values of finally selected PSFCHs on the multiple SL pools could exceed the maximum power of the UE.

- According to the RAN1 specification, the UE capability of the maximum number of PSFCHs that can be simultaneously transmitted is considered individually for each SL pool, and the number of transmitted PSFCHs in each SL pool is determined based on this restriction. When following this principle, the total number of finally selected PSFCHs on the multiple SL pools could exceed this capability of the UE.

- According to the current specification, the OLPC parameters for the PSFCH transmissions are configured independently for each SL pool, and in this case, only PSFCHs transmitted in the same SL pool are guaranteed to have the same power value.

- Reference

[1] R1-2300030, “LS on PSFCH configured power with multiple resource pools,” RAN4, LG Electronics.