3GPP TSG RAN WG1 #112                                   R1-2301545

Athens, Greece, February 27th – March 3rd, 2023

Source:	Sharp

Title:	Discussion on co-channel coexistence for LTE sidelink and NR sidelink

Agenda Item:	9.4.2

Document for:	Discussion and Decision

Introduction

A work item on NR sidelink evolution was approved in RAN#94e meeting [1], with one of the objectives to “study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any”, by “reusing the in-device coexistence framework defined in Rel-16 as much as possible”.

In this document, we share our views on a few aspects of co-channel coexistence for LTE sidelink and NR sidelink.

Discussion

Dynamic co-existence solution

Use of candidate information shared by SL LTE module

It was agreed in last RAN1 meeting that NR SL module uses shared information from the LTE SL module to exclude resources from its own candidate resource set when performing the resource (re)selection procedure in the PHY layer. Several points regarding how to exclude resource were left to FFS.

Agreement For dynamic resource pool sharing, the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, where - The NR SL module excludes resources based on the shared information from its own candidate resource set when performing the resource (re)selection procedure in the PHY layer. - FFS how to exclude resources at least based on the time and frequency locations of LTE SL transmissions that have been indicated in the shared candidate information. - FFS how the exclusion is performed according to clause 8.1.4 of TS 38.214. - FFS: whether/how NR SL module excludes resources not belonging to the generated LTE SL’s candidate resource set SB from its own candidate resource set.

Regarding how NR SL module use shared information from LTE SL module to perform resource exclusion, one way is that NR SL module excludes all time and frequency locations including both reserved resources by other LTE UE and resources used for its own LTE SL transmission when performing the resource (re)selection procedure for NR SL transmission. The way can contribute to adding a simple exclusion procedure to clause 8.1.4 of TS 38.214. For example, the exclusion procedure of time and frequency locations shared by LTE SL module can be performed at step 4) or step 7) without involvement in RSRP adjustment. Although the straightforward way ensures LTE SL transmission to be not affected by NR SL transmission, it seems to over-protect LTE SL transmission and to perform excessive resource exclusion. The way may lead to insufficient resource for NR SL resource selection such that NR SL module needs to use a very high RSRP to determine resource for NR SL transmission, which will cause high interference to other NR SL transmission and degrade the performance of NR SL transmission.

One alternative way is to separately treat time and frequency locations of reserved resources by other LTE UE and time and frequency locations of resources used for own LTE SL transmissions when performing resource (re)selection procedure for NR SL transmission. For time and frequency locations of reserved resources by other LTE UE, SL RSRP measurement results can be used to further determine whether to exclude corresponding reserved resource. NR SL transmission on candidate resource overlapping with reserved resources with low RSRP measurement results does not cause bad interference to LTE SL transmission on reserved resources with low RSRP measurement results. Then the exclusion procedure of time and frequency locations of reserved resources by other LTE UEs can be performed at step 6) of clause 8.1.4 of TS 38.214. On the other hand, for time and frequency locations of resources used for its own LTE SL transmission, if the resources are within the RSW, NR SL module can exclude them when performing the resource (re)selection procedure for NR SL transmission. The benefit is to ensure NR SL transmission to not block its own LTE SL transmission. The exclusion procedure of time and frequency locations of resources used for own LTE SL transmission can be performed at step 4) or step 7). For the subframes used for its own LTE SL transmission within the monitoring window (i.e. half-duplex subframes), since the candidate resources corresponding to half-duplex subframes may already be reserved by other LTE UE(s), LTE SL module excludes all the corresponding candidate resources within the RSW in step 5) of clause 14.1.1.6 of TS36.213. Similar exclusion procedure of NR SL module can be performed at step 5).

Proposal 1: For dynamic resource pool sharing, the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, where

- The NR SL module involves RSRP measurement results in determining whether or not to exclude time and frequency locations of reserved resources by other LTE UE.

  - The exclusion procedure can be performed at step 6) of clause 8.1.4 of TS 38.214.

- The NR SL module excludes time and frequency locations of resources used for own LTE SL transmission.

  - The exclusion procedure can be performed at step 4) or step 7) of clause 8.1.4 of TS 38.214.

- The NR SL module excludes candidate resources corresponding to non-monitored subframes of LTE SL module.

  - The exclusion procedure can be performed at step 5) of clause 8.1.4 of TS38.214.

PSFCH overlapping with LTE SL transmissions

It leads to AGC issue when PSFCH transmission overlaps with the LTE transmission. Solutions around how to handle PSFCH transmission overlapping with LTE SL transmission has been discussed and lasted for several RAN1 meetings but without consensus. The following in FL summary [2] are the latest proposal.

Proposal 1-1(VII): - For dynamic resource pool sharing, in NR SL resource pools with PSFCH configured and when HARQ-ACK is enabled, based on (pre-)configuration, when PSFCH resources overlap with resources to be used for LTE SL transmissions in the time domain, the NR SL UE - Always avoids transmissions on the PSFCH resources (Alt 1), or - FFS details including whether the TX UE avoid selecting resources for PSCCH/PSSCH transmissions with the overlapping PSFCH resources and/or RX UE does not transmit on the overlapping PSFCH resources. - Does not avoid transmission on the PSFCH resources (Alt 2), or - Conditionally avoids transmissions on a subset of the PSFCH resources. - FFS details of conditions including - a (pre-)configured subset, - the consideration of the LTE RSRP and LTE and/or NR priority, - presence of PSCCH/PSSCH transmission in the same time slot LTE subframe as PSFCH transmission with the same power by the same UE. - FFS for the case when there is an overlapping of time and frequency resources between PSFCH and LTE SL transmission - Introduce additional PSFCH periodicity of [5, 8 and] 10. - Note: Alignment between PSFCH periodicity and LTE logical subframes should be ensured by proper configuration. - FFS: Whether to confine the PSFCH transmission, in the time domain, within the guard symbol of the LTE SL subframe. - FFS details including the conditions.

In our view, the option of “always avoids transmissions on the PSFCH resources” in above FL proposal is a straightforward way to solve the AGC issue when collision between the PSFCH transmission and LTE SL transmissions occurs. For device type A, irrespective of whether the UE is PSCCH/PSSCH RX UE or PSCCH/PSSCH TX UE, NR SL module can use the candidate information shared by the LTE SL module to identify a set of resources which may be used for LTE SL transmission. Therefore, the PSCCH/PSSCH RX UE is able to be aware of whether the PSFCH transmission overlaps with LTE SL transmission. And if the PSFCH transmission overlaps with LTE SL transmission, the PSCCH/PSSCH RX UE can drop the PSFCH transmission. Likewise, when NR SL module of the PSCCH/PSSCH TX UE perform resource selection, the NR SL module can be aware of whether the PSFCH transmission associated with selected resources overlaps with LTE SL transmission or not. If PSFCH transmission associated with a resource for PSSCH transmission may overlap with the identified set of resources used for LTE SL transmission, the NR SL module of the PSCCH/PSSCH TX UE is likely to avoid selecting the resource for PSSCH transmission.

For the option of “Does not avoid transmission on the PSFCH resources”, the intention is to make LTE UE avoid transmitting LTE SL transmission on PSFCH resources if a new periodicity is introduced and the LTE UE measures SL RSSI on previous PSFCH resources. As have been concerned by companies regarding the SL RSSI measurement of 2-symbol PSFCH resource, it is also concerned to us whether the LTE SL UE can always and timely preclude the subframes overlapping with the periodically repeating set of PSFCH slots for SL transmission. Moreover, if both original NR periodicity and additional periodicity are used in a same resource pool, the option with additional periodicity may need to modify the existing implicit PSFCH configuration and mapping in Rel-16/17 to adapt PFSCH resource with unequal intervals in the basic set.

In view of the fact that the discussion has lasted for several RAN1 meetings and agreeing only one option seems an impossible way, we can accept the above FL proposal by introducing (pre-)configuration. Otherwise, the fundamental function of PSFCH in NR-SL cannot be supported in dynamic resource pool sharing. Details for each option can be further discussed to solve corresponding concerns after the direction of FL proposal by introducing (pre-)configuration is agreed.

Proposal 2:

- For dynamic resource pool sharing, in NR SL resource pools with PSFCH configured and when HARQ-ACK is enabled, based on (pre-)configuration, when PSFCH resources overlap with resources to be used for LTE SL transmissions in the time domain, the NR SL UE

  - Always avoids transmissions on the PSFCH resources (Alt 1), or

    - FFS details including whether the TX UE avoid selecting resources for PSCCH/PSSCH transmissions with the overlapping PSFCH resources and/or RX UE does not transmit on the overlapping PSFCH resources.

  - Does not avoid transmission on the PSFCH resources (Alt 2), or

  - Conditionally avoids transmissions on a subset of the PSFCH resources.

    - FFS details of conditions including

      - a (pre-)configured subset,

      - the consideration of the LTE RSRP and LTE and/or NR priority,

      - presence of PSCCH/PSSCH transmission in the same time slot LTE subframe as PSFCH transmission with the same power by the same UE.

  - FFS for the case when there is an overlapping of time and frequency resources between PSFCH and LTE SL transmission

- Introduce additional PSFCH periodicity of [5, 8 and] 10.

  - Note: Alignment between PSFCH periodicity and LTE logical subframes should be ensured by proper configuration.

- FFS: Whether to confine the PSFCH transmission, in the time domain, within the guard symbol of the LTE SL subframe.

- FFS details including the conditions.

Device types

In previous RAN1 meetings, there was a debate regarding the coexistence vs. backward-compatibility. We are of the opinion that the “Rel-18 co-channel coexistence” feature should have no or minimal dependences on other Rel-18 SL features, and that means e.g. a Rel-16 or Rel-17 UE can also implement such a feature (without implementing other Rel-18 SL features). In that sense there is no need to mention the release in the definition of a device type (for type A, type B and type C).

Observation 1: There is no need to mention the release of a device in the definition of a device type.

Regarding whether to additionally consider type B devices, we share a number of other companies’ view that this is not in the scope of the WID, with the reason that the WID requires to “reuse the in-device coexistence framework defined in Rel-16 as much as possible”, which does not make sense to a type B device. In our view, for type B devices, instead of concluding whether they should be “considered” or not in the study, it may be more appropriate to conclude that there is no specification work for such devices.

Proposal 3 (for conclusion): From RAN1 perspective, no specification work is envisioned for Type B devices (that contain only NR SL module, or contain a co-located LTE SL and NR SL module but the LTE SL module does not share any LTE sensing and resource reservation information).

FDM-based solutions

The following FL proposal was discussed in RAN1#110 meeting, with no consensus,

Proposal 2-2 (II): - For co-channel coexistence in Rel-18, RAN1 assumes that the FDM-based semi-static resource pool partitioning can be used based on Rel-16/17 specifications, and can be studied with the following constraints: - NR SL resource pool is configured with only 15 kHz SCS. - FFS other solutions to overcome the AGC issue caused by the differing SCSs between the NR SL and LTE SL resource pools. - Transmission/reception Configuration of PSFCH in resources overlapping with LTE SL subframes is not permitted. - FFS other solutions to overcome the AGC issues caused due to PSFCH being configured in NR SL resource pools. - FFS other constraints whether a guard band is required requirement. - Note: The LTE and NR resource pools do not overlap in frequency with each other in the FDM-based semi-static resource pool partitioning.

Our understanding is that for FDM-based semi-static resource pool partitioning, configuration of a same SCS (of 15 kHz) and PSFCH configuration without overlapping LTE SL subframe for NR SL and LTE SL is feasible, and not only avoid the AGC issue but also has no specification impact. If there is any concern about this in RAN1, e.g. if it is believed that there are some specification impacts, then the corresponding issues should be listed and discussed in RAN1. Otherwise we see no reason to keep this issue open. Guard band requirement between NR SL resource pool and LTE SL resource pool can be identified by RAN4. The guard band requirement should have no specification impact on RAN1.

Therefore, from RAN1 perspective, FDM-based semi-static resource pool partitioning with above constrains is one possible solution based on the existing Rel-16/17 specifications. The above FL proposal is acceptable to us and can be taken as conclusion.

Proposal 4 (for conclusion):

- For co-channel coexistence in Rel-18, RAN1 assumes that the FDM-based semi-static resource pool partitioning can be used based on Rel-16/17 specifications, and can be studied with the following constraints:

  - NR SL resource pool is configured with only 15 kHz SCS.

  - Configuration of PSFCH in resources overlapping with LTE SL subframes is not permitted.

  - FFS whether a guard band is required.

  - Note: The LTE and NR resource pools do not overlap in frequency with each other in the FDM-based semi-static resource pool partitioning.

Conclusion

In this contribution, we discuss a few aspects relating to co-channel coexistence for LTE sidelink and NR sidelink, and make the following observation and proposals.

Proposal 1: For dynamic resource pool sharing, the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, where

- The NR SL module involves RSRP measurement results in determining whether or not to exclude time and frequency locations of reserved resources by other LTE UE.

  - The exclusion procedure can be performed at step 6) of clause 8.1.4 of TS 38.214.

- The NR SL module excludes time and frequency location of resources used for own LTE SL transmission.

  - The exclusion procedure can be performed at step 4) or step 7) of clause 8.1.4 of TS 38.214.

- The NR SL module excludes candidate resources corresponding to non-monitored subframes of LTE SL module.

  - The exclusion procedure can be performed at step 5) of clause 8.1.4 of TS38.214.

Proposal 2:

- For dynamic resource pool sharing, in NR SL resource pools with PSFCH configured and when HARQ-ACK is enabled, based on (pre-)configuration, when PSFCH resources overlap with resources to be used for LTE SL transmissions in the time domain, the NR SL UE

  - Always avoids transmissions on the PSFCH resources (Alt 1), or

    - FFS details including whether the TX UE avoid selecting resources for PSCCH/PSSCH transmissions with the overlapping PSFCH resources and/or RX UE does not transmit on the overlapping PSFCH resources.

  - Does not avoid transmission on the PSFCH resources (Alt 2), or

  - Conditionally avoids transmissions on a subset of the PSFCH resources.

    - FFS details of conditions including

      - a (pre-)configured subset,

      - the consideration of the LTE RSRP and LTE and/or NR priority,

      - presence of PSCCH/PSSCH transmission in the same time slot LTE subframe as PSFCH transmission with the same power by the same UE.

  - FFS for the case when there is an overlapping of time and frequency resources between PSFCH and LTE SL transmission

- Introduce additional PSFCH periodicity of [5, 8 and] 10.

  - Note: Alignment between PSFCH periodicity and LTE logical subframes should be ensured by proper configuration.

- FFS: Whether to confine the PSFCH transmission, in the time domain, within the guard symbol of the LTE SL subframe.

- FFS details including the conditions.

Observation 1: There is no need to mention the release of a device in the definition of a device type.

Proposal 3 (for conclusion): From RAN1 perspective, no specification work is envisioned for Type B devices (that contain only NR SL module, or contain a co-located LTE SL and NR SL module but the LTE SL module does not share any LTE sensing and resource reservation information).

Proposal 4 (for conclusion):

- For co-channel coexistence in Rel-18, RAN1 assumes that the FDM-based semi-static resource pool partitioning can be used based on Rel-16/17 specifications, and can be studied with the following constraints:

  - NR SL resource pool is configured with only 15 kHz SCS.

  - Configuration of PSFCH in resources overlapping with LTE SL subframes is not permitted.

  - FFS whether a guard band is required.

  - Note: The LTE and NR resource pools do not overlap in frequency with each other in the FDM-based semi-static resource pool partitioning.

References

RP-213678, “New WID on NR sidelink evolution”, OPPO, LG Electronics, RAN#94e.

R1-2212540, “FL Summary EoM for AI 9.4.2 – Co-Channel Coexistence for LTE and NR Sidelink”, Moderator (Fraunhofer HHI), RAN1#111.