| 3GPP TSG RAN WG1 #112 | R1-2301588 |
| --- | --- |
| Athens, Greece, February 27th – March 3rd, 2023 |  |

Agenda Item: 9.2.2.2

Source: MediaTek Inc.

Title:	Other aspects on AI/ML for CSI feedback enhancement

Document for: Discussion & Decision

# Introduction

In the study item ‎[1] scope, it has been agreed to study CSI feedback enhancement including CSI compression and CSI prediction as representative sub use case. In this contribution, we study spec impact, requirements, and advantage of each sub use case.

# CSI Compression

## Training strategies of two-sided AI/ML models

Unlike other sub use cases, where AI/ML models can be merely deployed at a single entity, CSI compression sub use case focuses on two-sided AI/ML models. The two-sided AI/ML models in CSI compression are auto-encoders (AEs) from which encoders are used by UEs and decoders are employed by gNBs during the inference stage. The two-sided AI/ML models in CSI compression raise some fundamental issues. As each vendor may own its exclusive encoder or decoder depending on the vendor’s purpose, the performance of such an encoder/decoder in conjunction with other vendors’ decoder/encoder entails a big question mark. In fact, new training strategies are required to make encoders and decoders of different vendors inter-operable. To facilitate inter-operability of AI/ML models from different vendors, three types of training strategies have been focused in  ‎[2]. We cast our detailed view on these training strategies in the following.

Training type 1: Joint training at a single entity

Training type 1 aims at training a well-designed AE at a single entity (either UE or gNB) during the training phase and offering the pre-trained encoder or decoder of AE to other entities upon their request in the inference phase. This training type does not offer a solution for inter-operability of encoders and decoders from different vendors, instead it bypasses the problem by guaranteeing that the encoders used by UEs and decoders used by gNBs are already inter-operable (matched in structure and their performance has already been verified). Naturally such a training strategy has superior performance to all other training strategies that we will discuss shortly. Training type 1 can be pursued in two directions depending on which side (UE or gNB) takes the responsibility of training.

Training type 1 – training at UE:  As shown in Figure 1(a), in the training phase, UE takes full responsibility of training a whole AE including an encoder and a decoder. UE can leverage proprietary/public dataset and train AE in an individual forward pass (FP) and backpropagation (BP) loop. Once the training of AE is finished, UE obtains an encoder and a decoder which effectively perform compression and decompression, respectively. In the inference phase, the gNB requests pre-trained decoder to establish a two-sided AI/ML model with the aid of UE. UE uploads the decoder (exchangeable part of AE) for the gNB and keeps the encoder (unexchangeable part of AE). gNB deploys the received decoder and uses it for decompressing the CSI feedback afterward.

Training type 1 – training at gNB: As shown in Figure 1(b), in the training phase, gNB takes full responsibility of training an entire AE including an encoder and a decoder. gNB uses a proprietary/public dataset and trains the AE in an individual FP and BP loop. In the inference phase, UE requests pre-trained encoder for CSI compression. UE downloads the encoder (exchangeable part of AE), and gNB keeps the decoder (unexchangeable part of AE). UE and gNB leverage encoder and decoder to form a two-sided AI/ML model and perform CSI compression and decompression, respectively.

| - Type 1 - training at UE | - Type 1 – training at gNB |
| --- | --- |

Figure ‎2-1: Illustration of training type 1 when AE is trained at UE or gNB side

Whether UE or gNB trains the model, finally a two-sided AI/ML model is established in which encoder has learned how to provide rich latent vectors (compressed CSI maintaining essential information) and decoder has learned how to interpret the latent vectors. Thereby, inter-operability of vendors is no longer an issue. As revealed by our companion document ‎[3], training type 1 not only has superior performance to other types of training, but it also brings the following advantages which mainly roots in its centralized style of training:

- It does not need inter-vendor collaboration in the training phase

- It does not need inter-vendor signalling in the training phase

- BP and FP can maintain high precision for representing elements in latent vectors and gradients

- LCM is generally easier as all engineering efforts for re-training, debugging, and improving AI/ML model is centralized

Despite prominent advantages offered by training type 1 in the training phase, it has some drawbacks in the inference phase:

- The exchangeable part of AE may not be pre-tested and optimized at HW/SW architecture of the entity which uses the model trained at the other entity

- The exchangeable part of AE and processing related to it cannot be proprietary. For example, if UE trains AE, it should share information of decoder, dequantization, post-processing, and output type. If gNB trains AE, it should share information of encoder, quantization, and pre-processing.

- Depending on the number and size of AI/ML models that one entity may requests from others, the inference signalling overhead can be quite high.

Since the procedure and shared information in training “type 1-training at UE” and “type 1-training at gNB” are different, we believe the spec impact of each has to be discussed separately. Also, we note that alignment on different shared information including model format, structure, pre-/post-processing, and quantization as well as signalling format for request, download, and upload AI/ML models is required.

Categorize type 1 training into “type 1-training at UE” and “type 1-training at gNB”, and discuss the spec impact on each separately.

Discuss potential spec impact on exchangeable part of model in the inference phase including:

- Model format, pre/post-processing, quantization, input/output format.

- Signalling format to request and upload/download exchangeable part of AI/ML model

- Related UE capability

During the inference phase, training entity can keep half of AI/ML model (unexchangeable) part proprietary. However, this action may cripple non-training entity in performance monitoring. For example, in training “type 1-training at gNB”, if the gNB does not reveal the decoder part to UE, UE loses its capability of measuring CSI reconstruction accuracy. On the other hand, gNB lacks access to original estimated CSI at UE, which prohibits the performance monitoring using NMSE and SGCS intermediate KPIs. Thereby, it needs to be meticulously discussed if training entity reveals the entire AI/ML model for the sake of performance monitoring or not.

Discuss sharing full AI/ML model or only the exchangeable part of AI/ML model to non-training entity for performance monitoring.

Training type 2: Joint training at different entities

Training type 2 is introduced to avoid the model exchange issues in the training type 1 which breaks the proprietariness of AE and entails possible large signalling overhead in the inference phase. In training type 2, UE vendors participate with their encoders, and gNB vendors participate with their decoders in a training session. UE and gNB vendors jointly collaborate to train AEs. While the training is done jointly, each entity does not exchange any part of its AI/ML model and instead it exchanges other necessary information including latent vectors and gradient vectors to complete an FP and BP loop across two or more entities. Figure 2 shows the procedure of type 2 training for a single encoder and a single decoder pair, and it shows the signalling among encoder and decoder which completes the FP and BP loop across two different entities (i.e., UE and gNB). Simply put, a shared dataset is used by both encoder (at UE) and decoder (at gNB) for training purpose. The encoder generates the latent vectors and passes them to the decoder for CSI reconstruction. The reconstructed CSI samples will be compared to the target/label CSI samples, and loss will be calculated. The gradient of loss w.r.t. parameters will propagate through the decoder, and the decoder’s parameter will be updated accordingly. The decoder then will pass the gradient vector on its input layer to the encoder, and the encoder will resume BP and updating learnable parameters. While the encoder and decoder are proprietary, the applicable quantization/dequantization as well as format/precision of gradient vectors, latent vectors, and CSI samples may need to be aligned.

For training type 2, discuss alignment of quantization/dequantization as well as format/precision of gradient vectors, latent vectors, and CSI samples.

Figure ‎2-2: Illustration of training type 2 for a single encoder and single decoder

Unlike training type 1, in training type 2, the UE and gNB are not necessarily aware of detailed or type of AI/ML model structure used at the other side, and the structure of encoder and decoder may not match in type, number of layers, complexity, computational requirement, etc. For example, encoder may leverage a simple convolutional neural network (CNN) while decoder may use transformer (TF) as its core architecture. This architecture mismatch results a performance degradation compared to training type 1 where the training entity is implicitly responsible to train a matched architecture of encoder and decoder in its AE. In our companion contribution ‎[3], our evaluation shows that the encoder/decoder mismatch will cause 2.23% performance loss for UE and 2.26% performance loss for gNB. Despite this performance degradation, training type 2 has three prominent advantages:

- Maintains the proprietariness of encoders and decoders

- Naturally serves as performance upper-bound to training type 3

- Much less signalling (over the air interface) overhead in the inference phase

Extension to single-encoder multi-decoder setting

The extension of training type 2 to single-encoder multi-decoder setting is pretty straightforward as the single UE (encoder) can coordinate the training/updating its parameters and triggering different decoders in a Round-Robbin fashion or at the same time. We note that if the UE divides the training session into multiple sub-sessions and sequentially assigns each sub-session for exposing itself to only one of gNBs (decoders), it finally will be biased toward the last decoder which has been exposed to.

For single-encoder multi-decoder setting in training type 2, UE should not break down the training session into multiple single-encoder single-decoder training sub-sessions.

Extension to multi-encoder single-decoder setting

Extension of training type 2 to multi-encoder single-decoder scenario requires more provisions depending on data ownership and inter-vendor learning/update schedule. From data ownership point of view, two cases can be imagined, common/shared dataset and UE-specific datasets. With common dataset, all UEs and the gNB have access to the same CSI samples to generate the latent vectors and calculate CSI reconstruction loss. Using UE-specific datasets, each UE vendor does not share its CSI sample with other UE vendors, and it may share the CSI sample (with desired type, e.g., eigenvectors or raw CSI) with the gNB to make gNB able to calculate the CSI reconstruction loss. In this sense, if vendors use UE-specific datasets, the type of target CSI (shared with gNB) should be aligned among UE vendors.

In training type 2 for multi-encoder setting, if UE-specific datasets are used, the type of target CSI should be aligned among UE vendors.

From learning/update scheduling perspective, three major cases with different levels of coordination can be implemented: i) concurrent update scheduling, ii) alternating update scheduling, and iii) sequential update scheduling.

Concurrent update scheduling: In this case, for each parameters’ update at UEs’ encoders and gNB’s decoder, a batch (minibatch) of CSI data samples can flow through encoders and the decoder. As the CSI samples in the batches are shared with the gNB, gNB is able to calculate the joint loss accumulated from all CSI reconstructions and initiate BP. The procedure is shown in Figure 3.

Figure ‎2-3: Concurrent update scheduling in multi-encoder single-decoder setting for training type 2

Alternating update scheduling: Coordinating concurrent update may not be feasible due to alignment in size of batch, defining an appropriate joint loss, etc. In this case, UE vendors can individually trigger FP and BP using a batch or multiple batches of their specific CSI samples in an alternating pattern to maintain the fairness of their exposure to the common decoder. This training style is shown in Figure 4, where one parameters’ update (FP and BP) using a batch at UE vendor A is followed by a parameters’ update at UE vendor B, and these alternating updates last till the end of the training session. However, the inter-vendor synchronization is required at the UE side to not interfere parameter updates of each other.

Figure ‎2-4: Alternating update scheduling in multi-encoder single-decoder setting for training type 2

Sequential update scheduling: If concurrent or alternating update scheduling among UE vendors is not reachable/preferred, the training session can be divided into multiple sub-sessions during each of which one UE vendor uses its dataset (shared or specific) for training purpose as shown in Figure 5. While the synchronization requirement is almost rectified in this case, gNB vendor needs additional provisions to maintain its performance with the UE vendors which it has been exposed to them during initial sub-sessions.

Figure ‎2-5: Alternating update scheduling in multi-encoder single-decoder setting for training type 2

Discuss feasibility of synchronization/alignment required for different update scheduling in training type 2.

Another possible issue in multi-encoder setting is unintentional bias created by a UE-vendor such as excessive size of its dataset compared to the others, abnormal statistics of some datasets, unfair loss functions, etc. For example, consider gNB is using MSE function and dataset from a certain UE has samples with large values. Probably, gNB will be tempted to minimize loss function with focusing on large-valued samples and give up on the rest. As another example consider the case where one UE bombards gNB with its excessively large dataset.  If the underlying distribution of those CSI samples is different from CSI samples of other UEs, the gNB again trains a decoder that may not properly work for UEs with smaller datasets. As such, even if the UE vendors leverage UE-specific datasets, sharing information on training-related parameters such as size of datasets, statistics of datasets, training loss, update schedule, etc. is helpful to assure UE vendors about fairness of training sessions they are involved into.

If UE-specific datasets are used for multi-encoder training, consider sharing information on training-related parameters such as size of datasets, statistics of datasets, training loss, and update schedule.

In brief, the major advantage of training type 2 is enhancing the performance of unmatched encoder-decoder pair (as a single AE) in the inference phase by exposing them to each other in the training phase. The costs of this enhancement are frequent inter-vendor information exchange (e.g., latent vectors and gradient vectors) and the need for possible inter-vendor coordination.

Training type 3: Sequential separate training

Training type 3 relaxes the coordination requirements of training type 2 by offering a sequential separate training at UE and gNB sides. In this training type, either gNBs’ decoders or UEs’ encoders will be trained first, and then other parties will train their corresponding part of AEs accordingly. Based on possible order of training, the training type 3 has two categories: UE-first separate training and gNB-first separate training. A simple implementation of sequential separate training is described in the following for the most general setting where multiple encoders (at UEs) and multiple decoders (at gNBs) are trained in a single training session.

UE-first separate training: An example implementation of UE-first separate training may entail the following steps in order:

Step 1: Each UE leverages training type 1 to train an AE

Step 2: Each UE uses its trained encoder on an alignment dataset and generates latent vectors

Step 3: UEs provide compound datasets including CSI samples and corresponding latent vectors to gNBs

Step 4: gNBs collect compound datasets from all UEs and train their decoders.

gNB-first separate training: An example implementation of gNB-first separate training may entail the following steps in order:

Step 1: Each gNB leverages training type I to train an AE

Step 2: Each gNB uses its trained encoder on an alignment dataset and generates latent vectors

Step 3: gNBs provide compound datasets including CSI samples and corresponding latent vectors to UEs

Step 4: UEs collect compound datasets from all gNBs and train their encoders.

In our companion contribution ‎[3], we have evaluated training type 3 including both UE-first and gNB-first cases. In general, our results confirm the natural performance degradation of training type 3 compared to training 1 and training type 2. We also identified the risk of pairing unmatched encoder and decoders are higher in the training type 3, and between UE-first and gNB-first separate training, this risk is higher for gNB-first training. How to avoid such a degradation from unmatched pairs is a challenge that need information other than training inputs/outputs, i.e., latent vectors and CSI samples. Information such as encoders’/decoders’ types and complexity can be useful for the parties come second in the training order.

Consider sharing information about encoders’/decoders’ architecture type and complexity from entities doing training first to other entities.

## Quantization

Quantization in latent space

As shown in Figure ‎2-6, quantization is an indispensable step of CSI compression, where the quantizer module first converts the (latent) output of the encoder into a bitstream at the UE side, and later the de-quantizer module will translates back the bitstream into the latent input of the decoder at the gNB side. However, two-sidedness of the quantization, its natural non-differentiability, and abondance of quantization techniques bring unique challenges to the CSI compression sub use case, necessitating extensive study to identify scalable, accurate, and yet simple quantization technique which fit the budget of gNB/UE and tolerate multi-vendor wireless ecosystem.

Figure ‎2-6: Quantization framework in CSI compression

The goal of this contribution is to study and identify effective quantization methods for CSI compression and further analyse the requirements and spec impacts of each. To do so, we first numerate the major features that distinguish quantization methods, including training-awareness, learnability, and mapping (from latent to quantization space).

Training awareness

Training-aware indicates whether the quantization has been exposed to the encoder and decoder parts of AI/ML model in the training stage or not. This awareness is particularly helpful as exposure of the quantization to the AI/ML model helps it to compensate the inflicting error of quantization and adjust its parameters accordingly. This further avoids possible CSI reconstruction accuracy mismatch in the training and inference stages.

(a) Training-non-aware quantization

(b) Training-aware quantization

Figure ‎2-7: Illustration of the difference between training-aware and training-non-aware quantization

As shown in Figure ‎2-7(a), in a training-non-aware (TNA) approach, the training stage happens in the absence of the quantization, while the quantization method will be later deployed between the encoder and decoder of AI/ML model in the inference stage. Therefore, the sudden introduction of quantization may deteriorate the overall performance of AI /ML model. As shown in Figure ‎2-7(b), in training-aware (TA) approach, the quantization will be presented in the training stage, avoiding any mismatch between training and inference stages. As evaluation results in our companion contribution ‎[4] show, training awareness improves the overall performance of AI/ML models for CSI compression by 6.5%. Thereby, we propose to prioritize TA quantization methods.

Prioritize to study the training-aware quantization methods

Quantization (including quantizer and dequantizer modules) is a non-differentiable function by nature. Therefore, some provisions required to perform backpropagation in its presence. There are some techniques to rectify non-differentiability of quantization function such as artificial gradient, approximation of quantization in FP, approximation of quantization in BP, etc. However, pros and cons of each method are not well-studied. Also, it is not clear whether a general alignment is required among the techniques being used at gNB (dequantizer) and UE (quantizer) for training awareness.

Study alignment requirement and influence of different training awareness techniques for enabling backpropagation between quantizers and dequantizers.

Nevertheless, any training awareness technique which is adopted by gNB can highly impact the BP at  UE’s encoder if training type 2 is applied. As an example, let us assume during training type 2, gNB applies a dequantizer and does not treat its BP behavior at all. In this case, while decoder can perform BP without any problem, encoder fails to do it. As another example, let us assume gNB injects a large constant gradient for the sake of making BP able to go through dequantizer. Such an action highly de-stabilizes the training of the UE’s encoder. As such, we believe the gNB should inform UE about the training awareness technique used for its dequantizer at least for training type 2.

For training type 2, gNB should inform UE about the training awareness technique used for its dequantizer.

Learnability

Another aspect of quantization methods is learnability of their parameters/configurations. Learnability implies whether the quantization parameters/configurations will change in the course of training stage. Thereby, it is a unique feature of TA quantization methods. An example of non-learnable (NL) quantization is a fixed uniform quantization whose intervals and levels remain unchanged during the training stage. An example of learnable quantizer is a uniform quantizer whose levels and range will be adjusted in the course of training stage.

Mapping/Codeword assignment

Regardless of training awareness and learnability, any quantization method aims at discretizing the feasible space of the encoder’s output and mapping an entire sub-space into a single representative point (a.k.a. codeword). How to generate a codeword for each sub-space is another point of classification. There are two major approaches, “vector quantization (VQ)” and “scalar quantization (SQ)”. In the SQ, each element on the latent output will be mapped to a new discretized element, and the final codeword will be a vector of all individually discretized elements. In the VQ, however, there is unique mapping from multiple/all elements on latent output to multiple/all elements of discretized/quantization space. VQ also suffers from two problems. First, the codebook design for VQ is computationally expensive, and its complexity will increase with the dimension of its input. Second, the number of CSI samples in the training dataset should exceed the number of representative points in the codebook which is simply 2FB-OH. Therefore, for a high accuracy and large feedback overhead, the number of required CSI samples grows to an unpragmatic number, which hinders us to design a codebook that can handle the entire latent output together. To address these issues, it is inevitable to break down the latent vectors to smaller segments. With the aid of segmentation, a codebook will be designed for each segment as shown in Figure ‎2-8. In the inference stage, the same segmentation approach will be applied to the latent, and a codeword will be assigned to each segment. The final codeword will be the concatenation of all codewords of segments.

Figure ‎2-8: General framework of designing a VQ

In our companion contribution ‎[4], it is shown that segmentation harms the performance of VQ, and the SQ methods may outperform VQ as they do not suffer from the aforementioned issue. Also, VQ is very sensitive to any changes in AI/ML model. Thereby, we believe if VQ is used for quantization of latent space, it should be aligned at UE and gNB sides. Also, we could not find any advantage for VQ over SQ even though VQ needs more computational and alignment efforts, and its performance is not very promising.

For VQ, UE and gNB should align their codebook and segmentation approach.

Prioritize SQ methods over VQ.

On classification of different quantization methods

Figure ‎2-9: Classification of different quantization methods

We believe the training awareness, learnability, and mapping (codeword assignment) provide a comprehensive description of any quantization method. These three aspects can be used for classifying various approaches in quantizing the latent space as shown in Figure ‎2-9.

For describing a quantization method at UE and gNB, use training awareness, learnability, and mapping (codeword assignment) as its main attributes.

Quantization in gradient space

In training AI/ML models for CSI compression, quantization is not limited to the latent space. Other than latent output of CSI generation part (encoder), the gradient may be also exchanged among two entities of training, i.e., gNB and UE. The gradient exchange overhead is exactly equal to that of the latent outputs. Therefore, gradient quantization and latent quantization are equally important in controlling the information exchange overhead. However, this issue is overlooked in the previous meetings. It should be noted this is a unique issue for training type 2 where BP will happen across both gNB and UE entities, unlike other training types where gradient exchange does not happen.

Figure ‎2-10: Gradient exchange in training type 2

As shown in Figure ‎2-10, in training type 2, the gradient should be first quantized with a quantization which is not necessarily the same as latent quantizer. The quantized gradient will be sent to the UE for completing the BP loop. UE first dequantizes the gradient and continues BP through the latent quantizer and the rest of AI/ML model. The spec impact, performance impact, and possible alignment between latent and gradient quantization shall be further studied for training type 2.

For training type 2, discuss spec impact, performance impact, and possible alignment between latent and gradient quantization.

Quantization in data collection

Initial step in training an AI/ML model for any sub use case is data collection. It is widely expected that gNB establishes the data collection procedure; however, with disregarding UE’s capabilities, computational power, and storage budget, this procedure may inflict a huge burden on UEs. Also, UEs’ specs vary on both device and vendor basis. Therefore, we believe despite having a gNB-centric data collection framework, UE should provide gNB with a range of possible options for various aspects of the data collection procedures including but not limited to types of input CSI, types of assistant information, quantization parameters, periodicity of data collection, and maximum amount of data collected per period.

While gNB is main entity in establishing data collection procedure, UE should provide gNB with a range of possible options for configurations of the data collection procedure including but not limited to:

- Types of input CSI

- Types of assistant information

- Quantization parameters

- Periodicity of data collection

- Maximum amount of data collected per period

Nevertheless, the data collection stage comes with a huge overhead, mainly due to sending bulky CSI reports. To facilitate the data collection, it is inevitable to quantize CSI samples. It has been shown in ‎[5], the quantization can significantly reduce the overhead of data collection with negligible impact on the accuracy of the AI/ML model. However, it is still unclear which entity is responsible for configuring quantization and whether configuration changes per sample or remains unchanged for the entire data collection period.

Discuss the quantization of data in the following aspects:

- Decisioning entity

- Quantizable information (CSI samples and assistant information)

- Configuration changes (per sample or per dataset)

## Life Cycle Management

Life cycle management (LCM) includes the crucial steps of using the best CSI compression solution at any moment. LCM mainly consists of model deployment, monitoring, and vital actions such as activation/deactivation, switching, and fallback to non-AI/ML-based solution depending on AI/ML models performance and RF environment. In this section, we go over to the main steps of LCM.

Model management

It has already been agreed that UE and gNB have access to a set of AI/ML models each of which are associated with an ID and a range of representative information. However, there is no agreement what those representative attributes are. In brief, gNB can take a monitoring action based on detecting a monitoring event or inspecting the representative information of candidate AI/ML models. gNB then is able to inform UE about the ID of suitable AI/ML model that fits current RF environment or configuration. If no suitable AI/ML model is found among possible AI/ML models, gNB may decide to initiate fallback to non-AI/ML-based solution as well. So far, little progress has been made in detailing such a procedure, and we had just a consensus on the overall concept.

Study spec impact, signalling requirements, and candidate representative information of AI/ML models for activation, deactivation, switching, and fallback.

Also, all efforts have focused on monitoring the AI/ML models that are currently being used by UE and gNB. It is not discussed how to monitor the performance of other candidate AI/ML models which are not being used at the moment of monitoring. Then, the question is how UE and gNB can identify the best candidate AI/ML model if a monitoring event is detected.

Discuss methods and apparatus for monitoring AI/ML models other than the one which is already being used by UE and gNB.

Model monitoring

Model monitoring aims at tracking the changes of RF environment and AI/ML model effectiveness in CSI compression to adopt the best possible solution at any situation. While the decisioning entity in taking a monitoring action is gNB, monitoring entity can be either UE or gNB. To do this task, gNB and UE may rely on statistics of input/output CSI, intermediate KPIs, and system-level indicators (eventual KPIs).

Input/output-based model monitoring

It is natural that any changes in RF environment will be reflected in input CSI, and as there is a unique mapping between input CSI and output CSI, such changes will flow through output CSI as well. Therefore, it is possible for UE and gNB to identify monitoring events based on input CSI and output CSI, respectively. For the UE-side input-based model monitoring, UE is able to inspect statistics of input CSI samples like range, variance, distance to anchor points, etc. and compare to that of the training scenario and identify possible discrepancy. gNB can similarly monitor the discrepancy between statistics of output CSI in training and inference stages of an AI/ML model for detecting a monitoring event. It has been already shown such statistics (at least for input CSI) is able to identify monitoring events ‎[6]. This monitoring method inflicts no spec impact w.r.t. what already is introduced for general monitoring purposes. Also, the monitoring mechanism only resides at the UE or gNB. It neither necessitates AI/ML model disclosure of any party nor imposes large overhead of exchanging raw input/output CSI, making it an amenable candidate for practical implementation. However, accuracy of these methods is questionable and need to be further studied.

Given promising features of input/output-based monitoring, accuracy of possible solutions shall be further studied.

Intermediate-KPI-based model monitoring

The effectiveness of an AI/ML model for CSI compression cannot be better measured than monitoring the intermediate KPIs, such as GCS, SGCS, MSE, and NMSE. It is essentially the most accurate monitoring method to identify a monitoring event or compare a set of candidate AI/ML models. The intermediate-KPI-based monitoring can be either performed by UE or gNB as described in what follows.

UE-side intermediate-KPI-based model monitoring

In UE-side intermediate-KPI-based model monitoring, UE is the first entity to measure the intermediate KPI. UE will later inform gNB about monitoring events and intermediate KPIs to take possible monitoring actions. Such an approach can be implemented through two alternatives as shown in Figure ‎2-11.

| - UE-side monitoring - Alternative 1 | (b) UE-side monitoring - Alternative 2 |
| --- | --- |

Figure ‎2-11: Illustration of two different alternatives for UE-side monitoring

Alternative 1: As shown in Figure ‎2-11(a), gNB can send its decoder to the UE. Thereafter, UE can access to the entire AI/ML autoencoder model and measure intermediate KPIs upon estimating input CSI.

Alternative 2: As shown in Figure ‎2-11(b), gNB can send output CSI to the UE. The UE can measure intermediate KPIs as it has access to both input and output CSI samples.

Both alternatives come with significant shortcomings. In Alternative 1, gNB must disclose its AI/ML model, making it impossible to maintain any proprietary AI/ML model at the gNB side. Also, the gNB’s AI/ML model is not designed w.r.t. UE’s capabilities and budgets. UE may struggle in handling gNB’s AI/ML model due to excessive complexity. It may also go through the re-compiling, pruning, and quantization of AI/ML model as well to finally deploy the full AI/ML autoencoder model. Alternative 2 does not suffer from such issues. However, it comes with large airtime overhead in the downlink as the output CSI with equal overhead of input CSI should be sent to the UE.

gNB-side intermediate-KPI-based model monitoring

In gNB-side intermediate-KPI-based model monitoring, gNB is the first entity to measure the intermediate KPIs with the help of UE. gNB will use an intermediate KPI as a point of judgement to take possible monitoring actions. Such an approach can be implemented through two alternatives as shown in Figure ‎2-12.

| (a) gNB-side monitoring - Alternative 1 | (b) gNB-side monitoring - Alternative 2 |
| --- | --- |

Figure ‎2-12: Illustration of two different alternatives for gNB-side monitoring

Alternative 1: As shown in Figure ‎2-12(a), UE can send its encoder to the gNB. Thereafter, UE can access to the entire AI/ML autoencoder model. For the sake of monitoring, UE will send input CSI to the gNB, and gNB measures intermediate KPIs upon receiving input CSI and calculating output CSI.

Alternative 2: As shown in Figure ‎2-12 (b), UE will send latent output of its AI/ML model in conjugation with input CSI to the gNB. Having access to input CSI, gNB can measure intermediate KPIs upon calculating output CSI.

Both alternatives, however, suffer from large airtime overhead imposed by exchanging input CSI. Also, Alternative 1 mandates disclosure of AI/ML model at UE which is not appealing for UE vendors.

UE/gNB-side proxy-based monitoring

A promising alternative to previous intermediated-KPI-based monitoring is proxy-based monitoring where one party discloses a proxy AI/ML model instead of its actual model. Proxy AI/ML model can be used for constructing a proxy AI/ML autoencoder which results an intermediate KPI which is drifted from the actual one. The proxy AI/ML model is much simpler than the actual one and it does not necessarily bear the same structure type. The core idea is that any changes which can be captured by actual intermediate KPI, it can be captured by the drifted intermediate KPI as well. Our companion contribution ‎[4] has shown the feasibility of detecting monitoring events by drifted KPI. The UE/gNB-side proxy-based monitoring is made possible through the alternatives shown in Figure ‎2-13.

| (a) Proxy-based monitoring - Alternative 1 | (b) Proxy-based monitoring - Alternative 2 |
| --- | --- |

Figure ‎2-13: Illustration of two different alternatives for proxy-based monitoring

Alternative 1: As shown in Figure ‎2-13 (a), gNB can send a proxy AI/ML model for UE to enable forming a proxy AI/ML autoencoder model. Upon measuring the input CSI, UE is able to obtain the drifted KPI and share this information with gNB if a monitoring event is detected.

Alternative 2: As shown in Figure ‎2-13 (b), UE can send a proxy AI/ML model for gNB to enable forming a proxy AI/ML autoencoder model. Thereafter, UE will send input CSI for the sake of monitoring purposes, and gNB will calculate the drifted KPI for possible monitoring actions.

While these approaches secure proprietariness of AI/ML models at both parties, the second alternative comes with large airtime overhead due to input CSI transmission. However, this is not the case for Alternative 1. In overall,  Alternative 1 neither discloses the AI/ML model nor imposes large airtime overhead. It is indeed the best possible solution among all six alternatives for intermediate-KPI-based monitoring.

Prioritize UE-side (Alternative 1) proxy-based model monitoring as the initial monitoring method for tracking intermediate KPI.

System-level model monitoring

The system-level indicators/eventual KPIs, such as throughput, spectral efficiency, ACK/NACK rate, and BLER can also detect the monitoring events, e.g., when an AI/ML model  falls short in handing underlying RF environment and configurations. These indicators, however, bear the effects of both AI/ML model and RF environment which are impossible to be distinguished. As such, variation in system-level indicators cannot be regarded as the single point of decisioning for detection of monitoring events.

System-level indicators cannot be regarded as the single point of decisioning for detection of monitoring events.

Multi-stage model monitoring

We believe none of the aforementioned monitoring methods can individually offer an efficient monitoring tool in terms of overhead, accuracy, and proprietariness. Instead, it would be more pragmatic to adopt a multi-stage monitoring approach where a low-overhead low-accuracy method triggers a more accurate intermediate-KPI based solution with higher overhead as shown in Figure ‎2-14. Such an approach comes effective in terms of overhead, accuracy, and proprietariness of AI/ML models at all parties.

Figure ‎2-14: Multi-stage model monitoring

Study multi-stage monitoring approach where a low-overhead low-accuracy method triggers a more accurate intermediate-KPI based solution with higher overhead.

# CSI Prediction

## Potential Specification Impact

In the last RAN1 meeting, time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement. Although the potential specification impact has been deferred until RAN1#112b-e, we still provide our ideas in this document.

Several alternatives to handle CSI prediction exists and each would have their own specification impact. Since it is decided to apply the CSI prediction on the UE side, it is important to decide on how the CSI feedback to gNB is executed:

- Option 1: UE only feedback one instance CSI. In this case, on example is that gNB configures the UE with a prediction target of X milliseconds in the future (which is likely to be subject to UE capability). Then UE can use the following mechanism to compress the predicted CSI:

  - Existing codebook: The UE may feedback PMI/RI/CQI information using pre-Rel18 codebook (e.g., Rel-16 e-Type II codebook) to minimize the specification impact.

  - AI/ML assistance: The UE may compress the raw CSI channel using an AI/ML-based CSI compression approach. The spatial-frequency domain CSI compression can be used in this scenario.

- Option 2: UE feedback multiple instances of future CSI. In this case, we need to take the time domain fluctuations into account. Then UE can use the following mechanism to compress the predicted CSI:

  - Existing codebook: The UE may feedback the PMI/RI/CQI information using Rel-18 codebook (with Doppler domain information) to minimize the specification impact. Although the Rel-18 codebook is still developed in the Rel-18 MIMO agenda, it can be considered as an optional baseline scheme.

  - AI/ML assistance: The UE may compress the raw CSI channel using an AI/ML-based CSI compression approach. The temporal-spatial-frequency domain CSI compression can be used in this scenario.

For AI/ML-based CSI prediction, discuss the potential specification impact of CSI feedback mechanism. Codebook-based feedback can be used as a baseline (legacy codebook or Rel-18 codebook), but AI/ML-based CSI compression is not precluded.

## Potential Procedure

In this section, we proposed the potential procedure of CSI prediction. This procedure supports the periodic CSI-RS, semi-persistent CSI-RS, and aperiodic CSI-RS configuration. Before moving on the detailed procedure, the configuration of observation window and prediction window need to be clarified. Figure ‎3-1 shows the illustration of observation window and prediction window, which are composed of the number of CSI-RS instances and the time interval. It should be noted that the observation interval and the prediction interval can be different. For example, the observation interval is 4 slots in the figure, and the prediction interval is 1 slot. The number of observation instances and prediction instances are selected from a set of specified numbers, e.g., {4, 6, 8, 10, 15} samples. In addition, the observation interval and prediction interval are also selected from a set of specified numbers, e.g., {1, 4, 5} slots. These values can be configured by gNB or feedback by UE itself.

Figure ‎3-1: Illustration of observation window and prediction window

In addition to the signalling exchange between the UE side and the base station (BS) side, the AI cloud server is also included to handle more complex operations, such as model training and fine-tuning. The AI cloud servers can be deployed by the UE vendors or chipset vendors, depending on the implementation of different companies.

Figure ‎3-2 shows the potential procedure of AI/ML-based CSI prediction. The whole process includes the training and the inference stages, and is divided into ten steps:

- UE reports the maximum prediction length (e.g., 20 slots) based on its own capability. If the value is 0, it means no CSI prediction algorithm or corresponding AI/ML model in the UE side. If gNB receives a zero value, it will not trigger the CSI prediction process.

- In RRC connected mode, gNB configures the observation windows and prediction windows that UE can provide in the CSI feedback. The gNB may determine the parameters according to the environment.

- UE may decide whether to do the CSI prediction based on the following indicators:

  - CSI-RS periodicity (Ex. Not trigger CSI prediction in long CSI-RS periodicity, i.e., period > 8 slots)

  - UE velocity (Ex. Not trigger CSI prediction in low-speed scenario, i.e., UE speed < 20km/h)

- UE updates the observation window and the prediction window to AI cloud server based on BS configurations.

- In training stage: UE collect the training data, then transfer the training dataset to the AI cloud server. Or AI loud server can collect data by itself, so UE only needs to give an activation signal instead of transmitting a large amount of data

- In training stage: Model training or fine-tuning process is conducted in the AI cloud server.

- AI cloud server deliver the trained model to the UE (the format can be the ONNX format). Then UE stores the AI/ML model corresponding to the configured observation window and prediction window. If the memory size in the UE is sufficient, the UE can store many models of different observation/prediction window sets. It can avoid frequent retraining of the model.

- UE feedback an indicator (e.g., 1-bit) to show whether UE has the capability to perform the CSI prediction process.

  - If the indicator is 0: UE decline the request for the CSI prediction (the conditions are unsuitable or unworthy of UE to do CSI prediction, i.e., the UE speed is too low)

  - If the indicator is 1: UE has the capability to do the CSI prediction

- BS then configure the signalling to UE to activate/deactivate/modify (fine-tuning) the AI/ML model of CSI prediction.

- After UE receives the signalling, it provides the corresponding action:

- Active: Run the CSI prediction in inference stage and provide the results in next CSI feedback period

- Deactivate: Stop the AI/ML-based CSI prediction

- Modify: Re-collect the data and send it to AI cloud server for model fine-tuning

Figure ‎3-2: Procedure of AI/ML-based CSI prediction

In this procedure, we can observe that it needs the data collection and model transfer between the UE and the AI cloud server. Therefore, a unified interface is needed between them. The LCM related discussion can be left for the high-level discussion. In this section, we can focus on the potential specification impact of the procedure.

For AI/ML-based CSI prediction, discuss the potential specification impact of the signalling between UE and gNB.

# Conclusion

In summary, based on the above discussion, we have the following proposals:

- Categorize type 1 training into “type 1-training at UE” and “type 1-training at gNB”, and discuss the spec impact on each separately.

Discuss potential spec impact on exchangeable part of model in the inference phase including:

- Model format, pre/post-processing, quantization, input/output format.

- Signalling format to request and upload/download exchangeable part of AI/ML model

- Related UE capability

Discuss sharing full AI/ML model or only the exchangeable part of AI/ML model to non-training entity for performance monitoring.

For training type 2, discuss alignment of quantization/dequantization as well as format/precision of gradient vectors, latent vectors, and CSI samples.

For single-encoder multi-decoder setting in training type 2, UE should not break down the training session into multiple single-encoder single-decoder training sub-sessions.

In training type 2 for multi-encoder setting, if UE-specific datasets are used, the type of target CSI should be aligned among UE vendors.

Discuss feasibility of synchronization/alignment required for different update scheduling in training type 2.

If UE-specific datasets are used for multi-encoder training, consider sharing information on training-related parameters such as size of datasets, statistics of datasets, training loss, and update schedule.

Consider sharing information about encoders’/decoders’ architecture type and complexity from entities doing training first to other entities.

Prioritize to study the training-aware quantization methods

Study alignment requirement and influence of different training awareness techniques for enabling backpropagation between quantizers and dequantizers.

For training type 2, gNB should inform UE about the training awareness  technique used for its dequantizer.

For VQ, UE and gNB should align their codebook and segmentation approach.

Prioritize SQ methods over VQ.

For describing a quantization method at UE and gNB, use training awareness, learnability, and mapping (codeword assignment) as its main attributes.

For training type 2, discuss spec impact, performance impact, and possible alignment between latent and gradient quantization.

While gNB is main entity in establishing data collection procedure, UE should provide gNB with a range of possible options for configurations of the data collection procedure including but not limited to:

- Types of input CSI

- Types of assistant information

- Quantization parameters

- Periodicity of data collection

- Maximum amount of data collected per period

Discuss the quantization of data in the following aspects:

- Decisioning entity

- Quantizable information (CSI samples and assistant information)

- Configuration changes (per sample or per dataset)

Study spec impact, signalling requirements, and candidate representative information of AI/ML models for activation, deactivation, switching, and fallback.

Discuss methods and apparatus for monitoring AI/ML models other than the one which is already being used by UE and gNB.

Given promising features of input/output-based monitoring, accuracy of possible solutions shall be further studied.

Prioritize UE-side (Alternative 1) proxy-based model monitoring as the initial monitoring method for tracking intermediate KPI.

System-level indicators cannot be regarded as the single point of decisioning for detection of monitoring events.

Study multi-stage monitoring approach where a low-overhead low-accuracy method triggers a more accurate intermediate-KPI based solution with higher overhead.

For AI/ML-based CSI prediction, discuss the potential specification impact of CSI feedback mechanism. Codebook-based feedback can be used as a baseline (legacy codebook or Rel-18 codebook), but AI/ML-based CSI compression is not precluded.

For AI/ML-based CSI prediction, discuss the potential specification impact of the signalling between UE and gNB.

# References

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R1-2210753, “Summary#7 of [110bis-e-R18-AI/ML-02]”, Moderator (Huawei), Oct. 2022.

- R1-2212227, “Evaluation on AI/ML for CSI feedback enhancement”, MediaTek Inc., TSG-RAN WG1 Meeting #111, Toulouse, France, Nov. 2022.

- R1-2301587, “Evaluation on AI/ML for CSI feedback enhancement”, MediaTek Inc., TSG-RAN WG1 Meeting #112, Athens, Greece, Feb. 2023.

- R1-2210886, “Discussion on AI/ML for CSI feedback enhancement”, Huawei, TSG-RAN WG1 Meeting #111, Toulouse, France, Nov. 2022.

- R1-2212109, “Other aspects on AI/ML for CSI feedback enhancement”, Qualcomm Incorporated, TSG-RAN WG1 Meeting #111, Toulouse, France, Nov. 2022.